SECTION 07210

BATT INSULATION SYSTEM

PART 1 GENERAL

1.1 REFERENCES

1.2 DELIVERY AND STORAGE

PART 2 PRODUCTS

2.1 ASPHALT

2.2 ASPHALT PRIMER

2.3 SURFACE PROTECTION

PART 3 EXECUTION

3.1 SURFACE PROTECTION

3.2 APPLICATION

3.3 PROTECTIVE COVERING

# GENERAL

## referance

#### ASTM-D226.

#### ASTM-D227.

#### ASTM-D41.

#### ASTM-D4263.

#### ASTM-D43.

#### ASTM-D449

## Delivery and storage

### Deliver materials in sealed containers bearing manufacturer’s original labels. Labels shall include date of manufacture, contains of each container, performance standards that apply to the contents and recommended shelf life.

Specifier:  For California work use following:

- Comply with California quality assurance for insulating material.

Specifier:  Verify R value with mechanical.  Verify R value with products in Part 2.  If you have varying R values for exterior walls indicate here or delete this paragraph and indicate for each material.  Many states are adopting energy codes which dictate R value for walls and roofs.  Typical minimums are <RSI/m-1.8> {R-10} walls and up to <RSI/m-5.3> {R-30}roofs.  Verify.

- Insulation value exterior walls: RSI/m-1.8 R-10.

- Certification of minimum 20 percent recovered material content in fiberglass insulation.

- Certification of minimum 9 percent recovered material content in rigid plastic foams.

- Certification of minimum 75 percent recovered material content in cellulose insulation.

# PRODUCTS

## Asphalt

### ASTM-D449, Type I or Type II.

Specifier:  See Division 1 for definition of base and optional manufacturers.

Specifier:  UC Industries has insulation with <103, 172, 276 and 414 kPa>  {15, 25, 40 and 60 PSI} compressive strengths. Dow has <103, 172, 276, 414 and 758 kPa> {15, 25, 40, 60, 110 PSI} Decide which strength is required and insert.  Use <172> {25} for perimeter insulation.  Use <276> {40} or higher for plaza deck type use.  Do not use beadboard (molded foam.  The performance characteristics of extruded polystyrene are generally the best in the industry both in terms of retained "R" values and resistance to moisture absorption.  Extruded board specified here is generally for cavity wall or below grade installations.  Insulation for roofing is included in the appropriate roofing sections.

Foam plastic insulation may require a thermal barrier, consult applicable codes. Foam plastic insulation use may be restricted where termites may be present, consult applicable codes.

Green Use Discussion:

Green Use Discussion: Manufacturers of extruded board products still manufacture their boardstock products with HCFCs even though Tenneco and Dow use non-HCFC blowing agents for their fan baord.  Although the technology is available their strategy seems to be to adhere to the timetables set by the Montreal Protocol as oppossed to making a more aggressive commitment to phase out ozone depleting substances.  Styrene and dioxin (both carcinogens) emissions are also manufacturing problems. Currently (1997) only Tenneco uses 50 percent recycled content material in it's product, (AMOFOAM-RCX), but the R-value is 60 percent less than conventional extruded boardstock.  The EPA CPG program requires a minimum of 9 percent of total recoverd materials in rigid platic insulation.

- Minimum thickness: ___ mm ___ IN.

- Minimum thickness: As indicated.

- Minimum 9 percent total recovered material content per EPA/CPG guidelines.

Specifier:  The manufacture of expaned polystyrene does not use CFCs or HCFC's or contribute to ozone depletion.  Consider it when green factors are important.  Expanded does not have equal R value as extruded and will loose insulating characteristics when exposed to water.  While the manufacture of expanded board does not contibute to ozone depletion it does contribute to ground level smog.  An extensive recycling network has grown up around the use of expanded board.  This product should not be used below grade.

Foam plastic insulation mat require a thermal barrier, consult applicable codes. Foam plastic insulation use may be restricted where termites may be present, consult applicable codes.

- Insulation, rigid board:

- Molded (expanded) polysytrene.

- UL and FM listed for assembly indicated.

- Thickness: To provide R value indicated.

- Minimum 24 kg/m2 1.25 PCF. 5 Minimum 9 percent total recovered material content per EPA/CPG guidelines.

Specifier:  If batt insulation is used in exterior metal stud walls, use blanket or batt with no integral vapor retarder, with separate vapor retarder installed over it. Insulation <405 or 610 mm> {16 or 24 IN} wide for friction fitting in metal stud systems DOES NOT come with <25 mm> {1 IN} flanges on its vapor retarder.  Typical R values : <75 mm> {3 IN} = <RSI/m-1.9> {R-11}, <88 mm> {3-1/2 IN} = <RSI/m-2.3> {R-13}, <135 mm> {5-1/4 IN} = <RSI/m-3.5> {R-19}, <150 mm> {6 IN} = <RSI/m-3.9> {R-22}.  Select thickness.

Green Use Discussion:: Fiberglass  is currently listed by the Department of Health and Human Services as a possible carcinogen based on laboratory studies and emperical data. The North American Insulation Manufacturers Association, (NAIMA), has challenged these findings and has commissioned it's own studies in response.  At issue is fiberglass's similar physical characteristics to asbestos. From a practical standpoint, it is recognized that most fiberglass-health related problems occur when loose material is blown in attic or plenum spaces.  When fiberglass is manufactured with binders or "laminated" to face sheets the risk is not nearly as serious. .  Two companies are currenlty manufacturing formadehyde-free fiberglass insulation by engineereing the fiber properties so that a binder is unnecessary.  Also, the Johns Manville product, Miraflex, does not contain any unnecessary dyes or additives in the product.

.  Under EPA Comprehensive Procurement Guidelines (CPG) fiberglass is available with 20 to 25 percent total recovered materials content.  Owens Corning claims 30% recycled content with 4 percent post-consumer content and has been certified by the Scientific Certification System for recycled content.  Both Owens-Corning and Johns Manville are EPA Energy Star Partners. Verify that submittal requires certification.

Specifier:  Use for precast concrete.

- Insulation, rigid fiber:

- Inorganic fibers and resinous binders formed into semi-rigid blankets.

- Minimum thickness: 75 mm 3 IN.

- ASTM-E84 flame spread: Not greater than 25.

- Unfaced:

- 405 mm 16 IN wide.

- 610 mm 24 IN wide.

- Minimum 25 percent total recovered material content per EPA/CPG guidelines.

- Owens-Corning 700 Series.

Specifier:  Material specified for spandrel panels is same for curtainwall but should only be specified in this section for non-curtainwall applications such as spandrels in storefronts.  Curtainwall insulation that is part of a UL approved assembly is specified in Section 07270.

## Asphalt Primer

Specifier:  The AIA Environmental Resource Guide regards cellulose insulation as the "premier building thermal/ acoustical from an environmental perspective".  Cellulose insulation is manufactured from 75 to 80 percent recovered material, most of which is post-consumer.  Non-toxic additives are added to create permanent flame resistance.  Material also has better sound control properties than fiberglass.  The optimum application is for pneumatic spray-applied that forms a monolithic coating in the cavity that conforms to any substrate such as pipes and outlet boxes while covering cracks and forming a highly efficient thermal barrier.  Greenstone is a Energy Star partner.

### ASTM-D41

## Surface protection

### Protection board

### 1.Wood Fiber Board, ASTM-C208 or Perlite Board, ASTM C728.

Specifier:  Where vapor retarder abuts steel structure with fireproofing, or curtainwall system sheet metal vapor retarder may be required.  Indicate size on exterior detail or specify.

- Sheet metal vapor retarder:

- Minimum 0.6 mm 24 GA galvanized steel bent angle; size as required.

- Provide where vapor retarder abuts steel structure.

- Provide where vapor retarder abuts curtainwall.

Specifier:  This is a system to use on masonry or concrete walls where you want insulation with minimum loss of space in room.  Foam insulation is adhered to wall and stud strips (which look like mini-floor runners for GYPSUM WALLBOARD) are embedded into foam with fasteners shot thru them to hold insulation in space and to give surface to screw GYPSUM WALLBOARD to.  Verify insulation thickness for R value. TGIF system (subparagraph 5) gives better vapor transmission factor.  If electrical outlets are required in these walls, make sure electrical uses shallow boxes.

Specifier:  Add mechanical fasteners and mastic if required. Normally only used with precast and non-rigid insulations. Could be used in stud walls, but normally not.

- Mechanical fasteners:

- Pronged hangers and slotted washers or arrow pointed hangers.

- Sized to fit insulation thickness.

- Base: GEMCO (Goodloe E. Moore,); Gemco pronged hanger with WP washer.

- Optional: Stic-Klip Manufacturing.

- Mastic for use with mechanical fasteners: GEMCO (Goodloe E. Moore.)

Specifier:  ASTM-C516, Type II, Grade-0.  Primary usage is for waterproofing backside of precast, add if you have precast.

- Waterproof coating:

- Cement base, aggregate type.

- Harris Specialty Chemical, Thoro System Products, Thoroseal containing "Acryl 60".

# EXECUTION

## Surface protection

### Apply to clean dry surfaces. Moisture test in accordance with ASTM-D4263. If test indicates moisture, allow a minimum of 7 additional days after test completion for curing. If moisture still exist, redo test until substrate is dry.

## application

### Apply after priming coat is dry, but prior to any deterioration of primed surface, and when ambient temperature is above 4 degrees C 40 degree F.

### apply primer when ambient temperature is above 4 degrees C 40 degrees F and at rate of approximately four liters per 10 square meters one gallon per 100 square feet, fully covering entire surface to be dampproofed.

## protective covering

### Protect dampproofed surfaces against which backfill will be placed with.