FIFTEEN DOLLAR BOX

**Note, this system is very simple to build.  However, it does not work on all cable systems.

**It is designed for systems using old 1980’s technology.  If your cable system is behind the times,

the GO FOR IT!  If they have upgraded, then this will not work for you.    1980’s cable systems are characterized by a pulsating fuzzy picture and a beeping sound.

STEP-BY-STEP INSTRUCTIONS

1. Remove the plastic cover from the No.12 gauge solid copper wire (if any) and twist around a

3/8" nail or drill bit (the diameter is crucial)  to form a coil of 9 turns. (actually 9.5 turns to get both ends     of the coil facing the same direction.) Elongate coil to a length of 1 1/2" inches and form right angle bends on each end.

2.  Solder the variable capacitor to the coil.  It doesn't matter where

you solder it, it still does the same job.  The best place for it is in

the center with the adjustment screw facing upward.  Note: When it

comes time to place coil in box, the coil must be insulated from

grounding.  This can be done by crazy-gluing a piece of rubber to

the bottom of the box, and securing the coil to it.

3.  Attach chassis connectors center leads to the coil at points 2 1/2 turns from

each end of the coil.   Be sure to  solder them straight so that they will mount straight in the box.

4.  Solder resistor to center of coil and ground other end of resistor to

chassis box, using solder lug and small screw.

5.  Drill a 3/8" diameter hole in mini-box cover to permit adjustment

of the variable capacitor from the outside.  Inspect the device for

defects in workmanship and place cover on mini-box.  Tighten

securely.

6.  Place device in line with existing cable on either side of the

converter box and connect to television set with the short piece of

RG59 coaxial cable.  Set television set to HBO channel.

7.  Using a plastic screwdriver (non-metallic) adjust the variable

capacitor until picture tunes in. Sit back, relax, and enjoy!  ****ALSO: YOU MAY HAVE TO

EXPAND AND CONTRACT THE COIL TO GET A CLEAN PICTURE.

8.  Do not be discouraged if you can not get a clean channel.  In different areas HBO may be on a different channel.  This device will Descrambler one of your pay channels!  *

WARNING!!!

WHILE IT IS PERFECTLY LEGAL TO SELL, BUILD,  AND OWN A CABLE DESCRAMBLER,

IT IS ILLEGAL TO USE A DESCRAMBLER UNLESS YOU ARE PAYING YOUR LOCAL CABLE PROVIDER FOR THE CHANNEL YOU ARE DESCRAMBLING!!!!!!!!!!

I TAKE NO RESPONSIBILITY FOR ANYONE THAT IS PROSECUTED FOR ILLEGAL USE OF THE DEVICE  THESE PLANS ARE DESIGNED TO CREATE!

MATERIALS REQUIRED:

Quantity

1   	  - Radio Shack mini-box (RS #270-235) “numbers in brackets are RADIOSHACK part

numbers.”

1   	  - 1/4 watt resistor, 2.2k-2.4k ohm (RS #271-1325)

2   	  - F61a chassis-type coaxial connectors (RS #278-212)

12"	  - No. 12 solid copper wire

1   - 75pf-100pf variable capacitor  (YOU MAY USE ANY VARIABLE CAPACITOR AS LONG  AS IT INCLUDES THE RANGES SPECIFIED.  ((YOU WILL PROBABLY NEED TO GO TO YOUR LOCAL ELECTRONICS STORE FOR THIS PART.  RADIOSHACK DOES NOT STOCK IT.  HOWEVER THEY CAN ORDER IT FOR YOU.))  If you are unable to locate it, you may call DIGI-KEY at 1-800-344-4539.  (COST is 4.88 ea.)