Network Working Group                                     S. Waldbusser

Request for Comments: 2021                                          INS

Category: Standards Track                                  January 1997

Remote Network Monitoring Management Information Base

Version 2

using SMIv2

Status of this Memo

This document specifies an Internet standards track protocol for the

Internet community, and requests discussion and suggestions for

improvements.  Please refer to the current edition of the "Internet

Official Protocol Standards" (STD 1) for the standardization state

and status of this protocol.  Distribution of this memo is unlimited.

Abstract

This memo defines a portion of the Management Information Base (MIB)

for use with network management protocols in TCP/IP-based internets.

In particular, it defines objects for managing remote network

monitoring devices.

Table of Contents

1 The Network Management Framework ......................    2

2 Overview ..............................................    2

2.1 Remote Network Management Goals .....................    3

2.2 Structure of MIB ....................................    5

3 Control of Remote Network Monitoring Devices ..........    6

3.1 Resource Sharing Among  Multiple  Management  Sta-

tions ..............................................    7

3.2 Row Addition Among Multiple Management Stations .....    9

4 Conventions ...........................................   10

5 RMON 2 Conventions ....................................   10

5.1 Usage of the term Application Level .................   10

5.2 Protocol Directory and Limited Extensibility ........   11

5.3 Errors in packets ...................................   11

6 Definitions ...........................................   12

7 Security Considerations ...............................  122

8 Appendix - TimeFilter Implementation Notes ...........   123

9 Acknowledgments ......................................   129

10 References ...........................................  129

11 Author's Address......................................  130

Waldbusser                  Standards Track                     [Page 1]

RFC 2021             Remote Network Monitoring MIB          January 1997

1.  The Network Management Framework

The Internet-standard Network Management Framework consists of three

components.  They are:

RFC 1902 [1] which defines the SMI, the mechanisms used for

describing and naming objects for the purpose of management.

RFC 1213, STD 17, [3] which defines MIB-II, the core set of

managed objects for the Internet suite of protocols.

RFC 1905 [4] which defines the SNMP, the protocol used for

network access to managed objects.

The Framework permits new objects to be defined for the purpose of

experimentation and evaluation.

Managed objects are accessed via a virtual information store, termed

the Management Information Base or MIB.  Within a given MIB module,

objects are defined using the SMI's OBJECT-TYPE macro.  At a minimum,

each object has a name, a syntax, an access-level, and an

implementation-status.

The name is an object identifier, an administratively assigned name,

which specifies an object type.  The object type together with an

object instance serves to uniquely identify a specific instantiation

of the object.  For human convenience, we often use a textual string,

termed the object descriptor, to also refer to the object type.

The syntax of an object type defines the abstract data structure

corresponding to that object type.  The ASN.1 [6] language is used

for this purpose.  However, RFC 1902 purposely restricts the ASN.1

constructs which may be used.  These restrictions are explicitly made

for simplicity.

The access-level of an object type defines whether it makes "protocol

sense" to read and/or write the value of an instance of the object

type.  (This access-level is independent of any administrative

authorization policy.)

The implementation-status of an object type indicates whether the

object is mandatory, optional, obsolete, or deprecated.

2.  Overview

This document continues the architecture created in the RMON MIB [RFC

1757] by providing a major feature upgrade, primarily by providing

RMON analysis up to the application layer.

Waldbusser                  Standards Track                     [Page 2]

RFC 2021             Remote Network Monitoring MIB          January 1997

Remote network monitoring devices, often called monitors or probes,

are instruments that exist for the purpose of managing a network.

Often these remote probes are stand-alone devices and devote

significant internal resources for the sole purpose of managing a

network.  An organization may employ many of these devices, one per

network segment, to manage its internet.  In addition, these devices

may be used for a network management service provider to access a

client network, often geographically remote.

The objects defined in this document are intended as an interface

between an RMON agent and an RMON management application and are not

intended for direct manipulation by humans.  While some users may

tolerate the direct display of some of these objects, few will

tolerate the complexity of manually manipulating objects to

accomplish row creation.  These functions should be handled by the

management application.

2.1.  Remote Network Management Goals

o Offline Operation

There are sometimes conditions when a management

station will not be in constant contact with its

remote monitoring devices.  This is sometimes by

design in an attempt to lower communications costs

(especially when communicating over a WAN or

dialup link), or by accident as network failures

affect the communications between the management

station and the probe.

For this reason, this MIB allows a probe to be

configured to perform diagnostics and to collect

statistics continuously, even when communication with

the management station may not be possible or

efficient.  The probe may then attempt to notify

the management station when an exceptional condition

occurs.  Thus, even in circumstances where

communication between management station and probe is

not continuous, fault, performance, and configuration

information may be continuously accumulated and

communicated to the management station conveniently

and efficiently.

Waldbusser                  Standards Track                     [Page 3]

RFC 2021             Remote Network Monitoring MIB          January 1997

o Proactive Monitoring

Given the resources available on the monitor, it

is potentially helpful for it continuously to run

diagnostics and to log network performance.  The

monitor is always available at the onset of any

failure.  It can notify the management station of the

failure and can store historical statistical

information about the failure.  This historical

information can be played back by the management

station in an attempt to perform further diagnosis

into the cause of the problem.

o Problem Detection and Reporting

The monitor can be configured to recognize

conditions, most notably error conditions, and

continuously to check for them.  When one of these

conditions occurs, the event may be logged, and

management stations may be notified in a number of

ways.

o Value Added Data

Because a remote monitoring device represents a

network resource dedicated exclusively to network

management functions, and because it is located

directly on the monitored portion of the network, the

remote network monitoring device has the opportunity

to add significant value to the data it collects.

For instance, by highlighting those hosts on the

network that generate the most traffic or errors, the

probe can give the management station precisely the

information it needs to solve a class of problems.

o Multiple Managers

An organization may have multiple management stations

for different units of the organization, for different

functions (e.g. engineering and operations), and in an

attempt to provide disaster recovery.  Because

environments with multiple management stations are

common, the remote network monitoring device has to

deal with more than own management station,

potentially using its resources concurrently.

Waldbusser                  Standards Track                     [Page 4]

RFC 2021             Remote Network Monitoring MIB          January 1997

2.2.  Structure of MIB

The objects are arranged into the following groups:

- protocol directory

- protocol distribution

- address mapping

- network layer host

- network layer matrix

- application layer host

- application layer matrix

- user history

- probe configuration

These groups are the basic units of conformance.  If a remote

monitoring device implements a group, then it must implement all

objects in that group.  For example, a managed agent that implements

the network layer matrix group must implement the nlMatrixSDTable and

the nlMatrixDSTable.

Implementations of this MIB must also implement the system and

interfaces group of MIB-II [3].  MIB-II may also mandate the

implementation of additional groups.

These groups are defined to provide a means of assigning object

identifiers, and to provide a method for managed agents to know which

objects they must implement.

This document also contains enhancements to tables defined in the

RMON MIB [RFC 1757].  These enhancements include:

1) Adding the DroppedFrames and LastCreateTime

conventions to each table defined in the RMON MIB.

2) Augmenting the RMON filter table with a mechanism

that allows filtering based on an offset from the

beginning of a particular protocol, even if the

protocol headers are variable length.

Waldbusser                  Standards Track                     [Page 5]

RFC 2021             Remote Network Monitoring MIB          January 1997

3) Augmenting the RMON filter and capture status bits

with additional bits for WAN media and generic media.

These bits are defined here as:

Bit     Definition

6       For WAN media, this bit is set for packets

coming from one direction and cleared for

packets coming from the other direction.

It is an implementation specific matter

as to which bit is assigned to which

direction, but it must be consistent for

all packets received by the agent, and if

the agent knows which end of the link is

"local" and which end is "network", the bit

should be set for packets from the "local"

side and should be cleared for packets from

the "network" side.

7       For any media, this bit is set for any packet

with a physical layer error. This bit may be

set in addition to other media-specific bits

that denote the same condition.

8       For any media, this bit is set for any packet

that is too short for the media. This bit may

be set in addition to other media-specific

bits that denote the same condition.

9       For any media, this bit is set for any packet

that is too long for the media. This bit may

be set in addition to other media-specific bits

that denote the same condition.

These enhancements are implemented by RMON-2 probes that also

implement RMON and do not add any requirements to probes that are

compliant to just RMON.

3.  Control of Remote Network Monitoring Devices

Due to the complex nature of the available functions in these

devices, the functions often need user configuration.  In many cases,

the function requires parameters to be set up for a data collection

operation.  The operation can proceed only after these parameters are

fully set up.

Many functional groups in this MIB have one or more tables in which

to set up control parameters, and one or more data tables in which to

place the results of the operation.  The control tables are typically

read/write in nature, while the data tables are typically read/only.

Waldbusser                  Standards Track                     [Page 6]

RFC 2021             Remote Network Monitoring MIB          January 1997

Because the parameters in the control table often describe resulting

data in the data table, many of the parameters can be modified only

when the control entry is not active.  Thus, the method for modifying

these parameters is to de-activate the entry, perform the SNMP Set

operations to modify the entry, and then re-activate the entry.

Deleting the control entry causes the deletion of any associated data

entries, which also gives a convenient method for reclaiming the

resources used by the associated data.

Some objects in this MIB provide a mechanism to execute an action on

the remote monitoring device.  These objects may execute an action as

a result of a change in the state of the object.  For those objects

in this MIB, a request to set an object to the same value as it

currently holds would thus cause no action to occur.

To facilitate control by multiple managers, resources have to be

shared among the managers.  These resources are typically the memory

and computation resources that a function requires.

3.1.  Resource Sharing Among Multiple Management Stations

When multiple management stations wish to use functions that compete

for a finite amount of resources on a device, a method to facilitate

this sharing of resources is required.  Potential conflicts include:

o Two management stations wish to simultaneously use

resources that together would exceed the capability of

the device.

o A management station uses a significant amount of

resources for a long period of time.

o A management station uses resources and then crashes,

forgetting to free the resources so others may

use them.

The OwnerString mechanism is provided for each management station

initiated function in this MIB to avoid these conflicts and to help

resolve them when they occur.  Each function has a label identifying

the initiator (owner) of the function.  This label is set by the

initiator to provide for the following possibilities:

o A management station may recognize resources it owns

and no longer needs.

o A network operator can find the management station that

owns the resource and negotiate for it to be freed.

o A network operator may decide to unilaterally free

resources another network operator has reserved.

Waldbusser                  Standards Track                     [Page 7]

RFC 2021             Remote Network Monitoring MIB          January 1997

o Upon initialization, a management station may recognize

resources it had reserved in the past.  With this

information it may free the resources if it no longer

needs them.

Management stations and probes should support any format of the owner

string dictated by the local policy of the organization.  It is

suggested that this name contain one or more of the following: IP

address, management station name, network manager's name, location,

or phone number.  This information will help users to share the

resources more effectively.

There is often default functionality that the device or the

administrator of the probe (often the network administrator) wishes

to set up.  The resources associated with this functionality are then

owned by the device itself or by the network administrator, and are

intended to be long-lived.  In this case, the device or the

administrator will set the relevant owner object to a string starting

with 'monitor'.  Indiscriminate modification of the monitor-owned

configuration by network management stations is discouraged.  In

fact, a network management station should only modify these objects

under the direction of the administrator of the probe.

Resources on a probe are scarce and are typically allocated when

control rows are created by an application.  Since many applications

may be using a probe simultaneously, indiscriminate allocation of

resources to particular applications is very likely to cause resource

shortages in the probe.

When a network management station wishes to utilize a function in a

monitor, it is encouraged to first scan the control table of that

function to find an instance with similar parameters to share.  This

is especially true for those instances owned by the monitor, which

can be assumed to change infrequently.  If a management station

decides to share an instance owned by another management station, it

should understand that the management station that owns the instance

may indiscriminately modify or delete it.

It should be noted that a management application should have the most

trust in a monitor-owned row because it should be changed very

infrequently.  A row owned by the management application is less

long-lived because a network administrator is more likely to re-

assign resources from a row that is in use by one user than from a

monitor-owned row that is potentially in use by many users.  A row

owned by another application would be even less long-lived because

the other application may delete or modify that row completely at its

discretion.

Waldbusser                  Standards Track                     [Page 8]

RFC 2021             Remote Network Monitoring MIB          January 1997

3.2.  Row Addition Among Multiple Management Stations

The addition of new rows is achieved using the RowStatus method

described in RFC 1903 [2].  In this MIB, rows are often added to a

table in order to configure a function.  This configuration usually

involves parameters that control the operation of the function.  The

agent must check these parameters to make sure they are appropriate

given restrictions defined in this MIB as well as any implementation

specific restrictions such as lack of resources.  The agent

implementor may be confused as to when to check these parameters and

when to signal to the management station that the parameters are

invalid.  There are two opportunities:

o When the management station sets each parameter object.

o When the management station sets the row status object

to active.

If the latter is chosen, it would be unclear to the management

station which of the several parameters was invalid and caused the

badValue error to be emitted.  Thus, wherever possible, the

implementor should choose the former as it will provide more

information to the management station.

A problem can arise when multiple management stations attempt to set

configuration information simultaneously using SNMP.  When this

involves the addition of a new conceptual row in the same control

table, the managers may collide, attempting to create the same entry.

To guard against these collisions, each such control entry contains a

status object with special semantics that help to arbitrate among the

managers.  If an attempt is made with the row addition mechanism to

create such a status object and that object already exists, an error

is returned.  When more than one manager simultaneously attempts to

create the same conceptual row, only the first will succeed.  The

others will receive an error.

In the RMON MIB [RFC 1757], the EntryStatus textual convention was

introduced to provide this mutual exclusion function.  Since then,

this function was added to the SNMP framework as the RowStatus

textual convention.  The RowStatus textual convention is used for the

definition of all new tables.

When a manager wishes to create a new control entry, it needs to

choose an index for that row.  It may choose this index in a variety

of ways, hopefully minimizing the chances that the index is in use by

another manager.  If the index is in use, the mechanism mentioned

previously will guard against collisions.  Examples of schemes to

choose index values include random selection or scanning the control

Waldbusser                  Standards Track                     [Page 9]

RFC 2021             Remote Network Monitoring MIB          January 1997

table looking for the first unused index.  Because index values may

be any valid value in the range and they are chosen by the manager,

the agent must allow a row to be created with any unused index value

if it has the resources to create a new row.

Some tables in this MIB reference other tables within this MIB.  When

creating or deleting entries in these tables, it is generally

allowable for dangling references to exist.  There is no defined

order for creating or deleting entries in these tables.

4.  Conventions

The following conventions are used throughout the RMON MIB and its

companion documents.

Good Packets

Good packets are error-free packets that have a valid frame length.

For example, on Ethernet, good packets are error-free packets that

are between 64 octets long and 1518 octets long.  They follow the

form defined in IEEE 802.3 section 3.2.all.

Bad Packets

Bad packets are packets that have proper framing and are therefore

recognized as packets, but contain errors within the packet or have

an invalid length.  For example, on Ethernet, bad packets have a

valid preamble and SFD, but have a bad CRC, or are either shorter

than 64 octets or longer than 1518 octets.

5.  RMON 2 Conventions

The following practices and conventions are introduced in the RMON 2

MIB.

5.1.  Usage of the term Application Level

There are many cases in this MIB where the term Application Level is

used to describe a class of protocols or a capability.  This does not

typically mean a protocol that is an OSI Layer 7 protocol.  Rather,

it is used to identify a class of protocols that is not limited to

MAC-layer and network-layer protocols, but can also include

transport, session, presentation, and application-layer protocols.

Waldbusser                  Standards Track                    [Page 10]

RFC 2021             Remote Network Monitoring MIB          January 1997

5.2.  Protocol Directory and Limited Extensibility

Every RMON 2 implementation will have the capability to parse certain

types of packets and identify their protocol type at multiple levels,

The protocol directory presents an inventory of those protocol types

the probe is capable of monitoring, and allows the addition,

deletion, and configuration of protocol types in this list.

One concept deserves special attention: the "limited extensibility"

of the protocol directory table.  The RMON 2 model is that protocols

are detected by static software that has been written at

implementation time.  Therefore, as a matter of configuration, an

implementation does not have the ability to suddenly learn how to

parse new packet types.  However, an implementation may be written

such that the software knows where the demultiplexing field is for a

particular protocol, and can be written in such a way that the

decoding of the next layer up is table-driven.  This works when the

code has been written to accomodate it and can be extended no more

than one level higher.  This extensibility is called "limited

extensibility" to highlight these limitations.  However, this can be

a very useful tool.

For example, suppose that an implementation has C code that

understands how to decode IP packets on any of several ethernet

encapsulations, and also knows how to interpret the IP protocol field

to recognize UDP packets and how to decode the UDP port number

fields.  That implementation may be table- driven so that among the

many different UDP port numbers possible, it is configured to

recognize 161 as SNMP, port 53 as DNS, and port 69 as TFTP.  The

limited extensibility of the protocol directory table would allow an

SNMP operation to create an entry that would create an additional

table mapping for UDP that would recognize UDP port 123 as NTP and

begin counting such packets.

This limited extensibility is an option that an implementation can

choose to allow or disallow for any protocol that has child

protocols.

5.3.  Errors in packets

Packets with link-level errors are not counted anywhere in this MIB

because most variables in this MIB requires the decoding of the

contents of the packet, which is meaningless if there is a link-level

error.

Packets in which protocol errors are detected are counted for all

protocols below the layer in which the error was encountered.  The

implication of this is that packets in which errors are detected at

Waldbusser                  Standards Track                    [Page 11]

RFC 2021             Remote Network Monitoring MIB          January 1997

the network-layer are not counted anywhere in this MIB, while packets

with errors detected at the transport layer may have network-layer

statistics counted.

6.  Definitions

RMON2-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Counter32, Integer32,

Gauge32, IpAddress, TimeTicks            FROM SNMPv2-SMI

TEXTUAL-CONVENTION, RowStatus, DisplayString, TimeStamp

FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP FROM SNMPv2-CONF

mib-2, ifIndex                  FROM RFC1213-MIB

OwnerString, statistics, history, hosts,

matrix, filter, etherStatsEntry, historyControlEntry,

hostControlEntry, matrixControlEntry, filterEntry,

channelEntry                    FROM RMON-MIB

tokenRing, tokenRingMLStatsEntry, tokenRingPStatsEntry,

ringStationControlEntry, sourceRoutingStatsEntry

FROM TOKEN-RING-RMON-MIB;

--  Remote Network Monitoring MIB

rmon MODULE-IDENTITY

LAST-UPDATED "9605270000Z"

ORGANIZATION "IETF RMON MIB Working Group"

CONTACT-INFO

"Steve Waldbusser   (WG Editor)

Postal: International Network Services

650 Castro Street, Suite 260

Mountain View, CA 94041

Phone:  +1 415 254 4251

Email:  waldbusser@ins.com

Andy Bierman   (WG Chair)

Phone:  +1 805 648 2028

Email:  abierman@west.net"

DESCRIPTION

"The MIB module for managing remote monitoring

device implementations. This MIB module

augments the original RMON MIB as specified in

RFC 1757."

::= { mib-2 16 }

-- { rmon 1 } through { rmon 10 } are defined in RMON and

-- the Token Ring RMON MIB [RFC 1513]

Waldbusser                  Standards Track                    [Page 12]

RFC 2021             Remote Network Monitoring MIB          January 1997

protocolDir     OBJECT IDENTIFIER ::= { rmon 11 }

protocolDist    OBJECT IDENTIFIER ::= { rmon 12 }

addressMap      OBJECT IDENTIFIER ::= { rmon 13 }

nlHost          OBJECT IDENTIFIER ::= { rmon 14 }

nlMatrix        OBJECT IDENTIFIER ::= { rmon 15 }

alHost          OBJECT IDENTIFIER ::= { rmon 16 }

alMatrix        OBJECT IDENTIFIER ::= { rmon 17 }

usrHistory      OBJECT IDENTIFIER ::= { rmon 18 }

probeConfig     OBJECT IDENTIFIER ::= { rmon 19 }

rmonConformance OBJECT IDENTIFIER ::= { rmon 20 }

-- Textual Conventions

ZeroBasedCounter32 ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This TC describes an object which counts events with the

following semantics: objects of this type will be set to

zero(0) on creation and will thereafter count appropriate

events, wrapping back to zero(0) when the value 2^32 is

reached.

Provided that an application discovers the new object within

the minimum time to wrap it can use the initial value as a

delta since it last polled the table of which this object is

part.  It is important for a management station to be aware of

this minimum time and the actual time between polls, and to

discard data if the actual time is too long or there is no

defined minimum time.

Typically this TC is used in tables where the INDEX space is

constantly changing and/or the TimeFilter mechanism is in use."

SYNTAX Gauge32

LastCreateTime ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This TC describes an object that stores the last time its

entry was created.

This can be used for polling applications to determine that an

entry has been deleted and re-created between polls, causing

an otherwise undetectable discontinuity in the data."

SYNTAX TimeStamp

TimeFilter ::= TEXTUAL-CONVENTION

STATUS        current

DESCRIPTION

Waldbusser                  Standards Track                    [Page 13]

RFC 2021             Remote Network Monitoring MIB          January 1997

"To be used for the index to a table.  Allows an application

to download only those rows changed since a particular time.

A row is considered changed if the value of any object in the

row changes or if the row is created or deleted.

When sysUpTime is equal to zero, this table shall be empty.

One entry exists for each past value of sysUpTime, except that

the whole table is purged should sysUpTime wrap.

As this basic row is updated new conceptual rows are created

(which still share the now updated object values with all

other instances).  The number of instances which are created

is determined by the value of sysUpTime at which the basic row

was last updated.  One instance will exist for each value of

sysUpTime at the last update time for the row.  A new

timeMark instance is created for each new sysUpTime value.

Each new conceptual row will be associated with the timeMark

instance which was created at the value of sysUpTime with

which the conceptual row is to be associated.

By definition all conceptual rows were updated at or after

time zero and so at least one conceptual row (associated with

timeMark.0) must exist for each underlying (basic) row.

See the appendix for further discussion of this variable.

Consider the following fooTable:

fooTable ...

INDEX { fooTimeMark, fooIndex }

FooEntry {

fooTimeMark  TimeFilter

fooIndex     INTEGER,

fooCounts    Counter

}

Should there be two basic rows in this table (fooIndex == 1,

fooIndex == 2) and row 1 was updated most recently at time 6,

while row 2 was updated most recently at time 8, and both rows

had been updated on several earlier occasions such that the

current values were 5 and 9 respectively then the following

fooCounts instances would exist.

fooCounts.0.1  5

fooCounts.0.2  9

fooCounts.1.1  5

Waldbusser                  Standards Track                    [Page 14]

RFC 2021             Remote Network Monitoring MIB          January 1997

fooCounts.1.2  9

fooCounts.2.1  5

fooCounts.2.2  9

fooCounts.3.1  5

fooCounts.3.2  9

fooCounts.4.1  5

fooCounts.4.2  9

fooCounts.5.1  5

fooCounts.5.2  9

fooCounts.6.1  5

fooCounts.6.2  9

fooCounts.7.2  9    -- note that row 1 doesn't exist for

fooCounts.8.2  9    -- times 7 and 8"

SYNTAX    TimeTicks

DataSource ::= TEXTUAL-CONVENTION

STATUS        current

DESCRIPTION

"Identifies the source of the data that the associated

function is configured to analyze. This source can be any

interface on this device.

In order to identify a particular interface, this

object shall identify the instance of the ifIndex

object, defined in [3,5], for the desired interface.

For example, if an entry were to receive data from

interface #1, this object would be set to ifIndex.1."

SYNTAX      OBJECT IDENTIFIER

--

-- Protocol Directory Group

--

-- Lists the inventory of protocols the probe has the capability of

-- monitoring and allows the addition, deletion, and configuration of

-- entries in this list.

protocolDirLastChange OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of sysUpTime at the time the protocol directory

was last modified, either through insertions or deletions,

or through modifications of either the

protocolDirAddressMapConfig, protocolDirHostConfig, or

protocolDirMatrixConfig."

::= { protocolDir 1 }

Waldbusser                  Standards Track                    [Page 15]

RFC 2021             Remote Network Monitoring MIB          January 1997

protocolDirTable OBJECT-TYPE

SYNTAX      SEQUENCE OF ProtocolDirEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table lists the protocols that this agent has the

capability to decode and count.  There is one entry in this

table for each such protocol.  These protocols represent

different network layer, transport layer, and higher-layer

protocols.  The agent should boot up with this table

preconfigured with those protocols that it knows about and

wishes to monitor.  Implementations are strongly encouraged to

support protocols higher than the network layer (at least for

the protocol distribution group), even for implementations

that don't support the application layer groups."

::= { protocolDir 2 }

protocolDirEntry OBJECT-TYPE

SYNTAX      ProtocolDirEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the protocolDirTable.

An example of the indexing of this entry is

protocolDirLocalIndex.8.0.0.0.1.0.0.8.0.2.0.0, which is the

encoding of a length of 8, followed by 8 subids encoding the

protocolDirID of 1.2048, followed by a length of 2 and the

2 subids encoding zero-valued parameters."

INDEX { protocolDirID, protocolDirParameters }

::= { protocolDirTable  1 }

ProtocolDirEntry ::= SEQUENCE {

protocolDirID                   OCTET STRING,

protocolDirParameters           OCTET STRING,

protocolDirLocalIndex           Integer32,

protocolDirDescr                DisplayString,

protocolDirType                 BITS,

protocolDirAddressMapConfig     INTEGER,

protocolDirHostConfig           INTEGER,

protocolDirMatrixConfig         INTEGER,

protocolDirOwner                OwnerString,

protocolDirStatus               RowStatus

}

protocolDirID OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

Waldbusser                  Standards Track                    [Page 16]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS      current

DESCRIPTION

"A unique identifier for a particular protocol.  Standard

identifiers will be defined in a manner such that they

can often be used as specifications for new protocols - i.e.

a tree-structured assignment mechanism that matches the

protocol encapsulation `tree' and which has algorithmic

assignment mechanisms for certain subtrees. See RFC XXX for

more details.

Despite the algorithmic mechanism, the probe will only place

entries in here for those protocols it chooses to collect.  In

other words, it need not populate this table with all of the

possible ethernet protocol types, nor need it create them on

the fly when it sees them.  Whether or not it does these

things is a matter of product definition (cost/benefit,

usability), and is up to the designer of the product.

If an entry is written to this table with a protocolDirID that

the agent doesn't understand, either directly or

algorithmically, the SET request will be rejected with an

inconsistentName or badValue (for SNMPv1) error."

::= { protocolDirEntry 1 }

protocolDirParameters OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A set of parameters for the associated protocolDirID.

See the associated RMON2 Protocol Identifiers document

for a description of the possible parameters. There

will be one octet in this string for each sub-identifier in

the protocolDirID, and the parameters will appear here in the

same order as the associated sub-identifiers appear in the

protocolDirID.

Every node in the protocolDirID tree has a different, optional

set of parameters defined (that is, the definition of

parameters for a node is optional).  The proper parameter

value for each node is included in this string.  Note that the

inclusion of a parameter value in this string for each node is

not optional - what is optional is that a node may have no

parameters defined, in which case the parameter field for that

node will be zero."

::= { protocolDirEntry 2 }

protocolDirLocalIndex OBJECT-TYPE

Waldbusser                  Standards Track                    [Page 17]

RFC 2021             Remote Network Monitoring MIB          January 1997

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The locally arbitrary, but unique identifier associated

with this protocolDir entry.

The value for each supported protocol must remain constant at

least from one re-initialization of the entity's network

management system to the next re-initialization, except that

if a protocol is deleted and re-created, it must be re-created

with a new value that has not been used since the last

re-initialization.

The specific value is meaningful only within a given SNMP

entity. A protocolDirLocalIndex must not be re-used until the

next agent-restart in the event the protocol directory entry

is deleted."

::= { protocolDirEntry 3 }

protocolDirDescr OBJECT-TYPE

SYNTAX      DisplayString (SIZE (1..64))

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"A textual description of the protocol encapsulation.

A probe may choose to describe only a subset of the

entire encapsulation (e.g. only the highest layer).

This object is intended for human consumption only.

This object may not be modified if the associated

protocolDirStatus object is equal to active(1)."

::= { protocolDirEntry 4 }

protocolDirType OBJECT-TYPE

SYNTAX      BITS {

extensible(0),

addressRecognitionCapable(1)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object describes 2 attributes of this protocol

directory entry.

The presence or absence of the `extensible' bit describes

whether or not this protocol directory entry can be extended

Waldbusser                  Standards Track                    [Page 18]

RFC 2021             Remote Network Monitoring MIB          January 1997

by the user by creating protocol directory entries which are

children of this protocol.

An example of an entry that will often allow extensibility is

`ip.udp'.  The probe may automatically populate some children

of this node such as `ip.udp.snmp' and `ip.udp.dns'.

A probe administrator or user may also populate additional

children via remote SNMP requests that create entries in this

table.  When a child node is added for a protocol for which the

probe has no built in support, extending a parent node (for

which the probe does have built in support),

that child node is not extendible.  This is termed `limited

extensibility'.

When a child node is added through this extensibility

mechanism, the values of protocolDirLocalIndex and

protocolDirType shall be assigned by the agent.

The other objects in the entry will be assigned by the

manager who is creating the new entry.

This object also describes whether or not this agent can

recognize addresses for this protocol, should it be a network

level protocol.  That is, while a probe may be able to

recognize packets of a particular network layer protocol and

count them, it takes additional logic to be able to recognize

the addresses in this protocol and to populate network layer

or application layer tables with the addresses in this

protocol.  If this bit is set, the agent will recognize

network layer addresses for this protoocl and populate the

network and application layer host and matrix tables with

these protocols.

Note that when an entry is created, the agent will supply

values for the bits that match the capabilities of the agent

with respect to this protocol.  Note that since row creations

usually exercise the limited extensibility feature, these

bits will usually be set to zero."

::= { protocolDirEntry 5 }

protocolDirAddressMapConfig OBJECT-TYPE

SYNTAX      INTEGER {

notSupported(1),

supportedOff(2),

supportedOn(3)

}

MAX-ACCESS  read-create

STATUS      current

Waldbusser                  Standards Track                    [Page 19]

RFC 2021             Remote Network Monitoring MIB          January 1997

DESCRIPTION

"This object describes and configures the probe's support for

address mapping for this protocol.  When the probe creates

entries in this table for all protocols that it understands,

it will set the entry to notSupported(1) if it doesn't have

the capability to perform address mapping for the protocol or

if this protocol is not a network-layer protocol.  When

an entry is created in this table by a management operation as

part of the limited extensibility feature, the probe must set

this value to notSupported(1), because limited extensibility

of the protocolDirTable does not extend to interpreting

addresses of the extended protocols.

If the value of this object is notSupported(1), the probe

will not perform address mapping for this protocol and

shall not allow this object to be changed to any other value.

If the value of this object is supportedOn(3), the probe

supports address mapping for this protocol and is configured

to perform address mapping for this protocol for all

addressMappingControlEntries and all interfaces.

If the value of this object is supportedOff(2), the probe

supports address mapping for this protocol but is configured

to not perform address mapping for this protocol for any

addressMappingControlEntries and all interfaces.

Whenever this value changes from supportedOn(3) to

supportedOff(2), the probe shall delete all related entries in

the addressMappingTable."

::= { protocolDirEntry 6 }

protocolDirHostConfig OBJECT-TYPE

SYNTAX      INTEGER {

notSupported(1),

supportedOff(2),

supportedOn(3)

}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object describes and configures the probe's support for

the network layer and application layer host tables for this

protocol.  When the probe creates entries in this table for

all protocols that it understands, it will set the entry to

notSupported(1) if it doesn't have the capability to track the

nlHostTable for this protocol or if the alHostTable is

implemented but doesn't have the capability to track this

protocol.  Note that if the alHostTable is implemented, the

probe may only support a protocol if it is supported in both

the nlHostTable and the alHostTable.

Waldbusser                  Standards Track                    [Page 20]

RFC 2021             Remote Network Monitoring MIB          January 1997

If the associated protocolDirType object has the

addressRecognitionCapable bit set, then this is a network

layer protocol for which the probe recognizes addresses, and

thus the probe will populate the nlHostTable and alHostTable

with addresses it discovers for this protocol.

If the value of this object is notSupported(1), the probe

will not track the nlHostTable or alHostTable for this

protocol and shall not allow this object to be changed to any

other value. If the value of this object is supportedOn(3),

the probe supports tracking of the nlHostTable and alHostTable

for this protocol and is configured to track both tables

for this protocol for all control entries and all interfaces.

If the value of this object is supportedOff(2), the probe

supports tracking of the nlHostTable and alHostTable for this

protocol but is configured to not track these tables

for any control entries or interfaces.

Whenever this value changes from supportedOn(3) to

supportedOff(2), the probe shall delete all related entries in

the nlHostTable and alHostTable.

Note that since each alHostEntry references 2 protocol

directory entries, one for the network address and one for the

type of the highest protocol recognized, that an entry will

only be created in that table if this value is supportedOn(3)

for both protocols."

::= { protocolDirEntry 7 }

protocolDirMatrixConfig OBJECT-TYPE

SYNTAX      INTEGER {

notSupported(1),

supportedOff(2),

supportedOn(3)

}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object describes and configures the probe's support for

the network layer and application layer matrix tables for this

protocol.  When the probe creates entries in this table for

all protocols that it understands, it will set the entry to

notSupported(1) if it doesn't have the capability to track the

nlMatrixTables for this protocol or if the alMatrixTables are

implemented but don't have the capability to track this

protocol.  Note that if the alMatrix tables are implemented,

the probe may only support a protocol if it is supported in

the the both of the nlMatrixTables and both of the

alMatrixTables.

Waldbusser                  Standards Track                    [Page 21]

RFC 2021             Remote Network Monitoring MIB          January 1997

If the associated protocolDirType object has the

addressRecognitionCapable bit set, then this is a network

layer protocol for which the probe recognizes addresses, and

thus the probe will populate both of the nlMatrixTables and

both of the alMatrixTables with addresses it discovers for

this protocol.

If the value of this object is notSupported(1), the probe

will not track either of the nlMatrixTables or the

alMatrixTables for this protocol and shall not allow this

object to be changed to any other value. If the value of this

object is supportedOn(3), the probe supports tracking of both

of the nlMatrixTables and (if implemented) both of the

alMatrixTables for this protocol and is configured to track

these tables for this protocol for all control entries and all

interfaces. If the value of this object is supportedOff(2),

the probe supports tracking of both of the nlMatrixTables and

(if implemented) both of the alMatrixTables for this protocol

but is configured to not track these tables for this

protocol for any control entries or interfaces.

Whenever this value changes from supportedOn(3) to

supportedOff(2), the probe shall delete all related entries in

the nlMatrixTables and the alMatrixTables.

Note that since each alMatrixEntry references 2 protocol

directory entries, one for the network address and one for the

type of the highest protocol recognized, that an entry will

only be created in that table if this value is supportedOn(3)

for both protocols."

::= { protocolDirEntry 8 }

protocolDirOwner OBJECT-TYPE

SYNTAX      OwnerString

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { protocolDirEntry 9 }

protocolDirStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The status of this protocol directory entry.

An entry may not exist in the active state unless all

Waldbusser                  Standards Track                    [Page 22]

RFC 2021             Remote Network Monitoring MIB          January 1997

objects in the entry have an appropriate value.

If this object is not equal to active(1), all associated

entries in the nlHostTable, nlMatrixSDTable, nlMatrixDSTable,

alHostTable, alMatrixSDTable, and alMatrixDSTable shall be

deleted."

::= { protocolDirEntry 10 }

--

-- Protocol Distribution Group  (protocolDist)

--

-- Collects the relative amounts of octets and packets for the

-- different protocols detected on a network segment.

--    protocolDistControlTable,

--    protocolDistStatsTable

protocolDistControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF ProtocolDistControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Controls the setup of protocol type distribution statistics

tables.

Implementations are encouraged to add an entry per monitored

interface upon initialization so that a default collection

of protocol statistics is available.

Rationale:

This table controls collection of very basic statistics

for any or all of the protocols detected on a given interface.

An NMS can use this table to quickly determine bandwidth

allocation utilized by different protocols.

A media-specific statistics collection could also

be configured (e.g. etherStats, trPStats) to easily obtain

total frame, octet, and droppedEvents for the same

interface."

::= { protocolDist 1 }

protocolDistControlEntry OBJECT-TYPE

SYNTAX      ProtocolDistControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the protocolDistControlTable.

An example of the indexing of this entry is

Waldbusser                  Standards Track                    [Page 23]

RFC 2021             Remote Network Monitoring MIB          January 1997

protocolDistControlDroppedFrames.7"

INDEX { protocolDistControlIndex }

::= { protocolDistControlTable 1 }

ProtocolDistControlEntry ::= SEQUENCE {

protocolDistControlIndex                Integer32,

protocolDistControlDataSource           DataSource,

protocolDistControlDroppedFrames        Counter32,

protocolDistControlCreateTime           LastCreateTime,

protocolDistControlOwner                OwnerString,

protocolDistControlStatus               RowStatus

}

protocolDistControlIndex OBJECT-TYPE

SYNTAX      Integer32 (1..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique index for this protocolDistControlEntry."

::= { protocolDistControlEntry 1 }

protocolDistControlDataSource OBJECT-TYPE

SYNTAX      DataSource

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The source of data for the this protocol distribution.

The statistics in this group reflect all packets

on the local network segment attached to the

identified interface.

This object may not be modified if the associated

protocolDistControlStatus object is equal to active(1)."

::= { protocolDistControlEntry 2 }

protocolDistControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

Waldbusser                  Standards Track                    [Page 24]

RFC 2021             Remote Network Monitoring MIB          January 1997

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { protocolDistControlEntry 3 }

protocolDistControlCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last

activated. This can be used by the management station to

ensure that the table has not been deleted and recreated

between polls."

::= { protocolDistControlEntry 4 }

protocolDistControlOwner OBJECT-TYPE

SYNTAX      OwnerString

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { protocolDistControlEntry 5 }

protocolDistControlStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The status of this row.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all associated

entries in the protocolDistStatsTable shall be deleted."

::= { protocolDistControlEntry 6 }

-- per interface protocol distribution statistics table

protocolDistStatsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF ProtocolDistStatsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry is made in this table for every protocol in the

Waldbusser                  Standards Track                    [Page 25]

RFC 2021             Remote Network Monitoring MIB          January 1997

protocolDirTable which has been seen in at least one packet.

Counters are updated in this table for every protocol type

that is encountered when parsing a packet, but no counters are

updated for packets with MAC-layer errors.

Note that if a protocolDirEntry is deleted, all associated

entries in this table are removed."

::= { protocolDist 2 }

protocolDistStatsEntry OBJECT-TYPE

SYNTAX      ProtocolDistStatsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the protocolDistStatsTable.

The index is composed of the protocolDistControlIndex of the

associated protocolDistControlEntry followed by the

protocolDirLocalIndex of the associated protocol that this

entry represents.  In other words, the index identifies the

protocol distribution an entry is a part of as well as the

particular protocol that it represents.

An example of the indexing of this entry is

protocolDistStatsPkts.1.18"

INDEX { protocolDistControlIndex, protocolDirLocalIndex }

::= { protocolDistStatsTable 1 }

ProtocolDistStatsEntry ::= SEQUENCE {

protocolDistStatsPkts                    ZeroBasedCounter32,

protocolDistStatsOctets                  ZeroBasedCounter32

}

protocolDistStatsPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors received of this

protocol type.  Note that this is the number of link-layer

packets, so if a single network-layer packet is fragmented

into several link-layer frames, this counter is incremented

several times."

::= { protocolDistStatsEntry 1 }

protocolDistStatsOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

Waldbusser                  Standards Track                    [Page 26]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS      current

DESCRIPTION

"The number of octets in packets received of this protocol

type since it was added to the protocolDistStatsTable

(excluding framing bits but including FCS octets), except for

those octets in packets that contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { protocolDistStatsEntry 2 }

--

-- Address Map Group   (addressMap)

--

-- Lists MAC address to network address bindings discovered by the

-- probe and what interface they were last seen on.

--    addressMapControlTable

--    addressMapTable

addressMapInserts OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an address mapping entry has been

inserted into the addressMapTable.  If an entry is inserted,

then deleted, and then inserted, this counter will be

incremented by 2.

Note that the table size can be determined by subtracting

addressMapDeletes from addressMapInserts."

::= { addressMap 1 }

addressMapDeletes OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an address mapping entry has been

deleted from the addressMapTable (for any reason).  If

an entry is deleted, then inserted, and then deleted, this

counter will be incremented by 2.

Note that the table size can be determined by subtracting

addressMapDeletes from addressMapInserts."

::= { addressMap 2 }

Waldbusser                  Standards Track                    [Page 27]

RFC 2021             Remote Network Monitoring MIB          January 1997

addressMapMaxDesiredEntries OBJECT-TYPE

SYNTAX      Integer32 (-1..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum number of entries that are desired in the

addressMapTable. The probe will not create more than

this number of entries in the table, but may choose to create

fewer entries in this table for any reason including the lack

of resources.

If this object is set to a value less than the current number

of entries, enough entries are chosen in an

implementation-dependent manner and deleted so that the number

of entries in the table equals the value of this object.

If this value is set to -1, the probe may create any number

of entries in this table.

This object may be used to control how resources are allocated

on the probe for the various RMON functions."

::= { addressMap 3 }

addressMapControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AddressMapControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A table to control the collection of network layer address to

physical address to interface mappings.

Note that this is not like the typical RMON

controlTable and dataTable in which each entry creates

its own data table.  Each entry in this table enables the

discovery of addresses on a new interface and the placement

of address mappings into the central addressMapTable.

Implementations are encouraged to add an entry per monitored

interface upon initialization so that a default collection

of address mappings is available."

::= { addressMap 4 }

addressMapControlEntry OBJECT-TYPE

SYNTAX      AddressMapControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the addressMapControlTable.

Waldbusser                  Standards Track                    [Page 28]

RFC 2021             Remote Network Monitoring MIB          January 1997

An example of the indexing of this entry is

addressMapControlDroppedFrames.1"

INDEX { addressMapControlIndex }

::= { addressMapControlTable 1 }

AddressMapControlEntry ::= SEQUENCE {

addressMapControlIndex              Integer32,

addressMapControlDataSource         DataSource,

addressMapControlDroppedFrames      Counter32,

addressMapControlOwner              OwnerString,

addressMapControlStatus             RowStatus

}

addressMapControlIndex OBJECT-TYPE

SYNTAX      Integer32 (1..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique index for this entry in the addressMapControlTable."

::= { addressMapControlEntry 1 }

addressMapControlDataSource OBJECT-TYPE

SYNTAX      DataSource

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The source of data for this addressMapControlEntry."

::= { addressMapControlEntry 2 }

addressMapControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { addressMapControlEntry 3 }

Waldbusser                  Standards Track                    [Page 29]

RFC 2021             Remote Network Monitoring MIB          January 1997

addressMapControlOwner OBJECT-TYPE

SYNTAX      OwnerString

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { addressMapControlEntry 4 }

addressMapControlStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The status of this addressMap control entry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all associated

entries in the addressMapTable shall be deleted."

::= { addressMapControlEntry 5 }

addressMapTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AddressMapEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A table of network layer address to physical address to

interface mappings.

The probe will add entries to this table based on the source

MAC and network addresses seen in packets without MAC-level

errors. The probe will populate this table for all protocols

in the protocol directory table whose value of

protocolDirAddressMapConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirAddressMapConfig value of supportedOff(2)."

::= { addressMap 5 }

addressMapEntry OBJECT-TYPE

SYNTAX      AddressMapEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the addressMapTable.

The protocolDirLocalIndex in the index identifies the network

layer protocol of the addressMapNetworkAddress.

Waldbusser                  Standards Track                    [Page 30]

RFC 2021             Remote Network Monitoring MIB          January 1997

An example of the indexing of this entry is

addressMapSource.783495.18.4.128.2.6.6.11.1.3.6.1.2.1.2.2.1.1.1"

INDEX { addressMapTimeMark, protocolDirLocalIndex,

addressMapNetworkAddress, addressMapSource }

::= { addressMapTable 1 }

AddressMapEntry ::= SEQUENCE {

addressMapTimeMark                 TimeFilter,

addressMapNetworkAddress           OCTET STRING,

addressMapSource                   OBJECT IDENTIFIER,

addressMapPhysicalAddress          OCTET STRING,

addressMapLastChange               TimeStamp

}

addressMapTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { addressMapEntry 1 }

addressMapNetworkAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network address for this relation.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the

index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { addressMapEntry 2 }

addressMapSource OBJECT-TYPE

SYNTAX      OBJECT IDENTIFIER

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The interface or port on which the associated network

address was most recently seen.

Waldbusser                  Standards Track                    [Page 31]

RFC 2021             Remote Network Monitoring MIB          January 1997

If this address mapping was discovered on an interface, this

object shall identify the instance of the ifIndex

object, defined in [3,5], for the desired interface.

For example, if an entry were to receive data from

interface #1, this object would be set to ifIndex.1.

If this address mapping was discovered on a port, this

object shall identify the instance of the rptrGroupPortIndex

object, defined in [RFC1516], for the desired port.

For example, if an entry were to receive data from

group #1, port #1, this object would be set to

rptrGroupPortIndex.1.1.

Note that while the dataSource associated with this entry

may only point to index objects, this object may at times

point to repeater port objects. This situation occurs when

the dataSource points to an interface which is a locally

attached repeater and the agent has additional information

about the source port of traffic seen on that repeater."

::= { addressMapEntry 3 }

addressMapPhysicalAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The last source physical address on which the associated

network address was seen.  If the protocol of the associated

network address was encapsulated inside of a network-level or

higher protocol, this will be the address of the next-lower

protocol with the addressRecognitionCapable bit enabled and

will be formatted as specified for that protocol."

::= { addressMapEntry 4 }

addressMapLastChange OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of sysUpTime at the time this entry was last

created or the values of the physical address changed.

This can be used to help detect duplicate address problems, in

which case this object will be updated frequently."

::= { addressMapEntry 5 }

--

-- Network Layer Host Group

Waldbusser                  Standards Track                    [Page 32]

RFC 2021             Remote Network Monitoring MIB          January 1997

--

-- Counts the amount of traffic sent from and to each network address

-- discovered by the probe.

-- Note that while the hlHostControlTable also has objects that

-- control an optional alHostTable, implementation of the alHostTable is

-- not required to fully implement this group.

hlHostControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF HlHostControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of higher layer (i.e. non-MAC) host table control entries.

These entries will enable the collection of the network and

application level host tables indexed by network addresses.

Both the network and application level host tables are

controlled by this table is so that they will both be created

and deleted at the same time, further increasing the ease with

which they can be implemented as a single datastore (note that

if an implementation stores application layer host records in

memory, it can derive network layer host records from them).

Entries in the nlHostTable will be created on behalf of each

entry in this table. Additionally, if this probe implements

the alHostTable, entries in the alHostTable will be created on

behalf of each entry in this table.

Implementations are encouraged to add an entry per monitored

interface upon initialization so that a default collection

of host statistics is available."

::= { nlHost 1 }

hlHostControlEntry OBJECT-TYPE

SYNTAX      HlHostControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the hlHostControlTable.

An example of the indexing of this entry is

hlHostControlNlDroppedFrames.1"

INDEX { hlHostControlIndex }

::= { hlHostControlTable 1 }

HlHostControlEntry ::= SEQUENCE {

hlHostControlIndex               Integer32,

hlHostControlDataSource          DataSource,

Waldbusser                  Standards Track                    [Page 33]

RFC 2021             Remote Network Monitoring MIB          January 1997

hlHostControlNlDroppedFrames     Counter32,

hlHostControlNlInserts           Counter32,

hlHostControlNlDeletes           Counter32,

hlHostControlNlMaxDesiredEntries Integer32,

hlHostControlAlDroppedFrames     Counter32,

hlHostControlAlInserts           Counter32,

hlHostControlAlDeletes           Counter32,

hlHostControlAlMaxDesiredEntries Integer32,

hlHostControlOwner               OwnerString,

hlHostControlStatus              RowStatus

}

hlHostControlIndex OBJECT-TYPE

SYNTAX      Integer32 (1..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index that uniquely identifies an entry in the

hlHostControlTable.  Each such entry defines

a function that discovers hosts on a particular

interface and places statistics about them in the

nlHostTable, and optionally in the alHostTable, on

behalf of this hlHostControlEntry."

::= { hlHostControlEntry 1 }

hlHostControlDataSource OBJECT-TYPE

SYNTAX      DataSource

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The source of data for the associated host tables.

The statistics in this group reflect all packets

on the local network segment attached to the

identified interface.

This object may not be modified if the associated

hlHostControlStatus object is equal to active(1)."

::= { hlHostControlEntry 2 }

hlHostControlNlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for the associated

Waldbusser                  Standards Track                    [Page 34]

RFC 2021             Remote Network Monitoring MIB          January 1997

nlHost entries for whatever reason.  Most often, this event

occurs when the probe is out of some resources and decides to

shed load from this collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that if the nlHostTable is inactive because no protocols

are enabled in the protocol directory, this value should be 0.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { hlHostControlEntry 3 }

hlHostControlNlInserts OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an nlHost entry has been

inserted into the nlHost table.  If an entry is inserted, then

deleted, and then inserted, this counter will be incremented

by 2.

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlHostControlNlDeletes from hlHostControlNlInserts."

::= { hlHostControlEntry 4 }

hlHostControlNlDeletes OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an nlHost entry has been

deleted from the nlHost table (for any reason).  If an entry

is deleted, then inserted, and then deleted, this counter will

be incremented by 2.

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

Waldbusser                  Standards Track                    [Page 35]

RFC 2021             Remote Network Monitoring MIB          January 1997

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlHostControlNlDeletes from hlHostControlNlInserts."

::= { hlHostControlEntry 5 }

hlHostControlNlMaxDesiredEntries OBJECT-TYPE

SYNTAX      Integer32 (-1..2147483647)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum number of entries that are desired in the

nlHostTable on behalf of this control entry. The probe will

not create more than this number of associated entries in the

table, but may choose to create fewer entries in this table

for any reason including the lack of resources.

If this object is set to a value less than the current number

of entries, enough entries are chosen in an

implementation-dependent manner and deleted so that the number

of entries in the table equals the value of this object.

If this value is set to -1, the probe may create any number

of entries in this table.  If the associated

hlHostControlStatus object is equal to `active', this

object may not be modified.

This object may be used to control how resources are allocated

on the probe for the various RMON functions."

::= { hlHostControlEntry 6 }

hlHostControlAlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for the associated

alHost entries for whatever reason.  Most often, this event

occurs when the probe is out of some resources and decides to

shed load from this collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Waldbusser                  Standards Track                    [Page 36]

RFC 2021             Remote Network Monitoring MIB          January 1997

Note that if the alHostTable is not implemented or is inactive

because no protocols are enabled in the protocol directory,

this value should be 0.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { hlHostControlEntry 7 }

hlHostControlAlInserts OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an alHost entry has been

inserted into the alHost table.  If an entry is inserted, then

deleted, and then inserted, this counter will be incremented

by 2.

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlHostControlAlDeletes from hlHostControlAlInserts."

::= { hlHostControlEntry 8 }

hlHostControlAlDeletes OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an alHost entry has been

deleted from the alHost table (for any reason).  If an entry

is deleted, then inserted, and then deleted, this counter will

be incremented by 2.

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlHostControlAlDeletes from hlHostControlAlInserts."

Waldbusser                  Standards Track                    [Page 37]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { hlHostControlEntry 9 }

hlHostControlAlMaxDesiredEntries OBJECT-TYPE

SYNTAX      Integer32 (-1..2147483647)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum number of entries that are desired in the alHost

table on behalf of this control entry. The probe will not

create more than this number of associated entries in the

table, but may choose to create fewer entries in this table

for any reason including the lack of resources.

If this object is set to a value less than the current number

of entries, enough entries are chosen in an

implementation-dependent manner and deleted so that the number

of entries in the table equals the value of this object.

If this value is set to -1, the probe may create any number

of entries in this table.  If the associated

hlHostControlStatus object is equal to `active', this

object may not be modified.

This object may be used to control how resources are allocated

on the probe for the various RMON functions."

::= { hlHostControlEntry 10 }

hlHostControlOwner OBJECT-TYPE

SYNTAX      OwnerString

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { hlHostControlEntry 11 }

hlHostControlStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The status of this hlHostControlEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all associated

entries in the nlHostTable and alHostTable shall be deleted."

Waldbusser                  Standards Track                    [Page 38]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { hlHostControlEntry 12 }

nlHostTable OBJECT-TYPE

SYNTAX      SEQUENCE OF NlHostEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A collection of statistics for a particular network layer

address that has been discovered on an interface of this

device.

The probe will populate this table for all network layer

protocols in the protocol directory table whose value of

protocolDirHostConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirHostConfig value of supportedOff(2).

The probe will add to this table all addresses seen

as the source or destination address in all packets with no

MAC errors, and will increment octet and packet counts in the

table for all packets with no MAC errors."

::= { nlHost 2 }

nlHostEntry OBJECT-TYPE

SYNTAX      NlHostEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the nlHostTable.

The hlHostControlIndex value in the index identifies the

hlHostControlEntry on whose behalf this entry was created.

The protocolDirLocalIndex value in the index identifies the

network layer protocol of the nlHostAddress.

An example of the indexing of this entry is

nlHostOutPkts.1.783495.18.4.128.2.6.6."

INDEX { hlHostControlIndex, nlHostTimeMark,

protocolDirLocalIndex, nlHostAddress }

::= { nlHostTable 1 }

NlHostEntry ::= SEQUENCE {

nlHostTimeMark              TimeFilter,

nlHostAddress               OCTET STRING,

nlHostInPkts                ZeroBasedCounter32,

nlHostOutPkts               ZeroBasedCounter32,

nlHostInOctets              ZeroBasedCounter32,

nlHostOutOctets             ZeroBasedCounter32,

Waldbusser                  Standards Track                    [Page 39]

RFC 2021             Remote Network Monitoring MIB          January 1997

nlHostOutMacNonUnicastPkts  ZeroBasedCounter32,

nlHostCreateTime            LastCreateTime

}

nlHostTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { nlHostEntry 1 }

nlHostAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network address for this nlHostEntry.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlHostEntry 2 }

nlHostInPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors transmitted to

this address since it was added to the nlHostTable.  Note that

this is the number of link-layer packets, so if a single

network-layer packet is fragmented into several link-layer

frames, this counter is incremented several times."

::= { nlHostEntry 3 }

nlHostOutPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors transmitted by

Waldbusser                  Standards Track                    [Page 40]

RFC 2021             Remote Network Monitoring MIB          January 1997

this address since it was added to the nlHostTable.  Note that

this is the number of link-layer packets, so if a single

network-layer packet is fragmented into several link-layer

frames, this counter is incremented several times."

::= { nlHostEntry 4 }

nlHostInOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted to this address

since it was added to the nlHostTable (excluding

framing bits but including FCS octets), excluding

those octets in packets that contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { nlHostEntry 5 }

nlHostOutOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted by this address

since it was added to the nlHostTable (excluding

framing bits but including FCS octets), excluding

those octets in packets that contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { nlHostEntry 6 }

nlHostOutMacNonUnicastPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors transmitted by this

address that were directed to any MAC broadcast addresses

or to any MAC multicast addresses since this host was

added to the nlHostTable. Note that this is the number of

link-layer packets, so if a single network-layer packet is

fragmented into several link-layer frames, this counter is

incremented several times."

Waldbusser                  Standards Track                    [Page 41]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { nlHostEntry 7 }

nlHostCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { nlHostEntry 8 }

--

-- Network Layer Matrix Group

--

-- Counts the amount of traffic sent between each pair of network

-- addresses discovered by the probe.

-- Note that while the hlMatrixControlTable also has objects that

-- control optional alMatrixTables, implementation of the

-- alMatrixTables is not required to fully implement this group.

hlMatrixControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF HlMatrixControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of higher layer (i.e. non-MAC) matrix control entries.

These entries will enable the collection of the network and

application level matrix tables containing conversation

statistics indexed by pairs of network addresses.

Both the network and application level matrix tables are

controlled by this table is so that they will both be created

and deleted at the same time, further increasing the ease with

which they can be implemented as a single datastore (note that

if an implementation stores application layer matrix records

in memory, it can derive network layer matrix records from

them).

Entries in the nlMatrixSDTable and nlMatrixDSTable will be

created on behalf of each entry in this table.  Additionally,

if this probe implements the alMatrix tables, entries in the

alMatrix tables will be created on behalf of each entry in

this table."

::= { nlMatrix 1 }

hlMatrixControlEntry OBJECT-TYPE

SYNTAX      HlMatrixControlEntry

Waldbusser                  Standards Track                    [Page 42]

RFC 2021             Remote Network Monitoring MIB          January 1997

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the hlMatrixControlTable.

An example of indexing of this entry is

hlMatrixControlNlDroppedFrames.1"

INDEX { hlMatrixControlIndex }

::= { hlMatrixControlTable 1 }

HlMatrixControlEntry ::= SEQUENCE {

hlMatrixControlIndex                  Integer32,

hlMatrixControlDataSource             DataSource,

hlMatrixControlNlDroppedFrames        Counter32,

hlMatrixControlNlInserts              Counter32,

hlMatrixControlNlDeletes              Counter32,

hlMatrixControlNlMaxDesiredEntries    Integer32,

hlMatrixControlAlDroppedFrames        Counter32,

hlMatrixControlAlInserts              Counter32,

hlMatrixControlAlDeletes              Counter32,

hlMatrixControlAlMaxDesiredEntries    Integer32,

hlMatrixControlOwner                  OwnerString,

hlMatrixControlStatus                 RowStatus

}

hlMatrixControlIndex OBJECT-TYPE

SYNTAX      Integer32 (1..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index that uniquely identifies an entry in the

hlMatrixControlTable.  Each such entry defines

a function that discovers conversations on a particular

interface and places statistics about them in the

nlMatrixSDTable and the nlMatrixDSTable, and optionally the

alMatrixSDTable and alMatrixDSTable, on behalf of this

hlMatrixControlEntry."

::= { hlMatrixControlEntry 1 }

hlMatrixControlDataSource OBJECT-TYPE

SYNTAX      DataSource

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The source of the data for the associated matrix tables.

The statistics in this group reflect all packets

on the local network segment attached to the

Waldbusser                  Standards Track                    [Page 43]

RFC 2021             Remote Network Monitoring MIB          January 1997

identified interface.

This object may not be modified if the associated

hlMatrixControlStatus object is equal to active(1)."

::= { hlMatrixControlEntry 2 }

hlMatrixControlNlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that if the nlMatrixTables are inactive because no

protocols are enabled in the protocol directory, this value

should be 0.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { hlMatrixControlEntry 3 }

hlMatrixControlNlInserts OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an nlMatrix entry has been

inserted into the nlMatrix tables.  If an entry is inserted,

then deleted, and then inserted, this counter will be

incremented by 2.  The addition of a conversation into both

the nlMatrixSDTable and nlMatrixDSTable shall be counted as

two insertions (even though every addition into one table must

be accompanied by an insertion into the other).

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Waldbusser                  Standards Track                    [Page 44]

RFC 2021             Remote Network Monitoring MIB          January 1997

Note that the sum of then nlMatrixSDTable and nlMatrixDSTable

sizes can be determined by subtracting

hlMatrixControlNlDeletes from hlMatrixControlNlInserts."

::= { hlMatrixControlEntry 4 }

hlMatrixControlNlDeletes OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an nlMatrix entry has been

deleted from the nlMatrix tables (for any reason).  If an

entry is deleted, then inserted, and then deleted, this

counter will be incremented by 2.  The deletion of a

conversation from both the nlMatrixSDTable and nlMatrixDSTable

shall be counted as two deletions (even though every deletion

from one table must be accompanied by a deletion from the

other).

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlMatrixControlNlDeletes from hlMatrixControlNlInserts."

::= { hlMatrixControlEntry 5 }

hlMatrixControlNlMaxDesiredEntries OBJECT-TYPE

SYNTAX      Integer32 (-1..2147483647)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum number of entries that are desired in the

nlMatrix tables on behalf of this control entry. The probe

will not create more than this number of associated entries in

the table, but may choose to create fewer entries in this

table for any reason including the lack of resources.

If this object is set to a value less than the current number

of entries, enough entries are chosen in an

implementation-dependent manner and deleted so that the number

of entries in the table equals the value of this object.

If this value is set to -1, the probe may create any number

of entries in this table.  If the associated

Waldbusser                  Standards Track                    [Page 45]

RFC 2021             Remote Network Monitoring MIB          January 1997

hlMatrixControlStatus object is equal to `active', this

object may not be modified.

This object may be used to control how resources are allocated

on the probe for the various RMON functions."

::= { hlMatrixControlEntry 6 }

hlMatrixControlAlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that if the alMatrixTables are not implemented or are

inactive because no protocols are enabled in the protocol

directory, this value should be 0.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { hlMatrixControlEntry 7 }

hlMatrixControlAlInserts OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an alMatrix entry has been

inserted into the alMatrix tables.  If an entry is inserted,

then deleted, and then inserted, this counter will be

incremented by 2.  The addition of a conversation into both

the alMatrixSDTable and alMatrixDSTable shall be counted as

two insertions (even though every addition into one table must

be accompanied by an insertion into the other).

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

Waldbusser                  Standards Track                    [Page 46]

RFC 2021             Remote Network Monitoring MIB          January 1997

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlMatrixControlAlDeletes from hlMatrixControlAlInserts."

::= { hlMatrixControlEntry 8 }

hlMatrixControlAlDeletes OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of times an alMatrix entry has been

deleted from the alMatrix tables.  If an entry is deleted,

then inserted, and then deleted, this counter will be

incremented by 2.  The deletion of a conversation from both

the alMatrixSDTable and alMatrixDSTable shall be counted as

two deletions (even though every deletion from one table must

be accompanied by a deletion from the other).

To allow for efficient implementation strategies, agents may

delay updating this object for short periods of time.  For

example, an implementation strategy may allow internal

data structures to differ from those visible via SNMP for

short periods of time.  This counter may reflect the internal

data structures for those short periods of time.

Note that the table size can be determined by subtracting

hlMatrixControlAlDeletes from hlMatrixControlAlInserts."

::= { hlMatrixControlEntry 9 }

hlMatrixControlAlMaxDesiredEntries OBJECT-TYPE

SYNTAX      Integer32 (-1..2147483647)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum number of entries that are desired in the

alMatrix tables on behalf of this control entry. The probe

will not create more than this number of associated entries in

the table, but may choose to create fewer entries in this

table for any reason including the lack of resources.

If this object is set to a value less than the current number

of entries, enough entries are chosen in an

implementation-dependent manner and deleted so that the number

of entries in the table equals the value of this object.

If this value is set to -1, the probe may create any number

of entries in this table.  If the associated

Waldbusser                  Standards Track                    [Page 47]

RFC 2021             Remote Network Monitoring MIB          January 1997

hlMatrixControlStatus object is equal to `active', this

object may not be modified.

This object may be used to control how resources are allocated

on the probe for the various RMON functions."

::= { hlMatrixControlEntry 10 }

hlMatrixControlOwner OBJECT-TYPE

SYNTAX      OwnerString

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { hlMatrixControlEntry 11 }

hlMatrixControlStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The status of this hlMatrixControlEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all

associated entries in the nlMatrixSDTable,

nlMatrixDSTable, alMatrixSDTable, and the alMatrixDSTable

shall be deleted by the agent."

::= { hlMatrixControlEntry 12 }

nlMatrixSDTable OBJECT-TYPE

SYNTAX      SEQUENCE OF NlMatrixSDEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of traffic matrix entries which collect statistics for

conversations between two network-level addresses.  This table

is indexed first by the source address and then by the

destination address to make it convenient to collect all

conversations from a particular address.

The probe will populate this table for all network layer

protocols in the protocol directory table whose value of

protocolDirMatrixConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirMatrixConfig value of supportedOff(2).

Waldbusser                  Standards Track                    [Page 48]

RFC 2021             Remote Network Monitoring MIB          January 1997

The probe will add to this table all pairs of addresses

seen in all packets with no MAC errors, and will increment

octet and packet counts in the table for all packets with no

MAC errors.

Further, this table will only contain entries that have a

corresponding entry in the nlMatrixDSTable with the same

source address and destination address."

::= { nlMatrix 2 }

nlMatrixSDEntry OBJECT-TYPE

SYNTAX      NlMatrixSDEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the nlMatrixSDTable.

The hlMatrixControlIndex value in the index identifies the

hlMatrixControlEntry on whose behalf this entry was created.

The protocolDirLocalIndex value in the index identifies the

network layer protocol of the nlMatrixSDSourceAddress and

nlMatrixSDDestAddress.

An example of the indexing of this table is

nlMatrixSDPkts.1.783495.18.4.128.2.6.6.4.128.2.6.7"

INDEX { hlMatrixControlIndex, nlMatrixSDTimeMark,

protocolDirLocalIndex,

nlMatrixSDSourceAddress, nlMatrixSDDestAddress }

::= { nlMatrixSDTable 1 }

NlMatrixSDEntry ::= SEQUENCE {

nlMatrixSDTimeMark              TimeFilter,

nlMatrixSDSourceAddress         OCTET STRING,

nlMatrixSDDestAddress           OCTET STRING,

nlMatrixSDPkts                  ZeroBasedCounter32,

nlMatrixSDOctets                ZeroBasedCounter32,

nlMatrixSDCreateTime            LastCreateTime

}

nlMatrixSDTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { nlMatrixSDEntry 1 }

Waldbusser                  Standards Track                    [Page 49]

RFC 2021             Remote Network Monitoring MIB          January 1997

nlMatrixSDSourceAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network source address for this nlMatrixSDEntry.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixSDEntry 2 }

nlMatrixSDDestAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network destination address for this

nlMatrixSDEntry.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixSDEntry 3 }

nlMatrixSDPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors transmitted from the

source address to the destination address since this entry was

added to the nlMatrixSDTable.  Note that this is the number of

link-layer packets, so if a single network-layer packet is

fragmented into several link-layer frames, this counter is

incremented several times."

::= { nlMatrixSDEntry 4 }

Waldbusser                  Standards Track                    [Page 50]

RFC 2021             Remote Network Monitoring MIB          January 1997

nlMatrixSDOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted from the source address to

the destination address since this entry was added to the

nlMatrixSDTable (excluding framing bits but

including FCS octets), excluding those octets in packets that

contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { nlMatrixSDEntry 5 }

nlMatrixSDCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { nlMatrixSDEntry 6 }

-- Traffic matrix tables from destination to source

nlMatrixDSTable OBJECT-TYPE

SYNTAX      SEQUENCE OF NlMatrixDSEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of traffic matrix entries which collect statistics for

conversations between two network-level addresses.  This table

is indexed first by the destination address and then by the

source address to make it convenient to collect all

conversations to a particular address.

The probe will populate this table for all network layer

protocols in the protocol directory table whose value of

protocolDirMatrixConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirMatrixConfig value of supportedOff(2).

The probe will add to this table all pairs of addresses

seen in all packets with no MAC errors, and will increment

Waldbusser                  Standards Track                    [Page 51]

RFC 2021             Remote Network Monitoring MIB          January 1997

octet and packet counts in the table for all packets with no

MAC errors.

Further, this table will only contain entries that have a

corresponding entry in the nlMatrixSDTable with the same

source address and destination address."

::= { nlMatrix 3 }

nlMatrixDSEntry OBJECT-TYPE

SYNTAX      NlMatrixDSEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the nlMatrixDSTable.

The hlMatrixControlIndex value in the index identifies the

hlMatrixControlEntry on whose behalf this entry was created.

The protocolDirLocalIndex value in the index identifies the

network layer protocol of the nlMatrixDSSourceAddress and

nlMatrixDSDestAddress.

An example of the indexing of this table is

nlMatrixDSPkts.1.783495.18.4.128.2.6.7.4.128.2.6.6"

INDEX { hlMatrixControlIndex, nlMatrixDSTimeMark,

protocolDirLocalIndex,

nlMatrixDSDestAddress, nlMatrixDSSourceAddress }

::= { nlMatrixDSTable 1 }

NlMatrixDSEntry ::= SEQUENCE {

nlMatrixDSTimeMark                 TimeFilter,

nlMatrixDSSourceAddress            OCTET STRING,

nlMatrixDSDestAddress              OCTET STRING,

nlMatrixDSPkts                     ZeroBasedCounter32,

nlMatrixDSOctets                   ZeroBasedCounter32,

nlMatrixDSCreateTime               LastCreateTime

}

nlMatrixDSTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { nlMatrixDSEntry 1 }

nlMatrixDSSourceAddress OBJECT-TYPE

SYNTAX      OCTET STRING

Waldbusser                  Standards Track                    [Page 52]

RFC 2021             Remote Network Monitoring MIB          January 1997

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network source address for this nlMatrixDSEntry.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixDSEntry 2 }

nlMatrixDSDestAddress OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The network destination address for this

nlMatrixDSEntry.

This is represented as an octet string with

specific semantics and length as identified

by the protocolDirLocalIndex component of the index.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixDSEntry 3 }

nlMatrixDSPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets without errors transmitted from the

source address to the destination address since this entry was

added to the nlMatrixDSTable.  Note that this is the number of

link-layer packets, so if a single network-layer packet is

fragmented into several link-layer frames, this counter is

incremented several times."

::= { nlMatrixDSEntry 4 }

nlMatrixDSOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

Waldbusser                  Standards Track                    [Page 53]

RFC 2021             Remote Network Monitoring MIB          January 1997

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted from the source address

to the destination address since this entry was added to the

nlMatrixDSTable (excluding framing bits but

including FCS octets), excluding those octets in packets that

contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { nlMatrixDSEntry 5 }

nlMatrixDSCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { nlMatrixDSEntry 6 }

nlMatrixTopNControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF NlMatrixTopNControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A set of parameters that control the creation of a

report of the top N matrix entries according to

a selected metric."

::= { nlMatrix 4 }

nlMatrixTopNControlEntry OBJECT-TYPE

SYNTAX      NlMatrixTopNControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the nlMatrixTopNControlTable.

An example of the indexing of this table is

nlMatrixTopNControlDuration.3"

INDEX { nlMatrixTopNControlIndex }

::= { nlMatrixTopNControlTable 1 }

NlMatrixTopNControlEntry ::= SEQUENCE {

nlMatrixTopNControlIndex            Integer32,

Waldbusser                  Standards Track                    [Page 54]

RFC 2021             Remote Network Monitoring MIB          January 1997

nlMatrixTopNControlMatrixIndex      Integer32,

nlMatrixTopNControlRateBase         INTEGER,

nlMatrixTopNControlTimeRemaining    Integer32,

nlMatrixTopNControlGeneratedReports Counter32,

nlMatrixTopNControlDuration         Integer32,

nlMatrixTopNControlRequestedSize    Integer32,

nlMatrixTopNControlGrantedSize      Integer32,

nlMatrixTopNControlStartTime        TimeStamp,

nlMatrixTopNControlOwner            OwnerString,

nlMatrixTopNControlStatus           RowStatus

}

nlMatrixTopNControlIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An index that uniquely identifies an entry

in the nlMatrixTopNControlTable.  Each such

entry defines one top N report prepared for

one interface."

::= { nlMatrixTopNControlEntry 1 }

nlMatrixTopNControlMatrixIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The nlMatrix[SD/DS] table for which a top N report will be

prepared on behalf of this entry.  The nlMatrix[SD/DS] table

is identified by the value of the hlMatrixControlIndex

for that table - that value is used here to identify the

particular table.

This object may not be modified if the associated

nlMatrixTopNControlStatus object is equal to active(1)."

::= { nlMatrixTopNControlEntry 2 }

nlMatrixTopNControlRateBase OBJECT-TYPE

SYNTAX      INTEGER {

nlMatrixTopNPkts(1),

nlMatrixTopNOctets(2)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The variable for each nlMatrix[SD/DS] entry that the

nlMatrixTopNEntries are sorted by.

Waldbusser                  Standards Track                    [Page 55]

RFC 2021             Remote Network Monitoring MIB          January 1997

This object may not be modified if the associated

nlMatrixTopNControlStatus object is equal to active(1)."

::= { nlMatrixTopNControlEntry 3 }

nlMatrixTopNControlTimeRemaining OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The number of seconds left in the report currently

being collected.  When this object is modified by

the management station, a new collection is started,

possibly aborting a currently running report.  The

new value is used as the requested duration of this

report, and is immediately loaded into the associated

nlMatrixTopNControlDuration object.

When the report finishes, the probe will automatically

start another collection with the same initial value

of nlMatrixTopNControlTimeRemaining.  Thus the management

station may simply read the resulting reports repeatedly,

checking the startTime and duration each time to ensure that a

report was not missed or that the report parameters were not

changed.

While the value of this object is non-zero, it decrements

by one per second until it reaches zero.  At the time

that this object decrements to zero, the report is made

accessible in the nlMatrixTopNTable, overwriting any report

that may be there.

When this object is modified by the management station, any

associated entries in the nlMatrixTopNTable shall be deleted.

(Note that this is a different algorithm than the one used in

the hostTopNTable)."

DEFVAL { 1800 }

::= { nlMatrixTopNControlEntry 4 }

nlMatrixTopNControlGeneratedReports OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of reports that have been generated by this entry."

::= { nlMatrixTopNControlEntry 5 }

nlMatrixTopNControlDuration OBJECT-TYPE

SYNTAX     Integer32

Waldbusser                  Standards Track                    [Page 56]

RFC 2021             Remote Network Monitoring MIB          January 1997

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of seconds that this report has collected

during the last sampling interval.

When the associated nlMatrixTopNControlTimeRemaining object is

set, this object shall be set by the probe to the

same value and shall not be modified until the next

time the nlMatrixTopNControlTimeRemaining is set.

This value shall be zero if no reports have been

requested for this nlMatrixTopNControlEntry."

::= { nlMatrixTopNControlEntry 6 }

nlMatrixTopNControlRequestedSize OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The maximum number of matrix entries requested for this report.

When this object is created or modified, the probe

should set nlMatrixTopNControlGrantedSize as closely to this

object as is possible for the particular probe

implementation and available resources."

DEFVAL { 150 }

::= { nlMatrixTopNControlEntry 7 }

nlMatrixTopNControlGrantedSize OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum number of matrix entries in this report.

When the associated nlMatrixTopNControlRequestedSize object is

created or modified, the probe should set this

object as closely to the requested value as is

possible for the particular implementation and

available resources. The probe must not lower this

value except as a result of a set to the associated

nlMatrixTopNControlRequestedSize object.

If the value of nlMatrixTopNControlRateBase is equal to

nlMatrixTopNPkts, when the next topN report is generated,

matrix entries with the highest value of nlMatrixTopNPktRate

shall be placed in this table in decreasing order of this rate

until there is no more room or until there are no more

Waldbusser                  Standards Track                    [Page 57]

RFC 2021             Remote Network Monitoring MIB          January 1997

matrix entries.

If the value of nlMatrixTopNControlRateBase is equal to

nlMatrixTopNOctets, when the next topN report is generated,

matrix entries with the highest value of nlMatrixTopNOctetRate

shall be placed in this table in decreasing order of this rate

until there is no more room or until there are no more

matrix entries.

It is an implementation-specific matter how entries with the

same value of nlMatrixTopNPktRate or nlMatrixTopNOctetRate are

sorted.  It is also an implementation-specific matter as to

whether or not zero-valued entries are available."

::= { nlMatrixTopNControlEntry 8 }

nlMatrixTopNControlStartTime OBJECT-TYPE

SYNTAX     TimeStamp

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this top N report was

last started.  In other words, this is the time that

the associated nlMatrixTopNControlTimeRemaining object was

modified to start the requested report or the time

the report was last automatically (re)started.

This object may be used by the management station to

determine if a report was missed or not."

::= { nlMatrixTopNControlEntry 9 }

nlMatrixTopNControlOwner OBJECT-TYPE

SYNTAX     OwnerString

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { nlMatrixTopNControlEntry 10 }

nlMatrixTopNControlStatus OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The status of this nlMatrixTopNControlEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

Waldbusser                  Standards Track                    [Page 58]

RFC 2021             Remote Network Monitoring MIB          January 1997

If this object is not equal to active(1), all

associated entries in the nlMatrixTopNTable shall be deleted

by the agent."

::= { nlMatrixTopNControlEntry 11 }

nlMatrixTopNTable OBJECT-TYPE

SYNTAX     SEQUENCE OF NlMatrixTopNEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A set of statistics for those network layer matrix entries

that have counted the highest number of octets or packets."

::= { nlMatrix 5 }

nlMatrixTopNEntry OBJECT-TYPE

SYNTAX     NlMatrixTopNEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A conceptual row in the nlMatrixTopNTable.

The nlMatrixTopNControlIndex value in the index identifies the

nlMatrixTopNControlEntry on whose behalf this entry was

created.

An example of the indexing of this table is

nlMatrixTopNPktRate.3.10"

INDEX { nlMatrixTopNControlIndex, nlMatrixTopNIndex }

::= { nlMatrixTopNTable 1 }

NlMatrixTopNEntry ::= SEQUENCE {

nlMatrixTopNIndex                 Integer32,

nlMatrixTopNProtocolDirLocalIndex Integer32,

nlMatrixTopNSourceAddress         OCTET STRING,

nlMatrixTopNDestAddress           OCTET STRING,

nlMatrixTopNPktRate               Gauge32,

nlMatrixTopNReversePktRate        Gauge32,

nlMatrixTopNOctetRate             Gauge32,

nlMatrixTopNReverseOctetRate      Gauge32

}

nlMatrixTopNIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An index that uniquely identifies an entry in

the nlMatrixTopNTable among those in the same report.

Waldbusser                  Standards Track                    [Page 59]

RFC 2021             Remote Network Monitoring MIB          January 1997

This index is between 1 and N, where N is the

number of entries in this report.

If the value of nlMatrixTopNControlRateBase is equal to

nlMatrixTopNPkts, increasing values of nlMatrixTopNIndex shall

be assigned to entries with decreasing values of

nlMatrixTopNPktRate until index N is assigned or there are no

more nlMatrixTopNEntries.

If the value of nlMatrixTopNControlRateBase is equal to

nlMatrixTopNOctets, increasing values of nlMatrixTopNIndex

shall be assigned to entries with decreasing values of

nlMatrixTopNOctetRate until index N is assigned or there are

no more nlMatrixTopNEntries."

::= { nlMatrixTopNEntry 1 }

nlMatrixTopNProtocolDirLocalIndex OBJECT-TYPE

SYNTAX     Integer32 (1..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The protocolDirLocalIndex of the network layer protocol of

this entry's network address."

::= { nlMatrixTopNEntry 2 }

nlMatrixTopNSourceAddress OBJECT-TYPE

SYNTAX     OCTET STRING

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The network layer address of the source host in this

conversation.

This is represented as an octet string with

specific semantics and length as identified

by the associated nlMatrixTopNProtocolDirLocalIndex.

For example, if the protocolDirLocalIndex indicates an

encapsulation of ip, this object is encoded as a length

octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixTopNEntry 3 }

nlMatrixTopNDestAddress OBJECT-TYPE

SYNTAX     OCTET STRING

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

Waldbusser                  Standards Track                    [Page 60]

RFC 2021             Remote Network Monitoring MIB          January 1997

"The network layer address of the destination host in this

conversation.

This is represented as an octet string with

specific semantics and length as identified

by the associated nlMatrixTopNProtocolDirLocalIndex.

For example, if the nlMatrixTopNProtocolDirLocalIndex

indicates an encapsulation of ip, this object is encoded as a

length octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { nlMatrixTopNEntry 4 }

nlMatrixTopNPktRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of packets seen from the source host

to the destination host during this sampling interval, counted

using the rules for counting the nlMatrixSDPkts object.

If the value of nlMatrixTopNControlRateBase is

nlMatrixTopNPkts, this variable will be used to sort this

report."

::= { nlMatrixTopNEntry 5 }

nlMatrixTopNReversePktRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of packets seen from the destination host to the

source host during this sampling interval, counted

using the rules for counting the nlMatrixSDPkts object (note

that the corresponding nlMatrixSDPkts object selected is the

one whose source address is equal to nlMatrixTopNDestAddress

and whose destination address is equal to

nlMatrixTopNSourceAddress.)

Note that if the value of nlMatrixTopNControlRateBase is equal

to nlMatrixTopNPkts, the sort of topN entries is based

entirely on nlMatrixTopNPktRate, and not on the value of this

object."

::= { nlMatrixTopNEntry 6 }

nlMatrixTopNOctetRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

Waldbusser                  Standards Track                    [Page 61]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS     current

DESCRIPTION

"The number of octets seen from the source host

to the destination host during this sampling interval, counted

using the rules for counting the nlMatrixSDOctets object.  If

the value of nlMatrixTopNControlRateBase is

nlMatrixTopNOctets, this variable will be used to sort this

report."

::= { nlMatrixTopNEntry 7 }

nlMatrixTopNReverseOctetRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of octets seen from the destination host to the

source host during this sampling interval, counted

using the rules for counting the nlMatrixDSOctets object (note

that the corresponding nlMatrixSDOctets object selected is the

one whose source address is equal to nlMatrixTopNDestAddress

and whose destination address is equal to

nlMatrixTopNSourceAddress.)

Note that if the value of nlMatrixTopNControlRateBase is equal

to nlMatrixTopNOctets, the sort of topN entries is based

entirely on nlMatrixTopNOctetRate, and not on the value of

this object."

::= { nlMatrixTopNEntry 8 }

-- Application Layer Functions

--

-- The application layer host, matrix, and matrixTopN functions report

-- on protocol usage at the network layer or higher.  Note that the

-- use of the term application layer does not imply that only

-- application-layer protocols are counted, rather it means that

-- protocols up to and including the application layer are supported.

--

-- Application Layer Host Group

--

-- Counts the amount of traffic, by protocol, sent from and to each

-- network address discovered by the probe.

-- Implementation of this group requires implementation of the Network

-- Layer Host Group.

alHostTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AlHostEntry

MAX-ACCESS  not-accessible

Waldbusser                  Standards Track                    [Page 62]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS      current

DESCRIPTION

"A collection of statistics for a particular protocol from a

particular network address that has been discovered on an

interface of this device.

The probe will populate this table for all protocols in the

protocol directory table whose value of

protocolDirHostConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirHostConfig value of supportedOff(2).

The probe will add to this table all addresses

seen as the source or destination address in all packets with

no MAC errors, and will increment octet and packet counts in

the table for all packets with no MAC errors.  Further,

entries will only be added to this table if their address

exists in the nlHostTable and will be deleted from this table

if their address is deleted from the nlHostTable."

::= { alHost 1 }

alHostEntry OBJECT-TYPE

SYNTAX      AlHostEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the alHostTable.

The hlHostControlIndex value in the index identifies the

hlHostControlEntry on whose behalf this entry was created.

The first protocolDirLocalIndex value in the index identifies

the network layer protocol of the address.

The nlHostAddress value in the index identifies the network

layer address of this entry.

The second protocolDirLocalIndex value in the index identifies

the protocol that is counted by this entry.

An example of the indexing in this entry is

alHostOutPkts.1.783495.18.4.128.2.6.6.34"

INDEX { hlHostControlIndex, alHostTimeMark,

protocolDirLocalIndex, nlHostAddress,

protocolDirLocalIndex }

::= { alHostTable 1 }

AlHostEntry ::= SEQUENCE {

alHostTimeMark                 TimeFilter,

alHostInPkts                   ZeroBasedCounter32,

alHostOutPkts                  ZeroBasedCounter32,

Waldbusser                  Standards Track                    [Page 63]

RFC 2021             Remote Network Monitoring MIB          January 1997

alHostInOctets                 ZeroBasedCounter32,

alHostOutOctets                ZeroBasedCounter32,

alHostCreateTime               LastCreateTime

}

alHostTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { alHostEntry 1 }

alHostInPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets of this protocol type without errors

transmitted to this address since it was added to the

alHostTable.  Note that this is the number of link-layer

packets, so if a single network-layer packet is fragmented

into several link-layer frames, this counter is incremented

several times."

::= { alHostEntry 2 }

alHostOutPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets of this protocol type without errors

transmitted by this address since it was added to the

alHostTable.  Note that this is the number of link-layer

packets, so if a single network-layer packet is fragmented

into several link-layer frames, this counter is incremented

several times."

::= { alHostEntry 3 }

alHostInOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted to this address

of this protocol type since it was added to the

alHostTable (excluding framing bits but including

Waldbusser                  Standards Track                    [Page 64]

RFC 2021             Remote Network Monitoring MIB          January 1997

FCS octets), excluding those octets in packets that

contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { alHostEntry 4 }

alHostOutOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets transmitted by this address

of this protocol type since it was added to the

alHostTable (excluding framing bits but including

FCS octets), excluding those octets in packets that

contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { alHostEntry 5 }

alHostCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { alHostEntry 6 }

--

-- Application Layer Matrix Group

--

-- Counts the amount of traffic, by protocol, sent between each pair

-- of network addresses discovered by the probe.

-- Implementation of this group requires implementation of the Network

-- Layer Matrix Group.

alMatrixSDTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AlMatrixSDEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of application traffic matrix entries which collect

Waldbusser                  Standards Track                    [Page 65]

RFC 2021             Remote Network Monitoring MIB          January 1997

statistics for conversations of a particular protocol between

two network-level addresses.  This table is indexed first by

the source address and then by the destination address to make

it convenient to collect all statistics from a particular

address.

The probe will populate this table for all protocols in the

protocol directory table whose value of

protocolDirMatrixConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirMatrixConfig value of supportedOff(2).

The probe will add to this table all pairs of addresses for

all protocols seen in all packets with no MAC errors, and will

increment octet and packet counts in the table for all packets

with no MAC errors.  Further, entries will only be added to

this table if their address pair exists in the nlMatrixSDTable

and will be deleted from this table if the address pair is

deleted from the nlMatrixSDTable."

::= { alMatrix 1 }

alMatrixSDEntry OBJECT-TYPE

SYNTAX      AlMatrixSDEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the alMatrixSDTable.

The hlMatrixControlIndex value in the index identifies the

hlMatrixControlEntry on whose behalf this entry was created.

The first protocolDirLocalIndex value in the index identifies

the network layer protocol of the nlMatrixSDSourceAddress and

nlMatrixSDDestAddress.

The nlMatrixSDSourceAddress value in the index identifies the

network layer address of the source host in this conversation.

The nlMatrixSDDestAddress value in the index identifies the

network layer address of the destination host in this

conversation.

The second protocolDirLocalIndex value in the index identifies

the protocol that is counted by this entry.

An example of the indexing of this entry is

alMatrixSDPkts.1.783495.18.4.128.2.6.6.4.128.2.6.7.34"

INDEX { hlMatrixControlIndex, alMatrixSDTimeMark,

protocolDirLocalIndex,

nlMatrixSDSourceAddress, nlMatrixSDDestAddress,

protocolDirLocalIndex }

::= { alMatrixSDTable 1 }

Waldbusser                  Standards Track                    [Page 66]

RFC 2021             Remote Network Monitoring MIB          January 1997

AlMatrixSDEntry ::= SEQUENCE {

alMatrixSDTimeMark                 TimeFilter,

alMatrixSDPkts                     ZeroBasedCounter32,

alMatrixSDOctets                   ZeroBasedCounter32,

alMatrixSDCreateTime               LastCreateTime

}

alMatrixSDTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { alMatrixSDEntry 1 }

alMatrixSDPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets of this protocol type without errors

transmitted from the source address to the destination address

since this entry was added to the alMatrixSDTable.  Note that

this is the number of link-layer packets, so if a single

network-layer packet is fragmented into several link-layer

frames, this counter is incremented several times."

::= { alMatrixSDEntry 2 }

alMatrixSDOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of octets in packets of this protocol type

transmitted from the source address to the destination address

since this entry was added to the alMatrixSDTable (excluding

framing bits but including FCS octets), excluding those octets

in packets that contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { alMatrixSDEntry 3 }

alMatrixSDCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

Waldbusser                  Standards Track                    [Page 67]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { alMatrixSDEntry 4 }

-- Traffic matrix tables from destination to source

alMatrixDSTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AlMatrixDSEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A list of application traffic matrix entries which collect

statistics for conversations of a particular protocol between

two network-level addresses.  This table is indexed first by

the destination address and then by the source address to make

it convenient to collect all statistics to a particular

address.

The probe will populate this table for all protocols in the

protocol directory table whose value of

protocolDirMatrixConfig is equal to supportedOn(3), and

will delete any entries whose protocolDirEntry is deleted or

has a protocolDirMatrixConfig value of supportedOff(2).

The probe will add to this table all pairs of addresses for

all protocols seen in all packets with no MAC errors, and will

increment octet and packet counts in the table for all packets

with no MAC errors.  Further, entries will only be added to

this table if their address pair exists in the nlMatrixDSTable

and will be deleted from this table if the address pair is

deleted from the nlMatrixDSTable."

::= { alMatrix 2 }

alMatrixDSEntry OBJECT-TYPE

SYNTAX      AlMatrixDSEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the alMatrixDSTable.

The hlMatrixControlIndex value in the index identifies the

hlMatrixControlEntry on whose behalf this entry was created.

The first protocolDirLocalIndex value in the index identifies

the network layer protocol of the alMatrixDSSourceAddress and

alMatrixDSDestAddress.

Waldbusser                  Standards Track                    [Page 68]

RFC 2021             Remote Network Monitoring MIB          January 1997

The nlMatrixDSDestAddress value in the index identifies the

network layer address of the destination host in this

conversation.

The nlMatrixDSSourceAddress value in the index identifies the

network layer address of the source host in this conversation.

The second protocolDirLocalIndex value in the index identifies

the protocol that is counted by this entry.

An example of the indexing of this entry is

alMatrixDSPkts.1.783495.18.4.128.2.6.7.4.128.2.6.6.34"

INDEX { hlMatrixControlIndex, alMatrixDSTimeMark,

protocolDirLocalIndex,

nlMatrixDSDestAddress, nlMatrixDSSourceAddress,

protocolDirLocalIndex }

::= { alMatrixDSTable 1 }

AlMatrixDSEntry ::= SEQUENCE {

alMatrixDSTimeMark                 TimeFilter,

alMatrixDSPkts                     ZeroBasedCounter32,

alMatrixDSOctets                   ZeroBasedCounter32,

alMatrixDSCreateTime               LastCreateTime

}

alMatrixDSTimeMark OBJECT-TYPE

SYNTAX      TimeFilter

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A TimeFilter for this entry.  See the TimeFilter textual

convention to see how this works."

::= { alMatrixDSEntry 1 }

alMatrixDSPkts OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of packets of this protocol type without errors

transmitted from the source address to the destination address

since this entry was added to the alMatrixDSTable.  Note that

this is the number of link-layer packets, so if a single

network-layer packet is fragmented into several link-layer

frames, this counter is incremented several times."

::= { alMatrixDSEntry 2 }

alMatrixDSOctets OBJECT-TYPE

SYNTAX      ZeroBasedCounter32

MAX-ACCESS  read-only

Waldbusser                  Standards Track                    [Page 69]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS      current

DESCRIPTION

"The number of octets in packets of this protocol type

transmitted from the source address to the destination address

since this entry was added to the alMatrixDSTable (excluding

framing bits but including FCS octets), excluding those octets

in packets that contained errors.

Note this doesn't count just those octets in the particular

protocol frames, but includes the entire packet that contained

the protocol."

::= { alMatrixDSEntry 3 }

alMatrixDSCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this entry was last activated.

This can be used by the management station to ensure that the

entry has not been deleted and recreated between polls."

::= { alMatrixDSEntry 4 }

alMatrixTopNControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF AlMatrixTopNControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A set of parameters that control the creation of a

report of the top N matrix entries according to

a selected metric."

::= { alMatrix 3 }

alMatrixTopNControlEntry OBJECT-TYPE

SYNTAX      AlMatrixTopNControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A conceptual row in the alMatrixTopNControlTable.

An example of the indexing of this table is

alMatrixTopNControlDuration.3"

INDEX { alMatrixTopNControlIndex }

::= { alMatrixTopNControlTable 1 }

AlMatrixTopNControlEntry ::= SEQUENCE {

alMatrixTopNControlIndex            Integer32,

alMatrixTopNControlMatrixIndex      Integer32,

Waldbusser                  Standards Track                    [Page 70]

RFC 2021             Remote Network Monitoring MIB          January 1997

alMatrixTopNControlRateBase         INTEGER,

alMatrixTopNControlTimeRemaining    Integer32,

alMatrixTopNControlGeneratedReports Counter32,

alMatrixTopNControlDuration         Integer32,

alMatrixTopNControlRequestedSize    Integer32,

alMatrixTopNControlGrantedSize      Integer32,

alMatrixTopNControlStartTime        TimeStamp,

alMatrixTopNControlOwner            OwnerString,

alMatrixTopNControlStatus           RowStatus

}

alMatrixTopNControlIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An index that uniquely identifies an entry

in the alMatrixTopNControlTable.  Each such

entry defines one top N report prepared for

one interface."

::= { alMatrixTopNControlEntry 1 }

alMatrixTopNControlMatrixIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The alMatrix[SD/DS] table for which a top N report will be

prepared on behalf of this entry.  The alMatrix[SD/DS] table

is identified by the value of the hlMatrixControlIndex

for that table - that value is used here to identify the

particular table.

This object may not be modified if the associated

alMatrixTopNControlStatus object is equal to active(1)."

::= { alMatrixTopNControlEntry 2 }

alMatrixTopNControlRateBase OBJECT-TYPE

SYNTAX     INTEGER {

alMatrixTopNTerminalsPkts(1),

alMatrixTopNTerminalsOctets(2),

alMatrixTopNAllPkts(3),

alMatrixTopNAllOctets(4)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The variable for each alMatrix[SD/DS] entry that the

Waldbusser                  Standards Track                    [Page 71]

RFC 2021             Remote Network Monitoring MIB          January 1997

alMatrixTopNEntries are sorted by, as well as the

selector of the view of the matrix table that will be

used.

The values alMatrixTopNTerminalsPkts and

alMatrixTopNTerminalsOctets cause collection only from

protocols that have no child protocols that are counted.  The

values alMatrixTopNAllPkts and alMatrixTopNAllOctets cause

collection from all alMatrix entries.

This object may not be modified if the associated

alMatrixTopNControlStatus object is equal to active(1)."

::= { alMatrixTopNControlEntry 3 }

alMatrixTopNControlTimeRemaining OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The number of seconds left in the report currently

being collected.  When this object is modified by

the management station, a new collection is started,

possibly aborting a currently running report.  The

new value is used as the requested duration of this

report, and is immediately loaded into the associated

alMatrixTopNControlDuration object.

When the report finishes, the probe will automatically

start another collection with the same initial value

of alMatrixTopNControlTimeRemaining.  Thus the management

station may simply read the resulting reports repeatedly,

checking the startTime and duration each time to ensure that a

report was not missed or that the report parameters were not

changed.

While the value of this object is non-zero, it decrements

by one per second until it reaches zero.  At the time

that this object decrements to zero, the report is made

accessible in the alMatrixTopNTable, overwriting any report

that may be there.

When this object is modified by the management station, any

associated entries in the alMatrixTopNTable shall be deleted.

(Note that this is a different algorithm than the one used in

the hostTopNTable)."

DEFVAL { 1800 }

::= { alMatrixTopNControlEntry 4 }

Waldbusser                  Standards Track                    [Page 72]

RFC 2021             Remote Network Monitoring MIB          January 1997

alMatrixTopNControlGeneratedReports OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of reports that have been generated by this entry."

::= { alMatrixTopNControlEntry 5 }

alMatrixTopNControlDuration OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of seconds that this report has collected

during the last sampling interval.

When the associated alMatrixTopNControlTimeRemaining object

is set, this object shall be set by the probe to the

same value and shall not be modified until the next

time the alMatrixTopNControlTimeRemaining is set.

This value shall be zero if no reports have been

requested for this alMatrixTopNControlEntry."

::= { alMatrixTopNControlEntry 6 }

alMatrixTopNControlRequestedSize OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The maximum number of matrix entries requested for this report.

When this object is created or modified, the probe

should set alMatrixTopNControlGrantedSize as closely to this

object as is possible for the particular probe

implementation and available resources."

DEFVAL { 150 }

::= { alMatrixTopNControlEntry 7 }

alMatrixTopNControlGrantedSize OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum number of matrix entries in this report.

When the associated alMatrixTopNControlRequestedSize object

is created or modified, the probe should set this

Waldbusser                  Standards Track                    [Page 73]

RFC 2021             Remote Network Monitoring MIB          January 1997

object as closely to the requested value as is

possible for the particular implementation and

available resources. The probe must not lower this

value except as a result of a set to the associated

alMatrixTopNControlRequestedSize object.

If the value of alMatrixTopNControlRateBase is equal to

alMatrixTopNTerminalsPkts or alMatrixTopNAllPkts, when the

next topN report is generated, matrix entries with the highest

value of alMatrixTopNPktRate shall be placed in this table in

decreasing order of this rate until there is no more room or

until there are no more matrix entries.

If the value of alMatrixTopNControlRateBase is equal to

alMatrixTopNTerminalsOctets or alMatrixTopNAllOctets, when the

next topN report is generated, matrix entries with the highest

value of alMatrixTopNOctetRate shall be placed in this table

in decreasing order of this rate until there is no more room

or until there are no more matrix entries.

It is an implementation-specific matter how entries with the

same value of alMatrixTopNPktRate or alMatrixTopNOctetRate are

sorted.  It is also an implementation-specific matter as to

whether or not zero-valued entries are available."

::= { alMatrixTopNControlEntry 8 }

alMatrixTopNControlStartTime OBJECT-TYPE

SYNTAX     TimeStamp

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this top N report was

last started.  In other words, this is the time that

the associated alMatrixTopNControlTimeRemaining object

was modified to start the requested report or the time

the report was last automatically (re)started.

This object may be used by the management station to

determine if a report was missed or not."

::= { alMatrixTopNControlEntry 9 }

alMatrixTopNControlOwner OBJECT-TYPE

SYNTAX     OwnerString

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

Waldbusser                  Standards Track                    [Page 74]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { alMatrixTopNControlEntry 10 }

alMatrixTopNControlStatus OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The status of this alMatrixTopNControlEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all

associated entries in the alMatrixTopNTable shall be

deleted by the agent."

::= { alMatrixTopNControlEntry 11 }

alMatrixTopNTable OBJECT-TYPE

SYNTAX     SEQUENCE OF AlMatrixTopNEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A set of statistics for those application layer matrix

entries that have counted the highest number of octets or

packets."

::= { alMatrix 4 }

alMatrixTopNEntry OBJECT-TYPE

SYNTAX     AlMatrixTopNEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A conceptual row in the alMatrixTopNTable.

The alMatrixTopNControlIndex value in the index identifies

the alMatrixTopNControlEntry on whose behalf this entry was

created.

An example of the indexing of this table is

alMatrixTopNPktRate.3.10"

INDEX { alMatrixTopNControlIndex, alMatrixTopNIndex }

::= { alMatrixTopNTable 1 }

AlMatrixTopNEntry ::= SEQUENCE {

alMatrixTopNIndex                      Integer32,

alMatrixTopNProtocolDirLocalIndex      Integer32,

alMatrixTopNSourceAddress              OCTET STRING,

alMatrixTopNDestAddress                OCTET STRING,

Waldbusser                  Standards Track                    [Page 75]

RFC 2021             Remote Network Monitoring MIB          January 1997

alMatrixTopNAppProtocolDirLocalIndex   Integer32,

alMatrixTopNPktRate                    Gauge32,

alMatrixTopNReversePktRate             Gauge32,

alMatrixTopNOctetRate                  Gauge32,

alMatrixTopNReverseOctetRate           Gauge32

}

alMatrixTopNIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An index that uniquely identifies an entry in

the alMatrixTopNTable among those in the same report.

This index is between 1 and N, where N is the

number of entries in this report.

If the value of alMatrixTopNControlRateBase is equal to

alMatrixTopNTerminalsPkts or alMatrixTopNAllPkts, increasing

values of alMatrixTopNIndex shall be assigned to entries with

decreasing values of alMatrixTopNPktRate until index N is

assigned or there are no more alMatrixTopNEntries.

If the value of alMatrixTopNControlRateBase is equal to

alMatrixTopNTerminalsOctets or alMatrixTopNAllOctets,

increasing values of alMatrixTopNIndex shall be assigned to

entries with decreasing values of alMatrixTopNOctetRate until

index N is assigned or there are no more alMatrixTopNEntries."

::= { alMatrixTopNEntry 1 }

alMatrixTopNProtocolDirLocalIndex OBJECT-TYPE

SYNTAX     Integer32 (1..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The protocolDirLocalIndex of the network layer protocol of

this entry's network address."

::= { alMatrixTopNEntry 2 }

alMatrixTopNSourceAddress OBJECT-TYPE

SYNTAX     OCTET STRING

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The network layer address of the source host in this

conversation.

This is represented as an octet string with

specific semantics and length as identified

Waldbusser                  Standards Track                    [Page 76]

RFC 2021             Remote Network Monitoring MIB          January 1997

by the associated alMatrixTopNProtocolDirLocalIndex.

For example, if the alMatrixTopNProtocolDirLocalIndex

indicates an encapsulation of ip, this object is encoded as a

length octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { alMatrixTopNEntry 3 }

alMatrixTopNDestAddress OBJECT-TYPE

SYNTAX     OCTET STRING

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The network layer address of the destination host in this

conversation.

This is represented as an octet string with

specific semantics and length as identified

by the associated alMatrixTopNProtocolDirLocalIndex.

For example, if the alMatrixTopNProtocolDirLocalIndex

indicates an encapsulation of ip, this object is encoded as a

length octet of 4, followed by the 4 octets of the ip address,

in network byte order."

::= { alMatrixTopNEntry 4 }

alMatrixTopNAppProtocolDirLocalIndex OBJECT-TYPE

SYNTAX     Integer32 (1..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of the protocol counted by this matrix entry."

::= { alMatrixTopNEntry 5 }

alMatrixTopNPktRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of packets seen of this protocol from the source

host to the destination host during this sampling interval,

counted using the rules for counting the alMatrixSDPkts

object.

If the value of alMatrixTopNControlRateBase is

alMatrixTopNTerminalsPkts or alMatrixTopNAllPkts, this

variable will be used to sort this report."

::= { alMatrixTopNEntry 6 }

Waldbusser                  Standards Track                    [Page 77]

RFC 2021             Remote Network Monitoring MIB          January 1997

alMatrixTopNReversePktRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of packets seen of this protocol from the

destination host to the source host during this sampling

interval, counted using the rules for counting the

alMatrixDSPkts object  (note that the corresponding

alMatrixSDPkts object selected is the one whose source address

is equal to alMatrixTopNDestAddress and whose destination

address is equal to alMatrixTopNSourceAddress.)

Note that if the value of alMatrixTopNControlRateBase is equal

to alMatrixTopNTerminalsPkts or alMatrixTopNAllPkts, the sort

of topN entries is based entirely on alMatrixTopNPktRate, and

not on the value of this object."

::= { alMatrixTopNEntry 7 }

alMatrixTopNOctetRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of octets seen of this protocol from the source

host to the destination host during this sampling interval,

counted using the rules for counting the alMatrixSDOctets

object.

If the value of alMatrixTopNControlRateBase is

alMatrixTopNTerminalsOctets or alMatrixTopNAllOctets, this

variable will be used to sort this report."

::= { alMatrixTopNEntry 8 }

alMatrixTopNReverseOctetRate OBJECT-TYPE

SYNTAX     Gauge32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of octets seen of this protocol from the

destination host to the source host during this sampling

interval, counted using the rules for counting the

alMatrixDSOctets object  (note that the corresponding

alMatrixSDOctets object selected is the one whose source

address is equal to alMatrixTopNDestAddress and whose

destination address is equal to alMatrixTopNSourceAddress.)

Note that if the value of alMatrixTopNControlRateBase is equal

Waldbusser                  Standards Track                    [Page 78]

RFC 2021             Remote Network Monitoring MIB          January 1997

to alMatrixTopNTerminalsOctets or alMatrixTopNAllOctets, the

sort of topN entries is based entirely on

alMatrixTopNOctetRate, and not on the value of this object."

::= { alMatrixTopNEntry 9 }

--

-- User History Collection Group (usrHistory)

--

-- The usrHistory group combines mechanisms seen in the alarm and

-- history groups to provide user-specified history collection,

-- utilizing two additional control tables and one additional data

-- table. This function has traditionally been done by NMS

-- applications, via periodic polling.  The usrHistory group allows

-- this task to be offloaded to an RMON probe.

--

-- Data (an ASN.1 INTEGER based object) is collected in the same

-- manner as any history data table (e.g. etherHistoryTable) except

-- that the user specifies the MIB instances to be collected. Objects

-- are collected in bucket-groups, with the intent that all MIB

-- instances in the same bucket-group are collected as atomically as

-- possible by the RMON probe.

--

-- The usrHistoryControlTable is a one-dimensional read-create table.

-- Each row configures a collection of user history buckets, much

-- the same as a historyControlEntry, except that the creation of a

-- row in this table will cause one or more associated instances in

-- the usrHistoryObjectTable to be created. The user specifies the

-- number of bucket elements (rows in the usrHistoryObjectTable)

-- requested, as well as the number of buckets requested.

--

-- The usrHistoryObjectTable is a 2-d read-write table.

-- Each row configures a single MIB instance to be collected.

-- All rows with the same major index constitute a bucket-group.

--

-- The usrHistoryTable is a 3-d read-only table containing

-- the data of associated usrHistoryControlEntries. Each

-- entry represents the value of a single MIB instance

-- during a specific sampling interval (or the rate of

-- change during the interval).

--

-- A sample value is stored in two objects - an absolute value and

-- a status object. This allows numbers from -(2G-1) to +4G to be

-- stored.  The status object also indicates whether a sample is

-- valid. This allows data collection to continue if periodic

-- retrieval of a particular instance fails for any reason.

--

-- Row Creation Order Relationships

--

Waldbusser                  Standards Track                    [Page 79]

RFC 2021             Remote Network Monitoring MIB          January 1997

-- The static nature of the usrHistoryObjectTable creates

-- some row creation/modification issues. The rows in this

-- table need to be set before the associated

-- usrHistoryControlEntry can be activated.

--

-- Note that the usrHistoryObject entries associated with a

-- particular usrHistoryControlEntry are not required to

-- be active before the control entry is activated. However,

-- the usrHistory data entries associated with an inactive

-- usrHistoryObject entry will be inactive (i.e.

-- usrHistoryValStatus == valueNotAvailable).

--

usrHistoryControlTable OBJECT-TYPE

SYNTAX SEQUENCE OF UsrHistoryControlEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A list of data-collection configuration entries."

::= { usrHistory 1 }

usrHistoryControlEntry OBJECT-TYPE

SYNTAX UsrHistoryControlEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A list of parameters that set up a group of user-defined

MIB objects to be sampled periodically (called a

bucket-group).

For example, an instance of usrHistoryControlInterval

might be named usrHistoryControlInterval.1"

INDEX { usrHistoryControlIndex }

::= { usrHistoryControlTable 1 }

UsrHistoryControlEntry ::= SEQUENCE {

usrHistoryControlIndex             Integer32,

usrHistoryControlObjects           Integer32,

usrHistoryControlBucketsRequested  Integer32,

usrHistoryControlBucketsGranted    Integer32,

usrHistoryControlInterval          Integer32,

usrHistoryControlOwner             OwnerString,

usrHistoryControlStatus            RowStatus

}

usrHistoryControlIndex OBJECT-TYPE

SYNTAX Integer32 (1..65535)

MAX-ACCESS not-accessible

Waldbusser                  Standards Track                    [Page 80]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS current

DESCRIPTION

"An index that uniquely identifies an entry in the

usrHistoryControlTable.  Each such entry defines a

set of samples at a particular interval for a specified

set of MIB instances available from the managed system."

::= { usrHistoryControlEntry 1 }

usrHistoryControlObjects OBJECT-TYPE

SYNTAX Integer32 (1..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The number of MIB objects to be collected

in the portion of usrHistoryTable associated with this

usrHistoryControlEntry.

This object may not be modified if the associated instance

of usrHistoryControlStatus is equal to active(1)."

::= { usrHistoryControlEntry 2 }

usrHistoryControlBucketsRequested OBJECT-TYPE

SYNTAX Integer32 (1..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The requested number of discrete time intervals

over which data is to be saved in the part of the

usrHistoryTable associated with this usrHistoryControlEntry.

When this object is created or modified, the probe

should set usrHistoryControlBucketsGranted as closely to

this object as is possible for the particular probe

implementation and available resources."

DEFVAL { 50 }

::= { usrHistoryControlEntry 3 }

usrHistoryControlBucketsGranted OBJECT-TYPE

SYNTAX Integer32 (1..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of discrete sampling intervals

over which data shall be saved in the part of

the usrHistoryTable associated with this

usrHistoryControlEntry.

When the associated usrHistoryControlBucketsRequested

Waldbusser                  Standards Track                    [Page 81]

RFC 2021             Remote Network Monitoring MIB          January 1997

object is created or modified, the probe should set

this object as closely to the requested value as is

possible for the particular  probe implementation and

available resources.  The probe must not lower this

value except as a result of a modification to the associated

usrHistoryControlBucketsRequested object.

The associated usrHistoryControlBucketsRequested object

should be set before or at the same time as this object

to allow the probe to accurately estimate the resources

required for this usrHistoryControlEntry.

There will be times when the actual number of buckets

associated with this entry is less than the value of

this object.  In this case, at the end of each sampling

interval, a new bucket will be added to the usrHistoryTable.

When the number of buckets reaches the value of this object

and a new bucket is to be added to the usrHistoryTable,

the oldest bucket associated with this usrHistoryControlEntry

shall be deleted by the agent so that the new bucket can be

added.

When the value of this object changes to a value less than

the current value, entries are deleted from the

usrHistoryTable associated with this usrHistoryControlEntry.

Enough of the oldest of these entries shall be deleted by the

agent so that their number remains less than or equal to the

new value of this object.

When the value of this object changes to a value greater

than the current value, the number of associated usrHistory

entries may be allowed to grow."

::= { usrHistoryControlEntry 4 }

usrHistoryControlInterval OBJECT-TYPE

SYNTAX Integer32 (1..2147483647)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The interval in seconds over which the data is

sampled for each bucket in the part of the usrHistory

table associated with this usrHistoryControlEntry.

Because the counters in a bucket may overflow at their

maximum value with no indication, a prudent manager will

take into account the possibility of overflow in any of

Waldbusser                  Standards Track                    [Page 82]

RFC 2021             Remote Network Monitoring MIB          January 1997

the associated counters. It is important to consider the

minimum time in which any counter could overflow on a

particular media type and set the usrHistoryControlInterval

object to a value less than this interval.

This object may not be modified if the associated

usrHistoryControlStatus object is equal to active(1)."

DEFVAL { 1800 }

::= { usrHistoryControlEntry 5 }

usrHistoryControlOwner OBJECT-TYPE

SYNTAX OwnerString

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { usrHistoryControlEntry 6 }

usrHistoryControlStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status of this variable history control entry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value.

If this object is not equal to active(1), all associated

entries in the usrHistoryTable shall be deleted."

::= { usrHistoryControlEntry 7 }

-- Object table

usrHistoryObjectTable OBJECT-TYPE

SYNTAX SEQUENCE OF UsrHistoryObjectEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A list of data-collection configuration entries."

::= { usrHistory 2 }

usrHistoryObjectEntry OBJECT-TYPE

SYNTAX UsrHistoryObjectEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

Waldbusser                  Standards Track                    [Page 83]

RFC 2021             Remote Network Monitoring MIB          January 1997

"A list of MIB instances to be sampled periodically.

Entries in this table are created when an associated

usrHistoryControlObjects object is created.

The usrHistoryControlIndex value in the index is

that of the associated usrHistoryControlEntry.

For example, an instance of usrHistoryObjectVariable might be

usrHistoryObjectVariable.1.3"

INDEX { usrHistoryControlIndex, usrHistoryObjectIndex }

::= { usrHistoryObjectTable 1 }

UsrHistoryObjectEntry ::= SEQUENCE {

usrHistoryObjectIndex             Integer32,

usrHistoryObjectVariable          OBJECT IDENTIFIER,

usrHistoryObjectSampleType        INTEGER

}

usrHistoryObjectIndex OBJECT-TYPE

SYNTAX Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An index used to uniquely identify an entry in the

usrHistoryObject table.  Each such entry defines a

MIB instance to be collected periodically."

::= { usrHistoryObjectEntry 1 }

usrHistoryObjectVariable OBJECT-TYPE

SYNTAX OBJECT IDENTIFIER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The object identifier of the particular variable to be

sampled.

Only variables that resolve to an ASN.1 primitive type of

Integer32 (Integer32, Counter, Gauge, or TimeTicks) may be

sampled.

Because SNMP access control is articulated entirely in terms

of the contents of MIB views, no access control mechanism

exists that can restrict the value of this object to identify

only those objects that exist in a particular MIB view.

Because there is thus no acceptable means of restricting the

read access that could be obtained through the user history

Waldbusser                  Standards Track                    [Page 84]

RFC 2021             Remote Network Monitoring MIB          January 1997

mechanism, the probe must only grant write access to this

object in those views that have read access to all objects on

the probe.

During a set operation, if the supplied variable name is not

available in the selected MIB view, a badValue error must be

returned.

This object may not be modified if the associated

usrHistoryControlStatus object is equal to active(1)."

::= { usrHistoryObjectEntry 2 }

usrHistoryObjectSampleType OBJECT-TYPE

SYNTAX INTEGER {

absoluteValue(1),

deltaValue(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The method of sampling the selected variable for storage in

the usrHistoryTable.

If the value of this object is absoluteValue(1), the value of

the selected variable will be copied directly into the history

bucket.

If the value of this object is deltaValue(2), the value of the

selected variable at the last sample will be subtracted from

the current value, and the difference will be stored in the

history bucket. If the associated usrHistoryObjectVariable

instance could not be obtained at the previous sample

interval, then a delta sample is not possible, and the value

of the associated usrHistoryValStatus object for this interval

will be valueNotAvailable(1).

This object may not be modified if the associated

usrHistoryControlStatus object is equal to active(1)."

::= { usrHistoryObjectEntry 3 }

-- data table

usrHistoryTable OBJECT-TYPE

SYNTAX SEQUENCE OF UsrHistoryEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A list of user defined history entries."

Waldbusser                  Standards Track                    [Page 85]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { usrHistory 3 }

usrHistoryEntry OBJECT-TYPE

SYNTAX UsrHistoryEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A historical sample of user-defined variables.  This sample

is associated with the usrHistoryControlEntry which set up the

parameters for a regular collection of these samples.

The usrHistoryControlIndex value in the index identifies the

usrHistoryControlEntry on whose behalf this entry was created.

The usrHistoryObjectIndex value in the index identifies the

usrHistoryObjectEntry on whose behalf this entry was created.

For example, an instance of usrHistoryAbsValue, which represents

the 14th sample of a variable collected as specified by

usrHistoryControlEntry.1 and usrHistoryObjectEntry.1.5,

would be named usrHistoryAbsValue.1.14.5"

INDEX { usrHistoryControlIndex, usrHistorySampleIndex,

usrHistoryObjectIndex }

::= { usrHistoryTable 1 }

UsrHistoryEntry ::= SEQUENCE {

usrHistorySampleIndex   Integer32,

usrHistoryIntervalStart TimeStamp,

usrHistoryIntervalEnd   TimeStamp,

usrHistoryAbsValue      Gauge32,

usrHistoryValStatus     INTEGER

}

usrHistorySampleIndex OBJECT-TYPE

SYNTAX     Integer32 (1..2147483647)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An index that uniquely identifies the particular sample this

entry represents among all samples associated with the same

usrHistoryControlEntry. This index starts at 1 and increases

by one as each new sample is taken."

::= { usrHistoryEntry 1 }

usrHistoryIntervalStart OBJECT-TYPE

SYNTAX TimeStamp

MAX-ACCESS read-only

STATUS current

Waldbusser                  Standards Track                    [Page 86]

RFC 2021             Remote Network Monitoring MIB          January 1997

DESCRIPTION

"The value of sysUpTime at the start of the interval over

which this sample was measured.  If the probe keeps track of

the time of day, it should start the first sample of the

history at a time such that when the next hour of the day

begins, a sample is started at that instant.

Note that following this rule may require the probe to delay

collecting the first sample of the history, as each sample

must be of the same interval. Also note that the sample which

is currently being collected is not accessible in this table

until the end of its interval."

::= { usrHistoryEntry 2 }

usrHistoryIntervalEnd OBJECT-TYPE

SYNTAX TimeStamp

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of sysUpTime at the end of the interval over which

this sample was measured."

::= { usrHistoryEntry 3 }

usrHistoryAbsValue OBJECT-TYPE

SYNTAX Gauge32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The absolute value (i.e. unsigned value) of the

user-specified statistic during the last sampling period. The

value during the current sampling period is not made available

until the period is completed.

To obtain the true value for this sampling interval, the

associated instance of usrHistoryValStatus must be checked,

and usrHistoryAbsValue adjusted as necessary.

If the MIB instance could not be accessed during the sampling

interval, then this object will have a value of zero and the

associated instance of usrHistoryValStatus will be set to

'valueNotAvailable(1)'."

::= { usrHistoryEntry 4 }

usrHistoryValStatus OBJECT-TYPE

SYNTAX INTEGER {

valueNotAvailable(1),

valuePositive(2),

Waldbusser                  Standards Track                    [Page 87]

RFC 2021             Remote Network Monitoring MIB          January 1997

valueNegative(3)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the validity and sign of the data in

the associated instance of usrHistoryAbsValue.

If the MIB instance could not be accessed during the sampling

interval, then 'valueNotAvailable(1)' will be returned.

If the sample is valid and actual value of the sample is

greater than or equal to zero then 'valuePositive(2)' is

returned.

If the sample is valid and the actual value of the sample is

less than zero, 'valueNegative(3)' will be returned. The

associated instance of usrHistoryAbsValue should be multiplied

by -1 to obtain the true sample value."

::= { usrHistoryEntry 5 }

-- The Probe Configuration Group

--

-- This group controls the configuration of various operating

-- parameters of the probe.

ControlString ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This data type is used to communicate with a modem or a

serial data switch.  A ControlString contains embedded

commands to control how the device will interact with the

remote device through the serial interface.  Commands are

represented as two character sequences beginning with

the `^' character.

The following commands are recognized by the device (note

that command characters are case sensitive):

^s  Send string that follows which is terminated by the

next command or the end of string.

^c  Delay for the number of seconds that follows.  Toss

out any data received rather than storing it in a

buffer for parsing.

^t  Set timeout to the value represented by the decimal

digits that follow.  The default timeout is 20

seconds. Note that this timeout may be overridden

by a smaller serialTimeout configured for the

Waldbusser                  Standards Track                    [Page 88]

RFC 2021             Remote Network Monitoring MIB          January 1997

associated serial interface (see serialConfigTable).

^w  Wait for the reply string that follows which is

terminated by the next command or the end of string.

Partial and case insensitive matching is applied, ie.

if the reply string (any case combination) is found

anywhere in the received string, then the a match is

found.  If the current timeout elapses without a match,

then the remaining control string is ignored.

^!  The ^ character.

^d  Delay the number of seconds specified by the decimal

digits that follow.

^b  Send break for the number of milliseconds specified by

the decimal digits that follow.  If no digits follow,

break will be enforced for 250 milliseconds by default.

The following ASCII control characters may be inserted into

the `^s' send string or the `^w' reply string:

^@    0x00

^A    0x01

..

^M    0x0D

..

^Z    0x1A

^[    0x1B

^    0x1C

^]    0x1D

^^    0x1E

^_    0x1F

Binary data may also be inserted into the data stream.  The

control sequence for each byte of binary data is ^0x##, where

## is the hexadecimal representation of the data byte.  Two

ASCII characters (0-9, a-f, A-F) must follow the `^0x'

control prefix.  For example, `^0x0D^0x0A' is interpreted as a

carriage return followed by a line feed."

SYNTAX DisplayString

probeCapabilities OBJECT-TYPE

SYNTAX BITS {

etherStats(0),

historyControl(1),

etherHistory(2),

alarm(3),

hosts(4),

hostTopN(5),

matrix(6),

filter(7),

Waldbusser                  Standards Track                    [Page 89]

RFC 2021             Remote Network Monitoring MIB          January 1997

capture(8),

event(9),

tokenRingMLStats(10),

tokenRingPStats(11),

tokenRingMLHistory(12),

tokenRingPHistory(13),

ringStation(14),

ringStationOrder(15),

ringStationConfig(16),

sourceRouting(17),

protocolDirectory(18),

protocolDistribution(19),

addressMapping(20),

nlHost(21),

nlMatrix(22),

alHost(23),

alMatrix(24),

usrHistory(25),

probeConfig(26)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"An indication of the RMON MIB groups supported

on at least one interface by this probe."

::= { probeConfig 1 }

probeSoftwareRev  OBJECT-TYPE

SYNTAX     DisplayString (SIZE(0..15))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The software revision of this device.  This string will have

a zero length if the revision is unknown."

::= { probeConfig 2 }

probeHardwareRev  OBJECT-TYPE

SYNTAX     DisplayString (SIZE(0..31))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The hardware revision of this device.  This string will have

a zero length if the revision is unknown."

::= { probeConfig 3 }

probeDateTime  OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE (0 | 8 | 11))

MAX-ACCESS read-write

Waldbusser                  Standards Track                    [Page 90]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS     current

DESCRIPTION

"Probe's current date and time.

field  octets  contents                  range

-----  ------  --------                  -----

1      1-2   year                      0..65536

2       3    month                     1..12

3       4    day                       1..31

4       5    hour                      0..23

5       6    minutes                   0..59

6       7    seconds                   0..60

(use 60 for leap-second)

7       8    deci-seconds              0..9

8       9    direction from UTC        '+' / '-'

9      10    hours from UTC            0..11

10      11    minutes from UTC          0..59

For example, Tuesday May 26, 1992 at 1:30:15 PM

EDT would be displayed as:

1992-5-26,13:30:15.0,-4:0

Note that if only local time is known, then

timezone information (fields 8-10) is not

present, and if no time information is known, the null

string is returned."

::= { probeConfig 4 }

probeResetControl  OBJECT-TYPE

SYNTAX     INTEGER {

running(1),

warmBoot(2),

coldBoot(3)

}

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Setting this object to warmBoot(2) causes the device to

restart the application software with current configuration

parameters saved in non-volatile memory.  Setting this

object to coldBoot(3) causes the device to reinitialize

configuration parameters in non-volatile memory to default

values and restart the application software.  When the device

is running normally, this variable has a value of

running(1)."

::= { probeConfig 5 }

Waldbusser                  Standards Track                    [Page 91]

RFC 2021             Remote Network Monitoring MIB          January 1997

-- The following download objects do not restrict an implementation

-- from implementing additional download mechanisms (controlled in an

-- implementation-specific manner).  Further, in the case where the RMON

-- agent shares a processor with other types of systems, the

-- implementation is not required to download those non-RMON functions

-- with this mechanism.

probeDownloadFile  OBJECT-TYPE

SYNTAX     DisplayString (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The file name to be downloaded from the TFTP server when a

download is next requested via this MIB.  This value is set to

the zero length string when no file name has been specified."

::= { probeConfig 6 }

probeDownloadTFTPServer  OBJECT-TYPE

SYNTAX     IpAddress

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The IP address of the TFTP server that contains the boot

image to load when a download is next requested via this MIB.

This value is set to `0.0.0.0' when no IP address has been

specified."

::= { probeConfig 7 }

probeDownloadAction  OBJECT-TYPE

SYNTAX     INTEGER {

notDownloading(1),

downloadToPROM(2),

downloadToRAM(3)

}

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"When this object is set to downloadToRAM(2) or

downloadToPROM(3), the device will discontinue its

normal operation and begin download of the image specified

by probeDownloadFile from the server specified by

probeDownloadTFTPServer using the TFTP protocol.  If

downloadToRAM(2) is specified, the new image is copied

to RAM only (the old image remains unaltered in the flash

EPROM).  If downloadToPROM(3) is specified

the new image is written to the flash EPROM

memory after its checksum has been verified to be correct.

When the download process is completed, the device will

Waldbusser                  Standards Track                    [Page 92]

RFC 2021             Remote Network Monitoring MIB          January 1997

warm boot to restart the newly loaded application.

When the device is not downloading, this object will have

a value of notDownloading(1)."

::= { probeConfig 8 }

probeDownloadStatus  OBJECT-TYPE

SYNTAX     INTEGER {

downloadSuccess(1),

downloadStatusUnknown(2),

downloadGeneralError(3),

downloadNoResponseFromServer(4),

downloadChecksumError(5),

downloadIncompatibleImage(6),

downloadTftpFileNotFound(7),

downloadTftpAccessViolation(8)

}

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The status of the last download procedure, if any.  This

object will have a value of downloadStatusUnknown(2) if no

download process has been performed."

::= { probeConfig 9 }

serialConfigTable  OBJECT-TYPE

SYNTAX     SEQUENCE OF SerialConfigEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of serial interface configuration entries.  This data

will be stored in non-volatile memory and preserved across

probe resets or power loss."

::= { probeConfig 10 }

serialConfigEntry  OBJECT-TYPE

SYNTAX     SerialConfigEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A set of configuration parameters for a particular

serial interface on this device. If the device has no serial

interfaces, this table is empty.

The index is composed of the ifIndex assigned to this serial

line interface."

INDEX  { ifIndex }

::= { serialConfigTable 1 }

Waldbusser                  Standards Track                    [Page 93]

RFC 2021             Remote Network Monitoring MIB          January 1997

SerialConfigEntry ::= SEQUENCE {

serialMode                   INTEGER,

serialProtocol               INTEGER,

serialTimeout                Integer32 (1..65535),

serialModemInitString        ControlString (SIZE (0..255)),

serialModemHangUpString      ControlString (SIZE (0..255)),

serialModemConnectResp       DisplayString (SIZE (0..255)),

serialModemNoConnectResp     DisplayString (SIZE (0..255)),

serialDialoutTimeout         Integer32 (1..65535),

serialStatus                 RowStatus

}

serialMode  OBJECT-TYPE

SYNTAX     INTEGER {

direct(1),

modem(2)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The type of incoming connection to expect on this serial

interface."

DEFVAL { direct }

::= { serialConfigEntry 1 }

serialProtocol  OBJECT-TYPE

SYNTAX     INTEGER {

other(1),

slip(2),

ppp(3)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The type of data link encapsulation to be used on this

serial interface."

DEFVAL { slip }

::= { serialConfigEntry 2 }

serialTimeout  OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"This timeout value is used when the Management Station has

initiated the conversation over the serial link. This variable

represents the number of seconds of inactivity allowed before

terminating the connection on this serial interface. Use the

Waldbusser                  Standards Track                    [Page 94]

RFC 2021             Remote Network Monitoring MIB          January 1997

serialDialoutTimeout in the case where the probe has initiated

the connection for the purpose of sending a trap."

DEFVAL { 300 }

::= { serialConfigEntry 3 }

serialModemInitString  OBJECT-TYPE

SYNTAX     ControlString (SIZE (0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which controls how a modem attached to this

serial interface should be initialized.  The initialization

is performed once during startup and again after each

connection is terminated if the associated serialMode has the

value of modem(2).

A control string that is appropriate for a wide variety of

modems is: '^s^MATE0Q0V1X4 S0=1 S2=43^M'."

::= { serialConfigEntry 4 }

serialModemHangUpString  OBJECT-TYPE

SYNTAX     ControlString (SIZE (0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which specifies how to disconnect a modem

connection on this serial interface.  This object is only

meaningful if the associated serialMode has the value

of modem(2).

A control string that is appropriate for a wide variety of

modems is: '^d2^s+++^d2^sATH0^M^d2'."

::= { serialConfigEntry 5 }

serialModemConnectResp  OBJECT-TYPE

SYNTAX     DisplayString (SIZE (0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"An ASCII string containing substrings that describe the

expected modem connection response code and associated bps

rate.  The substrings are delimited by the first character

in the string, for example:

/CONNECT/300/CONNECT 1200/1200/CONNECT 2400/2400/

CONNECT 4800/4800/CONNECT 9600/9600

will be interpreted as:

response code    bps rate

CONNECT            300

CONNECT 1200      1200

Waldbusser                  Standards Track                    [Page 95]

RFC 2021             Remote Network Monitoring MIB          January 1997

CONNECT 2400      2400

CONNECT 4800      4800

CONNECT 9600      9600

The agent will use the information in this string to adjust

the bps rate of this serial interface once a modem connection

is established.

A value that is appropriate for a wide variety of modems is:

'/CONNECT/300/CONNECT 1200/1200/CONNECT 2400/2400/

CONNECT 4800/4800/CONNECT 9600/9600/CONNECT 14400/14400/

CONNECT 19200/19200/CONNECT 38400/38400/'."

::= { serialConfigEntry 6 }

serialModemNoConnectResp  OBJECT-TYPE

SYNTAX     DisplayString (SIZE (0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"An ASCII string containing response codes that may be

generated by a modem to report the reason why a connection

attempt has failed.  The response codes are delimited by

the first character in the string, for example:

/NO CARRIER/BUSY/NO DIALTONE/NO ANSWER/ERROR/

If one of these response codes is received via this serial

interface while attempting to make a modem connection,

the agent will issue the hang up command as specified by

serialModemHangUpString.

A value that is appropriate for a wide variety of modems is:

'/NO CARRIER/BUSY/NO DIALTONE/NO ANSWER/ERROR/'."

::= { serialConfigEntry 7 }

serialDialoutTimeout  OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"This timeout value is used when the probe initiates the

serial connection with the intention of contacting a

management station. This variable represents the number

of seconds of inactivity allowed before terminating the

connection on this serial interface."

DEFVAL { 20 }

::= { serialConfigEntry 8 }

serialStatus  OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

Waldbusser                  Standards Track                    [Page 96]

RFC 2021             Remote Network Monitoring MIB          January 1997

STATUS     current

DESCRIPTION

"The status of this serialConfigEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value."

::= { serialConfigEntry 9 }

netConfigTable  OBJECT-TYPE

SYNTAX     SEQUENCE OF NetConfigEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of netConfigEntries."

::= { probeConfig 11 }

netConfigEntry  OBJECT-TYPE

SYNTAX     NetConfigEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A set of configuration parameters for a particular

network interface on this device. If the device has no network

interface, this table is empty.

The index is composed of the ifIndex assigned to the

corresponding interface."

INDEX  { ifIndex }

::= { netConfigTable 1 }

NetConfigEntry ::= SEQUENCE {

netConfigIPAddress         IpAddress,

netConfigSubnetMask        IpAddress,

netConfigStatus            RowStatus

}

netConfigIPAddress  OBJECT-TYPE

SYNTAX     IpAddress

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The IP address of this Net interface.  The default value

for this object is 0.0.0.0.  If either the netConfigIPAddress

or netConfigSubnetMask are 0.0.0.0, then when the device

boots, it may use BOOTP to try to figure out what these

values should be. If BOOTP fails, before the device

can talk on the network, this value must be configured

(e.g., through a terminal attached to the device). If BOOTP is

Waldbusser                  Standards Track                    [Page 97]

RFC 2021             Remote Network Monitoring MIB          January 1997

used, care should be taken to not send BOOTP broadcasts too

frequently and to eventually send very infrequently if no

replies are received."

::= { netConfigEntry 1 }

netConfigSubnetMask  OBJECT-TYPE

SYNTAX     IpAddress

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The subnet mask of this Net interface.  The default value

for this object is 0.0.0.0.  If either the netConfigIPAddress

or netConfigSubnetMask are 0.0.0.0, then when the device

boots, it may use BOOTP to try to figure out what these

values should be. If BOOTP fails, before the device

can talk on the network, this value must be configured

(e.g., through a terminal attached to the device). If BOOTP is

used, care should be taken to not send BOOTP broadcasts too

frequently and to eventually send very infrequently if no

replies are received."

::= { netConfigEntry 2 }

netConfigStatus  OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The status of this netConfigEntry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value."

::= { netConfigEntry 3 }

netDefaultGateway  OBJECT-TYPE

SYNTAX     IpAddress

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The IP Address of the default gateway.  If this value is

undefined or unknown, it shall have the value 0.0.0.0."

::= { probeConfig 12 }

-- Trap Destination Table

--

-- This table defines the destination addresses for traps generated

-- from the device.  This table maps a community to one or more trap

-- destination entries.

--

Waldbusser                  Standards Track                    [Page 98]

RFC 2021             Remote Network Monitoring MIB          January 1997

-- The same trap will be sent to all destinations specified in the

-- entries that have the same trapDestCommunity as the eventCommunity

-- (as defined by RMON MIB).  Information in this table will be stored

-- in non-volatile memory.  If the device has gone through a hard

-- restart, this information will be reset to its default state.

trapDestTable  OBJECT-TYPE

SYNTAX     SEQUENCE OF TrapDestEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of trap destination entries."

::= { probeConfig 13 }

trapDestEntry  OBJECT-TYPE

SYNTAX     TrapDestEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This entry includes a destination IP address to which to send

traps for this community."

INDEX { trapDestIndex }

::= { trapDestTable 1 }

TrapDestEntry ::= SEQUENCE {

trapDestIndex               Integer32,

trapDestCommunity           OCTET STRING,

trapDestProtocol            INTEGER,

trapDestAddress             OCTET STRING,

trapDestOwner               OwnerString,

trapDestStatus              RowStatus

}

trapDestIndex  OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A value that uniquely identifies this trapDestEntry."

::= { trapDestEntry 1 }

trapDestCommunity  OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A community to which this destination address belongs.

This entry is associated with any eventEntries in the RMON

Waldbusser                  Standards Track                    [Page 99]

RFC 2021             Remote Network Monitoring MIB          January 1997

MIB whose value of eventCommunity is equal to the value of

this object.  Every time an associated event entry sends a

trap due to an event, that trap will be sent to each

address in the trapDestTable with a trapDestCommunity equal to

eventCommunity.

This object may not be modified if the associated

trapDestStatus object is equal to active(1)."

::= { trapDestEntry 2 }

trapDestProtocol OBJECT-TYPE

SYNTAX     INTEGER {

ip(1),

ipx(2)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The protocol with which to send this trap."

::= { trapDestEntry 3 }

trapDestAddress  OBJECT-TYPE

SYNTAX     OCTET STRING

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The address to send traps on behalf of this entry.

If the associated trapDestProtocol object is equal to ip(1),

the encoding of this object is the same as the snmpUDPAddress

textual convention in [RFC1906]:

-- for a SnmpUDPAddress of length 6:

--

-- octets   contents        encoding

--  1-4     IP-address      network-byte order

--  5-6     UDP-port        network-byte order

If the associated trapDestProtocol object is equal to ipx(2),

the encoding of this object is the same as the snmpIPXAddress

textual convention in [RFC1906]:

-- for a SnmpIPXAddress of length 12:

--

-- octets   contents            encoding

--  1-4     network-number      network-byte order

--  5-10    physical-address    network-byte order

-- 11-12    socket-number       network-byte order

This object may not be modified if the associated

Waldbusser                  Standards Track                   [Page 100]

RFC 2021             Remote Network Monitoring MIB          January 1997

trapDestStatus object is equal to active(1)."

::= { trapDestEntry 4 }

trapDestOwner  OBJECT-TYPE

SYNTAX     OwnerString

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { trapDestEntry 5 }

trapDestStatus  OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The status of this trap destination entry.

An entry may not exist in the active state unless all

objects in the entry have an appropriate value."

::= { trapDestEntry 6 }

-- Serial Connection Table

--

-- The device may communicate with a management station using

-- SLIP.  In order for the device to send traps via SLIP, it must

-- be able to initiate a connection over the serial interface.  The

-- serialConnectionTable stores the parameters for such connection

-- initiation.

serialConnectionTable  OBJECT-TYPE

SYNTAX     SEQUENCE OF SerialConnectionEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of serialConnectionEntries."

::= { probeConfig 14 }

serialConnectionEntry  OBJECT-TYPE

SYNTAX     SerialConnectionEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Configuration for a SLIP link over a serial line."

INDEX { serialConnectIndex }

::= { serialConnectionTable 1 }

Waldbusser                  Standards Track                   [Page 101]

RFC 2021             Remote Network Monitoring MIB          January 1997

SerialConnectionEntry ::= SEQUENCE {

serialConnectIndex                   Integer32,

serialConnectDestIpAddress           IpAddress,

serialConnectType                    INTEGER,

serialConnectDialString              ControlString,

serialConnectSwitchConnectSeq        ControlString,

serialConnectSwitchDisconnectSeq     ControlString,

serialConnectSwitchResetSeq          ControlString,

serialConnectOwner                   OwnerString,

serialConnectStatus                  RowStatus

}

serialConnectIndex  OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A value that uniquely identifies this serialConnection

entry."

::= { serialConnectionEntry 1 }

serialConnectDestIpAddress  OBJECT-TYPE

SYNTAX     IpAddress

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The IP Address that can be reached at the other end of this

serial connection.

This object may not be modified if the associated

serialConnectStatus object is equal to active(1)."

::= { serialConnectionEntry 2 }

serialConnectType  OBJECT-TYPE

SYNTAX     INTEGER {

direct(1),

modem(2),

switch(3),

modemSwitch(4)

}

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The type of outgoing connection to make.  If this object

has the value direct(1), then a direct serial connection

is assumed.  If this object has the value modem(2),

then serialConnectDialString will be used to make a modem

connection.  If this object has the value switch(3),

Waldbusser                  Standards Track                   [Page 102]

RFC 2021             Remote Network Monitoring MIB          January 1997

then serialConnectSwitchConnectSeq will be used to establish

the connection over a serial data switch, and

serialConnectSwitchDisconnectSeq will be used to terminate

the connection.  If this object has the value

modem-switch(4), then a modem connection will be made first

followed by the switch connection.

This object may not be modified if the associated

serialConnectStatus object is equal to active(1)."

DEFVAL { direct }

::= { serialConnectionEntry 3 }

serialConnectDialString  OBJECT-TYPE

SYNTAX     ControlString (SIZE(0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which specifies how to dial the phone

number in order to establish a modem connection.  The

string should include dialing prefix and suffix.  For

example: ``^s^MATD9,888-1234^M'' will instruct the Probe

to send a carriage return followed by the dialing prefix

``ATD'', the phone number ``9,888-1234'', and a carriage

return as the dialing suffix.

This object may not be modified if the associated

serialConnectStatus object is equal to active(1)."

::= { serialConnectionEntry 4 }

serialConnectSwitchConnectSeq  OBJECT-TYPE

SYNTAX     ControlString (SIZE(0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which specifies how to establish a

data switch connection.

This object may not be modified if the associated

serialConnectStatus object is equal to active(1)."

::= { serialConnectionEntry 5 }

serialConnectSwitchDisconnectSeq  OBJECT-TYPE

SYNTAX     ControlString (SIZE(0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which specifies how to terminate a

data switch connection.

This object may not be modified if the associated

Waldbusser                  Standards Track                   [Page 103]

RFC 2021             Remote Network Monitoring MIB          January 1997

serialConnectStatus object is equal to active(1)."

::= { serialConnectionEntry 6 }

serialConnectSwitchResetSeq  OBJECT-TYPE

SYNTAX     ControlString (SIZE(0..255))

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"A control string which specifies how to reset a data

switch in the event of a timeout.

This object may not be modified if the associated

serialConnectStatus object is equal to active(1)."

::= { serialConnectionEntry 7 }

serialConnectOwner  OBJECT-TYPE

SYNTAX     OwnerString

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The entity that configured this entry and is

therefore using the resources assigned to it."

::= { serialConnectionEntry 8 }

serialConnectStatus  OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"The status of this serialConnectionEntry.

If the manager attempts to set this object to active(1) when

the serialConnectType is set to modem(2) or modem-switch(4)

and the serialConnectDialString is a zero-length string or

cannot be correctly parsed as a ConnectString, the set

request will be rejected with badValue(3).

If the manager attempts to set this object to active(1) when

the serialConnectType is set to switch(3) or modem-switch(4)

and the serialConnectSwitchConnectSeq,

the serialConnectSwitchDisconnectSeq, or

the serialConnectSwitchResetSeq are zero-length strings

or cannot be correctly parsed as ConnectStrings, the set

request will be rejected with badValue(3).

An entry may not exist in the active state unless all

objects in the entry have an appropriate value."

::= { serialConnectionEntry 9 }

Waldbusser                  Standards Track                   [Page 104]

RFC 2021             Remote Network Monitoring MIB          January 1997

--

-- Extensions to the RMON 1 MIB for RMON 2 devices

--

-- These extensions include the standard LastCreateTime Textual

-- Convention for all control tables, as well as an augmentation of

-- the filter entry that provides variable-length offsets into

-- packets.

-- Each of the following, except for filterDroppedFrames, is a

-- read-only object which, if implemented, automatically appears when

-- the RMON1 row it is associated with is created.

etherStats2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF EtherStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { statistics 4 }

etherStats2Entry  OBJECT-TYPE

SYNTAX     EtherStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { etherStatsEntry }

::= { etherStats2Table 1 }

EtherStats2Entry ::= SEQUENCE {

etherStatsDroppedFrames     Counter32,

etherStatsCreateTime        LastCreateTime

}

etherStatsDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

Waldbusser                  Standards Track                   [Page 105]

RFC 2021             Remote Network Monitoring MIB          January 1997

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { etherStats2Entry 1 }

etherStatsCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last

activated. This can be used by the management station to

ensure that the table has not been deleted and recreated

between polls."

::= { etherStats2Entry 2 }

historyControl2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF HistoryControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { history 5 }

historyControl2Entry  OBJECT-TYPE

SYNTAX     HistoryControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { historyControlEntry }

::= { historyControl2Table 1 }

HistoryControl2Entry ::= SEQUENCE {

historyControlDroppedFrames Counter32

}

historyControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

Waldbusser                  Standards Track                   [Page 106]

RFC 2021             Remote Network Monitoring MIB          January 1997

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { historyControl2Entry 1 }

hostControl2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF HostControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { hosts 4 }

hostControl2Entry  OBJECT-TYPE

SYNTAX     HostControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { hostControlEntry }

::= { hostControl2Table 1 }

HostControl2Entry ::= SEQUENCE {

hostControlDroppedFrames    Counter32,

hostControlCreateTime       LastCreateTime

}

hostControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

Waldbusser                  Standards Track                   [Page 107]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { hostControl2Entry 1 }

hostControlCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last

activated. This can be used by the management station to

ensure that the table has not been deleted and recreated

between polls."

::= { hostControl2Entry 2 }

matrixControl2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF MatrixControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { matrix 4 }

matrixControl2Entry  OBJECT-TYPE

SYNTAX     MatrixControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { matrixControlEntry }

::= { matrixControl2Table 1 }

MatrixControl2Entry ::= SEQUENCE {

matrixControlDroppedFrames  Counter32,

matrixControlCreateTime     LastCreateTime

}

matrixControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

Waldbusser                  Standards Track                   [Page 108]

RFC 2021             Remote Network Monitoring MIB          January 1997

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { matrixControl2Entry 1 }

matrixControlCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last

activated. This can be used by the management station to

ensure that the table has not been deleted and recreated

between polls."

::= { matrixControl2Entry 2 }

channel2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF Channel2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { filter 3 }

channel2Entry  OBJECT-TYPE

SYNTAX     Channel2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { channelEntry }

::= { channel2Table 1 }

Channel2Entry ::= SEQUENCE {

channelDroppedFrames    Counter32,

channelCreateTime       LastCreateTime

}

channelDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

Waldbusser                  Standards Track                   [Page 109]

RFC 2021             Remote Network Monitoring MIB          January 1997

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { channel2Entry 1 }

channelCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last

activated. This can be used by the management station to

ensure that the table has not been deleted and recreated

between polls."

::= { channel2Entry 2 }

tokenRingMLStats2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF TokenRingMLStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { statistics 5 }

tokenRingMLStats2Entry  OBJECT-TYPE

SYNTAX     TokenRingMLStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { tokenRingMLStatsEntry }

::= { tokenRingMLStats2Table 1 }

TokenRingMLStats2Entry ::= SEQUENCE {

tokenRingMLStatsDroppedFrames       Counter32,

tokenRingMLStatsCreateTime          LastCreateTime

}

tokenRingMLStatsDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

Waldbusser                  Standards Track                   [Page 110]

RFC 2021             Remote Network Monitoring MIB          January 1997

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { tokenRingMLStats2Entry 1 }

tokenRingMLStatsCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last activated.

This can be used by the management station to ensure that the

table has not been deleted and recreated between polls."

::= { tokenRingMLStats2Entry 2 }

tokenRingPStats2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF TokenRingPStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { statistics 6 }

tokenRingPStats2Entry  OBJECT-TYPE

SYNTAX     TokenRingPStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS {  tokenRingPStatsEntry }

::= { tokenRingPStats2Table 1 }

TokenRingPStats2Entry ::= SEQUENCE {

tokenRingPStatsDroppedFrames    Counter32,

tokenRingPStatsCreateTime       LastCreateTime

}

tokenRingPStatsDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

Waldbusser                  Standards Track                   [Page 111]

RFC 2021             Remote Network Monitoring MIB          January 1997

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { tokenRingPStats2Entry 1 }

tokenRingPStatsCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last activated.

This can be used by the management station to ensure that the

table has not been deleted and recreated between polls."

::= { tokenRingPStats2Entry 2 }

ringStationControl2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF RingStationControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { tokenRing 7 }

ringStationControl2Entry  OBJECT-TYPE

SYNTAX     RingStationControl2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { ringStationControlEntry }

::= { ringStationControl2Table 1 }

RingStationControl2Entry ::= SEQUENCE {

ringStationControlDroppedFrames Counter32,

ringStationControlCreateTime    LastCreateTime

}

Waldbusser                  Standards Track                   [Page 112]

RFC 2021             Remote Network Monitoring MIB          January 1997

ringStationControlDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { ringStationControl2Entry 1 }

ringStationControlCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last activated.

This can be used by the management station to ensure that the

table has not been deleted and recreated between polls."

::= { ringStationControl2Entry 2 }

sourceRoutingStats2Table  OBJECT-TYPE

SYNTAX     SEQUENCE OF SourceRoutingStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

::= { tokenRing 8 }

sourceRoutingStats2Entry  OBJECT-TYPE

SYNTAX     SourceRoutingStats2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Contains the RMON-2 augmentations to RMON-1."

AUGMENTS { sourceRoutingStatsEntry }

::= { sourceRoutingStats2Table 1 }

SourceRoutingStats2Entry ::= SEQUENCE {

sourceRoutingStatsDroppedFrames Counter32,

Waldbusser                  Standards Track                   [Page 113]

RFC 2021             Remote Network Monitoring MIB          January 1997

sourceRoutingStatsCreateTime    LastCreateTime

}

sourceRoutingStatsDroppedFrames OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The total number of frames which were received by the probe

and therefore not accounted for in the *StatsDropEvents, but

for which the probe chose not to count for this entry for

whatever reason.  Most often, this event occurs when the probe

is out of some resources and decides to shed load from this

collection.

This count does not include packets that were not counted

because they had MAC-layer errors.

Note that, unlike the dropEvents counter, this number is the

exact number of frames dropped."

::= { sourceRoutingStats2Entry 1 }

sourceRoutingStatsCreateTime OBJECT-TYPE

SYNTAX     LastCreateTime

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime when this control entry was last activated.

This can be used by the management station to ensure that the

table has not been deleted and recreated between polls."

::= { sourceRoutingStats2Entry 2 }

filter2Table OBJECT-TYPE

SYNTAX     SEQUENCE OF Filter2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Provides a variable-length packet filter feature to the

RMON-1 filter table."

::= { filter 4 }

filter2Entry OBJECT-TYPE

SYNTAX     Filter2Entry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Provides a variable-length packet filter feature to the

RMON-1 filter table."

Waldbusser                  Standards Track                   [Page 114]

RFC 2021             Remote Network Monitoring MIB          January 1997

AUGMENTS { filterEntry }

::= { filter2Table 1 }

Filter2Entry ::= SEQUENCE {

filterProtocolDirDataLocalIndex     Integer32,

filterProtocolDirLocalIndex         Integer32

}

filterProtocolDirDataLocalIndex OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"When this object is set to a non-zero value, the filter that

it is associated with performs the following operations on

every packet:

1) - If the packet doesn't match the protocol directory entry

identified by this object, discard the packet and exit

(i.e., discard the packet if it is not of the identified

protocol).

2) - If the associated filterProtocolDirLocalIndex is non-zero

and the packet doesn't match the protocol directory

entry identified by that object, discard the packet and

exit

3) - If the packet matches, perform the regular filter

algorithm as if the beginning of this named protocol is

the beginning of the packet, potentially applying the

filterOffset value to move further into the packet."

DEFVAL { 0 }

::= { filter2Entry 1 }

filterProtocolDirLocalIndex OBJECT-TYPE

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-create

STATUS     current

DESCRIPTION

"When this object is set to a non-zero value, the filter that

it is associated with will discard the packet if the packet

doesn't match this protocol directory entry."

DEFVAL { 0 }

::= { filter2Entry 2 }

-- Conformance Macros

rmon2MIBCompliances OBJECT IDENTIFIER ::= { rmonConformance 1 }

rmon2MIBGroups      OBJECT IDENTIFIER ::= { rmonConformance 2 }

Waldbusser                  Standards Track                   [Page 115]

RFC 2021             Remote Network Monitoring MIB          January 1997

rmon2MIBCompliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"Describes the requirements for conformance to

the RMON2 MIB"

MODULE  -- this module

MANDATORY-GROUPS { protocolDirectoryGroup,

protocolDistributionGroup,

addressMapGroup,

nlHostGroup,

nlMatrixGroup,

usrHistoryGroup,

probeInformationGroup }

GROUP   rmon1EnhancementGroup

DESCRIPTION

"The rmon1EnhancementGroup is mandatory for systems which

implement RMON [RFC1757]"

::= { rmon2MIBCompliances 1 }

rmon2MIBApplicationLayerCompliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"Describes the requirements for conformance to

the RMON2 MIB with Application Layer Enhancements."

MODULE  -- this module

MANDATORY-GROUPS { protocolDirectoryGroup,

protocolDistributionGroup,

addressMapGroup,

nlHostGroup,

nlMatrixGroup,

alHostGroup,

alMatrixGroup,

usrHistoryGroup,

probeInformationGroup }

GROUP   rmon1EnhancementGroup

DESCRIPTION

"The rmon1EnhancementGroup is mandatory for systems which

implement RMON [RFC1757]"

::= { rmon2MIBCompliances 2 }

protocolDirectoryGroup OBJECT-GROUP

OBJECTS { protocolDirLastChange,

protocolDirLocalIndex, protocolDirDescr,

protocolDirType, protocolDirAddressMapConfig,

protocolDirHostConfig, protocolDirMatrixConfig,

Waldbusser                  Standards Track                   [Page 116]

RFC 2021             Remote Network Monitoring MIB          January 1997

protocolDirOwner, protocolDirStatus }

STATUS  current

DESCRIPTION

"Lists the inventory of protocols the probe has the capability

of monitoring and allows the addition, deletion, and

configuration of entries in this list."

::= { rmon2MIBGroups 1 }

protocolDistributionGroup OBJECT-GROUP

OBJECTS { protocolDistControlDataSource,

protocolDistControlDroppedFrames,

protocolDistControlCreateTime,

protocolDistControlOwner, protocolDistControlStatus,

protocolDistStatsPkts, protocolDistStatsOctets }

STATUS  current

DESCRIPTION

"Collects the relative amounts of octets and packets for the

different protocols detected on a network segment."

::= { rmon2MIBGroups 2 }

addressMapGroup OBJECT-GROUP

OBJECTS { addressMapInserts, addressMapDeletes,

addressMapMaxDesiredEntries,

addressMapControlDataSource,

addressMapControlDroppedFrames,

addressMapControlOwner, addressMapControlStatus,

addressMapPhysicalAddress,

addressMapLastChange }

STATUS  current

DESCRIPTION

"Lists MAC address to network address bindings discovered by

the probe and what interface they were last seen on."

::= { rmon2MIBGroups 3 }

nlHostGroup OBJECT-GROUP

OBJECTS { hlHostControlDataSource,

hlHostControlNlDroppedFrames, hlHostControlNlInserts,

hlHostControlNlDeletes,

hlHostControlNlMaxDesiredEntries,

hlHostControlAlDroppedFrames, hlHostControlAlInserts,

hlHostControlAlDeletes,

hlHostControlAlMaxDesiredEntries, hlHostControlOwner,

hlHostControlStatus, nlHostInPkts, nlHostOutPkts,

nlHostInOctets, nlHostOutOctets,

nlHostOutMacNonUnicastPkts, nlHostCreateTime }

STATUS  current

DESCRIPTION

"Counts the amount of traffic sent from and to each network

Waldbusser                  Standards Track                   [Page 117]

RFC 2021             Remote Network Monitoring MIB          January 1997

address discovered by the probe. Note that while the

hlHostControlTable also has objects that control an optional

alHostTable, implementation of the alHostTable is not required

to fully implement this group."

::= { rmon2MIBGroups 4 }

nlMatrixGroup OBJECT-GROUP

OBJECTS { hlMatrixControlDataSource,

hlMatrixControlNlDroppedFrames,

hlMatrixControlNlInserts, hlMatrixControlNlDeletes,

hlMatrixControlNlMaxDesiredEntries,

hlMatrixControlAlDroppedFrames,

hlMatrixControlAlInserts, hlMatrixControlAlDeletes,

hlMatrixControlAlMaxDesiredEntries,

hlMatrixControlOwner, hlMatrixControlStatus,

nlMatrixSDPkts, nlMatrixSDOctets, nlMatrixSDCreateTime,

nlMatrixDSPkts, nlMatrixDSOctets, nlMatrixDSCreateTime,

nlMatrixTopNControlMatrixIndex,

nlMatrixTopNControlRateBase,

nlMatrixTopNControlTimeRemaining,

nlMatrixTopNControlGeneratedReports,

nlMatrixTopNControlDuration,

nlMatrixTopNControlRequestedSize,

nlMatrixTopNControlGrantedSize,

nlMatrixTopNControlStartTime,

nlMatrixTopNControlOwner, nlMatrixTopNControlStatus,

nlMatrixTopNProtocolDirLocalIndex,

nlMatrixTopNSourceAddress, nlMatrixTopNDestAddress,

nlMatrixTopNPktRate, nlMatrixTopNReversePktRate,

nlMatrixTopNOctetRate, nlMatrixTopNReverseOctetRate }

STATUS  current

DESCRIPTION

"Counts the amount of traffic sent between each pair of

network addresses discovered by the probe. Note that while the

hlMatrixControlTable also has objects that control optional

alMatrixTables, implementation of the alMatrixTables is not

required to fully implement this group."

::= { rmon2MIBGroups 5 }

alHostGroup OBJECT-GROUP

OBJECTS { alHostInPkts, alHostOutPkts,

alHostInOctets, alHostOutOctets, alHostCreateTime }

STATUS  current

DESCRIPTION

"Counts the amount of traffic, by protocol, sent from and to

each network address discovered by the probe. Implementation

of this group requires implementation of the Network Layer

Host Group."

Waldbusser                  Standards Track                   [Page 118]

RFC 2021             Remote Network Monitoring MIB          January 1997

::= { rmon2MIBGroups 6 }

alMatrixGroup OBJECT-GROUP

OBJECTS { alMatrixSDPkts, alMatrixSDOctets, alMatrixSDCreateTime,

alMatrixDSPkts, alMatrixDSOctets, alMatrixDSCreateTime,

alMatrixTopNControlMatrixIndex,

alMatrixTopNControlRateBase,

alMatrixTopNControlTimeRemaining,

alMatrixTopNControlGeneratedReports,

alMatrixTopNControlDuration,

alMatrixTopNControlRequestedSize,

alMatrixTopNControlGrantedSize,

alMatrixTopNControlStartTime,

alMatrixTopNControlOwner, alMatrixTopNControlStatus,

alMatrixTopNProtocolDirLocalIndex,

alMatrixTopNSourceAddress, alMatrixTopNDestAddress,

alMatrixTopNAppProtocolDirLocalIndex,

alMatrixTopNPktRate, alMatrixTopNReversePktRate,

alMatrixTopNOctetRate, alMatrixTopNReverseOctetRate }

STATUS  current

DESCRIPTION

"Counts the amount of traffic, by protocol, sent between each

pair of network addresses discovered by the

probe. Implementation of this group requires implementation of

the Network Layer Matrix Group."

::= { rmon2MIBGroups 7 }

usrHistoryGroup OBJECT-GROUP

OBJECTS { usrHistoryControlObjects,

usrHistoryControlBucketsRequested,

usrHistoryControlBucketsGranted,

usrHistoryControlInterval,

usrHistoryControlOwner, usrHistoryControlStatus,

usrHistoryObjectVariable, usrHistoryObjectSampleType,

usrHistoryIntervalStart, usrHistoryIntervalEnd,

usrHistoryAbsValue, usrHistoryValStatus }

STATUS  current

DESCRIPTION

"The usrHistoryGroup provides user-defined collection of

historical information from MIB objects on the probe."

::= { rmon2MIBGroups 8 }

probeInformationGroup OBJECT-GROUP

OBJECTS { probeCapabilities,

probeSoftwareRev, probeHardwareRev, probeDateTime }

STATUS  current

DESCRIPTION

"This group describes various operating parameters of the

Waldbusser                  Standards Track                   [Page 119]

RFC 2021             Remote Network Monitoring MIB          January 1997

probe as well as controlling the local time of the probe."

::= { rmon2MIBGroups 9 }

probeConfigurationGroup OBJECT-GROUP

OBJECTS { probeResetControl, probeDownloadFile,

probeDownloadTFTPServer, probeDownloadAction,

probeDownloadStatus,

serialMode, serialProtocol, serialTimeout,

serialModemInitString, serialModemHangUpString,

serialModemConnectResp, serialModemNoConnectResp,

serialDialoutTimeout, serialStatus,

netConfigIPAddress, netConfigSubnetMask,

netConfigStatus, netDefaultGateway,

trapDestCommunity, trapDestProtocol, trapDestAddress,

trapDestOwner, trapDestStatus,

serialConnectDestIpAddress, serialConnectType,

serialConnectDialString, serialConnectSwitchConnectSeq,

serialConnectSwitchDisconnectSeq,

serialConnectSwitchResetSeq,

serialConnectOwner, serialConnectStatus }

STATUS  current

DESCRIPTION

"This group controls the configuration of various operating

parameters of the probe."

::= { rmon2MIBGroups 10 }

rmon1EnhancementGroup OBJECT-GROUP

OBJECTS { historyControlDroppedFrames, hostControlDroppedFrames,

hostControlCreateTime, matrixControlDroppedFrames,

matrixControlCreateTime, channelDroppedFrames,

channelCreateTime, filterProtocolDirDataLocalIndex,

filterProtocolDirLocalIndex }

STATUS  current

DESCRIPTION

"This group adds some enhancements to RMON-1 that help

management stations."

::= { rmon2MIBGroups 11 }

rmon1EthernetEnhancementGroup OBJECT-GROUP

OBJECTS { etherStatsDroppedFrames, etherStatsCreateTime }

STATUS  current

DESCRIPTION

"This group adds some enhancements to RMON-1 that help

management stations."

::= { rmon2MIBGroups 12 }

rmon1TokenRingEnhancementGroup OBJECT-GROUP

OBJECTS { tokenRingMLStatsDroppedFrames,

Waldbusser                  Standards Track                   [Page 120]

RFC 2021             Remote Network Monitoring MIB          January 1997

tokenRingMLStatsCreateTime,

tokenRingPStatsDroppedFrames, tokenRingPStatsCreateTime,

ringStationControlDroppedFrames,

ringStationControlCreateTime,

sourceRoutingStatsDroppedFrames,

sourceRoutingStatsCreateTime }

STATUS  current

DESCRIPTION

"This group adds some enhancements to RMON-1 that help

management stations."

::= { rmon2MIBGroups 13 }

END

Waldbusser                  Standards Track                   [Page 121]

RFC 2021             Remote Network Monitoring MIB          January 1997

7.  Security Considerations

In order to implement this MIB, a probe must capture all packets on

the locally-attached network, including packets between third

parties.  These packets are analyzed to collect network addresses,

protocol usage information, and conversation statistics. Data of this

nature may be considered sensitive in some environments. In such

environments the administrator may wish to restrict SNMP access to

the probe.

A probe implementing this MIB is likely to also implement RMON

[RFC1757], which includes functions for returning the contents of

captured packets, potentially including sensitive user data or

passwords. It is recommended that SNMP access to these functions be

restricted.

Waldbusser                  Standards Track                   [Page 122]

RFC 2021             Remote Network Monitoring MIB          January 1997

8.  Appendix - TimeFilter Implementation Notes

1) Theory of Operation

The TimeFilter mechanism allows an NMS to reduce the number of SNMP

transactions required for a 'table-update' operation.  Polling of

tables that incorporate a 'TimeFilter' INDEX can be reduced to a

theoretical minimum (if used correctly). It can be easily implemented

by an agent in a way independent of the number of NMS applications

using the same time-filtered table.

Although the name 'TimeFilter' may imply that a history of change

events is maintained by the agent, this is not the case.  A time-

filtered-value represents the current value of the object instance,

not the 'saved' value at the time indicated by the TimeFilter INDEX

value. Note that TimeFilter objects only appear in INDEX clauses

(always not-accessible), so their value is never retrieved. By

design, the actual value of a TimeFilter instance is not in itself

meaningful (it's not a 'last-change-timestamp').

The TimeFilter is a boolean filtering function applied in internal

Get* PDU processing. If the 'last-change-time' of the specified

instance is less than the particular TimeFilter INDEX value, then the

instance is considered 'not-present' (skipped for GetNext and GetBulk

PDUs; 'noSuchInstance' or returned to the requester.

1.1) Agent Implementation of a Time-Filtered Table

In implementation, the time-filtered rows (one for each tick of

sysUpTime) are only conceptual. The agent simply filters a real table

based on:

* the current value of sysUpTime

* the TimeFilter value passed in the varbind

* the last-update timestamp of each requested counter

(agent implementation requirement)

For example, to implement a time-filtered counter, an agent maintains

a timestamp in a 32-bit storage location, initialized to zero. This

is in addition to whatever instrumentation is needed for the counter.

Each time the counter is updated, the current value of sysUpTime is

recorded in the associated timestamp. If this is not possible or

practical, then a background polling process must 'refresh' the

timestamp by sampling counter values and comparing them to recorded

samples. The timestamp update must occur within 5 seconds of the

actual change event.

Waldbusser                  Standards Track                   [Page 123]

RFC 2021             Remote Network Monitoring MIB          January 1997

When an agent receives a Get, GetNext, or GetBulk PDU requesting a

time-filtered instance, the following agent has determined that the

instance is within the MIB view indicated by the community string in

the PDU.

/* return TRUE if the object is present */

boolean time_filter_test (

TimeFilter  last_modified_timestamp,

TimeFilter  index_value_in_pdu )

{

if (last_modified_timestamp < index_value_in_pdu)

return FALSE;

else

return TRUE;

}

The agent applies this function regardless of the lastActivationTime

of the conceptual row in question. In other words, counter

discontinuities are ignored (i.e.  conceptual row deleted and then

re-created later). An agent should consider a object instance

'changed' when it is created (either at restart time for scalars and

static objects, or row-creation-time for dynamic tables).

Note that using a timeFilter INDEX value of zero removes the

filtering functionality, as the instance will always be

1.2) NMS Implementation of a Time-Filtered Table

The particular TimeFilter INDEX values used by an NMS reflect the

polling interval of the NMS, relative to the particular agent's

notion of sysUpTime.

An NMS needs to maintain one timestamp variable per agent

(initialized to zero) for an arbitrary group of time-filtered MIB

objects that are gathered together in the same PDU.  Each time the

Get* PDU is sent, a request for sysUpTime is included. The retrieved

sysUpTime value is used as the timeFilter value in the next polling

cycle. If a polling sweep of a time-filtered group of objects

requires more than one SNMP transaction, then the sysUpTime value

retrieved in the first GetResponse PDU of the polling sweep is saved

as the next timeFilter value.

The actual last-update time of a given object is not indicated in the

returned GetResponse instance identifier, but rather the timeFilter

value passed in the Get*Request PDU is returned.

Waldbusser                  Standards Track                   [Page 124]

RFC 2021             Remote Network Monitoring MIB          January 1997

A "time-filtered get-next/bulk-sweep", done once per polling

cycle, is a series of GetNext or GetBulk transactions, and is

over when one of the following events occurs:

1) the TimeFilter index value returned in the GetResponse is

different than the TimeFilter index value passed in the

GetNext or GetBulk request. Counter values will still be

returned beyond this point (until the last-change-time is

reached), but most likely the same values will be

returned.

2) the return PDU includes instances lexigraphically greater

than the objects expected (i.e. same GetNext semantics as

if the TimeFilter wasn't there)

3) a noSuchName or other exception/error is returned.

Note that the use of a time-filtered table in combination with a

GetRequest PDU neutralizes any optimization that otherwise might be

achieved with the TimeFilter, because no PDU transactions are saved.

Either the current time-filtered object-value is returned, or a

'noSuchInstance' exception (SNMPv1c) or 'noSuchName' error (SNMPv1)

is returned.

If GetBulk PDUs are used, then the value selected for response PDUs

generated by the agent, since duplicate entries (one per size. An

appropriate of conceptual rows in the time-filtered table if known, or

equal to the number of instances expected to fit in a GetResponse PDU

without causing a 'tooBig' error from the agent.

2) TimeFilter Example

The following example demonstrates how an NMS and Agent might use a

table with a TimeFilter object in the INDEX. A static table is

assumed to keep the example simple, but dynamic tables can also be

supported.

2.1) General Assumptions

fooEntry INDEX { fooTimeMark, fooIfIndex }

FooEntry = SEQUENCE {

fooTimeMark    TimeFilter,

fooIfIndex     Integer32,

fooCounts      Counter32

}

The NMS polls the fooTable every 15 seconds and the

baseline

poll occurs when the agent has been up for 6 seconds,

and the NMS has been up for 10 seconds.

Waldbusser                  Standards Track                   [Page 125]

RFC 2021             Remote Network Monitoring MIB          January 1997

There are 2 static rows in this table at system

initialization

(fooCounts.0.1 and fooCounts.0.2).

Row 1 was updated as follows:

SysUpTime    fooCounts.*.1 value         500

1         900            2

2300            3

Row 2 was updated as follows:

SysUpTime    fooCounts.*.2 value

1100            1

1400            2

2.2) SNMP Transactions from NMS Perspective

Time nms-1000:

# NMS baseline poll -- get everything since last agent

restart

# TimeFilter == 0

get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

fooCounts.0);

returns:

sysUpTime.0 == 600

fooCounts.0.1 == 1  # incremented at time 500

fooCounts.0.2 == 0  # visible since created at time 0

Time nms-2500:

# NMS 1st poll

# TimeFilter index == 600

get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

fooCounts.600);

returns:

sysUpTime.0 == 2100

fooCounts.600.1 == 2   # incremented at time 900

fooCounts.600.2 == 2   # incremented at times 1100 and

1400

fooCounts.601.1 == 2   # indicates end of sweep

Waldbusser                  Standards Track                   [Page 126]

RFC 2021             Remote Network Monitoring MIB          January 1997

Time nms-4000:

# NMS 2nd poll

# TimeFilter == 2100

get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

fooCounts.2100);

returns:

sysUpTime.0 == 3600

fooCounts.2100.1 == 3  # incremented at time 2300

fooCounts.2102.1 == 3  # indicates end-of-sweep

# the counter value for row 2 is not returned because

# it hasn't changed since sysUpTime == 2100.

# The next timetick value for row 1 is returned instead

Time nms-5500:

# NMS 3rd poll

# TimeFilter == 3600

get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

fooCounts.3600);

returns:

sysUpTime.0 == 5100

some-instance-outside-the-fooTable == <don't care>

some-instance-outside-the-fooTable == <don't care>

# no 'fooTable' counter values at all are returned

because

# neither counter has been updated since sysUpTime ==

3600

2.3) Transactions and TimeFilter Maintenance: Agent

Perspective

Time agt-0:

# initialize fooTable

fooCounts.1 = 0; changed.1 = 0;

fooCounts.2 = 0; changed.2 = 0;

Time agt-500:

# increment fooCounts.1

++fooCounts.1; changed.1 = 500;

Waldbusser                  Standards Track                   [Page 127]

RFC 2021             Remote Network Monitoring MIB          January 1997

Time agt-600

# answer get-bulk

#   get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

#       fooCounts.0);

# (changed >= 0)

# return both counters

Time agt-900:

# increment fooCounts.1

++fooCounts.1; changed.1 = 900;

Time agt-1100:

# increment fooCounts.2

++fooCounts.2; changed.2 = 1100;

Time agt-1400:

# increment fooCounts.2

++fooCounts.2; changed.2 = 1400;

Time agt-2100

# answer get-bulk

# get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

#     fooCounts.600);

# (changed >= 600)

# return both counters

Time agt-2300:

# increment fooCounts.1

++fooCounts.1; changed.1 = 2300;

Time agt-3600:

# answer get-bulk

# get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

#     fooCounts.2100);

# (changed >= 2100)

# return only fooCounts.1 from the fooTable--twice

Time agt-5100:

# answer get-bulk

# get-bulk(nonRptrs=1, maxReps=2, sysUpTime.0,

#      fooCounts.3600);

# (changed >= 3600)

# return lexigraphically-next two MIB instances

Waldbusser                  Standards Track                   [Page 128]

RFC 2021             Remote Network Monitoring MIB          January 1997

9.  Acknowledgments

This document was produced by the IETF Remote Network Monitoring

Working Group.

10.  References

[1]  SNMPv2 Working Group, J. Case, K. McCloghrie, M. Rose, S.

Waldbusser, "Structure and Identification of Management

Information for Version 2 of the Simple Network

Management Protocol (SNMPv2)" RFC 1902, January 1996.

[2]  SNMPv2 Working Group, J. Case, K. McCloghrie, M. Rose, S.

Waldbusser, "Textual Conventions for Version 2 of the

Simple Network Management Protocol (SNMPv2)", RFC 1903

January 1996.

[3]  McCloghrie, K., and M. Rose, "Management Information Base

for Network Management of TCP/IP-based internets: MIB-II",

STD 17, RFC 1213, March 1991.

[4]  SNMPv2 Working Group, J. Case, K. McCloghrie, M. Rose, S.

Waldbusser, "Protocol Operations for version 2 of the

Simple Network Management Protocol (SNMPv2)",

RFC 1905, January 1996.

[5]  McCloghrie, K., and F. Kastenholz, "Evolution of the

Interfaces Group of MIB-II", RFC 1573, January 1994.

[6]  Information processing systems -- Open Systems

Interconnection -- Specification of Abstract Syntax

Notation One (ASN.1), International Organization for

Standardization.  International Standard 8824, (December,

1987).

[7]  Information processing systems -- Open Systems

Interconnection -- Specification of Basic Encoding Rules

for Abstract Notation One (ASN.1), International

Organization for Standardization.  International Standard

8825, (December, 1987).

[8]  Rose, M., Editor, "A Convention for Defining Traps for

use with the SNMP", RFC 1215, March 1991.

[9]  Waldbusser, S., "Remote Network Monitoring Management

Information Base", RFC 1757, February 1995.

Waldbusser                  Standards Track                   [Page 129]

RFC 2021             Remote Network Monitoring MIB          January 1997

[10] Waldbusser, S., "Token Ring Extensions to the Remote Network

Monitoring MIB", RFC 1513, September 1993.

11.  Author's Address

Steven Waldbusser

International Network Services

Phone: (415) 254-4251

EMail: waldbusser@ins.com

Waldbusser                  Standards Track                   [Page 130]