When you select Control Charts from the Graph menu, you see the Control Chart Launch dialog in Figure 25.1. Initially, the dialog shows three kinds of information: - process information - chart type information, - limits specification. Specific information shown for each section varies according to the kind of chart you request. Through interaction with the Launch dialog you specify exactly how you want charts created. The following sections describe the kinds of dialog information. Figure 25.1 Control Chart Launch Dialog Through interaction with the Launch dialog, you specify exactly how you want your charts created. The following sections describe the panels element. Process Information The Process panel displays a list of all columns in the current data table. Here you specify the variables to be analyzed and the subgroup sample size. Process selects variables for charting. - For variables charts, specify measurements as the process. - For attributes charts, specify the defect count or defective proportion as the process. Sample ID enables you to specify a variable whose values label the horizontal axis, and can also identify unequal subgroup sizes. If no sample ID variable is specified, the samples are identified by their sequence number. - If the subsamples are the same size, check the Sample Size Constant radio button and enter the size into the text box. If you entered a Sample ID variable, its values are used to label the horizontal axis. - If the subsamples have an unequal number or rows, or have missing values, the same size, check the Sample Grouped by Sample Label radio button and remove sample size information from the constant sample size text box. For attributes charts this variable is the subgroup sample size. In variables charts it identifies the sample. When the chart type is Individual, the Constant box becomes the Range Span box. The range span specifies the number of consecutive measurements from which the moving ranges are computed. For c and u chart types, the sample size shows as # of Inspection Units. The illustration in Figure 25.2 shows a Mean variables chart for a process with unequal subgroup sample sizes, using the Coating.jmp sample data from the QC folder. Figure 25.2 Variables Charts with Unequal Subgroup Sample Sizes Chart Type Information The Chart Type panel displays the popup menu shown below for chart-type specification. Options in the Chart Type panel vary according to chart type. Shewhart controls charts broadly classify as variables charts and attributes charts. Moving average charts and cusum charts can be thought of as special kinds of variables chart. - Shewhart variables charts are the Mean, R, S, and the IR menu selections. The Mean, R, S selection has check-box options for each chart. The Individual selection has check-box options for the individual measurement and moving range charts. - The uniformly weighted moving average (UWMA) and exponentially weighted moving average (EWMA) selections are special charts for means. - The Cusum chart is a special chart for means or individual measurements. - Shewhart attributes chart selections are the P, NP, C, and U charts. There are no additional specifications for attributes charts. The types of control charts are discussed in the sections Control Charts for Variables and Control Charts for Attributes. Limits Specification You specify computations for control limits by entering a value for k (K Sigma) or by entering a probability a (Alpha), or by retrieving a limits value from a previously created Limits Table. Limits Tables are discussed in the section Saving and Retrieving Limits later in this chapter. There must be a specification of either K Sigma or Alpha. The dialog default is K Sigma = 3. K Sigma allows specification of control limits in terms of a multiple of the sample standard error. K Sigma specifies control limits at k sample standard errors above and below the expected value, which shows as the center line. To specify k, the number of sigmas, click K Sigma and enter a positive k value into the box. The usual choice for k is three. The examples shown in Figure 25.3 compare the XBar chart for the Coating.jmp data with control lines drawn at K Sigma = 3 and K Sigma =4. Figure 25.3 K Sigma =3 (left) and K Sigma=4 (right) Control Limits Alpha specifies control limits (also called probability limits) in terms of the probability a that a single subgroup statistic exceeds its control limits, assuming that the process is in control. To specify alpha, click the Alpha radio button and enter the probability you want. Reasonable choices for a are .01 or .001. Using Known Statistics If you click the Specify Stats button on the Control Charts Launch dialog, a tab with editable fields is appended to the bottom of the launch dialog. This lets you enter known statistics for the process variable. The Quality Control platform uses those entries to construct control charts. The example to the right shows one (1) as the standard deviation of the process variable, and 20 as the mean measurement.

Previous Table of Contents Up Next

Previous Next

Tests for Special Causes The Tests option in the chart type popup menu displays the submenu shown here. You can select one or more tests for special causes (often called the Western Electric rules) in the Control Chart launch dialog or with the options popup menu. Nelson (1984) developed the numbering notation used to identify special tests on control charts. If a selected test is positive, the last point in the test pattern is labeled with the test number. When you select several tests for display and more than one test signals at a particular point, the label of the first test you specified appears beside the point. Table 25.1 lists and interprets the eight tests, and Figure 25.25 illustrates the tests. The following rules apply to each test: - The area between the upper and lower limits is divided into six zones, each with a width of one standard deviation. - The zones are labeled A, B, C, C, B, A with Zones C nearest the center line. - A point lies in Zone B or beyond if it lies beyond the line separating zones C and B. - Any point lying on a line separating two zones lies in or beyond the innermost zone. Tests 1 through 8 apply to mean and individual measurement charts. Tests 1 through 4 can also apply to p, np, c, and u charts. See Nelson (1984, 1985) for further recommendations on how to use these tests. Table 25.1 Description and Interpretation of Special Causes Tests Test 1 One point beyond Zone A detects a shift in the mean, an increase in the standard deviation, or a single aberration in the process. For interpreting Test 1, the R chart can be used to rule out increases in variation. Test 2 Nine points in a row in Zone C or beyond detects a shift in the process mean. Test 3 Six points in a row steadily increasing or decreasing detects a trend or drift in the process mean. Small trends will be signaled by this test before Test 1. Test 4 Fourteen points in a row alternating up and down detects systematic effects such as two alternately used machines, vendors, or operators. Test 5 Two out of three points in a row in Zone A or beyond detects a shift in the process average or increase in the standard deviation. Any two out of three points provide a positive test. Test 6 Four out of five points in Zone B or beyond detects a shift in the process mean. Any four out of five points provide a positive test. Test 7 Fifteen points in a row in Zone C, above and below the center line detects stratification of subgroups when the observations in a single subgroup come from various sources with different means. Test 8 Eight points in a row on both sides of the center line with none in Zones C detects stratification of subgroups when the observations in one subgroup come from a single source, but subgroups come from different sources with different means. Nelson (1984, 1985) Tests 1, 2, 5, and 6 apply to the upper and lower halves of the chart separately. Tests 3, 4, 7, and 8 apply to the whole chart. Figure 25.25 Illustration of Special Causes Tests, Nelson (1984, 1985)

Previous Table of Contents Up Next