Level and turnsã

Angle of Attack (AOA)

Controls and What They Do

Level Dynamics

Leveling Off from Climb

Level Cruise #1

Level Cruise #2

Level Cruise #3

Unable to fly level

Level Turn Dynamics

Bank recovery to a heading

Level Turns

Level turn exercises (Instructor)

Climbing Left turns

Climbing Right turns

Procedures:

Steep Turns

# Angle of Attack (AOA)

Your learning progress is directly related to your understanding of how the controls affect flight.  Accidents are the ultimate solution of a lack of understanding.  A pilot must understand the function of the rudder and angle of attack related to flight.  By definition, the angle of attack is the angle made by the chordline of the wing and the aircraft flight path.   At a certain critical angle of attack a wing or a part of it will stall regardless of speed or load factor. Stall warners are used because AOA indicators are difficult to install on small aircraft and even when installed the AOA at stall varies however slightly.

A wing can produce lift by increasing the AOA until reaching the stall AOA.  AOA is controlled by used of the elevators.  Any increase in speed will increase the lift.  In straight a level flight lift is equal to aircraft weight.  The fact is, airspeed does not cause a stall, AOA causes a stall

regardless of speed or load factor.  Load factor can be increased by turns,

abrupt control movements and dive recoveries;.  In these instances the aircraft may stall at a higher speed because of the load factor but the AOA

is always the same.

There is a relatively wide range of level flight speeds.  A pilot can by varying power and AOA, one against the other, transition through all the level flight speeds.  A fixed power setting and AOA allows a pilot to trim for a hands-off airspeed.  Changes of power only in a hands-off flight situation will cause the aircraft to change the nose, up or down, and, with dampening by hand, the aircraft will climb or descend at very near the original speed.

There is controversy between the aviation guru Wolfgang Langewiesche and the FAA as to what flight controls do.  Regardless, to place the aircraft into a given position the same essential movements are required.  Elevators do control the angle of attack and in so doing they control airspeed.  Elevators do not 'elevate' the aircraft except by converting excess airspeed into altitude.  The primary factor used to 'elevate' an aircraft is excess power.  The landing process is best stabilized by setting constants.  Power is the first and easiest constant to set.  With power set the elevator becomes the speed control and trim is the 'lock' that will set a constant speed.  Once a locked constant speed is attained, small power reductions can be used to control the glide path descent.  Maximum power is used to intercept a higher glide slope.

# Controls and What They Do

The primary controls are the elevators, ailerons and rudder.  These

provide primary movement around the axes of flight.  In combination, they give coordinated movement around the axes of flight.  Engine power is an additional primary control of pitch.  Again, in combination, it gives coordinated movement.  No change in one axis occurs without having some effect on the other axes.

Secondary controls include trim and flaps.  Devices that augment engine power and control operations, weight, center of gravity and load factor have secondary effect on control.  Complex aircraft may have additional controls.  The effect on all controls is dependent on conditions of altitude, speed, temperature and weather.

Neutral pitch is engineered into the placement of engine, wings. horizontal stabilizer and loading limits.  The pitch is moderated to a designed degree by elevator, engine power and trim.  Any change in elevator or engine power along with the rapidity of change requires coordinated control movement in the other axes.  To change only pitch, by whatever means, some additional combination of rudder and aileron is required.

Ailerons "control" bank angle, roll and roll rate but, in combination with the other controls.  On application of aileron in a turn, rudder must be “coordinated” to keep the tail behind the nose; elevator is used to counter loss of vertical lift.  Ailerons work in opposite directions, usually in differing distance and with an effect called adverse yaw.  The down aileron gives lift and drag  (induced).  The drag resists the turn so that rudder is applied for coordination.

Rudder is used most often in anticipation of known requirements from the other controls.  Rudder will induce roll as well as yaw.  The rudder can be used to raise a wing in a stall.  Anticipatory rudder is applied to counter the effects of power/pitch applications.  A rudder applied yaw is used to make possible crosswind landings.  P-factor, torque, precession and slipstream all require use of the rudder.  Skillful rudder on the ball and in anticipation is the distinctive mark of a good pilot.

Power is a pitch control.  Just adding power (no other control input) will cause the nose to rise and roll to the left.  Speed will decrease. In a turn, power will make the left turn possible with little or no rudder but require rudder to "lead" the right turn.  There are countless cause/effects

in the creation and control of a given airspeed and pitch condition.  If you are ever asked about what controls airspeed and pitch, just say, "The pilot".

## Anticipation

The ability to anticipate changes in control pressures required for a particular maneuver must be developed.  Failure to anticipate rudder movement required to move the nose as airspeed decreases is a most common flight error.  The behavior of instruments such as the airspeed indicator and vertical speed indicator that lag in relation to sound and attitude changes must be expected and understood.  Chasing the airspeed indicator is a common student fault.  Even worse is not recognizing that the VSI (vertical speed indicator) takes about 12 seconds before giving accurate indications unless the control movements are exceptionally smooth.  Starting the trim from a known position and keeping track of its movements in various flight configurations makes possible rapid/correct trim pressure corrections.

## Holding Headings

A pilot (not a student) is expected to hold a heading.  The PTS allows a + 10 degree or 20 degree range.  It is a mistake to be accepting of this range.  Successful flying is most dependent upon acquiring and holding a heading, not a range of headings.  Success in holding a heading is dependent upon a pilot's ability to 'hold' the yoke in one position while attention and movement is directed elsewhere.  It doesn't come easily or cheaply but it is there to be achieved.  Rudder alone will do the best job.

Turning to a heading is another much sought skill.  The variables in a turn far exceed those in level flight headings.  The turn has the angle of the bank, anticipation of yoke pressures, and airspeed as a factors.  The quality of the turn is measured by the pilot’s ability to determine when to begin rolling the wings level, when to stop at level and most of all how to keep it there during the transition.  For every degree of bank and airspeed we must learn what to do and when to do it.

Other opinions to the contrary, the thirty-degree bank is the safest and most controllable bank.  The turn can be cleared and completed in a minimal time.  The established bank is quite stable in comparison with others.  Making a standard bank procedure develops a sense of turn time and direction that is easily adapted to airport patterns.  This stability can be demonstrated by entering a 30-degree bank, putting in about 1/2 turn of trim to hold the nose and then holding the bank with light rudder.  It will hold both bank and altitude better than in any other banked condition.

The preferred method of recovering from a bank to a selected heading is to begin recover at half the number of degrees in the bank.  A thirty degree bank's recovery will begin at 15 degrees before the desired heading.  These markings are easily observed on the heading indicator.  With some adjustment in the recovery rate this method will work for all banks.  In the real instrument (IFR) world the standard-rate turn (3-degrees per second) recovery can be done quite quickly without regard to any rule.

# Level Dynamics

When a pilot has his aircraft flying so that the amount of propeller thrust is equal to the drag and the wing lift equals the weight plus the negative lift of the tail surfaces he is in level flight.  The weight will always be focused to the center of the earth.  Up to the wing's critical angle of attack an aircraft and power available will be able to maintain level flight over a wide range of speeds.  When the aircraft is flying slowly drag is mostly induced drag.  At high speeds drag is mostly parasitic drag.

# Leveling Off from Climb

An old saying among pilots is, "How long does it take a student pilot to level off?"  Thirty-five hours is the answer.  It should not take that long if the instructor is on the ball.  The student should know for leveling off from a climb at Vy will require a certain amount of anticipation, a certain amount of trim, a certain amount of acceleration, changing amounts of yoke pressure, a power adjustment, changing sounds and some fine tuning.  The trick is to put the aircraft into the desired attitude and leave/keep it there.

Demonstrate how pushing forward on the yoke will both lower the nose and level the bottom of the wing's surface with the distant horizon.  Have the student perform.  Have the student watch the wing and hold it locked level with his arm against the door.  Have him note the pressure required. Have him swing his eyes to measure the space between the nose and the horizon.  Have him touch the bottom most button on the trim wheel and move it full up while keeping the nose in position.  Have him note the gradual change in pressure as the plane accelerates.  Power should be reduced when reaching 85 kts.  Instructor might make any fine changes needed.  If the student needs to search for level flight and is carrying control pressure then he's doing something wrong.

# Level Cruise #1

As you reach a desired altitude sight on the horizon under the left wing and move the yoke forward until the wing tip is level.  Immediately bring your vision to the nose and hold the yoke so that the nose does not change position relatively to the horizon.  Trim to relive pressure but according to the final trim condition desired.  If you continue to adjust the trim while the aircraft is still accelerating you will create problems to yourself.  Let the aircraft reach its cruise speed  (85 kts) and power set before making final trim adjustments.

# Level Cruise #2

By going high and then diving to acquire cruise speed you can get to speed faster.  Some aircraft actually get on a 'step' like a speed boat and will maintain a higher than normal speed until the condition is disrupted.  The reason for not reducing the power initially to 2450 is that some deceleration occurs during leveling off.  This causes a 100 decrease in RPM.

# Level Cruise #3

The actual reason for having the student to look at the bottom side of the

wing for the nose level attitude is because the student does not yet know where to look over the nose.  The use of the wing can be substituted by a

marker on the windshield once level for the particular person has been determined.  Each persons ‘level’ will be slightly different.  Level flight is established and maintained by positioning the nose.  Repetition in positioning the nose is the best way for a student to learn where level is.  The more often you trim for hands off level the better able you will be able to discern just where level is.  With constant power there is one point on the windshield that keeps the plane level, every rate climb or descent has a specific position for placement of that point based on constant airspeed.  Airspeed is a relatively coarse way of nose adjustment.  It takes & makes a proficient pilot who uses the nose position for level flight.  The pilot who trims for his climb and descent airspeeds soon develops a set of constants for the aircraft.  With repetitive practice the pilot will learn to feel, hear, and SEE just where a particular configuration and attitude will position the nose.

# Unable to fly level

After you have been flying a while either with the instructor or solo a common phenomenon seems to occur where the new pilot is suddenly having difficulty in leveling off.  This is normal.  As we have trained and practiced we have developed along with the procedures for leveling a set of references.  We may have started with the wing on the horizon and gradually been able to reference the nose to the horizon.  Now, it doesn't seem to work.  We may oscillate in altitude, airspeed and trim for several minutes and still not get it right.  It is going to happen.

The reason it this occurs may be due to one factor or a combination of factors.  If the weather changes so that your usually clear horizon is  blocked by haze or cloud formations you have lost an essential reference.  Flying in mountains where the horizon cuts through the mountains can be a causal factor.  Perhaps due to a distraction you forget to trim.  Power control can cause the aircraft to fail to accelerate or to exceed cruise speed.  Any one of these or a combination can cause leveling off problems.  You might practice making deliberate errors in your leveling off procedure to ascertain the corrective procedure that works for you.

Most of the small movements evade detection of the eye but are sensed subconsciously by the peripheral vision, dangerously so.  In certain pattern turn conditions the peripheral vision can deceive your brain as to the true attitude of the nose.

# Level Turn Dynamics

A banked aircraft transfers some of the available wing lift away from the vertical into a turning force.  It is this transfer of lift that makes it necessary for the pilot to increase the wing's angle of attack to obtain the lift required for maintaining a constant altitude.  In this bank there is an apparent increase in weight caused by the horizontal centrifugal forces of the banked turn.  At a 60-degree level altitude bank the weight of everything is doubled. (2 G's)  A 30-degree bank has an effective weight increase of .15 Gs.

Since the most likely C-150 mid-air will come from a faster aircraft from the rear quarter, always look beyond 90 degrees when clearing but any aircraft above the horizon will pass overhead.  Any following aircraft should pass to the right, initiate clearing turns to the left.  There is nothing wrong with raising the wing for clearing.  The instinctive desire to see around the wing in the direction of the turn is both dangerous and inefficient.  You can't really see and you decrease your ability to hold

both bank and airspeed.  Keep your eyes on the nose/horizon during a turn.  Don't turn into an area you have not cleared.  Do not pull back on the yoke

to recover from a turn or bank, use the ailerons.

# Bank recovery to a heading

Lead your recovery from a left bank by applying right rudder.  Lead your heading recovery by 10 degrees in a 20 degree bank, 15 degrees in a 30° bank and 22 degrees in a 45° bank.  Every recovery from a bank also requires that some forward pressure be applied to prevent the 'pop-up' airspeed loss that will occur as the wings acquire added vertical lift when leveled.

# Level Turns

The turn is the only of the four basic maneuvers that exists in conjunction with the other three.  The level turn is a balanced condition, as with level flight, where the lift equals the aircraft weight.  With constant power the airspeed and angle of attack are controlled with the elevator.  Some airspeed is lost during the turn due to an increase in pitch.  The rudder keeps the tail behind the nose.  The quality of the turn is a blend of yaw, roll, pitch and power.  The blend is changed as the angle of the turn increase or if it occurs as level, climb or descent.  A climbing or descending bank requires a different blending of these factors.

Elevator controls pitch.  Elevator trim is for removing control pressures when a prolonged flight condition or attitude is to be maintained. Entering a 30 degree bank requires slightly forward yoke input on the elevator with the thumb.  This prevents excessive loss of airspeed.  On reaching 30 degrees a slight back pressure with the finger will give the pitch needed to maintain altitude.  Recovery from the bank requires slight forward pressure with the thumb again.  These finger applications are more pressures than movement.  If the turn is to the right, rudder pressure precedes aileron movement.  Recovery from a left turn requires that right rudder pressure precede aileron movement.

The only control difference between the left and right bank is the anticipation and lead required on the right rudder.  You lead the right turn with right rudder perceptibly before  you need to  with the left rudder in a left turn.  Again this is because of aerodynamic factors .  Likewise, the recovery from the left bank requires anticipation and leading with the right rudder before leveling off.  In this instance forward pressure is required to prevent the 'pop-up' from causing an altitude gain when leveling off.  The steeper the bank the greater the need for knowing about the amount of anticipation and firm forward  pressure required.

The design of most light aircraft gives a stable 30 degree bank hands off with just a little nose up trim.  The aircraft will tend to level off from any bank less than 30 and become steeper from any bank more than 30.  At 30 degrees the G-force is +1.15, at 20 degrees the G-force is 1.06, at 45 degrees you get +1.41 G, at 60 degrees the G force is +2.0.  Aileron must be held into the bank at less than 30 degrees, against the bank at more than 30 degrees and neutral at 30 degrees.  Any time the ailerons are not neutral there is induced yaw which must be countered by rudder.  Adverse yaw ceases when ailerons are neutral.

A similar maneuver will work with most any G.A. plane but the amount of trim will vary.  A bank of less that 30 will cause the aerodynamics of the plane cause it to want to level off.  Yoke must be held into the bank.  A bank of more than 30 will cause the plane to want to continue on over.  The yoke must be held against the bank to keep the bank from increasing.

# Level turn exercises (Instructor)

Select a flight course on a cardinal heading and toward a prominent point on the horizon.  Execute a series of left and right 30 degree banked turns of 90 degrees. The first turn should be to the left.  (Any passing traffic to your rear should be passing to your right.)  No turn should be initiated without visual clearing.  This clearing should include at least 30 degrees to your rear.  I recommend saying, "Clear left, turn left."  Once the turn has been cleared and the 90 degree reference point selected under the wing the pilot's eyes should be over the nose of the aircraft.  Since we are turning left P-factor will aid in coordinating the ball position so little left rudder will be required.  Once the 30 degree of bank has been attained the yoke must be neutralized as to bank. (The aircraft in a 30 degree bank is relatively stable.

An aircraft in less than a 30 degree bank wants to level out and must be held into the bank by yoke pressure.  An aircraft in more than a 30 degree bank wants to increase the bank and you pressure must be held against the bank.  This is part of the VFR to IFR transition problem.) The C-150 wing strut parallel to the ground makes a 30 degree angle bank.  Yoke pressure will be slightly forward initiating the bank; slightly back as the 30 degree bank is reached and slightly forward to prevent 'pop up' as the aircraft is leveled off.  (The automotive practice of leaning forward, then tilting and turning the head during a turn will cause problems in holding altitude.)  As the turn progresses toward the 90 degree point the 90 degree reference will come into view.  The leveling off from a left turn requires some anticipation of the pilot in leading the yoke pressure out of the bank with light right rudder pressure.  Forward yoke pressure is applied just as wings level occurs.

The turn to the right requires some slightly different techniques.  Clearing and selecting the 90 degree point is followed by, "Clear right, turn right."   Initial yoke movement into the bank is preceded by right rudder since there is no P-factor assistance.  Very little left rudder is required leveling off from a right turn.  All yoke pressures and movement are essentially the same.  (If a student increases the bank instead of leveling off it is probably because they are using and misinterpreting the attitude indicator instead of watching the horizon.)

The first turns should use visual references then use the heading indicator.  (I often reset the heading indicator to get the visual points to correspond to cardinal headings.)  Leveling off on a given heading requires only that the leveling pressures preceded the selected heading by a number of degrees half the angle of bank.  A 30 degree bank should begin leveling 15 degrees before the selected heading.

The accuracy of the visual reference turns can be demonstrated by having the student note the present heading and a 90 degree visual reference point.  Make the turn while the heading indicator is covered; checking only when level.  It is important to practice these turns only a few times during each flight.  Don't beat on a particular skill for too long.  Level turns have skills related to both climbing and descending turns.  It is important that the instructor utilize the instructional time and flight segments in the most efficient manner. A good way to provide a break between skill practice is to survey the area for check and reference points.

One of the very first exercies I do with a new student is to justify my insistance on 30 degree banks in all VFR maneuvers.  In the C-150, after clearing,  I have the student enter and hold a 30 degree bank and give about a half turn of nose up trim, then he is to let go of the yoke and fly only with rudder.

# Climbing Left turns

All turns that are going to exceed the angular range of windshield vision should be preceded by "clear R/L, Turn R/L"  Failure to clear will fail any flight test.

Since there is increased P-factor present in a climbing left turn, the then some right rudder might be required throughout the turn to keep the ball centered..  Even more right rudder will be required when leveling off.  The aircraft will tend to lose some indicated airspeed when all turns are initiated.  A slight, almost imperceptible forward pressure with the thumb will prevent this indicated speed loss.  As soon as the 30 degree bank is reached the thumb pressure is removed and replaced by sufficient one finger pressure to maintain both bank and airspeed.

In addition to P-factor that exists in a climb, in a climbing turn we introduce yaw.  Yaw in a turn is caused by drag.  Drag, in turn, is produced by a higher angle of attack.  The high wing in a turn has more yaw and more induced drag and a higher angle of attack because of the down aileron.  The fact that it is moving faster is a minor but existing parasitic drag factor.  It is the initial induced drag of the aileron's greater deflection when rolling in and out of banks that increases the need for more rudder.

# Climbing Right turns

Right rudder pressure is being held in the climb due to P-factor.

Even more is now required to initiate the right turn.  Anticipate the need to lead with right rudder in making right turns.  Yoke pressures and anticipation is much the same as with left turns.  Recovery from the bank requires only that the right rudder pressure be relaxed and then reset for P-factor to climb on heading.

# Procedures:

- Set bank and scan to position nose to horizon.  Variations of lower wingtip tell of changes

- Level - wingtips equal on both horizons

- IFR - Attitude Indicator to set bank by airspeed and check turn coordinator

- 30° bank - Most stable bank, set wing strut parallel to ground.  Lock angle on horizon, check with horizon and AI.  Gives only 1.15 G-load.  One-half down trim and bank can be flown hands off.

- 45° bank - The more quickly you get to 45 degrees the better you will be able to anticipate back pressure required.  Lock elbow/arm against door to hold pressure or trim two down as entering bank.  Recover at half angle with abrupt forward pressure.  Remove trim.

# Steep Turns

At some point during the first four flights steep turns should be demonstrated by the instructor.  You should use a prominent visual reference on the nose at a cardinal altitude.  While the PTS (Practical Test Standards) requires only one 360 degree turn, the most instructive steep turn consists of two full 360 degree turns, 45 degrees of bank, a constant altitude, and cruise power.  The bank entry to the 45 degree steep turn should be smooth and rapid.  Initially check the angle of bank on the horizon against the attitude indicator.  Once the angle has been achieved concentrate on the horizon and its angle.  Variations of five degrees of bank may be used to control altitude.  The new PTS requires only one 360° turn with recovery near heading.

After clearing, enter the steep turn smoothly and rapidly, lead with right rudder if to the right.  Sight on the horizon and anticipate the loss of lift with a locked elbow on the door and sufficient back pressure to prevent a loss of altitude.  Angle of bank may be varied from 45  + 5 degrees to adjust altitude.  Using the elevator to adjust altitude gives only an illusion of change.  Actually the turn is being made steeper with a resulting loss of altitude, increase in G-forces, airspeed and angle of attack.

Steep turns are precision maneuvers flown as a confidence builder.  The vertical lift lost by the steep bank must be replaced by increasing the angle of attack by applying back pressure.  The seemingly great pressure required is because of the increase in G force due to the bank.  The critical angle of attack of the wing remains the same but due to the increase in weight (G-force) the stall occurs at a much higher speed.  (A stall in this situation is called an accelerated stall because of the higher speed.)  Rudder is used to compensate for drag /adverse yaw from the raised wing.  Once in the turn, the raised wing will travel faster and provide more lift.  To compensate for this lift caused overbanking tendency the ailerons must be held against the bank.

The steep turn, properly performed as to bank and altitude, will, as the second 360 degrees of turn are performed, come in contact with the wake turbulence of the previous 360 degree turn.  This second 360 is no longer required by the PTS (Practical Test Standards) but it is the best way to self check performance of the maneuver.  Encountering the wake will cause the wings to rock and maintaining altitude typically becomes a problem.  The initial surprise seems to be the cause.  The student will instinctively relax pressure when it should be held or increased.  If more than 100 feet is lost the process should be started over from the beginning.  Since the bank is 45 degrees the leveling off should begin about 22 degrees early.  A very positive forward pressure must be applied to prevent a pop-up increase in altitude.  The turns should be performed both left and right but perhaps at different time since they may cause student distress.

There are two distinct ways the steep turn may be performed, with or without trim.  The unexpectedly high yoke pressures required to hold both the bank and the altitude is difficult for students but very instructive.  They should learn to press their arm against the door to lock the pressure and position.  The second way is easier but requires some timing. Airline instructors do not allow the use of trim.  At the moment the 45 degree bank is attained, give the trim wheel two quick full turns down.  This will release almost all of the pressure required to hold altitude.  Now most of attention can be devoted to bank angle and the slight changes needed for altitude.  The yoke release often caused by the surprise of wake turbulence will be compensated for by the trim setting.  However, when leveling off the trim must be removed very quickly before it aggravates the typical pop-up pressures of leveling off.

First: go as quickly into the bank to 45 as you can in both methods.  Easy way:  Using the tip of your right forefinger quickly make two top to bottom of the trim wheel.  Now a light touch will keep you in the bank and at the same altitude.  Lead your recovery by 22 degrees and again quickly remove the two turns of trim with your finger tip.  Do not pinch the trim wheel.

Second Way:

Go as quickly into the bank as you can.  Lock your arm and elbow against the cabin door.  Listen to the sounds!  As one of the others advised anticipation is the name of the game.  Watch the VSI with quick glances.  Watch the nose vs. the horizon the same way.  You will probably overreact to VSI movement so when you do react release half of the reaction and you will be about right.

The new PTS now requires only a 360 but I suggest that you do a 720.  On the second time around you will be in contact with your own wake turbulence if you have done the first 360 well.  The wake turbulence is a way to check your performance.  Don't do to many steep turns at one time.

Trimming makes the steep turn easier but it bypasses the instructional purpose of doing the steep turn.  The purpose of the steep turn is to develop the awareness and anticipation of the control forces and attitudes required.  Heavy control forces are designed into the performance of the aircraft when and where destructive G-loads exist.  If trim is allowed in steep turns to show the required attitude, then just as many turns should be made without trim to show the required control pressures to attain and maintain that attitude.

A common error related to steep turns is what the pilot does when a loss of altitude occurs.  The initial perception is that the loss is due to insufficient back pressure on the yoke.  This may well be the case but the recovery of this lost altitude by only increasing the back pressure is not the solution.  The increase in back pressure alone will tighten the turn by increasing the angle of bank which will lead to a further loss of altitude.  The geometry of the hand, arm and yoke cause this.  Any increase in back pressure must be accompanied by a decrease in bank angle at the same time.  Often loss of altitude can be corrected by just removing some of the bank.  It is this incorrect recovery from a banked turn that leads the graveyard spiral and spatial disorientation.  Get the wings level, recover altitude and start the steep turn again.  The essential is that the angle of bank be held so that no changes in back pressure are required.

Trimming during steep turns can be dangerous.  The potential is there if situational awareness is dimmed.  Any descent can become a high speed spiral at excessive G-load.  The steep bank should never be entered at speeds over Va.  Higher speeds than Va will overstress the aircraft.  A steep turn can only be recovered by leveling the wings.  Ailerons will still be effective. Rolling out of the trimmed bank without removing the trim can cause a low-speed unusual attitude.  Pulling back on the yoke to hold altitude or prevent a spiral dive is more likely to increase the bank and make the spiral more steep.  Avoid such trimming at night, IFR, or in low visual conditions.