# Preflightã

Flight Preflight

Pilot Preflight

Aircraft Preflight

Pre-start/Start Checklist.

Start/Post Start

Runup

Pre-takeoff Considerations

Pre-takeoff

Prelanding

Post-Landing

# Flight Preflight

1) Predeparture briefing aloud

2) Review departure and route

3) Density altitude vs. runway available vs. runway needed

4. Acceleration distance to rotation

5) Windshear conditions

6) Minimum safe altitude

7) Engine failure plan

8) Highest obstacle

9) Approach self briefing

# Pilot Preflight

..I’M SAFE--

NO..

Illness

Medication

Stress

Alcohol

Fatigue

Emotion

Experienced Advice:

- Play ... what if...

- Safety switch is...ON

- Be the best you can be.

- Shortcuts can be lethal

- Shortcuts switch is ...OFF

- Only proficiency ... COUNTS

- Complacently switch is ... OFF

- Attention is on edge of seat...

- Know your airplane...know it well

- Always leave yourself a way out...

- Play ground games... where to land?

- Your usual career is best left at home...

- Give every situation the ‘common sense test’

- Read safety rules of flying rules...the FARs

- If it’s not in doubt, you don’t need to prove it.

- Instinct is your enemy; calm reason wins every time.

- By controlling yourself you can control the airplane.

- The pilot is the most unreliable factor in an airplane.

- As a pilot you don’t need to prove anything to anybody.

- Anticipate with your brain rather than react with your body.

# Aircraft Preflight

FAR 91.103 refers to weather, airport data and POH information on aircraft performance.  FAR 91.7(b) refers to inspection of aircraft condition for safe flight.  If a situation exceeds your knowledge or experience, get expert advice.

We are now ready to go through the preflight.  Every effort is made to minimize the wasted time and movement about the aircraft while maximizing a through inspection.  As though walking up to the C-150 for the first time we unchain the left wing.  Note right main tire for yellow chalk mark showing that refueling has taken place since last flight.  (This is so that we don't have to walk over to it again when we roll the aircraft tires for inspection.) We use a single key to open the door and place it on the floor below the trim wheel.  (A single key is not as likely to result in an accidental turning off of the magnetos as is a key ring full of keys.)  (Placing the key on the interior cowling has caused many a key to disappear down the defroster hole.)

Fuel may be ordered by radio using 122.95.  This is the universal frequency used at airports with towers for Unicom services such as ordering a taxi or fuel.  At CCR we normally use Chevron and call the truck as “Chevron one”.  If the truck fails to answer contact “Chevron Base”.  Give your location as “East Ramp Golf two Cessna 6185K”.  With the new aircraft cover you should check the logbook to ascertain if any remark is there that would make the aircraft unairworthy.

Preset radios and transponder.  Remove the control lock, turn on the master, check the fuel gauges, and say, "Clear flaps" before lowering the flaps. (Get fuel as necessary.)  The "regulations" requires that a fuel gauge be accurate ONLY when reading full and empty.  Open right door and get fuel sump cup.  Drain and check fuel in left tank.  Place sump cup in right seat. (You don't need to carry it)  Check wheel faring (brake lines if visible), front and back side of left flap and antennas.  Check empennage, horizontal and vertical stabilizer, elevator hinges and movement on left side.  Remove tail chain.  Check rudder hinges and movement.  Check elevator (under side, too) and trim hinges and movement on right side.  Check empennage.  Check back side of right flap, front and back of right flap hinges, counterweight and movement.  Wing tip and right leading edge. Unchain right wing, check underside of right flap and drain right wing sump.  Return sump cup.  Check right fuel tank and cap.

Check nose wheel faring, strut, and damper, inside engine compartment, loosen oil stick with left hand, remove with right hand, clean with left hand, re-insert and check for 4 quarts minimum.  Pull fuel strainer with right hand and clean left hand with gasoline.  Oil stick can be hung on prop blade while adding oil.  (Be sure not to lose seal off lid of bottle into engine.)  Don't make oil cap too tight.  Check leading edge of propeller, spinner mounting plate for cracking, and cowling inlets.  Make sure air filter is secure and intact. Watch for 'working rivets, those that have gray powder around them. Roll aircraft at least 40 inches to check tires.  If diagonal cord shows in a smooth area the tire is unsafe for flight.  Check static air hole, overflow, pitot tube, and stall warner. Don't blow into any aircraft instrument intake hole.  Check left fuel tank and cap.  Check left leading edge and tip.  Check left aileron front back, movement and counterweight.  Rocking the wings and gently moving horizontal stabilizers is a good way to pick up internal damage via sound.  Do the 'squat test' to make sure everything is clear of the aircraft.  The only difference in preflighting a C-172 is the luggage door and alternator belt.

The preflight requires the student to be aware of the possible causes of accidental propeller movement.  That the key may possibly be removed from other than the off position. That the magneto can be otherwise grounded and allow a short movement of the propeller to start the engine.  That, while there is no absolute safe way to turn the propeller, backwards is the safest.  (Do not turn propeller of C-150 backwards). The only time you can have too much fuel is when you are on fire.  From full tanks the maximum safe flight time of a C-150 is three hours.  The tanks are full and the caps are tight only when checked by the pilot.  No C-150 flight should be undertaken with less than four quarts of oil.  Failure to monitor a radio frequency prior to transmitting as well as being poorly prepared to talk impinge on the flight safety of everyone.  Poor use of the radio is the most common failing of the incompetent pilot.

Throughout the first preflight the instructor keeps up a running commentary into the tape recorder as to the why's, wherefore’s and how to's for each check.  On successive flights additional operational checklists are added.  The first student effort is usually pages long.  The second effort is more concise.  For the third effort the instructor provides a twice folded 4 x 6 card cut half way through one fold.  This card can be made into a compact but complete checklist covering all operations from preflight to shutdown.  Arrows can be used below each list to indicate direction to fold card.  Emergency list is in outlined red.  Finally the pilot developed list is cross-checked with the manual approved one to make sure that all items are covered.

Where the student has previous flight experience the same process is used but the pilot is allowed to build on  his experience.  I feel it is vital that a pilot have his own personally developed checklist for every aircraft flown.  Build on first learned skills and habits.  The published and universal lists omit or rearrange the order of items so that mistakes often occur.  The checklist must contain all items from the aircraft manual BUT there are many supplementary items of radio, transponder, leaning, taxiing, braking, and clearing that are not mentioned.  Time is never wasted giving a careful preflight.

# Pre-start/Start Checklist.

Once in the aircraft we begin the pre-start tape recording.  Seats, doors, window open, belts.  FAR 91.107 requires the pilot to brief all passengers on how the seat belt and harness operates this may be done via an series of questions or an informational card.  The passengers must be advised to buckle up before the aircraft moves and again before takeoff and landing.FAR 91.105 requires the pilot, a required crewmember, keep his belt fastened at all times.  The shoulder harness need be fastened only during takeoff and landings if other duties at other times require its removal.

Two people of aggregate weight below 170 pounds may use one seat belt.  Under such usage the likelihood of injury is much greater.  As of 1992 child seats were approved and must be labeled as certified for use in aircraft.  This is in addition to the allowing of holding, outside of the belt, children less than two.  The requirement that passengers have seat belts available and used on takeoff applies to parachutists.

FAR 91.107(3) states that seat belts and shoulder harnesses are required for all occupants during takeoff, landing, and movement on the surface.  The FAR is unclear in this regard since requiring the belts to be available is not the same as requiring that they be worn.  The PIC is not required to make occupants wear belts.  The government-gods are crazy.

I require my students to advise me of my belts. (It is surprising how often my door seems to open on takeoff when student has failed to call it from a checklist.)  Key in. (Amazing how often the key is in the pocket beneath the tightened seatbelt.)  Check trim, radios set, Carb heat cold, and mixture rich. Prime.  In cold weather it helps to give the primer a squirt or two during the preflight.

PRE-START             	  START		Post Start	   TAXI

Seats/belts/doors/window   	"Clear"            800 rpm      Brake check

Radios set/Brakes         	 Brakes on	       Flaps up     Heading set

C.H./Prime/Mixture/Pump    	 Radios Off        Lean 1”     Wind direction

Key/Master/				 Start		 ATIS	on 	  Dive away

Oil Pressure Climb into

ATIS copy

# Start/Post Start

Make several dry-run engine starts on the tape recorder to develop the checklist.  Emphasize used of the left hand all the time holding the mike.  The right hand remains on the throttle unless using radios.  Try adding a pencil in the right hand.  There is no reason holding such objects in either hand needs to interfere with the use of these hands during flight operations.  Later on, this ability will become a very valuable asset.

After priming, hold the brakes, set the throttle for 1/4 inch in, yell "clear", look to both sides of the aircraft, and turn the key to start.  The position of the forefinger on the throttle is used to index the power at start at 800 RPM.  If the engine has not started by the time six blades have passed, stop cranking.  Review your starting procedure and try again.  As soon as the engine  has started reach over with your right hand and close the window.  Leaving the propeller blast through the open window to make a draft of cold air on the instructor's neck will not improve an already questionable personality.  Avoid initial rpm higher than 800.  This initial start is without oil pressure or lubrication and rpm should be kept low.  Turn on the radio and listen to ATIS while confirming oil pressure, amps, and suction.

Use the Prestart, Start, and Post Start lists during each dry run.  Show the student that an alternative for quick checking of a few things can be with fingers.  A different finger for each item. Thumb for beacon, index for radios, middle for flaps, ring for oil pressure and little for leaning..  Show how the throttle settings can be 'indexed' by holding index finger on the friction lock of Cessna aircraft.  Don't make the final start of the engine until you have practiced the radio work.

# Runup

Use a run-up checklist.  Always face the wind.  (Engine cooling is the reason.)  We are not teaching a flying career made only of C-150s.  We should teach with the C-150 the procedures that will follow over to high performance aircraft. An efficient runup should take only a minute or two at most.  When checking controls your thumb always points to the up aileron.  Index with your finger how much throttle will give the desired RPM.  Learn the sound/feel for 1700 and 800 to save time.  An airplane is just a expensive to operate on the ground as in the air.

Check your controls with the 'thumbs up' technique and multiple head movements.  The left thumb up always points to the up aileron and the head turn confirms both an up and down aileron.  Elevators are confirmed up and down and rudder left and right.

The right hand remains on the throttle while the left locates and touches the ampere meter and suction.  Touching the item on the checklist is an insurance confirmation for the observer. (examiner))  The power is indexed to 800 rpm by feel and sound.  (Instructor covers tachometer.)  Flight instruments and set and checked.  A frequent error is in setting the heading indicator with the compass.

I teach indexes for throttle position.  From idle at 800 rpm, the throttle held in the palm of the hand and the index finger moved back the length of a fingernail. Throttle is moved in until finger tip touches throttle clutch.  This should give the 1700 rpm required for the magneto check.  Repeat this exercise several times. Student should learn to do this by sound and feel.  (I have had several instances where this kind of operational knowledge of position and sound has changed emergency into routine.)  This indexing skill should be taught for both power settings and power reductions.

I present magneto checking as a smooth switching via count as 1-2, 1-2, 1,1 with the eyes focused on the tachometer to note rpm drop and difference. Some pilots tend to leave the engine operating on one magneto overly long.  It only takes a moment to check.  The hand moves to and pulls and pushes the carb heat while the eyes note drop when on and recovery when off.  If, during runup, you should accidentally go to OFF fully retard the throttle before turning magnetos back on.  This prevents after-firing that can and will damage the exhaust system.  It will blow any carburetor ice out in an emergency, however.

Knowing the airport altitude is an additional aid to knowing the altimeter setting.  The best altimeter setting is from an accurate copy of the ATIS information.

A good run-up should take only two or three minutes.  Any longer is indicative of inefficiency or poor checklist techniques.  Some pilots use a finger-count method to keep items in order.  Others work systematically across the panel.  Use whatever works best for you.  Using ‘cockpit’  checklist is just about the worst thing you can do short of not using a list at all.  Dry run the next radio call up procedures until they come out smoothly, accurately, and completely.  Discuss the taxiing, clearing turn to be made to check for aircraft on base leg and climb out departure plan.  No turn over 30 degrees should ever be made without both looking and saying "clear".

RUNUP         PRETAKEOFF

Into wind      Flap/(Pump)

Controls       Trim for climb

Mixture/RPM    Freq/vol/xpond

Mags/C.H.      Time Ck

Suction/Amps   Departure

RPM            1st CkPt

Instruments    Course

Radios set     Time

# Pre-takeoff Considerations

1. Gross Weight and Center of Gravity

Gross weight is the empty weight plus useful load.  You must check the POH to see if empty weight includes oil but it always includes unusable fuel.  Useful load includes useable fuel, (oil?), passengers, and luggage.

Gross weight directly affects stall, takeoff speed, and maneuvering speeds.  An improperly loaded aircraft will have undesirable flight qualities.  If you have doubts or don't know, refer to the POH.	The Va, maneuvering speed decreases as aircraft weight decreases.

2. Density Altitude

Density altitude combines the effect of pressure altitude and temperature.  This affects engine power, propeller thrust, and the speed/distance required to create takeoff list.

3. Wind

A headwind will reduce takeoff roll.  A tailwind will increase takeoff roll.  Regardless of the wind you always fly indicated POH airspeeds.  A 90-degree crosswind does not affect takeoff roll distance.  Liftoff during takeoffs in gusty wind conditions should be done at Vso + 5.

5. "What if... Planning

See Takeoff Emergency

# Pre-takeoff

Radio frequencies are changed and monitored before call up.  The transponder is activated, gauges and instruments are scanned.  Outside the aircraft is cleared.  The EMERGENCY checklist is positioned.   Practice the radio procedure for completeness, brevity, and smoothness in its entirety until it comes out in the correct order without pauses of punctuation.  Then the frequency is checked and the call made.

# Prelanding

Copy ATIS

Fuel/Mixture

Think:

Gauges/instruments

Type of entry

Altitude

Pattern altitude

Go-Around procedure

Position/traffic (See Drawer Mike)

Frequency

# Post-Landing

Once on the ground, Do not relax!  Hold the yoke back and properly positioned for taxi.  For any turns off the runway the yoke should be repositioned.  The flaps should be retracted prior to any brake application. You are not clear of the runway until you cross the hold-bar.  ATC will normally advise you to contact ground.  During any delay you should continue to "clean up" the cockpit.  This would include removing flaps, carburetor heat; turning off the strobes, unnecessary lights, transponder and  leaning the mixture.

Z-level should be part of every preflight

Best to consider 3000 foot level winds as surface winds affected by friction.