# Checkride questions (28 pages)ã

1. What must be in the pilots possession before flight?

2. What entries must be in a pilot's log book before he can legally carry 	passengers?

3. What are required papers for an aircraft?

4. What must preflight items include?

5. Name four specific use transponder codes.

6. Name four specific oxygen use levels.

7. Where is an altitude encoder required?

8. Name maximum speeds required by FAR's.

9. Discuss altitude requirements required by FAR's. Minimum safe, MC 	requirements, inhabited areas, populated stadiums, one person, cloud 	clearances/altitudes

10. Identify the ATC light signals.

11. Discuss existence, function, and requirements of Class Delta Airspace. 	  	SVFR in CZ uncontrolled airport.

12. How is traffic direction determined at differing airports?

13. When does the hemispheric rule not apply?

14. How is the Magnetic Course used?

15. How is Magnetic Course obtained?

16. What does the isogonic line show?

Answer

1. Pilot license good until revoked and current medical good 6 months, 1 	year, 2 years by class depending on age.  (radio not required in 	U. S.)

2. Biennial flight review or new rating acquired, verified takeoffs/landings 	under day/night conditions in last 90 days,  Checkout and endoresments 	in type, high altitude, complex aircraft, and high perromrance required.

3. Airworthiness, registration, (radio station for foreigh flight), manual or 	placards of limitations.  Weight and balance,  limitation placards, ELT 	battery life limit placard.  Engine log, aircraft log and ELT battery 	installation dated/signed, not required aboard but must be available.

4. All available information; weather; aircraft performance; fuel to 	destination plus 30 min. day, 45 min. night; destination runway 	lengths; alternatives.  Sectional not specifically mentioned.

5. 1200 VFR; 7700 emergency; 7600 radio emergency; 7500  hijack.

6. Below 12500 none; 12500 to 14000 1/2 hour none; 14000 to 15000 pilot; 	above 15000 everybody.

7. Above 10,000', in a TCA

8. 250kts below 10,000; 200kts below a TCA; 200kts in a ATA.

9. 1000 above 2000 lateral stadiums congested; 500 clear of  people sparse; 	able to land no damage/injury sparse;  No limit if intention to land; 	600' Contra Costa turn  limit at Concord (not FAR); 1000 above, 500 	below clouds; 3000 AGL above ATA; Above /below TCA without clearance; 	12500,14000,15000 oxygen; 12500 transponder; 180000 IFR

10. See chart in AIM.  flashing white no meaning in air, flashing red on 	ground clear runway.

11. ATA exists when tower open, communications requirements, controls 	traffic.  CZ exists specified times when weather is below VFR minimums 	to guarantee IFR aircraft separation. The ATA is 5 statute miles 	radius 	(new 4.1 nautical) from airport center and not shown on a 	sectional.  	The CZ is shown as --- 	on the sectional.  San Francisco no fixed wing 	SVFR. Requires contact with FSS or ATC (radar)

12. Standard is left; tower clearances/instructions; Use of  segmented 	circle; wind indicators; amber light for right.

13. Within 3000' of the ground.

14. Determines if hemispheric rule applies above 3000' AGL.

15. + amount indicated on isogonic line combined with True Course.

16. The angular difference between the north pole and magnetic north.

Question

1. How does the nose gear of a Cessna and a Piper differ?

2. Under what circumstance might full pressure on a Piper (fully 	operational) brake prove ineffective?

3. How high is an FAA tree?

4. How does a Warning Area differ from those in a Restricted area?

5. How do the Sectional representation of Federal and State Game Reserves 	differ in their presentation?

6. Why is there a yellow line down the middle of taxiways?

7. How can flying direct to a VOR kill you?

8. What is the significance of flights over 50 nautical miles?

9.  What is the difference between true course and true heading?

10. Where can you find an aircraft's true airspeed?

11. Explain how aircraft performance differs between Vx/Vy.

12. When can a private pilot accept money from passengers?

13. Explain the function of the magenta/blue tint areas of the sectional.

Answer

1. The Cessna nosegear is steerable by means of the rudder pedals through 	springs.  This is only when the strut is depressed.  In flight, the 	Cessna nosegear hangs down and aligns with the relative wind.  This 	means the wheel is aligned with the direction of travel (runway) in a 	crosswind landing even in a cross-control condition.

The Piper nosegear is directly linked to the rudder and rudder 	pedals both in air and on the ground.  In a crosswind landing the 	cross-control approach with the Piper results in the nose wheel being 	at an angle to the runway.  Any Piper crosswind landing must be 	initially on one main wheel only.  The nose wheel must be straightened 	before making ground contact.

2. The Piper has a cabin support bar across the cockpit just above the 	rudder 	pedals.  Inadvertent placement of the toes above the pedal toe-	stops 	can bring the feet into contact with this bar.  In this 	situation 	neither steering or braking is possible.(Hand brake still 	works)

3.  50'

4. The only difference is that Warning Areas exist over international waters 	that cannot be restricted from international travel.  The hazards of 	each are identical.

5. Federal minimum altitudes are 2000' AGL.  This is shown in a presentation 	on the margin of the sectional.  State's are 2000' AGL This is shown 	near each reserve in magenta.

6. A suggestion that an aircraft should taxi in the middle of the taxiway.

7. CCR VOR as an example.  It can be received from behind Mt. Diablo in such 	a manner as to fly the unaware aircraft into the mountain.

8. Logged as cross-country if landing is made when required for ratings.  	Otherwise, log flights to airports regardless of length. 50 hours 	required to take IFR practical test. after getting private rating.

9.  Wind correction angle

10.  In the manual

11.  Vx clears an obstacle in shorter distance but does not gain as much 	height in time.  Vy gains most altitude over given time.

12.  Only to share expenses.

13.  These transition areas become effective when weather is below VFR 	minimums.  If unable to maintain 1000' above,  500' below or 3 mile 	visibility, the non-IFR flight is required to descend to within 700' 	AGL/magenta areas or 1200' AGL blue (all other) areas and maintain 1 	mile clear of clouds.  This assures IFR flights safe clearances. (A 	special transition area exists northwest of Ukiah with set altitudes 	from 7500 to 9500 which avoids the up and down terrain avoidance 	required by the 700/1200 foot transition areas.

Question

1. What can we do to avoid vertigo?

2. What is required if you fail a written test for first time?

3. What is the definition of a complex aircraft?

4. How can the differences between adding power to an automobile and an 	airplane get you into trouble?

5. Why does the trim seem to work backwards?

6. What is meant by the stabilized approach?

7. What aspect of your flight training and technique will "break 	down" 	during the flight test, if not part of your usual practice?

8. How should you use excess airspeed in the event of an engine

failure emergency?

9. How do high density altitudes change your indicated approach speed for 	landings?

10.  How do you determine the dimensions and  limitations of a prohibited,  	restricted, or alert area.

11.  What are basic VFR minimums above/below 10000' in controlled airspace?

12.  What are basic VFR minimums within 1200' of the surface in uncontrolled 	airspace? Day vs night

13.  What is the VFR minimum in Class D airspace?  How do you tell if not 	VFR?

14.  What is the discrete use of the following frequencies?

121,5; 122,0; 123.6; 122.2, 126.2 122.95

15.  What limitation is placed on night SVFR?

16.  What is the meaning of an airport beacon operating in the daytime?

17.  Define the following and give the actual airspeeds for the aircraft 	used:  Vso, Vx, Vy, Va, Vne

18. Where can you find pressure altitude?

Answer

1. Vertigo sensations cannot be prevented, they can be ignored.  You don't 	appreciate the trained will power required to ignore vertigo until you 	have had it.

2. No wait, get a new instructor sign-off to take test.

3. Constant speed propeller, flaps, retractable gear.

4. Power alone will decrease an aircraft's airspeed by inducing a climb.

5. Raising the wheel lowers the nose.

6. Hands-off flight approach on glide path and at airspeed.

7. Use of a checklist

8. Gain altitude

9.IAS should be the same for every landing.  Apparent ground speeds can vary 	according to wind and density altitude.

10. Refer to the sectional upper margin, call a Flight Service Station.

11. 1000 above 1000 below 1 mile lateral 5 mile visibility;

1000 above 500 below 2000 lateral 3 mile visibility;

12. 1 mile clear of clouds in day; 3 mile 500/1000,2000 night unless in the	pattern 1/2 mile of airport.

13. 1000 ceiling, 3 mile visibility, ATIS, day rotating beacon out.

14. Emergency; flight watch; landing/departure from FSS only airport,

universal FSS frequency, military towers, controlled airport unicom.

15. Must be IFR current in IFR airplane.  CCR tower says that night SVFR would 	be permitted in the CZ if you could enter or leave the CZ in required 	night VFR conditions.

16. Weather below VFR minimums, SVFR clearances required

17. Minimum safe operating, best angle, best rate, never exceed.

18. Set altimeter to 29.92

## Question

1. What must be in the pilots possession before flight?

2. What must be in a pilot's log before he can legally carry passengers?

3. What are required papers for an aircraft?

4. What must preflight items include?

5. Name four specific use transponder codes.

6. Name four specific oxygen use levels.

7. Where is an altitude encoder required?

8. Name maximum speeds required by FAR's.

9. Discuss altitude requirements required by FAR's.

10. Identify the ATC light signals.

11. Discuss existence, function, and requirements of Class D airspace.

12. How is traffic direction determined at differing airports?

13. When does the hemispheric rule not apply?

14. How do you determine the dimensions and  limitations of a prohibited, 	restricted, or alert area.

Answer

1.	Pilot license and current medical(radio not required)

2. 	Biennial flight review or new rating acquired, verified takeoffs and 	landings under conditions in last 90 days, checkout in type.

3. 	Airworthyness, registration, radio station, manual or placards of 	limitations.  Weight and balance, engine log, aircraft log not required 	aboard but must be available.

4. 	All available information; weather; aircraft performance; fuel to 	destination plus 30 min. day, 45 min. night; runway lengths; 	alternatives.  Sectional not specifically mentioned.

5. 	1200 VFR; 7700 emergency; 7600 radio emergency; 7500 hijack.

6. 	Below 12500 none; 12500 to 14000 1/2 hour none; 14000 to 15000 pilot; 	above 15000 everybody.

7. 	Above 10,000 down to 2,500 AGL, within 30 miles of a Class B airspace 	primary airport, within and above all ARSAs, in ADIZ airspace, within 	10 miles of certain airports, and at all times if so equipped.

8.	250 kts below 10,000; 200 kts below Class B or C airspace, 200 kts in 	Class D airspace. See FAR 91.117

9. 	1000 above 2000 lateral stadiums congested; 500 clear of people in 	sparse areas; able to land no damage/injury sparse; No limit if 	intention to land; 600' Contra Costa County turn limit at Concord (not 	FAR); 1000 above, 500 below clouds; 3000 AGL above ATA; Above /below 	Class B without clearance; 12500, 14000, 15000 oxygen; 10,000 	transponder; 18,000 IFR

10.   See chart in AIM.  flashing white no meaning in air, flashing red on 	ground clear runway.

11.   Class D footprint exists when tower open, communications requirements,

controls traffic.  CZ when weather is below VFR minimums to guarantee 	IFR aircraft separation. Standard Class D is 4.4 nautical miles radius 	from airport center.  The airspace footprint is shown as --- on the 	sectional.  No SVFR fixed wing shown above airport data on ectional.

12.   Standard is left; tower clearances/instructions; Use of segmented 	circle; wind indicators; amber light for right taffic.

13.   Within 3000' of the ground.

14.   Refer to the sectional upper margin, call a Flight Service Station.

Question

1. What are basic VFR minimums above/below 10000' in controlled airspace?

2. What are basic VFR minimums within 1200' of the surface in uncontrolled 	airspace?

3. What is the VFR minimum in a Control Zone?

4. What is the discrete use of the following frequencies?  121,5; 122,0; 123.6 	(use ending with closing of FSS);

5.  What limitation is placed on night SVFR?

6.  What is the meaning of an airport beacon operating in the daytime?

7.  Define the following and give the actual airspeeds for the aircraft                                                            	used:  Vso, Vx, Vy, Va, Vne

8.  Explain how aircraft performance differs between Vx/Vy.

9.  When can a private pilot accept money from passengers?

10.  Explain the function of the magenta/blue tint areas of the sectional.

11.  How is Magnetic Course affected by altitude?

12.  What does the isogonic line show?

13.  Where can you find pressure altitude?

14.  What is the difference between true course and true heading?

15.  Where can you find an aircraft's true airspeed?

Answers:

1.   1000 above 1000 below 1 mile lateral 5 mile visibility; 1000 above 500 	below 2000 lateral 3 mile visibility

2.   1 mile clear of clouds.  Night where limits same as in answer #1.

3.   1000 ceiling, 3 mile visibility

4.  Emergency; flightwatch; landing/departure from FSS only airport

5.  Must be IFR current in IFR airplane

6.  Weather below VFR minimums, SVFR clearances required

7.  Minimum safe operating, best angle, best rate, never exceed.

8.  Vx clears an obstacle in shorter distance but does not gain as much height 	in time.  Vy gains most altitude over given time.

9.  Only to share expenses.

10.  Transition areas effective when weather is below VFR.  If unable to

maintain 1000' above,  500' below or 3 mile visibility, non-IFR flight

is required to descend to within 700' AGL/magenta areas or 1200' AGL (

(blue)and maintain 1 mile clear of clouds.  This assures IFR flights  	safe clearances.

11.  Determines if hemispheric rule applies.

12.  The angular difference between the north pole and magnetic north.

13.  Set altimeter to 29.92

14.  Wind correction angle

15.  Manual

Question

1.  No person may pilot an aircraft, with or without passengers, unless his logbook contains an endorsement that he has satisfactorily passed a flight review within the past _________ months.  FAR 61.56(c)

2. Requirements for pilots carrying passengers FAR 61.57

Daytime_______	Nighttime _______

3. What documents must be on board the aircraft at all times.  FAR 91.203, and 91.9 and Federal Communications Commission.

4. Discuss use of radar services for AIM 4-16

Answer

1. 61.56 (c)... Since the beginning of the 24th calendar month before the month in which that pilot acts a pilot in command.

2. 61.57 (c) General experience.  No person may act as pilot in command of an aircraft carrying passengers, ...unless within the preceding 90 days, he has made three takeoffs and three landings as the sole manipulator of the flight controls in an aircraft of the same category and class....  If the aircraft is a tailwheel airplane, the landings must have been made to a full stop in a tailwheel airplane.

61.57 (d) Night experience.  No person may act as pilot in command of an aircraft carrying passengers during the period beginning 1 hour after sunset and ending 1 hour before sunrise unless, within the preceding 90 days, he has made at least three takeoffs and three landings to a full stop during that period in the category and class of aircraft to be used.

4.  91.203  Civil aircraft: Certifications required.

(a)  ...no person may operate a civil aircraft unless it has within it the following:

(1) An appropriate and current airworthiness certificate.

(2) An effective U.S. registration certificate issued to its owner or, for operation within the United States, the second duplicate copy (pink) of the Aircraft Registration Application as provided for in Sec 47.31(b)

(b) No person may operate a civil aircraft unless the airworthiness certificate required by paragraph (a) of the section... is displayed at the cabin or in cockpit entrance so that it is legible to passengers or crew.

91.9 Civil aircraft flight manual, marking, and placard requirements

(a) ...no person may operate a civil aircraft without complying with the operating limitations specified in the approved Airplane Flight Manual, markings, and placards...

(b) No person may operate a U. S. -registered civil aircraft -

(1) For which an Airplane Flight Manual is required by Sec. 21.5 of this chapter, unless there is available in the aircraft a current approved Airplane Flight Manual...

(2) For which an Airplane Flight Manual is not requires by Sec. 21.5 of this chapter, unless there is available in the aircraft a current approved Airplane flight Manual, approved manual material, markings and placards, or any combination thereof.

(c) No person may operate a U. S. -registered civil aircraft unless that aircraft is identified in accordance with part 45 of this chapter.

RADIO The Federal Communications Commission must issue a radio station license specific to the aircraft or organization (group license).  A copy of that license is required to be aboard the aircraft at all times.  There is no longer a required radio license for the pilot in the United States.  Such a license may be required in countries outside the U. S.

4. AIM 4-16 TERMINAL RADAR PROGRAMS FOR VFR AIRCRAFT MUST BE READ IN ENTIRETY

Question

1. What documents must be in the possession of the pilot? 61.3

2. Oxygen requirements for flight above 12500, 14000, 15000. FAR 91.211

3. What are the aircraft equipment and pilot experience requirements for 	flight in Class B airspace?  FAR 91.131

4. What are the common frequencies for contacting the FSS?  Sectional

Answer

1.  61.3 Requirement for certificates, rating, and authorizations.

(a) Pilot certificate.  No person may act as pilot in command ...of a civil aircraft of United States registry unless he has in his personal possession a current pilot certificate issued to him under this part.

(c) Medical certificate.  ...no person may act as pilot in command ... under a certificate issued to him under this part unless he has in his personal possession an appropriate current medical certificate issued under Part 67 of this chapter.

2. 91.211 Supplemental oxygen

(a) General.

(1) At cabin pressure altitudes above 12,500' MSL up to and including 14,000 (MSL) unless the required minimum flight crew is provided with and uses supplemental oxygen for that part of the flight at those altitudes that is of more than 30 minutes duration.

(2) At cabin pressure altitudes above 14,000' (MSL) unless the requires minimum flight crew is provided with and uses supplemental oxygen during the entire flight time at those altitudes; and

(3) At cabin pressure altitudes above 15,000' (MSL) unless each occupant of the aircraft is provided with supplemental oxygen.

3. 91.131 Class Bravo airspace

(a) Operating rules.  No person may operate an aircraft within a terminal control area designated in part 71 of this chapter except in compliance with the following rules:

(b) Pilot requirements.

(1) No person may takeoff or land a civil aircraft at an airport within a terminal control area or operate a civil aircraft within a terminal control area unless:

(i) The pilot in command holds at least a private pilot 				certificate; or

(ii) The aircraft is operated by a student pilot who has met the requirements of Sec. 61.95 (Instruction and 90 day sign-off for operation in but not landing at SFO)

(c) Communications and navigation equipment requirements.

Unless otherwise authorized by ATC, no person may operate an aircraft within a terminal control area unless that aircraft is equipped with

(1) For IFR operations.  An operative VOR or TACAN receiver; and

(2) For all operations.  An operable two-way radio capable of 			communications with ATC on appropriate frequencies

(d) Transponder requirement.  No person may operate an aircraft in 		Class B airspace 	unless the aircraft is equipped with the 			 operating transponder and automatic altitude 					reporting equipment specified

4. FSS has one discrete frequency to be used when landing, operating on and 	taking off from an FSS only field.  123.6 (Obsolescent)

122.2 is a (nearly) universal frequency if a more discrete frequency 		is unknown.

The  heavy lined boxes at FSS locations have FSS Frequencies.

The Duplex system is being phased out except at the most remote VORs.

Question

1. Define the following: Squawk, Squawk VFR and Ident, Stop Squawk, Squawk 	alt, Stop squawk altitude  AIM Chapter 4

2.  When would a code other than 1200 be used? AIM Chapter 4

1. RADAR BEACON PHRASEOLOGY

Air traffic controllers, both civil and military, will use the following phraseology when referring to operation of the Air Traffic Control Radar Beacon System.  Instructions by ATC refer only to Mode A/3 or Mode C operation and do not affect the operation of the transponder on other modes.

1. SQUAWK (number) --Operate radar beacon transponder on designated 		code in Mode A/3

2. IDENT -- Engage the "IDENT" feature of the transponder.

3. SQUAWK (number) and IDENT -- Operate transponder on specified code 		in Mode A/3 and engage the "IDENT" feature.

4. SQUAWK STANDBY -- Switch transponder to standby position

5. SQUAWK LOW/NORMAL -- Operate transponder on low or normal 			sensitivity as specified.  Transponder is operated in "NORMAL"  		position unless ATC specifies "LOW", ("ON" is used instead of 		"NORMAL" as a master control label on some types of 				transponders.)

6. SQUAWK ALTITUDE --Activate MODE C with automatic altitude 			reporting.

7. STOP ALTITUDE SQUAWK --Turn off altitude reporting switch and 			continue transmitting MODE C framing pulses.  If your equipment 		does not have this capability, turn off MODE C.

8. STOP SQUAWK (mode in use) --Switch off specified mode.

9. STOP SQUAWK --Switch off transponder

SQUAWK MADAY -- Operate transponder in the emergency position 		(Mode A Code 7700 for the civil transponder.

11. SQUAWK VFR --Operate radar beacon transponder on code 1200 in the 		MODE A/3, or other appropriate VFR code.

AIM 6-11; 6-31; Assigned by ATC

2.  TRANSPONDER EMERGENCY OPERATION AIM Chapter 6 Section 2

a. When a distress or urgency condition is encountered, the pilot of an aircraft with a coded radar beacon transponder, who desires to alert a ground radar facility, should squawk Mode 3/A, Code 7700/emergency and MODE C altitude reporting and then immediately establish communications with the ATC facility.

b. Radar facilities are equipped so that Code 7700 normally triggers an alarm or special indicator at all control positions.  Pilots should understand that they might not be within a radar coverage area.  Therefore, they should continue squawking Code 7700 and establish radio communications as soon as possible.

AIM 6-4-2  TRANSPONDER OPERATION DURING TWO-WAY COMMUNICATIONS FAILURE.

a.  If a pilot of an aircraft with a coded radar beacon transponder experiences a loss of two-way radio capability he should:

1, Adjust his transponder to reply on MODE A/3, Code 7700 for a period of 1 minute.

2. Then change to Code 7600 and remain on 7600 for a period of 15 minutes or the remainder of the flight, whichever comes first.

3. Repeat steps (1) and (2) as practicable.

b. The pilot should understand that he may not be in an area of radar coverage.

Question

1. FAR 91.215  When is altitude reporting capability required

2. Maximum airspeed below 10,000'   FAR 91.117

Answer

1. 91.215  ATC transponder and altitude reporting equipment and use.

(b) All airspace.  No person may operate an aircraft in the airspace described in paragraphs (b)(1) through (b)(5) of this section, unless that aircraft is equipped with an operable coded radar beacon transponder ...and that aircraft is equipped with automatic pressure altitude reporting equipment, having a Mode C capability...

(1) All aircraft in Class A or Class B airspace;

(2) All aircraft.  In all airspace within 30 nautical miles of 			a Class B airspace primary airport from the surface upward to 				10,000' MSL.

(4) All aircraft.

(i)  In the airspace of an Class C airspace, and

(ii)  In all airspace above the ceiling and within the 				lateral boundaries  of Class C airspace, upward to 				10,000' MSL, and

(5) All aircraft...

(i) In all airspace of the 48 contiguous states and the District of Columbia:

(B)  At and above 10,000' MSL  and below the floor of 				a positive control area, excluding the airspace at 					and below 2,500' AGL; and

(ii)  In the airspace from the surface to 10,000' MSL 				within a 10 nautical mile radius of any airport listed in 				Appendix D of this part excluding the airspace below 1,200" 			AGL outside the airport traffic area for that airport.  				(There are no such airports in the Bay Area.

(c) Transponder on operation.  While in the airspace as specified in paragraph (b) of this section or in all controlled airspace, each person operating an aircraft equipped with an operable ATC transponder  maintained in accordance with SEC. 91.413 of this part shall operate the transponder, including Mode C equipment if installed, and shall reply on the appropriate code or as assigned by ATC.

2. 91.117  Aircraft speed.

(a)  Unless otherwise authorized by the Administrator, no person may operate an aircraft below 10,000' MSL at an indicated airspeed of more than

200 knots. FAR 91.117

Question

1. Maximum airspeed in an airport traffic area; below the base of a TCA.

Class B or C airspace

2. Discuss the various combinations of light signals,  green, red, white; 	steady or flashing. 91.125

3. What is the proper procedure for setting altimeter below 18,000'?

Answer

1. 	(b) Unless otherwise authorized or required by ATC, no person may operate an aircraft within an airport traffic area at an indicated airspeed of more than 200 knots.

(c)  No person may operate an aircraft in the airspace underlying a terminal control area, or in a VFR corridor designated through a terminal control area, at an indicated airspeed of more than 200 knots.

15. 91.205  Powered civil aircraft with standard category U. S. airworthiness certificates:  Instrument and equipment requirements.

(c)  Visual flight rules (night).  For VFR flight at night, the 			following instruments and equipment are required:

(2) Approved position lights.

(3) An approved aviation red or aviation white anticollision 			light system...

(4) If the aircraft is operate for hire, one electric landing 			light.

(5) An adequate source of electrical energy....

(6) One spare set of fuses, or three spare fuses of each kind 			required, that are accessible to the pilot in flight.

FAR 91.209  Aircraft lights

No person may, during the period from sunset to sunrise...

(a) Operate an aircraft unless it has lighted position lights;

(b) Park or move an aircraft in, or in dangerous proximity to, a night 	flight operations area of an airport unless the aircraft -

(1) Is clearly illuminated;

(2) Has lighted position lights; or

(3) Is in a area which is marked by obstruction lights.		(d) Operate an aircraft, required ...to be equipped with an anticollision light system...  However, the anticollision lights need not be lighted when the pilot in command determines that, because of operating conditions, it would be in the interest of safety to turn the lights off.

2. 91.125 ATC light signals.

Steady green Cleared for takeoff/land   Flashing cleared to taxi/return

Steady red Stop/give way to aircraft    Flashing taxi clear/don't land

Alternating red/green caution/caution   Flashing white return to starting

3.  91.121 Altimeter settings

(a) Each person operating an aircraft shall maintain the cruising altitude or flight level of that aircraft, as the case may be, by reference to an altimeter that is set, when operating -

(1)  Below 18,000' MSL, to -

(i)  The current reported altimeter setting of a station 				along the route and within 100 nautical miles of the 				aircraft.;

(ii)  If there is not station within the area prescribed 				in paragraph (a)(1) of this section, the current 				reported altimeter setting of an appropriate available station; or

(iii) In the case of an aircraft not equipped with a 					radio, the elevation of the departure airport or an 				appropriate altimeter setting available before departure; ...

Question

1. When are position lights required______?  What other equipment is 	required for night flight  FAR 91.205C; 91.209

2.  What are the minimum safe altitudes over congested area? _____ above and ______ horizontally.  Other than congested _______above.  Sparsely 	populated_______.  FAR 91.119

Answer

1. 91.205  Powered civil aircraft with standard category U. S. airworthiness certificates:  Instrument and equipment requirements.

(c)  Visual flight rules (night).  For VFR flight at night, the following instruments and equipment are required:

(2) Approved position lights.

(3) An approved aviation red or aviation white anticollision 			light system...

(4) If the aircraft is operate for hire, one electric landing 			light.

(5) An adequate source of electrical energy....

(6) One spare set of fuses, or three spare fuses of each kind 			required, that are accessible to the pilot in flight.

FAR 91.209  Aircraft lights

No person may, during the period from sunset to sunrise...

(a) Operate an aircraft unless it has lighted position lights;

(b) Park or move an aircraft in, or in dangerous proximity to, a night flight operations area of an airport unless the aircraft -

(10 Is clearly illuminated;

(2) Has lighted position lights; or

(3) Is in a area which is marked by obstruction lights.		(d) Operate an aircraft, required ...to be equipped with an anticollision light system...  However, the anticollision lights need not be lighted when the pilot in command determines that, because of operating conditions, it would be in the interest of safety to turn the lights off.

2. 91.119 Minimum safe altitudes: General.

Except when necessary for takeoff or landing, no person may operate an 	aircraft below the following altitudes:

(a) Anywhere.  An altitude allowing, if a power unit fails, an 			emergency landing without undue hazard to persons or property on 		the surface.

(b) Over congested areas.  Over any congested area of a city, town, or 		settlement, or over any open air assembly of persons, an 			altitude of 1,000' above the highest obstacle within a 			horizontal radius of 2,000' of the aircraft.

(c) Over other than congested areas.  An altitude of 500' above the 		surface, except over open water or sparsely populated areas.  In 		those cases, the aircraft may not be operated closer than 500' 		to any person, vessel, vehicle, or structure.

Question

1. FAR 91.133 How can you obtain a clearance into a prohibited or restricted 	area?

2. Far 91.159 Mandatory VFR cardinal altitudes begin at what AGL altitude?  	What are the appropriate VFR altitudes above the foregoing altitude?

3. What are the basic VFR minimums above 10,000'  FAR 91.155

4. Procedure for requesting VFR flight following from ATC.

5. How are VFR flight plans filed? Opened? Closed?

Answer

1.  91.133  Restricted and prohibited areas.

(a) No person may operate an aircraft within a restricted area (designated in Part 73) contrary to the restrictions imposed, or within a prohibited area, unless that person has the permission of the using or controlling agency, as appropriate....

2.  91.159 VFR cruising altitude or flight level.  Except while holding in a holding pattern of 2 minutes or less, or while turning, each person operating an aircraft under VFR in level cruising flight more than 3,000' above the surface shall maintain the appropriate altitude or flight level prescribed below, unless otherwise authorized by ATC:

(a) when operating below 18,000' MSL and -

(1) On a magnetic course of 0 degrees through 179 degrees, any 		odd thousand foot MSL altitude + 500'. (such as 3,500,5,500,

(2) On a magnetic course of 180 degrees through 359 degrees, any 		even thousand MSL altitude + 500' (such as 4,500, 6,500, 8,500).

3.  91.155 Basic VFR weather minimums.

(a) Except as provided in Sections. 91.155(b0 and 91.157, no person 		may operate an aircraft under VFR when the flight visibility is 		less, or at a distance from clouds that is less, than that 			prescribed for the corresponding altitude in the following 			table:

More than 1,200' above the surface and at or above 10,000' MSL

Visibility               Distance from clouds

5 statute miles          1,000' above	7,500)

1,000' below

1 mile horizontal

4. AIM Chapter 4

5. AIM Chapter 5  FARs

MUST BE READ IN ENTIRETY

Question

1. FAR 91.157, AIM Chapter 3 Section 2  What are the dimensions of Class D airspace?  How is it depicted on a sectional?  When must a clearance be given for operations in Class D airspace?

Answer

1. 91.157  Special VFR weather minimums.

(a) Except as provided in Sec. 93.113k, when a person has received an 		appropriate ATC clearance, the special weather minimums of this 		section instead of these contained in Sec. 91.155 apply to the 		operation of that aircraft by that person in a control zone 		under VFR.

(b) No person may operate an aircraft in Class D airspace under VFR 		except clear of clouds.

(c) No person may operate an aircraft...in Class D airspace under VFR 		unless flight visibility is at least 1 statute mile.

(d) No person may take off or land an aircraft...at any airport in 		Class D airspace under VFR -

(1) Unless ground visibility at that airport is at least 1 				statute mile; or

(2) If ground visibility is not reported at that airport, unless 			flight visibility during landing or takeoff is at least 1 			statute mile.

(e) No person may operate an aircraft... in Class D airspace under the 		special weather minimums of this section, between sunrise and 		sunset....unless:

(1) That person meets the applicable requirements for instrument 			flight under Part 61 of this chapter; and

(2) the aircraft is equipped as required in Sec. 91.205(d)

Class D Airspace

a. Class D airspaces are regulatory in nature and established as controlled airspace. They extend upward from the surface and terminate at 2500' AGL.  Class D airspace is based on a primary airport but may include one or more airports and is normally a circular area within a radius of 4.1 nautical miles around the primary airport, except that it may include extensions necessary to include instrument departure and arrival paths.

b. Some basic requirement for designating Class D airspace  is communications and weather observation reporting:

1. Communications capability with aircraft which normally operate within the Class D airspace must exist down to the runway surface of the primary airport.  Communications may be either direct from the ATC facility having jurisdiction over the control zone or by rapid relay through other communications facilities which are acceptable to that ATC facility.

2. Federally certificated weather observers take hourly and special weather observations at the primary airport in the control zone during the times Class D is designated.  The required weather observations must be forwarded expeditiously to the ATC facility having jurisdiction over the control zones.

c. Class D airspace is depicted on charts (for example on the Sectional Charts the Class D airspace is outlined by a broken blue line).  If Class D airspace is effective only during certain hours of the day (a part-time Class D airspace as prescribed in the regulation) it will be reflected on the charts.   AIM 3-2; and 4-84

Question

1. What are the common transponder codes?  AIM  FARs

Answer

1. AIM 1-2  AIR TRAFFIC CONTROL RADAR BEACON SYSTEM (ATCRBS)

1200 VFRl, 7700 emergency, 7600 no radio, 7500 hijack

a. the ATCRBS, sometimes referred to as secondary surveillance radar, consists of three main components:

1. Interrogator.  Primary radar relies on a signal transmitted from the radar antenna site and for this signal to be reflected or "bounced back" from an object (such as an aircraft).  this reflected signal is then displayed as a "target" on the controller's radarscope.  In the ATCRBS, the interrogator, a ground based radar beacon transmitter-receiver, scans in synchronism with the primary radar and transmits discrete radio signals which repetitiously request all transponders, on the mode being used, to reply.  The replies received are then mixed with the primary returns and both are displayed on the same radar scope.

2. Transponder.  This airborne radar beacon transmitter-receiver automatically receives the signals from the interrogator and selectively replies with a specific pulse group (code) only to those interrogations being received on the mode to which it is set.  These replies are independent of, and much stronger than a primary radar return.

3. Radarscope.  The radarscope used by the controller displays returns from both the primary radar system and the ATCRBS.  These returns, called targets, are what the controller refers to in the control and separation of traffic.

b. the job of identifying and maintaining identification of primary radar targets is a long and tedious task for the controller.  Some of the advantages of ATCRBS over primary radar area:

1. Reinforcement of radar targets

2. Rapid target identification

3. Unique display of selected codes.

c. a part of the ATCRBS ground equipment is the decoder.  This equipment enables the controller to assign discrete transponder codes to each aircraft under his control.  Normally only one code will be assigned for the entire flight.  Assignments are made by the ARTCC computer on the basis of the National Beacon Code Allocation Plan.  The equipment is also designed to receive MODE C altitude information from the aircraft.

AIM Chapter 4 is too long and should be read and studied from the AIM

Question

1. What are the basic VFR minimums between 1,200 and 10,000' MSL?  FAR 	91.155

2. What are the basic VFR minimums for flight within 1200' of the surface in 	controlled airspace?  Uncontrolled airspace day?  Night? FAR 91.155

3. Purpose for the following frequencies:

121.5, 122.0, 122.9, 122.8, 123.6, 122.2, 122.95, 122.75, 122.85

4.  Purpose of transponder codes?.

Answer

1. More than 1,200 feet above the surface but less than 10,000' MSL.

Visibility                Distance from clouds

Within controlled

airspace:		3 statute miles             500' below

1,000' above

2,000' horizontal

Outside controlled

airspace:         1 statute mile              500' below

1,000' above

2,000' horizontal

Anytime:

Night             3 statute miles             500' below

1,000' above

2,000' horizontal

2. 1200 feet or less above the surface-

Within controlled

airspace:          3 statute miles            500' below

1,000" above

2,000' horizontal

Outside controlled

airspace -   Day   1 statute mile           Clear of clouds

Night 3 statute miles             500' below

1,000' above

Exception                                    2,000' horizontal

Unless within 1/2 mile of airport and in pattern.

3.  AIM 4-8

121.5 Emergency frequency which is monitored by ATC and airlines.

122.0 Flight Watch for weather enroute

122.9 Multicom CTAF frequency at airports not having UNICOM.

122.8 One of several UNICOM CTAF frequencies where local FBO may 				answer.

123.6 FSS CTAF frequency to be used at FSS only airport for airport 			operations.

122.2 Almost universal FSS frequency to be tried if no more 				discrete frequency is unknown.

122.95 Universal UNICOM frequency at airports with ATC tower.

122.75 Aircraft to aircraft frequency

122.85 Aircraft to aircraft frequency(SPECIAL USE)

4. AIM Chapter 6

Question

1. The procedure for requesting a DF steer.

2. What is the lower limit of controlled airspace in the area not tinted?.  	What kinds of airspace is from the surface to 1200' AGL?

3. What is the upper limit of the controlled airspace depicted on the 	sectional?

4. How does a restricted area differ from a warning area?

Answer

1.  Establish contact with FSS.  Usually the FSS will refer an aircraft to a radar facility in an actual emergency.  A non-DF FSS has emergency capability to assist pilots by talking the pilot through the steps for a VOR fix or even a series of them as required.

If the aircraft is below/beyond radar coverage but in radio contact the DF may be the best option.  Rancho FSS near Sacramento has DF capability.  Indicate purpose for request of DF, practice, misplaced, finding airport, etc. Follow procedure instructions exactly.  Altitude is limiting factor for DF range.  Most common fault is failure to set heading indicator to correspond with compass.

Typical procedure:

1. FSS will assign discrete frequency and go through a list of questions covering, flight conditions, altitude, heading,  fuel on board, souls on board, last known position, VOR capability, etc. (It is essential that all assigned headings be flown precisely to insure accuracy.)

2.  Pilot will be told to key microphone for 5-10 seconds without speaking and then give aircraft identification. (The DF (direction finder)  will give FSS line from station to aircraft.)

OPTIONS:

3a. Pilot will be given heading to fly down the "beam" to the FSS station and airport.

3b. Pilot will be given heading to fly at right angles to "beam" and told to fly that heading(a given number of seconds) until told to key his radio again.  Using this information the FSS can easily compute how long it will take the pilot to fly to the station.

3c. The FSS will give directions as to selection of VOR frequency with nearest to 90 degree intersection. Pilot will be told to set frequency, ident, center needle FROM, and give radial from VOR.  FSS will draw intersection of "beam" and VOR radial for aircraft location.  Heading will be suggested and repeated "beam"/VOR readings can be used to determine track.  Aircraft can be directed by FSS to any location. FSS personnel are required to be proficient in these procedures and appreciate the opportunity to practice.

2.  All airspace that is not bordered by magenta shaded line makes up an 	instrument enroute transition area.  Transition areas are designed to 	contain IFR operations in controlled airspace.

3. Sectional legend.  18,000'

4. Warning areas are in international waters so only U. S. aircraft are

subject to flight restrictions

Question

1. In what instances does controlled airspace start at ground level.

2.  How is a airspace zone that does not allow Special VFR operations 	depicted on the Sectional?

3. What is Class A Airspace?

Answer

1. Class D Airspace

a. Class D airspace is regulatory in nature and established as controlled airspace.  They extend upward from the surface and terminate 2500' AGL.

Class B Airspace

a. Class B Airspace consists of controlled airspace extending upward from the surface or higher to specified altitudes, within which all aircraft are subject to operating rules and pilot/equipment requirements specified in FAR 91....

Class C Airspace

a. Class C airspace consists of controlled airspace extending upward from the surface or higher to specified altitudes, within which all aircraft are subject to operating rules and pilot and equipment requirements specified in FAR 91.

SPECIAL USE AIRSPACE

AIM 3-31 PROHIBITED AREA

Prohibited Areas contain airspace of defined dimensions identified by an area on the surface of the earth within which the flight of aircraft is prohibited...

AIM 3-32 RESTRICTED AREA

a. Restricted Areas contain airspace identified by an area on the surface of the earth within which the flight of aircraft, while not wholly prohibited, is subject to restrictions....

AIM 3-33 WARNING AREA

Warning Areas are airspace which may contain hazards to non-participating aircraft in international airspace.....

AIM  3-35 ALERT AREA

Alert Areas are depicted on aeronautical charts to inform non-participating pilots of areas that may contain a high volume of pilot training or an unusual type of aerial activity....

2. AIM 4-85 e. Special VFR operations by fixed-wing aircraft are prohibited in some Class B airspace due to the volume of IFR traffic.  A list of these control zones is contained FAR 93.113.  They are also depicted on Sectional Aeronautical Charts with a note indicating No SVFR.   San Francisco is such a control zone.  Helicopters are an exception.

3. Class A Airspace

Positive Control Area is airspace so designated as positive control 	area in FAR 71.193.  This area includes specified airspace within the coterminous U.S. from 18,000 feet to and including FL 600....

Question

1. What pilot and aircraft capabilities are required for night special VFR 	operations?

2. How is Class D airspace depicted on a sectional.

3. How is an airport that offers airport advisory service (not UNICOM) 	depicted on the sectional?

4. If controlled airspace starts at a level other than (1) ground, (2)

700' AGL, or (3) 1200' AGL how is the lower limit depicted on a 	sectional chart?

Answer

1.  FAR 91.157e No person may operate an aircraft in Class C or D airspace under the special weather minimums of this section, between sunset and sunrise...unless:

(1) That person meets the applicable requirements for instrument flight under Part 61 of this chapter; and

(2) The aircraft is equipped as required in Sec. 91.205(d)

AIM 4-85 g. Special VFR operations by fixed-wing aircraft are prohibited between sunset and sunrise unless the pilot is instrument rated (current) and the aircraft is equipped for IFR flight.(Simply: IFR pilot and aircraft)

2.  AIM Chapter 3  ...Class D airspace is indicated on aeronautical charts by the blue airport symbol with a dashed circle plus extensions.

AIM 3-2 shows 4.1 nautical mile radius circle up to but not including 2500'.  See also AIM 4-52

3. AIM AIRPORT ADVISORY AREA

a. the airport advisory area is the area within 10 statute miles of an airport where a control tower is not operating but where a FSS is located.  At such locations, the FSS provides advisory service to arriving and departing aircraft.  See AIM 4-8

b. It is not mandatory that pilots participate in the Airport Advisory Service program, but it is strongly recommended that they do. AIM COMMUNICATIONS FOR VFR FLIGHTS

a. FSS's are allocated frequencies for different functions; for example, 122.0 MHz is assigned as En Route Flight Advisory Service (Flight Watch) frequency at selected FSS's.  In addition, certain FSS’s provide Airport Advisory Service on 123.6 MHz.  Other FSS frequencies are listed in the Airport/Facility Directory.  If you are in doubt as to what frequency to use, 122.2 MHz is assigned to the majority of FSS's as a common enroute simplex frequency....

b. Certain VOR voice channels are being utilized for recorded broadcasts; i.e., ATIS, HIWAS, etc.  These services and appropriate frequencies are listed in the Airport/Facility Directory.  On VFR flights, pilots are urged to monitor these frequencies.  When in contact with a control facility, notify the controller if you plan to leave the frequency to monitor these broadcasts.

4.  Refer to latest San Francisco Sectional.  Representative areas north of Ukiah as shown with overlapping blocked blue lines.  Altitudes are along block borders or inside areas.  Extends along the Pacific Coast line at 5000'.   This negates the necessity of following the terrain at 1200'. and allows level daytime flight in these areas below the named altitudes with minimums of 1 mile and clear of clouds.  IFR and MOA operations will be above this level.

Question

1.  What is the significance of daytime operation of an airport beacon?

2. Define and give actual V airspeeds for the aircraft you are using.

Define and give actual V airspeeds for the aircraft you are using.

3.  What is the lower limit of controlled airspace in a transition area.

What is the purpose of a transition area? (No longer mentioned in AIM)

4.  Discuss the color coding on the airspeed indicator.

Answer

1.  AIM Chapter 2 AIRPORT (ROTATING) BEACONS

d. In Class D airspace, operations of the airport beacon during the hours of daylight often indicates that the ground visibility is less than 3 miles and/or the ceiling is less than 1,000 feet.  ATC clearance in accordance with FAR 91 (SVFR) is required for landing, takeoff and flight in the traffic pattern.

Pilots should not rely solely on the operation of the airport beacon to indicate if weather conditions are IFR or VFR.  At some locations with operating control towers, ATC personnel turn the beacon on or off when controls are in the tower.  At many airports the airport beacon is turned on by a photoelectric cell or timer and ATC personnel cannot control them.

There is no regulatory requirement for daylight operation and it is the pilot's responsibility to comply with proper preflight planning as required by FAR 91.103.  (No student SVFR)

2. Vso = the stalling speed or the minimum steady flight speed in the landing configuration; 35-40 kts.

Vs = the stalling speed or the minimum steady flight speed at which airplane is controllable; 40-50 kts.

Vx = speed for best angle of climb; 60 kts.  This is the speed that gets you over the 50' FAA tree.  Best altitude over distance.

Vy = speed for best rate of climb; 68 kts.  this is the speed that gets you highest in a given amount of time.

Vno = means maximum structural cruising speed.  This is where the green and orange arcs of the airspeed indicator meet.

Va = design maneuvering speed; 95 kts.   This is the speed below which the aircraft is "guaranteed" not to fold, spindle, or mutilate with abrupt control movements.

Vne = never exceed speed.  This is the red line speed of the airspeed indicator.

3. TRANSITION AREAS

a. Transition Areas are designated to contain IFR operations in controlled airspace. above uncontrolled airspace,  during portions of the terminal IFR operation and while transitioning between the terminal and enroute environment.

b. Transition Areas are controlled airspace extending upward from 700 feet or more above the surface when designated in conjunction with an airport for which an instrument approach procedure has been prescribed (magenta shaded borders; or from 1,200 feet or more above the surface when designated in conjunction with airway route structures or segments (rarely indicate by blue borders any longer).  Unless specified otherwise, Transition Areas terminate at the base of the overlying controlled airspace.

4. White arc - Normal flap operation speed range.

Yellow arc - Speed range where in turbulence reduction is required

Red line - Never exceed speed

Question

1. Above what altitude is VFR flight prohibited?  What is required above this altitude.

Answer

1.  91.135 Positive control areas and route segments. (Above 18,000')

(a) Except as provided in paragraph (b) of this section, no person may operate an aircraft within a positive control area or a positive control route segment designated in part 71 of this chapter unless the aircraft is -

(1) Operated under IFR at a specific flight level assigned by 	ATC;

(2) Equipped with instruments and equipment required for IFR operations;

91.205 (b)Visual flight rules (day).  For VFR flight during the 			day, the following instruments and equipment are required:

(1) Airspeed indicator.

(2) Altimeter

(3) Magnetic direction indicator

(4) Tachometer for each engine.

(5) Oil pressure gauge for each engine...

(7) Oil temperature gauge for each air cooled engine.

(8) Manifold pressure gauge for each altitude engine.

(9) Fuel gauge indicating the quantity of fuel in each tank.

(10) Landing gear position indicator...

(12) ...an approved safety belt with approved metal-to metal- 				latching device for each occupant 2 years of age or older.

(13)...an approved shoulder harness for each front seat....

(14) An emergency locator transmitter, if required by .91.207

(c) Visual flight rules (night).  floor VFR flight at night, the following 			instruments and equipment are required:

(1) Instruments and equipment specified in paragraph (b) of this section

(2) Approved position lights

(3) An approved aviation red or aviation white anticollision lights

(5) An adequate source of electrical energy for all installed 				electrical and radio equipment

(6) One spare set of fuses, or three spare fuses of each kind 				required, that are accessible to the pilot in flight.

(d) Instrument flight rules.  For IFR flight, the following 					instruments and equipment are required:

(1) Instruments and equipment specified in paragraph (b) of this 				section, and, for night flight, instruments and equipment 				specified in paragraph (c) of this section.

(2) Two-way radio communications system and navigation equipment 				appropriate to the ground facilities to be used.

(3) Gyroscopic rate of turn indicator...

(4) Slip/skid indicator

(5) Sensitive altimeter adjustable for barometric pressure.

(6) A clock displaying hours, minutes, and seconds...

(7) Generator or alternator of adequate capacity

(8) Gyroscopic pitch and bank indicator...

(9) Gyroscopic direction indicator...

(e) Flight at and above 24,000' MSL....equipped with approved DME..

(3) Flown by a pilot rated for instrument flight; and

(4) Equipped, when in a positive control area, with -

(i) The applicable equipment specified in Sec. 91.215

( 91.215 ATC transponder and altitude reporting equipment 		      and use.)

(b) (1) All aircraft.  In Class A and Class B airspace...

(ii) A radio providing direct pilot/controller communication on 			the frequency specified by ATC for the area concerned....

(b) ...ATC may authorize deviation on a continuing basis or for an 			individual flight, as appropriate.

Question

1. What items of information must preflight include as listed in FAR 91.103,

Answer

1. 91.103 Each pilot in command shall, before beginning a flight, become familiar with all available information concerning that flight.  This information must include -

(a) ...a flight not in the vicinity of an airport, weather reports and forecasts, fuel requirements, alternatives available if the planned flight cannot be competed, and any known traffic delays of which the pilot in command has been advised by ATC;

(b)  For any flight, runway lengths at airports of intended use, and the following takeoff and landing distance information:

(1) For civil aircraft for which an approved airplane...Flight Manual containing takeoff and landing distance data is requires, the takeoff and landing distance data contained therein; and

(2) For civil aircraft other than those specified in paragraph (b) (1) of this section, other reliable information appropriate to the aircraft, relating to aircraft performance under expected values of airport elevation and runway slope, aircraft gross weight, and wind and temperature.

AIM 5-1-2,  FOLLOW IFR PROCEDURES EVEN WHEN OPERATING VFR

a. to maintain IFR proficiency, pilots are urged to practice IFR procedures whenever possible, even when operating VFR.  Some suggested practices include:

1. Obtain a complete preflight and weather briefing.  Check the NOTAMS.

2, File a flight plan.  This is an excellent low cost insurance policy.  The cost is the time it takes to fill it out.  The insurance includes the knowledge that someone will be looking for you if  you become overdue at your destination.

3. Use current charts.

4. Use the navigation aids.  Practice maintaining a good course-keep the needle centered.

5. Maintain a constant altitude which is appropriate for the direction of flight.

6. Estimate enroute position times.

7. Make accurate and frequent position reports to the FSS's along your route of flight...

AIM 7-1-6. CATEGORICAL OUTLOOKS

a. Categorical outlook terms describing general ceiling and visibility conditions for advanced planning purposes, are defined as follows:

1. LIFR (Low IFR--Ceiling less than 500 feet and/or visibility less than a mile.

2. IFR--Ceiling 500 to less than 1,000 feet and/or visibility 1 to less than 3 miles.

3. MVFR (Marginal VFR)--Ceiling 1,000 to 3,000 feet and/or visibility 3 to 5 miles inclusive.

4. VFR--Ceiling greater than 3,000 feet and visibility greater than 5 miles; includes sky clear.

b. the cause of LIFR, IFR, or MVFR is indicated by either ceiling or visibility restrictions or both.  the contraction "CIG" and/or weather and obstructions to vision symbols are used.  If winds or gusts of 25 knots or greater are forecast for the outlook period, the word "WIND" is also includes for all categories including VFR.

Question

1. What is the normal traffic direction at uncontrolled airports?  What indicators are used to denote an irregular traffic direction? 91.127 AIM

Answer (two pages)

1. Far 91.127;   Operating on or in the vicinity of an airport: General rules

(a) Unless otherwise required by part 93 of this chapter, each person operating an aircraft on or in the vicinity of an airport shall comply with the requirements of this section and, if applicable, of Sec. 91.129

(b) Each person operating an aircraft to or from an airport without an operating control tower shall -

(1) In the case of an airplane approaching to land, make all turns of that airplane to the left unless the airport displays approved light signals or visual markings indicating that turns should be made to the right, in which case the pilot shall make all turns to the right;

TRAFFIC ADVISORY PRACTICES AT AIRPORTS WITHOUT OPERATING CONTROL TOWERS

a. Airport Operations Without Operating Control Tower

1. There is no substitute for alertness while in the vicinity of an airport.  It is essential that pilots be alert and look for other traffic and exchange traffic information when approaching or departing an airport without an operating control tower.  This is of particular importance since other aircraft may not have communication capability.

b. Communicating on a Common Frequency.

1. The key to communicating at an airport without an operating control tower is selection of the correct common frequency.  the acronym CTAF which stands for Common Traffic Advisory Frequency, is synonymous with this program.  A CTAF is a frequency designated for the purpose of carrying out airport advisory practices while operating to or from an airport without an operating control tower.  The CTAF may be a UNICOM, MULTICOM, FSS, or tower frequency and is identified in appropriate aeronautical publications.

2. The CTAF frequency for a particular airport is contained in the Airport/Facility Directory (A/FD etc...)Also, the CTAF frequency can be obtained by contacting any FSS.  Use the appropriate CTAF, combined with a visual alertness and application of recommended good operating practices, will enhance safety of flight into and out of all uncontrolled airports.

c. Recommended Traffic Advisory Practices.

1. Pilots of inbound traffic should monitor and communicate as appropriate on the designated CTAF from 10 miles to landing.  Pilots of departing aircraft should monitor/communicate on the appropriate frequency from start-up, during taxi, and until 10 miles from the airport unless the FARs or local procedures require otherwise.

2. Pilots of aircraft conducting other than arriving or departing operations at altitudes normally used by arriving and departing aircraft should monitory/communicate on the appropriate frequency while within 10 miles of the airport unless required to do otherwise by the FAR's or local procedures.  Such operations include parachute jumping/dropping, etc.

d. Airport Advisory Service Provided by an FSS.

1. Airport Advisory Service (AAS) is a service provided by an FSS physically located on an airport which does not have a control tower or where the tower is operated on a part-time basis.  The CTAF for FSSs which provide this service will be disseminated in appropriate aeronautical publications.  (123.6 at FSS only fields; usually tower frequency at FSS/tower fields)

AIM 4-52 VISUAL INDICATORS AT UNCONTROLLED AIRPORTS

a. At those airports without an operating control tower, a segmented circle visual indicator system, if installed, is designed to provide traffic pattern information.  The segmented circle system consists of the following components:

1. The segmented circle--Located in a position affording maximum visibility to pilots in the air and on the ground and providing a centralized location for other elements of the system.

2. The wind direction indicator--A wind cone, wind sock, or wind tee installed near the operational runway to indicate wind direction,  The large end of the wind cone/wind sock points into the wind as does the large end(cross bar) of the wind tee.  In lieu of a tetrahedron and where a wind sock or wind cone is collocated with a wind tee, the wind tee may be manually aligned with the runway in use to indicate landing direction.  These signaling devices may be located in the center of the segmented circle and may be lighted for night use.  Pilots are cautioned against using a tetrahedron to indicate wind direction.

3. The landing direction indicator--A tetrahedron is installed when conditions at the airport warrant its use.  It may be used to indicate the direction of landings and takeoffs.  A Tetrahedron may be located at the center of a segmented circle and may be lighted for night operations.  The small end of the tetrahedron points in the direction of landing.  Pilots are cautioned against using the tetrahedron for any purpose other than as an indicator of landing direction.  Further, pilots should use extreme caution when making runway selection by use of a tetrahedron in very light or calm wind conditions as the tetrahedron may not be aligned with the designated calm-wind runway.  At airports with control towers, the tetrahedron is should only be referenced with the control tower is not in operation.  Tower instructions supersede tetrahedron indications.

4. Landing strip indicators--Installed in pairs...and used to show the alignment of landing strips (runways)

5. Traffic pattern indicators--Arranged in pairs in conjunction with landing strip indicators and used to indicate the direction of turns when there is a variation from the normal left traffic pattern.  (If there is not segmented circle installed at the airport, traffic pattern indicators may be installed on or near the end of the runway.)

b. Preparatory to landing at an airport without a control tower, or when the control tower is not in operation, the pilot should concern himself with the indicator for the approach end of the runway to be used.  When approaching for landing, all turns must be made to the left unless a traffic pattern indicator indicates that turns should be made to the right.  If the pilot will mentally enlarge the indicator for the runway to be used, the base and final approach legs of the traffic pattern to be flown immediately become apparent.  Similar treatment of the indicator at the departure end of the runway will clearly indicate the direction of turn after takeoff.

c. when two or more aircraft are approaching an airport for the purpose of landing, the aircraft at the lower altitude has the right of way, but it shall not take advantage of this rule to cut in front of another which is on final approach to land, or to overtake that aircraft (FAR 91.113(f)).

4-53 TRAFFIC PATTERNS

At most airports and military air bases, traffic pattern altitudes for propeller-driven aircraft generally extend from 600 feet to as high as 1,500 feet above the ground.... Therefore, pilots of enroute aircraft should be constantly on the alert for other aircraft in the traffic patterns and avoid these areas whenever possible.  Traffic pattern altitudes should be maintained unless otherwise required by the applicable distance from cloud criteria (FAR 91.155)

Right traffic indicators

At night or any other time, an orange light on top of the wind sock.

The white traffic pattern indicators at right angles to the landing 		strip indicators show that a right turn is required. Traffic pattern 	indicators may be installed on or near the end of the runway.

Question

1.  Discuss the structure of a typical Class B Airspace. Use VFR Area chart; AIM Chapter 3

Answer (two pages)

1. AIM 3-2-2. Class B Airspace

a. Class B airspace  consists of controlled airspace extending upward from the surface or higher to 10,000, within which all aircraft are subject to the operating rules and pilot/equipment requirements specified in FAR 91.  Each location includes at least one primary airport around with the Class B airspace  is located.  Descriptions of Class B airspace can be found in FAR 71

b. Class B airspace is charted on Sectional, World Aeronautical, Enroute low Altitude, DOD FLIPS and Terminal Area Charts.

AIM 3-2-2. Class B OPERATION

a. Operating Rules and Pilot/Equipment Requirements.

REGARDLESS OF WEATHER CONDITIONS, AN ATC AUTHORIZATION IS REQUIRED PRIOR TO OPERATING WITHIN CLASS B AIRSPACE.  Pilots should not request an authorization to operate within a Class B unless the requirements of FAR 91.215 and Far 91. 131 are met.  Included among these requirements are:

1. Unless otherwise authorized by ATC, aircraft must be equipped with an 		operable two-way radio capable of communication with ATC on 			frequencies for that terminal control area.

2. No person may takeoff or land a civil aircraft at an airport within a 		Class B airspace or operate within Class B airspace unless:

(a) pilot-in-command holds at least a private pilot certificate;

(b) aircraft is operated by a student pilot who has met the 				requirements of FAR 61.95 (endorsement requirement);

(c) the following primary airports, no person may takeoff or land 			a civil aircraft unless the pilot-in-command holds at least 			a private pilot certificate. SAN FRANCISCO ...

4. Each aircraft must be equipped as follows:

(a) two-way radio capable of communications with ATC on 					appropriate frequencies for that area.

(b) IFR operations, an operable VOR or TACAN receiver.

(c) Unless otherwise authorized by ATC, an operable radar beacon 				transponder with automatic altitude reporting equipment.

(b) Flight Procedures.

1. IFR Flights. ...

2. VFR Flights must remain clear of clouds and 3 mile visibility.

(a) ARRIVING AIRCRAFT MUST OBTAIN AUTHORIZATION PRIOR TO ENTERING  A Class B AND MUST CONTACT ATC ON THE APPROPRIATE FREQUENCY, and in relation to geographical fixes showing on local charts.  Although a pilot may be operating beneath the floor of the Class B airspace on initial contact, communications, with ATC should be established in relation to the points indicated for spacing and sequencing purposes.

(b) Departing aircraft require a clearance to depart the Class B airspace and should advise the clearance delivery position of their intended altitude and route of flight.  ATC will normally advise VFR aircraft when leaving the geographical limits of the Class B.  Radar service is not automatically terminated unless specifically stated by the controller.

(c) Aircraft not landing or departing the primary airport may obtain ATC clearance to transit the Class B when traffic conditions permit and provides the requirements of FAR 91.1131 are met.  Such VFR aircraft are encouraged, to the extent possible, to operate at altitudes above or below the TCA or transit through established VFR corridors,.  Pilots operating in VFR corridors should use frequency 122.75 mHz for the exchange of information.

(d) VFR aircraft not cleared into the Class B airspace are cautioned against operating too closely to Class B boundaries, especially where the floor of the Class B is 3,000 feet or less or where VFR cruise altitudes are at or near the floor of higher levels.  Observance of this precaution will reduce the potential for encountering an aircraft operating at Class B floor altitudes.  Additionally, VFR aircraft are encouraged to utilize the VFR Planning Chart as a tool for planning flight in proximity to Class B airspace.  Charted VFR Flyway Planning charts are published on the back of the existing VFR Terminal Area Charts.

c. ATC Clearances and Separation

AN ATC AUTHORIZATION IS REQUIRED TO ENTER AND OPERATE WITHIN CLASS B AIRSPACE. VFR pilots are provided sequencing and separation from other aircraft while operating within Class B airspace.  (SEE AIM 4-16)

1. VFR aircraft are separated from all VFR/IFR aircraft which weight 19,000 pounds or less by a minimum of:

(a) Target resolution, or

(b) 500 feet vertical separation, or

(c) Visual separation.

2.  VFR aircraft are separated from all VFR/IFR aircraft which weigh more than 42,000 pounds and turbojets by no less than:

(a) 1 and 1/2 miles lateral separation, or

(b) 500 feet vertical separation, or

(c) Visual separation.

3. This program is not to be interpreted as relieving pilots of their responsibilities to see and avoid other traffic operating in basic VFR weather conditions, to adjust their operations and flight path as necessary to preclude serious wake encounters, to maintain appropriate terrain and obstruction clearances, or to remain in weather conditions equal to or better than the minimums requires by FAR 91.155.  Approach control should be advised and a revised clearance or instruction obtained when compliance with an assigned route, heading and/or altitude is likely to compromise pilot responsibility with respect to terrain and obstruction clearance, vortex exposure, and weather minimums.

4. ATC may assign altitudes to VFR aircraft that do not conform to FAR 91.159.  "Resume Appropriate VFR Altitudes" will be broadcast when the altitude assignment is no longer needed for separation or when leaving the Class B airspace.  Pilots must return to an altitude that conforms to FAR 91.159 as soon as practicable.

intentionally left blank

continue on...

B’s Question

1. Define Class D Airspace as it exists around an airport with tower operating and closed. AIM 3-2-5; FAR 91.127, 91.129  MORE

Answer

1. Standard Dimensions	____radius, up to but not including ____

FAR 91.127, 91.129  AIM 3-2-5, 91.127  Operating on or in the vicinity of an airport: General rules

(a) Unless otherwise required by part 93 of this chapter, each person operating an aircraft on or in the vicinity of an airport shall comply with the requirements of this section and, if applicable, of Sec. 91.129

(b) Each person operating an aircraft to or from an airport without an operating control tower shall -

(1) In the case of an airplane approaching to land, make all turns of that airplane to the left unless the airport displays approved light signals or visual markings indicating that turns should be made to the right, in which case the pilot shall make all turns to the right;

(3) In the case of an aircraft departing the airport, comply with any traffic patterns established for that airport in part 93.

(c) Unless otherwise authorized or required by ATC, no person may operate an aircraft within an airport traffic area except for the purpose of landing at, or taking off from, an airport within that area....

91.129  Operation at airports with operating control towers

(a) General.  Unless otherwise authorized or required by ATC, each person operating an aircraft to, from, or on an airport with an operating control tower shall comply with the applicable provisions of this section

(b) Communications with control towers operated by the United States.

No person may, within an airport traffic area operate an aircraft to, from, or on an airport having a control tower operated by the United States unless two-way radio communications are maintained between that aircraft and the control tower.  However, if the aircraft radio fails in flight, the pilot in command may operate that aircraft and land if weather conditions are at or above basic VFR weather minimums, visual contact with the tower is maintained, and a clearance to land is received....

(c) Communications with other control towers.  No person may, within an airport traffic area, operate an aircraft to, from, or on an airport having a control tower that is operated by any person other than the United States unless - (Example:  North Las Vegas has NFCT)

(1) If that aircraft's radio equipment so allows, two-way radio communications are maintained between the aircraft and the tower; or

(2) If that aircraft's radio equipment allows only reception from the tower, the pilot has the tower's frequency monitored.

(d) Minimum altitudes.  when operating to an airport with an operating control tower, each pilot of -

(3) an airplane approaching to land on a runway served by a visual approach indicator shall maintain an altitude at or above the glide slope until a lower altitude is necessary for a safe landing....

(e) Approaches.  When approaching to land at an airport with an operating control tower, each pilot of -

(1) an airplane shall circle to the left; and ...

(f) Departures.  No person may operate an aircraft taking off from an airport with an operating control tower expect in compliance with the following:

(1) Each pilot shall comply with any departure procedures established for that airport by the FAA.

(h) Clearances required.  No person may, at an airport with an operating control tower, operate an aircraft on a runway or taxiway, or take off or land an aircraft, unless an appropriate clearance is received from ATC.  A clearance to "taxi to" the takeoff runway assigned to the aircraft is not a clearance to cross that assigned takeoff runway or to taxi on that runway at any point, but is a clearance to cross other runways that intersect the taxi route to that assigned takeoff runway.  A clearance to "taxi to" any point other than an assigned takeoff runway is a clearance to cross all runways that intersect the taxi route to that point.

(X) Readback required.  Ground control is required to obtain a complete readback from the pilot of any taxi clearances which include the word "hold".

Class D Airspace

C. FAR 91.129 requires that unless otherwise authorized by ATC, a pilot operating to or from an airport served by an operating control tower must maintain two-way radio communications with the control tower while in the airport Class D surface area which includes the movement areas of that airport.  FAR 91.117 sets the maximum indicated airspeed for operations in an airport traffic area at 200 knots.

Questions for discussion with instructor

1. How are License/medical expiration dates determined.

2. How do you use of VOR to intercept a course

3. How do you use the following Airspace?

Visibility/cloud clearances in controlled airspace,

Visibility/cloud clearances in uncontrolled airspace,

SVFR and in Class Delta airspace

4. What is the procedure for NTSB notification for incidents/accidents

5. How do you plan a cross-country flight?

How to flight plan the navigation?

How to get and plan around weather?

How to get a weather briefing?

How to compute altitudes/fuel/ weight & balance

6. What are the procedures for operating in, over and under Class C airspace

7. How do you compute TAS/ETA/GS while flying

8. How will you plot a NEW course GS/ETA if diverted to another airport

9. Do you know definitions of IAS/CAS/TAS and how to find or calculate?

10.  How do you interpret segmented circle and pattern arms at uncontrolled

airports?

11. How do you perform slow flight and stalls under the hood?

12. How do you spiral down over point from altitude holding constant airspeed, 	turn radius, with 40 degree maximum bank and completing exercise in 	position for emergency landing.

13. How do you make "Constant altitude turns" at 45 degree bank.

14. What is the ‘magic’ of 1500 AGL while performing stalls?.

15. Can you use only 30 minutes to plan cross country to maximum range of

aircraft?

16. What are the ways to file a flight plan?

17. How do you reverse course during ground reference without getting out of 	position?

18. How do you used your checklist in an emergency with demonstrated forced 	landing?

19. Do you use an after takeoff checklist?

20. Did you know that at one time any turn to final may not be less than 1/4 	mile from runway?

21. Can you make a normal touchdown beyond but within 400' of point selected?

Short field within 200'?

22. Do you use of aircraft manual.  See Manual.