Nightã

Night flight checklist

Flying at Night Is Not

Physiology of night

Night sight skills

Night x-country

Night Taxiing

FAR Night Flight Requirements (instructor)

Night Takeoff

Night landings

Night Instruction

Night Lights

Night illusions

# Night flight checklist

Go or No-Go Decision

Media weather

Weather broadcasts

Patwas (Pilot's automatic telephone weather answering service)

TIBS (Telephone information briefing service)

No

Saying, "No, we won't go." makes you a PIC.  If you really know what you are doing you will know when to say "No!"  The strongest voice in opposition to "No!" is time.  Know your minimums and hold to them.

# Flying at Night Is Not

Just like Flying in Daytime

The hazards of night flying are directly related to the physiological limitations of the human body not the aircraft.  Humans do not perform well at night.  Night flying will be different.  Night flight is more stressful than day flying and very near to IFR flight without the required training.  It should be, a moderate amount of stress will improve performance, keep the pilot awake and motivated. However, subtle events occur at night that would be easily detectable in day light.  The solo pilot at night is at greater risk than when flying with an attentive passenger.  Night flight requires the pilot be very familiar with the area and have special knowledge that can be acquired only through experience.  I very much suggest you get this experience along with a pilot who have already acquired the experience.

Night flight is so completely different from day that it requires careful introduction.  Any pilot deficiencies become magnified at night. The night horizon is less visible and more indistinct.  Night flight is semi-IFR with considerable reliance on the instruments.  Clouds and terrain are from difficult to impossible to see.  On monocles nights, the objects seen are those which are illuminated enough to stand out.  There can be a gradual loss of visual clues when flying into darker terrain.  This leads to disorientation and loss of control.

Night flight adds to the risk of single-engine flying.  Emergency options are reduced.  The new five mile VFR minimums increase the impact of weather.  Mandated preparation for the flight such as lights and flashlights make a difference.  You will be much more able to cope if you maintain radio contact with ATC and have a readily available frequency list.  I avoid night training flights that have less than 1/4 moon.  Common mistake is flying when combination of pilot, conditions, aircraft, and preparation are not up to making the flight.  AIM recommends supplemental oxygen at 5000’ at night and  at 10,000’ daytime.

Flying a consistent profile is essential to safe night VFR.  Be so situationally aware that you do not descend below 1500' AGL until you are within engine out distance of the destination.  Plan to make a standard 45 entry so that you will reach pattern altitude when turning downwind.  If ATC gives you a straight in maintain pattern altitude until you are on two mile final.  Fly a VASI or PAPI if available.  If you know your ground speed, multiply it by five to get a 3 degree descent path.

In 1991 the night requirements for uncontrolled airspace were essentially raised to controlled airspace requirements.  Not having the instruments easily visible/readable is bothersome.  A 30 degree unobserved turn can cause complete disorientation.  The absence of a horizon can cause loss of control.  Both situational and geographic disorientation is more likely.

Note from 'Examiner Update' FAA Newsletter:

Pilot operation 9 (FAR 61.107) Pilot must have SOME night instruction before being qualified to be eligible to take flight test.  To be fully certified for night flight a student must have at least three hours of instruction at night with ten full-stop landings.

Night flight requires a more proficient instrument cross-check than does day.  In making maneuvers the horizon may be lost and spatial disorientation can occur.  Night flight has more inherent dangers and potential problems.  The obstacle you don’t know about will kill you at night.  More than ever, altitude at night is insurance.  Your policy is an accurate altimeter set.

# Physiology of night

Read up on the physiology of night vision so as to better understand the operation of the eye.  Over age 40, fatigue, and smoking affect visual acuity and adaptation to darkness.  Do not look directly at an object at night because the optic nerve location may not let you see it.  The decrease in oxygen above 4000' decreases visual efficiency.  Air Force requires full oxygen from the surface at night.  The light smoker is physiologically at 3000' before he gets into the plane.  Above 8000' at night it is a good idea to have oxygen.  Since we don't see as well as might be desired at night we must compensate using experience (brains) and technology.  The human eye performs poorly at night.

Fatigue has greater influence on pilot skills at night.  The retina is the first and fastest part of the body to react to reduction of blood oxygen.  Cigarette smokers start out with an immediate night vision problem.  Night vision can be improved by the use of oxygen.  Night flying errors happen because of human lack of capability.  Night vision is the key limiting factor.  Without surface lights, it is hard to know your altitude above the ground, with surface lights it is difficult to locate the airport beacon.  Most night accidents occur on 'dark night' flights.  Fatigue makes all of the safety factors involved to be more likely misjudged.  Raise you personal safety parameters at night and raise them even more when fatigued.  Skillful night flying is fragile, unused night flying skills must be polished regularly or they will be lost quickly.

The eye is much like a video camera.  A view is focused on the retina, converted electrically to data sent to the brain.  Rods and cones make the visual to electrical conversion.  Cones, near the focal center give colors, brightness and sharpness when light is good.  Rods are the night-vision part of seeing.  The peripheral region of the retina is rod territory.  Rods make it so we can see at night but not in color.  Complete night adaptation of the eye to darkness can take over 30 minutes and be destroyed in seconds.

The rods and cones that adapt to night conditions.  The cones are centered in the eye but are slow to adapt and then only by a factor of x 100.  Rods spread to the sides in the back of the eye.  They are more sensitive at night by a factor of x 100,000.  Rods take 30 minutes to recover from a bright light shock.

There is an oval shaped region of the retina known as the blind spot.  It cannot see light.  Binocular vision compensates for this in daytime.  At night we often are unable to see objects if we look directly at them.  To see at night we cannot look directly at what we want to see.  Your central vision is inoperative.  Looking off center at night uses peripheral vision which is 100,000 times more sensitive than central vision at night.  Your eyes can be adapted to night vision by wearing red glasses, patching one eye and using dimmed lighting.  No matter how well you do this one flash of a strobe taxiing out destroys it all.  It the lighted airports of today I prefer to work in a lighted cockpit.  I like to see my strange charts clearly with all obstacle heights known and avoided.  90% of our orientation is visual even in the cockpit.  NASA has proven that there is less oxygen at night than during the day.  The eye is quite susceptible to oxygen deficiency so vision at night at 9000’ gives the visual acuity that you would have at 15000’ during the day.

Your visual adaptability to light/darkness is reduced 50% every eleven years of your life.  Experience and frequency of night flight is the best compensation for this loss.  Any bright light effectively reduces night vision.  You might try protecting one eye from light until airborne.  Try wearing sunglasses at dusk.

The use of colors other than red in the cockpit has become more common in the 1990’s.  Light-emitting diodes are more efficient than other systems and will be in all cockpits of the future.  Blue lighting such as is common in military aircraft requires much more lighting than white lighting.

# Night sight skills

The ability to judge distances and heights at night is difficult at night.  The absence of haze or its presence can cause illusions at night.  Lights will vary in intensity and cause illusionary effects.  A mis-identified light source can cause total confusion.  A single light gives no altitude information.  Multiple lights may be in different geometric  visual planes.  Freeways become visible while country roads disappear.  Aircraft and lighted towers become visible for miles.  Airports have beacons.  The most common illusion is a narrow runway that appears to be longer than it is.  the narrow runway may make you think that you are too high.  Have a set procedure, allow an extra wide downwind at night.  Know the length of your destination runway.  Required FAR knowledge on all flights!  All illusions are made worse at night.

Preparation for night flight must be more intensive and comprehensive.  Make your initial night flight preflight during the day.  Check all the lights and carry a spare bulb.  Visual checkpoints are much closer.  Fuel reserves are doubled.  Charts are marked with black felt tip pens.  Frequencies are written large.  Terrain altitudes are noted and crossing extra altitude added.  Weather makes a big difference.  At night you can't see weather unless there is a moon.  We get very used to seeing weather change during the day.  Weather changes much the same way at night but quicker.  You must expect weather changes at night to occur suddenly simply because we cannot see the changes occurring as we can in daytime.  VFR to IFR at night by non-IFR pilots is usually fatale.  100  to 200 hour pilots have most such accidents.

One flashlight is not enough.  One (big) for preflight, one small for reading sectional etc., one backup.  No clouds, good dewpoint spread, minimum winds.  Don't fly at night into areas where you are not very familiar in daylight.  Have the legally required landings and carry another more experienced pilot (instructor) for cheap insurance.

# Night x-country

PREPARATION:

An honest assessment of skill and limitations

Routes, frequencies, weather, moon phase, airport information, terrain heights, FAR's related to night flight, night flight checklist

PLANNING:

- Take a blindfold test of the cockpit

- Older pilots need more light

- Limit night flying to familiar aircraft.

- An organized cockpit,

- Charts in order and folded for use, no red lines but well marked

- Closer checkpoints selected by time between and for night visibility

- Obstructions marked, ground routes/terrain studied

- Higher than normal altitude for terrain clearance

- Plan a what if...non-electronic flight possibility

- Airport, city-lights proximity route with VASI, PAPI runways

- minute VFR fuel reserve required

- Reduce range 1/3 keep track of wind direction and speeds

- No straight-in to strange airport, make high/steep approach

- Phone day before to an unfamiliar field for suggestions

- Extra careful preflight checking lights and spare fuse

- Always plan to get fuel before FBO's close for the day

- Weather notes on temperature/dew point spread

PRE-FLIGHT

Current sectional and TCA, flashlights, pens, radio backup? check all lighting.

FLIGHT:

- Night reference to instruments takes longer than in day light.

- Look outside more than inside

- Set all lights at lowest intensity that can be seen without 			effort.

- White lights increase mental alertness

- Clean windows

- Use landing light when near airports.

- Limit landing light use on ground due to over-heating of bulbs.

- Run-up creep is more likely to be undetected at night

- Use caution in proportion to darkness

- Lighting should be limited to preserve night vision

- Practice taxiing using navigational lights

- Tower can use light-gun to light center line of taxiway

- Know where you are at all times and know the nearest landing spot.

- If confused get assistance---CCCC

- Moonlit waters and freeways make good checkpoints

- Cloudy, moonless, windy nights are most difficult

- CAVU weather with full moon is best

- Recommend 1/4 moon as minimum unless IFR capable

- Keep altitude 'insurance' in force at all times.  Know your terrain.

- Check heading indicator/compass at checkpoints

- Be prepared to go on instruments and make 180 if you fly unexpectedly into a cloud.  Major cause of night flying accidents.

- Review causes of vertigo and disorientation

- Try not to pass a checkpoint without being oriented to next one

- Report your positions with extreme accuracy

- Night flight requires your highest level of precision and skill.

- Extreme levels of flying skill may be required all at once.

- Reference th A/FD to get frequency and procedure for turning on airport lights.

- Black hole takeoff and landing occurs in regions of few lights such as toward the ocean or a mountain.  You must be instrument competent or lucky to fly these.  Call me lucky.

- VASI or PAPI runways help you to avoid night landing illusions.

# Night Taxiing

Night taxiing is more difficult than any taxiing other than zero-zero conditions.  Night conditions are also difficult for the controller.  the controller may not know where your are any more than you do.  Be as specific as your can as to your last known location, your compass heading (you may not have set your HI) and the color of the line over your nose.  At night, don't do anything except taxi and keep track of your position.  The more experienced you are the more willing you seem to be to admit a problem of ignorance and a need for help.  When in doubt, stop.

Many aircraft have inadequate taxi lights and even lighted airports have unlighted areas.  A tower signal light can be used to show the center taxi line.  As age enters the picture, night vision fails.  A year or so ago I taxied the nosewheel into the mud while showing a student how to taxi without lights.  It was a good lesson, for the instructor.  When taxiing use as much lighting as you can and get any available assistance from ATC.  Being totally lost on your home airport is not uncommon.

I have had controllers use a light gun to show the taxiway centerlines when I had no taxi/landing light.  I have been at Sacramento Executive so far from the tower in the dark and fog that Airport Security had to show me to tiedown and later escort me to the active runway.  When you need help, get it.

Night ground operations are more difficult.  You may be able to follow the taxiway with a nose light while a wing-tip light makes it difficult to see the yellow line.  Be considerate of other pilots and don't use strobes while on the ground daytime and especially at night.

# FAR Night Flight Requirements (instructor)

Night FARs

- Night currency requires three full stop landings within past 90 days.

- Landings must take place one hour after sunset to count towards currency.

- Navigational lights must go on at sunset.

- FAR 91.209 requires that position lights and anti-collision lights be used from sunset to sunrise.  For logging purposes, night is the time between sunset and sunrise, for currency purposes night is from one-hour after sunset to one-hour before sunrise.

- SVFR at night is not allowed unless IFR current for both pilot and plane.  45-minute fuel reserve is required at night.

- Night visibility must be 3 miles except when remaining withing 1/2 mile of airport while doing pattern work.  Cloud clearances are 500, 1000, and 2000 feet.

- FAR Part 1 defines official "night" so pilots can log the conditions of flight, day or night required by FAR 61.51 (b)(3)(i). Twilight is the time of incomplete darkness after sunset or before sunrise.   FAR 6157(d) defines pilot proficiency requirement that must be complied with before the pilot can be pilot in command of an aircraft carrying passengers at night.   This is the time one hour after sunset and one hour before sunrise.  This more restrictive "night" is designed to assure pilot proficiency.  The less restrictive requirement can be used for meeting certificate requirements.

- Nothing in FAR Part 91 requires lights for takeoff or landing at night at either controlled or uncontrolled airports.  However ATC cannot clear you to land at an unlighted runway. They can say that any landing is at your own risk.  Most obstructions do not appear on ATC radar.  Obstruction clearance and avoidance is a pilot responsibility  and the eyeball is best supplemented by a sectional chart.  ATC will not issue a clearance that does not provide standard terrain clearance.  Pilot agreement to a given clearance is requires only in non-tower operations.  Terrain avoidance remains totally the pilot responsibility until minimum instrument altitude (MIA) is reached.  ATC cannot issue a clearance requiring pilot concurrence unless the pilot says that he can maintain obstacle and terrain clearance.  Pilot concurrence does not mean that the flight must be or will be flown in VFR conditions.

# Night Takeoff

Taking off into dark terrain may give an illusion that causes the pilot to make a bank or shallow descent.  Dark-terrain takeoffs should be made on instruments.  Night takeoffs should rely heavily on instruments until altitude is sufficient to allow for any monetary disorientation that is likely to occur using visual reference.

### Factors:

Weather

If you are cautious during the day, be doubly cautious at night.

If your strobes sparkle it indicates rain.

Visual illusions

Lack of lighting over final approach gives too high illusion.

Instrument competence

Greater attention to instruments at night.

Select altitude and airspeed and keep it

Abrupt power reduction gives nose down illusion. 				(somatogravic)

Workload

Biological clock problem

Be rested

If solo use ATC services

Anxiety

Experience and training

Don't activate lighting too early while taxiing out for 			takeoff or too early when arriving for landing.

Very dangerous to takeoff into total darkness like toward the 			ocean--you are IFR,

Check heading indicator to correspond to runway prior to takeoff.

No turns until at safe altitude.

Remember scattered lights on a mountain can be confused with 			stars if there is no horizon.

# Night landings

- Use your landing light.

- Avoid turns when cockpit workload is high.

- Don't hesitate to ask tower to turn up lights

- Pre-select an early go-around point and use it.

- Use a slightly longer/steeper final than normal.

- Know how to get lights at an uncontrolled field.

- Flare level and don't try for full stall landing.

- Maintain constant airspeed on final with power on.

- Remember distances can be very deceiving at night.

- Practice landings with and without landing lights.

- Practice landings with and without landing lights.

- Undershooting at night is greatest cause of accidents.

- Avoid long shallow approaches.  The steeper the better.

- Down slope lights cause high approaches and long landings.

- Up slope lights cause low approaches and landing short of runway.

- of flight takes place at night with 25% of the accidents--either fly a lot or NOT.

- If you are insecure flying at night, consider limiting bank angles to half of normal.

- Straight in approaches are most dangerous unless VASI or glide slope available, make high steep approach.

- If moisture reflects from landing light beam do not use for landing since it usually causes you to flare high.

You must know critical information (heights of terrain), that which can be seen vs unseen at night,  (freeways vs country roads), situational awareness becomes vital, willingness and ability to communicate to ATC. and knowing the performance parameters of the aircraft cold (without interior lighting) is beyond the just "nice to know" requirements of daytime flight.

The descent to the airport should be planned well before the actual occurrence.  Do what it takes to raise your level of mental alertness.  The safe performance of night landings depends on your ability to control approach speed and altitude.  If your daytime landing procedures are based upon a stabilized approach the transfer to night landings will be easier.  The stabilized approach gives you a reference form which to evaluate night landing illusions.  Your depth perception, visual cues, and runway perspective changes at night.  The desire to remain high and fast can overcome your training.  A high fast landing at night will be hard long and dangerous.  Like day landings a good night landing begins with a good approach but it is not exactly the same.  Avoid excess airspeed, use your instruments to confirm visual impressions, especially the altimeter and airspeed indicator.  Set your configuration of power, flaps, and airspeed as though doing a soft field landing.

Once you become aware of the many illusions that occur at night you will see the advantage of flying a pre-selected approach pattern.  Runway lighting gives you an impression of runway area.  There is an illusion as to runway length-to-width.  The vertical-position illusion occurs when there is no visible horizon.  It makes lights and visible objects to appear higher than they actually are.  The false-horizon illusion makes lights and stars appear to blend so that you cannot be certain as to where the horizon is. The foreground-occlusion illusion occurs when something ahead makes a light disappear.  Climb immediately.

Keep power on during a night landing.  The terrain cues needed for flight path correction are meager and undetectable at night.  Look at the lights toward the far end of the runway.  When these lights begin to flatten out you should enter your flare.  Do not try to make a full stall landing.  Your visual ability to determine altitude at night is seriously degraded.  Make your flare using the runway lights at the far end of the runway.  raise the nose until they are covered, but no more. Hold some power on as the plane gradually descends.  You don't want to hit the nose wheel first but a relatively flat landing at night is acceptable.

You need to be an active night flyer if you expect to make consistently good night landings.  I would recommend doing at least 1/4 of your flying at night to retain proficiency.  With experience a pilot can become consciously aware of the visual cues that are available and use them to improve night landing accuracy.

Likely faults by those whose proficiencies are due to lack of practice will be:

1. Approach speed too fast due to poor airspeed control.

2. Unable to detect/correct wind drift

3. Hard touchdown due to over-reactions during flare.

4. Likely to be below pattern altitude

THIS INSTRUCTOR WILL NOT AUTHORIZE STUDENT NIGHT FLIGHTS

# Night Instruction

The introductory night flight should begin at sunset and end just after total darkness.  The transition from day- time visual cues to a landing to those available at dusk and in darkness is a valuable lesson.  Under crosswind conditions the lesson is even more important.  Future night flights should include a well lighted urban area airport,  a small airport adjacent to a city, and an isolated field that is difficult to find at night.  The required FAA three hours is a minimum and not really enough for proficiency.

The normal tendency to get too close on downwind at a small runway is much worse at night.  It is not unusual to have a pilot confuse street lights with runway lights.  Be extra careful not to get confused by nearby airports.  It is not all that unusual to have a pilot communicating with one airport while landing at another.

Destination airports are difficult to find at night.  Once found, it is difficult to keep the airport in sight while planning your approach to a specific runway.  Use your heading indicator and get all the help you can from radar, tower, passengers, and other planes.  VFR radar advisories are provided only as ATC workload allows.  See and be seen applies.

If a runway has a downslope do not level off horizontally. A three-degree approaches glide will cause a one-degree upslope runway to give the ILLUSION that your approach is much too steep.  The same approach at a one-degree downslope runway will give the ILLUSION of too flat approach.  Find out the slope of any runway you expect to use at night.  Don't fly into a marginal field at night without first having been there in daylight.

Use the landing light on every other landing to maintain proficiency in case it should burn out.  Use the landing light to signal other aircraft that you see them.  With light shock occurring all the time in urban areas, my preference is to use cockpit lights all the time and rely on aircraft and airport lighting. In x-country flying I reduce cockpit lighting to a minimum and try to preserve my night vision in at least one eye.

# Night Lights

The rotating beacons all carry vital information:

Civil land:  alternating white and greeen

Beacon operates daytime when field below VFR minimums.

Military land: two quick white flashes and a green

Heliports: rapid flashes of green, white, or yellow

Lighted water: alternating white and yellow

Uncontrolled airport:

Pilot controlled on CTAF frequency:

low-intensity takes three clicks of mike switch

Medium-intensity takes five clicks.

High-intensity takes seven clicks.

VASI (visual approach slope indicator

Have two for G.A. aircraft or three bars for 747 types.

“Red over White, you’re all right.”

Three bar VASI’s will have two red over white for G.A. aircraft.

PAPI has four lights in a row on left side of runway

On glide path gives two red and two white

High gives three or four white

Low gives three or four red

# What to do if

#1 SITUATION:

Night, good VFR, moon, no terrain problem, non-emergency

1. Maintain aircraft control, avoid descent, trim.

2. Self orientation

Maintain VFR and extra altitude

Check sectional for terrain altitude minimums

Orient sectional to flight direction

Use VORs to determine position

Select course-turn and fly

Intercept VOR radial and fly to destination

Fly to city lights over 3000'

3. ATC assistance

Contact FSS-request DF steer

Contact Approach- request assistance.

4. Second Pilot

If you have doubts as to your experience

Good way to extend experience and lower cost

Lowers cockpit load

# 2 SITUATION:

Night, clouds, poor visibility, lost, fuel low

SUGGESTION:

For any one of the three...

Declare EMERGENCY  121.5/7700

# 3 SITUATION:

Mountain downdraft

SUGGESTION:

First of all I would not recommend any aircraft with less than 180 h.p. in mountains.  Never approach a ridge at 90 degrees.  A 45 degree approach gives a quicker get-away.  Try to have 2000' clearance if possible.  You may need the extra altitude if you hit a 2000' per minute downdraft in 40 kt winds.

It is important to get out of the downdraft quickly.  Instinctive desire to climb at Vx or Vy is wrong. Slow down in an updraft full power and speed in a downdraft.

# Night illusions

(see hood for more illusions)

Try some night airwork without cockpit lights.  It will make you listen more closely to the sounds of the aircraft.

Runway and approach light illusions will always be a problem if you are in an unfamiliar area.  For this reason it is always desirable to make a daylight familiarization flight to an airport before a first time night arrival.  If there is no VASI or VAPI for vertical guidance if you get too low the runway lights will begin to disappear.  A steep approach is always better at night.  If there is a strong crosswind and you are crabbing to the runway instead of slipping you will get the illusion of being inverted.  If the airport is well lighted in a surrounding dark area you will have an illusion of being higher than you actually are.  Again a steep approach has much advantage.  Rain on the windshield will give the illusion of being higher than you are.  An arrival at an airport with and approach lighting system (ALS) tends to be lower and at a shallower angle than otherwise.  If you are low and pitch the nose up as a correction or through the use of flaps, the illusion will indicate that you are rising.  Any reduction of power will cause you to land short.

At night, banking into or away from a line of lights will give the illusion that a dive or a climb is occurring.  The same dive or climb illusion can happen by a change in aircraft pitch occurs while flying toward a light.  Lights that appear dim, as seen through haze, will be reported as more distant than they are.  Example: I once reported an airport as in sight from twelve miles when the tower had me on radar at only five miles.

Pilots unconsciously make extensive use of their peripheral vision.  Level flight, banks, climbs, and descents rely 80% on peripheral vision.  (See downwind turns)  At low levels our peripheral vision gives us a sense of speed.  Over time we develop a peripheral sense as to what "normal" low level speeds are.  Add a tailwind, low altitude, a bank to final approach, and a peripheral sense of a "high" speed.  We now have an illusion causing a pilot response that says to pull back on the yoke to reduce the speed.  The pull merely makes the bank steeper and initiates a low level stall spin.  Recovery not possible.

A final approach over high terrain leading to the runway gives an illusion of a low fast approach.  An approach over terrain that makes the runway seem like an aircraft carrier will give an illusion of too high and too slow.

Night has its own illusions that are covered in the night flight lessons.  The distance of lights is greatly affected by the relative clearness and haze existing.  A region of no lights such as might exist off the end of a runway toward the ocean can cause disorientation because of IFR illusions.  The best solution is to go on instruments until established inland at altitude.