# Click on page number for instant topic scrolling.

# Practical Test standardsã

FAR 61 changes in August 1997

PRACTICAL TEST STANDARDS

Private Pilot Performance Standards

Universal Aspects

The Practical Test -

APPLICANT SUMMARY

Private Pilot Requirements

Paper work:

Logbook requirements

In possession for Checkride

EXPECTATIONS

THE EXAMINER

General Procedures

The PTS

PTS Failure

Descending Order of PTS Problems

Private Pilot Performance Standards

Performance sheet ASEL

Fear of failure:

Private Pilot Endorsements

# FAR 61 changes in August 1997

- Night requires 3 hours 10 landing and 100 mile flight.

- Cross country time requires a landing destination and use of a

navigational method.  No distance is required except for more than

50 NM straight line when required for getting a rating such as

private pilot.

- Pilot and medical certificates must be in personal possession.

- Complex and high performance airplanes require separate endorsements.

- Endorsments required for altitude, type rating, and tailwheel.

- 30 day wait between tests no longer exists.

- Student pilot solo time is PIC time.

- Second in command time requires you be both required and qualified.

- 24 month flight review required, new rating, or award program will 	count.

- Instrument recency requires 6 approaches, holding, tracking via nav.

- Instrument rating requires only 50 hours x-country time.  40 hours of    	time of which 15 is with CFII.  250 NM IFR flight with three 	approaches.

- Private pilot requires 3 hours hood, 3 hours x-country training, and 	100 nm night flight and only 150 nm x-country.

- Passing level of different written tests will vary.

# PRACTICAL TEST STANDARDS

Latest edition:  FAA-S-8081-14S

New Specifics:

- VFR minimums in all different airspaces.

- Indicate to examiner first indications of stall

- Explain FAR 91.312 on ADs and Minimum equipment

- Effects of over-counter drugs, stress, fatigue

- ...completes appropriate checklist in every phase of flight.

- x-country assigned day before.  longitude/latitude coordinates included 	and headings within 15° (15° Good Grief!)

- Ground reference maneuvers to make first turn to the left with two 	complete patterns before changing direction.

- O.k. to fly V-speeds in slow flight and minimum controllable.  Spin 	awareness testing increased.

- Emergency descents as though on fire.

# Private Pilot Performance Standards

The checkride is more of a test of the instructor’s training program than of student performance.

1997 Maneuver Tolerances:

Altitude + 200 feet

Heading + 10°

Speed + 5knots

Over the fence Vref + 5 knots or 1.2Vs1 (No exact POH figures)

Preflight & Cockpit Management

Inspects using checklist

Passenger briefing on belts and emergency

procedures from checklist

Tighten belts, open door, use of

radio, aircraft watch

Engine Starting

Clear, start and after-start with checklist

Taxiing

Brake check, speed control

Knows and obeys signs, lines, and (lights)

Yoke positions for wind

Radio use and acknowledgments and compliance

Run-up

Uses checklist

Airspeed, distances review

Emergency and departure review

Pre-takeoff

Uses checklist

Takeoff configuration

Radio use and acknowledgments and compliance

Clearing approach

Traffic Patterns

+ 100 feet and +10 knots.  The idea of a ‘key’ point on the downwind 		is no longer an FAA procedure.

Takeoffs

Normal and crosswind Vy + 10/- 5 knots

Soft field - in ground effect to Vy, climb at Vy +10/-5 knots

Short field - rotate at Vx or by POH, climb at Vx, +10/-5 knots to 50 	feet, then at Vy, +10/-5 knots

Landings on final

Normal and crosswind 1.3Vso or by POH +10/-5 knots (gust factor) 			within 400 feet

Soft field 1.3 Vso or POH +10/-5 knots (gust)

Short field 1.3 Vso or POH (gust factor, with touchdown within 200 		feet past a point.

Forward slips within 400 feet at ~ stall speed

Go-around

At Vy + 10/-5 knots

the go-around can be executed at any point in the pattern.  The go-	around on final is to be treated as an emergency.  90% of go-around

accidenrts occur on final.  Some go-arounds are not possible due to

limited aircraft performance and densitiy altitude.

Steep turns

Va or POH entry speed, into 360 turn with bank

of +10/-5 degrees of 45, + 100 feet, +10 knots,

perform in both directions or as directed

Ground reference

Turns around a point

600 to 1000 feet, 45 degree bank, + 100 feet, + 10 knots, to left turns with exit up-wind and course reversal into right turns or as directed

Rectangular course and S-turns

(1997 requirement)Entry MUST be on 45 to left downwind as though landing Departure will be as though leaving on left 45.  When making reversal of direction you must fly around the course and enter on a right 45 and depart of right 45 600 to 1000 feet, left then right, maxinmum bank 45 degrees +100 feet, + 10 knots, or as directed

Pilotage and dead reckoning

Course + 3 nautical miles,

# Universal Aspects

Tests now follow logical sequence of flight from preflight to shutdown.  There is an applicants checklist of what to bring to the test along with an examiner’s checklist of things to be covered.  The introduction tabulates the references to be used by the examiner.  Responsibility for teaching and testing of the test standards is explained along with special emphasis on aircraft operations 	critical to flight safety.  The standards of performance are deliniated between satisfacory and unsatisfactory.

The following is an effort to give the pilot to be, a comprehensive but not complete analysis of the PRACTICAL TEST STANDARDS.  In this one place you will find everything on the test but often greatly understated because of space constraints (Chronologically challenged instructors never admit knowledge limits).  Where practical, brief and often incomplete topical explanations are made suggestive of what might be required during the test.  Many aviation texts will give an expanded complete explanation.  If a particular items fails to bring recognition and understanding then go back to the books or your instructor.  Refer to the contents of this material as it exists in other areas.

The practical test is a two part test.  The oral part requires oral operating and semi-technical knowledge of the aircraft systems, navigation, weather, preflight requirements, and POH computations.  The oral test will concentrate on what you are expected to know as obtained from the sources you are expected to use.  Use the test as a learning situation.  You will learn those areas of your training for which you are not as well prepared as you should be.  Explain what you know as simply as you can.  Stick to the basics as you know and understand them.  Ask the examiner to reword a question that you think will give you difficulty.

No examiner asks a test question to which he has not pre-decided the best answer.  You must make your answer concise and complete enough to satisfy the examiner.  Avoid using technical terms where it is possible to use plain language.  Stick to answers that offer the safest way to operate the aircraft.  You will not be failed for knowing what you are supposed to know.  You can be failed for any guessing or showing of poor judgment.  Only knowledge will provide the degree of self confidence required.  Any test is easy if you have the answers.  You have many more answers than there are questions...your problem is to match your answers to the questions.  Take and pass your written test as close to the flight test as you can to reduce study time and enhance retention.

The flight test involves use of checklists, preflight, aircraft control, use of, radio, pilotage, weather facilities, radar, navigational facilities, and emergencies.  Everything in the PTS must be tested.  How long any part or item takes will depend on how well and quickly you please the examiner.

The test requires the performance of a multitude of tasks separately and in combination.  Effective division of attention is a central skill of safe flight so the examiner will present situations when you can demonstrate your ability to scan the variables, note the critical and discriminate accordingly.  Distraction is a part of the test.   Minimum standards must be met with smoothness, accuracy and good judgment predicated on good flying conditions.   If control is ever in doubt, tolerances exceeded, or prompt corrective action not taken, minimums will have been exceeded.  Failure to clear or scan area before an maneuver will be disqualifying.

# The Practical Test -

COMMON ASPECTS OF ALL PARTS OF THE TEST:

- Soundness of judgment

- Division of attention

- Airspeed and altitude limits

- Safety awareness

- Smooth and accurate aircraft control

- Ability to explain

- Where to find information

- Situational awareness.

The Practical Test -

Determines if applicant has had required instruction

Determines if applicant can safely perform required tasks

The major areas of required knowledge are:

- Preflight

- Flight Procedures

- Aircraft Performance

- Radio Navigation

- Pilotage

- Flight Planning

- Regulations

- ATC Procedures

- Airport Operations

- Communications

- Physiology of Flying

- Aircraft Systems

- Services Available

- Airspace

- Charts

- Meteorology

# Applicant Summary

You must have an application for a rating form, FAA Form 8710-1, 	a waiver form as required, a medical certificate, and radio license (for foreign flight).  FAA Form 8710-1 must be in ink or typed and signed in last 60 days to verify total logbook times. The CFI must print name under signature and CFR must follow certificate number.  The latest Examiner’s Update informatrion is that you cannot fail the checkride because of the manner in which you filled out the application.

You must be prepared with a selection of required papers, endorsements and forms.  Be sure to have valid picture identification such as your drivers license.  Present appropriate "notice of disapproval" or "letter of discontinuance" if test has been failed previously.  The written test score lists missed question areas.  Your instructor must verify all missed areas  as retaught.  In addition to your current medical certificate your student pilot certificate must have all the proper endorsements for solo in aircraft types and cross-country flight.  It would be nice if all endorsements were the responsibility of the instructor but it is up to the applicant to determine that all endorsements are completed.  You can blame everything else on the instructor but it won't change the outcome of the test.  Have photo identification and verification of residence.

Your logbook must have the required flight instruction signed off as to kind of instruction and types of hours flown.  Bring money.  It is money that makes an airplane fly.  You are expected to have current charts, plotter, computer, flight logs, pens and pencils.  A hood is required flight test equipment.  You should have a current copy (use mine) of the AIM, the FARs, Airport Facilities Directory, and the Practical Test Standards.  Be sure aircraft has its proper POH and take it with you for the oral test.  You are expected to know what is in the Practical Test Standards, to know that the examiner is required to test every item, and to know that you must be able to perform or discuss every item of the test.  What you don't know you must be able to reference quickly either in the AIM, PTS, POH or FARs.

The aircraft must be preflighted, airworthy, with all required papers, logbooks, and endorsements.  Post-it all required aircraft and student endorsements so they are easily located.  Know how to read AD code for required maintenance.  You are to know the operating limits of the aircraft, its performance, its systems, inoperative placarding, and emergency options.

Be familiar with all charts, graphs and operational numbers from the POH.  Don't show up with a dirty airplane.  Your appearance, hygiene, attitude and preparedness will make a difference.

# Private Pilot Requirements

At least 17, read, write, and speak English, 3rd class medical, written test, oral test, flight test.  61.103

A total of 40 hours instruction/solo flight time including:

20 hours instruction:

3 hours cross-country,

3 hours night (10 landings),

3 hours test preparation.

20 hours solo: (10 in airplanes)

10 hours cross-country,(One flight 300 nautical miles with

point 100 nm distant from  departure)

3 takeoff/landings from ATC airport

Logged and endorsed as competent in these PILOT OPERATIONS

(Note: pink test failure slip lists pilot operations by number)

- Elements of preflight

- Airport operations

- ground reference

- Critically slow speeds

- Landings, Normal/crosswind

- Basic instrument procedures

- Cross-country flying

- Maximum performance TO/landings

- Night flight

- Emergencies      61.107

Pilot in Command Time

There are five different ways to get logable pilot in command(PIC) time according to FAR 61.51(c)(2)

1. Sole manipulator of the controls in an aircraft rated for.

2. Sole occupant of aircraft

3. Certified Flight Instructor acting as CFI

4. Applies to ATPs

5. Applies to two pilots required for aircraft

# Paper work:

Endorsements:

1. Your student license must be endorsed for each type of aircraft 			flown and for cross-country flight.

2. Your log book must be endorsed for ground instruction given, 			flight instruction given, and solo flight along with written 		test results for each type aircraft.

3. You must have endorsements for flights between airports, each 		flight over 50 miles must be individually endorsed

4. The solo endorsement must be renewed every 90 days.

5. You must have an endorsement for spin awareness training, night 		instruction, and for an application for rating.

Records:

Maintenance records of aircraft and engine must be available to the examiner.  FAR 91.3  The pilot must be able to explain and interpret all of the maintenance records.  ARROW  Airworthiness, registration, radio license, POH or pilot operating handbook (equivalent), and weight/balance papers must be in the aircraft.

# Logbook requirements

- Student solo license endorsements for each type aircraft

- Student solo cross-country license endorsement

- Logbook signoff for ground instruction

- Pre-solo aeronautical knowledge and instruction endorsement

- Logbook signoff for aircraft test each aircraft

- Logbook solo requirements endorsement

- Logbook solo endorsement

- day solo endorsement

- Logbook local airport solo endorsement

- Individual signoff for EACH x-country

- Signoff for review of questions missed

- Aeronautical knowledge and instruction endorsement

- Take Practical Test endorsement

- Test after failure endorsement

# In possession for Checkride

- Logbook

- Medical/Student certificate

- Sectional (Must be current)

- Aircraft engine logbook (Get from mintenance)

- Aircraft airframe logbook (Get from maintenance)

- Hood

- Aircraft manual

- Registration

- Airworthiness

- Radio Station license

- AIM (Borrow mine)

- A/F directory,

Ground review:

- All frequencies

- Sectional use

- FARs

- Aircraft manual (one in aircraft-required)

- Aircraft papers/logbooks

Flight review:

- All slowflight

- All stalls

- All landings

- Go arounds

- Steep turns

- Spirals

- Unusual attitudes

- Emergencies

# Expectations

Stress is an important factor in the Practical Test.  The best possible stress reducer is to know what you are supposed to know.  The PTS publications delineates responsibility.  The applicant is responsible for knowing and preparing certain aspects.  The instructor is responsible for teaching to proficiency the required flying skills.  He is also expected to determine that the student has the required 'ground school' knowledge to pass the oral part of the PTS.  The examiner's responsibility is that he must cover everything in the PTS.  He is somewhat limited to the references listed in the PTS in his oral testing of knowledge skills.  He is expected to determine the applicants knowledge of the aircraft, its limitations, systems, and malfunctions.  An applicant who does not have knowledge to pass the oral will not even get to the airplane.

The flight planning should be 'practical'.  That is, the way you would do it. You are expected to know how to call up DUAT weather and interpret the printout.  The oral will include interpreting charts from AC 0045.  You will be expected to explain any aspects of the flight planning, aircraft operation, and maneuvers.  Oral questions may be used at any time during the test.  They can be used as distractions since distraction is considered part of the test.

Responsibility of the CFI

Applicant certified as ready and qualified as CFI signs application.

# THE EXAMINER

The FAA Designated Examiner is selected by need, experience, ability, recommendation and written test results.  He must give every oral and flight test based on acceptable standards for the objectives for every task in the Practical Test Standards (PTS).  The areas required for testing are not chosen by the examiner.  He is told what to do (everything) and the accepted standards of performance.  The examiner is required to ask questions to determine applicants knowledge and understanding. The examiner is to observe the applicant in the performance of routine, non-routine and emergency procedures. Oral questioning may be used at any time.  Questions will be practical and based on sources shown in the front of PTS text. Examiners are advised to avoid using other sources as references.  Testing will include all safety factors related to each operation.  References for each task are listed in each part of the PTS.  The examiner is a referee.

The tasks and the performance required are national standards.  This is objective as to required tasks and levels of performance.  The examiner is an impartial observer of how you can be expected to perform as a pilot.  He evaluates performance of routine, non-routine and emergency tasks.  He asks questions and assigns tasks as part of the process.  The purpose of the test is to objectively determine via standardized performance criteria the ability of a person to perform certain flight related tasks to a satisfactory level.

At any time during the test where a pilot does not meet the practical test standards, the examiner must terminate the test.  A re-test of the deficient area causing the halt and all other areas not completes is required.  The worst nightmare of any examiner is incomplete paperwork.  The back of the PTS has a checklist to cover this problem.  Organize your paperwork in a folder and put post-it tabs to indicate location of important endorsements in the aircraft, engine, and your logbook.

The examiner's method of emergency simulation should be discussed prior to the flight. The discussion would cover inoperative equipment, maximum performance takeoff and landing knowledge and judgment.  The successful conclusion of an in-flight emergency is always the primary objective.  Cross country flight planning  is a test of judgment as much as knowledge.  Fuel consumption, requirements and reserves are computed.  Weather, alternate plans and aircraft range enter into the planning.

During the test the examiner may place you into a situation where you are

uncertain as to just what the expectations may be.  Ask for a ‘time out’ to discuss your concerns with the examiner.  Ask not to continue until your concerns are satisfied.  Often the examiner will apologize for not having

properly or clearly conveyed his intent.

The basic requirements for the test is that the applicant must be master of the aircraft.  He must be able to put it where he wants it when he wants it there.  Flight must be both smooth and precise while showing good judgment and application of the appropriate FARs.  Procedures and maneuvers, including emergency simulations must always be performed within the operating limitations of the aircraft POH.

Each test, once begun, will result in:

- Pass - with white temporary certificate

- Fail - with "pink" Notice of Disapproval of Application

- Letter of Discontinuance - Applicant has 60 days to finish test.

Any subsequent test must be completed before expiration of written test deadline. (24 months after taken)

# General Procedures

See FAR 61.43 Flight Tests

a) Perform pilot operations...

1) Executing maneuvers within performance capability and limitations, 		including use of the aircraft's systems

2) Executing emergency procedures and maneuvers appropriate to the 		aircraft.

3) Piloting with smoothness and accuracy

4) Exercising judgment

5) Applying aeronautical knowledge

6) Mastery of aircraft with no procedure or maneuver in doubt.

b) Failure of required operations = failing flight test.  Must pass any 	areas failed.

c) Applicant or examiner can end test.  Applicant credited for those 	parts of the PTS passed.

# The PTS

Everything about the Practical Test is a given.  If you have studied the PTS you know what is going to happen.  You may not know when or just how but you can be sure of what.  There is more to being prepared than just knowing answers.  You need to know the 'why' of the answers.

The applicant must be able to:

- Use aircraft and systems within performance capabilities and 			limitations.

- Perform emergency procedures

- Show good judgment

- Show knowledge of aircraft/ATC procedures

- Control aircraft in maneuvers within minimums

The best way to learn the systems of an aircraft is to learn to draw them.  Study a pictorial diagram for a couple of minutes.  Close the book and make your best drawing.  Open the book to finish and correct the drawing.  Repeat the process until you can draw a complete diagram.  An examiner will be ‘impressed’ with this ability and probably not pursue the subject any further.

The private pilot of today needs to know more than the commercial pilot of just a few years ago.  With the automation of weather services, the pilot must be knowledgeable of weather and weather chart interpretation to the level required of the FSS technician two years ago.  Two years from now all the technician will be replaced by computers.  The aging of our aircraft fleet makes it vital that today's pilot be knowledgeable of maintenance factors never required when old planes were just replaced with new.  The radio skills required by the high density operations of ARSA and TCA go far beyond just getting into and out of a controlled airport.  While the flying skills are much the same, the knowledge skills required have greatly increased.

If a student fails to learn, someone has failed to teach.  It the instructor's fault that an area of knowledge is weak or a flight skill is deficient.  An instructor must demand the study time needed to detect and correct any weakness.

The practical test is the last hurdle to certification.  The test is to determine if you have the training and proficiency required.  The test is a presentation of tasks to be evaluated by oral questioning, observation or in combination.  Only question/answer references are those listed in the PTS.  Seldom used material is to be found by use of reference to POH, AIM or A/F Directory, etc.  The FAA highest priority is that the test be fair.  I have always found it so for the past twenty five years.

You must know the safe altitude for performance of any maneuver both with respect to your aircraft and its occupants and with respect to those on the ground.  Safety requires practice.  Pre-maneuver checklists such as STOP and CAPS will prevent you from blowing the safety part of the PTS checkride.

S = safety

C = Clearing turns

A = Altitude

P = Place (to land)

S = Speed

T = Tolerances (altitude, heading and speed

O = Objective

P = Procedure

The instructor won’t sign your certificate unless he is reasonably certain that you will pass the test.  The test, oral and flight, is divided into areas of operation and tasks.  All operations and tasks must be tested.  The examiner may vary the order of the test to promote efficiency and validity.  The reference for each area is listed in the PTS.

# PTS Failure

The habits and judgments we have developed cannot be concealed or changed just for the checkride.  Any bad habits or judgmental problems will show up in the checkride.  If you have any doubts as to your flying habits or ability to make the proper decisions, get them resolved during instruction.  You won't be able to fake your way through a checkride.

I have had a number of 'failures' in twenty years.  I have salvaged a number of 'failures' from other instructors.  I have never known an examiner to be arbitrary or unfair.  I have never known one not to give a student a second chance to improve on a failed performance.  I have seen where a 'failure' was instrumental in saving more than one pilot' life.

An applicant who fails his flight test and is issued a failure slip.  The only item he must demonstrate is a short field landing.  This demonstration must take place within 60 days and within the 24 month period allowed after passing the written.  If not the written must be retaken and the complete PTS test must be retaken.

# Descending Order of PTS Problems

- Student unable to slip due to instruction failure.

- Incorrect pattern entry.  Applicant should know to overfly airport 		for orientation instead of winging it.  Should be taught.

- Oral on weather, aircraft performance, weight and balance due to 		inadequate preparation.  (Instructor specific)

- ATC light signals.  Suggest use of multi-colored flashlight to 		practice.

- Steep turns, soft/short field and emergency landings.  Teach 			stabilized approach/constant glide angle.  Various landings should 	be practiced at many different airports.  Emergencies should be in 	all phases of flight at all airports.

The most common reason for failure of flying skills in the PTS is making crosswind landings.  Most examiners will allow an applicant a second try at a crosswind landing.  It won't really change anything because if you can't make the first one it is unlikely that the procedure for the second will be any better.  Don’t wait for the examiner to tell you to go around.  Don’t delay theg-around.  The second worst thing you can do is to try to salvage a poor flare.  The worst thing to salvage is a poor touchdown.

## Why Applicants Fail

- Taxiing across a runway without a clearance.

- "Paperwork failures" ;are not to be allowed as of 1997.

- Insufficient hours logged of a given type.

- Improper endorsements

- Improper aircraft logbooks

- Incorrect/no weight and balance data

- Incorrect aircraft handbook

- Out of date charts

- Unable to interpret AD and inspection data

- Reading weather

- Reading charts

- Emergency operation of equipment

- Location of extra fuses

- Pitot Heat

- Open door

- Ground fire while starting

- Forced Landings

- Slips

- On takeoff

- At altitude use of radio

- Getting help

- Cross-country emergency

- Turn in right direction first

- Then use Checklist

- Instrument flying

- Maintaining standard rate turns

- Avoiding airspeed excursions

- Cross-country planning

- ETA figures

- Legal requirements

- Manual limitations

- Cross-country flying

- Getting to first checkpoint

- Use of checkpoint checklist

- ETA figures

- Heading corrections for winds

- Normal takeoffs and landings

- Holding airspeed during turns

- Trim for stabilized approach

- Flight at minimum controllable airspeed.

- Avoiding stall

- Using power to hold altitude during turns

# Private Pilot Performance Standards

- Preflight & Cockpit Management

Inspects using checklist

Passenger briefing on belts and emergency procedures from checklist

- Engine Starting

Clear, start and after-start with checklist

- Taxiing

Brake check, speed control

Knows and obeys signs, lines, and (lights)

Yoke positions for wind

Radio use and acknowledgments and compliance

- Run-up

Uses checklist

Airspeed, distances review

Emergency and departure review

- Pre-takeoff

Uses checklist

Takeoff configuration

Radio use and acknowledgments and compliance

Clearing approach

- Traffic Patterns

+ 100 feet and +10 knots

- Takeoffs

Normal and crosswind Vy + 10/- 5 knots

Soft field - in ground effect to Vy, climb at Vy +10/-5 knots

Short field - rotate at Vx or by POH, climb at Vx, +10/-5 knots to 50 	feet, then at Vy, +10/-5 knots

- Landings on final

Normal and crosswind 1.3Vso or by POH +10/-5 knots (gust factor) 	within 400 feet

Soft field 1.3 Vso or POH +10/-5 knots (gust)

Short field 1.3 Vso or POH (gust factor, with touchdown within at 200 	feet past a point.

Forward slips within 400 feet at approximate stall speed

- Go-around at Vy + 10/-5 knots

- Slow flight

Not below 1500 feet AGL, speed as 1.2Vs1, +10/-5 knots, + 100 feet,

+ 10 degrees of heading, maximum bank of 30 degrees +0/-10 or as

directed, climbs and descents at 20 degrees, +0/-10, headings from 	turns + 10 degrees, levels off + 100 feet.

- Stalls

Power off, not beow 1500 feet AGL, stabilized approach, landing 		configuration (full flaps), heading + 10 degrees or up to 30 		degree bank + 0/-10 degrees.  Notes and announces buffet, first 		control decay, recovers at stall, to Vy before final flap 			retraction, returns to altitude and heading previously specified 		by DE.

Power on, not below 1500 feet, departure configuration, heading + 10 	degrees when inducing stall wings level, turn banks up to 20 		degrees +0/-10 degrees, notes and announces buffett, first control 	decay, recovers at stall, to Vy, altitude and heading as 			previously specified by DE.

- Spin Awareness (Discussion)

Where spins occur, how to recognize and recover, how to recover in 	this aircraft.

- IFR maneuvers

Straight and level control + 200 feet of altitude, + 20 degrees of 	heading, + 10 knots, climbs and descent, turns to headings

- Unusual attitude recoveries:

Nose up--Recover with pitch down, add power, level wings

Nose down--Recover with reduced power, level wings, pitch to level

- Steep turns

Va or POH entry speed, into 360 turn with bank of +10/-5 degrees of 	45, + 100 feet, +10 knots, 	perform in both directions or as 	directed

- Ground reference

Turns around a point

600 to 1000 feet, 45 degree bank, + 100 feet, + 10 knots, to left 	turns with exit up-wind and course reversal into right turns or as 	directed

Rectangular course and S-turns

600 to 1000 feet, left then right, maxinmum bank 45 degrees +100 	feet, + 10 knots, or as directed

- Communication, ATC facilities, Radar services

Seeks, acknowledges, follows ATC directives while flying + 200 feet 	and + 20 degrees heading, + 10 knots airspeed

- Emergency procedures

Use of checklist

Emergency descent

Recognizes need for and performs descent from 3000 feet to 1500 		feet in most rapid manner withing safety limits. Airspeed + 		5 knots of acceptable speed or POH

- Emergency landing

Best glide + 10 knots

- Systems malfunction

Use of checklist

Types of power loss, roughness, high termperature, carburetory or 	induction ice, low or no oil pressure, fuel problems, electrical 		problems, flight instrument problems, gear or flap problems, 		inoperative trim, open door or window, fire in engine or cockpit, 	other

- Equipment and survival gear

ELT, battery life, FAR requirements, operation

- Night Flight (discussion)

Use of checklist

Aircraft lighting, illusions, vision, physiology airport lighting, chart 		reading, effect of age, preflight, hazards, precautions

- Pilotage and dead reckoning

Course + 3 nautical miles, altitude + 200 feet, ETA + 5 minutes, heading 		+ 15 degrees

- VOR tracking

Altitude + 200 feet, uses radials to find position, interecepts and 			tracks selected radial, notes loss of signal and acts accordingly, 		describes station passage.  Always idents VOR.

- Diversion to alternate

Accurate immediate turn to estimated heading, finds ground speed, ETA, 			fuel conditions, flys + 200 feet, + 15 degrees

- Lost procedures

Holds course, finds landmarks, uses VORs, contacts ATC for help, plans 			precautionary landing

- Post landing

Use of checklist

- Parking and tiedown

Use of checklist, safe operation, use of tow-bar, post flight inspection

# Performance sheet ASEL

Weight and blance

CG at gross takeoff______CG at gross landing____

Shift weight from______ to _____

New CG ______

Takeoff

Sea level, 30C degrees OAT, 50 foot obstacle_______

4000 feet, 30C degrees OAT, 50 foot obstacle_______

Discuss effects and factors of density altitude 				performance

Landing

3000’ PA, standard OAT, 50 foot obstacle _________

Cross Country

Distance______; time_______; fuel______; remaining____;

remaining w/reserve_____; fuel stop??___

Best endurance?_________

Range at 75% power?______

Range at 65& power?______

# Fear of failure:

A mistake will not end the checkride unless it indicates a safety problem related to procedure or judgment.  Every flight with your instructor will include instruction in using the proper procedure and judgment.  On your own, make a tape recording of normal and emergency cockpit procedures.  Explain on the tape, to yourself how the systems and instruments work, show failure, and how related problems would be solved.  If you can't explain it to yourself, see your instructor.

I have had many students fail their flight tests.  Never have I had a student say that the test or the examiner was unfair, never gave him a chance, etc.   In many instances the student had knowingly flown off course, deviated from an altitude or neglected some procedure.  They had done so knowingly, but thinking that under the test circumstances the deviations were O.K. ; they weren't.  More often the students have failed on the oral.  They failed because the examiner found a weakness in their knowledge.  Like water working a weakness the examiner kept probing until the area constituted a PTS failure.

Areas of knowledge not related to the actual flying can be errors of commission, such as a wild guess at some unknown answer, or omission, leaving out an essential word or term.  The instructor has not done the job of seeking and finding these weaknesses.  The instructor doesn't know what you don't know but should find out.  Likewise, you might not know what you don't know.   More likely, you are aware of the weakness and are unable to ask for the required help.  Some students are under "pressure" to get the test over and hope that the weakness will slip through.  It won't.  What's more, any undetected weakness can kill you.

The endorsement gives the instructor power over behavior unequaled in any area.  No person can legally solo until a single instructor certifies that he has given the required instruction, and tests to the person.  The person is certified to have met the required performance standards and will be safe.  The pilot may not do anything contrary to the endorsement limitations of the instructor.

# Private Pilot Endorsements

An instructors endorsement indicates that he feels you have been instructed in and are competent to perform a given maneuver.  At that time.  Your logbook is a legal document.  You should use post-its to mark required instructor endorsements so they are readily available to the examiner.  Mark the type tests, first solo, any 90-day, spin awareness, cross countries, night flight W/landings, written test review, and test recommendation.  You are supposed to know the requirements.  The application must be properly filled out.  Preferably typed. The examiner is under the FAA gun to submit proper paper work so you should not offer anything that will cause him a problem.

It is the instructor's reputation that is on the line when he makes the written statement certifying that the applicant is prepared to pass the practical test.  The checklist for the PTS is at the back of the PTS booklet.

Aeronautical Knowledge

FAR 61.35(a)(f) and 61 105(a) Aeronautical knowledge

I certify that I have given (name) the ground instruction required by FAR 61.105(a)(1) through (6)

Flight Instruction/Proficiency Requirement

FAR 61.107(a) flight proficiency

I certify that I have given (name) the flight instruction required by FAR 61.107(a) (1) through (10) and find him competent to perform each pilot operation safely as a private pilot.

## Take Practical Test

FAR 61.39(a)(5) Completion of prerequisites for a practical test

I have given (name), flight instruction in preparation for a (type test) within the preceding 60 days and find him competent to pass the test and to have satisfactory knowledge of the subject areas in which the applicant was shown to be deficient by his airman written test.

Test after Failure

67. FAR 61.47 Re-testing within 30 days after first failure

I have given (name) additional (type) instruction and find him competent to pass the (type) test.

High Performance Aircraft Endorsement

(Some students learn in complex aircraft.)

I certify that I have given _____#_____the instruction required by FAR Section 	61.31 (e) and find him competent to act as pilot in command of high-	performance airplanes.

Cannot be limited to aircraft type.  A person needs only to receive instruction and an endorsement for either an airplane with more than 200 horsepower or an airplane with retractable gear, flaps, and a variable 	pitch propeller, or both to qualify as PIC in any high performance airplane. A high performance aircraft is any aircraft with more that 200 hp with retractable gear, flaps and a constant speed propeller. A complex airplane is any aircraft with retractable gear, flaps and constant speed propeller.  All complex airplanes are considers high performance but not all high performance airplanes are considered complex.