# RULES OF THUMBã

Basic Rule of Thumb:

You won’t remember a rule of thumb unless you use it.

Rule of thumb:

The more near the throttle is to full power the more even leaning 	will 	be among the cylinders.

Rule of thumb

If you don’t know best glide speed use best climb. (close)

Rule of thumb:

Cruise speed for planing purposes should be increased by one knot 	for every 100 pounds below gross weight.

Rule of thumb:

1. Standard distance between runway lights is 200'

Have passenger count lights to determine your aircraft 	performance.

2. Takeoff and landing distances in the book are of maximum performance 	type.  For normal operations double the book figures.

Rule of thumb

For recovery from a bank to a given heading is to lead by half the 	angle of bank.

Rule of Thumb

Ground speed at which hydroplaning will occur is seven times the 	square root of the tire pressure.

Rule of Thumb descent: (150kt range of ground speeds)

# 1. Take 1/3 of altitude-to-lose to get distance, descent at 1/2 		ground speed.

# 2. Take half of ground speed and add a zero for descent rate.

# 3. Ground speed times 5 to get VSI rate of descent giving 3° 			slope.

# 4. 	For establishing a 3 degree slope descent is to multiply 			your 	ground speed by 5 and add 50.  Not exact but good 			enough.

Rule of Thumb

Take three times your height above destination to the nearest 1000’.  Multiply by four.  Gives descent distance in miles to begin

3-degree descent.

Rule of thumb:

To get a 3 degree descent profile multiply your ground speed by 5 to get a fpm descent.

Rule of Thumb

To change from Celsius to Fahrenheit double the degrees of Celsius 	and subtract 10% and add 32.

Rule of Thumb

A 2% increase in weight will cause a 1% change in stall speed.

Rule of Thumb

The 22 rule for Reciprocals

Take any heading and the first digit, 0, 1, 2, or 3.  Add or 	subtract 2 	to the first digit to get a 0, 1, 2, or 3 for an 	answer.  If you add then subtract 2 from the second digit, if you 	subtract then add 2 to the second digit.  Third digit remains the 	same.

## Compass Turns

Compass turns can be made by time with a standard rate of 3 			degrees per second.  Rule of Thumb compass turns:

1. When turning Northerly, undershoot the heading by the latitude

in degrees plus half of the bank angle.

2. When turning Southerly, overshoot the heading by the latitude 			minus half the bank angle.

3. When turning East or West from the South roll out 5 degrees 			early.

4. When turning East or West from the North roll out 10 degrees 			early.

Rule of Thumb--trees

Calm		smoke is vertical

1-3 kts 	smoke drifts with wind

4-6 kts	leaves rustle, you feel wind

7-10 kts	leaves in motion all the time

11-16 kts	Dust & paper move

17-21 kts	Small trees move, waves have crests

28-33 kts	Trees sway, wires whistle

34-40 kts	Branches break, difficulty walking

41-47 kts	Signs fall

48-55	kts	trees fall, damage

56-71 kts	DAMAGE

## True Airspeed

Increase indicated airspeed by 2% per thousand feet of altitude. Wind effect on speed can be determined by finding difference between ground speed from DME and your calculated true airspeed.

The TAS = IAS * the square root of (standard sea level air density /

current air density). For the usual standard atmosphere, this can be

written:

TAS = IAS*[square root (T/288)] * [(1 - h/144000)^-2.75] where T = 273 +

OAT (C), and h is the pressure altitude. read ^-2.75 as "raised to the

minus 2.75 power".

## Wind correction angle

Divide crosswind velocity component as flown by your TAS in miles 	per minute.

## Wind component

A 45 degree wind velocity should be multiplied by .7 to find its effect on your course.  Every 15 degrees of change from the 45 is adjusted by .2 . Example:

Heading of 360; wind 150 at 24kts.  Wind is 30 degrees off tail.  Tailwind multiplier is .9 (.7 + .2) and crosswind multiplier is .5 (.7 -.2)  Tailwind component is 22kts, crosswind component is 12 kts.

## Bank Angle for Standard Rate Turn

Airspeed in knots divided by 10 and add half of that value.

Example:

IAS 100kts divided by 10 = 10 + 5 = 15 degree angle of bank

## Distance to Descend

Two times the ground speed in miles-per-minute times the altitude 	to lose in thousands of feet.

Example:

Ground speed in descent 120 kts (2 miles per minute).  To lose 	5000' from 8000 to 3000 you must start your descent 20 miles out 	from your 3000' mark.  2 x 2 = 4 x 5 = 20 miles out

## Increase in Landing Distance

Actual touchdown speed  . answer x itself (squared)

Proper touchdown speed

Use the number greater than 1 as a % .  This is the % greater the   	landing distance will be.

Use of this formula is a good reason to believe in the importance of precise speed control in landing.

Density Altitude

Density Altitude (DA): temperature deviation (in deg Celsius) from ISA at THE ALTITUDE you're interested in, multiplied by 120 ft + or - depending on whether its ISA + or -. you then add this to the pressure altitude and you have an approximate DA. So for the above problem normal ISA temp at 5000' = +5 deg C (15deg at MSL minus 2 deg/1000 ft)

50 deg F = approx 10 deg C

10 DEG C = ISA +5 @ 5000 FT

5*120=600 FT

5000 FT + 600 FT= 5600 FT DA