Private Pilot Performance Standardsã

As applied by one DE(designated examiner))

I have never known a DE not to give a student a fair shake on the flight test.  Often they will give a second chance if performance does nost meet PTS requirements.  Two-time-Tommys don’t get many applicants.  If, after going over the PTS you find an area where your procedure is different talk it over with your instructor and again with the DE before flying.  There are often several interpretations of just what is wanted during the flight test.  By the time you take the test you should be able to perform to the DE’s expectations what ever they may be.

Preflight & Cockpit Management

Inspects using checklist

Passenger briefing on belts and emergency procedures from 	checklist

Engine Starting

Clear, start and after-start with checklist

Taxiing

Brake check, speed control

Knows and obeys signs, lines, and (lights)

Yoke positions for wind

Radio use and acknowledgments and compliance

Run-up

Uses checklist

Airspeed, distances review

Emergency and departure review

Pre-takeoff

Uses checklist

Takeoff configuration

Radio use and acknowledgments and compliance

Clearing approach

Traffic Patterns

+ 100 feet and +10 knots

Takeoffs

Normal and crosswind Vy + 10/- 5 knots

Soft field - in ground effect to Vy, climb at Vy +10/-5 knots

Short field - rotate at Vx or by POH,

climb at Vx, +10/-5 knots to 50 feet, then at Vy, +10/-5 			knots

Landings on final

Normal and crosswind 1.3Vso or by POH +10/-5 knots (gust factor) 	within 400 feet

Soft field 1.3 Vso or POH +10/-5 knots (gust)

Short field 1.3 Vso or POH (gust factor, with touchdown at 200 	feet past a point

Forward slips within 400 feet at ~ stall speed

Go-around

At Vy + 10/-5 knots

Slow flight

Not below 1500 feet AGL, speed as 1.2Vs1, +10/-5 knots, + 100 	feet, + 10 degrees of heading, maximum bank of 30 degrees +0/-10 	or as directed, climbs and descents at 20 degrees, +0/-10, 	headings from turns + 10 degrees, levels off + 100 feet.

Stalls

Power off, not beow 1500 feet AGL, stabilized approach, landing 	configuration (full flaps), heading + 10 degrees or up to 30 	degree bank + 0/-10 degrees.

Notes and announces buffet, first control decay, recovers at 	stall, to Vy before final flap retraction, returns to altitude and 	heading previously specified by DE

Power on, not below 1500 feet, departure configuration, heading

+ 10 degrees when inducing stall wings level, turn banks up to 20 	degrees +0/-10 degrees,	notes and announces buffett, first control 	decay, recovers at stall, to Vy, altitude and heading as 		previously specified by DE

Spin Awareness (Discussion)

Where spins occur, how to recognize and recover, how to recover in 	this aircraft

Emergency descent

1500 foot clean descent at structural cruise speed but no descent 	below 1500’ AGL.  +/- 5 knots

IFR maneuvers

Straight and level control + 200 feet of altitude, + 20 degrees of 	heading, + 10 knots, climbs and descent, turns to headings

Unusual attitude recoveries

Nose up--

pitch down, add power, level wings

Nose down--

reduce power, level wings, pitch to level

Steep turns

Va or POH entry speed, into 360 turn with bank of +10/-5 degrees 	of 45, + 100 feet, +10 knots, perform in both directions or as 	directed

Ground reference

Turns around a point

600 to 1000 feet, 45 degree bank, + 100 feet, + 10 knots, to left 	turns with exit up-wind and course reversal into right turns or as 	directed

Rectangular course and S-turns

600 to 1000 feet, left then right, maxinmum bank 45 degrees +100 	`feet, + 10 knots, or as directed

Communication, ATC facilities, Radar services

Seeks, acknowledges, follows ATC directives while flying + 200 	feet and + 20 degrees heading, + 10 knots airspeed

Emergency procedures

Use of checklist

Emergency descent

Recognizes need for and performs descent from 3000 feet to 1500 	feet in most rapid manner withing safety limits. Airspeed + 5

knots of acceptable speed or POH

Emergency landing

Best glide + 10 knots

Systems malfunction

Use of checklist

Types of power loss, roughness, high termperature, 				carburetor or induction ice, low or no oil pressure, fuel 			problems, electrical problems, flight instrument problems, 			gear or flap problems, inoperative trim, open door or 			window, fire in engine or cockpit, other

Equipment and survival gear

ELT, battery life, FAR requirements, operation

Night Flight (discussion)

Use of checklist

Aircraft lighting, illusions, vision, physiology, chart reading, 	effect of age, preflight, hazards, precautions

Pilotage and dead reckoning

Course + 3 nautical miles, altitude + 200 feet, ETA + 5 minutes, 	heading + 15 degrees

VOR tracking

Altitude + 200 feet, uses radials to find position, interecepts 	and tracks selected radial, notes loss of signal and acts 	accordingly, describes station passage.  Always idents VOR.

Diversion to alternate

Accurate immediate turn to estimated heading, finds ground speed, 	ETA, fuel conditions, flys + 200 feet, + 15 degrees

Lost procedures

Holds course, finds landmarks, uses VORs, contacts ATC for help, 	plans precautionary landing

Post landing

Use of checklist

Parking and tiedown

Use of checklist, safe operation, use of tow-bar, post flight 	inspection

Performance sheet ASEL

Weight and blance

CG at gross takeoff______CG at gross landing____

Shift weight from______ to _____

New CG ______

Takeoff

Sea level, 30C degrees OAT, 50 foot obstacle_______

4000 feet, 30C degrees OAT, 50 foot obstacle_______

Discuss effects and factors of density altitude performance

Landing

3000’ PA, standard OAT, 50 foot obstacle _________

Cross Country

Distance______; time_______; fuel______; remaining____;

remaining w/reserve_____; fuel stop??___

Best endurance?_________

Range at 75% power?______

Range at 65& power?______