# Charts and moreã

All Available Information

Aviation Charts (first)

Reading the Sectional

World Aeronautical Charts (WAC)

Historical Interlude

It's about time

Way to Go

# All Available Information

FAR 91.103 says. “Each pilot in command shall, before beginning a flight, become familiar with all availavble information concerning that flight.

This includes weather, fuel, alternates, plane performance, runway/airport information, and possible delays.  Radio frequencies, ATC services, terrain, airspace, and local services are not in the FARS but belong in your “All Available Information” kit.  Materials such as the AIM require that you have a subscription to remain current.  The FBO AIM will not satisfy the AAI requirement.  Materials are available for on site use at all ATC facilities.

First get your charts, a World Aeronautical Chart is good for planning long trips, a sectional set is required but not mentioned specifically in the FARs, a VFR Area cheart is required by FAR if you plan to fly in, under, or over Class B airspace.  The first planning step is to draw a course line on the WAC and then trasfer it over to your Sectionals and Area Charts.

Using these course lines you are ready to locate any Special Use airspace along the route to note minimum altitudes, hours of operation, and areas where extra precautions may be adviseable.  Use the Airport/Facility Directory and any available airport guide to get enroute airport information to elaborate on the rather limited chart data.  You could use ‘Post-it’s’ but I prefer a black marker to give pattern altitudes and frequencies.  This is a good time to get any special frequencies that are not normally available such as Center.  You may need access to IFR charts for these as well as IFR approach/departure courses and altitudes.  In low visibility conditions you must plan to avoid IFR routes.

All information from charts and publications may be out of date anywhere from thirteen days to six months.  The accuracy of your information must be verified by getting all local and distant NOTAMs along the route.  Local NOTAMS-L may not be available until you approach an area while airborne.  NOTAMs are the latest valid information and may extend, cancel or replace any prior information that is printed.

While enroute listen to Flight Watch on 122.0 or even on the following frequencies that are intended for high altitude aircraft but are available as an option to a pilot who is flying in isolated areas such as behind the Sierras; 135.7 Oakland, 135.92 Seattle, 133.02 Salt Lake. and 135.9 Los Angeles.  Many VORs have weather broadcasts where the B|VOR box has a small black square in the corner.  TWEBs and HIWAS may be broadcast over navaids.

# Aviation Charts (first)

The first charts were published in 1926 of the principle airways.  They were long and narrow and extended only 40 miles to each side of the airway.  1 inch equaled 8 miles. (A sectional has 1 inch to 7.8914 miles).  Water was colored blue, cities yellow, railroad black and highways white.  Airports were red circles.  Sea level land was dark green, at 1000' it became light green, at 2000' light brown, above 3000' darker brown.  The charts were a big help but so narrow that any diversion for weather could fly you off the map.

# Reading the Sectional

Open the San Francisco Sectional and locate the lower left corner that has 36° with 125° below and to the right.  Now fold the chart so that only ocean is visible except at the upper right corner which shows Point Reyes.

Leave the Legend to the right out flat as you not that the vertical line is not straight but has a slight curve.  The bottom line is also curved as you should note by the width of the white margins.  Note the caution box below the 125 ° related to uncharted hazards below 200 feet.

The 36° line is the degrees of Northern Latitude it is parallel to the 37° line about 9 inches above.  The top of the sectional goes slightly beyond the 40° line of latitude or 40th parallel.  The space between every degree on the sectional is divided into 60 divisions and each division is a nautical mile.  This is true only on the vertical lines.  There are sixty divisions between the vertical lines but they vary in distance from 48 to 45 nautical miles due to the tapering of the distance toward to pole.  You never measure distance on the horizontal lines of latitude.

Vertically between 36° and 37° there are divisions and subdivisions.

Half way between every degree either vertically or horizontally there is a full line marking the thirty minute (30’)  It is very easy to confuse a 30’ line with a degree line.  Degree lines will have their numbers to the lower left at every interior intersection.  Horizontally each nautical mile is one-minute.  From the degree line every five degrees has a marking the extends farther to the left; every ten degrees has a marking extending farther both left and right.  Use of these subdivision markings make it easier to count degrees.

Go back to the lower left corner of the sectional.  Count up the 125°

line of west longitude or meridian five spaces.  Mark this line and then repeat the process on the 124° meridian.  Use a straight edge to connect the two points with a line.  This Line is knows as the 36° 05’ N.  Go back to the left corner and count over on the 36° parallel for 15 spaces and mark the crosshatch.  Go up to the 37° parallel and repeat the count and mark.  Draw a vertical line between the two marks. This line is known as the 125° 15’ W line of longitude or is it the 124° 45’ W meridian.  Which?

We have created a problem for ourselves.  36° 05’ N worked because we were counting toward 37°.  By counting from 125° to the right we may tend to use the 15 count when it is actually the 45’ point from 124°.  I deliberately tried to expose you to one of the most common errors in locating coordinates.  You must always start your count for latitude from the bottom and the count for longitude from the right.  The next most common error is in not using a long ruler to draw your lines.  It is very easy to draw a line between two points that are not going to give a parallel line.

Where these two lines cross is a point that is unique in the world.

36° 05’ N-124° 45’ W.  Every place on the earth can be so identified and located.  Locate Mt. Whitney near Death Valley and see what coordinates you get.  Check with the coordinates given on the chart legend.  Practice at least ten different locations by giving yourself coordinates that would be located on the San Francisco Sectional.  Next mark at least ten airports

on both sides of the sectional and determine the coordinates.

To locate a particular airport for which you do not have a designator. You can put the geographical coordinates into your Loran or GPS and the navaids will work just as well.  Fifteen years ago I did this while going to Medford, Oregon from the Nut Tree.  Crossing the threshold the Loran indicated 1/2 mile to airport center. (I taught LORAN in WWII)

The quadrangles bounded by the ticked lines are but 1/4 of the area in a degree square.  It isn’t a square or a rectangle but it does measure 60’ to a side.  Every quadrangle is 30 nautical miles high but the width is quite variable.  Call them wreckedangles?  Each figure that has land has a highest known obstruction elevation.  The given figure is rounded upward to the next hundred and then an additional hundred is added.  In mountains an additional 300’ is added.  These margins may only partially correct for altimeter setting errors.  You are required by FAR to have an altimeter setting from within 100 miles.  The closer the setting location the better.

The sectional aeronautical chart is essential to all forms of visual navigation.  Your eyes use a comparison of land features to chart features

to determine how relationships compare.  When comparisons match, you know where you are.

A sectional is aligned to true north.  The Lambert Conical Projection of the chart makes all straight lines very nearly the direction you fly before figuring in isogonic variation and deviation.  This works very well when you know that the winds given by the FSS and Weather Bureau are also measured by direction from true north.

The lines of longitude and latitude on the sectional divide it into a series of wrecked-angles (sic) with 30 ticks marked on each side.  Only the north-south ticks can be used for distance measuring since the lines of latitude are parallel.  Each wrecked-angle has a Maximum Elevation Figure which, within 100', tells the MSL altitude required to miss all obstacles.

Within many wrecked-angles the topography is shown in eight colors which shade from green to dark brown to indicate altitude.  Most chart symbols resemble earth features.  Small roads are shown only when deemed useful for navigation.  Fly with the chart open and pointed in the direction you are going.   In each quarter of a degree depicted there is a maximum elevation figure.  In most areas this figure is rounded to the next higher 100' but in mountainous areas it is rounded to the next higher 100' plus an additional 300'.

Airports are either magenta (uncontrolled) or blue (controlled).  It is important that the pilot become totally familiar with the chart legend as it applies to information available at the airports and through the use of radios.  Airspace depiction is shown by types of line/color combinations to cover Classes B, C, D, E, and G.  It is illegal to fly without a current chart for the area.

# World Aeronautical Charts (WAC)

WACs have half the scale of Sectional Charts so the price is cheaper for the area depicted.  Class B and C airspace is only outlined without altitudes.  Class D and Class E surface (CZ of non-tower) is not shown at all.  Space limits have eliminated many obstacles, towns, and frequencies.  Maximum elevation figures (MEF) cover 60 nautical mile wrecked angles (sic).  WACs are best for fast airplanes or for long flight planning.

(Required Practical Test Knowledge-frequent oral test question)

WAC charts show MOAs but do not show MTRs.  The width of a MTR can vary up to ten miles to each side.  See AIM 3-41 +.

# Historical Interlude

Different Miles and How they Came to Be

Under the Roman Empire, Rome became the center of the western world.  All roads led to Rome and all distances were measured from Rome.  The distances were based upon one-thousand Roman paces of the Roman soldier.  A Roman pace is equal to two of our steps and very near 64 inches.  The Latin for thousand is mille from which we derived the word mile.

The first paths for ships were called Porotan Charts.  These were lines drawn across the Mediterranean between the coastal ports.  Where many of these lines crossed the map makers would draw wind roses.  The wind rose initially varied but settled on the eight points.  The predecessor to the compass rose and our eight wind direction terms.

Thales of Miletus (640-546BC) made a gnomonic projection (use of shadows) of the region where he lived.  Hipparchus in the 2nd century B.C. had used sterographic (showing heights) and orthographic projections (perspective).  Eratosthenes in 3rd century B.C. calculated the size of the earth circumference to be 24,000 miles.  He developed a 16 point wind rose and use of 'degree".  He also wrote a description of known world.

Ptolemy, a 2nd century Greek, made a world map and made a world size error when he calculated size of world's circumference to be only 18,000 miles. This was not corrected until 1669 by Jean Picard, 200 years after Columbus.  Eratosthenes' calculations had been lost to the western world.  Ptolemy used the first conic projection plane map with the top as north.  This made possible drawing of rhumb (one direction) lines from point to point on the globe.  He devised the 60 minute and 60 second divisions of the 360 degrees in a circle.  A mile at sea, on this world of Ptolemy, was essentially equal to a mile on the land.  The length of a statute mile was 1000  (mille, from the Latin) Roman paces.  A Roman pace is two of our steps.  Each Roman road had occasional small obelisk statues placed to indicate the distance from Rome much as Mexico today does from Mexico City.  Hence, statute miles.

A 1466 Chart of Nicolaus Germanus divided the degree into 60 equal spaces called miles.  This was based upon an earth of 18,000 mile circumference and gave us a nautical mile the same length as a Roman statute mile.   Other cartographers including Hipparchus and  Mercator gave us a world with an overlying grid with numerical markings of longitude and latitude. Gerardus Mercator (Gerhard Kremer), Flemish, in 1569 drew world globe map with 180 degrees E/W longitude  0 to 90 N/S latitude.  He made errors which were corrected by Edward Wright who published the computations required as "Meridional Parts" and made this knowledge universal.  In combination, we now had a world which could be mapped in degrees of longitude and latitude.  Each degree of longitude had divisions of 60 miles equal to a statute mile  and each mile was again divided into 60 units called minutes and each minute was again divided into 60 units called seconds.

This was the kind of map and scale used by Columbus.  The navigators of his time had not the timing device to make possible the exact determination of longitude. The best 15th Century data available to Columbus came from Ptolemy.  The error by Ptolemy directly resulted in Columbus' declaring  that he had reached and was exploring India. Columbus thought he had sailed through enough degrees of longitude to have reached India.  He may well have, had the world been 18,000 statute miles in circumference.

When the world was computed to be 24,000 statute miles in circumference all the degrees and their divisions were longer and did not conform.  More accurate computation of the world's circumference kept changing and finally came to 24,902 statute miles.  The circumference of the earth has always been measured as 21,600 nautical miles (360 degrees X 60 nautical miles per degree).   However, the individual nautical mile has ballooned by nearly a third through this recalculation of the earth's size.  For many of the same reasons the U. S. has failed to convert to metric, later cartographers decided to use statute miles for land and the expanded nautical mile at sea.

Now we can see the background for the difference between nautical and statute miles and Columbus' reasoning.  We have Columbus sailing around an earth at least 1/3 larger than he was led to believe.  Based on available knowledge Columbus was quite justified to assume that he had actually reached and explored India.

For the navigator, it is very important that distance only be measured along the lines of longitude which has evenly spaced tick marks throughout.  The elongated orange peel appearance of the region between lines of longitude means that various latitude lines will have tick marks at differing intervals although always 60 ticks per degree.  Only at the Equator do the tick marks correspond to the size of those along the lines of longitude.

Johann Henrich Lambert from Alsace devised the lambert conformal conic projection in which the line you draw is the way you go. This is the charting used on aircraft.  As with any flat map of a round surface it has areas of inaccuracy which increase in one direction or another. Sectionals are most inaccurate (stretched) in the six inches at the top an bottom.  The center ten inches of the sectional for 5 inches up to five inches down from center is somewhat contracted in size.

# It's about time

A sailing ship's speed over a nautical a mile was, historically, measured by means of a knotted (knots) rope tied to a log.  A sand filled timing glass would be used to measure the time from leaving the log dead (much as a dead man might appear) in the water (dead reckoning) and the number of evenly spaced knots passed along the rope.  All of this would be recorded in the logbook.  Since the chronometer was yet to be invented, sailors had no way to determine longitude except by this dead reckoning.  Within crude limits, speed and compass indications could be used to determine estimated distance and estimated longitude.   Magellan in 1519 had access to charts, globe, theodolites, quadrants, compasses, magnetic needles, hour glasses, and timepieces.   He was unable to determine exact longitude.

An 18th Century a chronometer (weighed over 36 pounds) was first used to get longitude.  A chronometer differs from a clock or watch because it has a temperature adjustment for greater accuracy.  Captain Cook in 1768 had three different clocks for his voyage.  In 1779 he sailed with 4 chronometers and a nautical almanac which enabled him to determine longitude.  The very first effort to make a calculator was financed by the British to make the making of the nautical almanac easier.  The effort was stopped as the mechanical calculator was only a year from being completed.  The original design was completed in 1991 and found to work accurately.

Interesting to speculate where the world would be had it been completed in the 1700s.  30 years ago I knew a pharmacist who spent his evenings at an all-night pharmacy working out prime numbers on rolls of butcher paper with a pencil.  Did we miss a 300 year head start on computers by so little?

_______________________________________________________________________

revolutions per minute

First counted by paddle wheel ship captains._____________________

# Way to Go

The pole star was recognized by the ancients as being a constant reference for determining direction.  The Norsemen in the 11th century  used a needle of magnetic iron inserted in a straw and floated on water to point to the pole star.  Petrus Peregrinus de Maricourt invented the pivoted floating compass with lubberline and sight for bearing.  The modern compass is little more than one-hundred years old..

The compass card, due to wind rose origins is older than the magnetic needle.  A names of the cardinal compass points are from the ancient terms for wind direction.

Variation was understood by 1800 as a problem.  Edmond Halley at end of 17th century mapped lines of variation and drew isogonic lines (lines of variation) on his maps.  George Graham showed that variation was subject to diurnal (seasonal) changes with variation being less in winter.

Deviation was written about in 1627 by John Smith as a problem encountered through use of metal nails in his compass box.  Captain. Mathew Flinders in 1801-2 found way to correct by use of "Flinder's Bars as did Lord Kelvin through use of Kelvin spheres.  Placement of soft iron spheres at sides of compass could be used to correct deviation.

The development of the gyro compass began in 1851 when Leon Foucault used suspended cannon shot on a long wire pendulum to show the rotation of the earth as well as the inertia of the free swinging ball.  By 1852 he had created the gyroscope but had trouble applying continuous power.  By 1900 the electric gyroscope was invented by both Elmer A. Sperry and Anschutz-Kampfe of Germany.  By 1911 gyro compasses were in use soon to be followed by gyro repeaters (selysn(sp) units) and gyro pilots.