# Cessna 152ã

Cessna 152

C-152 Procedures

C152 Checklist

# Cessna 152

You should be warned about flying in a different kind of aircraft.  There is just enough difference in a C-152 to create stress and interfer with your thought processes.  The engineering of the C-152 is similar to the 150 except that where maximum flaps are only 30 degrees a 1/2 turn of trim seems sufficient for each 10 degrees of flap. After landing and flaps up, the aircraft is properly trimmed for climb.  The C-150 is trimmed for level.

Cessna in a 152 Owners Advisory Letter has indicated that a C-152 which is run at full rich will result in a 40% increase in consumpion along with a 70 minute reduction in flight time.  The same figures would probably apply to other Cessnas.

Get a complete breakdown of how a C-150 and C-152 differ.  Trim, flaps, electrical, and speeds.  Go up and review all the airwork in a strange aircraft before doing any landings.

How to ‘calibrate’ flap/trim relationships for C-152.

C-152 to get 1500 you need to reduce to 1600 (similar to C-172's

1700.  At altitude and imaginary numbers, power to 1600, hold altitude

and heading until reaching 60 knots.  Now quickly trim for hands-off

but be sure to keep track of how much is required.  Do the same for

each 10 degrees of flap as you descend.

You may need to do this a couple of times.  You are trying to index

the flap-trim ratios required to maintain 60 knots.  Once you have

developed a stabbilized procedure for maintaining 60 knots you should

be home free.

CESSNA 152

From 65kt climb

LEVEL OFF 2 TRIM UP

DESCENT

CH-1700 RPM

10 DEGREES FLAPS 1/2 UP TRIM

20 DEGREES FLAPS 1/2 UP TRIM

30 DEGREES FLAPS 1/2 UP TRIM

# C-152 Procedures

Short Field Takeoff:

Flaps set at 10 degrees.  Release brakes and accelerate on the runway centerline.  At 54 KIAS, the nose of the aircraft is raised smoothly and quickly to Vx pitch attitude - maintain 54 KIAS.

Establish normal climb 70-80 KIAS and retract the flaps.  Since the Vy speed will vary with weight, it might be better to use the lower 70 KIAS  when solo and 80 with two aboard. An old FAA manual once said that flap retraction should be made only after passing through 200'.

Soft Field Takeoff:

Flaps set at 10 degrees.  Elevator control to full nose up position.

As the nose of the aircraft comes up to Vy pitch attitude, release elevator pressure as necessary to Maintain a maximum nose high attitude just short of dragging the tail.  Level close to to Vy airspeed before attempting to climb out of ground effect.  Fly as close to the ground as you can.  This will maximize your effective engine power and speed up acceleration. Ground effect ceases at 1/2 wing span altitude.  With a 32' wingspan you have 16' of ground effect. The wing is already over 5' above the ground.  Any altitude of the wheels over 11' will degrade aircraft performance.  Stay low, accelerate and then climb.

If necessary, establish climb at 54 KIAS to clear any obstacles.  If not, normal climb 70 - 80 KIAS based on weight.  The FAA test does not include both a soft-short field requirement so the recommendation for airspeeds is good.  In the FAA PTS guide you should accelerate to the 70 - 80 KIAS before climbing.

Retract flaps once clear of obstacles or when normal climb speed is established.  200' is former FAA recommendation before flap retraction.

Traffic Pattern.

Normal climb airspeed (67-70 KIAS) should be maintained as should full power.

Opposite the proposed touchdown point on the runway, apply carb. heat, then reduce power to approximately 1600 RPM   Plane will decelerate to 60

knots and 1500 rpm.  Trim for hands-off.

Apply 10 degrees flap and trim the aircraft for a 60 KIAS glide. in the

1990s you should not leave pattern altitude until turning base.   Noise abatement procedure suggestsw not leaving pattern altitude until turning

base.  Try to keep the neighbors happy.

Turn base leg when runway numbers are at a 45 degree angle behind wing.  This point may vary with wind conditions.   The deceleration of Cessnas when the power is reduced to settle at 1500 RPM and altitude and heading are maintained the 45 degree or "key"  position is nearly automatic.  The speed which you go though the procedures should be faster as winds become stronger.

Bring the flaps to a 20 degree position, and retrim the aircraft to maintain 70 KIAS.  The base leg is where you decide to widen out, square your pattern or angle toward the runway as a first choice.  Second choice would be putting in additional flaps if high.  After these decisions have been tried then change power.  You should try to keep as many "constants" in your approaches as you can.

Lead the turn to final so as to roll out on the extended centerline.  Accuracy in final alignment is directly related to how well the downwind was flown according to wind conditions.  Even slight crosswinds on the ground must be considered as being stronger at altitude.  Never exceed a 30 degree bank in the pattern.  Standardized 30 degree banks will allow you to

make all your turns more accurately.  A 30 degree bank only increases the G force by .15.  A shallower bank extends the "blind" time in the turn.

Apply full flaps (30 degrees) as needed and establish 60 KIAS glide. (trim as needed)  The normal Cessna landing is made with full flaps.  Full flaps improve landing accuracy.  Crosswind or strong gusty winds are the only justification for using minimal flaps.  C-152s can be slipped with flaps according to the POH.

About 1970 the FAA came out with a recommendation that power off landings not be considered "normal"  Many instances of inability to re-apply power were occurring.  The FAA now recommends that power be maintained into the actual flare and only be taken off at actual touchdown.  Plan your approaches with the understanding that power will be applied.  Then if power is lost, the removal of flaps will allow a safe power off landing.

Go-Around, Full flaps

Full power, carb heat cold.  Lock arm to hold the yoke firmly forward to prevent pitch up and anticipatory right rudder to counteract P-factor.

Establish level pitch attitude while bringing the flaps up.  Milking flaps up or by notches at speeds below 60 is best way.  Climb on reaching Vy.

Short Field Landing

The traffic pattern for a short field landing is flown the same as a normal landing traffic pattern up to the final approach.

On final approach, establish a full flap, power on descent at 54 KIAS.

Power setting should be adjusted as necessary to establish a glide path which will bring the aircraft just above any obstacles.

Once it is apparent that the aircraft will clear the obstacle  without power, slowly close the throttled and maintain 54 KIAS.  Maintain 54 KIAS down to the flare and land the aircraft with a  minimum amount of float.

Once the aircraft is on the ground, retract flaps, lower the nose gently, and apply smooth but firm brake pressure.

Note: If the final approach on a short field landing is properly planned and executed, final approach will not involve any additions of power.  A lower approach speed can be used at lower weights.

Soft Field Landing:

The traffic pattern is flown just as it would be for a normal landing until final approach.  On final, a 60 KIAS full flap glide is established.

Once the runway is assured reduce power to idle RPM and flare normally over the runway.  Increase the pitch attitude prior to touchdown and bring power up to 1200 or 1500.

Once the main gear touches the runway, continue back elevator pressure as required to keep the nose wheel off the runway whiler bringing up the flaps.  Removal of the flaps will allow the pitch attitude to increase.

Keep the power on sufficinet to keep the nose off.  With proper power application it is possible to taxi clear of the runway without the nose wheel touching.

Crosswind landing:

Crosswind landing involve flying a crabbed traffic pattern with one addition.  On final approach, when the aircraft is aligned with the runway centerline, the aircraft will begin to drift.  The upwind wing is lowered to the point where the aircraft stops drifting and (the wing) is maintained in this down position.  Rudder pressure is applied in the opposite direction of the aileron deflection to keep the aircraft nose/tail lined up with the runway centerline.  Pressure must be maintained on both controls in order to keep the aircraft so aligned.  20 degrees of flaps rather than full flaps may be used on final, depending on wind strength and (crosswind angle) component with the runway.  touchdown is accomplished with the upwind wheel of the main gear touching the ground first.  Once on the ground, apply apply increasing upwind aileron gradually to full aileron, and maintain directional control with the rudder pedals.  I would make several suggestions.  Select your flap setting early on the approach before turning final.  20 degrees flap would be maximum for a 10 kt. crosswind.  Maintain your approach speed by lowering the nose as you establish your cross-control configuration.  On touchdown, hold the yoke well over.  The wing will not touch if the plane is kept rolling straight.

Takeoff and departure stall:

Clear area

Carb heat on

Reduce power to 1500 RPM

Slow aircraft to lift-off speed (55 KIAS)

Apply full power when reaching lift-off speed and increase back pressure firmly on the elevators until the stall occurs.  Right rudder pressure must be used to overcome P Factor.  When introducing this stall limit power

increase to 2000 RPM. Full power applications during "Proficiency".

If the stall is to be done while in a turn, a bank angle of approximately 20 degrees should be established when lift-off speed is reached.  Coordinated control pressure should be utilized as necessary to maintain this bank angle until the aircraft stalls.  Limit bank to 20 degrees

regardless.  An higher angle of bank greatly exaggerates the effect of uncoordinated controls.

Recovery:

Release the back pressure and level the wings utilizing coordinated control.  Allow nose to fall to or slightly below the horizon.  Then apply full power for the recovery at Vy climb.

Approach to a Landing Stall:

Clear the area .

Carburetor heat

Reduce power to 1500 RPM and when airspeed is in the white arc, apply full flaps and establish a 60 KIAS glide.

If the stall is to be done in a turn, establish a 20 degree bank angle.  Any bank beyond 20 degrees can be a prelude to a full flap spin.

Close the throttle slowly, gradually increase back pressure until the stall occurs.  Power can also be left at 1500 RPM.

Recovery:

Release back pressure, apply full power and level wings using coordinated control.  Get the nose down to level flight. Lock your elbow against the door to prevent abrupt pitch-up  as full power is applied.

When airspeed is on the upgrade retract flaps to 20 degrees without allowing sink to occur.  At 60 knots remove rest of flaps and climb.

CESSNA 152

60 KT FULL FLAP TRIM    HOLD HEADING

GIVES 65 KT CLIMB TRIM  1 FULL TRIM UP

LEVEL OFF 2 TRIM UP     FINE TRIM 60 KTS

1500 RPM-C.H. 1700 RPM  +8 COUNT FLAPS-30 DEGREES

4 FULL TRIM DOWN=60 KT  ANTICIPATE w/HEAVY FWD YOKE

10 DEGREES 1/2 UP TRIM  HOLD HEADING

20 DEGREES 1/2 UP TRIM  FINE TRIM 60 KTS

30 DEGREES 1/2 UP TRIM

# C152 Checklist

Pre-flight Inspection

Cockpit

Log book

"AR(R)OW" documents

Control lock

Shut off  ON

Ignition OFF

master switch ON,

full flaps,fuel gauges, fan

master switch OFF

fuselage, rear

fuel drain

fuselage

empenage

rudder gust lock

rear tie down

Right side fuselage

right side fuel drain

right wing

flap

aileron (

wing walk around

main gear

wheel chock

wing tie down

Fuel VISUALLY

Fuel Cap,

nose

Engine Oil level CHECK, do not operate with less than 4 quarts

Fuel sump

Cowl

Propeller

Spinner

Air  filter

Landing light

Nose wheel strut and tire

Static port

left wing

main gear nut/cotter key)

wheel chock

wing tie down

Fuel VISUALLY

Fuel Cap

Stall warning port, pitot tube, fuel vent

wing walk around

aileron

flap,

General walk around

required by law...

Before starting engine

pre-flight inspection

Passengers briefed

Seats, belts, shoulder harnesses

Doors, closed and latched

Fuel shut-off valve ON

radios and electrical equipment OFF

brakes- test and hold

check circuit breakers IN

Mixture -RICH

Carb heat- COLD

Prime- up to 3 strokes if cold, locked

Throttle- Open 1/2" (closed if engine warm)

Propeller area

Master switch

Beacon

Ignition

Throttle-- rpm or less

Oil Pressure

flaps

Radios

Transponder

Before Takeoff

Brakes

Cabin Doors

Flight Controls

Flight instruments

Fuel shut off valve

Mixture -- RICH (below 3000')

Elevator Trim

Run up-- Throttle to 1700 rpm

Magnetos--CHECK (drop should not exceed 125 rpms, or 50 rpm 			differential)

Carb heat-- CHECK for approx. 50 rpm drop

Engine instruments and ammeter

Suction gauge

Throttle to 1000 rpm or less

Radios

Strobe/Nav lights

Throttle friction

Transponder

Brakes

Normal Takeoff

Flaps up

Full throttle

rotate at 55 knots

pitch for Vy  (67knots) during climb out

Short Field Takeoff

Flaps-- SET 10 degrees

Full throttle (above 3000' lean mixture for maximum rpm)

release brakes at 2000 rpm

tail slightly low, pitch for Vx (54 knots) until clear of obstacle

In Flight Emergency Procedures

NOTE:  procedures for engine failure during takeoff run and immediately after takeoff are in the POH and should be memorized.

Emergency landing without engine power

level the wings,

ESTABLISH BEST GLIDE (60 knots)

choose a landing spot --

CHANGE COURSE TO TAKE YOU THERE

MAINTAIN CONTROL OF THE AIRCRAFT

check the following items:

primer--CHECK in and locked

master switch---ON

ignition--SET TO BOTH

carb heat--ON

mixture-RICH

fuel shut off--ON

fuel quantity--CHECK

oil pressure/temp--CHECK

Committed to a Forced Landing

Transponder--SET to 7700 and ident

COM radio --SET to emergency frequency 121.5

aircraft type and tail number

position and altitude

problem you are having

number of persons on board

intended landing spot

Seat belts ON and SECURE

Mixture-IDLE CUT-OFF

Fuel shut off valve-- OFF

Ignition switch--OFF

Flaps--UP until landing is assured

Master switch -- OFF

cabin doors-- OPEN SLIGHTLY

tail low

apply brakes heavily

Ditching

Transponder--SET to 7700 and ident

COM radio --SET to emergency frequency 121.5

aircraft type and tail number

position and altitude

problem you are having

number of persons on board

Jettison or secure heavy objects in baggage area

Approach:

High winds, heavy seas--INTO THE WIND

Light winds, heavy swells--PARALLEL TO SWELLS

Flaps--30 degrees

Power-- ESTABLISH 300 ft/min DESCENT AT 55 knots

Cabin doors-- UNLATCH

Touchdown-- LEVEL ATTITUDE AT 300 ft/min DESCENT

Face--CUSHION at touchdown with coat

EVACUATE aircraft through cabin doors

Life vests and Raft--INFLATE

In Flight Fires

Engine fire

Mixture--IDLE CUT-OFF

Fuel Shutoff Valve--OFF

Master switch--OFF

Cabin heat and air--OFF (except wing root vents)

Airspeed--85 knots (or whatever it takes to put out the fire)

Forced Landing-- EXECUTE as described in Emergency Landing Without Engine Power

Electrical fire

Master switch--OFF

All other switches (except ignition switch)--OFF

Vents/cabin air/heat--OFF

Fire Extinguisher--ACTIVATE (if available)

WARNING--After discharging an extinguisher in a closed cabin, ventilate the cabin

If fire appears out and electrical power is necessary to continue flight:

Master switch--ON

Circuit breakers--CHECK for faulty circuit DO NOT RESET

Radio/Electrical switches--ON one at a time, with delay until short is localized

Vents/ Cabin Air/ Heat--OPEN when ascertained that fire is completely extinguished

Cabin Fire

Master switch--OFF

Vents/cabin air/heat--OFF

Fire Extinguisher--ACTIVATE (if available)

WARNING--After discharging an extinguisher in a closed cabin, ventilate the cabin

Land the airplane as soon as possible to inspect for damage

Wing fire

Navigation light switch--OFF

Strobe light switch (if installed)--OFF

Pitot heat switch (if installed)-- OFF

NOTE:  Perform a side slip to keep flames away from fuel tank and cabin.

Land as soon as possible with flaps retracted

Icing

Inadvertent Icing Encounter

turn pitot heat switch ON (if installed)

Turn back or change altitude to obtain an outside air temp that is 		less conducive to icing

Pull cabin heat control to full out to obtain maximum defroster air

Open throttle to increase engine speed and minimize ice build up on 		propeller blades

Lean mixture for max rpm's if carb heat is used continuously.

Plan a landing at the nearest airport.  With extremely rapid ice 			build up, select a suitable

"off airport" landing site.

With an ice accumulation of 1/4 inch or more on the leading edges, be 		prepared for

significantly higher stall speed.

Leave flaps retracted. Flap extension could result in a loss of 			elevator effectiveness.

Open left window and, if practical, scrape ice from the windshiled 		for visibilty

Perform a landing approach using a forward slip, if necessary, for 		visibility

Approach at 65-75 knots depending on the amount of ice accumulation

Perform landing in a level attitude

Landings

Normal landing

Airspeed 60-70 knots (flaps up)

flaps-- AS DESIRED (below 85 knots)

Airspeed--55-65 knots (flaps down)

Power--REDUCE to idle as obstacle is cleared

Touch down--MAIN WHEELS FIRST

Landing roll--LOWER NOSE GENTLY

Braking--Minimum required

Short field landing

Airspeed 60-70 knots (flaps up)

flaps--30 degrees (below 85 knots)

Airspeed-MAINTAIN 54 knots

Power--REDUCE to idle as obstacle is cleared

Touch down-- MAIN WHEELS FIRST

Brakes--APPLY HEAVILY

Flaps--RETRACT

Balked landing (Go around)

Throttle--FULL OPEN

Carb heat--COLD

Flaps--RETRACT ONE NOTCH

Airspeed--55 knots

fly over the right edge of runway for traffic visibility

Flaps--RETRACT each notch when altimiter shows positive climb

After landing

Flaps--RETRACT

carb heat--COLD

Transponder--OFF

Engine Shut down (O-235-L2C only)

Throttle--1800rpm for 18 seconds

Throttle--1200rpm

Mixture--Idle Cut off

Securing airplane

Radios and electrical equipment--OFF

Mixture-IDLE CUT-OFF

Ignition switch--OFF

Master switch--OFF

Control Lock--INSTALL

Rudder gust lock--INSTALL

Tie downs--INSTALL

Pitot cover--INSTALL

Cabin cover--INSTALL (if available)

Log books--FILL OUT

CLOSE YOUR FLIGHT PLAN