Cessna 182ã

CESSNA 182 RG Transition

Adequate Checkout

Checkout in 61X

7561X

Retractable landing gear

The standby Vacuum System in 61X

Constant Speed Propeller

Leaning for takeoff

C-182 accidents

Emergency procedures:

Flight Transitions Chart

Another Checklist

COMPLETION OF TRANSITION TRAINING

FOUR DIFFERENT CHECKLISTS.

# CESSNA 182 RG Transition

Build on what you know about previous airplanes.  Standardize your procedures.  Increased complexity requires increased maintenance and decreased reliability.  Upgrade your proficiencies to meet aircraft requirements.  Flying the airplane will be harder until you understand how everything works.

Customize your checklists and procedures starting with the POH

Modify your checklists at least five times to make them efficient and useable.  Learn the performance figures for VFR and IFR.  This means set configuration, set power, set trim.  You are not competent until you are smooth.  Just because the aircraft systems are similar does not mean that they are the same.

A faster aircraft requires that this information be processed more quickly and accurately.  A fast aircraft can  get you into trouble much faster.  Mental and emotional saturation of the pilot occurs when the pilot is unable to keep up with the aircraft.  Unfamiliarity with an aircraft is one of the primary causal factors in accidents.  Accident levels are over twice as likely when you are below 50 hours in type.  If you forget the gear you have probably forgotten something else.  Use POH for engine operation, fuel pump, and performance recommendations.

# Adequate Checkout

Unfamiliarity in type is a greatly under-recorded accident type.  Low time in type accidents occur due to misuse in areas of engine operation, systems operation, emergency procedures, and flight peculiarities.  Reliance on the previous owner for your checkout is intellectually equivalent to going to a witch doctor.  Get the most experienced instructor you can for your checkout.

Converting to a new type usually involves converting to different engine operating procedures.  The more similar the engine the easier will be the engine checkout.  You must know the operating requirements, what to do, and what not to do.  Even aircraft of the same manufacture will have different fuel systems, capacities and consumption.  Learn the fuel system as it relates to the engine.  Fuel consumption figures seldom, if ever, match POH performance figures.  Run your own performance tests on every flight you make until your estimates approach actual.

Cockpit familiarization is important.  Know the how and where of all knobs, catches, latches, and controls.  Be prepared to ask about instruments and how to use them.  Seats, belts, doors, and luggage operation need to be worked out in conjunction with a weight and balance problem.  Practice operating the cowl flaps while keeping your eyes over the cowling.  Some pilots will have more difficulty than others.

Speaking of cowl flaps, consider reversing the leveling off checklist so that cowl flaps are closed shortly before leveling off and accelerating to cruise.  This will serve to reduce shock cooling the engine.  Likewise, consider not closing the cowl flaps as part of the post-landing checklist.  Allow the engine to warm up after its cooling descent to landing.  Give the engine about three minutes of taxi or shutdown time to adjust temperatures.  Select cowl flaps during this period to keep temperatures even.  Do anything you can to avoid extremes of heat and cooling.  Don't move the propeller after shut down until the metals have cooled down for at least an hour.  Differing expansion rates make warm engines have very tight tolerances.

The C-182 RG has rubber bladder fuel tanks.  These tanks have inherent problems for preflight and operation.  Preflight draining the fuel sumps is a two-man operation.  While the tail is held down, the wings should be rocked to move any water toward the sumps over ripples that often exist at the bottom of the fuel bladders.  Continue to hold the tail down while draining the sumps.  Drain the sumps a second time after the preflight is completed.

It is possible to get very mistaken fuel readings in a C-182.  The rubber bladder may collapse inside the wing.  Gauges will read full while you may have only a half-tank of gas.  Gauges may continue to read full when you are half empty.  Watch the fuel gauge movement to be able to separate normal from abnormal in individual aircraft.  The ramp and nose strut position of A C-182 can make considerable difference in the amount of fuel that the tanks will hold.  Start at level in all directions and see what differences occur and how they occur on slopes.  The tanks are inter-connected and you may need to go back to refill each tank a second time to get a maximum fuel capacity.  The only time you can have too much fuel is when you are on fire.

Several items of engine operation are worth noting.  Closing cowl flaps on the ground does not aid engine heating.  Over square operations are permissible and the additional full throttle fuel aids in engine cooling and reduces detonation.

Shock cooling can be reduced by making sure cowl flaps are closed and planning your descent from far enough away.  Begin gradual power reductions in cruise so engine cooling beings before descent.  Reduce manifold pressure in small increment every several minutes.  Keep the power in the green and mixture in the lean during any descent.  Smoothness is the key.

Emergency procedures, gear operations, and cruise operations must be covered on the ground and then be enhanced in the air.  Only 10% of the checkout time will be flight time.  In flight and trimmed for climb do a series of Dutch rolls to get a feel for control response.  Go to level cruise downwind and get a radar readout of ground speed.  Reverse direction and get another readout.  The average should give a no-wind ground speed.  This can also be done with LORAN and GPS.

Practice both slow flight and minimum controllable a couple of times before doing a stall series.  Using the speeds you have recorded, simulate both left and right landing patterns at altitude to a simulated touchdown and go around.  Go to an airport and make several full stop-taxi back landings.  After the flight review the flight and the flying.  Plan a second flight to polish any rough edges and do some work on other than normal landings.

The first surprise in flying the C-182 is the torque, P-factor, and acceleration on the first takeoff.  Prior warnings don't seem to make a difference.  Greater rudder pressures are required for all slow flight and minimum control maneuvers for proper coordination.  All controls will feel heavy.  Use of trim is essential.  Approach speeds will be higher.

# Checkout in 61X

FAR 61.31(e) requires instruction and logbook endorsement.  Insurance requires instruction and time in type.

A safe checkout requires that the instructor and student be familiar with the aircraft and its systems.  Begin by exchanging past experience, training, and qualifications.  Do a page by page review of the POH and work through the performance charts and weight and balance figures.  Review the systems both in the POH and in the aircraft as to operations, limitations and special considerations such as emergency factors.  A written quiz is desirable.  The quiz checks both that the instructor has covered the material and that the student has retained the required knowledge.

Begin the aircraft check in the cockpit.  Go over instruments, radios, switches and knobs.  Take at least an hour in the cockpit if the aircraft is well equipped.  Preflight should emphasize location of inlets, outlets, problem areas, switches, and potential hazards some of which may be peculiar to this make and model.  At least two flights are desirable.  The first flight is to confirm normal operations and procedures.  The second flight should explore the outer performance parameters and emergency operations.

Every aircraft make and type has a frequency of certain pilot errors.  The generic error is usually systems management as it is affected by decision making.  The type of flying will affect fuel management skills.  Long cross-country trips flown at different altitudes and power settings require vastly different computations since high or low can make up to 40% difference in range.

# 7561X

The NRI Board has that "touch and go" landings can be made in 7561X as long as the gear is not cycled in the process.  In addition, any pilot found responsible for "flats" on the tires are to be assessed $100.00 for tire damage.  Always check the tires before and after flying.  If damage  exists before you fly get a witness for verification.  The ways to avoid such damage are: (1) make touch down at minimum controllable speed,  (2) to avoid application of brakes while flaps are down, or (3) not to leave the runway with excess speed or excessive braking.

Flaps

Flap system damage occurs after repeated extension AT maximum approved speed.  Extension at the high speed limit causes increased system wear, damage, and maintenance.  61X received major flap damage when flaps were extended on the

ground in extremely windy conditions.

## Fuel tanks

The bladder type fuel tanks of 61X can, under certain conditions, give unreliable fuel quantity indications.  Check with the maintenance officer to determine last time fuel capacity has been checked (last annual?)    Bladders can be checked by mirror  to find wrinkles or clip detachment.    Never assume that full guages means full capacity.  Bladder equipped aircraft have a way of running out of fuel before reaching what would be a normal flight distance destination.  Different years of C-182RG aircraft have fuel capacities that vary from 50 to 81 gallons.

Are your tanks really full when they are topped-off?  The nose strut inflation, slope of the tarmac surface can make many gallons difference in what is there and in what is indicated.  Leave extra margins when you first start in type until you become familiar with any fuel gauge eccentricities.

The better you know the systems of a given aircraft type, the better you can

pick up on individual aircraft differences.

Oil

False oil level readings are also common to 61X.  This is because of the small o-ring on the  dip stick.  The o-ring forms an oil/air seal when it is removed and then full inserted.  The o-ring forces the oil down in the dip stick tube.  Immediate withdrawal will give a low reading.  It is important to wait a minute or longer before taking the dip stick reading.  This will allow the oil level to rise and give a true reading.

Gear warning

The gear up warning horn is set to come on at power reduction to about 15 inches as  for landing if the gear is not lowered.  However, it is possible to apply full power and cause an unintentional activation of the gear horn.  This problem is caused by the throttle linkage.  A very slight reduction of throttle will shut off the gear horn.  If the horn comes on at cruise power settings, the application of full throttle and then a reduction to desired power seems to correct the problem.

# Retractable landing gear

The kind of gear is determined by aircraft structure.  Cessna's complex gear is becoming more reliable and less complex.  Mooney's manual gear has become electric.  Retractables are electric, hydraulic or a combination.  Hydraulic systems are lighter but pressures are high and subject to leaks.   Cherokee gear is held down by over-center locks and retracted and held up by hydraulic pressure with no mechanical uplocks.

Emergency extension just requires release of pressure.  Cessna gear has down and up locks.  Complex electrical systems with switches to prevent ground retraction are required.  Unless you find a specific prohibition in the POH, it is a good idea to occasionally practice emergency gear extension.  Use the manual and follow the procedure precisely.

Hydraulic systems must be checked for leaks.  Over-center cams must be tight.  Switches must be clean and connections tight.  Lubrication showing is of no value.  Check for cracks.

Do not press gear operating speeds since a slightly lower gear lowering speed decreases by a factor of four the operating pressures against the mechanism.  If the gear lights don't come on, first check the lights for proper operation.  Wait before recycling the gear since you may have a fluid leakage problem.

The 182 RG system uses an electric motor to run a hydraulic pump.  The pressure from this pump is routed to hydraulic rams which are connected to the landing gear legs.  Each gear operates independently so it possible to have the nose gear extended but not the mains.  A gear up landing with the propeller stopped on horizontal does little damage.  61X has made such a landing.  It was not called an ‘accident’ by the FAA because of very minor damage.  However, the insurance company was unhappy.

The dual electric/hydraulic system is dependent on a complex system of valves, switches and locks.  Air in the pump can cause it to fail.  Lack of fluid or an excess of air will make the emergency extension inoperative.

The 182 RG uses two lights to indicate gear position.  Orange for gear up and green for gear down.  Lights can be exchanged to check if one is burned out.  When the power is reduced below 12 inches the gear horn is activated.  Sometimes the gear horn can be very intermittent giving a "static-like" noise which does not sound much like a horn.  The gear systems are full of malfunction opportunities, but 42% are due to pilot error.  Trying to extend the gear at the last minute is a mistake since you may not have enough time for a full lock.  Go around and start over.

I have had a Cessna gear experience where it would not lock down.

the wheels kept cycling about a foot forward and back and the green light would not come on.  We (my student and I) could not determine the cause, nor could our radio contact with an FBO.  We chose to make a NORDO landing because every time the master was on the gear would begin cycling.  We lowered the gear with the hand pump with the master off.  While the student pumped to keep the gear full down, I made a very full stall landing since the gear would remain locked once the aircraft weight was on it.

Cessna Pilots Association lists an experience where a Cessna gear would not move forward to lock into position either due to low pump pressure

from both electric and hand pump.  Locking was achieved by doing power on stall just as gear handle was moved down.  While plane was nose-down and before recovery all three gear moved forward and locked, and gear-down light came on.

I suggest that the pilot who makes a practice of mixing his flying with retractables and stiff-legs always call the "gear down" for every aircraft type.  Personally I call the "gear" three times, at pattern altitude when it is lowered, base check for "Green and wheel", and final.

You should know how to check the operation of the C-182 gear system.

If you have not read, studied, and made notes as to how the system works,

do so before your checkout.  Your best shot at making a malfunctioning system work is the first time.  If the gear malfunctions climb until clear and work on the problem.  Read the manual and slowly, carefully follow the recommended procedures.  If that should fail you could slow the aircraft and apply negative G-loads by flying a roller-coaster.  Positive Gs can be applied in steep turns.  Keep the airspeed low in performing the maneuvers to avoid over stressing the structure.

For all takeoffs the gear should not be retracted until there is no possibility of aborting to the departure runway.   For IFR situations it is

better to get the gear down earlier than later.  Get the aircraft configured and trimmed before the FAF.  On a short field takeoff over an obstacle you will be better off to leave the gear down until clearing the obstacle.

# The standby Vacuum System in 61X

The vacuum pumps usually last 400 hours and are replaced only on failure.  An IFR aircraft without some sort of backup faces potential trouble.  There is no way to practice using 61X's backup vacuum system.  The system can only be made to function during an actual failure (or having an A & P disconnect the vacuum pump).  The plus side of this system, however, is that in the event of failure you do get a warning light located on the far left of the panel.  This certainly beats having the system fail without warning while you follow an attitude indicator and heading indicator of ever decreasing reliability.

The system in 61X uses the difference in pressure between the static (atmospheric) pressure and the intake manifold pressure.  Unfortunately, the more power the engine develops the less pressure differential and the less pressure available for spooling up the vacuum gyros.   If a vacuum failure should occur the warning light on the extreme left of the panel will come on.  Pull the backup vacuum knob to open the valve to the intake manifold.  Reduce power to as near 12 inches of manifold pressure as you can.  The lower manifold reading you get the faster the gyros will spoolup. The system does not work without engine power.   The idea is to get the gyros to full speed and the instruments operating correctly.  You can then resume normal flight for a few minutes.  The further below full power you remain, the longer the gyros will retain their speed.  The spooling process will need to be repeated every few minutes for as long as the vacuum powered gyro instruments are required.  If not VFR head for VFR.

# Constant Speed Propeller

A constant speed propeller can change the pitch angle to allow full engine rpm at both cruise and slow speeds.  The available pitch angles are both more shallow and steep than that available on a fixed pitch propeller.  Using a fine pitch (related to going up a flight of stairs with many small steps) you can get full rated rpm and power for takeoff and climbs.  At cruise you can get a large propeller bite without having to run at high rpm.

You are able to select an rpm by adjusting the oil pressure to a semi-hydraulic pump in the propeller governor.  This pump works against springs to give a higher pitch.  Reducing the pressure allows the springs to flatten the pitch.  In a failure mode the propeller returns to a flat pitch.  This will cause the engine to overspeed.  If during runup the cycled propeller is very slow to lose rpm it means dirty oil.  An unplanned rise in rpm is symptomatic of decreasing oil pressure.  Engine failure and a runaway propeller is not far behind.  The emergency procedure in this event is to reduce power and raise the nose.

# Leaning for takeoff

Often the EGT instrument does not seem to work properly.   You can lean a constant speed propeller for takeoff power using the following technique.  Constant speed props have a fine-pitch and course-pitch stops that limit the range of available blade angles.  To use rpm as an indication of power, you have to use an engine speed below the fine-pitch stop.   Set about 2000 rpm and lean to get a 100-150 rpm increase.  This is peak power.  Doing this at the lower rpms will keep the pitch from changing.

Over square operations

Engine wear and fuel savings can be achieved by reducing rpm and increasing manifold pressure to maintain horsepower.  Modern engines have approved power ranges for which this practice is o.k.  These ranges are not

in the POH but can be found in the engine manufacturer’s handbook.

# C-182 accidents

The C-182 has 9.23 accidents per 100,000 hours of operation, somewhat higher than other makes of the same class.  In IFR conditions the rate is much lower than other makes at .09 per 100,000 hours.  Night accidents come in at 10.66 per 100,000 hours with non-IFR rated pilots making 75% of these.  Carburetor icing is a significant cause of C-182 accidents as well as density altitude operations.  Pilot complacency and over-confidence in the aircraft capabilities is the factor.  The Cessna 182 is not spin-certified since it cannot meet spin recovery requirements.

Landing accidents are at a much higher rate than similar aircraft of other makes.  The accidents usually do more damage to the aircraft than to the occupants.  The C-182 is heavy on the controls and this heaviness continues (unexpectedly) into the flare and touchdown.  Nearly half of C-182 landing accidents are due to hard landings that result in collapsed nose gears and bent firewalls.  The best solution to this difficulty is to always land with power on.  25% of the landing accidents are due to running off the end of a runway.  Almost as many are caused by crosswind incidents.  The go-around remains the best pilot option when things get beyond your abilities.

# Emergency procedures:

For the unfamiliar pilot there are several options that will make some

encounters and situations more resolvable.  If you get ‘behind’ the airplane or encounter weather/turbulence conditions, reduce power and lower the landing gear and put in 10 degrees of flaps or no flaps if turbulence is severe.  The greatest problem a pilot faces is that any loss of control may result in a high speed descent where control inpust may damage the airplane and make it uncontrollable.  Get slowed up sooner rather than later.

In some situations such as engine failure on takeoff or elsewhare it may be

desirable to extend the glide.  If the engine loses power you can retract the landing gear, remove any flaps, pull the propeller knob out to the full coarse setting.  Lastly, you could raise the nose to give the propeller an

opportunity to stop and possibily set is horizontal with the starter.

# Flight Transitions Chart

Flight Condition	Cowl	Flaps	Carb	Manifold	Prop	Speed Trim

Gear Position     Flaps			Pressure	rpm	Knots	+ = Nose up

Climb			open	  0	off	24"		2300	90	Neutral

Gear up

Level Cruise	Closed  0	off	24"		2300	140	N -1.3 down

Gear up

Cruise Descent	Closed  0   on    19"		2300	140	N - 1.3 ??

Gear up

Approach Level	Closed  10  on	19"		2300	100	N ??

Gear down

Approach Descent  Closed  10  on	15"		Full	100	N

Gear down

Non-Precision Descent

Gear down   Closed  10  on	12"		Full 	100   N

Holding

Gear up     Required 0  on    15"		2300    90  N + 1

Question: What is the suggested gear position during a C-182 RG go-around?

Leave the gear down.

Question: What is required to actually demonstrate the operation of the 		vacuum backup system?

Vacuum pump must be disconnected.

Question: What level of pilot proficiency should be expectedly the end of an 		aircraft checkout?

Performance to private pilot level.

# Another Checklist

Start				  Runup			IFR APPROACH  T T T T T T

CONTROLS FREE		  CONTROLS FREE	       TURN TIME TWIST

FUEL BOTH			  INSTRUMENTS

MIXTURE RICH		  AMMETER & FUSES        	CONFIRM..PLATE UP

COWL OPEN			  OIL TEMP		SET..VOR/ILS/NDB/DME/MARKER

CARB HEAT IN		  OIL PRESSURE	       		CONFIRM PRESSURE

PRIME 2-4 PUMPS (1 HOT)	  SUCTION		CONFIRM HEADING

MASTER ON			  FUEL PRESSURE	IAF/ALT/TIME/DIST TO MDA

THROTTLE 1/2"		  ART HORIZON		 	SET TOWER FREQ

FUEL PUMP ON-OFF		  D.G. WITH COMPASS	MISSED..HEADING ALT

CLEAR PROP			  BOTH TANKS

START-LEAN FOR TAXI	  ALTIMETER ERROR

FUEL PRESSURE		  RUNUP 1700 RPM

OIL PRESSURE		  MAGS CHECKED			LANDING

SUCTION			  CARB HEAT			GEAR DOWN/GREEN LITE

RADIOS			  CYCLE PROP 300 RPM    	CARB HEAT IN

SET COMS #1 & 2	  CHECK IDLE		PROP FULL

SET NAVS #1 & 2	  TRIM			MIXTURE RICH

SET FUEL CALC	  TRANSPONDER TO ALT		COWL CLOSED

SET ADF & LORAN	  INTERIOR		FLAPS SET

SET MARKER BEACON	  WINDOWS CLOSED	SEAT BELTS

TRANSPONDER 1200/STANDBY  DOORS LATCHED

COM SELECTOR AUTO/PHONE	  SEAT BELTS

ATIS				  PROP FULL

TAXI CLEARANCE		  STROBE/BEACON/LITES

IFR AUX VAC ON		  TIME OFF				SHUT DOWN

IFR PITOT HEAT							RADIO 121.5

MASTER RADIO OFF

TAXI				  TAKEOFF			LIGHTS OFF

NEEDLE/BALL			  MP CHECK			LEAN TO OFF

MAG COMPASS			  TAP BRAKES			MASTER OFF

SET FLAPS 10-DEGREES	  	RETRACT GEAR & FLAPS  		MAGS OFF/KEYS OUT

PITOT HEAT AS REQUIRED    MP TOP OF GREEN @ 85K 		YOKE PIN

FUEL SELECT -RIGHT

CRUISE			PITOT COVER ON

MP @ 23"

PROP @ 2300

LEAN

CLOSE COWL FLAPS

CLEARANCE                           TUNE AND VERIFY RADIOS

DEP ROUT			COMM 1  NAV 1   OBS  MDA    TIME

TRANSITION

VIA

VIA

VIA				COMM 2  NAV 2   OBS  MDA     TIME

DIRECT

ALTITUDE  CLIMB & MAIN

DEP FREQ

SQUAWK			NDB     HEAD     RW   MDA     TIME

TIME OFF

___________________________________________________________________

ATIS-INFO				ATIS INFO

FREQ						FREQ

CEIL						CEIL

VIS						VIS

TEMP						TEMP

DEW						DEW

WIND						WIND

PRESS						PRESS

ILS/LDA						ILS/LDA

RUNWAY						RUNWAY

MISC							MISC

GUMP, GRUMP, GRUMPS

Gas, (Radio), Undercarriage, Mixture, Propeller,(Seatbelt)

CHECKOUT

TRAINING PLAN FOR TRANSITION TO HIGH PERFORMANCE AIRPLANES Name____________________Date_________

Grade of  Certificate   ____________ Ratings and  Limitations _________________

Class of medical_______Date__________

Total flight time:____________ Aircraft to be used___________N#______________

Training at____________

GROUND INSTRUCTION

Airplane handbook and flight manual

General

Limitations

SYSTEMS AND PROCEDURES

Cockpit

Controls

Engine/propeller operation

Fuel

Gear

Flaps

Electrical

Hydraulic

Noise control

Ice/oxygen

FLIGHT PLANNING FOR AIRCRAFT

Performance data

Weight and Balance

Instrument operations

Servicing

CHECKLIST AND OPERATIONAL PROCEDURES

Pattern operation

Local procedures

Procedures to be covered in flight according to Practical Test 	Standards.

FLIGHT INSTRUCTION

PREFLIGHT INSPECTION

CHECKLIST AND PRESTART

STARTING

Normal

External power

NORMAL OPERATIONS

Taxiing

Pre takeoff check

Normal takeoff

Climb, checklist

Cruise, checklist

AIR WORK

Turns

Slow flights

Stalls

Systems emergency

IFR Unusual attitudes

Emergency descent

ARRIVALS

Descents, checklist

Landings

PATTERNS

All takeoffs and landings

Go-arounds

Aborted T/O

No flap landing

INSTRUMENT PROCEDURES

POST LANDING AND SHUTDOWN

TOTAL HOURS ____________

# COMPLETION OF TRANSITION TRAINING

PILOT ENDORSEMENT Remarks:________________________________________________________________________________________________________________                      (Instructor)_________Date________             I have received transition training to high performance airplanes and completed the ground and flight training noted above.      ___________________(signed) Date__________Duration of the flight._______

INSTRUCTOR ENDORSEMENT     Remarks:__________________________________________________________________________________________________________________  ____________________Date________Eugene L. Whitt      #1876572 CFII Expires 1-96 I have received transition training to high performance airplanes and completed the ground and flight training noted above.      ___________________(signed) Date__________

Copy to NRI Safety Officer

Endorsement for a pilot to act as PIC in high performance airplane.  FAR 61.31e

I certify that I have given flight instruction in a

high performance airplane to Mr./Ms __________,holder of

pilot certificate #__________, and

find him/her competent to

act as PIC in high performance airplanes.

SS     [date}       J.J. Jones  #__________CFI   Exp________

FLIGHT CHECKOUT PLAN AND CHECKLIST

Name________________________________________Date_____________

Grade of Certificate __________________#_____________________

Ratings and limitations______________________________________

Class of Medical __________________________Date______________

Total flight time________________Time in type________________

Aircraft: Make and model__________N#___________________

Location(s) of checkout ____________________________________

Ground Instruction___________________________________________

_____________________________________________________________

_____________________________________________________________

Review of maneuvers and procedures:

Handbook

Weight and balance

Radios

Preflight

Start and engine operation

Taxiing

4 basics

Stalls

Takeoff and landing

Cross country performance

Flight Instruction_______________________________________________________________________

______________________________________________________________________

COMPLETION OF CHECKOUT

Remarks

_____________________________________________________________

Signed __________________CFI #_____________Expiration________

I have received an aircraft checkout which consisted of the

ground and flight instruction noted above.

Signed_______________Date________

A suggested checklist.  There are other lists in the plane.  It is best that you make and use one that  follows your mode of operation.  All items in the Operating Manual checklist must be covered.  Any checklist should go through several revisions.  The final revision should be made so

that it is functionally in line with the way you operate the aircraft in all flight phases.  This is a "touch and verify" type list.

Another CESSNA 182 RG Checklist

Preflight			Prestart			Start

Gear down-green		Preflight-complete		Carb heat-cold

Avionics-off			Seats-belts-harness		Prop-high

Fuel quantity			Doors				Prime

Selector-both			Selector-Both			Mixture-rich

Baggage door			Avionics-set-Auto pilot	 Pump on/off

Tires				Electrical-off			Throttle 1/4"

Oil 6-8 quarts			Brakes-test-set		Master

Tanks-sumps-rock plane		Circuit breakers		Clear/start

Gear doors			Gear lever down		800 rpm

Lean

Flaps-up

Avionics

Pressure

Before Takeoff			Normal takeoff

Seats/belts/doors/windows		Flaps 0-20		Climb

Brake-set				Carb heat-cold		Square

Controls				Full throttle		 Power

Trim-set				Rotate 50 kias		 Prop

Selector-both			Climb 70/20 degrees 		Flaps-up

Mixture-rich			      80/0 degrees		Lean

rpm								Vx 54

Mags-L-R							Vy 80

Carb heat							Cruise 95

Prop-cycle

Idle

Flight instruments			Post landing

X-ponder				Flaps/flaps

Lights					Carb heat

Throttle friction			Lean

Brakes

Time

Cruise	Descent		Landing			Go around

Power			Seats/belts		Power

Prop			Selector-both		Carb heat

Mixture			Mixture	Cowl flaps	Flaps 20

Cowl flaps/Flaps 	Gear-green		70KIAS

VSI		       10 @140			100Kts Prop

95			Cowl flaps

Emergency

At takeoff

flaps up

full flaps

Maneuvering

to 89 at 1950lbs

Glide at gross

Another Checklist

Start			Runup			Landing

Controls free		Controls		Gear down

Selector-both		Instruments		Carb heat

Mixture			Oil			Prop full

Cowl flaps-open	temp			Mixture rich

Carb Heat-in		pressure		Cowl closed

Prime			Suction			Flaps set

Master			Fuel pressure		Seat belts

Throttle 1/2" 		AI

Pump on-off		HI			Shut down

Clear/start		Selector-both		Radio 121.5

Lean for taxi		Altimeter		Master radio off

Fuel pressure		error			Lights off

Fuel quantity/set	Runup			Mixture lean to off

Suction			1700 rpm		Master off

Radios Set		Magnetos		Mags off/keys on cowl

ATIS			Carb heat		Yoke pin in

Clearance 		Cycle prop		Pitot cover

Idle

Taxi 			Trim			Cruise

Needle/ball		 Transponder to alt		23 inches

Compass			 Interior windows/doors	Prop 2300 rpm

flaps			Prop full			Lean to 1450 TIT

Strobe/beacon/lights	Close cowl flaps

Note time off

A suggested checklist

There are other lists in the plane.  It  is best that you make and use one that  follows your mode of operation.  All items in the Operating Manual checklist must be covered.

-EXTERIOR- 						   PRE-PREFLIGHT

MAGS

1. L SUMP           15. E INSIDE            25. L TANK   	GEAR

2. L FLAP           16. OIL 6-8             26. WARNER   	CONTROLS

3. LUGGAGE          17. SUMP                27. PITOT    	LOG TIMES

4. ANTENNAE         18. PROP                28. VENT     	PAPERS

5. EMPENNAGE        19. SPINNER             29. TIP      	RADIO MASTER

6. SURFACES         20. GEAR                30. L AILERON  	SWITCHES

7. CHAIN            21. DOORS               31. GEAR            TRIM

8. TAB              22. 3 TIRES                                 MASTER-on

9. R FLAP           24. STATIC AIR                		FLAPS (2)

10. R AILERON							MASTER -off

11. R TIP

12. R CHAIN

13. R SUMP

14. R TANK

-PRESTART-         -START-                   -TAXI-                -RUN UP-

SEATS         1. "CLEAR"                 1. BRAKES              1. WIND

BELTS         2. 6 BLADES                2. CONTROLS            2. Seats-Belts-Doors

DOORS           3. PRESSURE                3. D.G.              3. CONTROLS

KEY             4. MIXTURE LEAN                                4. MIXTURE

BREAKERS         5. RADIO MASTER                                5. 1700 RPM

FUEL             6. RADIOS (9)                                       ALTERNATOR

MIXTURE          7. ATIS                                             VACUUM

CARB HEAT        8. INSTRUMENTS                                      AMPS

PRIME            9. FLAPS                                       6. MAG CHECK

THROTTLE         10. RADIO                                      7. C. H.

800 RPM

-TAKEOFF-            -CLIMB-                 -LEVEL-            9. TRIM

LIGHTS                1000'                  1. TRIM            10. INSTRUMENTS

X-PONDER             TOP OF GREEN  ,         2. COWL F          11.

TIME                 2. MANIFOLD UP          3. POWER           12. RADIO

FLAPS (2)            3. TRIM                 4. LEAN

OFF AT 60K

88K RATE           SPEEDS IN KNOTS

GEAR                         ROTATE   60   Va  3100#   112             BEST GLIDE 80, 72, 64

FLAPS                         Vx       64           Va  2550#   101        APPROACH     70

TRIM                          Vy       88           Va  2000#    89        SHORT FIELD  63

ENROUTE 100                                         GEAR        140 (120)     MOMENT

--EMERGENCY--              FUEL  75 GAL      7561X    EMPTY WEIGHT 1857.5     64.4

CHECKLIST                 USE   13.5 GPH    FULL LL FUEL          450.0      21.5

80kts - TRIM              OIL   6-8         FRONT                 400.0      15.0

FIELD AND WIND            GROSS 3,100#      REAR                  390.0      29.5

ELECT. PUMP               CABIN LOAD 792.5#                      3097.5     130.4

RESTART             	     Va 125 to 145

121.5/7700

COMMUNICATE

PRE-CRASH

-DESCENT-    -DESCENT #2-   -PRELANDING-   -LANDING-         -POSTLANDING-     SHUTDOWN

MIXTURE     1.  MIXTURE     1.  MIXTURE    1.  MIXTURE       1.  FLAPS/FLAPS 1. 121.5  OFF

MANIFOLD    2.  MANIFOLD    2.  MANIFOLD       DOWNWIND      2.  MIN BRAKES  2. ELECTRICAL

5 FOR  5    3.  HOLD ALT    3.  GAUGES     2.  12 INCHES     3.  RADIO       3.  MAG CHECK

AIRSPEED    4.  GEAR        4.  GEAR       3.  GEAR          4. CONTROLS     4.  MIXTURE

YELLOW      5.  AIRSPEED                   4.  PROP/100KTS   5.  MIXTURE     5.  MAGS  OFF

TRIM - 3 DN	        	  6.  LOG TIME

90 KTS

10 FLAPS/TRIM

BASE

12 INCHES

GEAR CHECK

PROP FULL

20 FLAPS/TRIM

80 KTS

FINAL

30  FLAPS/TRIM

70 KTS

GEAR CHECK

FULL STALL

Mnemonic checklist

Start

S - switches off

L - Lean control full rich

I - ignition on

M - master on

S - safety (clear)

After start

R -RPM to idle

O - Oil presssure check

A - ammeter check

R - Radios on

S - suction check

Runup

C - controls

I - instruments

G - gas selector & quantity

A - Attitude set (trim, flaps)

R s- runup

S - suction check

Takeoff

L - lights as required

C - camera (transponder)

A - Action (call tower_

Post-landing

TLC - tender, loving care

T - Transponder off

L - Lights off

C - Cancel IFR, close VFR

Shutdown

S - Switches off

L - lean control out

I - Ignition off

M - master off

S - secure plane

Still another checklist made up of two cards with items on both sides.  Three of the sides

have two columns each.

Card 1-Side I

Column 1		Column 2

PRE-FLIGHT

Refueling mark		R tire

L chain			R flap

Pitot cover		R sump

Doors			R tank

Ladder			Oil 6 qt minimum

Key on floor		Engine sump

Control lock		Prop/leaks

Log book		Spinner

Magneto/key check	Belt

R master		Cowling

Fuel to both		Nose gear

Guages			Nose tire

Flaps 10°/Cowl flaps	Roll tires

Fuel pump /		Static air

Master OFF		L tank

Get strainer		Stall warner

L sump			Overflow /

L flap			L tip

Antennae		L aileron

Elevator		L tire

Rudder

Tail chain

R flap

R aileron

R tip

R wing

Card 1-Side II

Emergency list

Checklist

Fuel pump

Best glide

Gross   3100 @ 80

2550 @ 70

2000 @ 65

Trim nose high

Field & wind

Restart

Selector

Guages

Mixture

Magnetos

Fly to field

121.5/7700 Words x3

Flaps when certain

Pre-crash

Harness tight

Doors cracked

Selector OFF

Masster OFF

Full stall

Card 2-Side 1

Column 1		Column 2

PRIME

Passenger checks	RUN-UP

Emergency		Into wind

Gear check		control /

Traffic		1700 rpm

Other			Magneto /

START			C. H.

Trim /			Vacuum

Seats, belts, doors	Amp meter

Key			Cycle prop

Mixture	 rich		 3 times or

Cowl flaps /		 1 time

C. H. /			800 rpm

Prime /			Altimeter/

Throttle prime/set	H. I.

Window “clear”		X-ponder /

Master			Flaps 10°

Mags to start		mixture rich

Mixture to lean

Radio master		POST TAKEOFF

X-ponder to stby	1000’ Top of greens

Fuel pressure

Oil pressure

Card 2 - Side 1

Column 1		Column 2

CRUISE			SHUT DOWN

Throttle  22”		121.5

Prop 2200		Radio master

Cowl flaps		Lights

Master off

Mixture

Mags/key off/out

PRE LANDING

Selector both

120 knots

Gear

Flaps 10°

Wheel and green

C. H.

power to 14”

Hold heading/altitude

100Kts

Prop full

Mixture

flaps as required

POST LANDING

Flaps/flaps

mixture

C. H.

X-ponder stby