# Click on page number for instant scrolling.

Reading weatherã

METAR

METAR sequence

Terminal Area Forecasts

About TAFs

TAF Coverage:

Amendment Criteria of TAFs

Prognostic TAF qualifiers

Uncommon TAF Terms

# METAR

- Meteorological reports--aviation routine (METAR). METAR (Surface Report). List surface winds, visibility (sm), weather and sky cover.

- Hourly METARs list winds, visibility, runway visual range (RVR), present weather, sky conditon, temperature, dewpoint, and altimeter setting (pressure) and REMARKS.

- Metars exclude trend forecasts.

# METAR sequence

When or SPECI (special weather report) sometimes data is omitted user must know and recognize sequence.  Exceptions are RVR(runway visibility range) and SLP (sea level pressure)  SPECI remarks will be in plain language.

- wind information

- prevailing visibility in statute miles

- RVR information

- Cloud layers and description as (FEW, SCT, BKN, OVC) with heights

- in three digits.

- temperature and dewpoint in Celsius

- Altimeter settings are begun with A or Q plus four digits

A2992 means given in inches and hundredths of mercury

Q1013 means hecto Pascals or centimeters of mercury

- RMK begins the remarks section which will include the sea level pressure

- Always read in the past tense

- METAR sequence another way

- where

K first letter of all U. S. three letter identifiers making four 	letters. Alaska begins with PA, Hawaii with PH

- when

- First METAR time is six digits followed by Z.  First two digits are     	day of month, last four are time of observation. All other times are 	four 	digits read in groups of two.  From (or between) xx to xx.

- wind

- First three digits are wind direction to nearer 10°.  Next digits are 	wind speed usually in KT (knots)  V between wind directions that vary 	more than 60° and greater than 6 knots.  VRB (variable) when less 	than 6 knots.  Peak winds in remarks section with /time past hour of 	occurrence.

- Visibility

- Fractions shown by /.SM (statute miles) follows.

- Visual range uses R+runway/hundreds of feet.  Letters other than R 	may be M(below sensor ability) P (above sensor ability) V (variable).

- Weather

- Two letter descriptor followed by two character weather type.  	Intensity - = light, none = moderate, + = heavy.

- Clouds

- Categorized on eights of the sky. (octas)

- SKC = sky clear; FEW 1-2/8;  SCT 3-4/8;  BKN 5-7/8;  PVC 8/8.

- VV = vertical visibility;  VV/// = indefinite ceiling

- Temperature/dewpoint

- Listed in Celsius with / between.  M = minus temperatures

- Altimeter

- A2992 if in inches. Q if metric.

- ASOS/AWOS

- AUTO included unless human makes remarks.  Remarks tells number of sensors.  CLR means no clouds below 12K.

- Usually from airports

- Hourly at 50 minutes after hour

- Specials preceded by S

- Reports (METAR) whenever visibility is less than 6 miles.

## Demo sequence

OSM FG VV000

visibility, zero statuted miles; fog; vertical visibility, zero

Demo sequence

KOAK 091720Z 1818 22020KT 4 SM -SHRA BKN020CB

### Interpretation

Oakland, 9th day of month made at 1720 valid 1800 9th to 1800 10th

Winds are as 220 degrees at 20KT as direction and velocity,

variable winds given as 180V220 or as VRB08

Visibility given as 4 miles prevailing.  If greater than 6 miles gives as P6S(statute)

Weather given as light showers (-SH) and rain (RA)

Clouds are broken (BKN) at 2000' (020) and cumulonimbus.

KPHL 1550Z 24007KT 7SM BKN009 OVC020 18/16 A3005

Interpretation

Philadelphia, PA 1505Z observation

winds 270 degrees at 7 knots,

visibility 7 statute miles,

the ceiling is 900 broken

with an overcast cloud deck at 2000 feet,

temperature is 18 degrees centigrade, dew point 16 degrees centigrade,

altimeter 30.05 inches

KLAX 1550Z 21008KT 1/2SM R14/3500V5500FT FG OVC 003 14/14 A3018

Interpretation

Los Angeles, CA 1550Z observation,

winds 210 degrees at 8 knots,

visibility 1/2 mile (statute),

runway 14 runway visual range (RVR) is 3500 feet variable to 5500 feet

in fog.

the ceiling is 300 feet overcast,

temperature 14 degrees centigrade, dew point 14 degrees centigrade,

altimeter 30.18

KACY 1545Z 00000KT 11/4SM -SHSN BKNOO6 OVC010 MO3/M03 A2942 RMK SB30

Interpretation

Atlantic City, NJ 1545Z observation,

winds calm,

visibility 1 and 1/4 statute miles

in light snow showers,

the ceiling is 600 feet broken with

an overcast cloud deck at 1000 feet,

temperature is minus 3 degrees centigrade, dewpoint is minus 3 degrees centigrade,

the altimeter is 29.42, and

remarks indicate that the snow began falling at 1530Z.

KBUF 1549Z 35015G27KT 1/4SM BLSN R31/1200FT VV003 01/M01 A3OI3

Interpretation

Buffalo, NY 1549Z  Observation,

winds are 330 degrees at 15 knots gusting to 27 knots

with 1/4 mile )statute) visibility,

runway 31 runway visual Range (RVR) is only 1200 feet, and

the ceiling is indefinite 300 feet,

temperature is 1 degree centigrade,

dew point is minus 1 degree centigrade and

the altimeter is 30.13

KBOS 1549Z 15012KT6SM -RASN SCT004 BKN009 VOC015 02/M01 A2949 RMK PRESFR

Boston, Mass. 1549Z observation,

winds from 150 degrees at 12 knots,

visibility 6 statute miles

in mixed light rain and snow,

scattered clouds at 400 feet,

a broken ceiling at 900 feet and

an overcast deck at 1500 feet,

temperature is 2 degrees centigrade, dewpoint is minimum 1 degree       			centigrade,

the altimeter is 29.49 and

remarks indicate that the pressure is falling rapidly.

KBWI 1550Z S09008KT 3/4SM R10/3800FT -FZDZ PE 008OVC M01/M01 A2992

Baltimore, MD 1550Z Observation,

winds directly out of the East (090 degrees) at 8 knots,

with only 3/4’s of a statute miles visibility,

runway 10 has a runway visual range (RVR) of only 3800 feet

in light freezing drizzle and ice pellets,

with a ceiling of only 800 feet,

temperature is minus 1 degree centigrade, a dew point of minus 1 degree 			centigrade and

the altimeter is 	29.90

METAR HIAD 081055 A 21019G27KT 1/2SM RO4/3000FT-SN BR SCT011 OVC015 O1/O3 			A2945 RMK SLP045

METAR - hourly report

HIAD - ICAO location indicator

081055 - Issuance time:” ALL times in UTC as a two digit date and a four digit	time.

A - ??

21019G27KT - three digit true north direction, nearest 10 degrees or VRB (variable); gusts to 27 knots.

1/2SM prevailing visibility in statute miles

R04/3000FT - runway number and visual range in feet

-SN BR - Light snow and mist (French)

SCT 011 - Scattered clouds at 1100’

OVC015 - Overcast 1500’

temperature 1° Celsius, dewpoint 3° Celsious

A2945 - Altimeter 29.45 inches of barometric pressure

RMK - remarks  SLP045 - Sea level pressure 3045

KPHL 1550z24007KT 7SM BKN009 OVC020 18/16 A3005

Philadelphia, PA 1550Z observation.  Winds 240 degrees at 7 knots, visibility 7 statute miles, the ceiling is 900 feet broken with an overcast cloud deck at 2000 feet, temperature 18 degrees centigrade, dewpoint 16 degrees centigrade, altimeter 30.05 inches of mercury.

KLAX 1550Z 21008KT 1/2SM R14/3500V5500FT FG OVC003 14/14 A3018

Los Angeles, CA 1550Z observation, winds 210 degrees at 8 knots, visibility 1/2 statute mile, runway 14 runway visual range (RVR) is 3500 feet variable to 5500 feet in fog, the ceiling is 300 feet overcast, temperature is 14 degrees centigrade, dewpoint 14 degrees centigrade, altimeter 30.18 inches of mercury.

KACY 1545z 00000KT 11/4SM -SHSN BKN005 OVC010 M03/MO3 A2942 RMK SB30

Atlantic City, NJ 1545Z observation, winds calm, visibility 1 1/4 statute miles in light snow showers, the ceiling is 600 feet broken with an overcast cloud deck at 1000 feet, temperature is minus 3 degrees centigrade, dewpoint is minus 3 degrees centigrade, the altimeter is 29.42 inches of mercury, and remarks indicate that the snow began falling at 1530Z.

KBUF 1549z 35015G27KT 1/4SM BLSN R31/1200FT VV003 01/MO1 A3013

Buffalo, NY 1549Z observation, winds are 350 degrees at 15 knots gusting to 27 knots with 1/4 statute mile visibility in blowing snow, runway 31 runway visual range (RVR) is only 1200 feet, and the ceiling is indefinite 300 feet, temperature is 1 edge centigrade, dewpoint is minus 1 degree centigrade, and the altimeter is 30.13 inches of mercury.

KBOS 1549z 15012KT 6SM -RASN SCT004 BKN009 OVC015 02/M01 A29.49 RMK PRESFR

Boston, Mass 1549Z observation, wind from 150 degrees at 12 knots, visibility 6 statute miles in mixed light rain and snow, scattered clouds at 400 feet, a broken ceiling at 900 feet and an overcast deck at 1500 feet, temperature is 2 degrees centigrade, dewpoint is minus 1 degree centigrade, the altimeter is 29.49 inches of mercury and remarks indicate that the pressure is falling rapidly.

KBWI 1550Z 09008KT 3/4SM R10/3800FT -FZDZ PE OO80OVC M01/M01 A2990

Baltimore, MD 1550Z observation, winds directly out of the east at 8 knots, with only 3/4 statute mile visibility, runway 10 has a runway visual range of only 3800 feet in light freezing drizzle and ice pellets, with a ceiling of only 800 feet, temperature is minus 1 degree centigrade, a dew point of minus one degree centigrade and an altimeter setting of 29.90 inches of mercury.

# Terminal Area Forecasts

Using the area of a particular airport TAF predicts the next 24 hour weather beginning with where and when.  The 24 hours Zulu time is divided into periods of weather change. The first time group gives forecast wind, visibility, sky condition, and weather important to aviation.  A change in any one area will trigger a new time period directed only to those areas of change.  All other elements in the first time period not changing remain the same and though not mentioned are carried forward but not written as unchanged into the second time period. All three-letter station identifiers will be preceded by a K as KOAK for Oakland.  Day of the month and time group is followed by valid time group.  TAF format has been changed so that each time change now begins on a new line.

First break the TAF down into specific time periods.  Sequence of wind, visibility, significant weather, cloud cover and cloud height follows for each time block.  Basically the same forecast for a definite time period.  The initial weather group and the FM (FROM) group always include the wind, visibility and sky condition.  Cloud conditions are divided into eights or  If no visiblity or weather is mentioned it is assumed to be unrestricted or non-existant.  WS in a TAF is wind shear not part of convective activity.  Qualifiers such as TEMPO is used with two digit beginning hour and two digit ending hour.

Get copy of NWS Federal Meteorological Handbook (FMH1A) from government printing office.  It has TAF Decoding Chart but takes some time to learn.

# About TAFs

- Six TAFs cover U.S. SFO, (West coast),  SLC, DFW, CHI, BOS and MIA.

- Contains written flight precautions with broad brush (crying wolf)

with nothing left to  chance.

- Times forecast for weather changes included in the text.

- Most likely errors to be in time rather than occurrence

- An inaccurate forecast tells you about actual as compared to expected.

- Serve as an alert to seek other weather information.

- Flight precautions in airmet category of TAF and amended by Airmets

# TAF Coverage:

- IFR conditions

- Mountain obscurations

- Moderate turbulence, icing, 30 kt winds

- Coincides with Airmet information

- Three times a day.  A 12 hour forecast with 6 hour outlook that is   	`printed 15 minutes before an hour to all stations

- Hazards/flight precautions

- IFR conditions/thunderstorms

- Icing, turbulence, obscurations

- Visibility when below 5 miles

- Winds over 30 kts.

- Synopsis is a Total overview that is good for six hours

- Clouds and Weather good for 12 hours referenced to mean sea level

- Ceilings referenced to ground level

- Outlook good for 18 hours

- TAF excludes temperature, turbulence and icing forecasts.

- Ceiling is not designated: defined as lowest broken or overcast 	layer or the vertical visibility.

# Amendment Criteria of TAFs

Visibility

9000 meters = 56. miles

8000 meters = 5 miles  (File IFR)

7000 meters = 4.3 miles

6000 meters = 3.7 miles

4800 meters = 3 miles

1600 meters = 1 mile  Don’t fly)

9999 means more than 6 miles visibility

Shortcut is to subtract 3 from the thousands place and call it miles.

Ceiling

If the forecast is				Amend if...

No ceiling or ceiling above 3000'	Forecast is not representative		or ceiling is 3000' or less.

3000' to 2000'	   Ceiling  = or < 3000' or below 2000'

Less than 2000' to 1000'      = or < 2000' or below 1000'

Less than 1000' to 600'		= or < 1000' or below 600'

Less than 600' to 200'		= or < 600' or below 200'

Less than 200'			= or < 200'

Visibility

Forecast					Amend if:

< 5 miles				Decreases to 5 miles or less

3 to 5 miles			< than 5 or below 3 miles

2 to > 3 miles			= or < 3 or below 2 miles

1 to > 2 miles			= or < 2 or below 2 miles

1/2 to > 1 mile			= or < 1 or below 1/2 mile

> 1/2 mile				= or < 1/2 mile

### Qualifing terms of TAFs

LIFR (Low IFR)

Ceiling below 500' visibility below 1 mile

IFR

Ceiling 1000 to 500, visibility 1 to 3 miles

MVFR (Marginal VFR)

Ceiling 1000 to 3000, visibility 3-5 miles

VFR

Ceiling over 3,000, visibility over 5 miles

Intensity/proximity

- light, +heavy, no sign = moderate

VC: Vicinity 5-10 sm from runway center

Description

MI shallow,  BC patches,  DR drifting,  TS Thunderstorm, PR Partial

BL blowing,  SH showers,  FZ freezing  PO dust devil

Weather

DZ drizzle,  RA rain,  SN snow,  SG snow grains

IC diamond dust,  PE ice pellets,  GR  hail (French),  GS small hail

Obscuration

BR mist (French),  FG fog,  FU smoke(French),  VA volcanic ash

SA sand,  HZ haze, SY blowing spray,  DU dust  PY spray

Other

SK sky clear  CLR clear  TCU towering cumulus

SQ squall,  SS sand storm,  DS dust storm,  PO dust whirls

FC funnel cloud  MI shallow  SA sand

CAVOK = CAVU = WX NIL or SKC

CIG010 = ceiling 1000

SKY-X = sky partially obscured

9//005 Sky obscured(9), clouds not visible (//) 500' vertical 	visibility

Octas (eights)

2CU030 = 2/8 scattered cumulus at 3000’

# Prognostic TAF qualifiers

Permanent shifts

GRADU = gradual change over 1/2 hour as with fog burning off

RAPID = with time a quick (less than 1/2 hour)  turnabout

FRONT = rarely used

Short term variations

TEMPO = CHC less than 1 hour of variation

INTER = OCNL  quick changes in ceilings/visibility

PROB = VRBL less certain of changes.

Others:

00000 = calm winds

VRB means variable direction of winds

1920 from 1900Z to 2000Z

OCNL, OCNLY	= Occasional, occasionally means greater than 50% chance for 	less than half of the forecast period.

OTLK = outlook covers six hours following forecast period.

CHC = chance means 30 to 50% probability during forecast period.

SLGT CHC = 10 to 20% chance

ISOLD = isolated is lowest thunderstorm/precipitation alert level

WDLY SCT = widely scattered is less than 25% chance

SCT = scattered 25 to 54% chance

NMRS = numerous is more than 54% chance

VCNTY = vicinity between 5 and 25 miles

VC-SHRA =  rain showers in vicinity

NSW = no significant weather

9999 = visibility over 6 miles

BR = mist (baby rain)

T16/11 = temperature 16°C at 1100Z

6CU030 = 6/8th cumulus at 3000’

FEW = 1/8 to 2/8 sky coverage

COR = correction

SPECI = special or Special Report)

KPIT U.S. station identifiers begin with K followed by three letters.

201955Z 20th day 1955Z time of issue. It is always 6 digits and "Z"

COR for correction to observation) ...follows the time field (when 	present)?

RMK SLP013 remarks, sea level pressure 30.13

FM1930  FroM and 2-digit hour and 2-digit minutes indicates 	significant change in weather.  Each FM group has a separate 	line which is indented five spaces.

PROB40 2022  PROBability and 2-digit percent; probable condition 	during 2-digit beginning and 2-digit ending time period.

Intensity or proximity

- = light  no sign = moderate    + = heavy

VC = vicinity   This covers only weather in vicinity of airport up to

ten miles.

TAFs tend to be more accurate than FAs.

Three FTs a day, valid for 24 hours

18 hour forecast with additional 6 a categorical outlook

Expected ceilings, clouds/coverage, visibility, weather, vision

surface winds

Outlook of last 6 hours is VFR, MVFR, IFR, and low IFR

Ceilings as C in 100s of feet above ground

Clouds in ascending order

Visibility in miles and fractions up to 2 miles.  Over 6 not 			reported.

Winds reported to neared 10 degrees from true north in knots.

Extends only 5 miles from airport center

VCTNY in forecast means within 25 miles

Never use an FT after the station has closed for the day.

If the surface report and FT are not available at destination have

alternate.

2/10 of weather coverage (rain) means that 80% of area is rain free.

When METAR data is missing from the body of the report (e.g. dew point), it is simply omitted and the user must know the sequence to recognize this. Some exceptions apply in remarks such as RVRNO, or SLPNO when RVR or SLP are normally reported but not currently available.  METARs exclude trend forecast.

# Uncommon TAF Terms

9M04/// means temperature -4 degC, dewpoint not available.

FM1930 = FroM  gives a two digit hour and two digit minute as time of

significant change.  Each FM begins a new line indented five spaces.

PROB = PROBability

PROB40 2022 Has a two digit percent during time between two digit beginning 	and end.

TEMPO = TEMPOrary

TEMPO0407  Changes less than an hour duration and less than half of time 	between two digit beginning and two digit end.

BECOMING = BECoMinG

BECMG	1315  Change expected from beginning two digit and ending two digit time

period.

BINOVC breaks in overcast

BRKS N breaks in layer below 1000' covers 6/10+ of sky

ACCAS ALQDS Altocumulus castellanus all quadrants

CUFRA W cumulus factus clouds west

IC4 ice crystals 4/10 of sky

PRESSRR pressure rising rapidly

RADAT 87120  87 is relative humidity of freezing level, 120(00) is altitude

RADAT ZERO freezing level at ground

F5 fog covering 5/10 of sky

RWU E  Rain showers of unknown intensity east

OCNL LTGIC  occasional lightning in clouds

FQT LTGCCCA  Frequent lightning cloud to cloud and cloud to atmosphere

SB50 PRESSFR Snow began 50 minutes after the hour pressure falling rapidly

KOCTY smoke over the city

CLDS APCHG STN clouds approaching station

PRJMP 8/1012/18 pressure jump  .8inch at 10:12 stopped at 18 after 10:00

TWRINUN control tower in unknown

AB13E20 HLSTO1/2  Hail began 13 after and ended 20 after 1/2" hailstones

SU N snow of unknown intensity north

SHLW GFDEP 4 Shallow ground fog 4 feet deep

SNOINCR 1/4/8 snow depth increase 1inch per hour/4 inches last six 8” deep

PCPN 103  1.03 precipitation in past hour

ACSL altocumulus standing lenticular clouds

CCSL Cirrocumulus standing lenticular

CBMAMcumulonimbus mama

P (Plus)as a letter imposed before visibility means ‘greater than’ otherwise means +. (Plus)

NSC means no significant clouds apparently differs from SKC (sky clear).

NSW means no significant weather

VC means vicinity...at or within five miles

DSNT means distant...at or beyond ten miles

SKC means sky clear

FEW means cloud coverage 1 to 2 eights sky coverage

TEMPO (Means "conditions temporary about half of time)  Weather and visibility from another time period carries forward through TEMPO unless mentioned.

PROB40 (40% chance of...),

BECMG (becoming...) After qualifiers components omitted are expected to remain the same.  Weather not expected to continue into BECMG or TEMPO time periods is written as NSW (no significant weather)

KRNO

141624Z 1818 28010KT P6SM SCT035 SCT100 BKN250 BECMG 1920 25012KT

SCT075 BKN100 TEMPO 1901 VRB12G40KT 11/2SM +TSRA OVC015CB BECMG

OOO1 26006KT SCT730 SCT100 BKN250 BECMG 0909 00000KT 5SM BR SCT250

BECMG 1213 25007KT NSW SCT025=

First six digits give (14th) day of month, (1624Z)time of issue.  Four digits (1818) give valid time of forecast from 1800Z the 14th to 1800Z the 15th.

Forecast wind (28010KT) is from 280 at 10 kts.

Visibility expected is (P)over (6SM) six statute miles.

(SCT055) Scattered clouds at 5,500’.  Next layer is scattered at  10,000’ (Only CU or CB cloud types are ever mentioned.) Broken clouds are at 25,000’.

Becoming a change (BECMG) between (1920) 1900Z and 2000Z wind (2512KT) from 250 at 12 kts a layer of (SCT075) scattered clouds at 7,500’ and a layer (BKN100) of broken clouds at 10,000’.

Visibility when not given is unlimited.

(TEMPO) Temporarily (up to half of the time) between 1900Z and 0100Z a variable (VBL12G40) direction wind with velocities between 12 and 40 knots.

Visibility is one and one-half (11/2SM) statute miles with heavy (+TSRA) thunder showers with temporary ceiling (OVC015CB) at 1,500 and cumulus buildups.

Between midnight and 0100Z (BECMG 0001)wind becoming from 260 degrees at six knots with cloud layers of Scattered at 7,300, another scattered layer at 10,000’, and broken clouds at 25,000’.

Becoming (BECMG) between 0900Z and 0900 calm wind conditions (000000KT) with five ((5SM) statute mile visibility in mist (BR) was a scattered layer at 25,000’.

By the period between (1213)1200Z and 1300Z becoming a wind from 250 at seven knots, (NSW) no significant weather other than scattered clouds a 25,000’.

TAF

TAF KIAD 271725Z 271818 21006KT P6SM VCSH SCT050 BKN090 TEMPO

1802 BKN050 BECMG 0002 23006KT KP6SM SCT060 TEMPO

0413 4SM BR SCDT010 BKN060

FM1300 28012G20KT P6SM BKN045 OVC080 PROB30 1318 5SM

-SHRA=

Interpretation

KIAD Location: Dulles International airport (K is the USA)

271725 Issued  on the 27th day at 1725 Zulu

271818 Forecast 24-hour period from 1800Z the 27th to 1800Z the 28th

21006KT Wind forecast from 210 (TAF uses 3-digit wind) degrees at 6 knots

P6SM Visibility P stands for ‘plus’ greater than six statute miles.

Where weather is not given none relates to aviation.

SCT050 Sky conditions have scattered clouds at 5000’ AGL (U.S. uses feet)

Scattered means 3 to 4 eights cloud coverage.

BKN090 Broken clouds at 9000’ covering 5 to 7 octas of the sky.  This is a

ceiling but must be a factor known to the pilot.

TEMPO 1802 Says a temporary change is forecast between 1800 Zulu and 0200 	Zulu.

BKN050 Says the temporary change is a ceiling of 5000’ over a period of 	eight hours only.

BECMG Means a gradual permanent change between 0002 and 0000 Zulu and 0200

Zulu there will be...

23006kt a Wind shift to 230 at 6 knots

P6SM  visibility will be better than 6 statude miles along with

SCT060 Scattered clouds at 6000’

TEMPO 0413 Between 0400 Zulu and 1300 Zulu a temporary change  will occur 	in which  (4SM BR)

visibility will decrease to four statute miles in misty conditions.

BR means that you can see more than 5/8th of a mile.  Fog has 		visibility less than 5/8th of a mile.

SCT010 a scattered layer of clouds will be at 1000’

BKN060 forecasts a broken layer of 5 to 7 octa at 6000’

FM begins a new line and indicates a change will take less than an hour.

1300 is the time the change begins

28012G20 knots winds will be from 280 at 12 knots gusting to 20 knots.

P6SM visibility over six statute miles

BKN045 cloud layer at 4500’ AGL

OVC080 overcast layer at 8000’ AGL

PROB30 is a 30% chance of a significant change between ...

1300 Z and 1800Z to

5SM visibility of five statute miles

-SHRA The weather visibility is reduced due to rain showers.

TAF  KCRP 2371730z 271818 14020G29KT P6SM VCSH BKN020

OVC030 TEMPO 1822 5SM-SHRA BKN017

FM0100 15015G21KT P6SM SCT015 BKN025 WS020/17035KT

Interpretation

Issued on the 27th at 1730 and valid from 1800Z to 1800Z the 28th.

Wind will be from 140 at 20 knots gusting to 29 knots.

Visibility will be greater than 6 miles

.

Weather will consist of showers in the (VCSH) vicinity (within five miles)of the airport.

Broken clouds (ceiling) will be at 2000’ and overcast at 3000’

There will be a temporary change from 1800Z to 2200Z with visibility of five statute miles due to light rain showers with broken (ceiling) at 1700’

From 0100Z winds will be 150 at 15 knots gusting to21 knots.

Visibility will be more than six statute miles. with scattered clouds (3 to 5 eights) at1500’ and a broken cloud (ceiling) covering six to eight octas at 2500’.

Winds at 2000’ will cause wind shear while blowing from 170 at 35 knots.

## TAF (Terminal Aerodrome (Aviation) Forecast)

TAFKLIZ0606 31022/39 O700 71SN 9//008 QNH2990INSCIG008 INTER 0714 0000

39BLSN 9//000 620003 CIG000 GRADU 1415 33025 38 BLSN 8SC020 QNH 2993INS CIG 020 TEMP 1517 85SNSH ...GRADU 2021 32010 WXNIL 3S020 RAPID 22 VRB04 9999 SKC;

### Interpretation

TAF = Terminal aerodrome forecast

KLIZ = K is the United States, LIX is station identifier

0606  TAS every six hours, 0000, 0500, 1200, 1800. = 0600 one day to 0600 	the next.

31022/39 = Wind from 310 at 22 knots gusts to 39 knots.

0700 = meters of visibility, 3/8s of a mile

71SN = Uses code lattice 71 to indicate SN snow.  No apparent reason for

the 	numbers since SN = snow.

9//008 = obscured sky with 800 feet vertical visibility

QNH2990INS = Altimeter MSL is 29.90 inches.  May be lowest of forecast

period.

CIG008 = Ceiling of 800 feet

INTER0714 = Says  from 0700 to 1400 conditions will occur frequently for 	less 	than an hour.

0000 = zero-zero visibility

39BLSN = Uses code lattice 39 to indicate blowing snow. No reason for 	numbers.

9//000 = Sky obscured with zero-zero visibility vertically

620003CIG000 = 6 = icing group symbol , 2 = light rime icing in cloud,

000 = lowest height AGL, 3 = 300 feet, CIG = ceiling, 000 = height 	above AGL (zero)

GRADU 1415 = A gradual change from 1400 to 1500

33025 = wind from 330 degrees at 25 knots

38 BLSN = 38 uses code lattice to indicate blowing snow, BLSN means blowing 	snow.

8SC020 = 8/10 stratocumulus sky cover. 0 temperature group symbol. 20 	degrees Celsius.

QNH2993ins = Altimeter setting 29.93 inches of mercury

CIG 020 = Ceilings (?), 020 = 2000 feet (?)

TEMP 1517 = For less than 1/2 hour periods between 1500 and 1700

85SNSH = snow showers

...GRADU 2021 = gradual change from 2000 to 2100 zulu

32010 = wind at 320 degrees at 10 knots

WXNIL = no significant weather

3SC020 = 3/10 sky covered with stratocumulus,  at 2000 feet

RAPID 22VRB04 = Rapid change of wind from 22 knots to 4 knots

9999 = Visibility greater than six miles

SKC = sky clear

WS015/30045KT  wind shear field direction(s) and velocity

TAF	KJKF 251720Z 1818 21007KT 5SM -SHRA BKN030 OVC 080

FM 22Z 33015G25KT P6SM 030BKN BKN080 PROB40 2301 1SM TSRA

FM05 32005 P6SM 050SCT 100SCT TEMPO 0709 00000KT 1SM FG

FM12 33010 4SM FGHZ BECMG 1416 30010 P6SM NSW SKC

This is the terminal forecast for John F. Kennedy airport for the 25th day of the month, prepared at 1720Z for

the period from 1800Z of the 25th until 1800Z on the 26th.

-The TAF calls for, from the beginning of the forecast period (1800Z) until 2200Z, winds out of the southwest (210°) at 7 knots, with

the visibility to be 5 statute miles

in light rain showers, with a (broken) ceiling of 3000 feet and

an overcast deck at 8000 feet.  Note use of term deck instead of overcast)

After FM) 2200Z, it calls for winds out of the northwest (wind shift to 330°) at 15 to 25 knots,

with visibility improving to more than (P) 6 statute miles

with just broken clouds, one deck (ceiling) at 3000 feet and

an overcast deck at 8000 feet with a probability of (occasionally)

Between  2300Z and 0100Z,

visibility dropping to 1 statute mile in thunderstorms and rain showers.

-After 0500Z the winds are expected to be out of the Northwest (320°)at only 5 knots,

visibility more than (P)6 statute miles with just

scattered clouds at 5000 and 10000 feet, but

temporarily (chance of), Between 0700Z and 0900Z, the winds becoming calm and visibility dropping to 1 statute mile in fog.

-After 1200, the forecast calls for winds out of the northwest 330° at 10 knots with

visibility 4 statute miles in fog and haze

becoming between (or by) 1400-1600Z winds out of the Northwest (300°) at 10 knots,

visibility more than 6 statute miles with

no significant weather and clear skies.

TAF KMCI  011725Z  011818  24020KT  2SM FZRA OVC 010

MCI

Eleventh day of the month at 1725 Zulu

Eleventh day of the month from 1800 zulu to 1800 zulu next day

wind  240 at 20 knots

2 statute miles visibility

freezing rain and overcast ceiling at 1000 feet.

TAF 131530Z KABC 0716 27015KT 10SCT015 TEMPO 1116 4000 +SHRA PROB30 TEMPO 1416 TSRA BKN010CB

### Interpretation

131530z		thirteenth day of month at 1530 zulu

KABC			K is U. S. code and three letter station identifier

0716              date and time of forecast elements  0600 Z to 1600 Z

27015KT		wind 270 at 15kts

10SCT015		sky 10,000' scattered visibility 15 miles

TEMPO			temporary

1116			between 1100 and 1600 zulu always 4 digits separated.

4000			4000’ ceiling with

+SHRA			heavy rain showers

PROB30  		30% chance

TEMPO 1416  	between 1400 and 1600Z

TSRA			moderate chance of thunderstorm

BKN101CB  		Sky going to 1000' broken with cumulonimbus clouds

TAFs

ABC 1410 AMD VRB05KT 9000 BR SCT020 BKN050 OVCO80 QNH3020 CIG050 BCMG 1314 VRB05KT 1600 BR SCT020 BKN040 OVC100 QNH3018INS CIG040 VC-RA

Amended forecast 1400Z to tomorrows 1000Z”wind” variable to five knots 9000 meters visibility (5.6 miles) mist scattered clouds at 2000 feet broken at 5000 and overcast at 8000 feet altimeter 30.2 ceiling is 5000 feet becoming  “between” 1300Z and 1400Z ind” variable at 5 knots 1600 meters visibility (1-mile) with misty (French word) 2000 scattered 4000 broken overcast at 10,000 altimeter 30.18 inches ceiling at 4000 rain in vicinity.

KJKF 251720Z 1818 21007KT 5SM-SHRA BKN030 OVC080

FM 22Z 33015G25KT P6SM BKN030 BKN080 PROB40 2301 1SM TSRA

FM05 32005 P6SM 050SCT 100SCT TEMPO 0709  00000KT 1SM FG

FM12 33010 4SM FGHZ BECMG 1416 300010 P6SM NSW SKC

This is the terminal forecast for John f. Kennedy airport for the 25th day of the month, prepared at 1720Z , for the period from 1800Z of the 25th day until 1800Z on the 26th.  The TAF calls for, from the beginning of the forecast period (1800Z) until 2200Z, winds out of the southwest at 7 knots, with the visibility to be 5 statute miles in light rain showers, with a ceiling of 3000 feet and an overcast deck at 8000.  After 2200Z, it calls for winds out of the northwest (note wind shift) at 15 to 25 knots, with visibility improving to more than 6 statute miles with just broken clouds, one deck at 3000 feet and the other at 8000 feet with a probability of (occasionally) between 2300Z and 0100Z, visibility dropping to 1 statute mile in thunderstorms and rain showers.  After 0500Z, the winds are expected to be out of the northwest at only 5 knots, visibility more than 6 statute miles with just scattered clouds at 5000 and 10,000 feet, but temporally (chance of) between 0700Z and 0900Z, the winds becoming calm and visibility dropping to 1 statute mile in fog.  After 1200Z, the forecast calls for winds out of the northwest at 10 knots with visibility 4 statute mines in fog and haze becoming between (or by) 1400-1600Z, winds out of the northwest at 10 knots, visibility more than 6 statute miles with no significant weather and clear skies.