Cessna 150ã

Nose-wheel (Instructor)

C-150 flaps

Trim and the C-150

Basic Maneuvers

C-150 POH

Building a Checklist

Performance Synopsis

# Nose-wheel (Instructor)

Conventional gear aircraft fly much the same as tricycle aircraft.  It is the landings that require special instruction and endorsement.  This is because of the predilection to ground-loop.  Tricycle aircraft. if weight is kept off the nose wheel will align with the direction of travel.  There is a significant design difference of the nose wheel between Cessna and Piper.  On the ground, Piper steers heavily and positively; Cessnas have springs that will gradually pull the wheel into the turn.  The left and right turning springs of the Cessna are usually of different tension.  This means that more differential braking will be required in one direction than in the other.

In the air, a Piper nose wheel will move with the rudder.  On the ground it moves with the rudder.  In a cross-wind landing it moves with the rudder.  This means that a cross-control landing slip in a Piper will have the nose wheel cocked away from the low wing and into the wind.  Severe landing loads are put on the nose wheel if it is allowed to touch down in this cocked position.  It may ever result in a ground-loop.  The nose wheel of a Piper should never be allowed to touch the runway during landing without having been first aligned with the direction of motion.

In the air, a Cessna nose wheel will hang down about a foot.  This frees it from the socket that has the turning springs.  The Cessna nose wheel will face forward and not move with the rudder.  In a cross-wind landing the Cessna nose wheel is aligned with the direction of aircraft motion.  The nose remains so aligned even in a cross-control landing.  Again, the nose wheel should not be allowed to touch down during the initial landing but only because it is not designed to with stand such shock.  The turning springs of the Cessna will not function until the nose wheel strut is sufficiently compressed.  If you make a landing that seems to prevent steering except through differential braking, apply the brakes to compress the nose strut.  (see nose-wheel landings)

# C-150 flaps

The C-150 has slotted Fowler "barn doors" that, along with inadequate instruction, caused the C-152 to be "improved(?)" with only 30 degrees of flap extension.  Flap speeds should never be exceeded since they put structural strain on the wings' rear spar.  In most Cessna a 10-knot buffer the white arc has much to recommend it.  Any parasitic drag caused by flaps increases as the square of the speed.  In some cases 10 degrees of flap may be used for takeoff but they should be removed at a safe altitude to improve climb performance.

C-150 flaps have evolved from the Piper like Johnson bar through various flap switches and indent levers. Be careful some switches work 'backwards'. The later models of the C-150 have electric flaps with indicators marked in 10 degrees to 40 degrees.  There is a flap-shaped switch which applies power to an electric motor and a worm gear to the flap actuating rod.  Extension takes about nine seconds and airstream assisted retraction takes about five seconds.  Any count system used to move flaps should be figured accordingly.  The flap indicator works on a cable + pulley system which has variations in accuracy.

A C-150 after landing with full flaps will be trimmed for level flight and will require 1 trim down for climb.  The C-172 will be trimmed for climb and will require 1 trim+ for leveling off.  Most Cessnas do not recommend flaps for short field takeoff where Vx is required to get over an obstacle.  Getting off the ground early will not help you get UP.  The C-150 can have an abrupt pitch up when full power is applied in a full flap configuration.  This could result in a departure type stall.  The nose must be prevented from rising above the horizon by locking the elbow before applying any power.  See "Go Around.

# Trim and the C-150

From cruise trim to best rate climb is one full trim down.  From cruise to 1500 rpm four trims down and then back to 2000 rpm gives no flap minimum controllable.  The same operation with only three trims down gives no flap slow flight at 60 knots.  The same operation again but leaving the power at 1500 gives a 60 kt descent. The transition from pattern slow flight to descent requires only a reduction of power,  Easy.

Every 10 degrees of flap has a one to one relationship to a full turn of trim from the above configuration.  From the 60 kt 1500 RPM power glide an application of 10 degrees flap lowers the speed to 50 kts.  A full trim movement up returns the speed to 60 kts. 20 degrees does the same as will 30/40 degrees.  When doing this process with a student for the first time be sure to bring to their attention that you are in a descent.  A descent which will be stopped at a given point.  Descents are an early source of student anxiety as are clouds, mountains, bumps, water, etc, etc.

A Cessna 150 has nearly the same trim setting for level cruise flight as is needed at 1500 RPM, full flaps and 60 kts descent.  Thus, the full flap short approach requires no change in trim unless power is off.  This setting is constant for nearly every loading. The instructor will help the student determine this initial setting.  If the C-150 has been landed with full flaps it will be near the correct setting for level cruise but not for takeoff climb.  From this starting point of the trim wheel the following apply:

1.   For a takeoff climb at 65 kts, the C-150 trim wheel must be moved down 	one full turn from the full flap landing/level cruise position.

2.   To level off from this climb it must be moved up one full turn.

3.   Three full down turns from level will give descent at 60 kts at 1500 	RPM.

4.   Trim down four for no flap glide at 60 kts with power off.

5.   Trim full down four for minimum controllable without flaps.  About 	2000 RPM no flaps.

6.   For full flap slow flight or minimum controllable trim up one. 	Full power.

7.   Most Cessnas have one full turn of the trim between level and climb 	settings.

8.   In each case fine trim movements may be required.

Removal of the flaps during the go-around finds you trimmed for level cruise.  One full trim down will give Vy climb at 65 knots.  This same procedure can illustrate why, when making a short approach, reduction of power to 1500 and application of full flaps at the white arc will give you a hands-off approach speed of 60 knots.

# Basic Maneuvers

The following are designed as a talk/think through series of steps specific for the basic maneuvers of a C-150.  Very slight variations are required for the C-152, C-172, and other Cessna models.  When you go on a practice flight make a copy of several of these procedures and take them with you for review.  An unplanned practice session wastes money.  An explanation for certain specifics is included.

C-150 Procedures

TAKEOFF/CLIMBS       LEVEL CRUISE #1       LEVEL CRUISE #2

Plane is trimmed        From normal climb        From normal climb

for cruise or has       LEVEL WINGTIP            CLIMB 100' HIGH

landed full-flaps.      HOLD NOSE LEVEL          DIVE TO ALTITUDE

CLIMB ATTITUDE          1 FULL TRIM UP           1 1/4 TRIM UP

FULL POWER              ACCELERATE               REDUCE TO 2550

1 FULL TRIM DOWN        2450 RPM                 FINE TRIM

RIGHT RUDDER REQD.      FINE TRIM                2450 RPM

FINE TRIM 65KTS                                  Takes 20 seconds

CLIMBING LEFT TURNS   CLIMBING RIGHT TURNS      LEVEL TURNS

"clear left"              "clear right"          "clear r-l"

LITTLE/NO RUDDER          LEAD R-RUDDER          LEAD R-RUDDER TO RIGHT

YOKE-SLIGHT FWD.          YOKE-SLIGHT FWD        30 DEGREE BANK

30 DEGREE BANK            30 DEGREE BANK         LEAD R-RUDDER FROM LEFT

ONE FINGER                ONE FINGER                   30 DEGREE BANKS

BACK PRESSURE 65KTS       BACK PRESSURE 65KTS       LT BACK YOKE/CRUISE

LEAD +15 DEGREES          LEAD +15 DEGREES          LEAD + 15 DEGREES

R-RUDDER/LEVEL            RELAX R-RUDDER/LEVEL      YOKE-RELAX/CRUISE

YOKE FWD.  65KTS          YOKE-SLIGHT FWD. 65KTS    LITTLE/NO RUDDER

R-RUDDER AS REQ.          R-RUDDER AS REQUIRED

60 KT DESCENT           60 KT DESCENT W/FLAPS     TURNS IN DESCENT

From  cruise            From 60 kt descent        LEAD R-RUD. TO RIGHT

CARB HEAT               4 COUNT FLAPS-10 DEG.     LEAD R-RUDDER FROM LEFT

1500 RPM                ANTICIPATE w/ FWD YOKE    HEAVY RUDDER REQD.

HOLD HDG. & ALT.        HOLD HEADING              AS FLAPS ADDED

3 FULL TRIM DOWN        1 FULL TRIM UP            GREATER FWD/BACK YOKE

FINE TRIM 60 KTS        FINE TRIM 60 KTS REQD.    w/MORE FLAPS

REPEAT FOR EACH 4 COUNT   TO HOLD AIRSPEED

ANTICIPATE W/FWD YOKE

NO FLAP SLOW FLIGHT     RT SLOW FLIGHT TURNS      LFT SLOW FLIGHT TURNS

CARB HEAT               "clear right"             "clear left"

1500 RPM                LEAD R-RUDDER              L-AILERON/LT R-RUDDER

HOLD HDG. & ALT.        BK PRESSURE/+POWER         BK PRESSURE/+POWER

4-TO FULL TRIM DOWN     LEFT AILERON               10-30 DEGREE BANK

2000 RPM                10-30 DEG. BANK            1/2 ANGLE RECOVERY

R-RUDDER                1/2 ANGLE RECOVERY         LEAD R-RUDDER

50 KT TO MIN.CONTROL.   L-AILERON TO LEVEL         AILERON TO LEVEL

RELAX R-RUDDER             POWER TO HOLD ALT.

POWER TO HOLD ALT.

Can lead to cross-control stall

NO FLAP RECOVERIES

FULL POWER

R-RUDDER

LOWER/HOLD NOSE

REMOVE TRIM

HOLD HEADING & ALT.

CLIMBING LEFT TURNS    CLIMBING RIGHT TURNS    LEVEL TURNS

"clear left"            "clear right"            "clear r-l"

LITTLE/NO RUDDER        LEAD R-RUDDER            LEAD R-RUDDER TO RIGHT

YOKE-SLIGHT FWD.        YOKE-SLIGHT FWD          LEAD R-RUDDER FROM LEFT

30 DEGREE BANK          30 DEGREE BANK           30 DEGREE BANKS

BACK PRESSURE 65KTS     BACK PRESSURE 65KTS      LT BACK YOKE/CRUISE

LEAD +15 DEGREES        LEAD +15 DEGREES         LEAD + 15 DEGREES

R-RUDDER/LEVEL          RELAX R-RUDDER/LEVEL     YOKE-RELAX/CRUISE

YOKE FWD.  65KTS        YOKE-SLIGHT FWD. 65KTS   LITTLE/NO RUDDER

R-RUDDER AS REQ.        R-RUDDER AS REQUIRED

SPIN RECOVERY

POWER OFF         DIRECTION OF SPIN

FLAPS UP          OPPOSITE RUDDER (TURN COORDINATOR GIVES SPIN DIRECTION.)

YOKE FORWARD      GENTLE RECOVERY

LEVEL/CLIMB       FAA REQUIRES SPIN KNOWLEDGE BUT NOT PERFORMANCE

UNIVERSAL SPIN RECOVERY

POWER TO IDLE, FLAPS UP, TRIM NEUTRAL, LET GO OF EVERYTHING.

FF-SLOW FLIGHT   F-F SLOW FLIGHT R TURNS   F-F SLOW FLIGHT L TURNS

CARB HEAT         "clear right"             "clear left"

1500 RPM          LEAD HEAVY R-RUDDER       LEAD LEFT AILERON

WHITE ARC/F-F     LEFT AILERON FOR BANK     R-RUDDER NO OVERBANK

YOKE FWD THEN BK  10-30 DEGREE BANK         10-30 DEGREE BANK

HOLD HDG. & ALT.  +POWER REQ.FOR ALT.       +POWER REQ. FOR ALT.

FULL PWR/R-RUDDER  1/2 ANGLE RECOVERY       1/2 ANGLE RECOVERY

1 FULL TRIM UP    +L-AILERON TO LEVEL       LEAD R-RUDDER

40 KTS MIN. CONTRL. RELAX R-RUDDER          LEVEL w/AILERON

+-POWER FOR ALTITUDE

FINE TRIM

F-F SLOW FLIGHT RECOVERY          45 DEGREE STEEP TURNS

FULL POWER                        Begin from level cruise

R-RUDDER                          LEAD HEAVY R-L RUDDER

MILK FLAPS TO 20 DEGR.            QUICKLY TO 45 DEGREE BANK

HOLD ALT. & ACCELERATE            BACK PRESSURE TO HOLD NOSE

CLIMB SPEED-FLAPS UP              LOCK ELBOW TO HOLD YOKE

1 FULL TRIM DOWN                  LOCK HORIZON/NOSE POSITION

CLIMB OR LEVEL CRUISE             NOISE & VSI BEST CUES FINE

TRIM                              2 FULL TRIM DOWN OPTIONAL

+-POWER                           WAKE TURBULENCE 2ND 360

IF WAKE TURB. BACK

PRESSURE/DECREASE BANK

During the proficiency phase      + 5 DEGREES OF BANK

if training all slow flight    	    + 50' ALTITUDE

should be transitional into

minimum controllable and back		STEEP TURN RECOVERY

to slow flight as a training		22 1/2 DEGREES EARLY

exercise in holding altitudes		SHARP FORWARD YOKE

and headings.				CENTER BALL

TRIM IF REQUIRED

+ 10 DEGREES OF HEADING

POWER-OFF STALL      POWER-ON STALL        DEPARTURE STALL

CLEARING TURNS          CLEARING TURNS           CLEARING TURNS

CARB HEAT               CARB HEAT                CARB HEAT

PWR TO OFF              PWR TO 1500              PWR TO 1500 HOLD

HOLD HDG. & ALT.        HOLD HDG. & ALT.         HOLD HDG. & ALT.

R-RUDDER AS REQD.       R-RUDDER AS REQD         R-RUDDER AS REQD.

BUFFET OR STALL         SLOW TO 55 KTS           SLOW TO 55 KTS

YOKE RELAX FWARD        PWR UP 2000/FULL         PWR UP 2000/FULL

NOSE TO/BLOW HORIZON    R-RUDDER                 20 DEGREES-CENTER BALL

LEVEL WINGS             BUFFET OR STALL          BUFFET OR STALL FULL

POWER-CLIMB 65 KTS      YOKE FORWARD             YOKE FORWARD

R-RUDDER/HOLD HDG.      NOSE TO/BLOW HORIZON     NOSE TO/BELOW HORIZON

LEVEL WINGS             LEVEL WINGS           The power-on/departure Stalls

F-POWER-CLIMB 65K       F-POWER-CLIMB 65K     should be with full power

R-RUDDER TO HOLD HDG.   R-RUDDER TO HOLD HDG. during the proficiency phase

DUTCH ROLLS IN A CLIMB

YOU ARE ALREADY HOLDING RIGHT RUDDER.

PICK A POINT OR A HEADING

BANKS MUST BE THE SAME TO EACH SIDE

TAP LEFT RUDDER

START TO BANK TO LEFT OF 10, 20 OR 30 DEGREES

USE RIGHT RUDDER BEFORE NOSE CAN MOVE

REVERSE DIRECTION OF BANK

RELAX RIGHT RUDDER BEFORE NOSE MOVES RIGHT

TAP LEFT RUDDER AND USE LEFT AILERON

START BANKING LEFT

10-30 DEGREE BANK

R-RUDDER BEFORE NOSE MOVES LEFT     Pilots usually take two lessons

BEFORE NOSE MOVES  L                Students take five lessons.

KEEP BANK CHANGING                  This is a required skill for crosswind

HOLD R-RUDDER                       landings.

RELAX R-RUDDER

APPROACH STALL        ACCELERATED  STALL     	FULL FLAP GO AROUND

CLEARING TURN         CLEARING TURNS            FULL POWER

CARB HEAT             45 DEGREE STEEP TURN      HOLD NOSE LEVEL

PWR TO 1500           CARB HEAT/YOKE BACK       R-RUDDER-HOLD HDG.

HOLD HDG. & ALT.      REDUCE PWR/YOKE BACK      FLAPS UP 20

WHITE ARC             REDUCE PWR/YOKE BACK      MILK BELOW 50KTS

FULL FLAPS            REDUCE PWR/YOKE BACK      60-65 KTS

HOLD HDG. & ALT.      PWR OFF/YOKE FULL BACK    FLAPS UP

20 DEGREE L-R BANK    IF ANY LOSS OF ALT.       CLIMB

HOLD ALTITUDE         START OVER                R-RUDDER-HOLD HDG.

BUFFET OR STALL       BUFFET OR STALL

YOKE FORWARD          USE AILERONS TO LEVEL

LEVEL WINGS           FULL POWER

FULL POWER            R-RUDDER TO HOLD HDG.

FLAPS UP TO 20 DEGREES  (Not FAA required)

CLIMB SPEED 60-65 KTS

FLAPS UP

CLIMB 65 KTS

R-RUDDER TO HOLD HDG.

EMERGENCY CHECKLIST

CHECKLIST                      PRE-CRASH

BEST GLIDE 60 KTS               BELTS/HARNESS

HOLD NOSE UP                    FUEL OFF

3-4 DOWN TRIM                   CRACK DOORS

FIELD AND WIND                  ELECTRIC OFF

RE-START(Guages,Fuel,Mixture,Magnetos)

121.5/7700

MAYDAY ETC X3

FLAPS WHEN CERTAIN

FULL STALL LANDING

Side Notes:

Pilots do not usually have the best-glide numbers available.  A short cut is just to trim the aircraft full nose-up without flaps and go for the ride.   You can gain more distance if you decelerate slowly.  Once fully trimmed, fly with just rudder.

SOFT FIELD PROCEDURES           SHORT FIELD PROCEDURES

TAKEOFF           LANDING           TAKEOFF          LANDING

10 DEGREE FLAPS    60 KTS/1500 RPM       ROLLING or         FINAL 55KTS

YOKE WELL BACK     FULL FLAPS            LOCKED BRAKE       FULL FLAPS

KEEP IT ROLLING    FLARE LEVEL 60K       FULL POWER         1500 RPM

RWY HDG/F-POWER    REDUCE POWER-OFF      HOLD LEVEL         AIM SHORT

RAISE NOSE-MORE       ROTATE 50-55       ROTATE SHARP       MAX.ANGLE LIFTOFF

R-RUDDER-HDG.      HOLD OFF W/YOKE.      CLIMB 60 KTS       POWER BACK

CLOSE TO GRND.     PWR 1300 RPM          10 COUNT 50'       BRAKES ON

LOWER NOSE,        NOSE HIGH             CLIMB 65 KTS       FLAPS UP

60-65 KTS CLIMB    MAIN GEAR ONLY

200' FLAPS UP      FLAPS UP

CLIMB 65 KTS       HOLD YOKE BACKKKKK

TRIM

GO AROUND #2

GO AROUND #1                  AIRSPEED BELOW 60 KTS

SPEED 60 KTS UP                  FULL POWER/CH OFF

FULL POWER/CH OFF                LOCK ELBOW ON DOOR

FLAPS UP                         MILK FLAPS UP 20 DEGREES

CLIMB ATTITUDE                   HOLD LEVEL/ACCELERATE

R RUDDER/TRIM                    AT 60 KTS CLIMB

LOCK ELBOW ON DOOR               R RUDDER/TRIM

SHORT APPROACH                  	SHORT-SHORT APPROACH

Abeam numbers                       Abeam numbers

C.H./1500 RPM                       C. H. POWER OFF

FULL FLAPS                          FULL FLAPS

Fine trim 60 kts                    IMMEDIATE TURN TO BASE

BASE TURN 45 DEGREES                Fine trim for 55 kts

# C-150 POH

1976 Cessna 150 Required Information

Conditions are standard, and weight at gross unless otherwise stated.

Maximum Range and endurance with 45 minute reserve

@ 7000'  340 NM for 3.3 hours       Clean stall     48 KIAS

Sea level climb 670 fpm             Flap stall      42 KIAS

Service ceiling 14,000'

Service ceiling is the maximum altitude at which an aircraft can continue to climb 100 fpm.  The safety margin between the highest terrain and the service ceiling should be at least 5000'.

Absolute ceiling is the maximum altitude an aircraft can attain.  At this point the cruise speed, best climb speed and stall speed all equalize.  The aircraft becomes unstable and controls are unable to stabilize flight.

T.O. S.L. Ground Roll  735          Fuel 26 total 22.5 useful

Over 50'    1385          Oil maximum 7 with filter

Landing   Ground roll  445              6 maximum indicated

Over 50'    1075              4 minimum indicated

Maximum Gross Weight 1600 lbs

Wing Span   32?                         Prop length 69"

Wing Loading  10.0 lbs per squat. ft.

Power Loading 16.0 lbs per horsepower

Va (Maneuvering speed) 97 KIAS @ 1600lb  88 KIAS @ 1300 lb.

Vfe 85 KIAS

Vno Structural cruise speed  107 KIAS

Vne 141 KIAS

Vs Minimum controllable

Vso Minimum controllable (with forward C. G.)

Vx best angle  56 all altitudes

Vy best rate  S. L. 68 to 62 @ 10,000'

White arc 42 to 85

Green arc 47 to 107

Yellow arc 107 to 141

The bottom of the yellow arc represents the airspeed where the airframe can sustain a specific design gust without exceeding the limit load.  This means the plane will not fold, spindle or mutilate if the gust is less than the maximum design gust of 50 feet per second (30 on older planes).

Red line @ 141 KIAS

Chandelles, lazy-eights, steep turns and spin entries 95 Kts.  Spins are prohibited with flaps down.

The red panel light on the far right indicates high or low voltage.  The split switches can act as a circuit breaker to protect the system  Reset by operating switches off and back on.  Do this only one time.

Maximum glide speed is 60 KIAS and windmilling prop

What if prop is stopped?

Power off landings:  Without flaps @ 65 KIAS

With flaps    @ 55 KIAS

Ground fire procedures?

Electrical fire procedures?

Glide range (no wind) at 60 KIAS

@6000'  8 NM

@3000'  4.5 NM

Altitude required to execute power off 180 degree turn.

Altitude required to execute power off 240 degree turn.

Carburetor Ice as affected by application/removal of carb heat.

Short field approach speed with 40 degrees of flaps is 52 KIAS

Maximum crosswind component is 13 Knots.

Figure fuel remaining in C-150 for flight from CCR to RNO.

What is the maximum weight authorized for the luggage compartment.

At gross weight the allowed C.G. travel is only five inches.

Fuel Consumption

Fuel consumption will vary widely from those of the aircraft manual.  Cessna 150's have been known to consume up to 9 gallons per hour.  A PA-28 180 can be out of fuel in 3 1/2 hours, from full tanks, if the carburetor is out of adjustment.  Failure to know the current, as loaded, fuel consumption is just as dangerous as not checking the tanks in the first place.  The psychological readiness of a pilot is in a large part made up of his intellectual awareness of aircraft and his own capabilities.  Be on the ground after three hours.

Gauges

By FAR the fuel gauges are required to read accurately only when the tanks are completely full OR empty.  When the gauge reads at the quarter full marker on both tanks  the manufacturer says there are only

three gallons of fuel left.  At best this is about 20 minutes worth.

# Building a Checklist

The transition to a higher performance aircraft will bring into the situation the good, bad, and ugly habits that have been learned while in training.  Essential to any transition is use of trim, throttle, and

airspeed control.

The POH checklist is designed to remind the pilot of the minimums required for safe operation.  It is a memory aid used to overcome short-term memory loss.  The checklist places in order a number of actions or procedures that require action or verification.  A checklist is not a "how-to" manual.

A checklist has items ranked in importance.   Critical items affect safety, non-critical items affect efficiency and convenience.  A checklist title for a given phase of flight would identify when the critical items are to be checked, list them in an orderly sequence, and provide for verification.  This touch and verify method is recommended by the FAA for single pilot operations.  The non-critical items list should be triggered by the completion of the last critical item.

Checklists should be done when they will least cause flying problems.  A heads-up positioning of the list is best.  The closer you are to making the landing approach the less likely will be the completion of a checklist.

I suggest checklist "triggers" such as being established on an assigned particular heading, on the 45 entry, at pattern altitude, etc. to help you decide "When".  The initiation of a checklist should be keyed to a specific event common to your flying.

Most important that you develop habit of use unless you plan to

limit your operations to most simple of aircraft. Using the POH you should develop your own checklist to fit your method of operation.  You should no more use someone else’s checklist than wear their shoes.  I have found several ideas convenient.

(1) Since preflight is longest make it on a 3x8 card hung

around neck.  Use backside for emergency check list in

red.

(2) 3X5 card with pre-takeoff ground lists on one side and

post-landing ground operation on the other.  Different

colors of ink.

(3) Flight operations card with climb, cruise, and checkpoint

lists on one side and descent, pre-landing, and landing on

the other.

(4) Make yoke clip using broom clip and wide notebook clip.  			Bolt them together using four nuts to prevent movement.

(5) To make your own checklist use a tape recorder to record 			everything you do for a given operation.  Use recording to 	  	      make a rough draft.  Crosscheck your draft to include every 		item from the POH.

I have found that it takes up to five revisons before a list should

be considered suitable for lamination.  (See checklist)

Checklist errors are of at least five types.

1. The list is ignored

2. A critical item is omitted

3. Verification is falsely noted

4. Use of the list is delayed

5. The list is not completed

Once you have developed a complete checklist, you should begin to refine and systematize the material over and over.  The list of items should be shortened, combined, and revised until it actually fits into your flying as something you usually do, in the order you do it, and when you do it.

The emergency checklist should be a memorized for immediate action.  Do what you need to do and then use the list to verify that nothing has been omitted.  An inaccessible emergency checklist is useless.

Sample recommendations:

1. Use laser or jet printer if possible

2. Extended text should be in lower case.

3. Limit to 2 types of font for emphasis

4. Use black on white except for emergency

5. Avoid multi-colors except for different aircraft types

6. Laminate (after fifth revision) with non-glare plastic    		7. Print size should vary with age of pilot.

# Performance Synopsis

Cessna-back of front cover:

Weight, speed, range, climb.

Service Ceiling, take-off, landing, stall

Empty weight, useful

Fuel, oil, propeller, engine

OPERATION:

Checklists

Preflight to shutdown

Fuel system-cautions/operation

Electrical system-cautions/operation

Gear system-cautions/operation

Engine system-cautions/leaning/operation

PERFORMANCE:

Take-off/landings/flap settings/gear

Climb/cruise/cold/hot

EMERGENCY:

Engine/fire/electrical/gear

LIMITATIONS:

Gross/load factors/speeds/engine

Weight-balance chart/graph

OPERATIONAL DATA:

Airspeed corrections

Stall speeds

Takeoff/climb/landing charts

Cruise/glide

1.  Don’t try to climb to 9,500 in a C-150 unless you must.  A C-150 flight 	to Las Vegas can be a 12 hour roundtrip regardless of wind direction.

2.  Select an altitude appropriate to the distance.  Avoid 3000’.  Remember 	local flights tend to stay below 3000.  No need to go high for short 	distances.  Most pilots tend to fly at even 500s even below 3000.  	Choose a unique altitude so as to avoid traffic.  Above 3000 AGL you 	must fly according to the hemispheric rule.  Fly at 7,500 or 8,500 to 	minimize traffic conflicts but be aware when you cross, parallel or 	fly airways.  Be sure to check with FSS prior to flight or with 	Flight Watch if your flight will cross military training routes.

3.  Choose an altitude appropriate to the terrain and airports.  This 	means that route selection may be predicated on several factors.

4.  Select an altitude appropriate to the winds.  Winds usually increase 	in velocity with altitude.  Plan accordingly.

5.  Select an altitude with reference to special airspace restrictions, 	local hazards and cloud layers.  (It is more likely to be a rough 	flight below clouds.)