# Click on page number for instant scrolling.

Getting the weatherã

Fax Weather

WEB Weather

Radio Weather

Getting the Weather

The New FSS

Route Forecasts

WinDs and temperatures Aloft Forecast (FD)

Prognostic Charts

Weather Depiction Chart

Radar Summary (SD

In-Flight Advisories

AIRMET (WA)

SIGMET (WS)

CONVECTIVE SIGMETS

PIREPS (UA)

NOTAMS

NOTAM TYPES

NOTAM "D"

NOTAM "L"

FDC NOTAM

PLANNING WITH DUATS

# Fax Weather

Accuweather 800 438-9847

American  Flight Service Systems  800 432-3265

Weatherfax  800 359-4242

ZFX  800 876-1232

JeppFax

You only pay for what you use

Information-- 800/621-Jepp

Interpretation-- 800/FLT-0126

Computerized Weather:

Evergreen Software

970 731-9550

DUATS (Direct User Access System)

Contel (800) 767-9989

DTC (800) 245-3828

FSS (800) WX-BRIEF

Advantages:

Hard copy

Clearer than Government issue

Redrawn maps

Composite chart of radar + weather systems

Two-panel chart option

Near-real-time radar

WeatherMation PC Modem Briefing Numbers  (Fresno) (209) 498-1027

Information 1-800-367-6602

# WEB Weather

http://www.avweb.com

http://thunder.atms.purdue.edu/

http://cirrus.sprl.umich.edu/wxnet/radsat.html

http://thunder.met.fsu.edu:80/nws/public_html/wxhwy.html

http://www.usatoday.com/weather/wpilots0.htm

http://www.usatoday.com/weather/wpice0.htm

http://rap.ucar.edu/staff/gthompsn/cur_wx/aviation/html

http://www.awc-KC.NOAA.gov/awc/Neural_NetIcing.html

http://www.awc-KC.NOAA.gov/awc/AviationWeatherCenter.html

http://www.awc-KC.NOAA.gov/awc/awc-airmets.html

http://www.nws.noaa.gov

http://www.atmos.uiuc.edu

http://wxp.atms.purdue.edu

http://www.aviation.uiuc.edu/institute/avilinks/weather.html

# Radio Weather

Any FSS

122.1 and listen on VOR is almost completely phased out

122.0 Flight Watch can be contacted almost nation wide if plane is

5000' AGL

135.7 Flight Watch for high altitudes and as alternate to 122.0

122.2 Nearly universal FSS frequency if you can't use a discrete one.

TWEB  Transcribed weather enroute broadcast.

Over NAVAID frequencies with weather changes as they occur,

advisories, winds, and local NOTAMS

HIWAS High Altitude weather advisory service

Transcribed in flight advisories using AWW, WST, WS, WA, CWA, and 		FSS

# Getting the Weather

You can learn the weather by watching the sky.  As clouds occur, correlate their types, altitudes and seasons with flight conditions.  Clouds may be classified by shape but its trend of change is more significant.  Changes in color, from pearl-like to lead-like give a feel for the presence of trouble or lack of it.  Clouds can grow vertically, flatten, fall apart, change color, transparency, and luminosity.  Clouds precede weather frontal systems.  Where one weather front meets with another we have a zone of discontinuity between air masses giving a change in weather.  Clouds are the advance salesmen of weather fronts.

The difference between the world systems occurred because the U.S. had limited coding ability via Teletype.  The rest of the world could transmit

faster and developed a different code.

There are seventeen terminal forecast sites from Bakersfield  to the northern Calif border to make SFO  forecast.  14 more are south of Bakersfield for LAX forecast.  Nevada has 6 for Reno forecast.  California has three upper air wind and temperature forecast sites; 5 more give wind and temperatures.  Sacramento is the only northern weather radar site.  Southern Calif has 6 such sites.  California has eleven inflight weather advisory (Flight Watch) sites remoted to OAK and LAX.

# The New FSS

115 Weather Forecast Offices in the U.S.

Data automation, Doppler radar, satellite pictures

Briefer

Standard briefing:

Synopsis - where weather is (watch TV) This tells you where the 		fronts and pressure systems are and where they are ‘expected’ to 	move.  This is the core information all the rest is details, details, 	and more details.

Hazardous weather - Try not to interrupt but this might be where you cancel 	flight.  Ask for additional information or alternative routes.

Current weather - Ask questions of Flight Watch (122.0)

when real time and forecast weather differ.  Get frequent weather 	updates if conditions begin to change.  Monitor 122.0 to keep up to 	date with what others are encountering.

Forecasts from route terminals - This tells you how the weather is 	‘supposed’ to progress.  By getting an update before departure you 	will get an idea as to how accurate the forcast is.  Time is the 	greatest variable in accuracy.

Area forecast - Winds aloft - When weather does not occur as forcast it can 	be detected in changes in the winds aloft tables.  Weather moves with 	the wind.

- NOTAMS  (DUAT best source for unpublished notams. but giving 			"L"s is not in their contract)

- You must ask briefer for published notams.

- Local user requirements

- 'D" Distant beyond FSS area

- 'FDC' Regulatory system-wide

Ask briefer for Class II notams and consult the Airport/Facilities directory.

# Route Forecasts

300 route forecasts

Routes are numbered

Weather to 25 miles each side of route

All airmets, sigmets and convective sigment from Kansas City

# WinDs and temperatures Aloft Forecast (FD)

- Winds and temperatures aloft from 176 locations in continental U.S. and are generated from Washington, D. C.

- Header states day and time for data base.  Valid time is a twelve hour prognosis that is valid for only nine hours.

- Winds in 10 degrees segments from true north

- Winds aloft information is usually inaccurate but rarely off by more than 30-degrees or 10-knots.

- Temperatures in Celsius

Very close approximation to Celsius temperature in Fahrenheit can be 	Obtained by doubling the Celsius ° and then subtracting 10%.  Works!

- Require interpolation for intermediate altitudes.

- Radar system in west is FAA, in East combined FAA and NWS

- Clues to upper-level wind shear and z-level.

Metar suppemental information given as

WS13 means windshear and two digit runway number or ALL for

all runways

RE is a recent weather of significance indicator

RMK  is an indicator of remarks to follow, U.S. only.

Metar supplemental information given if:

- Turbulence if 4 to 8-kt change per 1000' between levels

- Turbulence if 40 kt wind speed change in 150 miles

## Charts

Radar and satellite tell what is happening now.  Charts depict either old or what may happen.

- Surface Analysis Chart

Maps position of fronts, highs, lows, and isobar lines of equal 		pressure indicative of wind direction and strength.

- Weather Depiction Chart

Shows similar to Surface Analysis chart but shads in areas of 		visual, instrument, and MVFR weather.

- Radar Summary

Shows area of precipitation

- Upper Air constant Pressure Analysis Chart

5,000, 18,000 and 30,000 feet.

If these show U-shaped bend indicates cold front. Best advice is 	never to fly to the east side of such a trough.  Over 40 knot winds 	above 10,00’ mean possible mountain waves if winds are perpendicular 	to mountain ridges. 500 mb charts are at 18,000’ sthese winds steer 	storm movements.  500 mb flows across the country fortell good 	weather.

- Composite moisture Stability Chart

Outlines stable/unstable area and moisture content

- Severe Weather Outlook Chart

Plots area of thunderstorms.  Hatched areas is possible T+'s		Convective areas outlined.

- Low-Level significant Weather Prognosis Charts

Specific multi-panel shadings of forecast for 12, 24, 36, and 48 		hours.  Expected positions of fronts, weather systems, precipitation 	and VFR, MVFR, and IFR conditions.

- Severe Weather Watch

Posted for specific times, routes, and areas of T-storms.

- Convective Outlook

Shows where T-storms are possible, mentions stability, and were 		precipitation is falling.

- Convective SIGMETS  (WSTs)

Describes area of actual storm activity

- Center Weather Advisories (CWAs)

From Air Route Traffic control Centers (ARTCCs) to advise of 		weather avoidance routes.

- Surface Observation (SAs)

- Pilot Reports (PIREPS) (UA)

- Terminal Forecast (FT)

- Area Forecast (FA)

# Prognostic Charts

Pictorial of expected weather good for planning

- Prognostic below 24,000 feet

- 4 times daily of two sections

- 4 panel, 2 -12 hour and 2 24 hour

- Legend to determine IFR, marginal VFR, turbulence and 				tops of turbulence

- Second section gives 36 and 48 hour forecasts.  The further off 			 the more inaccurate.

# Weather Depiction Chart

From 01Z every 3 hours

Sky cover/ceiling

Weather vision obstructions and visibility

- IFR Ceiling below 1000, visibility below 3 miles

- MVFR Ceiling 1 to 3000, visibility 3 to 5 miles

- VFR Ceiling over 3000, visibility over 5 miles

Does not give route weather

# Radar Summary (SD

The SD is a good indicator of past forecast accuracy.  If the SD shows

precipitation not forecast for that time period it indicates inaccuracy as would the absence of precipitation.  The chart is a pre-planning aid that must be updated before flight.

The SD is a Polaroid of a moment at 35 after the hour that will not be issued for another twenty minutes and become available only after the hour.  This means that any SD you see could be as much as an hour and a half old.  This be significant to the pilot only as a predictor of accuracy since thunderstorms seldom last over thirty minutes.

Radar information is a composite of many different radars that helps fill in the picture of weather over the past few hours.  East of the Rockies and the Pacific Coast is given better coverage than the mountain regions.  Radar is a detector of water in the atmosphere.  The processing of a chart takes about an hour and the data may be two hours old.  the vest way to get current radar is to visit an FSS having a weather radar scope.  Radar is poor as detecting clouds unless the moisture level is quite high as in thunderstorms.  The SD is commonly used to detect thunder storms  as they were, a trend indicator of development, but not as they are.

The SD chart tells you where the storms were, where they came from, and where they seem to be heading.  The echoes are given as areas of returns, lines of precipitation, and as cells.  Areas between radar sites may show returns even though there is no actual coverage.   A dashed line box indicates either a WS (storm watch) or WT (tornado watch)  The number after WS/WT tells how many this year.  NE = no echoes with radar working, OM = out for maintenance there may be echoes,  NA = radar out of service there may be echoes.  A + or - after a radar indication is not related to intensity it indicates the returns are getting stronger or weaker.

Shaded areas show rain showers the number of contour lines inside the shaded areas indicate the intensity.  Each line counts two levels of intensity..  Level I,II has an outline,  III,IV has one interior contour and V, VI has two interior lines.

Generally the higher the top the worse the storm.  Bases over three thousand imply virga and microbursts.  When virga evaporates, it cools and

this cooling increases down drafts.  At extra cost radar information is available through DUAT.

## Radar (SD) Information

- Areas and movements of precipitation and thunderstorms

- Type, intensity, trend, configuration, coverage, tops, bases and movement.

- No clouds or fog

- Gives Density/velocity of weather out to 125 miles

- Gives vertical information and can distinguish between types and movement of weather systems.

Printout:

Jan 1835 AREA 5TRW-/+ 140/92 221/102 340/42 010/62 080/80

C2327 MT 380 AT 150/43

Interpretation:

35 minutes after the hour, area 5/10s covered with light thunderstorms expected to increase in intensity.  Azimuth/distance of cell activity.  Cells moving from 230 at 27-kts.  Maximum tops 38,000 azimuth of 150 and distance of 43 miles.

# In-Flight Advisories

All are issued from Kansas City Center Weather Advisory (CWA)

# AIRMET (WA)

Alphabetical

Sierra,

for moderate icing

Tango

for moderate turbulence

30+ kt winds

Zulu

IFR conditions

Ceilings below 1000

Visibility below 3 miles

Mountains obscured

Example:

ZCZC MKCWA5Z

WAUS1 KSLC 232045

SLCZ WA 232045

AIRMENT ZULU UPDT 3 FOR ICE AND FRZLVL VALID UNTIL 240300

AIRMET ICE...ID MT NV WA OR CA AND CSTL WTRS

FROM YQL TO BZN TO EKO TO RNO TO 150W UKI TO 120W TOU TO YDC TO YQL OCNL MOD RIME/MXD ICGICIP ABV FRZLVL TO FL200...WITH LCLY SEV CONDS POSS IN AREAS OF FZDZ/FZRA SFC AND ALF OVR WA/NRN OR/NRN ID/WRN MT. FRZLVL SFC-080. CONDS OVR WA/OR/ADJ CSTL WTRS SPRDG E AND S OF RMNDR AREA BY 02-06Z...CONTG THRU 09Z

Translation:

AIRMET for icing over Idaho, Montana, Nevada, Washington, Oregon, California and coastal waters from Lethbridge, Alberta, to Bozeman, Montana, to Elko, Nevada, to Reno, Nevada, to 150 miles west of Ukiah, California, to 120 miles west of Tatoosh, Washington, to Princeton, British Columbia to Lethbridge, Alberta.  Occasional moderate rime and mixed icing in clouds and precipitation above the freezing level to Flight Level 200 (twenty thousand) with locally severe conditions possible in areas of freezing drizzle and freezing rain at the surface and aloft over Washington, northern Oregon, northern Idaho, western Montana.  Freezing level from the surface to 8999 feet above sea level.  Conditions over Washington, Oregon and adjacent coastal waters spreading east and south over the reminder of the area by 0200 Zulu to 0600 Zulu, continuing through 0900 Zulu.

# SIGMET (WS)

Alphabetical

November through Yankee

Severe icing (not TRW)

Severe turbulence (not TRW)

Visibility below 3 miles

# CONVECTIVE SIGMETS

Severe thunderstorms forecast based on latest radar reports

for two-hour period.  Reliable.  Should ground light aircraft.

Tornadoes

Lines of thunderstorms

Embedded thunderstorms

40% TRW coverage level 4 or better

3/4" hail

winds s50+ kts

volcanic eruption

# PIREPS (UA)

(Pilot reports)

When in-flight advisories are in effect it is important that PIREPS be given to confirm or negate such advisories.  The FAA can take enforcement action against a pilot who reports icing even though that information may be vital to the safety of other pilots.

Pilot reports include:

- Type of aircraft, position, altitude, and flight conditions

- Cloud cover- bases and tops

- Turbulence and icing

- Visibility/restrictions

- OAT (air temperature)

Other

PIREP Elements     Code              Contents............................

Station identity	xxx		Nearest station to report

Report type				UA or UUA  Routine or Urgent

Location		/OV		Relative to a VOR location

Time			/TM		Coordinated Universal Time

Altitude		/FL		Essential for turbulence/icing information

Type aircraft	/TP		Essential for turbulence/icing information

Sky cover		/SK		Cloud height and coverage

Weather		/WX		Visibility, precipitation, restrictions

Temperature		/TA		Degrees Celsius

Wind			/WV		Direction TRUE, speed in knots

Turbulence		/TB		AIM 7-21

Icing			/IC		AIM 7-20

Remarks		/RM		To clarify

A PIREP is the closest we can now come to real time weather in the cockpit that applies directly to a specific flight.  An weather that affect aircraft safety or operations justifies a PIREP.  Occasionally two pilots will give differing perspectives of the same weather phenomena.  The absence of PIREPS is a major problem.

# NOTAMS

(Notices to Airmen)

Time critical information to pilots.  Every 14 days NOTAMS are published in "Notices to Airmen" (NTAP)($125) and are dropped from FSS Service "A" weather briefings.  Published information is not part of a standard weather briefing.  Permanent changes will later appear in the Airport/Facility Directory and will never again appear as a NOTAM.

# NOTAM TYPES

148,000 notices a year	with an average of 1,400 active per day.

Uses National Weather Service data from:

FA (TAF) Terminal Area forecasts

FT Terminal Forecasts

SA Sequence reports

WW

AWW Severe Weather Forecast Alerts

WST convective sigmets

WS Sigmets

Wa Airmets

Center advisories

# NOTAM "D"

(distant)

- Important hazards or changes

- Would mention runway problem but not taxiway problem

- Appended to hourly weather reports

- Mostly temporary

- In FSS data if unpublished

- Can be issued by any airport

# NOTAM "L"

- Potential hazards

- Would mention taxiway problem but not runway problem

- Regional only.  Can be issued by any airport

With closing of local FSS "regional" has become much 	larger.  If you call the 800 FSS number and get connected with Walla Walla Washington, as I once did, you may not get any "L"s because the briefer does not have them.  I use a local FSS phone number if possible.  Distributed Service "B" by FSS to local ATC facilities

# FDC NOTAM

Data related to procedures added, eliminated, changed.

- Flight Data Center (FDC) NOTAMS

Regulatory to IFR system

Issued by FAA Air Traffic Control, Airports or by FAA 			Flight Standards

Generally permanent changes approach charts and other charts

May include temporary airspace restrictions.

- Local FDC NOTAMS will be given but over 400 miles away must be 				specifically requested from FSS

# PLANNING WITH DUATS

Based on the following weather can you make a five and a half hour round trip flight with a one lunch/fuel stop from Kansas City to Brainerd MI and back.  The distance is 490 nm from KOJC TO KBRD BY WAY OF STJ VOR, OVR VOR, AND KSUX, THEN TO DDL IA, OTG MI, RWF MI, ILL MI, KBRD.  all leg times should be converted to ZULU time to make the weather report times conincide with your ETAs.  Departure is 1900Z and landing about 0100Z.  The outlook for SD, ME, KA, MI, IA and MO is VFR with 8000’ clouds.  MI shows scattered clouds at 2500 between 1300 and 1500Z.  Our departue will be 4 hours after this occurance.  Turbulence is to east and south of route.

The AIRMET Zulu has ice in clouds below 14 thousand. around DSM.  VFR

should not be a problem at lower altitudes in scattered clouds.  METARs show 10 mile visibility below 12 thousand in cold air but wide dewpoint spread. No fog that way.

TAFs show 6 mile visibility and good VFR below 12 thousand but by KSUX we will have overcast at 10 thousand.  At KBRD at 0100 the wind will be 320 at 12 knots so we must pick an appropriate runway.  The NOTAM warns about braking.

The next day the reverse route for a 1600Z departure the TAFs between 1300-1500Z the ceiling is broken at 4 thousand and lowering at KBRD.  KSUX will be getting worse down to MVFR.  KOMA is much the same and trending lower.

Any precipitation is going to be snow because of the temperature.  This means snow on any airport runways.  No go.

FA

EE AIRMET SIERRA FOR IFR CONDS AND MTN OBSCN. TS IMPLY SEV OR GTR TURB SEV ICE LLWS AND IFR CONDS. NON MSL HGTS DENOTED BY AGL. OR CIG.

SYNOPIS...11Z CDFNT ALG A SSM-MBS-EVV-30N ARG-OSW-GLD LM...CONTG AS A QSTNRY FNT ALG A GLD-BFF LN. ANOTHER CDFNT ALG A 50N MQT-DLH-AXN LN. HIGH INVOF LBF WITH TROF EXTDG ALG A LBF-DHT LN. HIGH INVOF MSP

SD: WRN/CNTRL...AGL SCT025 SCT-BKN-SCT080-100. CLDS LYRD FL2500. 15-17Z

AGL SCT080 SCT-BKN1500. OTLK...VFR. ERN...AGL SCT-BKN080-100 CLDS LYRD FL2500. OTLK...VFR

NE: N CNTRL/NERN...AGL SCT025 SCT-BKN100 TOPS 130. 18-20Z BKN080-100. OTLK...VFR. RMNDR...SCT CI. OCNL SCT100. OTLK...VFR.

KS: NRN HLF...SKC. OCNL SCT CI. 15-18Z SCT150 SCT-BKN CI ABV. OTLK...VFR. SRN HLF...SKC OTLK...VFR.

MN: WRN HLF...AGL SCT-BKN010-020 TOPS 050. OCNL VIS 5SM BLSN. 13-15Z SKC.OCNL AGL SCT025. OTLK...VFR.ERN HLF...AGL SCT-BKN010 CIG BKN030-040 TOPS 070. VIS 3-05SM -SHSN. 13-15Z AGL SCTO25=035. OTLK...VFR

IA: WRN HLF...SKC. 15-18Z SCT100. OTLK...VFR. ERN HALF...AGL SCT-BKN 020-030 TOPS 070. 13-15Z SCT CI. OCNL AGL SCT025 SCT100 NRN SXNS. OTLK...VFR.

MO: SKC. 18-020Z SCT CI. OTLK...VFR.

AIRMETS

AIRMET TANGO UPDT 3 FOR TURB VALID UNTIL 062100

AIRMET TURB..MN IA WI IL MI IN KY LS LM LH LA TN MS AL..UPDT FROM YWG TO YQT TO YVV TO DTW TO FWA TO CVG TO HNN TO TRI TO CHA TO 50SW ABY TO 40W CEW TO 90SE MSY TO BTR TO MEI TO MSL TO STL TO YWG. OCNL MOD..ISOL SEV..TURB BLW..080 OVR ERN KY/ERN KY..BLW 060 ELSW DUE TO OCNLYSSTG AND GUSTY NWLY LOW LVL FLOW ACRS AREA. ISOL SEV CONDS MAINLY AL PTNS AREA. CONDS CONTG BYD 21Z...ENDG BY 03Z.

AIRMET TURB...MO IL IN KY AR TN MS AL. FROM BRL TO CVG TO HNN TO TRI TO CHA TO 50SW ATL TO TXK TO MKC TO BRL. OCNL LGT/MOD TURB BTWL FL 180 AND FL370 DUE TO WINDSHEAR. CONDS MOVG EWD AND ENDG BY 21Z.

AIRMET ZULU UPDT 2 FOR ICE AND FRZLVL VALID UNTIL 062100

ND SD MN IA: LGT..ISOL MOD..RIME ICGIC BLW 140 IN AREA BNDD BY 50NNW ISN-MOT-RWF-MCW-DSM-FSD-RAP-90SW DIK-50NNW ISN.  CONDS DVLPG/SPRDG EWD DURG PD... CONTG BYD 21Z THRU 03Z.

FRZLVL...SFC-040..N OF BFF-LBF-BUM-ARG LN ..040-080..S OF BFF-LBF-BUM -ARG LN

SURFACE OBSERVATIONS

METAR KOJC 061653Z 00000KT 10SM CLR 06/M07 A3044 RMK A02 SLP 303

T00561072 PNO$

METAR KSTJ 061652Z 36007KT 10SM CLR 01/M08 A3047 RMK A02 SLP 324

T00111083

METAR KOMA 061652Z 00000KT 10SM CLR M03/M11 A3048 RMK A02 SLP332

T10331111

METAR KSUX 061655Z 10SM CLR M06/M16 A3047 RMK A02 SLP337

T10611161

METAR KILL 061655Z AUTO 29007KT 10SM CLR M16/M18 A3041 RMK A02

METAR KILL 061715Z AUTO 3006KT 10SM CLR M16/M18 A3041 RMK AO2

METAR KILL 061735Z AUTO 29006KT 10SM CLR M15/M18 A3041 RMK AO2

METAR KBRD 061653Z AUTO VRBO4KT 10SM CLR M12/M18 A3032 RMK AOT

SLP 297 T11171183 FZRANO TSNO

TERMINAL FORECASTS

TAF KOMA 061130Z 061212 35010KT P6SM SKC BECMG 1718 VRB05KT SCT120

FM2200 12010KT P6SM SCT100 BECMG 0506 BKN100

FM1200 12012KT P6SMBKN025

TAF KSUX 061130Z 051212 31012KT P6SM OVC100 BECMG 2022 VRB05KT

FM0100 12008KT -6SM BKN050 0VC100 PROB040 0106 5SM -SN BKN35

FM0600 14010KT P6SM OVC030 TEMPO 0610 3SM -SN BR

TAF KBRD 061735Z 061818 31012KT P6SM SKC BECMG 0002 VRB04KT

FM0500 VRB03KT P6SM BKN100

FM1100 VRB05KT P6SM BKN060 BECMG 1215 BKN040