# Click on page number for instant scrolling.

Non Directional Beacon approachesã

The secret to flying these off airport NDB approaches is: after station

passage turn to the final approach course and maintain that course for

at least 30 seconds, despite what the needle is telling you ... this

is what the examiner wanted me to do (I passed yesterday).

Keeping this lesson in mind should make flying NDB approaches

much simpler in the future.

The NDB approach is generally considered one of the tougher

items on the flight test for the instrument rating.  If you

asked a bunch of instrument-rated pilots to list their favorite

approach equipment, you'd probably get a list something like this:

#1   ILS

#2   LOC only (no glideslope)

#3   VOR

#4   NDB

The ILS (Instrument Landing System) is every IFR pilot's

favorite.  It consists essentially of two indicators, one

for "go right/go left" and another for "go up/go down".

The LOC (localizer) only and VOR approaches look pretty much the

same as an ILS except that they don't have the "go up/go down"

glideslope needle, just a "go right/go left" needle.

Simple NDB Approach

The difference between the NDB and the others is that there isn't a single dial in your cockpit that you can refer to, to see your tracking error.  You must combine the information presented by the Hi and the ADF, if you wish to accurately track to or from an NDB.

You can practice learning to track using the NDB.  When flying cross-country, work on finding the proper crab angle.  Good visualization

practice.  But during the approach, people under stress may misinterpret the dials and ‘correct the wrong way.

Flying the NDB approach, which has an NDB located 4 miles west of the threshold of runway 09.

__________________

270  <-----   /------- * -- 090 --> 9__________________|

/

045 / 225

When cleared for the NDB 09 approach, so you could simply

“home” to the NDB, which is your IAF (Initial Approach Fix).

‘Homing’ with an ADF simply consists of keeping the needle on

the ‘nose’ of the aircraft, or on the zero at the top of the ADF

indicator.

With a crosswind, our just flying the needle will get us there.  Spend any excess time studying the plate.

get the important items:

A pproach   - the right plate?

M inimums   - MSA, PT, FAF, MDA

O vershoot  - quickly review which way, how high, how far

R adios     - ADF tuned and identified.  Radar, ATC, CTAF

T imes      - get a time to the Missed Approach Point (MAP), VDP?

S peeds     - groundspeed based on wind

Passing the NDB outbound, some type of procedure turn.  Course reversal is easiest but always to the published turn side.  Otherwise, visualize what the wind is doing to you on each straight leg of the procedure turn, and make a correction for it.  A crosswind, crab into it.  A headwind, increase your time significantly.  A tailwind, decrease your time slightly.  Headwinds hurt more than tailwinds.

Cockpit checks now.  Check your Heading Indicator against the compass.  You cannot fly an accurate NDB approach without an accurate heading reference.

Final Approach Track.

Turn initially to a heading of 090 and begin descent started to get your altitude for the FAF (Final Approach Fix).  This is usually lower than your procedure turn altitude.  Be sure to keep a 500 fpm descent rate going, use 600 or 700 fpm if you have a tailwind.

If there's no wind, a heading of 090 will take us right to the NDB. Rather than using any tracking and correction technique just simply

home to the NDB.  Needle on the nose, back to the NDB.

By homing to the NDB, we are assured of a station passage.  On Passing the NDB (the FAF) inbound, turn back to 090 and start a descent, down to your MDA (Minimum Descent Altitude).  Eessentially, after station passage just fly a heading of 090 until you see the runway, or your timer runs out at the MAP.  At a ground speed of 90 knots, it will take 2 minutes to travel 3 nautical miles to a 1 mile final for runway 09, at which time you should be visual with the runway, and start your descent

from the MDA.

But let's say we've got a 30 knot direct crosswind, which is a lot at 500 feet AGL.  If we get a good station passage, and just ignore the wind by flying a heading of 090, after 2 minutes the crosswind will have pushed us 1 nm off course.  Instead of being 1 mile from the runway threshold, we will be 1.4 miles, which is a pretty good return for such a simple procedure.

If you think about it carefully, it's actually going to be better than that.  With a strong wind from the north, while homing to the NDB after the procedure turn for the FAF, let's say we eventually weathervaned to a heading of 070 at station passage.

After we pass the station inbound, during our turn from 070 back to 090 we will travel somewhat north, or upwind of the desired track, so we probably won't drift a whole 1 nm downwind.

Also, if you have a higher groundspeed than 90K, you will be affected less by the crosswind.  There's less time for the crosswind blow you off course after passing the FAF.  The neat thing about the above NDB approach is that at no time did you ever have to track using the ADF.  sYou always flew and maintained headings

If you’re fussy:

Slightly modify the above procedure.  With a groundspeed of

90K, we know that two minutes after passing the FAF we should

be visual with the runway, and in position to start descending

on final.

So, passing the NDB inbound, turn to a heading of 090 and hold

it as accurately as you can for one minute.  After one minute,

observe the ADF indication.  Let's say that it's now 190 relative.

This means we have drifted south (to the right) 10 degrees during the

one minute after we passed the FAF.

To correct for it, we turn left 20 degrees to a new heading of 070 for the remaining one minute.  This 20 degree correction for a 10 degree drift is exactly the same track correction method you used as a student pilot on your private pilot flight test.

ADF quadrantal error   (ADF installation error)

ADF quadrantal error is the error due to the slight mismatch in gain between the two orthogonal directional antennas.  From a mathmatical standpoint ADFs find the arctan of the signal ratio from two directional loop antennas with a cos(theta) pattern.  At 0, 90, 180, and 270 degrees the ADF transmitter is in the null of one of the two loops and the ADF error due to mismatched gains is 0.  However when the station is between these cardinal points there is an error.  The quadrantal error is often primarily due to the geometry of the airframe that the antenna is mounted on.  GA aircraft sometimes have an adjustment for quadrantal error.  On newer large air transport planes the quadrantal error correction is sometimes set via jumpers in the avionics mounting tray connector.  In either case the correction is accomplished by adjusting the gain of one of the loop antenna channels to offset the gain difference due to the airframe geometry.