# Click on page number for instant scrolling.

IFR Trainingã

Requirements

Cockpit Check

Knowledge Areas Required

PRACTICAL TEST STANDARDS

Acceptable standards

Reasons for failure

Resource management/judgment test.

Simulation during IFR Practical Test

IFR Checkride

Application form

ORAL

Instrument weather

Electronic Bulletin Board Access Information

Why?

How Sweet It Is When...

IFR % Accident Frequency as Compared to VFR

# Requirements (Changed August 1997)

FAA decides what training is required as a MINIMUM

- 50 hours must be cross country (over 50 nautical with landing)PIC

- 15 hours of CFII instruction

- The instrument cometency check is now an instrument proficiency check.

and must include holding procedures.

- NDB not required on long X-country which must include three different approaches.

- Human factors and decision-making training required

Instruction should be sure to include experiences such as:

Spatial disorientation

Severe weather, turbulence

Malfunctions

Partial panel

FAR 91.205(a)

No person may operate ...unless that aircraft contains the instruments and equipment specified for that type of operation and...operable condition.

# Cockpit Check

In the PTS the cockpit check comes under preflight procedures Task C of FAA-S-8081-4B dated Oct., 1994.

1)	You must know how instruments work, why the check is needed, the FARs that apply, and how defects and failure can be noted.  Options that exist with failed equipment.

2) 	You must have a functional checklist that includes the POH list.

3) 	Many preflight specifics are required:

1.  Setting and use of audio panel.

2.  Use of all navigation aids

VORs, GPS, ADF, ILS, and Markers

(See 1) above)

3.  Compass

Fluid level can be checked as part of cockpit check.

4.  Gyro instruments

(Instrument Handbook 35 to 54)

Know 'your' airplane.

Clock, pitot heat, turn and slip, static

` When preflighting you should check pitot heat, hot prop and 		  pre-set radios as additionals.  When taxiing we should check 		  AI, HI, and TC during turns.

4) Will any noted discrepancy make the aircraft unsafe for instrument 	flight.

# Knowledge Areas Required

...to be Ready for the Test

Written test, aircraft papers, pilot papers, application

Cross country flight planning

Preflight - IFR specific

Mnemonic

IFR add to VFR requirements

CHART GAD

clock, heading indicator, attitude indicator, two way radio, turn coordinator, generator, altimeter, DME above 24K.

Checklists

Instrument cockpit check

Systems operation

Systems failure

Antennas

Collision avoidance techniques

Compliance with ATC Clearances

Flight reference to the instruments

Constant airspeed climb

Straight and level

Change of airspeed

Constant airspeed descents

Constant airspeed climbs./

Steep turns

Constant rate climbs

Timed turns to compass headings

Holding patterns

Recovery from unusual attitudes

Intercepting and tracking VOR radials

Intercepting and Tracking NDB bearings

VOR approaches

NDB approaches

LOC approaches

ILS approaches

Partial panel

No-gyro approaches

Circling approach procedures

Unusual attitudes

Debrief

# PRACTICAL TEST STANDARDS

# Acceptable standards

An Instrument Rating applicant is not permitted more than three-quarter-scale CDI or glide slope deflection during a VOR or ILS approach and no more than one-quarter scale CDI or glide slope deflection during an ILS approach.  Less that half-scale CDI deflection for VOR approaches, and plus or minus 5 degrees for NDB approaches..  You must be "established" on course when operating on an IFR route segment within those standards.  Instrument pilots are just like athletes; with conditioning, practice, repetition and concentration you will get it right.

# Reasons for failure

Lack of preparation, all plates, altitudes, frequencies

Radar:

Vectors if plate says approach frequency (center may give 			approach vectors but minimums are high.)  Once on vector heading

review plates, set/ident frequencies,

Non-radar:

Altitude and crossing restrictions

Procedure turn/direction

Reasons for failure:

Situational awareness

Reliance on radar

Instrument interpretation

Wrong direction during procedure turn

Late descent for non-precision approach

Turning in wrong direction to heading.

On takeoff we would prefer to have the aircraft cleaned for climb before entering actual.

While the step by step process of configuring the aircraft is best for teaching,  The most likely cause of an approach screw-up is just plain getting behind the plane.

Reference to refer to on how to do a particular maneuver required by the PTS can be located in the PTS below the task required section in each area of operation. The PTS provides a reference list of required documents, materials, aircraft and instruction.  Confirm by reference to the PTS that you are being trained to meet all of the tasks and objectives listed in the PTS.

# Resource management/judgment test.

Use ILS if available it actual emergency but examiner may use other approach to check partial panel ability on non-precision approaches.

The instrument PTS regarding an ILS approach states that the standards are no more than 3/4 scale deflection of the localizer or glide slope indications and within 10 kts airspeed.

PTS Emergencies (Keep all plates available)IFR Practical Test

# Simulation during IFR Practical Test

Gyroscopic failure

During the in-flight simulation of partial panel instrument approach, because of the physical position of the magnetic compass in some aircraft, the applicant is likely to receive visual clues not normally available.  Once this occurs, a meaningful evaluation is compromised.  Examiners may request applicant no longer refer to magnetic compass.  If actual ATC assistance is unavailable the examiner may simulate ATC assistance procedures.

Instrument rating PTS requires non-precision approach under partial panel.  Emphasis will be on loss of gyros and loss of radios.  Test will include timed turns, compass turns to headings. climbs, descents and unusual attitudes. In actual situation ILS would be more desirable.

The expectation during simulated emergencies is that a successful conclusion is the primary objective.  Applicants must demonstrate knowledge of resources available.  (Advise ATC, request radar vectors, call fixes, no gyro assistance, nearest VFR, nearest airport, etc.)

# IFR Checkride

Examiner comments related to failures:

- Unable to handle ATC communications.  Failure to repeat back clearances and instructions.  Problems with frequencies on missed approach.

- Did not know preferred routes, symbols or abbreviations

- Did not have airport/facility Directory

- Use of radials on VOR.  Confusing VOR ident with localizer.  Flew with reverse sensing of VOR.

- Premature descent especially during procedure turns

- Failure to set altimeter prior to approach

- Unable to fly plane while using magnetic compass for headings.

- Failed to recognize that glide slope failure required localizer minimums and timing.

- Failed to adhere to departure procedures.

- Excessive and even incorrect course corrections using CDI.

- NDB approach used homing instead of tracking.

- Failed to fly to missed approach point before turning.

The oral began with a check of my application form, current pilot

certificate, and medical.  Then it was straight to the airplane

logs.  The examiner wanted me to prove that the airplane is legal

fly IFR.  I had gone over the logs the day before, so I was able

to quickly find the log entry for the pitot-static and altitude

reporting inspections as well as the one for the most recent annual,

which happened to also be within the last 100 hours.  Then the

examiner asked about the airworthiness directives (AD) for the

airplane.

My FBO uses a system that calls for of all of the maintenance

information for the airplane to be kept in a three-ring binder.

Finding the log entries is simplified, and the AD information is

right at your fingertips;  each AD has a single page dedicated to

it, upon which is the exact wording of the AD and space for the

mechanic to describe the fix and sign off the AD.  Unfortunately,

I had not looked closely enough at some of the AD entries, and the

particular AD that we looked at was a recurring AD that, according

to the text, must be inspected and signed off every 100 hours.  Alas,

it had been more than 100 hours since the last signoff.  Before we

flew, we had to get a mechanic to check the applicable aircraft

systems and sign the log.

Then came the expected questions about the currency of the pilot,

including the use of a simulator, and general guidelines and

regulations for IFR flying.  I was asked to diagram and discuss

the pitot-static system (could have been the fuel or electrical

system as well).  What is a VOR-A approach?  When should circling

minimums be used instead of straight-in minimums?  Is there ever

any restriction on which direction a circling maneuver may be

made?  (Yes, when so stated on the chart, e.g., circling NA north

of runways 10/28.)  We looked at approach charts and enroute

charts, showing that I know what the symbols means.  What are

MEA, MCA, MOCA, MAA, etc...?  What are the ways of checking a VOR?

One aspect of the oral that I found quite interesting was the fact

that the examiner asked questions that took into account the knowledge

that I obtained during training for my private certificate a year

ago.  For example, an interesting question was a problem that was

posed using an example of a real-life route on my chart, and in the

airplane that I was about to fly.  I was given the following

conditions:  I'm in a Cessna 172 at max gross weight on an airway at

the MEA, 4000 feet.  I'm told that a nearby airport is reporting a

temperature of 102 degrees F.  I'm following a route upon which I'm

approaching a fix that has an MCA of 5000 feet, and another fix that's

just a few miles after that with an MCA of 9000 feet.  Suppose that I

lose two-way communications before crossing the first fix.  When should

I commence my climb?  Of course, my answer was that I should be to 5000

by the first fix and 9000 by the second fix.  But then, after some prodding

by the examiner, I take the temperature and the fact that I'm at max gross

weight into consideration, and into the airplane manual I go, and to

my surprise, I find that I would not be able to make 9000 feet by the

second fix if I'm only at 5000 feet by the first fix, based on climb

gradient information taken from the current conditions. It was an

excellent problem.  And it a very good extension of what I had to know

for my private practical:  density altitude, climb rates (ft/NM) at full

gross weight, etc..

Another question that was interesting regards what I'd do if I noticed

that my vacuum was slowly dropping.  I said that I'd keep in mind that

my AI and DG would likely become inaccurate, and thus I'd rely solely

on the other instruments for pitch and bank information.  The examiner

asked me to imagine this in reality, and we came to the conclusion that

everyone should carry something with which the instruments could be

covered should they become inoperative.  This seems like a good idea to

me, because I would likely still include the AI and DG in my scan, because that's what I'm trained to to.  (Most of our training on partial panel is done with the instruments covered, eh?  So why leave them uncovered if they are actually broken?  And hey, there's a good marketing slogan in there for companies that sell those little instrument inop stickers...

Another question that I found interesting was regarding the maximum bank

angle at which a wet compass is still reliable.  The formula that I've

seen for bank angle for a standard rate turn is approximated by

[(KTAS/10) + 7],

and thus if you're going 120 KTAS, that's 19 degrees of bank (and more

if you're going faster).  Check the literature and you'll see that this

is over the theoretical limit for a wet compass.  This is good

information to consider when losing vacuum;  at certain speeds, one might wish to inform the controller that half-standard-rate turns would be used.

Another question was the following scenario:  I take off from my home

airport, which does not have a precision approach, and weather is

below the lowest minimums at that airport, and as soon as I get into

the soup, my alternator dies.  So I have a short amount of electrical

(battery)  time left;  what would I do?  I thought that the examiner

was testing my knowledge of lost comm procedures when in fact, the

idea was that there is an airport nearby with a precision approach,

and with my remaining electrical juice, it might be better to simply

declare an emergency and shoot that approach, instead of remaining in

the system and risking further problems.  The point of the question was

the idea of situational awareness.  Always have a plan for emergencies.

This, another excellent question, drove the point home.

In general, the oral was a set of questions that required analysis of

the information at hand, rather than simple repetition of memorized

information.

Then came the flight.  Fortunately, from my home airport, there's only

one direction in which to fly where there is an NDB within a reasonable

distance, so my instructor and I concentrated our practice flights

in that area, and this is the direction in which the flight went.  I

am moderately familiar with the airways, radials, and intersections in

that area.

I offered to do a VOR check during run-up, but since I had answered

the questions regarding VOR checks during the oral, I only had to

show in the permanent log that the VORs had been checked within the

past 30 days.

The examiner gave me a clearance to fly, and I made sure that I

could fly it before accepting it (thanks Roy Smith).  The examiner

played ATC during most of the flight.  The route was unique, in that

I had flown a similar one only once before.  It included "...radar

vectors SUNOL intersection victor 195...", and during the flight,

I was told to resume my own navigation and thus fly to the intersection,

which is defined by two VOR radials.  I tried to use my DME and

immediately, the examiner said that it was inoperative until further

notice.

There was more wind and turbulence than on any of my training flights,

and I had about 20 degrees of wind correction at one point while flying

the airway.  I was given hold instructions and asked how I would enter

the hold, then the instructions were amended to expect the VOR approach

at a nearby airport.

The VOR approach went well;  the wind was varying by about 30 degrees

during the approach, and the controller was doing an excellent job

of keeping we pilots up to date regarding its direction and magnitude.

And its magnitude was strong enough to cause me to use a different

approach groundspeed than I would normally use for timing calculation

from the FAF to the MAP.

Next were vectors for the NDB approach.  I wish I had done this approach

better.  I intercepted the inbound course quite well, and there was

little wind correction, but as soon as I passed the FAF, a compass locator, the controller gave a wind report that made be believe that I should change my heading, and I did.  In retrospect, the controller's statement that the wind had changed didn't mean that it changed for me right over the FAF, but I corrected for it anyway, which was a mistake.  I should have stayed with the heading that got me to the fix for at least a minute -- and I know this -- and then corrected based on the needle indications thereafter.   Thus, upon taking off the hood, I was dismayed to see the airport off to my left.

Next was the ILS, which went smoothly, even with the turbulence

through a thousand feet.  The only interesting thing about the ILS

is that the examiner intended for me to remove the hood at DH and continue along the glideslope in visual conditions to a landing, but I started the missed as soon as my altitude was within thirty or so feet of my DH.  I was just going by my experience with my instructor, which is that if he doesn't say anything, I leave the foggles on and go missed.  There are probably a variety of ways to communicate whether a landing is desired or not, and I'll bet this is a problem with student/instructor combinations as well.

Then came the airwork.  There was a vector to climb to a certain

altitude given a heading, and then instructions to do a steep turn

once stabilized.  (After the approaches and before the airwork, the

examiner allowed me to remove the foggles for a minute or so.  Then

I was instructed, "Put the foggles back on when you're comfortable",

to which I responded, "Okay, I'll put them on when we're back on the

ground", to which the examiner responded with a laugh, "Foggles on NOW!"

:-)  I nailed the steep turn.  Then the AI and DG were covered, and I

did compass turns, altitude changes during turns, and unusual attitudes.

After all this turning about, I was asked to show on a map where I was,

and at first I read my OBS indication incorrectly after centering the

CDI needle;  I was off by 10 degrees, but I quickly realized that I

was 10 degrees off when told to fly to intercept an airway and fly to

a fix, and thus correctly stated my position moments after incorrectly

stating it.

On the way to the fix, I determined the type of entry correctly, cross-

checking myself with two different methods.  (This is something that I

learned very late in my training:  I draw the hold on the chart and

draw my direction of flight into it and use that to determine the type

of entry.  Then, if I there is enough time enroute to the fix, I double-

check that with the exercise of super-imposing the 70-110-degree lines

on the DG.  Since determining hold entries was somewhat difficult for

me, I would make mistakes somewhat frequently;  this double-checking

greatly reduced my mistakes.)

There was also enough time enroute to the hold to allow me to consider

some of the winds that I'd be dealing with, and it was great that I

thought about that because my DME and second NAV/COMM became inoperative

(per the examiner) immediately preceding my entry into the hold.  Thus I

was using a single NAV radio to determine the fix, which was defined by

crossing VOR radials, during the hold.  And the hold itself was interesting because my first inbound leg was fifteen seconds long (quick:  how long should your outbound leg be?), and a 20-degree correction on my first outbound leg still didn't seem to be enough.

And then it was over.  The examiner said, "Take off the hood and take

me home."  And here was another way the examiner could determine whether

I've retained my VFR abilities, too:  We returned to an airport that's

under both class B and class C airspace, and thus I had to be down below

the airspace by a certain point.  The flight back was also an opportunity to discuss the flight and the oral portion, but I remained sure that I kept my mind fully on the flying.

In retrospect, the experience was an educational one, and even though I

was nervous, I had fun!  I recognize that I'm not a great and one-hundred percent proficient instrument pilot at this time, but I look forward to using the rating.

I hope this note was interesting reading and I hope that it helps those

that are studying now for their instrument rating.  Other stories in r.a.ifr helped me.

# Application form

During certification check the PTS and FARs to make sure that you meet the requirements.  A most common error is not following direction and completing all required spaces in the application.

Areas which are causing the most frequent problems in the oral phase of the practical test:

# ORAL

# Instrument weather

Examiner will use FAA aviation weather services publication AC 00-45C

Applicant must know how to obtain, read and analyze weather reports and forecasts from AC 00-45C

Weather reports and forecasts

PIREP and radar reports, surface analysis charts.  Radar summary charts, prog charts, winds and temperature aloft, freezing lever charts, stability charts, severe weather, outlook, constant pressure charts, high level prognosis charts.  SIGMETS, AIRMETS, ATIS reports

(Automatic Weather Observation Station)

(Automatic Meteorological Observation station)

What they do...What they give...How different

Reason examiner requires this:

Raise knowledge lever of weather theory

DUATS requires knowing how to read reports and forecasts.

Self briefings are a growing requirement

Weather minimums:

When is an alternate required

Night IFR at uncontrolled airport

Below minimums is O.K. if in pattern within 1/2 mile

SVFR procedures

Navigation:

15% of applicants do not know how to get magnetic course.  Know how to enter ARSA below TCA  (3000' critical)

How WAC charts are different from sectionals

IFR departure from uncontrolled airport

Partial panel approach

Aircraft systems:

Carburetor ice

Induction ice and alternate air source

Static air intake and alternate static source

ANTENNAS - GLIDE SLOPE, marker beacon

An Examiners question:

How does the wire from the wing to the tail help to hold the tail on?

The wire running from the top of the cabin to the top of the vertical stabilizer is the ADF sense antenna? Where is Marker Beacon antenna?

Know gyro instruments and pressure instruments

Know how turn/slip gyro differs from turn coordinator.		Know how attitude indicator erects, dies, acts under low vacuum, 			etc.

Know primary and supporting instruments

LEVEL     TURN    CLIMB   DESCENT

Primary

Secondary

Flight Plan

DVFR when crossing  ADIZ.

Where/what is the ADIZ in the Bay Area?

Required References:

PRACTICAL TEST STANDARDS

FAR PART 61  CERTIFICATION OF PILOTS AND FLIGHT INSTRUCTORS

FAR PART 91  GENERAL OPERATING AND FLIGHT RULES

AC 00-6      AVIATION WEATHER

AC 00-45     AVIATION WEATHER SERVICES

AC 61-21     FLIGHT TRAINING HANDBOOK

AC 61-23     PILOT'S HANDBOOK OF AERONAUTICAL KNOWLEDGE

AC 61-27     INSTRUMENT FLYING HANDBOOK

AC 61-84     ROLE OF PREFLIGHT PREPARATION

AC 90-48     PILOT'S ROLE IN COLLISION AVOIDANCE

AIM          AIRMAN'S INFORMATION MANUAL

SID          STANDARD INSTRUMENT DEPARTURES

STAR         STANDARD TERMINAL ARRIVALS

AFD          AIRPORT FACILITY DIRECTORY

FDC NOTAM    NATIONAL FLIGHT DATA CENTER/NOTICE TO AIRMEN

IAP          INSTRUMENT APPROACH PROCEDURES

Government instrument approach procedures (IAP) are 				published every 56 days, updated every 28 days and 				NOTAMS as required.

Pertinent pilot operating handbooks

FAA approved flight manuals

Enroute low altitude chart

Instrument rating eligibility:

Aeronautical eligibility 61.65 (e and (i

Total

50 hours cross country other than student solo

61.65(c)(4)

250 nautical mile cross country which must include an ILS,

a VOR and NDB approaches. (check)61.65 (II)(III)

IFR Student Requirements

To qualify for the long cross-country IFR flight must include at least two airports 250 NM in a straight line distance apart and 1 VOR, 1 ILS and 1 NDB approach.

Total hours is 15 by a CFII instructor in an airplane.

Maximum of 20 hours in a simulator

GET ON GOVERNMENT MAILING LIST: (See VFR material)

USE    AC 00-2   AND  AC  00-2.7   Oct. 1994

Address:

DEPARTMENT OF TRANSPORTATION  M 494.1

DISTRIBUTION REQUIREMENTS SECTION

WASHINGTON D. C.  20590

# Electronic Bulletin Board Access Information

Safety and Standards

Modem(202)267-5205 Operator Rick Marinelli, (202)267-7669

Policy, Plans and Management

Modem(202) 267-5697 Operator Mike Lee (202) 267-3332

Now that you have your rating comes the hard part.  The on-going need to maintain the minimum of 6-6-6 will continue as long as you fly IFR.  The initial issue certificate is just a license to continue your learning and training.  Now you must be more than just legally current to be safe.

# Why?

Getting your rating made you a better pilot.  You became more selective as to what made weather safe for flying. You were capable of absorbing  multiple instructions while maneuvering the aircraft.  You set new personal minimums for VFR flying because of your capability to go IFR.  Even the FSS talked to you differently after you said, "IFR capable".  When entering VFR conditions on an IFR flight plan you must see and avoid other traffic.

We will make a practice of setting up the missed approach as much as possible during the approach to be followed by a holding pattern at some point prior to the next approach.

# How Sweet It Is When...

All your avionics work.

A weather report is accurate

A vector is a short cut

Ceiling is above your personal minima

Tops well below you.

One power setting all the way in.

All the needles where you want them.

A clearance you can copy.

A warm feeling on landing.

# IFR % Accident Frequency as Compared to VFR

12% related to low-level maneuvering.

12% Fuel exhaustion (Often caused by headwinds and multiple approaches)

50% Trying VFR in IFR conditions

3%  Low level flight into terrain(Circling approaches)

3%  Midair while VFR

14% Failure to select fuel or put on carburetor heat.

50% Improper approach procedure

20% Stall on approach

50% Into Terrain on approach/icing

33% Of IFR loss of control occurred in IFR conditions.  Most of these happenedduring departure into overcast or on approach.

50% IFR control loss on departure

50% IFR control loss in weather

16% IFR control loss in turbulence

0% Lost or disoriented

25% of IFR accidents were related to mechanical problems of aircraft systems.  Knowing how to shut off autopilots for example.  Engine failure implicated in 11% of all IFR accidents.

50% IFR control loss due to vacuum pump failure

25% Into terrain at night

50% IFR control loss due to static system failure