# Click on page number for instant scrolling.

# IFR flyingã

Attitude Indicator	2

Attitude Indicator Errors

Attitude Indicator

AI Demonstration

Turn Coordinator

Needle and Ball

Heading indicator

Compass

Visibility

Determining Visibility

Flight Visibility:

Ground Visibility:

Low Visibility IFR

Emergencies

Electric System Failure

Vacuum Failure

Pitot-Static Failure

System Cross-Check for Failure

Test of Judgment

Partial panel

Disorientation in turbulence

Unusual Attitudes

No-Go Decisions

Flight Plan

Flying in the System

FAA Way

Not the FAA Way

Real World IFR

IFR CCR to SCK

# Attitude Indicator

The attitude indicator gives instantaneous indication of pitch and bank.  It is the only instrument on the panel that provides a clear picture of the

flight attitude of the aircraft.  Most modern AIs permit full 360-degree rotation about the roll axis with pitch stops at 60 degrees.  Marked to show + 50 degrees nose up/down and 20 degrees when inverted. but no specific degrees by markings. Do not have caging knobs.

The attitude indicator (called the artificial horizon in former years) has a vertical gyro as its spin axis.  They do precess but it has an erection system activated by gravity that resets it back to the vertical.  The AI has bank markings up to 90 degree banks.  The first thirty  degrees is divided into 10 degree units.  Very close to the standard rate turns can be achieved by reference to the AI.  Every airspeed has a degree of bank (coordinated) for a standard rate level turn; this is about 15% of airspeed;  15 degrees at 100 kts, 12 degrees at 90 knots.  Use turn coordinator with ball centered to confirm angle required.

# Attitude Indicator Errors

The erecting mechanism operates continuously but is limited to 3 degrees per minute to avoid errors due to sensed 'gravity' during banks.  During prolonged banks in one direction this error can be significant.

Very shallow bank turns or flying out of rudder trim for long periods can produce errors.  Any maneuver that displaces straight and level will result in AI error if it lasts long enough.  Coordinated and uncoordinated turns will do this.  It is for this reason that holding patterns have the one-minute level flight legs after each minute of turn.  This leg allows the AI to settle itself.  This may be the reason the FAA prefers the 45/180 for the procedure turn.

When making a 180 degree steep turn and then rolling out your attitude indicator will show a nose up and wing down in a direction opposite the turn.  The error is inherent to the erecting mechanism.  The errors tend to cancel in 360 degrees.  Taxiing turns are always skidding turns.  It is normal for the AI to show up to five degrees of tilt during taxi turns.  Any more requires investigation.  AI gyrations during initial start of the engine is normal.  If the bar fails to stay horizontal or tips over 5 degrees during taxi, it must be deemed unreliable.

A vacuum powered attitude indicator gives warning of failure.  It will be slow to erect and may go through gyrations while erecting.  It may be sluggish in flight.  To see what an instrument does when it loses power--when the  vacuum pump fails, for example--watch it run down after shutdown. There is no way to predict what an instrument with an internal problem will do.  Don't chance flying IFR with an instrument that has failed and then recovered.  Don't fly with one that is even suspected of being faulty.

Being on partial panel (real failure) in IMC requires landing at the nearest suitable airport.  Report failure to ATC.

Airports with ASR approaches can give no-gyro approaches.  On final, turns should be at half-standard rate. If controller offers no-gyro approach take it.

The AI and TC operate on vacuum and electricity respectively.  If the information is conflicting (you know the standard rate reading of the AI for the different airspeeds) suspect one system has failed.  Go immediately to the Compass as a friend you can trust.

# Attitude Indicator

A Navy study found that during major attitude changes 85% of experienced IFR pilots focused on the AI.	 When the AI is set for level flight its movement can be set to position the nose for any selected climb speed.  There will be a consistent correlation between power, trim, nose attitude and AI attitude.  Knowing this removes the aircraft as a problem.

Instrument interpretation  means to look at the instrument and make an appropriate correction for the indication.  Instruments in a given flight condition are selected for pitch, bank and power.  You set the aircraft attitude and power to get the trend you want.  You fine tune using the instruments with numbers.  Begin any maneuver using the attitude indicator (AI).  Set the AI, check the trend of climb, descent, level, and turn.  Now go to the numbers for making the selected flight condition precise.

Ability to scan from the Set AI to check the trend to the numbers back to the set requires prior knowledge of where you are going to move your eyes. In level flight, with a predetermined power and airspeed, the scan  can be a relative easy and slow AI, HI, AI, Alt, AI.  The scan must be changed and accelerated if a change of heading is required and even more speed if a descending turn is called for.  The sequence of required instrument scan is not so important as keeping the eyes moving always back to the AI

Use the AI as the central instrument.  It gives direct indication of pitch and bank information.  It is the best single source of aircraft attitude.  Flying the AI makes you safe.  Make your turns with AI, check TC for accuracy, the VSI for perfection and use the HI (numbers) to measure results.  Scan should include the AI in every second or third fixation. (You can't see when your eye is moving.)

The AI gives pitch attitude, bank attitude and bank angle

- Pitch

AI-

Airspeed and VSI (VSI shows vertical trend after a few seconds)

Altimeter (FAA primary-with the numbers)

- Bank

AI

TC gives roll into turn rate, turn rate and coordination

HI (FAA primary-with the numbers)

- Power

RPM

Airspeed indicator-with the numbers

# AI Demonstration

- Cover everything but AI (Simulates instrument failure of aircraft with (HSI) and intercept an airway or VOR radial in level flight.  Fly radial using only AI.

- Cover everything but AI and establish best rate pitch attitude going into successive left and right banks and then to one bar descent with left and right banks.  Success depends upon power settings and changes.

- Cover everything but AI and make timed turn to heading using knowledge of airspeed/angle of bank as function of turn rate.  At the end of each timed turn check altimeter.  Practice until you can make timed turns without changing altitude using only AI and your ability to fly aircraft.

- Cover everything but AI and make timed climbs and descents using

a set sequence of attitude, power and trim.  Before recovery make a                          Check (peek) of VSI and altimeter to see how your settings are performing.

# Turn Coordinator

The turn coordinator is usually an electrically driven gyroscope.  It is mounted at an angle of 30 degrees with the back slanted downward.  It is dampened to reduce the reaction to turbulence.  It originally served to control single axis autopilot's. It senses both yaw mostly and roll rate slightly.  Initially it senses roll and when the bank is established it senses yaw (rudder) input.  Knowing the airspeed,  the angle of bank can be inferred.  The TC ball (slip-skid) indicator is smaller than that on the older needle and ball instrument.    The TC will indicate the direction of a spin so you will know which rudder application is opposite.  This is not true if the spin is inverted.  In an inverted spin the turn coordinator will give incorrect information for recovery.  A failed TC will "park" level.  A very good reason not to use as a level flight indicator except in emergency.

Overhaul calibration of both TC and Needle is done by adjusting centering springs.  As the springs weaken with age sensitivity to yaw increases so does gyro friction increase with age with a decrease in sensitivity.  The net effect of these changes are unpredictable.  Ground check of T.C. operation is that in a left turn the ball moves to the right and aircraft remains level.

# Needle and Ball

The needle's gyro axis is mounted horizontally and spins up and away from the pilot.  It tilts on the roll axis up to 45 degrees.  It cannot move on the vertical axis.  A yaw of the aircraft causes the gyro to yaw in the opposite direction.  Reversing linkage gives correct direction and amount.  Oscillations are prevented by a dashpot mechanism.

# Heading indicator

Use 45 degree markers on heading indicator to fly 45 degree intercepts to airways and 45 degree holding pattern entries.  Common procedure is to only set HI in level unaccelerated flight.  Setting the HI should be part of every instrument approach checklist but especially the NDB approaches.

# Compass

The compass is the least likely to fail and any change in its numbers would indicate the opposite direction of turn.  If ever the turn coordinator disagrees with the attitude indicator use the compass to break the tie.  Heading indicator as a vacuum partner with the attitude indicator is a biased juror.

This material is included in the VFR material but is repeated because of the partial panel requirements of the IFR PTS.  Mounted on the face of an aircraft compass is a chart.  This chart is a record of compass error called deviation.  As a pilot of a particular aircraft you should copy this chart for use in navigational planning.

Many airports have an area set aside for a compass rose.  This rose is aligned using the variation between the True North and Magnetic North for this specific area.  It shows the magnetic directions such as are used to align the airport runways.  It is a place where aircraft are positioned and 'swung' through the various magnetic directions.  This 'swinging of the compass' is done with all electric circuits functioning so that the operation of the compass will reflect this fact.

As the aircraft is positioned on the eight magnetic courses a small pair of adjustable magnets are moved so as to get the most accurate compass reading possible.  As these adjustments are made, a record is made of the compass direction of the aircraft on the rose as compared with the best adjusted reading of the aircraft compass.  This record is transcribed on to the deviation chart affixed to the compass.

In most cases this deviation is only one or two degrees.  This exceeds the straight line flying ability of a pilot.  However, with the advent of the Global Position System, it is becoming important to note deviation as a factor in navigation beyond the Practical Test Standards or the FAA written.

The numbers on the compass are opposite in order and direction from the HI.  The reversal of the two numbering systems requires us to be consciously aware of the difference.  Setting the compass to the HI requires that we note the compass lubber line is between two numbers either centered or near one than the other.  These two numbers are then located on the HI and the HI set to correspond.  A usual student error is to make a mistake setting the compass.

Flying with the compass is quite different from using the directional gyro.  The compass has several inherent errors relating to turn, speed and geographic latitude.  The compass should only be "read" in level unaccelerated flight.  It is best (easiest) to make compass turns using the turn coordinator and time.  At 3 degrees a second a turn of two minutes is 360 degrees, one minute 180 degrees, 30 seconds 90 degrees and 10 seconds 30 degrees.  A normal count of 1, 2, 3, (4) will be close to 10 degrees.  On the ground a compass should only be checked while taxiing straight or when stopped at a known heading.

The swinging and dipping of the compass during a turn or acceleration due to changes in speed or direction is a physical phenomenon caused when the north seeking end of the compass dives toward the north magnetic pole.  The higher the latitude the greater the dipping tendency.  Turns from a northerly heading lag behind the turn; turns from a southerly heading lead the turn.  When the card is banked the compass dips to the low side of the turn.  ANDS is the mnemonic for acceleration errors.  In a shallow 360 turn the compass is most accurate at 90 and 270 degrees. In a smooth turn from a south heading the rate shown on the compass exceeds that of the actual turn at a diminishing rate until at 90 or 270 degrees.  See AC 61-27C pg. 44.

In the planning of a flight the use of the mnemonic "True virgins make dull company" gives the order true course, variation, magnetic course, deviation, compass course, wind, wind correction angle, to provide the compass heading required for the flight.  In level unaccelerated flight the compass is unaffected by turning or acceleration errors.  the compass is the self contained and independent means of determining flight direction.

# Visibility

Visibility Defined:

FAR 91.175(c)(2) defines  visibility prescribed for an approach.

FAR Part 1 defines both flight and ground visibility, and there Is 	a difference in the two meanings.

# Determining Visibility

- Decide what you will look for before beginning the approach.

- Study the approach lighting and runway length.

- MALS lighting system is 1400’; MALSR is 2400’

- Learn to count runway lengths.

Factors

Part 91 takeoffs have no minimums but...

Part 91 landings have three prerequsites

1. Normal descent

2. Minimum visibility

3. Runway in sight

You cannot descend below DH or MDA unless you have minimum flight visibility required by the approach plate.  This determination of minimum is done by the pilot.   The IFR training program does not teach

you how to determine visibility.  Reliance on a 45 minute old ATIS is

likely neither valid nor reliable.  The most difficult phase of any approach in actual conditions is where the transition for instruments to

visual actually occurs.

You will be at the decision height about an eighth of a mile before you should see a runway at ILS minimums of a half-mile.  Seeing

the runway before reaching decision height means you have the required visiblity.  Seeing the approach lights but not the runway allows you to

continue but the missed is your best option.  Knowing that each group of

lights are 200’ apart gives you a handy distance reference to the runway.  If you see all of the MALSR you have required visibility.  Seeing all of a MALS system is less than a half-mile of visiblity, go-

around.

# Flight Visibility:

Average forward horizontal distance, from the cockpit of an aircraft in flight, at which prominent unlighted objects may be see and identified by day and prominent lighted objects may be seen and identified at night.  Flight visibility is determined by pilot.  A pilot may not land an aircraft unless the flight visibility is as prescribed in the approach procedure.

# Ground Visibility:

Prevailing horizontal visibility near the earth's surface as reported by an accredited observer.   Reported ground visibility has no reflection on actual flight visibility.  Landing and takeoff visibilities are ground-based measurements.

Visibility above the minimums is a two-way street.  It makes the takeoff safer but does not, necessarily assure a safe landing.  Regardless of conditions, planning and flexibility are the keys.

# Low Visibility IFR

Zero/zero takeoffs are in themselves not particularly hazardous.

Its the ‘what-ifs’ related to engine failure on takeoff or having an immediate need for an alternate landing field that makes such takeoffs

ill advised.  A straight-ahead landing  requires that you know what is

there even though you can’t see.  You must review and know the departure area to gain even some element of survival.

Low visibility takeoffs require that you maintain runway headings and low visibility landings require a minimum visibility.  A Part 91 pilot can depart in zero/zero but must have visibility minimums and the runway environment in sight before desceding below MDA or DH.  What you hear on the ATIS is not controlling.  It’s what you see.

Clouds or rain are most often as stated.  Fog can be variable and

change rapidly.  If fog is a factor be prepared to go to an alternate.

Use a second pilot to call outside the cockpit while the first pilot

stays on the instruments.  In single pilot operations the greatest hazard in such conditions is to let your sink rate increase while you

are looking for an opportunity to dive for the runway.  Don’t!

# Emergencies

ATC facility malfunction

Rare but happens due to lightning strike, phone line cuts,

computer crashes, saturation of system

Aircrew problems

Repeated failure to execute approach, severe weather beyond

crew competence, fuel, situational awareness

Aircraft equipment failure

Maintenance, capability

Don't let a situation become dire before taking actions and

know what to do ahead of time.  Most dangerous inadvertent unusual attitude is the spiral dive with increasing airspeed:

- Reduce power

- Level wings

- Level flight by watching altimeter.

- Greatest hazard is over control.

FAR 91.3 states:  "In an in-flight emergency requiring immediate action, the pilot in command may deviate from any rule of this part to the extent required to meet that emergency."  Two-way com failure is an emergency.

# Electric System Failure

Safe IFR flight is possible following an electrical system

failure with only the pitot-static and vacuum instruments. Electrical systems usually fail slowly if the problem is in the alternator.

Reduce electrical load to save battery.  X-ponder only is good option.

Radios on only for assistance.  No nav radios if being vectored.  Know where nearest VFR lies and head that way.  ATC will provide traffic separation.

# Vacuum Failure

Note: On average only one accident a year occurs solely due to vacuum

pump failure where the pilot detected the failure and flew for nearly an

hour before task overload precipitated the accident.  Keep your partial

panel skills and cover failed instruments.

Early warning of vacuum failure is the need to repeatedly reset the heading indicator.  Vacuum systems tend to fail gradually.  If the autopilot is coupled to the AI, the failure of vacuum pressure will cause autopilot to follow AI as it spins down until the autopilot's limits are exceeded.  The inability of the pilot to detect this failure is believed to be the cause of many 'structural failure' accidents.  The vacuum pressure gauge must be part of the scan.

If the vacuum pump fails, an electric turn coordinator and the pitot-static instruments provide sufficient information for safe instrument flight.  An electric AI backup vacuum system is a worthwhile installation.  See material on C-182 RG and it's vacuum back-up system.

Of the three systems subject to failure, only the vacuum pump is normally flown until it fails.  Fly long enough and you will experience vacuum pump failure.  Given a choice vacuum pumps seem to prefer to fail in IFR conditions. A vacuum failure will cause the AI to indicate a slow turn where none was made or intended.  The HI will begin to spin.

# Pitot-Static Failure

In a thunderstorm pitot-static instruments (airspeed, altimeter, VSI) become unreliable due to radical pressure differences.  The pitot heat should be part of the "actual" IFR checklist.  Use pitot heat at first sign of visible moisture or loss of airspeed, expect high electric drain.  Heavy rain/thunderstorm pressure changes may affect IAS accuracy.   Pitot/static systems can be checked by use of alternate air.  Breaking the VSI glass will cause reverse reading by the needle.  Best sign of blocked static is constant altitude.

# System Cross-Check for Failure

There are three mutually independent instruments that are available in IFR flight to correct bank.  If one of the instruments differs from the other two, believe the two and cover the one. The AI, vacuum powered, is a bank (and pitch) instrument.  The turn coordinator is electric.  The compass is the third independent bank indicator.  The failure of a single system only reduces the redundancy available to the pilot, not the capability.

# Test of Judgment

- Advise ATC, request assistance

- Request vectors to best airport

- Request radar assisted approach

Vectors to final

Shallow intercept outside FAF

Surveillance and course deviation information

# Partial panel

The lack of partial panel practice will deteriorate your scan skills.  Partial panel proficiency is the most perishable of instrument skills.  Your skills usually coincide with your equipment and the frequency of use.  You tend to use well what is there to use.  Being reduced to basic instruments requires the use of skills seldom practiced.  Accidents happen when the abnormal occurs.  The basics of partial panel are heading, altitude, and airspeed.  Don’t expect ATC to fully understand your partial panel problem unless you give a full explanation.

Knowing what power settings provide the control equilibrium for any desired flight condition makes partial panel approach feasible.  You want to get lined up on final as far out as possible.  As an approach indicator the compass is just to erratic especially inside the FAF.  Once you know the course fly with wings level and the ball centered.  Don’t chase the needles, use rudder for instant, quick and small corrections.  Since you are flying headings, keep the wings level.

It would be a rare instance for pitot-static, vacuum and electric system to fail together.  The best defense of a failure of one is to have an operational/available backup system.  The better your cross-check the sooner you will notice a problem. AI cross-check is Altimeter, VSI and airspeed.  Heading cross-check is the TC and compass.  airspeed check is altimeter, AI VSI and power.  Altitude check is with VSI, AI and airspeed.  Practice partial panel often.

Partial panel practice exercises.  Hold altitude and airspeed as constants.  With power as a variable, control airspeed with power and altitude with pitch.  Practice 180 and 360 compass turns both left and right.  Vary airspeed while maintaining level flight.  Make timed turns to headings.  Practice rate-climbs and descents.  Practice stalls and unusual attitude recoveries.  Allowable error margins of 100', 10 knots and 10 degrees.

Failed vacuum eliminates the attitude indicator, most heading indicators and the HSI (horizontal situation indicator).  A blocked pitot tube kills the airspeed indicator.  A blocked static hole invalidates readings of the altimeter, airspeed, and vertical speed indicators.  Electrical failure of the turn coordinator has a backup in the attitude indicator and knowing the angle of bank for a given airspeed.

Cover inoperative instruments at once.  Keep control of the aircraft.  Advise ATC of problem and intentions.  Request No Gyro assistance and possible vector to VFR.  Good practice is, when in VFR conditions, to cover heading and attitude indicators while on approach.  The logic of this is that the more we simulate emergencies and train for the possibility, the better our chances of survival.  Gyro failure is an inherent risk of IFR flight.

Many pilots find partial panel easier.  This is only possible if flying skill reduces the pilot's dependence on scan.  Some disorientation will occur regardless of experience.  The time period for this to occur will vary among pilots.  At some point even the most experienced and best of us will experience some confusion.  In a seven year period of the 80s 330 accidents occurred due to spatial disorientation.  28% of the pilots involved had instrument ratings.  Currency alone is not sufficient, you must be proficient as a partial panel pilot as well.  You start losing  instrument competence as soon as you get out of the plane.  Three flights a month are required to maintain minimum competence.

The best currency is actual IFR.  Instrument cross check skill includes your ability to lock the arm/elbow while looking at charts.  three seconds out of the scan is maximum.

If you have trimmed your aircraft for the mode of flight don't change trim.  Power should first be set and then used to adjust altitude in small increments.  Fly hands off but hold rudder as required.  Don't hurry.  If the TC is level and the ball centered the aircraft is not turning.  Needle, ball and airspeed has flown IFR in the past and can be flown today.

VFR partial panel can be practiced by flying an ILS or localizer.  With a legal safety pilot aboard, cover the heading indicator.  Watch the CDI and make 1/2 standard rate turns with rudder only toward the needle when it moves.  Repeat the process as requires.  As you react to the needle with will center and rudder corrections will decrease until only slight pressures are required.  It can be done down to MDA or DH and will get better with practice.

Daytime partial panel is not a realistic simulation.  Try practicing partial panel at night or in actual conditions but never as a single pilot operation.   A systems failure in actual conditions is an emergency. A partial-panel approach in IMC is a full-up emergency.  Call it.  Cover all failed instruments.  Advise ATC of the problem and make the declaration.  Your flying skills must have habit patterns so well developed that your flight procedures will work in the worst circumstance.

The pilot should know the systems that operate the instruments as well as how the instruments operate.  Knowing this, the pilot should include in his IFR instrument scan a check of the vacuum and electrical operation.   the functioning of one instrument should be checked against another.  The AI vs the turn coordinator etc.  ATC offers options such as vectors, no-gyro approaches and advice as to options.  Cover all dead instruments since they become a visual attraction that will destroy any partial panel scan.

## Partial Panel Turns

Use 18 degrees or less of bank to make a standard rate turn.  The maximum turning error is the latitude of the aircraft.  At 36 degrees latitude you will have a maximum of 36 degrees error when turning to north or south.  This has been called the OSUN method since you overshoot the compass reading for turns to the south and undershoot for

turns to the north.  Little correction is required if turning east or west since the error is zero.  Turns to intermediate headings by 30 degree increments will be 12 or 24 degrees.  Lead roll-out by 1/2 angle of bank for northern turns and delay roll-out by 1/2 angle of bank for turns to the south.

Partial panel turns can be aided by using the ADF card.  Set card to heading.  Which way to turn is quickly determined as well as timing

since each quadrant is three ten second divisions or thirty seconds.

Timed partial panel compass turns are easier to make.  Determine the number of degrees you plan to turn and divide by 3 to get the number of seconds you will turn.  A sweep second hand makes this easier to determine.  3 degree heading corrections can be made by rolling in and immediately out of a 1/2 standard rate turn.  Personally, I use the rudder for such a correction.  Use a standard rate turn for a 6 degree change.  I find that if I hold a standard rate turn for a count of ten I can get a ten degree change.

# Disorientation in turbulence

Turbulence will cause spatial disorientation   In the event of turbulence the pilot should immediatly reduce the speed to Va as

determined by weight.  The lighter you are the slower you go.

In turbulence altitude is a variable.  Let ATC know of the problem

and they will assign you a range (block) altitude instead of a

hard altitude.

Your concentration in turbulence must be on aircraft control.  Believe

your instruments, listen to what is happening.  If an instrument is not

performing as it should, cover it.  ATC can be a help or a hinderance by

the way they communicate.  If their communication becomes a problem as in an amended clearance have them do it one step at a time as you

perform.  Don’t let communications interfere with your control of the aircraft.

The end all of turbulence is...keep the ball centered.  You won’t spin.

Wings level can be flow using the ADF and keeping it on the nose or tail.  Another way is to fly 180 on the magnetic compass.  Proficiency is the only absolute solution.

# Unusual Attitudes

Any flight condition you don't expect is an unusual attitude.  Just as mistakes compound so do unusual attitudes.  You must recognize and correct any unexpected flight condition before it becomes worse.  Sound is a good way to detect the unexpected.  You should memorize the sounds of your aircraft performance so that you can recognize the sound of a nose high or low condition.  Bank is more difficult to detect by sound but most unusual attitudes are a combination of pitch and bank.  Get the pitch under control if it is high, first.  Get the bank under control if the pitch attitude is low.  Get the wings level first.  Without the proper mental set, pilots tend to pull back on the yoke, which merely serves to tighten the spiral descent.  Aerobatic training is the best way to know what to do in the event of an inverted condition or unintentional roll.  If beset by a vortex induced roll, continue the roll.

Instinctive reactions must be aggressively overcome.   When inverted the elevator control must be pushed firmly forward.  Roll input from the ailerons is usually inadvertent due to distraction or body movements.  A spiral descent will soon follow as below.  Releasing the yoke when distracted seems like a valid alternative.

## Graveyard spiral recovery

In the event of a failure that leads to a bank and the sound of increasing airspeed, first reduce power.  Aircraft structural limits can be exceeded the insidious forces of the graveyard spiral entered due to

instrument failure.  Pulling back on the yoke will only continue the descent.  Do not apply back elevator until the wings are level.  Use any indicator you can to get wings level.  Request no-gryo assistance if you are in radar contact.

## IFR accidents

An IFR rated pilot is more likely to have an IMC accident than a VFR pilot.  VFR into IFR accidents have an 85% fatality rate. They usually result from flight into rising terrain.  Not having a sectional showing terrain is a common part of accident evidence.  Complete competency can be overcome by bad judgment.

Causes of IFR accidents include, pilot out of currency, over-confidence in equipment, effort to avoid ATC system and a pilot who is deficient in system malfunction training.  An Unknown number of IFR accidents seem to be related to chart reading!  Most pilots catch their interpretation errors before they become problems or accidents.  Errors tend to be related to confusing typographical depiction, clutter, unclear notes, omissions, errors and differences between NOS and Jeppesen.  Some approaches are not even published by NOS or Jeppesen but may be given to the pilot by approach as though they were. The ILS approach is five to seven times less likely to result in an accident as is a non-precision approach.

Unless you have a routine pattern for flying IFR you will have problems

in dealing with an emergency.  Your routine is the basis for the way you

perform all IFR tasks.  Any proficiency training is devoted to improving

and fixing in place this routine.

Part of this routine involves the way you prepare the radios, where you

keep your alternate information, and the priority you give to flying the airplane and maintaining your scan.  To what extent will a problem distract you from your routine.  It will, but how much and how long?   Your salvation lies in having consistent habits.  Consistency in configuring the plane allows you to devote extra effort to the outside cockpit problems.  This reduces anxiety and stress even though what is happening is not totally predictable.  The highest level of learning is your ability to learn from the routines of others.

# No-Go Decisions

The decision is based not only on equipment but on pilot competence.  Always have an alternate plan when faced with icing, thunderstorms, low ceilings and no alternate.  Just being current is not sufficient, you must be competent.

If there is no alternate within reasonable distagnce, cancel.  A zero-zero departure is quite different when simulated from the actual.  In an actual zero-zero you have no chance of returning.  Ego-based decisions are most often accident prone.  always fly with a tad of humility.  If you lack humility then IFR flight will eventually give you some.  You must be confident of your ability but not so much as to do something stupid.   Time in type under IFR is essential.

Accident result from pilot decision-making.  Saying “no” is just as difficult as making the flight.  Make the decision the safe one.

# Flight Plan

IFR and VFR flight plans are done on Center computers as part of flight data processing (FDP)  The computer knows every route, published and unpublished.  It knows every navaid, most intersections and all airports.  It knows preferential routes and knows that forecast or real time weather may change arrival routes.  Center computers do not overlap in coverage or information with other Centers.  Flight requests not honored in one must be repeated in the next.  Centers and TRACONs now have a computer graphic that can show every aircraft in the U. S. that is on a flight plan as to its flight plan information and present position.

You can specify routing by remarks such as "no over water", "No altitudes over 12,000'" etc.  The ETE destination point on an IFR flight is the point of intended landing.  FAR 91.169 which refers you to FAR 91.153 (a)(6)

FAR sets rules and items that pilots should do to fly safely.  To operate IFR you must file a flight plan and obtain a clearance before entering controlled airspace.  FAR 91.173  If you are on a VFR/IFR or an IFR/VFR flight plan you MUST open and close the VFR parts of any plan through a Flight Service Station.  This appears to be part of an FAA make work program.

Flying according to airline standards, even though Part 91 capable, is much safer.

Box # 3 Without approach capable GPS file as /R.  Approach capable GPS files as /G

Box # 5 Know that the navaid or intersection is in the ATC airspace to be contacted if other than an airport.

Box # 6 You can extend the two-hour life of a pre-filed plan by filing a time 30 minutes later than you actually plan to file.

Box # 7 File your final altitude but check the ATC preferred altitudes to avoid the ATC preferred routing that may screw up your flight planned route.

Box # 8 Using preferred routes will avoid having to copy a clearance when airborne.

Box # 9 By always filing an alternate you avoid forgetting that one is

required where filing to an airport without an approach.

Box # 11 Mention GPS, instructor, or multiple approaches.

Box # 17 Provide both departure and destination phone numbers where someone will be available.

# Flying in the System

ATC operates on the basis that information is exchanged with the pilot.  ATC issues a clearance that the pilot reads back and flies in compliance.  ATC radar monitors the flight, issues advisories and confirms compliance with route and altitudes.  ATC warns the pilot of deviations and provides corrective vectors and information.

With the expansion of radar there has been a corresponding reduction in the acquisition and maintenance of independent orientation by pilots.  I mean that the pilot should be utilizing all available facilities and equipment to keep his own record of where he is, where he is going and where his escape outlet lies.  Only if you know where you are, can you know when ATC has forgotten you, failed to give a vector, descent, or clearance.  When you know where you are you are better able to protect yourself.  ATC can and does make mistakes.

Complacency is one factor that can give unexpected and undesirable  IFR adventures.  Complacency is responsible for many more accidents than is realized.  Complacency seems to affect most insidiously those with the most experience.  Complacency is often caused by agnosia, you don't know what you don't know.  By maturity you should have learned not to trust the anybody or anything with your well-being, property or airplane. Keep a backdoor open.

As an IFR pilot you must anticipate a potential problem before it arrives.  A circle to land may be indicative of a tailwind on approach (CCR)that requires correcting for a tailwind effect ground speed. If the tailwind is strong it may cause your ground speed to rise to Category B.  The radius of turn then goes from 1.3 to 1.5 nautical miles.  Will left or right turns in a hold make for an easier departure outbound (APC)?  Will one departure be better for you (not ATC) than another.  Taking positive assertive action over the radio to ease your situation is an attitude called aggression.  When it affects the safety of your flight it is aggressive safety.  Your flight safety and convenience does not just happen through the good graces of ATC; you must make your own flight safety.  A pilot can become disoriented while maneuvering, looking for the runway, scanning the instruments, and talking to ATC.  Protect yourself by knowing the rules, limitations of systems and equipment, and your own limits.

The pilot of a plane has command authority for which he is often inadequately trained.  The pilot who does not recognize an inappropriate vector, clearance or altitude is not taking charge.  Go not quietly into that thunderstorm or icing altitude.  Continued flight in known icing conditions is prohibited in the FARs.  As a pilot you have the command authority to decline, request, suggest or to act for the safety of your aircraft.

In recent years we have become too reliant on ATC.  As a part of every instrument procedure we should include several fixes of our own to supplement those on the approach charts.  Our fixes should provide us with situational awareness while under radar vectors. Knowing where you are will help you anticipate radar handoffs, heading changes, when to anticipate airspeed, and altitude changes.  Flying IFR means understanding where the airplane was before, where it is now, and where it is going.  Keep your mind ahead of the airplane; know what comes next.

Don’t ever feel that your acquired skills and experience will allow you to overcome the safety net built into the FARs.  Breaking the FARs is not an IFR option.  The fundamentals of IFR rules make breaking them offer benefits much outweighed by the risks.  You have options.

There are things you should do and things you should not do.  Some of the things you should not do change into things you must not do.  The

always available option in IFR flight is to go somewhere else.

The instrument pilot who relys on ATC to give assistance on the approach is apt to be unasqare that radar is relatively inaccurate at extended ranges.  Even vectors are only approximations.  If you are in a situation where you MUST rely on radar you would do better to fly the whole instrument procedure.

The pilot who does not fly instruments regularly is going to regress in basic skills, be liable to the effects of distraction, and be behind the aircraft.  the more experience and the more recent it is the more able will the pilot be to deal with the basics of the approach and the unexpected things that always occur.

# FAA Way

The IFR flight using control/performance instead of the FAA method of primary and support is growing in use simply because the FAA method has the value of each instrument changes for every maneuver.

Flying the FAA way:(Primary has numbers)

For level cruise primary pitch is altimeter support is VSI

For bank primary bank is heading indicator support is turn 				coordinator.

For power primary is airspeed support is engine instruments

For constant ias climb primary for pitch is airspeed

For normal climb primary for pitch is VSI

You will note in the FAA way the attitude indicator is a minor factor.

# Not the FAA Way

- Using control/performance we have three categories of instruments.

- control, performance, and navigation.

- Attitude indicator is main reference for pitch and bank

- Power instruments reference power

- Performance is monitored by altimeter, airspeed, VSI, heading indicator 	and turn coordinator.

- Navigation instruments tell where you are.

First establish an attitude, set power for desired performance, hold attitude while acquiring desired performance, trim off pressure.  Check

performance and adjust attitude, power, and trim as required.

The better you know your attitudes, know your power settings, and know your trim settings, the more precise will be your flying.  You must train your ability to anticipate what must be done to acquire the desired performance.  Flying must not be allowed to become a part of the IFR problem.  Knowing ahead of time what to do and set minimizes the cross-check requirements in constant performance situations and reduces cross-check requirements when performance is changing.  Knowing in advance what to do when simplifies the entire process.  Flying is done by anticipation not be reaction.

Vectors

Departure vectors come from the tower either by reference to a SID (Standard Instrument Departure) a specific heading or a turn to a

heading.  On the handoff to departure these initial vectors may be

supplemented for transition to the enroute structure.

Arrival vectors begin by an initial vector which is an ATC guess as to what conditions will require.  Following the initial vectors will be vectors to intercept the final approach course, for spacing/sequence,

for base leg, delay, penalty, radom, forgotten and bad.  The fewer the vectors the higher skill displayed by the controller.  Sequencing and spacing are standard procedures.  As a pilot you can request any one

of the various vector types if you feel ‘pressed’ by the situation.

“Radar contact” means that you and ATC are using radar procedures.

ATC will advise you if they take you across the final approach course.

This is sometinmes done for spacing but more often because they forgot

to vector you in time for the intercept.  Being vectored doew not mean that you are being protected from all traffic.  Vectors provide only guidance and do not guarantee radio coverage, terrain cleanance, or a clear missed approach.

The approach gate is an imaginary point used as a vasis for vectoring to the final apprach course.  Ceilings determine how far out the gate the

intercept vector should occur.  the intercept may be at the gate if the

ceiling is 500’ above the minimum vectoring altiude with VFR visibility.

Center can give vectors just as do terminal but the different type of radar antenna turn slower by I/3.  Center scope PPI (Plan Position Indicator) are mosaic pictures from several radar sites.  Center radar has 150 mile range so a 10 mile approach procedure will be only an inch long on the scope.  The scope symbols showing the approach are always the same size regardless of the scope scale (range).  Any center vector must have an assigned altitude that is 1000’ AGL or 2000’ AGL when in mountainous terrain.  On occasion, this altitude requirement can be evaded by vectoring a localizer/glideslope intercept at a higher altitude than the FAF by increasing the distance.

# Real World IFR

Try to share any IFR flight you make take with other IFR pilots.  Single pilot IFR is not the best way to go.  Until you develop the hours and experience where you can take a younger pilot, try to fly with the most experienced pilot available.  Expose yourself and the other pilot to real turbulence, overwhelming fatigue, poor vectors to final, tight vectors,

instrument failure and rain so hard it activates the stall warner.  Cancel IFR to avoid a vector into a CB and execute a missed and depart for an alternate.  Always have a good alternate.  Redundant pilots are just as valuable as redundant instruments and power.

# IFR CCR to SCK

Depart CCR and at 600' go direct to VOR

Scan priority

Heading 	approximate 360 with centered needle

Altitude	Climb to altitude assigned and note time of a delayed

altitude assignment enroute.

Looking for VOR passage

At VOR passage use the 5 T's

Turn	to intercept heading of next route

Time	time of passage and time of route segment

Twist	to new course radial

Throttle	no change until level-off

Talk	report VOR if requested

Scan priority

Heading	tracking radial of course with needle centered

Altitude	as assigned

Looking for intersection, next intersection

At each VOR, mileage break, or intersection, the 5 T's will 		aid the memory in correct selection of new heading, VOR, 			etc.  #2 VOR is always used just prior to the approach so as 		to allow setting of #1 NAV to approach navaid.  As the 			approach begins, the Looking for prompt will aid 				recognition of station passages and expected reaction.

Looking for VOR, ADF, glide slope, localizer, time to MAP,

decision altitude, MDA, etc