# Click on page number for instant scrolling.

IFR Systemsã

ATC at work

Major ATC problems occurrences:

Charts

Plate Calls

Minimum Safe Altitude

Inoperative Tables

Transition Routes (Feeders)

Preferred Routes

Alternates

NOS Charts

NOS Layout

NOS Inoperative Tables

Open Triangle

Black Triangle T

Black triangle A

NOS Enroute

ILS Components

Part 1 the Localizer

Part 2 the Glideslope

Part 3 Outer marker

Part 4 the Middle marker (amber)

Part 5 Approach lighting system (ALS)

Compass Locator

Markers:

Radio Marker

Fan Marker

Bone Marker

Z Marker

Final approach

Block Altitudes

RADAR ATC sectors

Separation Standards

IFR separation from IFR

IFR separation from VFR

VFR separation from VFR

Visual separation

Diverging Course

Radar surface movement

When ATC Screws-up

Radar failure

IFR Without Radar

ARTCC Weather Radar

Radar Service Terminated

LORAN

Global Positioning System (GPS)

Uncontrolled Airspace

ATC Weather

Runways

Runway Departure Safety Zones

Runway Visual Range

Lighting

VASI

PAPI

AWOS Weather

Terminology:

IFR Terminology vs Meaning

# ATC at work

The VFR conditions responsibility to see and avoid rests with you, the

pilot.  ATC’s responsibility, authority and accountability for separation exists only under IFR conditions for IFR vs IFR flights.

A controller who ‘violates’ minimum separation standards is under zero- tolerance standards of performance.  All other ATC services are subordinate to IFR vs IFR separation.  Because of this responsibility and accountability the controller has the authority to assign IFR aircraft altritudes, headings, routes, and clearances.

Under the same VFR circumstances the controller can ignore VFR flights but he cannot order or command.  For VFR aircraft the controler is limited to advisories and suggestions.  As a VFR PIC you can reject

any ATC advisories or suggestions.

A computer detected non-collision bust of 500' or less will be addressed by administrative action. (Warning letter)  A loss of separation reported by the computer can require recertification of the controller. Reading back ATC clearances as you have heard them is a good practice in the event of ATC's failure to correct the readback.  If you are directed to call a particular number by ATC reply "Roger" but you do not "have" to make the call.  Instead fill out the NASA form to protect your behind.

If an approach has "radar required" as a note, the approach cannot be flown except under a radar watch.  On any approach it is good procedure for you to request a "call" for any fix even though you may be able to determine the fix with your equipment.  Some radar fixes do not appear on the charts.  You can request that ATC call the FAF for you if you wish.

On an approach the radar vector approach gate is normally a mile outside the marker or the FAF.  Radar normally tries to vector you well outside the gate but can on request give you a close in or far out vector. Don't let ATC vector you in so close to the marker that you don't have time to stabilize the approach.  Don't accept vectors to the marker.

You are asked to maintain a speed to match that of the following jet.  ATC has three techniques to place you into this situation: the tight vector to the FAF, the high speed final, and the slam dunk.  All of these are contrary to what you would use in training.  I have, on occasion, requested a vector 360 so as not to be pressed by following traffic.  ATC may request that you maintain speed up to a certain point, even the threshold.  If ATC asks for a higher approach speed than you can handle..just say "unable".  You are PIC.  Ask for another option.  If ATC gives you a too steep/fast arrival from above the glide slope, miss the approach and ask for another vector.  It is important that both ATC and you have an understanding as to when you will slow down.

The next problem is being held well above approach altitude.  Years ago at Las Vegas I heard an airliner being so held and then told to descend for the approach.  The pilot complained throughout the descent and finally had to execute a missed.  I later found out over coffee and doughnuts with a neighbor that he had been the Captain involved.  Getting down for the small aircraft involves getting the aircraft as dirty as possible and keeping the engine warm.  You could try to do this after a high speed descent but you risk engine damage.  It is wise to pre-plan your procedure selection before it happens and advise ATC of your intentions.  Don't promise ATC anything you can't deliver.

# Major ATC problems occurrences:

- Entering ARSA/TCA without authorization

- Runway incursions

- Altitude deviations.

# Charts

For essentials, use post-its, and any other aid that comes to mind.  As you develop preparation skills you will be building toward the time when you can meet the demands of reading the plate in the cockpit.  Diagram the airport on the approach plate to orient yourself for a minimums break-out, or circling approaches.  Very important for airports with intersecting runways.

Highlight the plates:

Main NAV frequency

Final approach heading

First part of missed approach

Minimum descent altitude/decision altitude

Time

Missed approach fix

# Plate Calls

The training program should so prepare you that there is no guess work.  Planning and preparation makes it so you are ready when there is more to do and less time to do it.  Configure the aircraft and follow the black lines.  To help you focus on the specific information you need you will prepare two  Post-Its with plate calls.  One is for you and the other for the second pilot.  A more permanent plate might be made for frequently used plates.  A plate call is the numbers of the black lines on the plate.  Some of these lines are in the plan view. others are from the profile view, the last is the initial missed approach figures.

Always list the columns and numbers in the same order for a plate call as left column Com frequency, Nav frequencies/idents, right column airspeed, heading, altitude, DME and time (as required).  Airspeed implies aircraft configuration.  There is no use practicing approaches until you have the power settings, airspeeds, trim, and configuration for the aircraft set so that you can takeoff and climb, cruise, approach, descend and configure with assurance, ease, and smoothness.  Once you can fly the aircraft in a consistent profile you are ready to set up approaches.  One  sequence of plate calls are:

Column  1		 	 Column 2	  	Column 3

1. Approach 		Frequencies		ias, HI, alt

2. Inbound						DME, time

3. Marker x-ing

4. DH-time

5. Missed

The training expectation is that you have studied, prepared and even memorized specific aspects of the charts at your destination.  You need to have  FAF altitude, time, DH/MDAs and initial missed information memorized, post-it posted or highlighted on the chart.  I, personally, try to prepare all three.  Frequencies I do separately.

Jeppeson shows instrument airports by using all capital letters. Jeppesen are indexed by city; not airport name.  Jeppesen charts have range rings are called airport reference circles of 5 statute miles in diameter, centered on the airport (See NOS rings).  Jeppesen chart stepdowns are as they would be flown.  A descent, level flight to the next step, descent, level flight etc.   Jepps put frequencies in upper left corner. Sometimes difficult to tell if frequency goes with facility above or below.

Practice improvement in your chart reading skills by taking a

strange plate.  Looking up mentally ask yourself to  locate some information that should be on the chart.  Give yourself a count of three to get that information before looking up.  Do this repeatedly until all essential information is acquired.  Any chart search that takes over three seconds would indicate that you have probably initiated a heading excursion.

Some pilots never remove their plates from the binder.  They find

an innate advantage exists in having alternate plates instantly available.  Plates currently in use can be held with heavy paper clips.  Each instrument airport has a minimum of two plates - the airport chart and the approach chart.

Jeppesen "notes that kill" contain special instructions for altimeter settings, equipment requirements, circling instructions.  Circling minimums are predicated on performance parameters of aircraft.  Circling in the wrong area is a big NO.  Circling minimums can never be lower than straight-in minimums.

The plan view is drawn to scale.  This view gives transition routes, minimum altitudes, courses and initial missed route.  It includes information required for determining fixes and highest obstacle.  The profile view, below the plan view, gives the vertical cross section needed to determine crossing altitudes at fixes, procedure turn altitude and TDZ altitude of runway.

Jeppeson has added colored contours when terrain exceeds 4000' above airport reference point (ARP).

# Minimum Safe Altitude

The Minimum Safe Altitude (MSA) circle has nothing to do with the approach or the initial approach altitude or the approach segments or the procedure.  The MSA is for emergency purposes only.  Minimum vector altitudes (MVA)are usually below the MSA.  The required obstacle clearance (ROC)of the TERPS procedures for making an instrument approach procedure also set that the ROC of the MSA must be 1000’ higher than any obstacle within the circle.  The MSA circle may be divided into sectors for any approach except for the GPS approaches.  The MSA is essentially a non-functional part of the approach plate.

# Inoperative Tables

The Chart gives the inoperative components tables, whys and wherefores of straight-ins versus circling and the category differences.   When there are inoperative components plan for the missed approach   The middle marker no longer changes minimums (1993)	Your minimums do not change with a failure of equipment unless the minimums box list new DH/MDA or visibility for that failure.

# Transition Routes (Feeders)

Transition routes are made so that an approach can be flown without ATC involvement.   Pilots call them transitions and ATC calls them feeders.  Feeders exist where the initial approach fix is not part of the enroute structure.  Feeders don't exist if the IAF is an intersection or a VOR shown on both charts and plates.  An approved transition route is usually, but not always, depicted as a heavy line with a minimum altitude, course and distance.  In a cluttered plate the routes may be a finer line with the information placed elsewhere on the plate.  In the radar environment these routes are under used.  The IAF is required in a non-radar situation.  Some dead reckoning is allowed.

With a clearance you can fly off-route but become responsible for obstacle clearance.  Transitions have minimum altitudes and distance given on the chart.  A defining radial does not have these.  A transition can be flown by DR but most have positive course guidance.

# Preferred Routes

Computerized IFR routes really care nothing about how you have filed.  Computers have canned standard preferred routes and tower enroute clearances.  Some are published while others are both unpublished and unknown except by the local controllers.  The difference between them is altitude.  Between points filing at one altitude will/may give a completely different direction and route.  The computer wants an end-to-end continuous route line.  If the computer rejects your plan the specialist can specify "full route clearance" that tells clearance delivery to read the approved plan to you in full.  You will not get "cleared as filed" in such an instance.  No all preferred routes are published.  If you don't like a particular clearance tell ATC.  If you properly explain your situation the specialist will try to help.

The best way to face this situation in the Bay Area is to file airways CCR to MYV (Marysville)and see what happens.  I could tell you but I won't.  If you don't like a particular route clearance, wait until you get in radar contact and attempt to negotiate something else.  One trick is to file only to the first fix of a preferred route going your own preferred direction and then negotiate the route. If a change in routing is desired, make your change in small bites rather than multiple legs. If you plan a fuel stop, pick one with other than just fuel facilities.  Different preferred often specify altitudes.  An approach control or Center might know distant preferred routing better than an FSS.  (see Tower enroute)

# Alternates

Back of Lead chart as a "to-file-as alternate" block.  If NA, it means that it cannot be filed as an alternate but it may still be used.

# NOS Charts

Printed every 56 days with revisions on the 28th day as a Change Notice.  Charts are indexed by city not be airport name.  There is no consistency in this in sectionals or en route charts.  Pilots need to search throughout NOS plates for information that is not consistently located.  A/FD or airport guide required.  Changes are best posted on to the chart on reception.  You should never assume that a flip chart has not changed no matter how often you fly the approach.  16 booklets cover the U.S.  FAA exam is bases on NOS charts.

# NOS Layout

Index of airport terminal chart pages

Approach charts

"Notes that can kill you are below the minima box."

Airport diagram charts

Do not give center of airport for LORAN or GPS

Standard Instrument Departure (SID)

IFR takeoff minimums and departure procedures

Textual/graphic display of standard terminal route 				clearances.

IFR alternate minimums

General information and abbreviations

STAR and profile descent charts

Airport Terminal charts

# NOS Inoperative Tables

The Chart gives the inoperative components tables, whys and wherefores of straight-ins versus circling and the category differences.   When there are inoperative components plan for the missed approach   The middle marker no longer changes minimums (1993)

# Open Triangle

NOS charts use an open triangle to show obstacles.  The elevation is shown as MSL and a + indicates the position is only accurate to 600'.  All obstacles do not appear on a chart.   Power lines less than 200' high are not shown.

# Black Triangle T

Nonstandard takeoff minimums apply when the plate displays a black triangle with a T inside beneath the minima section.  You must turn to the takeoff minimums section of the booklet.  If your IFR departure instructions are not given in the clearance and the triangle T is there, you must refer to the takeoff minimums section and check for a special IFR departure procedure.

# Black triangle A

Nonstandard alternate airport plate displays an A inside a triangle beneath the minima section.  You must turn to alternate minima section in the book to find specifics.

Other

- Frequencies in small type may be on left or right.  Frequencies may not be visible if overprinted on water depiction.

- NOS rings are called distance rings.  Usually but not always 10 nautical miles in diameter.  May not be centered on airport.

- If an approach fix requires DME, the approach requires DME and should so state on the approach plate.

- The lightning bolt where the glide slope intersects the arrival altitude set the FAF for the ILS.

- The Maltese cross on the glide slope sets the FAF for non-precision approaches.

- NOS plate has continuous descent line with no level flight depicted for a step-down approach.

- Offset localizer has small dot-circle to warn chart reader.  Offset is shown on airport view in lower right.

- Plates are not to scale beyond 15 miles.

# NOS Enroute

- Enroute charts are reissued every 8 weeks.  Jeppesen lists changes.

- Printed in four colors.

- VOR airways and data are black.

- Civil only instrument airports are green.

- Special, B, and C airspace, ARTCC boundaries, and 				Civil/military instrument airports are blue.

- NDBs, MOAs, MTRs and non-instrument airports are brown.

- Local Airport Advisory frequencies (LAA) are in shadow box with 			FSS name and frequency above.

- VORs latitude and longitude are given in degrees, minutes, and 			seconds.

- Minimum Crossing altitudes are noted by X-flag with restriction.

- MOCAs are preceded with an asterisk.

- ORTOCAs by NOS or MORAs by Jeppesen or off-route obstruction clearance altitudes are now being included on enroute charts.  This gives terrain clearance related to the chart one degree rectangles.  The numerals, very much the same as    those on the sectional, give 1000’ clearance over level terrain and 2000’ in the mountains.  the exact location of the obstacle used for this determination is not given.

- MEA GAP indicates that communications/navigation may be lost.  			Such intersections are marked with an R-flag.

- DME distances are inside an oper arrowed D pointing to the 				intersection.

- The size of the compass rose is for clarity and has no

- significance.

- The small bar bell is a military FSS facility

- A circled G means a military radar facility

- A small brown arrow on a chart indicates that a LOM (Localizer outer marker) has an enroute function such as identifying an intersection.

- The flag x symbolizes a minimum crossing altitude beyond which a higher MEA applies.

- The routes between two VORs may often have non-reciprocal

- radials due to magnetic variation differences.

- Some L-class VOR have expanded service volume for the

- purpose of identifying intersections.

- A blue airport has an instrument approach.  A brown airport does not.

- The cross-hatch area represents the 30-mile mode C veil.	Jeppseen does not show this.

- Blue MRAs (minimum reception altitudes) apply to VORs, brown ones to NDBs.

- VOT (test ) frequencies are not given.

- Airways do not exist below published MEA or MOCA

- NOS scale is near WAC scale of 1" = 14 nautical but will vary from 10 to 20 nautical as from New York to Montana.

# ILS Components

Item:

Flying the ILS requires the pilot to process four times the information a surgeon doing a major operation.

The ILS has five parts.

# Part 1 the Localizer

The localizer at departure end of runway provides lateral references.  Frequencies are 108.l by odd tenths to 111.9.  It is identified by "I" (..) as first of four letters.  Fan width varies from 3 to 6 degrees and may not be aligned with the runway.  It is useable within 35 degrees of course.  An LDA’s range of tolerance is identical to that of the ILS’s but is more likely to be in the 6 degree range.It is 4 times more sensitive than VORs.  One degree of localizer fan sensitivity is only 25' at one mile. Backcourse should not be used for navigation unless authorized.  An ADF (simplified directional facility) is fixed at either 6 or 12 degrees, whichever is best at the site.

# Part 2 the Glideslope

The glideslope is abeam 1000' marker of runway and provides threshold crossing height (TCH).  It is not useable if crossing threshold below 50'.  The course width of the glide slope is 1.4 degrees.  Useable course distance is 10 nautical miles.  On the glide slope approach will give a touchdown at 1000' markers.  UHF frequencies at 330 paired with localizer.  Same angle as VASI. The reason for a glideslope interception altitude is because of higher "false slopes".  May have DME but more often not. (see Reno).  No leveling off at decision height (DH).  Reverse sensing on backcourse standard VOR but not on HSI.

The glideslope angle is usually 3 degrees.  You can determine the descent rate of the 3 degree slope by using the charts in Jeppesen or a rule of thumb based on your ground speed.  The thumb rule is good for practice.  Multiply your ground speed by 5 and add 50.  (90 kts x 5 + 50 = 500 (Should be 485)

# Part 3 Outer marker

You cannot fly an ILS if the outer marker is out and no other authorized definition/substitution for the fix exists. AIM 1010 and FAR 91.175(k) The outer marker is five miles from runway.  It is the Blue light. Interception of ILS glide slope may be before, at or after OM. ILS final approach fix often is different from localizer only approach.  Timing of localizer approach is always from OM as the non-precision FAF. Put this altitude into your need to know approach checklist. ILS Outer Marker (OM) is fan-shaped across ILS localizer course about 4 1/2 miles from approach end of runway.

# Part 4 the Middle marker (amber)

The middle marker is 1/2 (3500 feet) mile from runway at decision height which is typically 200' AGL above TDZ.  MM inoperative does not change minimums as of 1993.  Check the marker crossing altitude as an altimeter check.  The middle marker is not a required component for full ILS minimums nor the localizer.  Now called the Inner Marker (IM).  It is fan shaped across ILS localizer course about 1/2 mile from approach end of runway.  On a back-course the marker, if used, is white.

Up until the 1970s there were still LF/MF four course radio ranges in use known as radio range, Adcock range, standard radio range, A/N range and L/MF range.  The range uses a 200-425 kilocycle (Hertz) signal to provide course guidance through the ADF.  These ranges are still in use worldwide.  Older aircraft will have an "Airway Marker Beacon" light along with Outer and Inner marker lights which transmit on 75 megacycles (Hertz) to provide definite position information. These are Marker Beacons.

- Fan Marker:

100 watts power with 3000 cps (cycle per second) tone emitted in a 	fan shape three miles wide and 12 miles across an airway.

- Dumbbell Marker

Same as fan marker but narrow middle is only 1-1/2 miles wide.

- Z Marker (Station Locator Marker)

Located at radio range site and identifies center of range. (cone 	of silence.

# Part 5 Approach lighting system (ALS)

(See AIM for variations)

MALSR and SSALR have runway alignment lights (R means runway)

MALS and SSALS do NOT have runway alightment lights

# Compass Locator

The compass locator while not technically part of an ILS procedure serves to maintain situational awareness especially in regard to being vectored outside or inside the FAF.  Its range may be lower than 15 miles.  Using the compass locator select a heading that will intercept the localizer outside the FAF.  On interception a HI reference heading is selected.  Expect reference headings to change on the approach.

# Markers:

A 1954 edition of the Pilots Radio Handbook revised by the Civil Aeronautics Administration describes the LF/MF (Low frequency/medium frequency) four course radio range as a ground located radio transmitting facility which defines four navigational radio courses which the pilot of an aircraft can locate and fly using a receiver operating in the 200-425 kilocycle band.  This system was known under such names as four-course radio, radio range, Adcock range, standard radio range, A/N range and L/MF range.  The last U.S. continental range was decommissioned in Maine about 1970. The ‘Radio Beacon” is a general term for LF/MF navigational aids for use with a direction finder to provide bearing information.

# Radio Marker

The Radio Marker is a fixed ground transmitter operating on the frequency of 75 megacycles to provide definite position information to an aircraft directly above it.  A high directional signal is transmitted directly above the ground antenna.  The aircraft marker “marks” the passage over the antenna by coded tone bursts and lights.  Although all at the same 75 megacycles frequency the different functions of the beacons are determined by their locations, power output, coded signals, and light color.

# Fan Marker

Fan Marker (FM) at 100 watts emits a 3000 cycle tone  in a elliptical pattern.  At 1000’ it is 4 NM wide and 12 NM long, at 10,000’ it is 12 wide and 35 long.

# Bone Marker

Or Dumb Bell Marker because of its shape gives a more precise check when exactly on course.  At 1000’ it is 3 NM wide in the center and 12 NM wide.

# Z Marker

Z Marker or Station Location Marker is located in the center of a radio range site and gives a positive indication of the cone-of-silence by radiating a 5 watt signal vertically.  Shows white light.  These are still in use in other parts of the world.  ILS Outer Marker (OM) transmits a fan about 4 ½ miles out on the ILS approach path. Shows blue light.  ILS Middle Marker (MM) gives fan about 3500’ from approach end of runway.

# Final approach

On final you should realized that at the outer marker you are five miles out and 1500’ above the runway.  Within the lateral limits of the CDI the slope or steps of the approach will keep us above any obstacles.

As we descend to 200’ above decision height the lateral limits become so constrained that full CDI deflection occurs at the end of the runway

in only 500’.  Properly flow you have guaranteed obstacle clearance.

The new TERPS critera obstacle clearance above obstructions at five miles is 755’ and at DH  the new criteria provide 122’.  Non-precision

approaches provide 250’ obstruction clearance throughout the approach.

# Block Altitudes

There will be times when IFR flight conditions make it difficult to impossible to maintain an altitude.  Actually almost any time you can

request a block altitude.  The block gives you a practice playground between two altitudes that are above the minimum instrument altitude of FAR 91.177 or minimum vectoring altitude.  This way you can

legally fly so as to be actual by picking up clouds a few hundred feet

higher or lower inside your block.  Cloud scooping it is called.

# RADAR ATC sectors

Controllers live by rules in 7110.65.  A radar controller separates aircraft as they move through "his" airspace.  ARTCC or Air Route Traffic Control Center cover all the wide open spaces between Terminal facilities.  Terminals funnel aircraft to and from their airports.  Airspace is transferred from ARTCC to Terminals and thence to airports by Letter of Agreement (LOA)s.  "Letters of Agreement", often unpublished and unknown to pilots, exist between ATC facilities.  These letters allow special flight routes, altitudes and procedures.

"Radar contact" does not guarantee terrain clearance any time you are below MOCA or MEA altitudes.  ATC radar airspace is different than a pilot's airspace.  A tower may by LOA (letter of agreement) lose control of a part of its space under certain weather conditions or because of the needs of an adjacent airport approach.  It helps the pilot to know the ATC preferred direction of traffic and airspace alignment.  Hayward has a LOA with Tracon to release the top 1000' of its Class D airspace at all times.  Alameda has a similar agreement.  CCR and APC release to Travis control of airspace above a fixed altitude during IFR conditions.  This appears to the pilot as part of the SVFR clearance.

Some sectors and frequencies are more "quiet" than others.  On weekends sectors may be combined so the frequency you normally select will be only monitored so as to assign the active frequency.   One sector will feed into another sector by means of a "handoff"  Several such sectors may feed to a final controller.  If too many aircraft are loading up the final sector some of the outlying sectors may be required to "spin" (hold) aircraft to lessen the final controller's load.  The "spin" may consist of a completely different routing.

The handing off of an aircraft from one sector to another requires coordination.  This means asking approval via phone/computer of the adjoining sector's controller if the will accept another aircraft.  Controllers are not allowed to violate adjoining sectors.  Your vector may be simply to avoid an adjacent sector.  Every favor one pilot receives, delays another.  The controllers can't make all the phone calls in time to meet requests in complex environments.  Computerized handoffs are rapidly replacing the phone.

Aircraft on the controllers screen have a data tag that gives call sign, type, groundspeed, altitude readout and may include destination, type of flight and a controller letter of identification.  The new automated handoff has some problems since the airspace sectors is often subdivided several times.  When a controller notes that an aircraft is leaving his airspace he tells the computer.  The computer then decided who gets the aircraft.  The originating controller may not know who gets it.  If the aircraft fails to make the proper radio contact we have a loose cannon in the system.  When the Mode S transponders get used every aircraft will have a permanent code.

A "pointout" is similar to a handoff but allows an aircraft to nip through a corner of a controllers airspace without changing frequency or making radio contact.  ATC can do this without the pilots knowledge.

# Separation Standards

Some ATC calls are courtesy calls, some are ‘point outs’, some are

advisories, and some are mandatory advisories.  The pilot must learn to

listen to the tonal variations of the controller to separate one from the other since there is no other obvious distinction.

ATC contract..."provide separation".  If standards are violated a "deal" occurs which causes economic and training problems for controller.  To protect themselves from the minimums controllers maintain greater than minimum required separation.  Separation can be both horizontal or vertical and need not be both. a ‘snitch patch’ in the computer tells if separation is less than standard.  Makes a ‘deal’ for the controller

Standard separation is determined by aircraft type, altitude, type of ATC facility, stage of flight (departure, cruise, approach) weather, and antenna distance.  If visual separation exists by pilots or by ATC the separation may be much closer than 5 miles. If aircraft are separated by altitude the separation may be as little as 2.5 miles.

A limiting separation factor is the distance of the aircraft from the antenna, class of airspace and size of aircraft (heavy).  Minimum is

miles within 40 nm of antenna; 5 miles beyond 40 nm.  Center (ARTCC) uses 5 nm standard separation  and 1000’ below FL290 regardless.  Terminal areas most likely 3 nm and 1000’ with visual separation allowed.  Vertical rules are same everywhere.  There  are no standards of separation for VFR aircraft.  Only in Classed B and C is IFR separation mandatory.  Class B separation of IFR/VFR is 500’ and 1500’.  Class C separation is 500’ and ‘green between.  In Class C there are no mandatory advisories.  VFR advisories are given as load allows.

IFR aircraft fly in a cylinder 10 nm  (ARTCC) to 6 nm (terminal) diameter.  The cylinder extends 1000' above and below the aircraft,  ATC  normally allows much more space.

Your encoding altimeter must be within 300' because the "snitch patch"  of center's radar is set to alarm if altitudes encroach on 700' separation.  An encoder off by 300' could set off the alarm.  If you should wander more than 300' off IFR assigned altitude and ATC questions, delay your response until altitude is within 300'.  Better yet, advise "unreadable" while making the correction.  "Snitch" is found (1993) only in ARTCC radars.

radar target may not be in ATC contact so the altitude is only ‘indicated’ not confirmed.   Traffic advisories are a part of flight following which includes weather advisories, terrain, obstruction, and low altitude alerts.  asking for flight following and being given flight following leaves out the rest of radar flight services.  You can even request that traffic advisories be omitted from flight following.

An IFR flight with visual contact on VFR aircraft is allowed to maneuver to avoid without regard to an ATC clearance.  The radar controller is required to advise you if your target merges with another on the screen.  The advisory may just to indicate how far above or below you the altimeter reading indicates.  Giving ATC your indication of visual contact relieves them of responsibility.  ‘Mode-C intruders’ are aircraft that have encoders but are not in communication contact with ATC.  Many radars can give you two-three minute warnings if such an aircraft is in conflict with your route.

# IFR separation from IFR

VFR-on-Top and visual climb/descents responsibility:

ARTCC 5 miles/1000'		Relieves ATC only during altitude change.

Class B 3 miles/1000'		Other aircraft need not be visible.

Class C 3 miles/1000'		Use when possible. A VFR on top clearance 					is all right below a cloud deck or between 					layers as long as you can remain VFR and 						meet cloud clearance-criteria.

# IFR separation from VFR

ARTCC  nothing			When IFR pilot agrees to keep visual

Class B 1.5 miles/500'		separation, ATC is relieved of respond-

Class C  conflict/500'		visibility.  Pilot may climb, descend or

TRSA 1.5 miles/500'		get closer than minimums.

# VFR separation from VFR

ARTCC  nothing

Class B  1,5 miles		No requirement but you may get traffic

Class C  nothing			advisories and safety alerts.			TRSA  1.5 miles		Be careful in Class C.

# Visual separation

ARTCC  limited		Must report aircraft in sight to ARTCC and

Class B  allowed		then must report passing clear.  Seldom used.

Class C  allowed		ATC controller uses his visual contact of TRSA     				allowed aircraft involved to expedite movement.  				When a pilot acknowledges visual contact he 					relieves ATC of responsibility for separation.

Note:

There is no time duration limit on this exchange of separation responsibility.  A pilot can hand back this responsibility to ATC by advising that visual contact cannot be maintained or is lost.

# Diverging Course

ARTCC   no

Class B  allowed		This means IFR aircraft at same altitude may

Class C  allowed		be allowed to break minimums if divergent.

TRSA     allowed

# Radar surface movement

Guidance and control system (SMGCS)

Special low visibility, RVR less than 1200', taxi routes with lighting and surface markings.  Green taxiway centerline lights,  2 red lights on each side of the taxiway serve as stopbars, these are ATC controlled.  At no time shall a pilot cross an illuminated red stop bar.  Flashing yellow lights are across taxiway in runup area as taxi-holding position lights.  Position markings are pink circular numbered marking along taxiway.   3 amber lights across center of taxiway are holding bar indicators.

See the AIM for latest on airport markings...a MUST

# When ATC Screws-up

The ATC system requires that the pilot maintain the situational awareness required to know when something is missing from a clearance, a route, frequency change, or a procedure.  Be prepared to question ATC instead of waiting for them to recognize a problem.  Be prepared to protect yourself by knowing what is supposed to happen.

# Radar failure

If approach facilities do not have radar...no traffic can be seen.

When radar fails at a radar facility two backups exist:

DARC is used by Center

Cenrap is used by approach

When ARTS computer fails all aircraft are given the same discrete code, slashes appear instead of numbers.  When a complete radar and computer failure occurs we have a reversion to CENRAP which will use a radar signal from one of centers antennae.  Separation becomes 5 miles instead of 3.  Mode C is unusable so all altitudes must be reported.  No altitude or conflict alerts can be given.  Aircraft are put into lines at different altitudes and processed  to the approach and landing in order.  Pilots should  expect requests for position and altitude.

# IFR Without Radar

An approach is called "single threaded" if only one IFR operation at time is allowed due to lack of radar coverage.

- Plan to execute the full approach procedure in non-radar conditions.

- Know and obey the altitude restrictions.

- You must make the required position reports.

- Leaving an altitude

- Procedure turn inbound

- Final approach fix

# ARTCC Weather Radar

ARTCC radar has three keys WX 1-2 and 3. which will give rain return information on a digitized circuit.  It takes 4000 feet of clouds to make rain.  If its wet where you are, figure to climb at least 4000' to top the clouds.  This information must be requested by the pilot.  Digitized Hs on his screen indicates weather to be avoided.  The controller is obligated to give you the worst possible interpretation of any weather return.

1995 NEXRAD radar coverage is 100% east of North Dakota line to the Gulf.  WSR-88 display precipitation and wind if particulates are present in the air.  Data will go to Center Weather Service Units and Flight Service Stations.  175 antennas will give circular coverage out to 285 miles up to 10,000'.  Most will be away from major airports about 10 miles.   It can sector scan over airport and down main weather runway.  In the future these displays will be in the cockpit.

# Radar Service Terminated

Becoming overly dependent on radar can cause problems.  You should be prepared to operate in a non-radar environment.  This means defining your own fixes, required reporting points, and altitudes.  Without radar to help, your are responsible that your position and altitude will provide terrain clearance.  Your preflight preparation made you aware of terrain and obstacles that intruded into your approach route.  Review the chart to make sure that the correct frequencies are set.  abandon the approach if you feel that something isn't right.

Radar service terminated means that the pilot will become totally

responsible for the altitudes, headings, terrain, and aircraft on his

flight route.  Some radar servides are automatically terminated in IFR

conditions as:

1. Cancellation of IFR flight plan except where basic radar service is always provided as in Airspace Classes, B, C, and TRSAs.

2. When on an instrument approach ATC advises to change to the

CTAF or tower frequency.

3. On completion of a radar approach.

(4). VFR aircraft told to change frequency, or squawk VFR.

# LORAN

If you have non IFR LORAN or GPS they can be used as advisory.

LORAN like GPS can fly you into terrain unless it has the correct data base.  The mutual use of Loran or GPS by two aircraft in the vicinity of a given intersection or navaid  greatly increases the likelihood of a mid-air.  The accuracy of these makes such an accident probable where with the use of VORs it was unlikely but possible.

# Global Positioning System (GPS)

Paper charts will still be required in addition to a local GPS transmitter that may or may not be capable of avoiding any military induced GPS errors.   A faulty navigation signal is flagged within 30 seconds.

For Receiver Autonomous Integrity Monitoring (RAIM) six signals are only required so many areas and approaches will have RAIM "holes'.  GPS overlays will be added to current approaches primarily to improve course guidance.  FAR 91.205(d)(2) says that for IFR navigations equipment appropriate to the gound facilities to be used must be operational and on the aircraft.  The greatest problem to be resolved is the IFR approved GPS which allows DME and ADF substitutions on a published overlay.

GPS can confirm the information on the HI, altimeter, airspeed indicator, VOR, ADF and correct the DME.  GPS can be used to point toward the airport.  (See GPS material under VFR cross-country)  The military accuracy will be available to all as of April 1996.  If flying with a hand-held or panel mounted GPS not certified as IFR, just file ‘GPS equipped’ under remarks

There are 4600 GPS overlay approaches.  Fixes have been converted to GPS waypoints.  There is no vertical information at the waypoints.  All GPS navigation is TO a waypoint.  Some synthetic fixes have been created to allow outbound headings and for increased GPS sensitivity on final approach.  Timing is losing its importance for determining the missed approach point.

There are GPS Phase II and III overlay procedure charts.  On NOS charts there is no way to tell the phase.

Jeppesen provides a list.

Phase II requires that navaids be available but you are not required to use 	them.

Phase III charts have GPS or ... this form of GPS is a stand alone 	procedure.

On airways you can use GPS if you also have standard navaid devices.

# Uncontrolled Airspace

Uncontrolled airspace is just that.  Any current instrument pilot can fly IFR in uncontrolled airspace wherever it's IFR.  Airspace below 700/1200 AGL is uncontrolled.  You don't need an IFR flight plan or clearance to operate there, regardless of weather.  You do need to follow 91.177 and 91.179 regarding minimums, however.   Can't be done?   ATC says the pilot is on his own until reaching controlled airspace.

The ultimate IFR question is whether what you did was the right thing to do.  Any arrival or departure from an uncontrolled airport will involve

IFR flight in Class G airspace.  This means at least some small portion

of the flight does not have the services of ATC for IFR separation.

The purposes of the 1200 and 700 foot transitions areas is meant to

maintain IFR separation services on the approach corridor for as long

and as low as possible.

There are a few Class E airports and a number of Class D airports that have Class E extensions acivated by below VFR minimums.  These Class E

airspaces touch the ground and have ATC IFR separation services.  The

Jepp charts show such airports with a small E but NOS charts show nothing.  The sectional is the best place to look.  Look where an FSS

airport used to exist or where an official weather observer was once available or has been replaced with ASOS/AWOS.  The type of approach has nothing to do with the type of airspace.VFR flights will require a SVFR clearance.

The FAA can cite you for a violation when flying IFR in uncontrolled airspace even though FAR 91.173 says you don't need a flight plan or a clearance.  FAR 91.155 allows you to fly IFR in uncontrolled airspace.  Once you get above 700’ AGL you IMMEDIATELY must comply with the required VFR cloud clearance.  Better to stay low and find some 1200’ transition airspace before looking for an area where you can climb with required cloud clearance.  The FAA will catch you on FAR 91.13 for careless and reckless flight if you don’t have the cloud clearance.

# ATC Weather

Within limits wind determines the runway ATC will select.  When wind is less than 5 knots ATC may select a noise-abatement runway or even a downwind runway.  Visibility determines the right to do an approach, land, or takeoff.  ATC is a source of advice but the action is up to the PIC.

Ceiling requires a decision on an alternate, if the destination weather is forecasts or reported not to give a ceiling of 2000' one hour before or after plus three miles of visibility at ETA.   Precision alternates must have 600 and 2, non-precision 800 and 2.  Ceilings from any source are legal for Part 91.

# Runways

- Precision

- This is the safe bet with DH 200’HAT, lights and 1/2 visibility

- Precision with obstacles inside DH

- DH 250 or higher, 1 mile visibility, may have lights.  Over 250 DH means obstacles inside DH.

- Non-precision

- Could have 1/2 mile visibility, clear approach path on centerline.

- VASI and/or VDP, usually the ILS runway

- Non-precision with obstacles below the MDA

VASI say no obstacles on final from four miles out.  If you are

unfamiliar you may or may not have obstacles.

# Runway Departure Safety Zones

An ATC clearance does not assure obstacle avoidance during departure. Obstacle clearance occurs only when ATC radar provides navigational direction such as radar vectors.  Until such assistance is obtained it is the responsibility of the pilot use visual charts to avoid hitting anything.  SIDs allow departure planning and reduce

communications.  Use an area chart or sectional to confirm terrain and location of obstacles.  Advise departure of your need for a specific

departure or SID due to limited climb performance.  ATC will accommodate

your request.

Each instrument runway departure has three zones of obstacle clearance.

- The first zone is from the departure end, 500' to each side and 35' high in a 15-degree fan to each side for a distance of two miles in a 40:1 slope or 152 ft/nm.

- Zone 2 extends from a point on the centerline of the runway and 2000' from the approach end.  This zone extends in a semicircle arc toward the departure end  at a 40:1 slope until reaching minimum enroute altitude (MEA).

- Zone 3 extends from the same point in a 40:1 slope toward the approach end of the runway until reaching the MEA.

# Runway Visual Range

An approach requires RVR of 2400'.  If the RVR equipment

fails a ground visibility of one-half mile must be reported.

FAR 91.175.  Reported ground visibility or RVR  doesn't enter into the approach/landing decision for Part 91 operations..  For flight below MDA or DH you must be continuously in a position to make a landing with a normal rate of descent, have the runway or its environment in sight and have the flight visibility required for the approach.  The exception is

with an approach lighting with sequenced flashing lights. (ALSF)  Descent to, but not below, 100 feet above touchdown zone elevation is allowed unless the red siderow or terminating bars are clearly visible.

# Lighting

Every approach lighting system has a 'decision bar' of white lights 1000 feet from the threshold  green lights.  The 'rabbit' lights stop running at the 'decision bar'.

If you are at the middle marker (.6 mile from the threshold) and you can see the decision bar you have 1/2 mile visibility.  This may be the required visibility for the approach.

# VASI

If you have visual contact with the runway the FAA recommends flying the VASI if it is available.  If there is a visual descent point (VDP)You can descend below the MDA if you have visual reference as required by FAR 91.175(c)(3).  You cannot descend below the MDA before the VDP.

According to FAR 91.129 (d)(3) a flight into a VASI at a controlled airport with the tower open must remain at or above the glidepath until lower is required for landing.  The rule applies only to a VASI and not to other glideslope light system.  The rule applies only if the tower is operating.

The three-bar VASIs  has two glide paths.  The far two bars are for large aircraft; the two near bars are for small planes.  Normal glide path is 3 degrees and upper glide path is 1/4 degree higher.  Some locations have 4.5 degree glide path to give obstacle clearance.  VASI is not part of instrument landing system.  An inoperative VASI does not affect IFR minimums.  Obstacle clearance up to 4 miles or less and 10 degrees to each side.

Tri-color VASI

Green is on glide path, white is above and amber/red when below.

# PAPI

Four lights to left of runway.  4 white  for high 2 with/two red for just right, four red for low.

# AWOS Weather

- AWOS-3 is official weather.

- Airport may qualify as alternate airport with  AWOS-3.

- AWOS-3 gives altimeter, ceiling, visibility, wind,

- temperature, dewpoint, and density altitude above 1000'

- At controlled airports AWOS-3 is available only when ATIS is not.

Some AWOS have a phone number listed in A/F Directory.

# Terminology:

- "VFR over the top" is when a VFR pilot flies over a ceiling

- "IFR over the top" is the act of flying VFR over a ceiling on

an IFR flight plan

- "VFR on top" is the clearance an IFR pilot must request while

on an IFR flight

If on an IFR flight but in ‘on top’ conditions you are refused an IFR climb to remain on top, you can request an ‘on top’ climb in which you have effectively told ATC that you will remain VFR during the climb and accept traffic avoidance responsibility.  When climbing and when on top you are expected to:

- Fly the center line of the airway,

- Maintain a listening watch on the assigned frequency,

- Fly the clearance route,

- Make IFR position reports,

- Maintain VFR,

- Be responsible for traffic avoidance.

When nearing your destination you can request an ‘on top’ descent which will follow the same strictures as the ‘on top’ climb.  ATC likes this because it relieves them of the traffic minimums imposed by IFR.  Any time you cannot maintain VFR be sure to request a “hard’ altitude, climb or descent well ahead of time.

# IFR Terminology vs Meaning

- Altitude restrictions

Traffic procedure in which aircraft are assigned altitudes to 	expedite traffic, traffic separation or to balance workload.

- “You are held for release”

You may have your clearance but don’t leave until I tell you to.

- Call for release

Actual departure time is to be announced later by ATC..  One ATC 	facility asks another to find a spot.

- Controlled time of arrival

The time ATC uses to figure your departure time.  May involve 	ground delay.  Used to keep arrival runway within arrival 	capacity.

- Static restrictions

Routine restrictions on routes or at fixes to cope with high 	demand.

- Ground stop

ATC refusal to allow departures due to traffic load.  Only as 	necessary.

- Fix balancing

Rerouting of traffic to ease work load.  Allows traffic to arrive 	by other routes.

- Gate hold

Absorbs delays and reduces workload by holding aircraft before 	taxi or takeoff.  Applied with delays of over 15 minutes.

- Metering in progress

ATC control of traffic by assigning fix crossing times for 	arriving traffic.  Use of timing keeps arrivals within airport 	acceptance rate.

- “Radar Contact”

You are on my radar screen.  Don’t run into anything.

- “Radar Contact, fly heading...”

You are on my radar screen.  Do what I tell you to do and you 	won’t run into anything.  If not on a vector, terrain avoidance is 	the responsibility of the pilot.

- The pilot is well advised to fly the published IDP if you think an assigned heading will lead to CFIT.

- “When able, proceed...”

Go to planned or assigned point but don’t run into anything along 	the way.