Lapboard and Checklists

IFRã Checklist......................................2

Pop-up

Flight Planning IFR Flight

Preflight Checklist......................................3

Instrument Takeoffs.......................................4

Altitude Checklist

Aircraft at Cruise

Pre-approach Checklist

7 Tees

The 6 Tees ....................................5

Pre-approach Checklists

MALHIC

Lapboard Spaces

Cockpit Resouce Management.....................................7

Charted Items

Required Reporting Points

One Pilot's IFR Checklist......................................8

Aircraft Operational Quich Reference Chart

VOR Test Record

IFR Checklist

Missed Approach................................9

IFR Flight Form...............................11

George's Checklists...........................12

IFR Approach List.............................15

Bachelder's IFR Checklist

IFR Problem Areas.............................16

Instruction Preflight.....................................17

Practial Test Standards

IFR Checklists

You must tailor the generic POH list, the complex aircraft requirements, the IFR requirements and your requirements to ensure that everything necessary or desirable has been considered.  This allows you to include both the necessary and nice-to-have as you consider logical. The following items are only suggested for incorporation into you checklist.

Plan for at least five revisions.

In the preflight operation how often are steps retraced?

What checklist?  It's my plane.

How many wasted motions in the start operation?

What checklist?  I've done this thousands of times.

How often are the radio knobs turned the long way around to a

frequency?  You can change frequencies by counting clicks.

How many times to get the ATIS?

POP-UP   CCR

ATIS

Pitot heat / Alt Air

Nav #1      108.5

Nav #2   117.0

ADF    335

.Which way_ 181

How low  440/740

How long_2:16

VDP   1:30

Missed  - left to VOR

Alternate plates

Flight Planning IFR flight

The preflight

Before IFR flight FAR 91.103 in addition to all available

information requires ...

Pilot

Confirm personal papers of license, medical, BFR or AFR ,

currency passengers & night, 6, 6, 6, instrument.

No Alcohol within past 8 hours.  (I can usually tell if used within

hours by student performance)

The Plane:

To be legal the aircraft must have its ARROW papers.  Its  A & P Inspections , Airworthiness Directives (ADs) ELT check, Transponder Check, must be  recorded in aircraft and engine logbooks. Static system and altimeter tested.  (If inspected and approved

early January 1993.  Reinspection to remain current for

IFR flight required before January 31 1995.)  FAR 91.411

For legal IFR flight VORs must have been checked  and correctly logged within

the last 30 days.  FAR 91.171.

Instruments

required

Airspeed

Turn

Altimeter  Clock

Attitude  Directional

Radio/Nav

IFR flight requires gyroscopic heading indicator

.

Mnemonic

A TACADRN (elephant)  An IFR  ELEPHANT never forgets!

Not required equipment

The VSI is not a required IFR instrument.  Do not rely on VSI for pitch information unless it is an IVSI.  Dual VORs, ADF. ILS are not required

nor is pitot heat

Oxygen required

Oxygen for unpressurized aircraft 1/2 hour up to 14,000; Crew

above 14000' and everybody above 15000'.  FAR 91.211

Instrument Competency Check

The 6-6-6 rule requires six approaches and six hours within the past six months.  This rule can be circumvented by an Instrument Competency Check ride and an endorsement.

I certify that __#________ successfully completed the instrument competency check required by FAR Section 61.57 (e),(2) on (date)

Weather reports and forecasts

Forecaster tries to decide what is going to happen over a given period.  You must understand what is there and how it could change.  It is nice to get the expected; it is wise to expect the unexpected.

Known delays

DUAT has Flow Control Messages Menu under the Flight Planning section that shows where aircraft are being held on the ground (Gate Hold) because of traffic congestions.  This most often is caused by weather requiring alternate routing.  Keeping up with the weather can warn you if such may be happening enroute. This is usually initiated by Center.

NOTAMs

Airport information

Any known traffic delays advised by ATC

Runway lengths at airports of intended use

AFD Notams, L & D Notams

Temporary NOTAMS have 120 day life.

Notices to Airmen publication every two weeks.  Once a NOTAM is 			printed it will be dropped from the FSS briefing unless a search 		is specially requested

Fuel

requirements

You need enough fuel to fly to this airport,

fly to an alternate and fly thereafter for 45 minutes at

normal cruise.  FAR 91.167

Alternatives if flight cannot be

completed

IFR airport of intended landing is 1500 OVC and 3 (see 			minimums) at

planned ETA.

(See uncontrolled airports)

Preflight Checklist

Pitot heat/anti-ice

Positive check is only by feel

Taxiing

AI, TC, HI

Pre-takeoff

Pitot heat/anti-ice (A preventative not removal)

X-ponder

Radios(7)

AI, HI

Actual conditions

Pitot heat

Instrument takeoffs (Not recommended)

Pre-takeoff

Confirm pitot heat  (ventrui effect can freeze at above z-level)

Set attitude indicator for level

Confirm alternator output

On the runway

Align with runway centerline

Set HI

X-ponder and lights

Brakes on and power to 1500 (Spools up gyros)

Altitude checklist

(PRICE)

Pressure and quantity

Regulator

Indicator

Connections (secure)

Emergency (what to do)

Aircraft at Cruise

Fuel on fullest

Belts

Gear

Clock set

Nav radios set

Speed

Pre-approach Checklist:

ATIS

Adjust altimeter, heading gyro

Airspeed (pitch and power)

Avionics

Approach plate review

Notes review

fixes,

altitudes,

headings,

time

MDA/DH,

MAP/procedures

7 Tees

Turn, Time, Throttle, Tune, Twist, Talk, Track

THE 6 TEES

(TUNE)

TURN

TIME

TWIST

THROTTLE

TALK

TRACK

TURN

Every  time you TURN then TIME, TWIST, THROTTLE, TALK AND TRACK

TIME

Every time you TIME, then TWIST, THROTTLE, TALK AND TRACK

TWIST

Every time you TWIST, THROTTLE, TALK AND TRACK

THROTTLE

Every time you THROTTLE, TALK AND TRACK

TALK

Every time you TALK, TRACK

TRACK

What this means is that, in every change of direction  or altitude when in departure, enroute or approach go through the 6 Ts until it becomes an ingrained habitual procedure.  Not all of the 6 Ts will be used every time but going through the procedure with all six is a important skill to develop from the very beginning.

Pre-approach Checklist

Aircraft:Preparation

Every time					IFR conditions

Fullest tank				Pitot heat

Pump						Instrument failure check

Seat belts and harness			Engine check

Set heading indicator			Systems check

Timer set

Altimeter set

Gear down

Radios:

Frequencies and identified

OBS settings

ATIS

Clearance

Wind:

Direction and velocity

Time/speed estimates set

Plates:

Obstructions

Minimums

Straight-in

circling

Restrictions

Missed approach procedure

Memorize

MDA/DH + Personal minimums rounded

Time

Visual Descent point time (Normal landing)

First part of missed

Pre-approach 			Pre-departure

Frequency/idents			Procedure or SID

Course and altitude		Heading and altitude

FAF and altitude			Transponder code/on

Time and DA/MDA			Frequencies set/ident

Missed procedure

Mnemonic:

MALHIC

M - minimums, missed approach, markers

A - ATIS, Altimeter, ADF

L - Localizer, landing checklist, lights

H - heading inbound, #1 height, #2 height

I - ident navaids,

C - compass, clock, communications

Idea:	1. Use clock minute hand to make tank changes L to R.

Missed approach time

Missed approach route

Configuration:

Power

Speed

Gear

Flaps

Mixture

Fix

Lapboard Spaces

Time sheet (minutes after hour)

ETA/ATA

Takeoff___/___

VOR    ___/___

1st    ___/___

2nd    ___/___

etc.

Aircraft Performance

AT 90 knots climb   At 100 knots cruise

Feet per mile 		3 minutes = distance

6 minutes

5 Column frequency chart

Com 1|Com 2|Nav 1 OBS|Nav 2 OBS|ADF|Mkrs|

6 TEEs (For each checkpoint)

T/O |VOR|PITTS| ETC.

Turn  350  091  xx

Time  :17  :21  :25

Twist 350  071  071

Throttle full  full  40-

Talk  Track  VOR

Track TO   FROM  FROM

Cockpit Resource Management

Similar to good office management.

Go/no go weather, aircraft performance, physical 				condition, pilot ability, passenger limitations,

flight materials

Developing an IFR Checklist:

Preparation:

Emergency equipment

tools

lights

Charts and plates

VFR/IFR

Notams

Weather

Go/NoGo

Minimums

Departure/enroute/Arrival conditions

Aircraft

Papers

Maintenance

ELT according to FARs

Transponder according to FARs

Pitot/Static according to FARs

Pitot heat

VSI/alternate air

Preflight

Aircraft Performance (from manual)

Weight/balance

Range/altitudes

T/O and landing requirements

Fuel/oil

Pito heat

Planned flight time & conditions

Training procedures:

Walk/talk through headings

Walk/talk through altitudes

Work/talk through communications frequencies

Work/talk through navigational frequencies

Times between check point estimates/actual

Set the standards of performance

Charted Items

Headings #1    #2    Missed

Altitudes #1   #2    Minimum

Time      :

Required Reporting Points

Final Approach Fix

Leaving an Altitude

Failure of equipment

Unable 500fpm climb

3 minutes ETA

Missed Approach

Have traffic to follow

(more)

One Pilot's IFR Checklist

In range checklist

3 M's

Markers/ATIS/Altimeter/D.G./VOR's

Identified &

set/ADF  (I would break up this list-Gene)

Minimums memorized

MAP memorized

Landing checklist				FIX

Pump 					Time

Gas                            Turn

Undercarriage			Throttle

Mixture                        Tune

Prop                           Talk

Seat Belts

Develop your own IFR checklist:

Prestart:Wt & balance, fuel-range-times- Preset

radios on/test, VSI, charts in order

Start:  Clearance, runway, how-to,

Taxi: Attitude, heading, turn, brakes

Pretakeoff:  altimeter, frequencies, OBSs,

Transponder ON, time check, DG check,

pitot heat, emergency list

Confirm ARROW papers for aircraft are aboard.

Confirm that

ELT, static (altimeter per 91.171), transponder, annual and

100 hr inspections and tests are current.

Make a list of all airspeeds and power, flap and trim

settings to get them.

Aircraft Operational Quick Reference Chart

Power settings and airspeeds for climb, cruise, cruise

descent, approach & descent for ILS and non-precision. Give a

count of trim settings needed to get from one to the other.

To fly by the numbers you must know the numbers.

Control: AI and power

Performance: airspeed, altimeter, TC, HI, VSI

VOR Test Record

Since VOR test records seem to get lost.  Keep your own in

the back of your logbook.  Be sure to include type of check,

frequency used, error noted both to & from for both OBS's,

sign and date.

IFR Checklist

Pre-approach check list:

1. a. Pre-landing list

b, Heading set

c. Timer

2. a. ATIS

b. Frequencies (4)

c. Wind

3. a. Approach plates

b. Plate sequences

c. Memorize rounded numbers to high side.

1. MDA

2. Time to MAP

3. Missed approach

4. Approach speed point

Prepare the 6 T/s at every fix

1. Time

2. Turn

3. Throttle

4. Tune

5. Talk

6. Trim

Outbound:

1. Figure wind to get 2-3 minutes away from FAF inbound

Winds at CCR change direction and velocity at shoreline

2. Intercept outbound before making procedure turn.

3. Begin descent when needles start to move.

Procedure turn: (See preceding material on Procedure Turn)

1. If equipment permits put in 10 nm procedure turn limit

on #2 VOR

2. Determine pitch and power required for level and descent

at constant airspeed.

3. Gear/flap extension point selection

At the FAF:

1. 6 T,s

2. Descent to MDA. Begin to include the windshield in

your scan at reaching the MDA.

3. Fly the time

a. No further descent without runway in sight

b. VDP  make one if not published.

Making a Visual Descent Point

1. 10% of MDA altitude as seconds.

2. Subtract from approach time

3. If runway is not in sight plan

for missed approach.

Missed Approach:

1. Start the missed approach climb at any time but continue 			direct to missed approach point.  A  turn on final 				approach will not have

assured obstacle clearance regardless of altitude 				until	the missed approach point.

2. Start the missed approach turn at the missed approach 			point	regardless of altitude.

Cockpit Preparation

VOR check

Every pilot should keep his own log in back of log book.

Date of check

Aircraft

Dual VOR check

What frequency

Location

Error to

Error from

Signature

Prestart VFR list +

Set clocks

Set Nav radios and OBS

Set Com frequencies for ATIS and taxi

Start VFR list +

All radios ON and set (7) in all

ADF test

Markers test

(DME)

VSI check

Taxi VFR list +

Turn check of turn coordinator, heading indicator, and attitude indicator

Pre-takeoff list +

Time check and verified into tape recorder

Hood fitted

Takeoff VFR list +

Heading check

Time off

Climb VFR list +

Time to climb

Cruise VFR list +

Time to ^point

VFR^ Checklist +

6 TEEs

Pre-Approach checklist

Approach checklist

Set up missed

IFR CHECKLISTS:

IFR PREFLIGHT

Pitot heat

Alternate air

Lighting

PRESTART                      CLEARANCE           TAXI

Charts                       Limit              Heading

Sequence of:              Departure          Turn/Bank

Headings               Transition         Attitude

Frequencies            Altitude            #1 Ident

Altitudes              EFC Point/time      #2 Ident

Intersections          Frequency          Confirm route

ATIS                   squawk

MDA &TIME              Confirm clearance

Radios

X-ponder                     CHECKPOINT     FINAL APPCH FIX

#1Com #1Nav-OBS set            Time record    Time

#2Com #2Nav-OBS set            Heading bug   Turn

ADF                            Altitude v     Throttle

DME                            Airspeed v     Tune

Markers                        Frequency v    Talk

Clocks                             ATIS v      Missed Appch v

Ident

PRE TAKEOFF

Time check

X-ponder

Heading

Ident Navs

ACTUAL CONDITIONS CHECKLIST

Pitot heat

Carburetor heat

Beacon off

Landing light off

IFR Flight Form

Course__________to ____________         Useable fuel _______gal

Altitude        ft  Airspeed _____Kts         Fuel burn

___  ___  ___  ___  ___   ___ = ___     Start/Taxi/Takeoff  ___gal

TC + WCA  TH + VAR  MH + DEV    CH      Climb..............___gal

_______________________________________ (to______ft at___NM in___min

Depart:_______         Airport Diagram  at ______rpm and _____ gph)

ATIS ______  FSS_____   Pattern alt ___

Cl D ______  (_______)       			   Total............_____ gal

GRD  ______  CTR_____

CT   ______  (_______)				   __________________________

DEP  ______

_________________________			   Reserve = _______ gal

Departure time _______			        Flight time

_________________________			   (_______ min at _______ rpm

___Open Flight Plan __

Enroute time				   _______MP     _____% Power

___ hr  ___ min

___________________________________________________________________

Kts   from   CUM   Total NM  ETE ATE  Cum  NOTES  Center   Fuel

GS		   NM   Distance  ETA ATA  Time				 Notes

Leg NM

Distance

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

________  ___ ___  ___

___________________________________________________________________

-------------------------------------------------------------------

VOR   _____   _____   ______   ______   ______   ______   _____

Freq

-------------------------------------------------------------------

Arrive ______         Airport diagram			ATIS

FSS_____  ATIS _____  Pattern Alt ______

(______)  CT   _____

APP ____  GRD  _____

(______)

---------------------

Arrival time ________

----

____CLOSE FLIGHT PLAN

George's Checklists

IFR Approach Checklist (added to VFR list

Cockpit

Headset/intercom

Plates

Prestart

Nav Radios/freq/ OBS

ADF/on/freq

Markers/on/test

Alternate Air (Knob and Door checks)

Taxi

Instrument check

AI, HI, TC

Climb/Cruise

Pitot heat/clouds

Pretakeoff

Timer/Recorder

Time check

Heading Set

In Range

ATIS

Altimeter

Heading Set

Coms Freq/vol

Davs Freq/vol

ADF freq/vol

Markers Test/vol

IAF

6 T's

Clock set

MDA check

FAF

6t's

Clock start

Heading set

Pre-landing

GUMP

(* indicates specific to IFR)

Cockpit

Headphones/intercom

Key/off

Seats/belts			Suggest- Alternate Air Check

*Plates

Prestart

Com radios/freq

*Nav radios/freq/OBS

*Markers/on/phones

Transponder/standby

*ADF/on/freq

Circuit breakers/switches

Mixture rich/fuel both

Carb heat off

Throttle 1/8"

Brakes on

Start

Radio master off

Master switch on		Suggest: Prime

"Clear" start

Radio master on

rotating beacon on

1. mixture lean

2. Flaps up

3. Oil pressure

Suction/amps

ATIS/1st time

Set Altimeter/HI

Call Grnd/Clrnce

Taxi

Brake check

Aileron for Wind/HI

Climb into/Dive away

*Instrument Check

*AI level

HI turning

TC turning

Before takeoff

Instrument check

Transponder/strobes on

Landing light on

Carb heat off

Mixture as required

Primer locked			Suggest: Flight route review

Fuel both tanks

Flaps up/trim set

Door latches/belts

*Advise Grnd/Clearance

Timer/recorder set

Call tower/wind

*Time chk/write dwn

*Set HI on Rwy

Climbing thru 1000'

landing light off

Cruise

Mixture as required

Set HI/scan chk

*Pitot Heat/clouds

Prelanding

ATIS/unicom

Carb heat on/descent

Engine instruments/gauges

Set HI

G as  to both tanks

U undercarriage

M mixture as required

P rop pumps/pressure

S safety belts/lights

Call tower

After landing

Flaps up

Carb heat off

Mixture leaned

Transponder/strobes off

Landing light off

Aileron for wind/HI

Climb into/dive away

Call ground

Shut down

Radio 1221.5/off

All electrical off

Master Sw off

Mixture out

Throttle out

Key off/out

Lock up

Tie downs

Control lock

Time log sheet

Headphones etc.

Clean Cockpit

Seat belts

Sun Shades

Lock up

IFR APPROACH LIST

IN RANGE

*ATIS/obtain

*Altimeter set			Suggest: Last check of DH/MDA

*Heading indicator set				time, course

*Com 1 & 2 Freq/vol

*Nav 1 & 2 FReq/Vol/OBS

*ADF/frq/vol

*Markers/test/vol

IAF

6 T's

Clock/set

MDA/DH confirm

FAF

*6T's

*Clock start

*Heading ind/set

Pre-landing

Gas

Undercarriage

Mixture

Prop

Safety

Batchelder's IFR Checklist

IFR WEATHER BRIEFING

DEPARTURE              WEATHER TRENDS

ENROUTE         FREEZING LVL, TURB

DESTINATION            NOTAMS

ALTERNATE              SIGMETS, AIRMETS

ESCAPE ROUTE           PIREPS

PITOT HEAT-OPERATIVE

VOR CHECK

PAST 30 DAYS  DATE-PLACE BEARING CHECK- SIGNATURE

TRANSPONDER-ALTIMETER

SYSTEMS CHECK -PAST 24 MONTHS

ALTERNATE AIR

LIGHTING

CALIBRATION OF TURN COORDINATOR

TAXIING

RUN UP

TURN COORDINATOR                   #1 VOR  #1 COM ON FREQ

HEADING INDICATOR                          #2 VOR  #2 COM ON FREQ

ATTITUDE INDICATOR                         DME ON

TRANSPONDER-ON-TEST-STNDBY                        	                                     ****TUNE AND VERIFY*****

AIRSPEED INDICATOR CHECK                   MARKER BEACONS-ON/TEST

ALTIMETER CHECK (WITHIN 75')               ADF-ON FREQ-TEST

ALTERNATE STATIC SOURCE

LINE UP CHECK:                   POSITIVE RATE OF CLIMB

TIME OFF/TRANS/MODE C-ON           ESTAB THEN GEAR UP

STROBES-ON                         A.I. & VISUALLY

LANDING LITES-ON                   V.S.I. CLIMB TREND

PITOT HT-ON IF NECESSARY           ALTITUDE

/H.I.

MAG COMPASS=RUNWAY ALIGN           THEN GEAR UP

POSITION REPORTS                   FIX (5 T's)

ATC FACILITY                       TIME-TURN-TWIST-

AIRCRAFT ID                        THROTTLE-TALK

P-T-A-T-E-N

POSITION- TIME- ALTITUDE- TYPE OF FLIGHT PLAN- ETA NEXT

REPORTING POINT- NAME OF NEXT REPORTING POINT.

PRE APPROACH                       APPROACH

#1 COM ON FREQ                       FINAL APP COURSE ____

#2 COMM ON FREQ                      ALTITUDE AWARENESS

ATIS / TIMING CLOCK                  F A F (5 T's)

MARKER BEACON- ON                    TIME _________________

#1 NAV TUNE AND VERIFY               DH OR MDA ________

#2 NAV TUNE AND VERIFY               MISSED APP PROCEDURE

ADF APPROACH - VOLUME UP

#1 NAV- TUNE VERIFY

ALTIMETER SETTING

MISSED APP PROCEDURE - REVIEW

#2 NAV- TUNE VERIFY

C-G-U-M-P-C

ANOTHER APPROACH OR

COWL FLAP-GAS-UNDERCARRIAGE-          ALTERNATE

MIXTURE-PROPELLER-CARB HT, FUEL SITUATION

IFR Problem Areas

FSS CONTACT/FILING FLIGHT PLAN

IFR PRESTART CHECKLIST

FREQUENCY LIST FOR COM/NAV #1/2

DEPARTURE/ARRIVAL PLATES

CLEARANCE PRIOR TO TAXI PROCEDURES

ALL RADIOS SET AND ON

IFR TAXIING CHECKLIST

INSTRUMENT CHECK

PRE TAKEOFF CHECKLIST

TIME

TRANSPONDER

HEADING CHECK

IDENT VORS?

Instruction Preflight

Preparing for IFR Student flight:

Walk through all headings.

Walk through all headings and altitudes

Walk through all headings altitudes and fixes.

Walk through all headings, altitudes, fixes and

radios.

Draw heading map

Draw heading map and altitudes.

Draw heading map with altitudes and fixes (OBS and

frequencies)

Make five column radio frequency chart

#1 Com #2 Com  #1 Nav #2 Nav  DME/ADF

markers

Make approach Chart summary

IAP (Initial approach fix)

Heading to___From_____

altitude	altitude

Other fixes

Heading to_____ From_______

altitude    altitude

FAF (Final approach fix)

Heading to_____ From ______

altitude    Altitude

Time

COPY CLEARANCE

_________CLD TO THE ______________________-

M. ___________

DEP.__________

SQK __________

TIME NOW ______-EXPECT FURTHER

CLEARANCE _______-

AMENDED CLEARANCE

ATIS_____

HDG_____

M.______

Practial Test Standards

This is an abbreviation of the Practical Test Standards.  Some

omissions have occurred.  Refer to FAA-S-8081-4A

I. AREA OF OPERATION(FAA-S-8081-4A)

GROUND PHASE

A. TASK: OBTAINING WEATHER INFORMATION

PILOT OPERATION - 4

REFERENCES:  FAR Part 61; AC 00-6 and AC 00-45; and AIM

Objective,

1, Able to obtain, read, and interpret weather information

2, Weather related to this flight

B. 	TASK:  CROSS-COUNTRY FLIGHT PLANNING

PILOT OPERATION - 4

REFERENCES: FAR Parts 61 and 91; AC 61-27; and AIM

Objective:

l. Plan flight within regulations and flight rules.

2, Know aircraft performance and handbook calculations as 			required for this IFR flight

3. IFR chart selection and interpretation

4. Selection and interpretation of NOTANS

5. Knows aircraft capability and handbook calculations

(FAA pays by the word.  Seems much the same as #3)

6. Files and flies flight as planned.

C.   TASK: AIRCRAFT SYSTEMS RELATED TO IFR OPERATION

PILOT OPERATION - 4

REFERENCES:  FAR Part 61; and AC 61-27

Objective. Knowledge of aircraft systems and operation of:

1. Airframe

2. Propeller/intakes

3. Fuel system

4. pitot-static

D. TASK: AIRCRAFT FLIGHT INSTRUMENTS AND NAVIGATION EQUIPMENT

PILOT OPERATION - 4

REFERENCES:  FAR Part 61 and AC 61-27

Objective.

1. Knowledge of system and operation of:

a. pitot-static

b. altimeter

c. airspeed indicator

d. vertical speed indicator

e. attitude indicator

f. horizontal situation indicator

g. magnetic compass

h. turn-and-slip indicator/turn coordinator

i. heading indicator

1. Knowledge and use of navigational systems:

a. VOR

b. DME

c. ILS and MLS

d. Markers

e. Transponder

f. Automatic direction finder (ADF)

E. TASK: INSTRUMENT COCKPIT CHECK

PILOT OPERATION - 5

REFERENCES: FAR Parts 61 and 91; and AC 61-27

Objective.

1. Preflight check of instruments and why.

2. Preflight use of checklist

3. Determines safety and legality of flight:

a. Radios

b. Navigation equipment

(1) VOR

(2) ADF

(3) ILS

(4) Markers

c. Compass

d. Heading indicator

e. Attitude indicator

f. Altimeter

g. Turn-and-slip indicator/turn coordinator

h. Vertical speed indicator

i. Airspeed indicator

j. Clock

k. Vacuum/electric

l. pitot heat

4. Discrepancies and legality to fly.

II. AREA OF OPERATION

AIR TRAFFIC CONTROL CLEARANCES AND PROCEDURES

(Actual or simulated)

A. TASK: AIR TRAFFIC CONTROL CLEARANCES

PILOT OPERATION- 2

REFERENCES:  FAR Parts 61 and 91; AC 61-27; and AIM

Objective.

1. Knows elements of clearance, division of 					responsibilities, and types of clearances

2. Copies clearance as issued

3. Checks clearance against aircraft capabilities and 				charts.

4. Requests changes, clarifications or verifications

5. Correctly reads back clearance as received

6. Use of standard phraseology

7. Presets radio, navaid, and transponder

B. TASK: COMPLIANCE WITH DEPARTURE, EN ROUTE, AND ARRIVAL 			PROCEDURES AND CLEARANCES

PILOT OPERATION - 2

REFERENCES: FAR Parts 61 and 91; AC 61-27; SID; En 					Route Low Altitude Charts and  STAR

Objective.

1. Knows charts and division of responsibility

2. Use of current and appropriate charts

3. Use of correct nav/com frequencies and 	settings

4. Use of checklists for each phase of flight

5. Communications with correct phraseology

6. Compliance with ATC instructions and restrictions

7. Knows radio failure procedures

8. Flies route as cleared

9. Within flight tolerances of 10/10/100 and track

C. TASK; HOLDING PROCEDURES

(DME only if equipped)

PILOT OPERATION - 2

REFERENCES: FAR Parts 61 and 91; AC 61-27; and AIM

Objective.

1. Knows how to hold

2. Knows when to slow down for hold (3 minutes)

3. Uses FAA entry procedure (?) for holding

4. Initiates hold at fix

5. Knows when report required to ATC

6. Uses time as required by ATC

7. Uses DME for holding leg lengths

8. Uses adjustments for wind and time in holding

9. Holds within 10/10/100 and holding tolerances

III. AREA OF OPERATION

FLIGHT BY REFERENCE TO INSTRUMENTS

(Partial panel required)

A. TASK: STRAIGHT-AND-LEVEL FLIGHT

PILOT OPERATION -1

REFERENCES: FAR Part 61; and AC 61-27

Objective.

1. Able to fly straight-and-level

2. Able to flight straight-and-level while changing 				configuration

3. Keeps tolerances of 10/10/100

4. Uses proper(?) cross-check with appropriate(?) pitch, 			bank, power and trim

B. TASK: CHANGE OF AIRSPEED

PILOT OPERATION - 1

REFERENCES: FAR Part 61; and AC 61-27

Objective.

1. Able to change airspeed in straight-and-level and 				turns

2. Proper use of power in changing airspeed

3. Maintains 10/10/100 tolerances

4. Uses proper (?) cross-check and appropriate (?) 				pitch, bank, power and trim

C. TASK: CONSTANT AIRSPEED CLIMBS AND DESCENTS

PILOT OPERATION - 1

REFERENCES: FAR PART 61; AND ac 61-27

Objective.

1. Able to fly constant airspeed climbs and descents

2. Able to fly constant airspeed climbs and descents 				in turns between specific altitudes

3. Flies from specific altitudes and headings into 				constant airspeed climbs and descents

4. Sets attitude and power for selected climb or 				descent performance.

5. Keeps 10/10/100 tolerances

6. All level-offs within 100'

7. Uses proper (?) cross-check and appropriate pitch, 				bank, power and trim

D. TASK: RATE CLIMBS AND DESCENTS

PILOT OPERATION - 1

REFERENCES: FAR Part 61; and AC 61-27

Objective.

1. Able to fly rate climbs and descents

2. Able to fly rate climbs and descents between 				altitudes  straight or in turns

3. Able to enter rate climbs and descents from 					specific altitude, airspeed and heading

4. Sets pitch and power and bank for rate climb or 				descent

5. Keeps 10/10/100  and 5 degree bank tolerances

6. All level-offs within 100'

7. Uses proper (?)cross-check and appropriate pitch, 				bank, power and trim

E. TASK: TIMED TURNS TO MAGNETIC COMPASS HEADINGS

PILOT 0OPERATION - 1

REFERENCES: FAR PART 61; AND ac 61-27

Objective.

1. Understands calibration of turn coordinator, 				operation of compass and the making of timed 				turns

2. Can fly left and right standard rate turns

3. Uses clock to calibrate turn coordinator

4. Adjusts turn coordinator position for standard rate

turns

5. Makes compass timed turns

6. Keeps 10/10/100 and + 5 degree rate turns with 				recovery to selected heading within 10 degrees

F. TASK: STEEP TURNS

PILOT OPERATION -1

REFERENCES: FAR Part 61; and AC 61-27				Objectives.

1. Able to fly steep turns IFR

2. Makes 45 degree steep bank

3. 360 or 180 degrees left and right

4. 10/10/100 tolerances (10 on rollout + 5 degrees of 				bank

5. Uses proper cross-check and appropriate pitch, 				bank, power and trim

G. TASK: RECOVERY FROM UNUSUAL FLIGHT ATTITUDES

PILOT OPERATION - 5

REFERENCES: FAR Part 61; and AC 61-27

Objectives.

1. Can recover from nose high/low attitudes IFR

2. Recovers using proper cross-check and appropriate 				pitch, bank and power in correct sequence

IV. AREA OF OPERATING

NAVIGATION AIDS

A. TASK: INTERCEPTING AND TRACKING VOR/VORTAC RADIALS AND DME ARCS

PILOT OPERATION -2

REFERENCES: FAR Parts 61 and 91; AC 61-27; and AIM

Objectives.

1. Understands VOR/VORTAC and DME arc interception and 				tracking.

2. Tunes and identifies before using navaids

3. Correctly sets OBS to desired radial (RMI)

4. Intercepts radial TO/FROM navaid at appropriate 				angle

5. Flies interception and tracking of radials at 				10/5/100 (Note: headings within five degrees)

6. Flies radial within 3/4 deflection of CDI (Course 				deviation indicator) or 10 degrees of RMI

7. Can determine position relative to navaid

8. Can intercept DME arc and stay within mile of arc

9. Recognizes navaid failure and reports to ATC

B. TASK: INTERCEPTING AND TRACKING NDB

PILOT OPERATION - 2

REFERENCES: FAR Parts 61 and 91; AC 61-27 and AIM

Objective.

1. Able to intercept and track NDB bearing

2. Tunes and identifies navaids

3. Monitors navaid operation

4. Determines relative bearing of NDB

5. Uses double the angle interception to or from NDB

6. Tracks to NDB at 10/5/100

7. Heading to correct track within 10 degrees

8. Knows position relative to NDB

9. Knows of ADF/NDB failure and reports to ATC

V. AREA OF OPERATION:

INSTRUMENT APPROACH PROCEDURES

A. TASK: VOR?VORTAC INSTRUMENT APPROACH PROCEDURE

PILOT OPERATION - 3

REFERENCES FAR Parts 61 and 91; AC 61-17; Standard 			instrument approach procedure chart; and AIM

Objective.

1. Knows elements of VOR/VORTAC approach procedure

2. Uses appropriate VOR/VORTAC procedures

3. Uses appropriate radios and phraseology throughout

4. Selects, tunes, identifies and monitors to assure 				status of navaids (keeps volume audible)

5. Complies with clearances of ATC or examiner

6. Detects failure of AI or HI (resets) advises ATC

7. Advises ATC or examiner if aircraft unable to 				comply with a clearance

8. Sets approach configuration and speed and uses 				checklists

9. Keeps 10/10/100 and less than full scale CDI 				deviation

10. Makes MDA adjustments required by:

a. FDC and Class II NOTAMS

b. Inoperative aircraft/navaids

c. Inoperative visuals

d. Weather reports and criteria

11. Flies descent and track to reach MDA prior to MAP 				with aircraft able to make normal landing

12. On final approach segment keeps within 10 kts and 				3/4 scale deflection of CDI (10 degrees RMI)

13. Maintains the MDA when reached +100/-0 to MAP

14. Executes missed if visual reference not acquired 				at MAP

15. Executes normal landing as instructed by examiner

B. TASK: NDB INSTRUMENT APPROACH PROCEDURE

PILOT OPERATION - 3

REFERENCES: FAR Parts 61 and 91; AC 61-27; Standard 				instrument approach procedure chart; and AIM

Objective.

1. Knows elements of NDB approach procedure

2. Flies appropriate NDB approach procedure

3. Uses appropriate radio phraseology throughout

4. Selects, tunes, identifies, confirms and monitors 				navaid status

5. Complies with clearances issued by ATC or examiner

6. Detects AI or HI failure or inaccuracy advises ATC

7. Advises ATC when aircraft unable to comply with 				clearance.

8. Uses checklist to configure aircraft for phase of 				flight

9. Flies within 10/10/100 tolerances

10.  Makes MDA adjustments required by

a. FDC and Class II NOTAMS

b. Inoperative aircraft/navaids

c. Inoperative visuals

d. Weather reports and criteria

11. Flies descent and track to reach MDA prior to MAP 				with aircraft able to make normal landing

12. On final approach segment keeps within 10 kts and 				3/4 scale deflection of CDI (10 degrees RMI)

13. Maintains the MDA when reached +100/-0 to MAP

14. Executes missed if visual reference not acquired 				at MAP

15. Executes normal landing as instructed by examiner

C. TASK: ILS/MLS INSTRUMENT APPROACH PROCEDURE

PILOT OPERATION - 3

REFERENCES: FAR Parts 61 and 91; AC 61-27; Standard 			instrument approach procedure chart; and AIM

Objective.

1. Knows elements of ILS/MLS approach procedure

2. Uses appropriate ILS/MLS procedures

3. Uses appropriate radios and phraseology throughout

4. Selects, tunes, identifies and monitors to assure 				status of navaids

5. Complies with clearances of ATC or examiner

6. Advises when aircraft unable to comply

7. Sets approach configuration and speed and uses 				checklists

8. Keeps 10/10/100 tolerances

9. Makes MDA adjustments required by:

a. FDC and Class II NOTAMS

b. Inoperative aircraft/navaids

c. Inoperative visuals

d. Weather reports and criteria

10. Flies from interception a descent that follows

glide slope

11. Flies within 10 kts and 3/4 deflection of 					localizer and glide slope

12. No descent below DH before making missed or 				landing

13. Executes missed at DH if visual reference not 				acquired

14. Executes normal landing

D. TASK: MISSED APPROACH PROCEDURES

PILOT OPERATION - 5

REFERENCES: FAR Parts 61 and 91; AC 61-27; Standard 			instrument approach procedure chart; and AIM

Objective.

1. Knows missed approach procedures and chart 					interpretation

2. Initiates by power, pitch attitude, configuration 				and trim

3. Advises ATC of missed

4. Complies with published or alternate procedures

5. Advises ATC or examiner if aircraft unable to 				comply with a clearance

6. Uses checklist

7. Requests ATC clearance to alternate

8. Keeps 10/10/100 tolerances during missed

E. TASK: CIRCLING APPROACH PROCEDURE

PILOT OPERATION -3

REFERENCES: FAR Parts 61 and 91; AC 61-27; Standard 			instrument approach procedures chart; and AIM

Objective.

1. Knows elements of circling approach procedure

2. Flies appropriate circling procedure within 					capabilities of aircraft

3. Complies with pattern, restrictions and directions 				of ATC and examiner

4. Maintains criteria of visibility and altitude until

established for normal landing

F. TASK: LANDING FROM A STRAIGHT-IN OR CIRCLING APPROACH PROCEDURE

PILOT OPERATION - 3

REFERENCES: FAR Parts 61 and 91; AC 61-27; and AIM

Objective.

1. Knows responsibilities to the environment, the 				operational area and meteorological factors for 				types of landings.

2. Makes normal landing from DH, MDA, or VDP

3. Adheres to all advisories related to operations 				considerations

4. Uses checklists for all phases of flight