# Click on page number for instant scrolling.

Approachesã

DME Arc

Three ways to fly a DME arc

Bracketing

Centered method:

10 Degree Method:

ILS Method

ILS another way

Single Pilot ILS

Fly the ILS with Rudder

ILS Partial panel

A Better ILS

Downwind ILS

ILS Failure

Catching ILS Malfunctions

Emergency

ILS Test:

Non-precision Approaches

The Best of a Bad Approach

Straight-in Approach

Localizer Approaches

Checking Localizer

VOR Approaches

Intercepting a Radial

VOR Checks

ADF Features

Emergency ADF use:

NDB Cold approach

Visual Procedures:

Visual approach

Contact Approach

Charted Flight Visual Procedures (CFVP)

PAR Approach (obsolete)

Airport Surveillance Radar Approach (ASR)

No-Gyro Approach

# DME Arc

Arc distances vary from 7 to 30 miles.  At 100 kts lead turn from radial to arc by 1/2 mile and 90 degrees change (tangent heading).  You should know that the obstacle clearance on a DME arc is the same as an airway.  Four nautical miles to each side at the specified altitude. Minimum vertical obstacle clearance is 1000' or 2000' if mountainous.  For straight final DME segments the obstacle clearance is 2 to five miles wide and 250'.  If final is an arc, it is 8 miles and 500'.  DME arcs are usually initial segments but can be intermediate, final or missed approach.  Since you need to refer to the DME chart often, be sure of your competency to make rapid visual checks of the charts without losing aircraft control.  Regardless of the method the lead radial is where you should change over to the ILS frequency. DME arcs are usually NoPT.

DME arc practice can consist of flying semi-circle arcs and varying distances.  Flying the arc can become easy by flying tangent headings while comparing the OBS setting and HI. this corrects for wind as well.  The radio magnetic indicator (RMI) is a must for flying DME arcs except for the most proficient.

A DME arc procedure flown by your own navigation must begin at a IAF.  ATC actually has the option of ignoring the arc and vectoring you into the initial or intermediate segment of the approach.  Training can be augmented by departing via an airway to intercept the arc from the inside.  Makes student identify and turn correct way.

# Three ways to fly a DME arc

The PTS requires tracking the arc within 1 mile of the published distance.

# Bracketing

Know the 90 degree heading required when intercepting the arc from a radial.  If the DME reading increases, turn into it by 10 degrees.  If the DME reading decreases, turn away by 10 degrees.  This bracketing method does not provide position guidance and can be difficult in strong winds.  It is the low workload method.

# Centered method:

- Turn 90 degrees from the interception radial to the arc.

- Turn the OBS to keep the needle constantly centered

- Keep your heading 90 degrees to the OBS radial setting.

- Any changes in DME readings must be corrected as in method #1.

# 10 Degree Method:

- Turn 90 degrees from the interception radial to the arc.

- Set OBS 5 degrees ahead.

- It is better to initially set 1/2 needle deflection.

- Fly until 1/2 needle deflection to the other side.

- This gives better control over the 10 degrees than with full 			deflection to one side

- Turn the OBS ahead another 10 degrees.

- Bracket the DME readings as in method #1.

# ILS Method

- Fly a heading

- Initial heading changes to find the reference heading should be

- 5 degrees.  (10 degrees only if 1/2 deflection.)

- Make all turns by reference to heading indicator.

- Stabilize your heading and airspeed based on the wind.

- Interception angle of 10 degrees maximum to the ILS.

- Set power for glide slope descent -- correct glide slope with yoke (slight changes).

- 1/2 scale glide slope errors require power changes to avoid airspeed excursions.

- Power is used since yoke corrections would cause excessive airspeed changes.

- ILS Descent rule of thumb:  Ground speed times five add fifty.

Flying the  localizer/glide slope gets more difficult below 500' of threshold crossing height (TCH) because of increased sensitivity and visual search.  If another pilot is aboard let him do the looking while you fly.

The instructor can observe how well the student is holding the airspeed and rate of descent.  Changes in the rate of descent must be accomplished with no change in airspeed.  Learn the skill of keeping the sound constant.  As the rate of descent is less than or more than 500 fpm practice making rate changes without airspeed changes.  Time the descents as well.  The essence of this skill development is to control attitude and power while making small heading and vertical speed changes.  This is good preparation for flying a PAR (Precision Approach Radar) approach at a military field.

An additional practice with the glideslope/localizer needles can be flown by intercepting one of the upper ghost slopes.  The reason there is an intercept altitude for an ILS is that it is possible to get the needles of the ILS to function on other signals that provide a much steeper slope.  Fly to an ILS runway at a high constant altitude and watch the needles. You don't usually find this in the textbook literature.

It is a good practice to initiate the descent 1/4 dot distance early.  An illustrative practice of the ILS descent skill can be flown in VFR with the needles covered.  At ILS intercept and initiation of the descent cover the needles and fly the descent based on the projected ground speed.  Once the descent is stabilized, occasionally uncover (15 seconds) to make checks for needed corrections.  A variation of this would be to establish a stabilized descent on one of the ghost slopes and fly the rate of descent with covered needles and occasional 15 second checks for performance.  Once you set your known power setting for the known airspeed and known descent rate for the approach, you can control your glideslope with pitch and trim.  You may find this technique much smoother and easier as a way to stay in the doughnut.

The sought for result of these training exercises is for the student to recognize that flying the ILS means stabilizing the attitude that gives the desired rate of descent and heading.  When you maintain a correct approach attitude for the ILS, your airspeed and  rate of descent have solved the glide slope part of the ILS equation. Scan only the AI, altimeter and needles.  With a constant attitude, changes in the needles will be due to wind.  If the needles move from center, first make the change required to stop the change and them make the correction.  At DH take a look make your choice.

If ILS is not monitored no alternate minimums will be published.  If tower is part time alternate minimums do not apply during time tower is closed.  To be an alternate the procedure must be monitored and weather reporting must be available.  If terminal forecast is not there use area forecast.  Where nonstandard alternate minimums are published for an airport they must be used instead of the standard minimums.

# ILS another way

You can fly an ILS without chasing the magnetic compass.  Fly into the localizer needle when it moves; when it stops you stop turning.  The VSI and CDI needle can be used as primary instrument for an ILS approach to minimums.  Only fingertip pressure at all times to check the pressure

of the trim setting...trim is the name of the game.  Dance with your eyes over the instruments.  The lower you get, the more flicky the needles and the smaller any corrections. Never go nbelow minimums

Compass turns can be made by time with a standard rate of 3 degrees per second.

Rule of Thumb compass turns:

- When turning Northerly, undershoot the heading by the latitude in degrees plus half of the bank angle.

- When turning Southerly, overshoot the heading by the latitude minus half the bank angle.

- When turning East or West from the South roll out 5 degrees early.

- When turning East or West from the North roll out 10 degrees early.

# Single Pilot ILS

Divides the ILS approach into eight stages, arranges priorities of when, where, and how to perform each stage.

- Stage 1

- On downwind vector do prelanding of

- ATIS to determine minimums, runway, wind, localizer minimums.

- Altimeter setting and altitude to cross check Marker altitude

- Airspeed slowed to approach speed

- Avionics set all radios (4 to 7 depending)

- Stage 2

- Vector to intercept.  Set localizer ident.

- Stage 3

- Set gear and flaps for glide slope descent on intercept.

- Stage 4

Six Ts

- Turn--not required on ILS

- Time--only if localizer approach possible

- Twist--Set up missed approach

- Throttle--Throttle as required

- Talk--Call Marker inbound and check altimeter

- Track--Needle in doughnut and on heading

- Stage 5

- DH + thousand feet

- Stage 6

- Circling minimums or localizer approach option

- Stage 7

- DH + 100 feet (Personal minimums)

- Stage 8

- DH, have runway, have clearance, declare missed.

# Fly the ILS with Rudder

Without rudder input holding headings and making heading corrections of less than 10 digress is nearly impossible.  Use the rudder to hold headings and make corrections of less than 120 digress.  It's quicker, more accurate, and less likely to induce a correction back in the other direction.  Sure, it's a skid, just hold the wings level and kick the rudder as required.

Fly "bugs"; "gates" and 5 degrees.	Fly the localizer 5 degrees at a time by making "gate" corrections.  A "gate" is the five degree space between the tics on the heading indicator.  You can think of making a correctional turn on the localizer as being one or two "gates".

# ILS Partial panel

Losing the AI is a good partial panel exercise.  The turns or lack of turns then rely on the turn coordinator or needle.  Try controlling  this with the rudder only.  Wings level =  no turn.  The plane can be flown in climb, level, descent and turns without use of the yoke.  It takes a lot of confidence and judicious use of throttle trim and rudder.  It would be wise to know if your auto-pilot remains coupled during gyro failure so you should uncouple "George" in the event of gyro failure.  The turn coordinator is the instrument of choice for recovery from unusual attitudes under partial panel.

# A Better ILS

Problems related to the ILS reside mostly in faulty techniques

in using the aircraft controls and lack of system understanding.  We can arrive on the ILS not knowing where we are.   Instead of trying to do

everything at once get on the localizer established and configured

before reaching the marker. Predecide the DH, and MDA/Time for localizer approach as well as the missed approach procedure and initial heading.  Remember, the FAF is at the intersection of the initial approach altitude and the glide slope.  Set power and pitch configuration to fly the localizer descent.

The final approach can be a handsoff affair stabilized approach

if you have done the preliminatries correctly and know your airplane.

On the localizer limit initial heading changes to five degrees.  I

usually put in the five and then take off half of it  shortly afterwards

since the tendency is to overreact.  Make your adjustments according to

your rates of descent and the heading indicator.  Once stabilized, make

glide slope adjustments by elevator and headings with rudder.  Do notoverreact to apparent large changes in the needles when close in.  A large change close in is actually a small change.  Remember allow zero tolerance below the glide slope.  You will be high over the threshold but still have plenty of time to land.

# Downwind ILS

A tailwind will have the effect of increasing your ground speed over your indicated air speed.  You must adjust power or drag (Try full flaps to keep engine warm) to obtain a more steep approach than normal.  You must change your numbers.  The rate of descent will exceed 500 fpm.  Time must be shortened.  DH remains the same but a VDP should be moved away from the threshold.  If there is water on the runway get slow early so as not to waste runway.

# ILS Failure

Be aware that ILS needles can show "perfect" approach when electrically shorted and unable to show "OFF" flags.  Wise to get back-up information when things are "perfect".

# Catching ILS Malfunctions

- Check approach plate for date, name, and critical data.

- Audio ident LOC during approach

- Call critical altitudes on approach

- Call marker crossing altitude and compare altimeter

- Set GPS as backup

- Monitor rate of descent to plate.

- Standard descent rate is ground speed divided by 2 plus a zero.

- Glide Slope Failure

- ATC can clear a pilot for the ILS with inoperative components.  ATC will tell the pilot if the glide slope is out of service.  It is up to the pilot to determine if a non precision approach can be flown.  If other ILS components are inoperative the ILS may need to be flown at higher minimums.

When on an ILS approach you have doubts as to your instrument indications, execute a missed approach.  get a safe altitude from ATC and sort out the problem.

Was on an ILS approach where ATC had held us over a thousand feet high.  Autopilot locked on to false glide slope.  I was safety pilot and

could see point of outer marker.  There was no way the autopilot was going to make the intercept at the proper altitude.  Interestingly, the

instruments gave no such indication.  Only a study of the plate and use

of DME could have revealed the problem.  At his point pilot was happy

with the way the glide slope and localizer were tracking.  I should not

have jumped on the problem.  It would have been a much better lesson to

have the outer marker crossed a thousand or more feet high.  If that ever happens to you, execute the missed but don’t turn until the time runs out.

# Emergency

Commit to memory as much as possible.

One bar will give minimum sink flaps up.

Use of flaps will increase vertical sink

Open doors

most ILS accidents (20-30 per year) seem to occur within a mile or less of the runway but 1/3  crash on the runway.  Over half of the accidents occur at night when only 1/4 of the ILS approaches are made.  Transitioning to the visual is the most demanding and dangerous part of an ILS but even more so at night.  There is no margin for error in an ILS.

Runway accidents seems to be related to contact on slick runways and higher than normal speed.  1/6 of ILS accidents occur while making second or third approaches.  An ATC warning of course or altitude is sufficient notice to begin the missed.  Flights on the ILS to an airport known to be below minimums should be flown to DH for practice only with a planned missed.  Don't fly a no-approach light ILS at night.

# ILS Test:

Fly last 200' of descent on an ILS with all instruments covered except for the AI.

# Non-precision Approaches

The purpose of the nonprecision approach is not to put you on the runway.  Its purpose is to get you down out of the clouds and in sight of the runway.  The pilot is expected to navigate to the missed approach point.  Only if clear of the clouds will he will be allowed to maneuver visually to the runway.  MDA are usually below standard pattern altitudes and will require a higher level of maneuvering skill.  Practice your short approaches in better conditions.

Localizer types of non-precision approaches are the localizer, LDA and SDA.  Other nonprecision approaches are the VOR, and the Airport surveillance radar approach (ASR).  ASR approaches require a published plate.  A given approach may be entered as (1)a straight in approach or (2) as a full approach.

# The Best of a Bad Approach

Preparation can improve the precision of a non-precision approach.

Letter designators such as VOR-A or NDB-B  or if the descent angle is steeper than normal are indicative that only a circling approach is authorized.   If making a non-precision approach that has a steep approach angle you had best configure the flaps to get the desired rate of descent.  Any descent rate over 700 fpm is not in the normal range.

Clearance for the approach should include distance to FAF and allow you to descend to intermediate segment altitude.  If you feel that you are being vectored in at too high an altitude or too close word your request according to your needs.  Watch out for dumb mistakes.  Use the checklist.  Don't descend until you know where you are.  Know the visual descent point and pre-plan one of your own.  (Use the first two digits of the MDA as seconds subtracted from total approach time to set time reaching VDP for normal landing approach.)

Essential to any nonprecision approach is that the glide path be stabilized.  A stabilized approach means that you have several constants

established.  Power is a constant.  Trim is a constant.  Airspeed is a constant.  Altitude is a constant.  These constants are maintained by constant reference to the instruments and a constant light touch on the controls.  The pilot knows the power, attitude, trim, attitude and airspeed required for any pitch change to another.   All changes are pre-planned and smoothly accomplished.  This is practiced and perfected by practice and repetition in VFR conditions.

# Straight-in Approach

The tower is not responsible for any traffic separation until that traffic is on the ground but arriving traffic must be far enough out to allow surface traffic to clear the runway before touchdown.  There is no ATC separation by the tower while you are in the air even though you may be talking to the tower.  There is a hazard associated with the straight-in approach.  Aircraft viewed directly from the rear are difficult to see since there is no relative motion.  Motion is the first thing the eye sees.  An airplane both in front of you and lower is even

more difficult to detect.  Use the radio.

The straight in approach is so called because it does not use a course reversal to become established on final course.  Radar vectors can be used to bring you so as to intercept the final approach course.  A no procedure turn (NoPT) transition is possible when the feeder route has both direction and altitude for an easy intercept and descent.  An approved transition must have a minimum altitude, distance and heading.  Approaching from a hold is allowed when the inbound leg is aligned with the final approach course.  May be prohibited by note.  The DME arc is a curve route to intercept the final approach course.  NoPT is required but DME is.

Straight-in minimums are not published or available when the final approach course is not withing 30 degrees of the runway centerline OR a nomal rate of descent cannot be used from the MDA to the runway.  When the approach plate has a number as part of its title then the runway is

within 30 degrees and the straight-in is authorized and the minimums are published.  Any time you fly such an approach with a crosswind component

corrected by your heading, the airport will not be over the nose of the airplane.

At many airports the approach to the runway on the chart may require either a downwind landing or a circling approach.  Lower minimums will apply to the downwind landing.  You will need to adjust your airspeed to get the desired ground speed caused by the tailwind.  If the tailwind is only 10 kts it will double your landing distance.

# Localizer Approaches

6 degree course accuracy to threshold.  OBS should be set to approach course as a reference heading only.(disconnected).  Backcourse ILS is nonprecision localizer approach.  Localizer directional approach (LDA) flies like a localizer approach but can bring you in to runway from 30 degree angle.  Some LDAs have glide slope.  Simplified directional facility (SDF) approaches uses poor quality antenna with course width of 6 to 12 degrees.

- Use approach chart to obtain reference descent rate in fpm.

- Bracket this rate to get and keep correct slope.

- Your primary pitch instrument for straight and level at the MDA is the altimeter (the one with the numbers).

- A clearance to the ILS approach is likewise a clearance for the localizer but with higher minimums.

- An ILS/localizer approach has two FAF's only one of which is timed.

- The interception point of altitude with glide slope is the FAF for the ILS.

- You cannot fly the Localizer if you have flown the ILS to below the MDA for the localizer.

# Checking Localizer

A localizer can be checked for accuracy by departing a runway served by that localizer.  Tune to LOC frequency while taxiing and note CDI movement as you move on to runway.  CDI should center as you line up on runway centerline.  This will not work at CCR 19R except at the very end of the runway due to offset.

# VOR Approaches

Item

VORs with HIWAS do not have two-way communications.

On a VOR approach, the VOR can be located at:

- Airport

- Missed approach point

- Initial approach fix

- Final approach fix or

- Miles from the airport.

Off airport facility

The course is aimed at the airport reference point.

On airport navaid

The final course is aimed at the navaid but will intercept 			the runway centerline optimally at 3000' from threshold for 		circling.  Straight-in is aimed at the threshold.

VOR is four times laterally less accurate than an ILS localizer.  This means that every 1 degree of a VOR has a built in it an error.  The error is easily computed by taking the distance from the VOR in nautical miles and adding two zeros.

Consider that a VOR is allowed up to six degrees of error (airborne) to either side of a course line.  Take a VOR distance such as CCR at 3.1 miles from the airport.  This is about 300' error per degree.  A full 6 degree error would amount to missing the end of the runway by 1800'.  This does not include possible pilot error such as a 1-2 degree error in setting the OBS.

Pilots should never fly anything other than the course radial on the OBS.  You should look to left and right of your course for the airport.  How far to each side you look would depend on the distance between the airport and the VOR.  You never track an incorrect radial in anticipation of possible equipment error.  Use Airmans A/F Directory to determine useable directions and altitudes of all navaids.  CCR VOR can fly you into Mt. Diablo from the southeast.

Use ADF "0" position to set altitudes assigned by ATC.

# Intercepting a Radial

Along an airway a 45 degree intercept

Approach use 30 degrees

10 degrees maximum on the ILS.

Use rudder for 5 degree corrections

In some instances a greater than 30-degree arrival may not require a hold.  The FAA may design an approach where the hold is so removed from the final approach fix (5 nm) that sufficient time for course alignment exists.

# VOR Checks

There are 6 ways a VOR can be checked for errors:

- VOT

- Ground check point

- Dual VOR check

- Radio shop check

- Designated airborne check

- "Homemade" check

# ADF Features

The NDB approach is designed to have protected minimum obstacle clearance is 300 feet vertical and 1.25 miles wide at the facility.  A formula is used to determine the protected space as the missed approach point which amounts to 6000' wide per mile.  You have 30 degrees angular space, 15 degrees to each side, to miss the runway and still have protected airspace.  The PTS standards allow only 10 degrees bearing error to the approach course.

Low frequency radio 200-1600 KHz with both loop and sense antennas.  Not limited to line-of-sight but by power.  HH power good for 200 miles down to compass locator or locator outer marker (LOM)at 15 miles.

The ADF course is the intended magnetic flight direction.  When ATC directs you to fly with reference to an NDB they will always give a bearing from  or to the station.  The to or from may be either way by ATC while meaning the same thing.  If you are in doubt as to ATC intentions reverse the term and direction used by ATC and ask for confirmation.

I usually teach the NDB/ADF procedure with the azimuth card fixed at "0".  Using this method the magnetic bearing can be read by mentally superimposing the ADF needle on to the heading indicator.  The FAA written test uses problems that require use of the formula MB=RB+MHs.  This means that to find the magnetic direction to the NDB to add the relative bearing to your magnetic heading.  The math involved in this procedure when flying can be avoided by rotating the azimuth card.  ADF needles always move into the wind.  When holding an approach heading the needle will point toward the course line.

Rotating the azimuth card makes the ADF into a pseudo Radio Magnetic Indicator (RMI) without the remote compass.  In the RMI the azimuth card rotates automatically.  With the ADF you must set the card to the desired magnetic heading.  The heading selected in the course line + the predicted wind angle.  You are trying to get the tail of the needle to indicate the approach course line.  When the tail of the needle indicates to the right of the course line you will turn to the right for about 30 seconds before returning to the WCA heading.  Works the same if the tail moves to the left.  Once you have turned the card it is best to leave it alone unless you have grossly misjudged the WCA.

# Emergency ADF use:

If you lose your AI, HI, etc. and need to fly a particular heading, you can use the ADF as a very useful and reliable aid.  Tune to the most powerful station available.  With that as a guide you will be able to fly straight and make turns by the required number of degrees.

# NDB Cold approach

The NDB is the easiest 'cold approach' to make.

THE COLD APPROACH (no opportunity to use the 6 p's)

- Frequencies at top of chart

- Locate VOR's relative to approach

- Check/recheck OBS settings

- Write time and MDA/DH on hand

- The side of the published procedure turn is mandatory

- A published teardrop procedure is mandatory

- The completion distance from the fix is mandatory

- It is TOO LATE to study the plate inside the FAF

You should know that the NDB power and range are in the Airport/

Facility Directory.  The ADF needle always points to the station and the tail of the needle give the aircraft position.  To get your magnetic bearing to an NDB you must use your imagination to put the ADF needle over the HI.  Changing the ADF card makes the process more difficult, leave it on zero.  Base all wind correntions on the point of the ADF needle.  The arrowhead will point the direction the wind is going.

# Visual Procedures:

A visual approach allows an aircraft on an IFR flight plan to fly in VFR conditions under ATC control to an airport.  Clearance requires VFR conditions at airport and enroute, airport in sight and any traffic in sight.

You are not on an instrument approach procedure even though you are adhering to instrument flight rules.  If there is any doubt as to the success of a visual procedure, you should remain IFR.  Both visual and contact approaches must be flown in VMC.  A visual approach can be flown to any airport if ceiling and visibility are above minimums of 500' above minimum vectoring which is always at least 1500 AGL.  Though similar, the visual and contact approach are not interchangeable.

Either ATC or pilot can initiate visual approach when you can tell ATC you have the airport in sight or if  you have preceding traffic in sight and will maintain separation and wake avoidance.  If you see the airport but not the traffic ATC must provide separation.  Radar service is terminated when you change to tower frequency.  The visual approach clearance does not include missed approach instructions.  Missed approaches are handled as a go-around.

A visual approach requires a 1000’ceiling and 3-mile visibility.  What you do visually depends on the ATC clearance.  If you are cleared for a

named and charted visual approach, you must fly the charted procedure.  If your clearance is not named and charted you can fly VFR to get where you are going.

To conduct a visual approach you must be on an IFR flight plan and have airport or traffic in sight with weather at least 1000 and 3.  ATC can give clearance at an airport without reported weather if 1000 and 3 is expected.  Visual approaches are allowed even if vectoring weather minima do not exist as long as 1000 and 3 do exist.  If there is no weather observer the radar controller must be told that you are VFR before he can give you a visual approach.  Once you have accepted a visual approach, you are responsible for all traffic separation.

Imagine that the airport/approach is a beautiful woman. You can have visual

any time you want, but you've got to request contact.

# Visual approach

- ATC or pilot initiation

- Ceiling 1500' or better or pilot must be able to maintain VFR.

- Pilot maintains separation and wake avoidance

# Contact Approach

- The contact approach can only be asked for by the pilot.

- Minimum conditions are 1 mile visibility and clear of clouds.

- ATC does not like the contact approach since its safety is directly related to how familiar the pilot is with the area.

- ATC will never clear a contact approach aircraft below the local minimum safe altitude.

- Requested by pilot

- Clear of clouds and

- 1 mile visibility

- Pilot responsible for

- obstacle clearance

ATC:

- Determines airport visibility

- Gives alternate (missed) instructions

- Provides separation until

- handed off to tower

Comparison

Contact Approach				 Visual Approach

Only pilot initiated               Pilot or ATC initiated

Airport required to have           Vectors possible

instrument procedure charts	   Basic VFR minimums exist and

Clearance must include missed		can be maintained

approach instructions

ATC has 1 mile at airport

ATC radar separation

Weather minimums				Weather minimums

SVFR minimums	for pilot and        1500' and 3 miles

ATC

Pilot responsibility			Pilot responsibility

Pilot obstacle clearance             Maintain basic VFR

When cleared to follow

Advantages

Enables file IFR to 								    airport without weather 								    reporting.

Area weather must forecast

VFR. (New night VFR)

Limitations                         Limitations

Only at airport with	      	    Clearance limit to nearby 	instrument procedure.			    intersection

Airport reporting	statute

mile ground visibility.

Not to be used to transition

to another airport.

The release of an aircraft to tower freqency ends radar service.  There have been many instances where the has been verbal confusion between the pilot and ATC.  ATC asks you to have runway in sight, you respond with,

“airport in sight”.  there is a difference and the difference has resulted in many wrong-runway landings as well as wrong airport landings.  Additionally ATC records show that pilots are often mistaken when reporting, “Have traffic”.

# Approach by Vectors

Use P-position

T-turn

A-altitude

C-clearance

Do not repeat everything. ...cleared for approach is enough.

Important item:  Cleared for approach is not, repeat not, a clearance to descend below any altitude shown on the plate.

- ATC position given is only informational and need not be repeated.

- ATC turn heading is advisory and need not be repeated or even 			followed if pilot judgment suggest a better alternative.

- ATC altitude is usually present altitude which need not be 				repeated.  An altitude restriction should be repeated.

- ATC reporting requirement should be acknowledged.

Primary duty of pilot is to fly approach and acknowledge the clearance with any restrictions or requirements given by ATC.

Clearance usually gives a fix, crossing altitude, and approach 		clearance.  Repeat nothing acknowledge only that you have the 		clearance.

# Charted Flight Visual Procedures (CFVP)

Charted flight visual procedures are used for noise abatement in radar/VFR situations.  original intent was to speed up arrivals.  It is not an instrument nor a visual approach and does not have a missed approach segment.  Not to be logged as an approach.  Missed approaches are VFR go-arounds.  Determining factor is visibility

Pilot responsibilities:

- Acceptance of visual clearance makes pilot responsible.

- Acknowledgment of traffic makes pilot responsible for separation and            	turbulence avoidance.

- If unable to land you must remain VFR until obtaining additional    	clearance.

- Must advise ATC if unable to follow

ATC responsibilities

- ATC will clear when pilot has airport or aircraft to follow in sight.

- Minimums are 1500' and three miles visibility with the ceiling 500' 	above minimum vectoring altitude (1500')

- Radar service canceled when you change to tower(advisory) frequency

# PAR Approach (obsolete)

This approach is a highly-skilled, labor intensive system being shut down by the military.  It provides glideslope and azimuth direction where all turns and descents are at the direction of the radar controller. On being given the instruction:

"All turns will be standard rate until established on the final approach.  Then they will be made half-standard rate.  If radio contact is lost for more than seconds, contact tower on (frequency) and execute the prescribed missed-approach procedure."

Every few seconds or so the final controller will issue course and glidepath corrections leading to the runway threshold.  I was talked down to within 10 feet of the runway at Travis on a PAR twenty years ago.  Great feeling.  Once on the ground you may need taxi assistance.

On such approaches the controller would need to know when you had acquired visual references such as lights so that you could be cleared to land visually or to execute the missed approach.

# Airport Surveillance Radar Approach (ASR)

This approach has replaced the PAR in most places with radar.  Controllers are requires to practice these approaches and may be more than willing to give you a practice approach.  The procedure requires that the airport have the required radar capable of giving heading information.  ASR altitudes are recommended only.  On final you will be given range to airport and stepdown altitude recommendations for each mile.  Altitudes will be higher than for standard instrument approaches.

# No-Gyro Approach

#1 Cover up the heading indicator.  #2 Keep aircraft under control.  #3 Do what you are told to do when you are told to do it.  When it is a dark and stormy night and the gyros quit the controller can give you assistance by means of the turn coordinator.  He will tell you to turn at standard or half-rate turns to the right or left and tell you when to stop turn.  An ASR approach requires that the radar range be reduced which effectively blows-up the picture size for the controller.  Usually an entire scope and controller is dedicated for an ASR approach.  There will be no handoff to the tower.  The radar controller handles the landing clearance, rollout, and advises when to contact ground.  A missed approach will continue with the radar advisories.