# Click on page number for instant scrolling.

Holdingã

The Way Holding Should Be

Sayings:

Holding

The Fix

Timing

Holding Airspace

Directions

Sum-of-the-digits.

The Pilot Must Know:

Radar/Radio Surveillance

Situational Awareness

Wind

Getting Reciprocals

NDB Holding

Entries

NDB Holding Simplified

FAF Using NDB

Radios

Changing the OBS

Basic Components.

Holding Directions

More Pattern Analysis

How to Study

Direct Entries

Non-direct Entries

Simple Parallels

Y-Intersections

Changing Frequencies and OBS

Using the Heading Indicator to Hold

Simple Holding

Thinking Through the Hold

Off Airways

Single VOR Holding

DME Holding

The Clearance

Clearance Types

Standards

Course reversal holding and procedure turns:

Approach Holds

Clearance Simulation

Holding Peculiarities

Bay Area Intersections

Exercise II

Intersections and Airways

# The Way Holding Should Be

## Sayings:

Climbing outbound on a procedures inbound radial is a sure way of 			meeting someone.

Prepare your departure and arrival strategies ahead of time.

Desire to be good is required to be good.

An accident occurs when you have exhausted your box of options.

Getting away with something stupid is a learning experience, not an 	invitation to try again.

# Holding

Under the revised FARs you are now required to log holds every 6 monts for IfR currency.  You are expected to use a procedure that will keep you within

the protected airspace of the holding pattern.  The entry guaranteed to keep

you in the airspace is the course reversal.  Both the parallel and teardrop entries can be substituted by the course reversal.  First you slow down.

You much remember second  to just turn to the outbound heading over the fix, perform the course reversal to the left for right standards holding patterns and to the right for non-standard left patterns.  Draw it out, if you must.

Go for tripling the wind correction angle while outbound but maintain your

standard rate bank angles.  Ask for two, three minute legs to give you more

time to study your plates.

For the Practical Test Standards (PTS) you will need to know but not use the FAA recommended entries.  An applicant who uses other than the recommended procedure must remain within the holding pattern protected airspace.  Reference: AFS-600 Designee Update Vol 6, No. 2. April 1994.  After you have your instrument rating it doesn't make any difference how you do a hold after you get your instrument rating, as long as you remain in protected airspace.  The FAA is de-emphasizing the 70/110 method of determining holding pattern entries.

Aircraft holding patterns are a way of parking or delaying an aircraft along a route much as a railroad might use a railway siding.  It may be used as a descent maneuver which avoids terrain or as an altitude stacking procedure to align aircraft as might be required in non-radar IFR situations.  Holding is done for the convenience of ATC.  Do not let their convenience jeopardize the FAR mandated fuel requirements.  Be prepared to give minimum fuel advisories or to declare an emergency. Holding patters are used for traffic separation enroute, for sequencing at terminals and as part of the approach procedure.  The hold provides protected airspace.

Flight instructors are requires to teach the recommended holding pattern entries defined in the Instrument Flying Handbook and the AIM.  This is so even though other entry procedures may enable the aircraft to enter the holding pattern and remain in protected airspace.

Entering holding patterns with excess speed increases the probability that you will fly out of protected airspace.  the examiner will not accept reduction of speed sooner than three minutes before reaching the fix.  You can get ATC approval if you wish to slow up sooner, however.

You may use a non-standard entry if it keeps you within protected airspace.

Examiners may question you about the standard procedures if you choose to

fly a non-standard pattern.

# The Fix

The hold is a race track pattern  which, in no-wind conditions, is

made up of one minute straight legs and one minute turns at each end.  At one 'corner' of the pattern is a FIX.  A FIX may be the airway intersection of two VOR radials, a VOR, an NDB, by DME along an airway radial or by direction. The pattern is normally flown at an economic fuel saving speed.  If you are being flown toward a holding pattern by ATC, make a request for a lower (endurance) speed enroute.  This lower speed may eliminate the holding requirement. Always plan to slow down before reaching the fix.  Sooner is better than too late.  The turns are not normally timed  since a standard rate turn is used and may be more or less than 180 degrees for windage.

Once established in the holding pattern at an intersection fix the needles will be centered at the fix, both needles will be off on the outbound leg and one centered on the inbound leg.  At a VOR the fix is reached at the moment of TO/FROM reversal.  The needle is off to one side on the  outbound leg.  NDB holding will be covered later. The first big step to holding is aircraft control. You must have your power/trim settings for the aircraft so locked in that neither airspeed nor altitude become distractions or added work load.

Historically, the name of every intersection made some  reference to omething near that location.  Originally the  words were spelled out.  With the advent of computers all intersections consist of five letters which may phonetically resemble the original.  LODI became LODDI, RIO VISTA became REJOY and VISTA.  Only an active imagination or an old memory bank makes possible many of the associations.   VORs have only three letter designators.

Early attitude indicator gyros were quite subject to precession if a continuous turn were made.  To prevent this as a problem holding patterns were designed with one-minute straight legs.  Hence, the holding pattern as we know it.

In the real world of holding where it is an every flight occurrence at certain Class B airspaces, the pilot knows just by 'situational awareness' what he must do to position the aircraft to the fix and into the holding entry.  You just know what to do and how to do it.  There is no need for diagrams, angles, or mental gymnastics.

The General Aviation pilot will do more holds getting the IFR rating that he will ever do the rest of his flying life.  It is because of this lack of use that the pilot must have an easy, available method for interpreting the clearance and determining the holding procedure.

Information provided by ATC:

- The direction from the fix is given (confusion isssue) including the  radial or bearing to hold.  This holding direction is the side to hold on but is not the course for the inbound direction.

- Know when you arrive at the fix and make your outbound turn within six seconds.

- One minute holds are made up to and including 14,000’.  Know that the true airspeed over indicated airspeed increases with altitude.

- Using a DME hold requires that you change your time for a leg into a distance for a leg.  Thus, at 90 knots a leg would be 1.5 miles for one minute.  Using DME you would turn after 1.5 outbound regardless of time.

- Expect further clearance (EFC) is required infomration for a hold.  If ATC fails to give it, ask for it.

- You are expected to used all available resources.

# Timing

You start timing the outbound leg of intersection, VOR, and DME holds when you are on the selected outbound heading as corrected for wind.  The traditional practice has been to double the outbound wind correction angle on the outbound heading.  As of January 6, 1995, the AIM  uses the word triple' instead of double.  At an NDB or if you can establish when abeam a VOR or a fix start your time when abeam.  VOR: Outbound timing starts when to/from indicator reverses as you pass abeam the VOR.

Intersection:  Outbound timing starts at completion of outbound turn since magnetic bearing cannot be determined.

NDB:  Outbound timing starts when ADF relative bearing is 90-degrees minus drift correction angle.

Subject: Re: E6-B Flight Computer

Correct description is:

Set the actual outbound time, on the outer ring, opposite the actual

inbound time, on the inner ring. Now read off the correct outbound time

from the outer ring opposite the 60 second arrow on the inner ring.

So, for example, if your first outbound leg is 60 seconds, and your

inbound leg after that is 45 seconds (because you have a tailwind

inbound), you'd set 60 seconds on the outer ring opposite :45 on the

inner ring, then read off 1:20 on the outer ring opposite the 60 second

arrow on the inner.

So, let's say you fly the next outbound leg for 1:20, and on the inbound

you still only take :50. Shift the ring so that :50 is opposite the 1:20

(that used to be next to the 60 second arrow); you should now see 1:36

opposite the 60 second arrow, so that's what you should fly the next

outbound leg. All it is doing is 60 * (outbound / inbound).

# Holding Airspace

The space available for the hold is fairly standardized both as to length and size to each side of the inbound course.  There is a primary area, maneuvering zone, and secondary area.  The overall shape is like that of an elongated avocado cut in half lengthwise.  The fix is on the course line.  One nm away, at right angles to the course at the fix, is the center (nameless) of the radii for the small end of the avocado. A five nm 180 degree area of arc forms the small end of the avocado.  Ten nm away on the outbound course there is another point (nameless) on the same side of the course but two nm miles from the course.  The six nm radii from this point form the big end of the avocado.  The offset from the course line of these two points gives both ends of the maneuvering zone. The course line  is thus offset from the center line of the avocado by one nm on the small end and two nm on the big end.

HHHHH

Draw an avocado.  The holding inbound course line is offset to the non-holding side one mile from the center of the arc of small end and is offset two miles to the non-holding side of the arc of the large end.  The radius of the small end arc is 5 miles plus a two-mile secondary zone which extends like a heavy skin around the avocado.  At its longest the avocado is 25 miles long.  At the large end it is 15 miles wide and at the fix about 14 miles wide.  The holding side gives us a over half an avocado that is 25 miles long, and widens from 7 miles to 10 miles on the holding side.  The non-holding side is 23 miles long and 7 miles wide.

Holding patterns are used instead of procedure turns to reduce the amount of airspace required.  The procedure turn can be any time you want as long as it is to the correct (protected) side.  Caveat: Some Category A-only require reversal within 5 miles.  It does not need to be as depicted unless it is a teardrop.  Personally, I much prefer the 90°/270° since it is quicker and reduces potential wind effect.

## Directions

They are;

NORTH (exactly 360 but ranging between 340 and 020);

NORTHEAST (exactly 045 but ranging between 020 and 070);

EAST (exactly 090 but ranging between 070 and 110); 				SOUTHEAST (EXACTLY 135 but ranging between 115 and 160); 			SOUTH (exactly 180 but ranging between 160 and 200); 				SOUTHWEST (exactly 225 but ranging 200 to 250);

WEST (exactly 270 but ranging between  250 to 290);

NORTHWEST (exactly 315 but ranging between 290 and 340)

## Sum-of-the-digits.

Notice, bold digits when added equal 9.

Notice, italicized digits when added equal 7

Notice, normal digits when added equal 2  (2+9+0 = 11; 1 + 1= 2 )

# The Pilot Must Know:

How to fly from cruise to 90 knots and back again at altitude

Right from left

That a radial is from a VOR, a bearing is TO  an NDB

The numbers and terms for major headings of the compass

How to get reciprocals quickly. (+ 2 and + 2).

Using the DG to get reciprocals, 30 and 45 degree angles

How to fly with minimum effort TO/FROM a VOR radial.

How to intercept and track a given bearing to an NDB

How to quickly tune and set radios is important.

# Radar/Radio Surveillance

ATC communications are guaranteed at MEA and expected at IAF and

at Missed Approach altitude but not necessarily on approach.

Where possible ATC is required to detect and advise any time the holding airspace is exceeded.  However, it is the pilot responsibility to maintain the hold, not ATC's.

If, when under radar, you should ever acknowledge that you have visual contact with pointed-out traffic it becomes your responsibility to both see and avoid.  ATC has effectively handed this responsibility to you until you should again tell ATC that you can no longer see the traffic.

Yes the ATC system does fail but more common are mistaken frequency assignment or a dropped handoff.  You can protect yourself by anticipating

your next frequency.  This can be done in your preflight planning, referring to your charts, using the A/FD listings, or most easily listening to the handoffs that occur in front of your flight.  The last method is best because it is the least likely to be out-of-date.  During busy and non-busy periods ATC will eliminate or combine station frequencies.  When you know which period exists during your flight you can anticipate the required frequency.

# Situational Awareness

No matter how you determine your holding procedure, your knowledge of where you are in relationship to the fix, the holding pattern, your arrival and departure routes are essential givens. The perceptual key to successful holding is to remain oriented.  Don't even think about orientation until you are headed toward the fix or on an intercept to one of its legs.

Each of the three types of holds have certain characteristics as to entry, number and direction of turns, NAV/OBS changes, and orientation difficulties.  The better they are understood the better you will stay oriented.  The different kinds of holding fixes, the multiplicity of holds, the variations of instructions all contribute to the orientation problem.  Since intersections are seldom at right angles (90 degrees) some eyeballing of the holds and turns will be required in actual  practice.  The airspace allotted for holding takes into account a considerable amount of pilot error.

If you must... ALWAYS-ALWAYS DRAW THE INBOUND SIDE AND HEADING FIRST when drawing any holding pattern.  Doing this will at least improve the chance of getting the turn direction correct.

As you go through the following study process first do the drawing.  Get away from the drawing as soon as you are able.  It is essential that you be able to mentally visualize any hold from an oral description consisting of only a fix name, a direction and the turn direction.  i.e." Hold at RAGGE, south in left turns."  or "Hold southeast at SNUPPY."  When no direction for the turns is given 'standard' is right turns.

# Wind

A requirement for any holding entry is that the pilot be able to make some predetermination of wind direction and velocity.  Your best insight will be your own inflight calculation of the wind.  Do all you can to get the wind direction and velocity while you are flying to the fix.  Your second best insight will be the FSS forecasts for your altitude.

Once you know the wind, triple your wind correction on the outbound legs for at least one minute of that leg.  (As of January 6, 1995, the AIM  uses the word 'triple' instead of double.) This is a DR skill.  Wind correcting headings are used on both the outbound (double the outbound) and inbound to

fix courses.  Wind correcting times are also used on the outbound headings to establish one minute inbound legs.

Course is the line you are trying to fly, heading is where the nose points in order to fly the course line.  Course and heading are the same only when there is no crosswind.  Since the holding side is "protected" airspace considerable allowance is made while the pilot tries to sort out the wind effect.  Three times around should get the pattern into pretty good shape.

# Getting Reciprocals

At this point is important for you, the pilot, to know how to get your reciprocals quickly and accurately.  It is wise to memorize the eight

cardinal headings and their reciprocals.  The difficulty of adding or subtracting 180 can be overcome by several methods. Always say course, headings, bearings, or radials as consisting of three digits.  You should have started this way as a student pilot.

Easy way #1

Just set the original number at the bottom of the OBS dial.

Easier way #2

Add 200 subtract 20/subtract 200 add 20.

Since you need to either add or subtract 180 degrees

To a number less than 180 add two hundred and subtract twenty

i.e. 045 + 200 = 245 - 20 = 225

To a number more than 180 subtract 200 and add twenty.

i.e. 266 - 200 = 66 + 20 = 086

Easiest way #3

Since every number on the compass consists of 3 digits, just add or     	   subtract two from the first two digits. The third digit remains the    	   same.

Course = 0 4 5 		Course 2 2 5

+2-2__			-2+2__

becomes  2 2 5 			 0 4 5

Easiest: Take two from first digit and add it to second

digit; take two from second digit and add to first digit.

Another way:  The last digit of reciprocals always remains the same.

The middle number always increases or decreases by eight.  There are only

four different initial digits.  First write down the last digit.  To find the

first digit you must know how the numbers on opposite quadrants of the compass rose change.  The first digit threes are in the opposite quadrant from the ones.  The zeros are opposite the twos.  It works but might not be that easy under pressure.

Knowing the sum of the digits for all 90, 180, and 270 points from a given heading are equal.  For example, 135 degrees adding the digits equals 9.  The 180 degree reciprocal is 315 which also equals 9.  The 90/270 degrees are 225 and 045 and the sums of the digits equal 9.  This will work for any compass points including 45 degree intervals, 22.5 degree intervals and 11.25 degree intervals.  Occasionally, during a turn you may have failed to determine a 90 or 180 degree heading point.  The above sum of the digits knowledge may save the turn.  Likewise, you can just reference the initial heading in an appropriate 90 or 180 position on the HI.

Mark all published holding patterns on plates and area charts with a diagonal line having arrows and numbers for headings in both directions.

Highlight important frequencies, directions, times, and altitudes.  Be aware that once you have learned the ‘approved’ FAA method and obtained your certificate you may want to use only the ‘course reversal’ method explained later.

# NDB Holding

A holding fix at an NDB or equivalent is usually flown direct to the NDB using the ADF.  If a specific bearing for arrival at the NDB fix is given then an intercept procedure is required.  Flight using the ADF requires that very accurate headings be flown.  An accurate heading makes it possible to determine the effect of any wind.  Flight into a crosswind will be tracking accurately on course if the heading is constant while the needle likewise remains  constant even though not on '0'.  Loran and GPS makes tracking so easy that the FAA will not allow it.

In many respects holding with an ADF is easier than with a VOR fix of any kind.  With the ADF you always know where the fix is.  You can always

fly right to the fix even if by a circuitous route due to winds.  Once at the fix you determine the bearing on which ATC has determined the holding direction and if non-standard turns.  No mental reversals of inbound/outbound courses are required.  No frequency changes.  Just get there and do it.

#8 is to left

side of entry and makes right turns in pattern.

Entries

#1 is to right

side of entry

and makes left

turns in pattern

Teardrop Teardrop

8	   1

R-turns  L-turns

#6 is to left side of entry and makes right turns in pattern

Direct	7			Direct	2

L-turns			   R-turns

#3 is to right

side of entry and makes left turns in pattern

Parallel			  Parallel

6				3

R-turns			  L-turns

#5 and #7 are to the left of

entry and make left turns	  5	   4	  #2 and #4 are to the right of

in the pattern					   entry and make right turns in

the pattern

Entry  Direction

Using a clockwise eight count for each pattern from 12 o’clock we find that only #4 and #5 give you instant inbound entry into the pattern. #1 and #8 require that you turn to depart the fix at a 30 degree angle  from the outbound direction for one minute before tracking back to the fix.  #3 and #6 allow you to track outbound on the inbound bearing before turning back to the fix through the holding pattern.  #2 and #7  Require that you fly outbound on a heading until the ADF needle point tracks to the 30 degree point.  This should take one minute.

Once you have tracked back to the fix you are established in the hold and should begin turns in the assigned direction.  Your outbound leg is made on a heading designed to correct for the wind as guessimated or forecast.  The preferred procedure is to determine the inbound correction required. Once the inbound correction has been approximated be sure to triple the angle of correction of correction on the outbound leg.  (As of January 6, 1995, the AIM  uses the word 'triple' instead of double.)  This will make the holding pattern egg shaped as required by the wind.

One of the beauties of the ADF is that it is relatively easy to determine when to begin timing the outbound leg.  When the Fix is at the 90 degree point, left or right, start your time.  In no wind conditions the needle will be at the 30 degree rear position at the completion of the one minute outbound leg.  The + effect of the wind will change the angle slightly as well as the outbound time required to set the inbound leg time to one minute.

Fly a NDB holding pattern in calm conditions and note how the needle will be at 90/270 as when to begin timing the outbound course.  Note how needle will tract to your rear, left or right, 30 degree position when you have reached your time for beginning your inbound turn. Next try this on a windy day and note how the needle is not nearly as much help in either determining your timing points or wind correction angle.

# NDB Holding Simplified

In the "real world" bearings are seldom flown exactly.  If the NDB can be located on a LORAN or GPS such flight is very easy.  (Flying an NDB approach to an airport becomes a very accurate affair if the airport location is put into the LORAN or GPS.)

When holding at NDB the first inbound turn is flown direct to the NDB.  The heading required to fly direct gives some indication of wind correction required.  Make a guess as to the amount of wind correction required.  Triple this correction angle into the wind on the outbound leg. When you come out of the turn to your inbound heading if the ADF needle is '0' then your wind correction is correct.  Change your inbound heading to correct for the wind again.  Make any fine adjustment required.  Repeat the procedure as necessary

to correct for wind and time.

# FAF Using NDB

Identify fixes using an NDB bearing is a check-ride Catch 22.  There are FAFs that use a wing tip bearing for identification.

# Radios

Regarding Frequency changes, turns, and OBS settings, there is a pattern for you to learn.  The NDB hold requires no changes of any kind.   The VOR hold's only change is required after crossing to set the inbound (reciprocal) of the assigned radial on the OBS.  Direct entries #4 & #5 require no changes.  Teardrops #1 and #8 require only an OBS reversal on the #1 OBS.  Parallels holds # 3  & #6 and direct entries #2 & 7 require a complete change in frequencies and OBS settings.  Each hold requires a sequence of changes.  There is a pattern to the changes in frequency and OBS settings required for each hold. By working through each hold slowly you can learn the appropriate pattern.  I would suggest that you memorize and practice making frequency/OBS changes on the ground so that every knob will be turned initially in the right direction and the right amount.  Anything you can do to lighten your load under single pilot IFR should be practiced.

For the intersection hold the interception settings of the two VORs are maintained until reaching the intersection and making the initial turn.  It is desirable to set all NAV/OBS on the outbound leg and  reset/check them on the inbound leg.

When flying to a holding fix, marked by an intercepting VOR radial, don't wait for more than a one dot movement of your intercept needle before slowing

down.  Set up a timing intercept radial for NDB and VOR fixes as Poorman's DME. (See instructor)

One of the ways to better plan holding is to develop a course/ VOR #1-OBS/VOR and a #2 VOR/OBS chart.  Chart should Specify intercept heading, course inbound, and all headings after turns.  The word time or the letter T should be written under all headings that require timing.  The #1 VOR should always be the one being tracked to  or from.  The #2 should always be the interception one.  Although it may seem easier in some patterns not to do this it can lead to complete disorientation under stress situations.

Remember that on outbound legs while established in the holding pattern the needles will not be centered.  The degrees of turn inbound to intercept the inbound leg of the parallel hold may be increased /slowed to allow easier interception and tracking to the fix.

# Changing the OBS

Once you have a good command of how to enter the hold, practice using the OBS.  Try to turn the shortest distance.  Some settings are easy to misplace.  If a setting is  039 set 040 and back off a fraction.  Watch out for parallax in your view of the numbers and setting.  Be aware than some older dials have the OBS setting on the bottom rather than on top.  If you have trouble getting reciprocals, just put your number on the bottom.  If the number you want on the OBS is to your right, turn the OBS knob to the right.  One full twist between the finger and thumb will approximate 90 degrees. Practice setting changes required for every hold until you can make required changes in less than 5 seconds.

# Basic Components.

There are three different holding pattern entries

direct

teardrop

parallel

There are two different holding directions

right (standard) need not be assigned or published

left - must be assigned or published

We will always be flying toward the fix.  Every hold will be to the right or left of the course to the fix.  Turns are either standard or specified (left).  The major variable is in the instructions for the hold.

Frequently the holding direction makes a second approach clumsy.  An experienced controller will reverse the holding direction if he knows ahead of time that you plan to do multiple approaches.

# Holding Directions

Every four way intersection has eight holds.

Four are direct entries.

Of the four direct entries the two to your left are in 		left turns and the two to your right are in right turns.  Note

pattern:  Left/left and right/right.  See, diagram above.

Two are teardrop.  Two are parallel

The teardrop and parallel are ambidextrous; the patterns 			to your right are in left turns, those to your left are 			in right turns.  Pattern:  Left/right and right/left.

The teardrop entries are both across the fix extending in your 		line of flight.  The parallel entries extend to the left and 		right on the approaching side of the fix.

Diagramming the Hold

Use a + shape to denote the fix.  Let the top of the + be the direction of flight.  You are arriving from the bottom of the + and flying toward the top of the +.  You always fly to the holding fix.  The fix is the middle of the +.  Draw a + and put in the two teardrop holds. They will always be to each side of the top vertical.  Draw another + fix and put in the two parallel holds.  They will always be below the horizontal and extending to each side.  Draw a third + fix and put in the four direct entries.  Two will be to each side of the bottom vertical.  Two will be above the horizontal and extending to each side.

Number the holds of a four inch  +  from one to eight clockwise, beginning at the top right.  Now draw  small R-arrows and L-arrows along the patterns on the outbound legs to confirm turn direction.  Begin drawing every series of arrows on the inbound leg of the holding pattern.  You should be able to detect a  pattern.  There is a relationship between the location of the fix and the turn direction.  Look for it.

# More Pattern Analysis

4 are direct entries

Two holds are in right turns; both are to the right of your course

Hold #2  extends like a wing to your right.  To enter you fly across the intersection turn right 90o fly for one minute, make a 180o right turn and fly inbound to the fix. Continue in right turns.

Hold #4  extends like a banana along your route of entry toward the fix.  You begin making a right 180o turn and fly back the way you came for one minute before turning right toward the fix.  No NAV/OBS changes required.

Two holds are in left turns; both are to the left of your course.

Hold #5  extends like a banana toward you. At the fix you begin making a left 180o and fly back the way you came for one minute before turning left toward the fix.  No NAV/OBS changes required.

Hold #7  extends like a wing to your left. To enter you fly across the intersection turn left 90o fly for one minute, make a 180o left turn and fly inbound to the fix.  Continue in left turns.

2 are teardrop entries

Hold #8 in right turns; extends like a banana away from you.  It is to the left side of your arrival course. It is entered at the fix by a 30o left turn and after one minute a right turn back to the fix is followed by all right turns.

Hold #1 in left turns; extends like a banana away from you.  It is to the right side of your arrival course.  It is entered at the fix by a 30o right turn and after one minute a left turn back to the fix is followed by all left turns.

2 are parallel entries

Hold #6 in right turns; extends like a wing to your left.  It is flown by making a left 90o flying for one minute and making a left 220 back to the fix.  All subsequent turns are to the right.

Hold #3 in left turns; extends like a wing to your right.  It is flown by making a right 90o and flying for one minute and taking a right 220 back to the fix.  All subsequent turns are to the left.

# How to Study

When you are driving and arrive at a four way

intersection stop light you are looking at eight different potential

holding patterns.  The very center of the intersection is the fix.  The end of each holding pattern overlap somewhat at the fix.  For ease of visualization we will always be (flying) driving through the intersection.  Number these patterns clockwise from the top right as 1 through 8.  1 & 8 are to the north and  4 & 5 to the south.

You can actually be studying your holding patterns as you come to a stop light while driving.  Give yourself successive clearances around the eight holds as you come to each stoplight.  Go through your radio arrival

and hold changes.

Another  method of study might be to first draw the eight holding patterns usually given at a four way intersection.  Flight is always from the bottom of the page toward the top.  Draw and study the following numbers and their related patterns thoroughly before  proceeding. First locate the hold from your direction as #1. #2, #3, #4, #5, #6, #7, or #8,   Highlighted patterns are direct entry, #1 & #8 are teardrop, #3 & #6 are parallel.  Direct entries to your right are in right turns; direct entries to your left are in left turns. Any hold to your left with right turns is either parallel or teardrop; those to your right in left turns will be parallel or teardrop.  Teardrop entries #8 to your left is in right turns & #1 to your right is in left turns. They extend across the fix away from your course.  Parallel entries #3 to your right is in left turns & #6 to your left is in right turns. They extend like wings on the arrival side of the intersection.

# Direct Entries

We are going to begin with the four direct entry patterns.  Patterns 4 & 5 initially and then 2 & 7.  A direct entry  means that the pattern turns and holding pattern side relative to your heading are to the same side.  Right side, right turns; left side left turns.  Just fly to the fix and start turning.  In a direct entry, once you have flown through the fix, set up the fix on your radios.

Direct entries #4 and  #5 which extend toward you from the fix do not require any frequency or OBS changes.  The frequency/OBS changes for direct entry #2 is paired with parallel entry #3 and direct entry #7 is paired with parallel entry #6. (covered later)

#4 and #5 Direct Entries

Now draw the intersection but put only patterns 4 & 5 on

the drawing.  They should extend toward the bottom of the

page like two sausages side by side.  You are flying (drawing and then walking) directly up the center line of the street toward the fix.  This is called the inbound leg.  The pattern to your right (#4) is in right turns and

the pattern to your left (#5) is in left turns.

#4 and #5 Radio Set

No NAV/OBS changes are required whether you are flying to an NDB, VOR or intersection for patterns #4 & #5

#4 and #5 Clearance

The area of student difficulty in orienting to #4 and #5 lies in the

terminology of the instructions for the holding pattern at the fix.  The holding direction is given as opposite to your direction of flight.

You are flying north and told to hold SOUTH.  The holding radial may be given as the same or the reciprocal to your course depending on the location of the VOR (ahead or behind).

Simply:

If the holding direction is opposite to your direction of flight; the pattern extends toward you from the fix, you don't need to make any NAV/OBS changes and the entry is direct.

#2 and #7 Direct Entries

Yow draw a new intersection, you are flying toward the top of the page, and put in patterns 2 & 6. These should extend to the side like wings.  They are on the  upper side of the horizontal line.

You are flying (driving, walking) toward the fix and the patterns extend, wing like, to your right and left.  The one on the right (#2) is in right turns; the one on the left (#6) in left turns.  At the fix a series of NAV/OBS changes are necessary and will be covered later.  At the fix first turn to the outbound heading and start the time.

During your flight to the fix you should have tried to determine both direction and velocity of the wind.  It is appropriate that you adjust your initial outbound flight according to your estimate of the wind.  Otherwise fly outbound heading for one minute.

#2 and #7 Radio Set

You have this one minute to make the NAV/OBS changes.

Ample time if you have practiced the settings.  In addition, it provides the interception and tracking information required for flying toward the fix.

The #2 Nav/OBS is used to set up the interception at the fix.  Both the frequencies and the OBS settings need to be reversed between the #1 and #2 radios.  Use an intercept radial from another VOR if at all possible at a single VOR fix.  It's nice to have the use of a VOR intercept radial even at an NDB fix.

# Non-direct Entries

Of the remaining four holding patterns, two will be teardrop and two will be parallel entries.  These entries require that you get the airplane turned around going to the fix.  It is this turning around that causes pilot disorientation.  The purpose of the teardrop and parallel procedure is merely to turn the aircraft around in the FAA’s conception of the simplest manner possible.

#1 and #8 Teardrop Entries

If the ATC specified direction of the hold is the same, or nearly so, as

your course the entry is a teardrop.  These holds will be within 30 degrees of #8 & #1.

If the ATC specified turn direction is to the right then the pattern is to your left and vice versa.  The hold to the RIGHT will be in LEFT turns.  The hold to the LEFT will be in RIGHT turns.  You will fly through the

fix and diagonally through the holding pattern bisecting it at 30 degrees.  (Just as the first turn on a 45 entry to right traffic is to the left, so will your first turn into a teardrop calling for left turns be to the right and the teardrop calling for right turns be to the left.)

If you have an approach plate that frequently requires teardrop entries, it is a good idea to draw in the 30 degree angle and heading numbers on the plate.  In the teardrop clearance calling for right turns only the first 30 degree turn is to the left.  Likewise, the holding pattern calling for a teardrop in left turn has an initial turn to the right.  You time the diagonal for one minute and turn about (wind) 210 degrees to intercept the inbound leg.  The 210 degree turn and all subsequent turns will be the opposite direction of the initial 30 degree turn and in the direction specified in the clearance.

Once established on the inbound leg you are in the holding pattern.

#1 and #8 Radio Set

The VOR frequencies on #1 and #8 remain the same.  While on the outbound

diagonal the #1 OBS must be reversed  180oto the inbound course.  No changes to the #2 radio are needed.  Again, for teardrop entries, the only change will be a 180o reversal of the #1 Nav OBS.

#8 and #1 Clearance

If the holding direction is given as within 30 degrees of the reciprocal to your direction of flight, the entries will be teardrop.  The pattern to your right will be in left turns with the initial turn a 30 degree right turn.  All subsequent turns are to the left.  The pattern to your left will be in right turns with the initial turn a 30 degree left turn.  All subsequent turns are to the right.

#3 and #6 Parallel Entries

The remaining two patterns #3 & #6 are parallel entries.  Commonly called PARALLEL OUTBOUND since your first turn of 90 degrees parallels the outbound leg while actually flying the reciprocal course and radial of the inbound leg.

The parallel holding patterns extend at approximate right angles to your initial inbound course.  You must fly through the tips of these patterns to get to the fix.  On arrival at the fix you turn left if the holding turns are right or right if the holding turns are left, to parallel and intercept the outbound course.  Estimated wind correction may be applied.  Time this outbound leg for one minute as soon as established out of the turn.  At the end of the minute an additional 210 degree turn in the same initial 90 degree turn direction will give you at least a 30 degree interception back to the fix.

The first two turns from the fix of 90 and about 210 degrees will be in the same direction with all subsequent turns as part of the holding pattern in the opposite direction.  You will have turned right 90 degrees for the pattern to the right of your course.  You will, after one minute, turn right again about 210 degrees to fly diagonally through the pattern and intercept the inbound radial to the fix.   A total of about 300 degrees to your right in order to reverse the aircraft flight direction.  All subsequent turns will be to the left.

or

You will have turned left 90 degrees for the pattern to the left of your course.  You will, after one minute, turn left again about 210 degrees to fly diagonally through the holding pattern and intercept the inbound radial to the fix.  All subsequent turns will be to the right.

## Simple Parallels

If the parallel pattern is to your right, your first two turns are to the right in order to reverse your flight direction.  The pattern turns are to the left thereafter.

If the parallel pattern is to your left, your first two turns are to the left in order to reverse your flight direction.  The pattern turns are to the right thereafter.

#3 and #6 Radio set

The Nav/OBS changes for patterns #6 & 3  has four variables.  You can be tracking to or from a VOR and the interception VOR may be to your left or right.  However there are a number of constants.  The first is that the #1 and #2 navigational frequencies will be reversed.  The #1 OBS will be turned 90 degrees to the right for patterns #6 & 7 and 90o to the left for patterns #2 and #3.

There is a pattern but no easy way to set the #2 OBS for any of the four patterns.  You must know if the intercept station is going to be to your left or right.  The OBS change will always be only 90o and is the same for #6 & 7 in each situation.  The OBS change will always be only 90 degrees and is the same for # 2 & 3 in each situation.

# Y-Intersections

Y-intersections will offer variations. Six holding patterns are possible.  The direction of your arrival at the intersection gives a direct entry to at least two patterns.  The teardrop to your left will be left turns; the teardrop to your right will be right turns.  The parallels are as usual,

to the right left turns, to the left right turns. Any entry that requires more than a 90 degree turn is wrong.

# Changing Frequencies and OBS

Use a chart to simulate a flight with holds at every intersection of the route using the suggestions given.  Giving your own clearances will make you better understand ATC’s.

#1.  At the first intersection hold in the direction of your flight with a teardrop entry and left turns.  Verbalize the clearance ATC would give you to define the hold at that fix.  Making your own clearances will help you

understand them.

Talk through the frequency settings, headings and OBS changes of that hold. Make a chart for #1 and #1 frequencies and OBS settings as well as #2 frequencies and OBS settings that will be required for arrival and then for the hold.  There is a logical pattern and sequence.  Once hold is established proceed...

#2.  At the next intersection hold to your right in a direct entry and right turns.  Talk through the wording ATC might give for your clearance.  Chart the frequency settings, headings and OBS changes of that hold.  Once hold is established proceed...

#3.   At the third intersection hold again to your right in a parallel entry and left turns.  Talk through the clearance.  Chart the frequency settings, headings and OBS changes of that hold.  Once hold is established proceed.

4.   At the fourth intersection hold to your right rear in a direct entry and right turns.  Talk through the clearance.  Chart the frequency settings, headings and OBS settings of that hold.  Once hold is established proceed...

5.   At the fifth intersection hold to your left rear in a direct entry and left turns.  Talk through the clearance.  Chart frequency settings, headings and OBS settings of that hold.  Once hold is established proceed...

6.  At the sixth intersection hold to your left in a parallel entry and right turns.  Talk through the clearance.  Chart frequency settings, headings and OBS settings of that hold.  Once h old is established proceed...

7.  At the seventh intersection hold to your left in a direct entry and left turns.  Talk through the frequency settings, headings and OBS settings of that hold.  Once hold is established proceed...

8.  At the eighth intersection hold to your left in a teardrop entry and right turns.  Talk through the  clearance.  Chart frequency settings, headings and OBS settings of that hold.  Once hold is established proceed.

# Using the Heading Indicator to Hold

Procedure:

Locate your plane,

Direction to fix,

Location of hold.

1.  Valid only when flying toward the fix:

2.  The fix is at the center of the HI

3.  The plane is always at the bottom

4.  On airways, HI index number is to the fix

# Simple Holding

Fly to the fix

Turn to nearest outbound or teardrop heading

fly for one minute,

turn back to fix on holding side

...as published

...as given by ATC

Fly time/distance (DR)

Turn inbound on holding side and intercept inbound course.  Correct for wind during entry.  Established, triple the wind correction outbound.  (As of January 6, 1995, the AIM  uses the word 'triple' instead of double.) Stay in protected airspace.  This is easiest with the NDB/ADF holds.

You can expect a controller to give you a hold that is in front of you.  You will be directed to hold in a magnetic direction for a publixhed hold or on a radial otherwise.  Draw the hold on the chart.  If you have a problem tell the controller and request a heading or vector into the holding pattern.

Simple hold is, “Turn to the outbound heading, do a course reveral to the holding side and enter direct.”

# Thinking Through the Hold

There are several flight items that must be decided  before trying to enter a holding pattern.  With some repetition you will find that each hold has a pattern both as to turns and sequence of radio changes.  By knowing them you can anticipate and stay ahead of the airplane.

1. Be sure to get the aircraft power and trim set to holding speed before 	reaching the fix.

2. You must fly to intercept a radial determining the fix or fly directly 	through (to) the fix.

3. Think of the fix as being in the center of your heading indicator.

4. You are flying toward the center of your heading indicator from the bottom 	up.

5. The number at the top of the card is the direction from the fix.

6. At the fix turn to the outbound heading for one minute.  Then urn back inbound on the holding side.

# Off Airways

On occasion you may be off airway and told to fly and hold at a specific fix.  Locate the fix.  Make an approximate turn toward the fix.  Set in the frequency and OBS (FROM) settings for both VORs determining the fix.  From the position of the two needles select one to intercept prior to arrival

at the fix.  The radial intercept that will provide the easiest entry will be the holding inbound course or its reciprocal will produce the easiest

holding entries.  On airways a 45 degree intercept is recommended while on approaches 30 degree maximum is to be preferred.  Failure to take a good intercept angle will create a problem if flying into a strong cross wind.  Be

sure to reverse the selected OBS (FROM to TO) setting for interception

of a radial if flying inbound toward a VOR.

# Single VOR Holding

All of the previous VOR holding has assumed that you have dual VORs.

Now you should try holding along an airway using only one Nav radio.  This requires that you have identified two VORs as operational and are keeping the volume at a level sufficient to hear the code.  You need to have both frequencies and OBS settings required for the fix posted/memorized.  You will be busy making the changes so practice the way to turn the frequency knobs and the OBS for the most efficient changes.  Counting frequency change clicks is one way while you keep your eyes on the instruments.  You don't need to watch the frequency knob change.  Learn how far the OBS turns as you normally twist it and make the twists required before looking at the numbers.

# DME Holding

The DME fix will be along a radial perhaps of an airway.  If you are flying the radial be aware that your flight direction must be either away from (FROM) the VOR or through the VOR (TO).  The DME fix is always named as being

on the radial FROM the VOR so this can be confusing.   Be sure that you are reading the DME in such a manner as to be flying toward the fix.  This type of DME fix has only four possible holds.  All the holds are along the radial.  Two will be toward the VOR and two will be away from the VOR.  Two will be direct entry and two will be teardrop.  If you are flying toward the fix and the VOR your OBS will be TO as will your heading.  BUT the fix is named as DME on the radial FROM.  This can cause confusion.

Prior to reaching the fix you must determine two things about the hold.  Determine the direction of the hold and the direction of the turns.  If your flight will reach the holds before reaching the fix then the entries will be direct whether right or left turns.  If your flight will reach the fix before reaching the holds then your entries will be teardrop.  The teardrop with left turns will be to your right. The teardrop with right turns will be to your left.  Make your 30 degree diagonal turn across the hold accordingly.  Time for one minute.  Since the teardrop is actually a course reversal

you must reverse the OBS for the inbound heading to avoid reverse sensing.  If the entries to the DME fix are direct no changes of the OBS will be required.  It would be most unusual to be asked to hold at a DME fix at right angles to the radial determining the fix.

# The Clearance

The designation of a hold can take several forms.  A most frequent area of confusion involves the use of cardinal heading terminology.  The eight headings are north, northeast, east, south east, south, southwest, west, northwest.  You can be asked to hold northwest at a fix while on a southeast heading. You might be told to hold southwest while entering a fix from the south east.  Know the directions and their related numbers.

You may be given a radial from a VOR to hold on that requires a TO OBS setting.  A DME fix may require an OBS reversal or not.  While not mandatory it is considered preferable to always fly the #1 VOR head and use the #2 for interceptions.  The exception might be when flying to intercept an inbound approach heading.

The holding clearance will give first a holding direction.  The

holding direction may be given as a bearing to/from an NDB, as a radial from a VOR, or as one of the eight cardinal

directions .

## Clearance Types

Items essential to a holding clearance.  These items can be given in several ways published, standard, assigned by ATC.

Published

A charted hold can be given by ATC by giving the name, the holding leg direction and the term "as published", and the EFC (expect further clearance) time.

Assigned

1. EFC time is the last essential assigned item for any holding clearance.

An established no wind hold takes four (4) minutes with two one minute

legs and one minute standard rate turns at each end.

2. Definition of the fix.

Definition is by name of navaid, intersection, or geographic location.

3. The holding leg.

This may be a stated direction, radial, course, bearing azimuth, airway 	or route.

4. Leg length

Defined in miles or time if other than standard.  Standard on inbound 	leg is one minute below 14,000' MSL and 1 1/2 above.  Adjustment of 	outbound leg is required to obtain required inbound leg time.

# Standards

5. Direction of turns

Right turns are standard, left turns must be requested or assigned by 	ATC.

6. Altitude to be specified unless same as last assigned which is standard.

7. Speed maximum standards are set by FAR of 175 Kias for all other than Jets 	(265 KIAS). ATC cannot require speed reductions in holding patterns.

Best economy speed makes most sense for all concerned.

8. It is standard to triple the wind correction angle of the inbound leg on 	the outbound leg.  This compensates for the wind's effect during 	turns.  	(As of January 6, 1995, the AIM  uses the word 'triple' 	instead of 	double.)

The holding direction may be given as on a radial from a VOR which is actually the  reciprocal of the inbound holding course.  Knowing that this is the standard ATC representation means that you must mentally reverse the direction and radial for your inbound direction. (For example: Being told to, "Hold at a VOR northeast on the 030 radial" really means that your inbound heading will be to the southwest on the 210 radial.)

The same directions given as a radial to or from a VOR passing through an intersection at an airway intersection may or may not require the mental reversal.  The direction may be given as a bearing to an NDB in which the bearing and  inbound heading are identical but the actual hold will be on the opposite side of the fix.  You could be told to hold WEST (270), on the 090 bearing to the NDB.

It is the wording of the instructions to a hold is more often than not the cause of the perceptual difficulties pilots have.  While flying north to a fix you might be instructed to hold south.  This means that your inbound leg to the fix will be to the north while the body of the holding pattern will be to the south.  Draw it out and see.  The entry will be direct with no frequency/OBS changes required.

You must stay oriented.  If assigned to hold on a radial, you must realize that your inbound leg heading TO or at the VOR will be the reciprocal of the numbers given in the holding instructions.  If holding FROM the VOR (not at the VOR) the assigned radial and the inbound leg will be the same radial.

Again both situations should be drawn out to show understanding.

With reference to the eight cardinal directions you will be given

(1) The general direction of the holding pattern (airspace) from the 			fix as a cardinal direction. (i.e. Southeast)

(2) The name of the fix which can be a DME point as well.

(3) Specific direction numbers as a reference.  Whenever the holding 			leg is other than one of the cardinal directions the 				degrees will be given.  Radials are always FROM and 				bearings are always TO unless stated as FROM.

(4) The direction of the turns, right is standard.

(5) EFC Expect further clearance.

(6) Time now.

There is a way to help your interpretation of a clearance.  Learn to say the clearance as it would be given by ATC with all the requirements as you practice the sequence of holds below.  This will clarify your understanding of how the words define the hold.  Knowing how the clearance tells where you will be and what you will be doing will give you the situational awareness required to keep the brain functioning normally.  Learn to act as your own controller and give the clearance information you need to correctly approach the fix, determine the holding direction, and the turn direction. The cardinal direction is usually defined by assigning a radial number or airway.

As a pilot you need to think ahead of the holding pattern to how you will depart toward your destination.  The incorrect clearance can make that departure more difficult.  If ATC should give you a hold that is going to make your departure less than the most efficient you can suggest a change.  Just the difference of right turns to left turns can be significant.

# Course reversal holding and procedure turns:

The consistent use of a course reversal whenever it is necessary to reverse aircraft direction has desirable qualities.  A procedure turn done by course reversal does not require timing and focuses inbound attention on interception.  The course reversal very easily substitutes for all the intricacies required for parallel and teardrop holding patterns.

Just fly outbound on the inbound course and do the reversal to the holding side after one minute.  Simple.

# Approach Holds

There are two different depictions of approach holds on charts.  The

bold depiction of a hold is part of the procedure.  The bold hold is used as a course reversal segment.  Where the holding pattern is lightly depicted on the plate it is part of the missed approach but not part of the procedure.  You cannot correctly fly the procedure from a lightly depicted holding pattern.

The TERPS has different altitude minimums for each variation of the holding pattern.  Where a light holding pattern exists you must get established on the outbound course before doing a course reversal by

procedure turn or otherwise.  Only proceeding this way will insure that you remain in protected airspace.

# Clearance Simulation

It is possible to learn your job better as a pilot faced with a holding clearance by knowing the job of your ATC counterpart who has to issue the clearance.

# Holding Peculiarities

There are 31 different holding pattern sizes based on different aircraft speeds and altitudes.  In many situations ATC can, at its option change the size of a holding airspace protected area once an aircraft is

established in the hold.  The holding minimum pattern altitude (MHA) may

be less than the minimum instrument altitude (MIA) in mountainous areas.

Not all holding patterns considered to be ‘published’ are on charts or plates.   It doesn’t matter.  ATC can require you to hold at any time as

long as you remain above specified altitudes.  You do not need to be in

radar contact for an ATC hold.  Because of protected areas minimums all instrument approach procedure course reversal holding patterns are limited to 200 knots or less.  TERPS design of the holding patterns protects the

course reversal area under the assumption that the outbound course will be flown.

Study Session

Use the San Francisco Area Chart.  Draw, walk or 'dry run' the following routes and make the holds as assigned.  If making a 'dry run' in an aircraft work through the frequency/OBS changes required at each hold.  Make a 4 column chart of the changes.  Try using parts of the charts on an actual flight.

# Bay Area Intersections

The IFR student may have no idea as to just where his checkride will go or by what route.  The following is an exercise which will familiarize the student with at least one region of the Bay Area.  Recognition of where you are in relation to your surrounding space is an essential to all flying but even more so with IFR flight.  It should be noted in passing that every airway intersection has a name which bears some relationship with the ground below or its history.  Many holds appear only on approach plates.

The following is a plate/chart study process whereby the student proceeds to study the route of the charts and plates and thereby familiarize himself with the area and at the same time develop a sense of how to make any one of the possible holds that might be assigned. The course might well be reversed in direction at the end and make the same holds coming back but write/say your own clearances.

Depart CCR APT direct CCR VOR, direct FLEAT, hold at FLEAT as published.

Depart FLEAT via V150 direct CROIT, hold northwest at CROIT in right turns.

Depart CROIT via V195 direct SABLO, hold north at SABLO in left turns.

Depart SABLO direct CCR VOR, hold on CCR 071 radial in right turns.

Depart CCR direct CROIT, hold northwest at CROIT in right turns.

Depart CROIT direct SABLO, hold southeast at SABLO on the SGD 127 radial in right turns.

Depart SABLO direct CROIT, hold east at CROIT in left turns.

Depart CROIT direct SABLO, hold northeast at SABLO on the CCR 230 radial in right turns.

Depart SABLO direct CROIT, hold southwest at CROIT in right turns.

Depart CROIT direct SABLO, hold southerly on the OAK 347 radial in left turns.

Depart SABLO direct CROIT, hold northeast on V 150 at CROIT in left turns.

Depart CROIT direct CORDD at cruise speed, hold northeast at CORDD in right turns.

Depart CORDD direct SGD VOR at cruise speed, hold west at SGD in right turns.

Depart SGD direct CROIT at cruise speed, hold northwest on V 108 at CROIT in left turns.

Depart CROIT direct CCR VOR at cruise speed, hold south at CCR in left turns.

Depart CCR direct SABLO at cruise speed, hold northeast on CCR 230 radial at SABLO in right turns.

Depart SABLO direct CROIT while climbing 500', hold easterly on the 259 radial of CCR at CROIT in right turns.

Depart Croit direct SGD VOR while descending to 1000', hold southerly on V 87 in right turns.

Depart SGD VOR direct KCBS 740 KHz, climb to 2,200', hold west in left turns.

## Exercise II

Depart CCR direct CCR VOR direct FLEAT, hold at FLEAT as published.

Depart FLEAT on the SGD 089 radial to intercept  V195 direct CORDD, hold north on V195 in right turns.

Depart CORDD on 061 radial of SGD VOR, hold northeast on 061 radial at SGD 6 DME fix in right turns.

Depart this fix direct SGD, hold on SGD 347 radial at SGD in left turns.

Depart SGD via V 108 114 radial, hold northwest at 8 DME fix in right turns.

Depart this fix direct CROIT and hold at CROIT east on CCR 259 radial in right turns.

Depart CROIT direct FLEAT, hold at FLEAT as published.

Depart FLEAT direct CCR VOR.  Hold at CCR Southeast in left turns.

Depart CCR VOR via V 108 direct CROIT, hold northeast at CROIT on V 150 in left turns.

Depart CROIT via SGD 114 radial, hold northwest 7 DME SGD fix in right turns.

Depart this fix direct CROIT, hold southwest on SAU 035 radial at 20 DME fix in left turns.

Depart this fix direct SABLO, hold south on OAK 347 radial at SABLO in right turns.

Depart SABLO direct CROIT. Hold westerly at CROIT on the CCR 259 radial in right turns.

Depart CROIT  direct SGD  VOR, hold west at SGD in left turns.

Depart SGD direct FLEAT, hold as published at FLEAT.

Depart FLEAT direct CROIT, hold northeast at CROIT in left turns.

Depart CROIT to intercept APC 36 localizer, hold south at outer marker in right turns. Do not intrude on Class D airspace.

# Intersections and Airways

Mentally proceed on this flight.  Hold as indicated.  Select several to actually fly.  (In Marin County the proximity to the TCA makes it inadvisable to fly unless on IFR clearance.) Pick legal altitudes least likely to be flown by other aircraft.  Remember some holds and fixes are shown only on approach plates

Proceed via V 108 to:

1. SDG hold on 347 radial in left turns, then to

2. MICRA hold northwest in left turns, then to

3. PYE hold northwest on V 107 in left turns, then to

4. FREES hold northwest in right turns, then to

5. GETER hold north in left turns, then to

6. LAPED hold west in right turns, then to

7. DALON hold north in right turns, then to

8. RUMSEY hold southwest in right turns, then to

9. ILA hold northeast in right turns, then to

10. BESSA hold west in left turns, then to

11. RAGGS hold east in right turns, then to

12. CORRD hold north in right turns, then to

13. CROIT hold east on CCR 259 in right turns, then to

14. SABLO hold south in left turns, then to

15. CCR hold north in left turns, then to

16. PITTS hold east in left turns, then to

17. RYMAR hold as published, then to

18. REJOY hold east on ECA 288 radial in left turns, then to

19. COUPS hold south in right turns, then to

20. SAC hold northeast in right turns, then to

21. VISTO hold north in left turns, then to

22. OAKEY hold south in right turns, then to

23. PITTS hold east in left turns, then to

24. CCR hold WEST in left turns, then to

25. CROIT hold south in right turns, then to

26. FLEAT hold as published, then to

27. EMBER hold northwest in right turns, then to

28. SAC hold northeast in left turns, then to

29. CAPTO hold southeast in left turns, then to

30. GRIME hold southeast in left turns, then to

31. ILA hold north in right turns, then to

32. RUMSEY hold southwest in right turns, then to

33. MAUCH hold east in right turns, then to

34. POPES hold west in left turns, then to

35. RAGGS, hold north in right turns, then to

36. CORRD hold south in left turns, then to

37. SGD as published (see approach plate), then to

38. YUBBA via V 87, V 200, hold as published, then to

39. HALOW  hold as published, then to

40. SAC direct hold as published, then to

41. METRE via SAC 337 radial, hold as published, then to

42. LANEE via 124 bearing, hold as published, then to

43. WAGER via radar vectors SAC 137, hold as published, then to

44. ORANG via V 585, hold as published, then to

45. CEA hold as published, then to

46. MOD via ECA 124 radial, hold as published, then to

47. WOWAR via IMOD, hold as published, then to

48. QUADS via direct LIN via 285 radial, hold as published. (see LODI), then

49. TRACY via ECA direct via 229 radial, hold as published (See LVK plate), 	then to

50. MERET via TZZ VOR via 349 radial, hold as published(see ?? plate) hold 	as published, shoot approach and fly missed approach, then to

51. AGGIE hold as published, then to #36 and fly back.