# Click on page number for instant scrolling.

Proceduresã

PART 91 IFR

IFR FARs

Airway Clearances

Standard Instrument Departures (SIDS)

Standard Terminal Arrivals (STARS)

Alternate Airport

Alternate Alternates

Sample Approach Briefing

Preparation

You need:

One planning system:

Flying Approaches

Preflight Planning Do’s

Don’ts

Clearances

IFR Shorthand:

Types of Clearances

As filed

Tower Enroute

Pop-up

Pop-up checklist

Procedures

Timing

Select a heading:

Cleared for the Approach

Common Approach Elements

Approach Segments:

Transition

Beginning of approach

Initial Segment

Intermediate Segment

Final Segment

Missed Approach Segment

Step-Down Approach

Stabilized Approach

Full approach

Holding Pattern Approach

Missed Approach

IFR Dead Reckoning

Speed changes

Altimeter Setting

Remote altimeter setting source (RASS)

Required Fuel

Meanings of ‘Established’

Reversing Direction for Approach

Procedure Turns

NoPT (No Procedure Turn)

IFR Descents

Clearance on unpublished route

Procedure

Cautions:

Descent (Decision) Height

Airport in Sight

Decent Procedures

Initial Approach Fix

Uncontrolled Airport Arrival

Cruise Clearance

IFR in VFR

Uncontrolled Airport Departure

Clearance Void time

Hold for Release

Controlled departure time

Non-radar IFR

Non-radar Approach

Select a heading:

Visual Descent Point

# PART 91 IFR

Only FAR 91 operations can depart regardless of visibility and get a "look-see" privilege on approaches.  Someone operating under FAR Part 91 can legally takeoff from an airport but not have the landing minimums needed to return and land at the same airport after takeoff.  Not a good 'go' choice.  Under FAR Part 91 you can legally takeoff when a commercial/airline operator would be grounded.  Yeah, takeoff but can you get back?

Since there are no Part 91 departure standards, prudence would dictate:

- No departure be made if an arrival is not possible.

- Compliance with a published IFR procedure or with a SID is not mandatory unless it is included in the clearance.

- If no IAP is published, what will be the safest way to go?

- Is an alternate obstruction clearance route available?

- Will visual clearances be possible? Know the terrain, the minimum climb rates compared with your aircraft capability.

- I will do zero-zero departures for training under VFR conditions.

Departure obstacles are based on a 152' per mile slope from 35' above the departure end of the runway.   As a standard, procedure designers assume that the aircraft will cross the end of the runway by at least 35' above the ground and will then climb at least 200 feet per nautical mile.  The aircraft is expected to climb to 400' AGL before turning.  If an obstacle penetrates this slope the airport will have a non-standard takeoff minimums. (Small T in black triangle) and greater performance will be required.   If a climb of more than 200 fpnm is required on the plate to a certain altitude, a climb of 200 fp/nm will suffice after that altitude.

If marginal conditions exist, you must check takeoff and alternate minimums. The standard minimums apply only if there are no nonstandard alternate minimums published.  The published departure procedure gives an obstruction clearance route.  You are expected to climb to pattern altitude before turning on course or to the assigned heading.  For Part 91 operations, reported ceiling does not make a difference.  Only visibility controls.

A technique to avoid vectors around areas like LA Basin, Chicago, or in the North East works well .  File IFR from VOR on the other side of these areas to your destination.  In remarks section of flight plan indicate time you will pick up IFR plan over VOR.   Takeoff VFR fly to

VOR and pick up IFR flight plan.  You must remain VFR.  This avoids all the wasteful vectors ATC gives IFR flights around the dense areas.  The more off-hours you fly the more likely you are to get your way instead of ATC's way.

# IFR FARs

You cannot file IFR at any time unless you are IFR rated.  A non-rated IFR student can, with instructors authorization, file in the instructor's name, for a dual IFR training flight.  You cannot file IFR or fly in IFR conditions if you have not flown your 6,6,& 6.  Only an instrument competency check can overcome the 6,6,&6 requirement.

# Airway Clearances

If a pilot who is operating along an airway receives a clearance to operate below the MEA it can be accepted if:

1.  If the altitude is no lower than the MOCA and the aircraft is 	    within 22 nautical miles of the applicable fix or navaid.

2.  If radar navigational guidance is provided along with lost 		    communications instructions.

3.  Do not accept a clearance that is proximate to an obstacle

# Standard Instrument Departures (SIDS)

An airport with an instrument approach will have a SID when obstacles exceed a 40-to-1 plane.  A SIDS depicts all nav aids, courses, and altitudes for the departure.  The presentation is both graphic, textual and computer coded.  The standard climb gradient of 200 feet per nautical mile is expected unless a steeper gradient is specified.  Required frequencies are part of the SID plate.

SIDs come in three types: pilot nav SIDs, vector SIDs, and in combination.  Most SIDS are combination.  SID plates change rather often so keep your SIDs up to date.  I recently had one more current than the one given by ATC.  SIDs may require specific aircraft performance to meet SID requirements.  Check the gradient chart to see if you comply.  Standard is 200 feet per mile or 333 feet per minute at 100 kts.

If there is no SID because obstacles do not break the 40-to-1 plane, the pilot is expected to do what is required to assure a safe departure.  You should check the terrain, gather local advice, plan for visual avoidance of terrain, or otherwise fly the published departure procedure.

ATC expects the pilot to fly the SID so that required separations will be met.  If you are below radar coverage you are on your own until reaching controlled airspace or an assigned fix.

# Standard Terminal Arrivals (STARS)

A STAR can be a profile or textual description of a route whose purpose is to get you on the instrument approach.  Different classes of aircraft may have different ‘notes’ to follow.  If you go to an airport that has STAR procedures you will probably get one even though you did not file one.  Some ATC procedures are STAR like without being called a STAR.  They often coincide with PARs or preferred arrival routes.  Controllers will often cut the corners off of STAR routes to same time.  Always confirm with ATC if your routing is different than indicated on the STAR.

STARs have altitudes that are MEAs.  If you lose your radios do not fly the STAR altitudes.  STAR altitudes and descents are valid only when you have been cleared for descent or on a cruise/approach clearance.  A STAR may not be at an IAF (initial approach fix).  Center may use the end of such a STAR to hand you over for vectors by TRACON.

A STAR has the same features as a SID with the addition of specific altitudes and speeds.  Mostly required for heavy jets.  Know what the STARs are so you know what ATC is doing.  What you usually see on a STAR is what you will normally get as a clearance when filing  "No STAR".  Star transitions are just branches from the root route.  Required frequencies are part of the STAR.

A STAR is not a profile descent.  When descent clearance is issued ask for clarification if ATC gives mix of descents and STAR clearances.  Don’t descend to the altitudes shown on a STAR until an unrestricted descent clearance is issued.  STAR altitudes are to be ‘expected’ and must be validated by a clearance.  Being cleared for approach while on a STAR route gives you the right to descend to all published MEAs, otherwise you must stay at your last assigned.

# Alternate Airport

An alternate is not required if the first airport of intended landing has a standard instrument approach procedure and for at least one hour before and one hour after the estimated time of arrival, the weather reports or forecasts, or any combination of them indicate the ceiling will be at least 2000' above the airport elevation and the visibility will be at least three statute miles.  Weather reports or forecasts, or any combination of them may be used to determine if an alternate airport is required. (A terminal forecast from the destination airport is required.) FAR 91.169(b)

If no instrument approach procedure has been published in FAR Part 97 for an airport, the ceiling and visibility minimums are those allowing descent from the MEA, approach, and landing under basic VFR.  FAR Part 91.169(C)(2)

# Alternate Alternates

Far 91.103 gives legal requirements but the your real need may differ.  With your destination predicted to be above 2000' and 3 miles for an hour either side of your arrival time you don't need an alternate.  If you can't land at your destination you will head for the closest ILS with over 600' and 2 miles or a nonprecision with 800 and two miles.  The "paper alternate" exists to assure you of protected airspace to cover lost communications.

Never rely on an alternate's forecast.  Weather is what it is; not what it's supposed to be.  Monitor your alternate weather enroute.  Barring the forced use of the "paper" alternate, you can use anywhere you want as the alternate.  The choice  of alternates declines at night.  Radar and towers close.  Keep your alternate within range.

If your destination doesn't have an approved approach procedure you're required to file an alternate regardless of the weather. Some airports are not legal alternates with tower closed, having no weather observer, or navaids out of service.  The rules for alternates exist so pilots have a way out.  Your alternate doesn't need an approach procedure if you can descend from the MEA, approach and land under basic VFR.

# Sample Approach Briefing

1. This will be a ______ to runway ____ at ___________

2. Field elevation is ______ feet

3. Time for the FAF to MAP will be ______min ______sec.

4. The MDA/DH for this approach will be  ______ feet.

5. The required rate of descent will be ______FPM at ______Kts.

Get 3-degree descent by G. S.  X  10

6. Approach notes are 			  2

7. Missed approach procedure is...

# Preparation

By being prepared and organized you can concentrate on your 	flying.

- Get the ATIS ahead of time.  If no ATIS get weather and NOTAMs ahead of time.

- Set radio for best reception

- Heading indicator set

- Listen selectively for what applies to you and that you need.

# You need:

- Ceiling and visibility

- Memorize chart essentials.  Debrief the approach as to headings, altitudes, frequencies, and missed approach.

- Direction (Which way).  Will vectors affect your orientation.  How

- will you protect yourself by knowing where you are.

- Altitudes (How low)

- Nearby published holds.

- Time  (How long)

- Initial heading and Altitude of missed procedure

- Know how and where to locate chart information.  Have all charts for flight available.

- Use the other person in the cockpit

- Terrain

- Aircraft performance settings for level, descent, and missed.

- Fuel requirements.  Escape routes.

- Instrument performance check.

# One planning system:

1. Inside the initial approach fix (IAF)

Five "A"s

ATIS

Altimeter

Airspeed

Avionics

Approach briefing

Which Way

How low

How long

Configuration

2. Course interception

Needle alive

Callout Six "T"s

Turn

Time

Twist

Throttle

Talk

Track

3. Intercepting radial

"Cross radial alive"

Callout

4. Descent from Final Approach Fix (FAF)

Callout Six "T"s

Configuration check

5. Circling minimums

Callout

Visual Descent point (VDP)

Gear down

6. Missed approach point (MAP)

Callout

Missed

Runway in sight

Each approach may require a slightly different arrangement of the above.

# Flying Approaches

A successful approach begins by homework before takeoff.  Many IFR students overlook the planning and organization required before you leave the ground.  You have planned and decided when to set your radio stack for the approach, when to slow to approach speed, when to do the pre-landing, when to remind ATC that they are creating a problem for you, and how you plan to maintain situational awareness (ADF), the effect of a head or tail wind, a descent, and a visual descent point, etc.  With the new Jeppesen charts showing terrain in color the pilot should form a mental picture as part of the pre approach planning.

# Preflight Planning Do’s

- Review approach procedures, all of them

- Confirm alternate requirements

- Confirm aircraft has required instruments

- Check category and category minimums

- Get all NOTAMs

- Determine personal minimums based on experience at this field.

- Plan to fly as published

- Plan your descent and the Missed Approach

- Know what it takes to have airport in sight

- Expect to do the missed and plan to fly it as mapped

- Turn toward runway if a circle to land missed is required.

- If proceeding VFR be sure to cancel IFR

# Don’ts

- Don’t fly actual to minimums unless all aids are operating in the aircraft and on the surface.

- Don’t descent below DH or MDA until you see the runway indicators

- Don’t fly circle-to-land at-straight in minimums

- Don’t deviate from IFR procedures until VFR and have cancelled IFR

# Clearances

CRAFTS (mnemonic)

C = clearance limit

R = route

A = altitude

F = Frequency

T = transponder code

S = special instructions

# IFR Shorthand:

Always slash a zero to avoid confusing with O

Put parentheses around instructions

15 = fifteen minutes after the hour

HFR = hold for release

AF = as filed

RV = radar vectors

RH = runway heading

/_______ climb and maintain,

\______ descend

and maintain,

rr report reaching,

CZ control zone,

___________

___________ A line above and below means maintain

A line below means at or above

A line above means at or below

O with arrow into, out of, or through means to enter, leave, fly through

C cleared,

EFC expect further clearance,	expect ten thousand in ten minutes

If cleared to 2000 and expect 6000 in 10 minutes...Write

2- 6 x 10

_/____ intercept,

x cross,

rh runway heading,

rt   right turn,...etc

D = With arrow = direct

15k with up arrow = climb 15000'

Put transponder code in rectangle

V = victor airway or void as in void time

X = cross as in crossing fix or VOR

# Types of Clearances

An full approach clearance does not include airways before the feeder routes and associated initial approach segments for which the aircraft is equipped.

# As filed

If ATC chooses not to allow "cleared as filed" then the full route clearance must be read to pilot, copied by pilot and read back by pilot.

# Tower Enroute

When on a Tower Enroute Clearance you may expect to contact Center  (ARTCC) if the Letters of Agreement LOAs apply to your route or altitude and other facilities do not have coverage.  Example: CCR to STS. (Santa Rosa or Napa)

The FAA has charted tower enroute control(TEC) procedures for most of California and other heavily traveled areas.  There are also called

prefered routes and are given a separate listing  by Jeppeson.  Prop aircraft have selected routes and altitudes that avoid the jet and

turboporp traffic.  If you file out Class-B areas you are going to get a preferred route.  This, as in the Los Angeles and San Francisco areas

will usually fly you all the way around the Class-B airspace.  You can avoid this by filing in short segments, airport to airport until reaching the outer edge of the Class-B.

# Pop-up

An on-the-spot request for a clearance directly with ATC.  This request is made for practice or when faced with VFR to IFR conditions enroute.  More often than not ATC will come back with request for aircraft identification, type and equipment (If you didn't remember to get it in the request).  Then will come the route, altitude and squawk code.  Radar contact is not required but it helps.  ATC can give you a clearance without radar contact if the pilot agrees to maintain terrain/obstacle clearance.

The pop-up may be made as a VFR or IFR flight.  It may also be

a practice VFR or practice IFR flight.  The IFR pop-up flights are given separation as for all other IFR flights.  The VFR or practice VFR procedure must be approved by ATC on a ‘work load permitting’ basis.

The word ‘practice’ separates the two.  ATC on approval will advise that

separation is not provided for VFR or practice approaches. Additionally, unless the option on the approach is requested a missed approach is

given only by request and ATC approval.

At a controlled airport a common pop-up is "IFR to VFR-on-top".  This means that you are allowed to depart in IFR conditions with the expectation that you will reach VFR very quickly (CCR).  Once you are on top and over 1000' above the clouds you cancel with approach and proceed VFR.  Give cloud base and top reports.

When you pop-up into a radar controllers airspace he may direct you to fly to a fix before issuing a clearance or giving a vector. Given the problem of flying to a given fix from an off airways position.

- FIRST, make your best judgment turn to a heading toward the fix.

- SECOND, go about setting in your frequencies and OBS settings.

- THIRD, plan your 45 degree intercept of the airway.

There is at least one instance (1981) where failure to complete the flight plan form with all the particulars was deemed a violation.  ATC in a pop-up may not even bother to write the information but it will be on tape. that you complied with the FARs.

# Pop-up checklist

ATIS

Configuration

Nav # 1

Nav # 2

ADF

Marker

DME/LORAN

Pitot

Which way

How low

How long

VDP

Missed

# Procedures

Variations of transitions, segments and fixes make approaches.  The simplest approach on an airport will have only a final segment, MDA and missed approach.  No timing, no intermediate segment, intermediate fix or final fix.  Visual descent point (VDP) may exist.  There is no glide slope information.  Final approach may have several stepdown fixes.  At a minimum descent altitude (MDA) level flight continues, usually by time, to the missed approach point.

- ATC cannot vector you to intercept the ILS above glide slope.

- Vectoring altitude is 1000' AGL in flat terrain and 1500/2000' AGL where mountainous.

- The "Maltese Cross" on approach plates is the final approach

- fix for non-precision approaches only.

- The final approach fix for precision approaches is glide slope

- interception.

- If the ILS becomes a localizer approach then the time over the Maltese Cross must be noted.

- Ground speed can be determined electronically by DME, LORAN or GPS more commonly by using the wind direction and velocity from whatever source.

- Use of the airport wind is very uncertain since wind speeds vary greatly with altitude.  The variability of the approach ground speed along with other instrument factors are what makes most approaches non-precision.

# Timing

Timing, whether you use a traditional or electronic timer makes no difference.  Time is the second T of the 5/7 Ts. for every segment of the instrument approach.  You may not use it every time but it should remain as part of the sequence.  The time required for an approach depends on ground speed.  The timing of an approach involves more than just noting the time off the chart.  Since the time is based on ground speed we must factor into it the effect of the wind, our proficiency at holding an airspeed during established flight and transitions.  The result of your timing efforts will only be as accurate as your data input.

The "Maltese Cross" on approach plates is the final approach

fix for non-precision approaches only.  An ILS should be timed passing the non-precision FAF (Maltese cross).  This guarantees positional awareness if the glideslope fails and you need to continue with a localizer-only approach.  The FAF for an ILS is the point you intercept the glideslope at the designated altitude on the chart. The localizer FAF may be inside or outside marker, begin descent at interception.(See Livermore chart) The final approach fix for precision approaches is glide slope interception.

The greatest hazard associated with nonprecision approaches is the descent from MDA to the runway.  In relatively poor conditions the pilot must change from IFR to VFR flight.  Your localizer is only good within three degrees of centerline.  An LDA’s precision is even less.  It is not unusual to miss the runway by 1/2 mile at the MDA of a NDB approach.

Where a VASI or PAPI serves as a glide path assistant some degree of obstacle clearance is assured.

# Select a heading:

Reference Heading is the heading that will keep the needle centered.  Because of wind you must bracket this heading through referral to the Heading Indicator  One system uses 1/2 angle corrections of ever decreasing amounts.  (see instructor)

Initial changes to find the reference heading should be 5 degrees.  (10 degrees only if 1/2 deflection.)  All turns by reference to heading indicator. Stabilize your heading and airspeed based on the wind.

# Cleared for the Approach

If a pilot who is "Cleared for the Approach" does not understand his responsibility to adhere to the charted altitudes of that approach, he could be preparing for an accident.  The phrase is one of the most misunderstood and ambiguous terms used by ATC.  Controllers expect pilots to know what they are supposed to know.  Pilots who don't know what they don't know will fail to follow the expectations of the controller.  FAR 91.175 (i) states--"when a pilot is cleared for an IFR approach, he shall maintain the last assigned altitude until established on a published route for which a lower minimum altitude is published."

Any route leading to an IAP (instrument approach procedure) becomes part of that procedure when the approach clearance is given.  If you don’t get an approach clearance you can’t descend.  The approach clearance should relieve you from all altitude restrictions.  Just report leaving  your present altitude

The pilot who has a malfunction of equipment or systems on an IFR flight is required to report the problem to ATC (FAR 91.187).  The pilot should know that being cleared for the approach requires him to fly the altitudes and routes as charted.  You can remain at higher than charted altitudes but never at lower than charted.  If ATC provides vectors then altitude restrictions must be included.  If ATC fails to provide the required instructions or information then the pilot needs to know enough to pick up the error.  Most of all the pilot must know enough of his situation to say "NO" to ATC when it is justified.

The term "radar contact" does not mean that ATC will provide obstruction  avoidance.  ATC will not provide advisories as to traffic and terrain below the minimum vectoring altitude unless specifically requested.  The informed pilot understands the performance rules of ATC radar.  Minimum IFR altitudes (MIA) is the minimum vectoring altitude for Center radar.  Minimum vectoring altitude (MVA) apples to terminal facilities.

# Common Approach Elements

Preparation:

- Study the plate.  Highly essential.  Memorize what you can.

- Be ready to fly the full approach.

- Always assume you'll have to fly the missed.

- If you need to look at the approach plate inside the outer marker 		you have not properly prepared.

- Configure the aircraft for approach before reaching a critical 			point on the approach.

- Anticipate what will be required

- Use a pre-approach checklist

- Take your time.  Slow the aircraft down. Try to determine the effect 	of wind. (Tailwind approach)

- Remain in a hold until YOU are ready for the clearance and 	approach.

- The inbound hold will probably be aligned with the approach course.

- Use what you have

- Use all available equipment and devices.

# Approach Segments:

Each part is defined as to:

- Beginning

- End

- Course

- Altitude

# Transition

--Inbound to IAF--

- Beginning - the route to IAF

- End

# Beginning of approach

-- the Initial Approach Fix --

- Course

- Altitude

# Initial Segment

--Between IAF and end of procedure turn--

- Beginning - IAF

- End - to end of procedure turn

- Course

- Altitude

# Intermediate Segment

--Established inbound to FAF--

- Beginning - from procedure turn

- End - to final approach fix (FAF)

Fan markers can be used as FAF but such use is rare.

- Course

- Altitude

# Final Segment

--Between FAF and MAP--

- Beginning - FAF

- End - missed approach point (MAP)

- Course

- Altitude based on 40:1 climb gradient of 152’ per nautical mile.

# Missed Approach Segment

This segment begins at the decision height (DH) or at a specific point in non-precision approaches.  The chart will show an altitude, direction, and clearance limit.  Figure minimum climb of 200’ per nautical mile.  Given a choice between a timed and DME MAP, take DME.

Any departure course within 15 degrees is considered straight-out.

On an ILS do not make any heading changes until above 400’ AGL and on non-precision make your turn immediately.  You can always climb before the MAP but never turn before the MAP.

There may be more than one missed approach procedure but only one will be charted.

--Between MAP and holding fix--

- Beginning - MAP

- End - to holding fix

- Course

- Altitude

Any approach in actual conditions should be flown with the upermost idea that the missed approach is an anticipated outcome.  Be prepared to configure the aircraft.  The landing gear is the least critical item

of any re-configuration.

# Step-Down Approach

First of all, slow down.  Know the pitch attitude, power setting, and configuration that will give the most rapid descent you can control at the 90 kts approach speed.  Practice this descent and level off until it is not part of the problem.  The step-down approach requires you to be able to control the maximum performance capabilities of your aircraft.  Doing this gives you time to pick up the runway, configure the aircraft, and execute a normal landing.

# Stabilized Approach

- Determine primary heading

- Established outside FAF

- + 5° to MAP

# Full approach

The FULL Approach always includes the procedure turn.  Anything else is considered a straight-in approach via:  (Within 30 degrees of runway alignment)

- Vectors

- NoPT Transition

- From a holding pattern

- DME arc

# Holding Pattern Approach

If a holding pattern is part of the approach the pattern maneuver is completed when the aircraft is established on the inbound course after executing the appropriate entry. (AIM 5-48)  ATC expects you to proceed straight-in when crossing the fix.  ATC does not expect/require a holding turn from any entry that allows you to arrive at the holding fix within 30-degrees and at altitude. AIM 5-48(a)(4)  The holding pattern maneuver is considered complete when the aircraft executes an entry that establishes it on the inbound course.  A radio report is required. If you wish to do the complete pattern one or more times, for practice or to become better established, advise ATC.

Coupled Approach

The use of autopilot may not be allowed due to roughness (wavering) of signals.  The faster the aircraft the more likely the problem.  If you have this equipment be moderate in its use and your reliance on it.   Use the autopilot only enough to maintain currency in its use.  Fly basic instruments on raw data and partial panel as opportunities arise.  Allowing the autopilot to become the dominant factor over personal proficiency is a mistake.

# Missed Approach

The best way to remove missed approach concerns is through standardization of your planning and execution.  The missed approach is a standard and normal part of the approach procedure. You must always pre-plan the missed approach at the same time that you prepare for the approach.  The missed is not a contingency.  It is part of the approach that is planned and prepared for.  If you need to look at the chart inside the outer marker, you have not properly prepared for the approach.  Don't even think about descending below the DH.

Make a stick-up with critical missed DH/MDA information near the AI.  One very useful technique is to write the "missed' procedure on the window with a grease pencil. The Missed Approach Point can be simplified for your post-it or window by using a standardized format such as:

How low  _________

How long _________

Which way_________

How far to my secondary airport________

The missed approach is always a viable alternative to landing.  The pilot, in preparation for the approach, has become informed of the procedure and requirements.  The "plan" is compared with the weather minimums, personal minimums and procedure options.  If the safer option is inconvenient, take it.  If you are going to make a creative missed (different than published) clear in with ATC first.   You can climb before reaching the missed approach point but you can’t legally turn until reaching it.  Given the choice fly an ATC heading as assigned instead of the published procedure which is more likely to put you into a holding pattern.

Landing expectancy is a "mind set" at the subconscious level. The perception of the situation may not be accurate but rather a expected, hoped for, desired situation.  The further the distance flown and the closer the landing the stronger the expectancy becomes.  The more insecure and uncertain the pilot is with the missed approach procedure the greater will be the drive toward landing expectancy.

Setting up the missed approach on the #2 NAV during the approach should be an integral part of the cockpit resource management system of the pilot.  The missed approach point on a non-precision approach occurs

when the TIME runs out.  It is much to late to look at the plate when the approach time runs out.  You should know what to expect by the time you reach your VDP.  The risk-benefit ratio can get pretty lopsided if you see the runway just as you pass over it.  (This probably caused the Sun Valley Mall accident) Reaching DH/MDA and probing for the runway is the most hazardous part of an instrument approach in low weather.

Consider executing the missed approach in your approach configuration.  It shouldn't affect performance that much and will reduce the workload.  Clean up the plane once you are established toward the fix. If equipped, use heading bug to set assigned headings.  It's always better to go around for another try.  You now have first-hand knowledge of the situation.

Most judgment errors in non-precision approaches occur during the descent from MDA to the runway as the transition from IFR to VFR takes place.  You cannot climb back to circling minimums once you have descended below them except for making the missed.  Some non-precision approaches have charted visual descent points (VDP) before the missed approach time runs out.  This is the point from which from the MDA a normal landing can be accomplished.  Visual descent points are usually DME fixes.  They provide normal descent to airport.

Just being able to see the airport does not mean that it is safe to get to the runway.  It is far better to execute a missed than to force your way to the runway.  The purpose of a VDP, even of your own making, is to provide a ‘normal’ landing approach.  Be sure to write where your VDP exists on the plate and the time from the FAF.

You can make your own VDP on any chart by taking 10% of the MDA and use that figure as the number of seconds to be subtracted from the approach time.  This method automatically adjusts for altitude, speed and time.  If you do not see the airport references at your calculated VDP you will be required to make a missed because you will be below circling minimums.  Remember, the missed climb can begin at any time but the turn part of the missed cannot begin until the time has run out.

Your ability to land at reaching minimums will depend on your ability to make the IFR /visual transition, configure the aircraft, and acquire the required runway visual features. (More to come) Flying the time out will usually make a landing at a mile long runway impossible.  Run these figures as part of your preflight chart study.

You should pre-determine when you expect to break the reported ceiling.  Your plan is that from 200’ above down to the MDA you will flick your scan ‘outside’ every four instrument scans.  Use a windshield spot as the scan point.  That is where you expect to see the runway.  This spot will be different for every aircraft and loading.  Every time you look outside at least two seconds is required to focus the eye on a 15° field of view.  At night you must force yourself to look slightly to the side of where you expect to see the airport.

It is very likely that you will be approaching a runway with the wind from one side or the other.  Keep in mind the angle of correction you are holding on the HI.  If you are holding a right correction you must look to the left of your windshield aiming spot.  It will make a

great difference where you can find the airport if you are in a constant

descent or already at MDA and flying level.  Fly both approaches to the same runway, descent and slope.  Notice how the approach affects where

you find the airport up until the VDP.  At the VDP both approaches are

at the same windshield spot.

The landing comes when you see the runway environment; the missed is everything else.  Nobody likes to miss an instrument approach.  The leading cause of IFR approach accidents is improper IFR procedures--especially recognizing when a missed approach is needed.  A rapidly evolving error chain in an approach is best indicated by any full deflection of the needles.  Start the missed approach as soon as there is uncertainty confirmed by instrument indications.  Execute the missed on any full-scale deflection of the localizer or glide slope.

The missed must be flown precisely to maintain obstacle clearance. The missed approach procedure will be immediately executed if at any time the aircraft descends below MDA.  You do not have to be at the missed approach point to initiate the missed approach. You can execute the climb part of the missed approach at any time.   However, you must continue the approach course, even while climbing, and initiate the missed approach procedures (turns) from that point in time. Virtually all missed-approach procedures require an immediate climb and a turn at the MAP.

If your approach goes bad, apply power and climb. You cannot execute a turn from a missed approach until you are at the time expiration for the DH on a precision or at the timed MAP on a non-precision.  Obstacle clearances before the MAP or DH are quite different from those that apply after the turn.  Fly the published procedure unless VFR and canceling.

A minimum climb of 152' per mile is required.  The area for the missed procedure is similar to that of the circle to land procedure.  Accidents during missed approaches are caused by failure to initiate, delay in initiating, and improper procedure.  Reliance on ATC radar to resolve difficulties is becoming a common source of problems.  (See FAR 91.175)

- Execute the missed if needles are erratic.

- Set your own decision height above the published according to

- your proficiency.

- If you make and lose visual contact with the airport below decision height execute the missed immediately.

- Landings get three shots but approaches get only one.  Leave

- while you're still alive.

- Don't even try if the weather is below minimums.

- FAR 91.175 tells when you must execute the missed.

- You will probably not be as near the runway when you do the missed from the glide slope minimum as you will using the elapsed time of a non-precision.

On occasion you may decide that a misseed approach other than that published will better fit your safety requirements.  Be sure to coordinate this with ATC so that they can give you appropriate headings and altitudes to fly.  An alternate missed procedure may not be possible.

# IFR Dead Reckoning

FAR 61.65(b)(2) required instrument pilots to be instructed in DR.

- DR is required for non-radar transitions to final approach

- DR is required in doing holding patterns

- DR is required in a non-precision approach when a navaid does not

- define to missed approach point.

- DR can legally be used on an IFR flight that extends beyond the 			"service volume" (range) of a navaid.  Such flight can be

- initiated by either pilot or controller.  Radar monitoring will be 			provided.

- DR provides situational awareness which should be augmented by a 			time and distance log record of the flight.

Have your plates marked so you don't have to look around for the times distances or altitudes.  Memorize them if you can.  Plan your descent so as to reach the MDA (minimum descent altitude) at least 1 mile before reaching the MAP (missed approach point).

# Speed changes

Speed changes are used by ATC to more efficiently move traffic.  Any non-compliance, errors or inability to make the changes are likely

to affect your arrival.  The only speed change that ATC cannot directly

order is the 250 knots above 10,000 of 91.117.

ATC deals with indicated, true, and ground speed.  True airspeed is used for flight planned enroute speeds.  The pilot flys indicated speeds and ATC corrects his radar measured ground speed accordingly.

ATC wants to know if your speed changes + 10 knots or by 5%.  When you

file your flight plan the speed you use  is the ATC reference speed

from which changes are measured.  If ATC assigns you a speed, that speed becomes a clearance.  Clearances are not violated.

# Altimeter Setting

All altimeter settings must come from a national Weather Service official observer or approved source such as AWOS or ASOS.

There are two different altimeter settings used in aviation.  European pilots and glider pilots tend to use QEF.  Americans use QNH for most operations.  The selection to be used for the decision height of an approach makes a difference.  Jeppeson provides both numbers.  NOS only QNH.

- QEF is when the altimeter is set to read zero at the airport elevation.

- QNH is when the altimeter is set to read elevation from a datum plane

which is at the oceans mean sea level (MSL).

# Remote altimeter setting source (RASS)

Most remote settings are close enough to the field’s setting that

you may be tempted to think the difference can be ignored.  When a low

exists in the area the difference can be critical.  The higher remote minimums must be applied to intermediate, final segments, to step-down and circling minimums.  The DH of precision approaches applys the difference but it is not applied to MSAs (Minimum safe altitueds , initial segments, airway segments or feeder routes.

# Required Fuel

There is some question as to whether, when you reach into the 45-minutes required fuel reserve, it constitutes an emergency.  You are allowed to use this reserve to complete a flight to an alternate landing but should you land at such an alternate with less than the required 45-minute reserve you would be in violation of FAR 91.151.  Catch 22?

# Meanings of ‘Established’

"Established" is a variable, it may mean "needle alive and moving toward center",  half-way, touching the doughnut or even centered.  A centered needle provides the greatest obstacle clearance.  One opinion is that when needle starts to move begin your turn to the Reference Heading.  When needle stops --stop turning.  For descent to published altitude "established" means making the procedure turn or crossing the fix for the first time in a holding pattern.

Some instructors use a three item list for being ‘established’.

- Cleared for the approach

- Within 10° for NDB, others needle alive

- Within procedure distance of FAF.

Obstacle clearance is the prime consideration.  Exceed any of the three items above and your clearance is not guaranteed.  I prefer to use

the second dot on the VOR/LOC head as being established and allowing a turn to the required heading.  Many VOR/LOC signals are unreliable outside the second dot.

The major consideration for being ‘established’ is obstacle clearance.  A centered needle gives the best clearance.  The standard procedure turn if taken to its ten-mile limits and to full needle

deflection is at the edge of obstacle protection.  Errors of needle deflection may work either for or against obstacle clearance.  The better you know the terrain and maintain your situational awareness

the more ‘established’ can vary from the centered needle definition.

# Reversing Direction for Approach

The procedure turn was part of the radio range system that preceded the VOR NDB, and ILS soon to be phased out by GPS.  The radio range sent out four radio beams in four quadrants.  An A-N audible tone could be heard to each side of a beam.  A was -.; N was .- when you were flying on the beam the tone was steady.  Any deviation off course gave either an A or N code indicating the direction required to correct.

At the juncture of the four beams was the null or cone of silence.  This

told you exactly where you were.  From this point you fly outbound on a beam for a timed period, execute a procedure turn, intercept the beam inbound and begin your descent, knowing that you have obstacle clearance back to the null and the airport so long as you stay on the A N beam.

The same course reversal procedure is used at most general aviation airports except when superseded by radar vectors.

Any course reversal procedure must be completed within 10 miles of a designated fix usually the FAF.  There are three types of full approaches used to turn the plane back into the final approach course:

- The standard procedure turn may illustrate a 45-180-45 to final.

AIM 5-48 NOS chart's barbed arrow indicates only direction/side of outbound course.  Point of commencement, type and rate of turn is discretionary.

- A published tear-drop pattern is required as published.

- A holding pattern is also required as published.  You do not need to make the full loop if your entry allows you to become established inbound.

You may used the “course-reversal” for your procedure turns or holding pattern entries as long as you remain in protected airspace.  I find these especially good for ADF approaches.

- On a published IAP any course reversal is mandatory for the pilot unless:

According to FAR 91.175(j)

1. Radar vectors are available

2. NoPT is published on plate

3. A timed approach from a holding fix with tower in operation and 		timing procedures in use.

# Procedure Turns

The procedure turn is prescribed as a method for turning an aircraft around and reversing its course when on the intermediate or final approach parts of an instrument procedure.  The approach place specifies the procedure turn fix, the inbound and outbound course, the distance required for completion and the direction r/l of the turn.  The procedure turn is required except when the NoPT is indicated, radar vectors are provided, when a holding pattern is shown in lieu or (rare) with timed approaches.  Only teardrop or racetrack procedure turns are specifically required.

The necessity for the teardrop comes when a very narrow corridor requires a significant loss of altitude.  This procedure is known as a penetration turn due to the space and altitutde constraints.  To make the intercept from a teardrop it is very important that you know your distance from the FAF.  A smooth turn at the nine mile point should give a smooth intercept.

In every other case how the procedure is accomplished within the allotted airspace as well as where to turn and how to turn is discretionary to the pilot.  To stay within bounds it is important the descent to the procedure altitude  and outbound route be at least "bumped" before beginning the procedure turn outbound.  Failure to do this could take you outside protected airspace.  You have ten miles from the fix outbound to complete the turn.  Allow at least one minute to get established inbound to the fix.

If ATC does not specifically state that you will be given radar vectors, you as PIC can decide if a procedure turn is required.  Being cleared for the approach by ATC means that they assume that you know what to do.  This includes, that you know you are required to remain above the charted altitudes for the route.  How you do it or if you do it is up to you.  Remember, if something goes wrong the FAA gets to second guess any decision you make.  See FAR 97.3 and AIM 5-48a.  Once established inbound you can descend to published inbound altitude.  Established means that the intercept needle is at least half way to center.  Cleared for the approach does NOT include a clearance to land.

If a descent is required allow a minute for every 500'.  Turning from the outbound part of the turn to the inbound part is a required reporting point.  The inbound turn leg is  when you use the mid-point of any needle deflection to initiate descent to the inbound altitude.  You should be in approach configuration with your approach checklist completed as you become inbound to the fix.   At the fix use your Ts list.  At MDA/DH land or make the missed.

The maneuvering zone is the wide side of the allocated space from the course line for making the course reversal.  To the other side of the course line there is a primary area which is five nm wide.  Around the whole is a two nm secondary area.  You have 8 nm of airspace on the procedure turn side of the course line and 4 nm on the non-procedure side both with  at least 1000' of obstacle clearance at assigned altitude.   The 10 nm turn requirement distance has 16 nm of protected space. Your knowledge of this available space should reduce the plucker-factor if you overshoot or misjudge wind effect.  It is the safest procedure to assure flight to the center of the procedure area.

The turn options that work are, the often published 45-180 which requires diverted attention to timing, an 80-260 (90-270 course reversal) which does not require timing,  and the Army 45 (for 40 seconds)-225. In strong crosswind conditions a 260-80 might be preferred with the initial turn into the wind. Get the winds before you make the turn or you had best go as published.  When a crosswind exists at the point for the procedure turn the 45 degree outbound leg should be 40 seconds plus lengthened or shortened one second for every degree of wind correction required on the outbound leg.

NOS charts just show an arrow to the turn side.  Jeppesen uses the 45-180. Be sure to advise anyone in the cockpit of what you plan to do. Try to use the procedure that will most reduce the effect of the wind.  ATC usually requests that you advise when 'procedure turn inbound'.

An ILS should be timed passing the non-precision FAF (Maltese cross).  This guarantees positional awareness if the glideslope fails and you need to continue with a localizer-only approach.  If the glideslope should fail you can climb but you cannot turn until the localizer time runs out.  The obstacle clearance allowances are much narrower on the approach side of the localizer transmitter than on the missed approach (back course side).  The FAF for an ILS is the point you intercept the glideslope at the designated altitude on the chart. You can always ask ATC for a marker or fix call or even your position if you are in a radar environment.  (See Livermore chart)  Every pilot flying an ILS must understand the chart symbology both for a full ILS and for the Localizer approach.  There is a difference between a precision FAF and a non-precision FAF

# NoPT (No Procedure Turn)

Procedure turns are charted at most general aviation airports but are more often not required due to radar vectors.  The continue charting of the turns is because the possibility of radar failure always exists.  Flying the full procedure does train the positional awareness that is frequently lost under radar vectors.  Some GPS approaches will have procedure turns to conform to historical practice an no other reason.

There is often confusion as to whether or not a pilot is required to do a procedure turn and confusion on the part of ATC when a pilot performs the maneuver when none is expected.   ATC can stop the confusion by making their approach clearances that are expected to be straight in include the term “straight in”.

If the procedure turn is published and you are not vectored to the

final approach course, you are expected to fly the procedure so as to perform the procedure turn.

# IFR Descents

Approach descents

- Must have clearance

- Must be on published route, or

- Descend when:

Cleared for approach and on a published route.

Do not descend flying direct to a fix unless on a published approach.

# Clearance on unpublished route

- ATC gives minimum for crossing fix, or

- Maintain last assigned altitude

- MSA for emergencies only

# Procedure

Procedure 1:

Gear down at FAF or downwind.

Procedure 2:

Gear and Flaps at initial descent from cruise.  This allows 		early detection of gear/flap problems and allows power to be

retained to avoid shock cooling.

# Cautions:

- Keep the engine happy by maintaining some power.

# Descent (Decision) Height

This is the height at which a decision must be made during a precision approach to either continue descent or to execute a missed approach.   Since a descent below the DH is not allowed unless the choice between the two decisions is made, the choice must be made before reaching DH.  This means that the required visual references must be acquired before reaching DH.  Every pilot should have personal minimums as to the DH and should consider executing the missed well before reaching the DH.

The ability of a Part 91 operation to do almost anything that is not in the IFR rules allows the pilot to descend to DH and then take a look.  Part 91 operations do not require the pilot have reported weather at the

destination.  He can take a look.  POH performance charts are predicated on a TCH of 50' over the threshold.  A displaced ILS or VASI that result in a Threshold Crossing Height (TCH) greater than 50' is going to affect your landing distance.

# Airport in Sight

There are many illusions associated with the transition from IFR approach to the VFR landing.  You don’t rely on ground contact.  Other illusions of distance, position, are both possible and likely when looking for the airport and runway.  Reaching minimums and then finding the airport is not a recommended single pilot operation.

As a single pilot you want to reach visual conditions before reaching MDA or DH.  The sooner the better so as to give you a better opportunity to locate the required runway indicators.  Regardless, you remain on the ILS, localizer, or heading that will take you to the runway.  If circling is required you must not lose sight of the airport nor enter the clouds.  Aircraft control is the essential ingredient.

The winning hand on any IFR approach is to have the airport in sight.  If visual conditions are marginal you must have horizontal visibility, especially for circling to land.  The pre-determined missed approach option is always there as a choice.

# Decent Procedures

- On an ILS approach, once you have descended below circling minimums as given for the localizer approach there is no going back up.

- ILS FAF in case you lose the glide slope. Punch your clock.

- Failure to note the time at the Localizer FAF requires you to execute a missed approach any time you lose the ILS glide slope inside the FAF.

- This does not apply if you can identify the missed approach point by other means.

- Move the throttle so that the changes can be heard.

- Set the power to give the speed required during the descent.  It is power management that holds the glide slope.

- A 3-4 degree heading change will change needle by one dot.

- Tendency is to fly the needle instead of the heading indicator as

you get below 800 feet.

- It is better to accept needle two dots off than to make violent maneuvers.

- The wind does not always decrease at lower altitudes.

# Initial Approach Fix

The FAA has determined (1994) that the IAF is a required route in a non-radar environment unless you have a contrary clearance.  The IAF can be part of the enroute flight where the enroute 'feeds' traffic into the IAF.  Sometimes these 'feeder routes' are not part of the enroute structure.  The instrument approach begins at the IAF.  The IAF may be along an arc, radial, course, heading, vector, or any combination of these.  The IAP chart assures terrain and obstruction clearance and contact with the airport.

# Uncontrolled Airport Arrival

It is possible to operate IFR into and out of just about any airport and you can get into some fields even if they don't have approaches. You are filing an IFR flight plan into an uncontrolled airport.  There is not instrument approach.  The airport has 50 mile visibility and ceiling unlimited.  An alternate airport is required!  The use of or dependence on a phone both to file and close can create problems since the "single-threading" (one aircraft at a time) of traffic may effectively exclude other IFR traffic due to phone communications failure.

If you should file an alternate and decide to fly there instead of your primary desination, you must refile another alternate.  The pop-up and tower enroute flight plan may not be considered by ATC as a legal flight plan since no alternate is given.  The correct way to file an IFR plan is through Duats or an FSS.  An air file should be done by relay to the FSS.

If in the course of a flight you find that updating weather indicates that your alternate is not legal, find and file for one that meets requirements.  The new selection should appear in the remarks section of the flight plan.  Every IFR flight is required to have an alternate for IFR operations if the destination is expected to be less than 2000’ ceiling and 3 mile visibity for one hour each side of your ETA.  The alternate must have either a precision approach with at least 600 and 2 or a non-precision with 800 and 2.

AWOS/ASOS as now being installed at uncontrolled airports gives a pilot a head start on predicting the need for an alternate.  Get the AWOS/ASOS as far away as possible and get an indication of any trend.  The trend can motivate an early departure for an alternate.  When planning a flight in near-minimum conditions remember the requirements, select obtainable alternates and keep alternate alternates open.  Failure to depart for an alternate in a timely manner is an indication of poor planning and judgment.

The IFR clearance to an uncontrolled airport is not a clearance to land.  ATC would (normally) like you to cancel IFR well before you get to the airport if conditions permit.  Otherwise, you must cancel by phone after landing.  They have a way of suggesting for you to close in flight.  They remind you be sure to close after landing.  If there is the possibility of a missed approach do not cancel your approach into an uncontrolled airport.  In the event of an accident you will be out of the system and ATC has no way of knowing that you are overdue.  There are enough cracks in the system without you falling through one of your own making.  You may still have advisories but any warning of minimum safe altitude (MSAW) and low altitude alert (LAAS) will not be there.

It would be best to monitor the advisory frequency during the approach for other traffic on your #2 radio.  When told to contact CTAF be sure to advise traffic that you are on the IFR approach.  If circling, maneuver shortest path to base or downwind that conforms with the local direction of traffic so as to avoid conflict with a local requirement.  Try to overfly to determine runway and pattern. See and be seen applies. Use the radio.  Just because you are IFR does not mean that you have priority over VFR traffic at an uncontrolled airport.  Your IFR straight-in approach does not give the traffic "see and be seen" advantages of the preferred 45 entry.

### VFR-on-top

You file an IFR flight plan with VFR-on-top.  At 180

degrees M.C. you should fly at even plus 500' altitudes.  A pilot must comply with VFR visibility, distance from cloud criteria and minimum IFR altitudes from FAR Part 91.  ATC services for separation in Class B, Class C and TRSAs.  FAR 91.159, AIM Para 266, and FAA Order 7110.65.

You can file (through the remarks section) for a climb to VFR conditions with stated intention of canceling IFR on reaching VFR conditions.  This clearance must be asked for by the pilot.  ATC will advise of any top reports if available.  If you reach a clearance limit before reaching VFR you can expect to hold.  Don't ask for a climb to VFR unless you are certain you will reach VFR at the altitude you've selected.

When traffic prevents a climb or descent on an IFR flight plan being made in visual conditions, you can request a VFR-on-top.  When the VFR-on-top is approved by ATC you must obey both IFR and VFR FARs.  Your altitude must be correct for VFR hemispheric rule and above IFR minimums.  VFR cloud clearances and visibility are required and you are liable for your own traffic avoidance.   IFR position reporting and following ATC clearances is required.  Altitude changes must be reported to ATC.  When being vectored by center in an area where there is no MEA you should query ATC to confirm that the MVA (minimum vectoring altitude is also the minimum IFR altitude.

# Cruise Clearance

A cruise clearance allows a pilot to fly from the minimum IFR published altitude up to the maximum specified in the clearance.  If you can't get there from here, refuse the clearance.  If you don't know minimum altitudes ask ATC.

# IFR in VFR

IFR operations have radar separation that is not available to VFR

operations.  The assumption is that VFR aircraft can only be in VFR conditions.  Under certain mixed flight conditions these two flights could merge with out warning.  In Class B airspace the radar service must provide separation.  VFR traffic on a practice IFR approach  places primary and ultimate separation responsibility on the pilot.  VFR traffic has a low priority for radar separation.

The VFR-On-top is requested by the pilot who is allowed to select an altitude below Class A airspace.  This clearance requires you to operate in basic VFR conditions while following IFR strictures related to minimum altitudes  and radio procedures.  In Class B airspace this clearance shows on radar as VFR but gives IFR separation

A pilot can reject or refuse an ATC initiated visual approach.  An accepted or initiated visual approach requires the pilot to proceed visually and clear of clouds to the airport.  FAR 91.155 requirement for VFR minimums does not apply.  The pilot may proceed visually and clear of clouds. The visual approach does not contain a missed procedure so you may need to expedite your request for an new clearance.  The visual approach may be given by ATC at uncontrolled airports when VFR conditions are expected to prevail.

The Charted Visual Approach uses an IFR chart to show a route at a minimum altitude which navigates by visual markers as charted. The visual approach eliminates IFR procedures is NOT an IFR procedure.  The charted altitudes are minimums and suggested only.  There is no missed approach.

Additionally:

- Radar service ends when changing frequency.

- Flight plans must be closed by the pilot at uncontrolled fields.

# Uncontrolled Airport Departure

IFR pilots tend to believe that ATC will keep them clear of obstacles.  Except for airports with published instrument departure procedures, ATC does not provide terrain and obstruction clearance--not until you reach a published route, minimum vectoring altitude or minimum instrument altitude.  In a mountainous area an aircraft may not have the required IFR departure performance.  Your option is to request a "visual" (not VFR) which ATC understands to be visual climb as part of the IFR flight during which the pilot is responsible for obstacle avoidance.

The only place you will find terrain avoidance information will be on a current sectional or TCA chart.

Only 7% of airports have control towers. For IFR you must know how to depart safely, how to get your clearance, and what to do if you decide to go back.  Some airports have approaches but no departure procedure.  Usually you will be told in your clearance to proceed to a particular fix on entering controlled airspace.  Use a departure procedure to do this if it exists, otherwise, make your own safe routing.  Controlled airspace is not necessarily obstacle free.  The IFR departure procedure is solely for obstruction avoidance.  On filing you have a choice of filing a SID or of following the ATC departure procedure.

A pilot may do an approach to a controlled airport but cancel on the approach in VFR and proceed to another nearby airport.  The ETE destination point on an IFR flight is the point of intended landing.  FAR 91.169 which refers you to FAR 91.153(a)(6). Pilots often file to an IFR airport with intention to cancel and diverting to actual non-IFR destination.

You must show up at the place and time ATC expects.  How you get into controlled airspace and to minimum vectoring altitude (MVA) is up to you but until you reach MVA obstacle avoidance is a pilot responsibility.  If you are in doubt about a heading breing a vector with ATC responsibility just tell ATC that you will fly the IDP.  In uncontrolled airspace you don't need a clearance.  Up to 700 or 1200' AGL you can fly in the fog but you must be VFR above that.

There are several ways to get an uncontrolled airport IFR departure clearance. A remote communications outlet (RCO) may be within range,  A nearby ATC facility (tower) may be used.  You can use a phone and get a void-time clearance or even depart and get your clearance in uncontrolled airspace. Your initial clearance will be to a nearby fix not to your destination. The fix location usually assures radar contact before reaching.

A departure procedure exists if an obstacle is determined to affect plane of departure from either end of runway.  Minimum is 200 feet per nautical mile unless higher minimum rate is specified.  NOS uses T and Jeppesen lists it on airport page.  Pilot is responsible even if radar is available.  Radar assumes responsibility for obstacles only when initiating vectors.

# Clearance Void time

A clearance or clearance void time at an uncontrolled airport must be obtained through ATC/FSS once your reservation becomes available.  A clearance void time (CVT) from an uncontrolled airport requires that the takeoff occur before that time.  Radio contact time is not mentioned. Departure at that time is not authorized.  FAR 91,173.  The void-time clearance usually has a 10 minutes or less time during which you are expected depart to establish radio contact.  In non-busy environments a longer time may be requested. The void-time clearance will not contain departure instructions but will tell you that on entering controlled airspace to proceed to a certain fix. ATC must be advised within 30 minutes of intentions from the ground if departure cannot be made before that time.  If no ATC notification is made the aircraft is considered missing. This automatically cancels the IFR clearance but the IFR flight plan is still in effect.

A VFR departure with an airborne IFR pick-up of a pop-up clearance is both practical and safe.  It is much more efficient than a void-time clearance and is to be preferred by pilots.  ATC likes it better, too.

# Hold for Release

ATC has you in the starting position for your flight.  You are under IFR procedures now.  You cannot depart (VFR or IFR) after being told to "hold for IFR release" without ATC acknowledgment and concurrence.

# Controlled departure time

This is not a clearance void time  A CDT is an ATC procedure to reduce traffic congestion.  It is a reservation that may or may not come on time.  You are expected to allow 1000' vertical (2000' in mountains) and 4 miles horizontal from highest obstacle.

# Non-radar IFR

Non-radar separation is premised on timely and accurate radio position reports along with assigned altitudes.  If radar contact is lost while on an IFR flight, the pilot is expected to resume reports at shaded triangle intersections (mandatory).  Use PTAEN mnemonic for position-time-altitude-estimate-next-name.  On request position reports are at open triangle intersections.  See FAR 91.183.

If you should miss a mandatory reporting call, the controller

is required to query you after five minutes.

ATC is required to increase separation when radar contact is lost.  The controller has to make computational estimates so that your aircraft, based on its past performance, will not conflict with approaching, following, or crossing traffic.  Standard separation is 5 miles to the side, and 1K vertical to 29,000 where it becomes 2K.   Without radar separation must be 10 minutes or 20 DME.  At this juncture it becomes vital that pilots advise ATC when ETA change by three minutes or more.  With a change in ETA expect ATC to give more complicated clearances and holds.

Pilots can help in the non-radar environment by limiting requests for deviations and by giving accurate position reports and estimates.  Non-compulsory points need be reported only on ATC request but you may wish to report them anyway.  It is always a good idea to make periodic reports just to make sure ATC is still there and caring about you.  Mistakes in trip routes happen.  Such errors can be picked up by ATC if your position reporting follows the correct format.

You are IFR in controlled airspace and the #1 VOR (only one with glide slope and localizer capability) fails.  #2 has no ILS capability.  You should report the malfunction immediately.  FAR 91.187

When faced with a non-radar transition in some remote areas or in event of a Center radar failure as does happen, you are expected to do the entire approach procedure unless:

1. You are vectored to final approach course or a segment of the 			initial approach procedure.

2. You are making a visual approach

3. You are making a contact approach

Any non-radar approach must begin at an IAF followed by a course reversal unless charted as NPT, timed from a holding fix, or not published course reversal.  Go to and stay on published routes to assure yourself obstacle clearance when out of radar contact.

If you are not getting vectors you must have studied the published route to the transition to an approach.  It is well advised to contact the approach prior to becoming airborne to pre-determine minimum instrument altitudes and minimum vectoring altitudes.  I have seem minimum vectoring altitudes within 100’ of terrain.  Scary!  Approaches at an airport may have different required transition altitudes.  Get too low and you may not be able to make every approach available.  By staying at published altitudes you are assured of both lateral and vertical obstacle clearance but not that every approach is available.  You may wish to stay higher than published altitudes to retain all approach options.  Without radar it is the pilot who must avoid running into anything.  When flying in unknown areas this requires special effort.  If the engine quits head for the lowlands.  There you will find people.

When flying a non-radar procedure, you are expected to fly the charted approach along with the procedure turn, if required.  Even if VFR conditions prevail you must fly the entire procedure unless you cancel

IFR or get a visual/contact approach.

# Non-radar Approach

- The approach must begin at an IAF.  Full procedure must be flown to assure gradients, communication and safe obstruction avoidance.

- A DME arc is not allowed as a substitute for an IAF.

- A charted procedure turn in an approach is required if the pilot is cleared by ATC for the approach except when:

- Radar vectored to final approach

- Making a timed approach from a holding fix.  A timed approach can 	  only be made with a tower open and ATC advising that such     	  	  approaches are in progress.

- On a NoPT segment.

- VOR Failure/Errors

- Where VOR head fails, 'OFF" flags will not appear.

ATC will provide separation and seal off the airport airspace as the classification allows.  If out of radar contact you are expected to use AIM 5-3 reporting procedures.  That means you will continue to report leaving an altitude, missed approaches, VFR-on-top altitude changes, holding reports, equipment failures, the FAF inbound and any ATC requests such as procedure turn inbound, a specific fix and canceling IFR.

# Select a heading:

Reference Heading is the heading that will keep the needle centered.  Because of wind you must bracket this heading through referral to the Heading Indicator  One system uses 1/2 angle corrections of ever decreasing amounts.  (see instructor)

Initial changes to find the reference heading should be 5 degrees.  (10 degrees only if 1/2 deflection.)  All turns by reference to heading indicator. Stabilize your heading and airspeed based on the wind.

# Visual Descent Point

- Where a non-precision approach has DME, FAA policy is to give a 			visual descent point VDP.

- Pilots use of a VDP is optional.  Localizer only is not to have 			VDPs.

- VDP  requires local altimeter setting.

You can adapt the approach time and the MDA given off the plates to a VDP.  Convert the first two digits of the altitude to seconds and subtract those seconds from the approach time.  You should be able to make a very normal approach from the MDA from this reduction in time, providing the runway is in sight.