Questionsã

1. What is the purpose of an FDC notam?

2. To get a visual approach, you must be...

3. What are minimum requirements for a contact approach.

4. What differentiates a visual from a contact approach?

5. What specifics are mentioned as being part of “all available 	information for IFR flights?

6. Must ATC advise you if a facility is notamed out of service?

7. ATC is required to pass along PIREPS?

8. What pilot response is expected when you are cleared for descent at 	pilots discretion?

9. What is the significance to the pilot of an ATC directive to do 	something when able?

10. When are pilot reports required?

11. You must know the weather before shooting a Part 91 approach.

12. When can you log an approach as IFR?

13. Can an FSS provide separation where no tower or approach exists?

14. When is a missed approach procedure not an assumed option?

15. What separation remains with ATC even on a Practice approach?

16. What obstacle clearance do you have on feeder routes?

Answer

1. FDC notams are regulations covering changes in instrument approach procedures and temporary flight restrictions.  FDCs must be location specific to DUAT.  FDCs within 400 miles are maintained by FSS until published in biweekly Notices to Airmen Publication (NTAP).  FDCs are not included in weather briefings unless asked for.  (See AIM 5-1-3 for NOTAM contractions)

2. Aircraft on IFR flight plan while airport is VFR, in sight, and traffic to follow also in sight.

3. Aircraft on IFR plan may request contact approach if they are clear of clouds and expect to remain clear of clouds and have one mile flight visibility to the airport.

4. Pilot must request contact approach.  ATC can assign visual approach which must have higher weather minimums.

5. A pilot must have weather reports and forecasts, required fuel plus reserves, flight alternatives, known traffic delays, and airport runway lengths.

6. ATC must give you alternative routs and may tell you why this is required.  Ask.

7. ATC is required to pass pertinent flight information, including PIREPS but it doesn’t always happen.

8. A pilot should always report leaving last assigned altitude.

9. Any clearance or direction by ATC for the pilot to perform something when able, means just that.  The performance is entirely at the pilot’s determination of “able”.

10. Pilot reports are required in the event of equipment failure, leaving an assigned altitude, departing a holding pattern, making a missed approach...

11. Not required for Part 91 to know or have weather.  Take a look is

authorized.  Better have landing visibility minimums if you land.

12. If any part of the approach is flown in IFR conditions the entire approach can be logged as an IFR approach.

13. It is unlikely that an FSS will give more than relayed clearances, releases, and weather.

14. The missed approach may not be flown on VFR or Practice approaches

unless specifically requested and approved by ATC.

15. ATC is always for runway separation and all ground separation.

16. Feeder routes have normal obstacle clearance of 1000’ and in mountains 2000’.

# Question

1. What is the course width of an ILS localizer.

2. What is the course width of a localizer-type directional aid(LDA)?

3. What is the course width of a simplified directional facility (SDF)?

4. What is the course width of a glideslope?

5. What is the standard useable distance for using a glideslope?

6. What is the standard useable distance for a localizer?

7. The middle marker is how far from the runway?

8. The marker light that flashes on a back course is what color?

9. On an ILS approach the white light indicates what marker?

10. On an ILS approach the color of the light over the middle marker is?

11. The Standard with of the localizer back course is?

12. When is it acceptable to descend 50' below DH?  (FAR 91.175)

13. Can you descend into ground fog from clear conditions to below DH?

14. What are the three conditions that allow descent below DH?

15. What does "Cleared for the approach" mean?

16. What is the controlling visibility for Part 91 operations?

17. When can a pilot change from an ILS to a Localizer approach.

18. Is a Part 91 pilot required to abide by an IFR departure procedure?

19. Do monitors shut down defective glide slopes?

Answers:

1. 3-6 degrees full width to give 700' at runway threshold. (AIM 1-10(b)(2))

2. Same as ILS above. (AIM 1-10 (c)(1))

3. 6-12 degrees adjusted for safest operation. (AIM 1-11 (f))

4. 1.4 degrees vertical width.  (1-10 (d) (3))

5. Standard glideslope distance is 10 nautical miles.  (AIM 1-10 (d) (3))

6. Course guidance is 18 nautical miles up to 4500' above antenna and 1000' 	above course terrain.  (AIM 1-10 (B)(5))

7. Middle marker is 3500' from threshold.

8. Back course marker is white.

9. On the ILS a white marker indicates the inner marker.

10. The middle marker light is amber.

11. The back course localizer at the threshold is 400' standard.

12. According to the FAA, "Never".

13. No

14. 1) Ability to execute normal landing; 2) Visibility not less than 	charted; 3) Required visual runway references (10 of them) (91.175)

1. Approach lights

2. threshold

3. Threshold markings

4. Threshold lights

5. End identifier lights

6. Visual approach slope indicator

7. Touchdown zone or markings

8. Touchdown zone lights

9. Runway or runway markings

10. Runway lights

15. Fly the approach as charted at charted altitudes or higher

16. Part 91 operations are controlled by flight visibility as determined by 	the pilot.

17. A pilot can change from ILS to Localizer any time before descent to 	localizer minimums.  An additional ATC clearance is not required.

18. A Part 91 pilot need only to abide by the procedure that is in the ATC 	clearance.

19. Old ones did but no longer.  You must check flags.

Question

1. Is a pilot required to begin an approach at an IAF?

2. When is an approach an IFR approach?

3. Which notams are about navigational facilities, public airports, 	etc.?

4. Which notams are about taxiway closures, taxiway lighting, and 	beacons?

5. Which notams apply to regulations, charts and flight restrictions?

6. What is the requirement to advise ATC when you change departure time?

7. What must you do if you miss a clearance void time takeoff?

8. How is obstacle clearance provides during an IFR departure?

9. When must you advise ATC of a change in airspeed?

10. When should ATC give a clearance beyond your clearance limit if 	there are no delays?

11. What does MSA (minimum safe altitude) have to do with approaches?

12. What are the limitations of the MVA.

13. What does 'cleared for the option' mean on an IFR approach?

14. How is a VDP (visual descent point) used?

15. What is the meaning of 'minimum fuel advisory"?

16. During a visual approach, when is radar service terminated?

17. What is the minimum climb rate required for an unpublished IFR 	departure?

18 Transitioning from enroute to an approach transitions what airspace?

19. What is the universal definition of uncontrolled airspace?

20. What clearances can be issued to a pilot by ATC without being asked?

Answers

1. There is no requirement to begin an approach at an IAF.

2. An IFR approach is an IFR approach when any part of the approach is 	IFR or under the hood.

3. Distant notams AIM 5-3

4. Local notams  AIM 5-3

5. Flight Data Center notams, FDCs.  AIM 5-3

6. Notify ATC if you are delayed more than an hour.   AIM 5-11

7. You cannot depart after a clearance void time and you must advise ATC 	within 30 minutes. AIM 5-23

8. It is the pilot's responsibility as to procedure and obstacle 	clearance. AIM 5-25

9.  If your flight true airspeed varies from your filed true airspeed by

5% or 10 knots, ATC expects to be advised.

10. ATC should issue a clearance beyond the fix as soon as possible and 	at least five minutes before reaching the clearance limit. AIM 4-	82

11. MSA is for emergency use only.  Measured from NDB or VOR.  AIM 5-44

12. MVA always provides 1000' terrain clearance but may not provide 	2000' in mountains. AIM 5-44

13. "Option" means you can make low approach, missed approach, touch-	and-go, stop-and-go, or full stop.  AIM 4-71

14. Visual descent points are used only on non-precision straight in 	approaches.  AIM 5-44

15. Minimum fuel advisory by the pilot does not give priority.  AIM 5-84

16. Radar service is terminated when you are told to contact the tower.

AIM 5-80

17. 200 fpm.

18. Transitions use airspace from 700/1200 AGL to the base of the 	overlying 	airspace.

19. ATC does not control aircraft in uncontrolled airspace.

20. ATC can issue a pilot a visual approach, STAR, or SID when it wants 	to.

Question

1. At what distance from the airport do you usually intercept the 	glideslope?

2. Why do some glide slopes prohibit coupled approaches?

3. Marker crossing altitudes on the charts are given above (what)

4. How far out can you intercept and track a glideslope?

5. What is required for ATC to authorize visual separation?

6. What are minimum visual separation standards?

7. What are Class B visual separation standards?

8. What are visual separation standards if radar is not available?

9. What is the effect of a pilot’s admission of visual contact?

10. Where is visual separation not allowed.

11. On accepting visual separation what does ATC expect of the pilot?

12. Can two departing IFR aircraft request ‘visual separation’?

13. What is the visual separation limitations for Class E airspace.

14. What will the alternate static source do to instrument readings?

15. What is the effect on the VSI of a blocked static source?

16. Why do we vary the angle of attack during a level turn?

17. What procedure will correctly give the climb lead necessary to level 	off at a given altitude?

18. What factors influence how to lead a level off from a descent?

Answer

1. Glide slopes are usually intercepted 10 miles from the threshold.

2. If terrain caused glideslope problems can't be fixed, no coupled 	approaches are allowed.

3. Both NOS and Jepp as MSL and AGL above TDZE.

4. Watch out for false slopes but there is no regulation saying when to 	intercept.

5. Conditions must allow continual visual contact of one aircraft with  	another.

6. Pilot decision determines visual separation minimums.

7. See answer # 6.

8. Radar is not a factor is\n visual separation.

9. Once a pilot has visual contact, separation responsibility rests 	entirely with the pilot.

10. Class A airspace does not allow visual separation.

11. The pilot is expected to maneuver to maintain visual separation once

it has been granted by ATC.

12. Two departing IFR aircraft can be granted ‘visual separation’ by ATC 	but the entire burden of such responsibility rests on the pilot.

13. There are no visual separation limitations in Class E airspace.

14. With an active alternate static source the altimeter will read 	higher than it should.  Indicated airspeed will read higher than 	it should.  The VSI will show initial climb.

15. Whatever the VSI is reading at the moment of blockage will remain 	constant regardless of the aircraft behavior.  The VSI works on 	differential pressures of a sealed chamber and the static air 	source.

16. In a level turn the vertical lift component is decreased by the 	amount of force required to make the turn.  To hold altitude, this 	decrease must be compensated by an increase in angle of attack.

17. The easiest way to lead your leveling off at a given altitude is to

use 10% of your vertical climb speed.

18. The lead required to level off from a descent is determined by the 	speed of descent and the selected level airspeed.  If descent and

level are to be at the same speed use 10% of descent rate.  If 	level speed is to be higher then bring up power at 20% of descent 	rate point and hold nose for speed increase but allow to rise at 	% point for level flight at higher than descent speed.

Question

1. If the ATIS does not give sky conditions you are supposed to know...

2. How does 'see and avoid' apply to IFR flight?

3. Where is VFR-on-top not allowed?

4. How do you determine a proper altitude if cleared VFR-on-top?

5. What reports are required when VFR-on-top?

6. What reports require a specific ATC request?

7. At what point should the outbound timing begin when holding at an 	NDB?

8. When cleared for a visual approach, when can you commence descending?

9. Are you required to report leaving an altitude?

10. What does 'cruise 8000' mean?

11. What climb/descent rate does ATC expect from you?

12. What does 'expedite' mean when used by ATC over the radio?

13. Breaking the VSI glass will restore the pitot-static system except for 	the VSI.  When will the VSI read accurately?

14. How long will a terminal forecast (FT) be valid?

15. Transcribed Weather Broadcast route forecasts are issued twice a day 	and are valid for how long?

16. How will a briefer inform you that flight under VFR is questionable?

17. Why would an airmet not be issued over an area with moderate icing, 	moderate turbulence, 30+ knot winds, and ceilings of below 1000' 	and visibility less than three miles with extensive mountain 	obscuration?

18. Where can you expect microbursts.

19. Why is holding prohibited over the areas of a glideslope?

Answer

1. No sky condition in the ATIS means ceiling over 5000' and 5 mile 	visibility.  AIM 4-13

2.  When conditions permit IFR pilots are expected to "see and avoid".

FAR 91.113

3. No VFR of any kind is allowed in Class A airspace.

4. All VFR flight must comply with the hemispheric rule as determined by 	Magnetic Course when 3000' or more AGL.  far 91.159

5. VFR-on-top flights must give reports as though non-radar IFR.

6. ATC must make a specific request for a FAF report.

7. When abeam the NDB the outbound timing should begin.

8. Descent is at your discretion unless restricted by ATC.

9. It is recommended in AIM 5-33 that you report leaving an altitude.  I 	feel that ATC likes to know a pilot is competent and 	knowledgeable.

10. It let you fly at any altitude from the IFR minimum to 6000'.  You 	can climb and descend at your discretion but you cannot climb 	again if you have reported to ATC that your are leaving an 	altitude.  A cruise clearance is also a clearance to your 	destination.  Rarely used.

11. ATC expects a minimum of 500 fpm.  According to AIM 5-33 you are 	expected to advise ATC if you are unable to perform.

12. "Expedite' is used by ATC when immediate compliance is required.

13. Level flight.  All climb and descent readings will be backward.

14. 24 hours

15. The morning forecast is valid for 12 hours.  The evening for 18 	hours.

16. "VFR flight is not recommended.

17. An airmet will not be issued if the information in the terminal 	forecast (FA).

18. Microbursts can be expected wherever there is convective activity.

19. An aircraft in a holding pattern can cause glideslope distortion.

Question

1. When does a microburst die?

2. What should you do if you enter a thunderstorm?

3. What distance is considers as safe for avoiding thunderstorms?

4. What is indicated by the presence of virga?

5. What is required for a visual approach?

6. What is required for a contact approach.

7. Where do we find the FDC notams?

8. IFR flight requires that you have all available information plus...

9. How does the AI react differently in a roll Vs a loop?

10. to recover from a steep diving unusual attitude what is the best 	instrument to use for recovery?

11. In unusual attitude recovery, why level wings before fixing pitch?

12. Why do aircraft recovery from changes in pitch attitude?

13. How can you identify the horizon in recovery from an unusual 	attitude?

14. How does the localizer of Category II approaches differ from 	standard?

15. What is the meaning of RA345 on an approach chart?

16. Why might a Category II approach have two DHs?

17. What should you expect if crossing the threshold on the glide slope?

Answer

1. A microburst becomes stronger for about five minutes and then takes 	another ten minutes to dissipate

2. Continue straight ahead, ignore altitude changes, slow to Va, don't 	turn, and stay level.

3. 20 miles.

4. Strong downdrafts with moderate + turbulence.

5. A visual approach must be authorized by ATC, an IFR flight plan, must 	have airport or preceding aircraft in sight, and be able to 	maintain VFR to the airport.

6. For a contact approach you must be on an IFR flight plan, clear of 	clouds in one mile visibility, conditions expected to remain 	enroute to airport, must be requested by pilot.

7. Every two weeks the Notices to Airmen publishes the NOTAM Ds expected 	to remain active and the FDC Notams currently effective.

8. In addition to all available information you must have weather 	reports forecasts, needed fuel, alternatives, known traffic delays 	and runway lengths.

9. AIs do not tumble during a roll.  In roll they will read correctly.  	In pitch, when a loop exceeds the 80-degree stops in up/down pitch 	and then slowly read correctly.

10. If the dive is not inverted, the turn coordinator is easiest to use.

11. Leveling wings reduces load factor and prevents a spiral dive.

12. Positive dynamic stability is the design quality of an aircraft 	which allows it to make a self-recovery from pitch changes.

13. The horizon can be identified in unusual attitude recovery when the 	VSI, altimeter and airspeed reverse trend direction.

14. Category II localizers are no more accurate but signal quality is

higher.

15. RA 345 on an approach chart is a radar altitude which can only be 	determined if you have a radar altimeter.

16. After a Category II pilot has gained experience he can take lower 	DHs.

17. Crossing the threshold on the ILS on the glide slope will give 	touchdown at the 1000' markers.

Question

1. Are glide slope and localizer equipped for shutdown on failure?

2. Why are low altitude holds prohibited over any part of the ILS?

3. What is the service range of an ILS signal?

4. What are false glide slopes?

5. Can you intercept a glide slope from far away and fly it inbound?

6. How does a attitude indicator precess under acceleration?

7. What kind of bank does a coordinated turn show on the turn 	coordinator?

8. What is the best way to increase the rate of turn while decreasing 	the radius of the turn in a standard rate turn?

9. What is the effect of temperature on true airspeed?

10. What is the effect of true altitude on true airspeed?

11. How do you determine pressure altitude?

12, With an altimeter setting of 30.43 at an altitude of 6000 feet what

will the pressure altitude be?

13. When making a standard rate turn how fast are you turning?

Answer

1. Shutdown is not automatic.  Always check flags for glide slope and 	localizer.

2. Aircraft at moderate altitudes and lower can cause localizer 	distortion.

3. ILS standard service volume is 10 miles but may be farther.

4. False glide slopes are allowed to exist above the published 	glideslope.  The key identifier is the crossing altitude of the 	marker.  False glide slopes will have a steeper angle of descent.

5. You can fly a  glideslope from beyond 10 miles out but you have no 	way to determine if it is false or true until you get an altitude 	check.

6. When accelerated the AI moves down so that corrections will cause an

unwarranted descent.

7. Turn coordinator does not show bank.  It only shows rate of turn 	regardless of airspeed.

8. If maintaining a standard rate turn it will be the same regardless of

airspeed so rate cannot be changed.  The radius of the turn can be 	reduced by slowing the TAS of the aircraft.

9. True airspeed will vary directly with changes of temperature.  TAS 	increases as temperature rise and decreases as temperatures fall.

10. True airspeed will vary directly with changes of true altitude.  TAS 	increases as true altitude rises and decreases as true altitude 	becomes lower.

11. Pressure altitude is determined by setting altimeter to 29.92.

12. The difference is .51 or 510 feet. Pressure altitude is 5490 feet.

13. Three degrees a second, thirty degrees in 10 seconds, 90 degrees in

thirty seconds, 180 degrees per minute.

Question

1. What common flight instrument is not required for IFR flight?

2. Where do false glide slopes exist?

3. How do you overcome spatial disorientation?

4. What kinesthetic sense is created by a smoothly coordinated 1.5G

turn?

5. When can level flight from an unusual attitude can be noted on the 	instruments?

6. How do you recover from a spiraling descent?

7. What the difference between flying "over the top" and "on top."

8. What constitutes the end of radar service?

9. When on an IFR flight, ATC advises that radar contact has been lost.

What do you do?

10. Non-radar separation standards are?

11. What depends on your distance from the antenna?

12. When cleared for approach why might ATC issue an altitude 	restriction?

Answer

1. The VSI is not a required IFR flight instrument.

2. False glide slopes are permitted to exist only above the real one.

3. Spatial disorientation cannot be prevented.  It can be overcome by

total reliance on the instruments.

4. The illusion produced is the entrance into a climb if the head is

held erect eyes covered but open.  With the eyes closed the 	illusion is that of a dive. (needs confirmation)

5. Approximate level occurs when airspeed and altimeter slow their rate 	of change and the VSI reverses.

6. Recovery from a spiraling descent is best achieved by power 	reduction, leveling of wings, and raising the nose.  The closer 	together these occur the better will be the recovery.

7.  "VFR over the top" is when a VFR pilot flies over a ceiling

"IFR over the top" is the act of flying VFR over a ceiling on

an IFR flight plan

"VFR on top" is the clearance an IFR pilot must request while

on an IFR flight

8. The controller may make a specific statement or convention says that 	being told to contact the tower or the advisory frequency 	constitutes a termination of radar service.

9.  When radar contact is lost resume making all standard and requested 	position reports.

10. Single - thread procedures usually prevail in non-radar situations.

One aircraft at a time is taken in or out until radar can make 	contact and provide separation.

11. Terminal separation depends on the aircraft distance from the 	antenna.  3 miles is standard within 40 miles and 5 is standard 	beyond that the same as Center.

12. Any time ATC issues an altitude restriction as part of an approach 	clearance you should assume that an obstacle is responsible.

Question

1. What is the IFR departure required for Part 91 aircraft in Class D

2. What is unusual about Part 91 IFR takeoff minimums?

3. Where takeoff minimums exist for FAR Parts they relate only to...

4. At what altitude AGL may an IFR turn be initiated?

5. How are commerical operations different than Part 91?

6. Aircraft approach categories under FAR97 are determined by...

7. If an emergency requires deviation under FAR 91 you must...

8. ATC priority handling by ATC in without violation of FAR 91 	requires..

9. Part 91 takeoff minimums are...

10. On an IFR approach when must you execute a missed approach?

11. When on a VASI runway you are required to...

12. What is the maximum speed allowed in Class D airspace?

13. Who determines if an aircraft is airworthy?

14. Who determines if an aircraft is not airworthy?

15. Who deterimes if an aircraft is in compliance with required

Part 91 inspections?

16. Do all aircraft have minimum equipment lists?

17. Can an aircraft be flown without an ELT

18. What are the basics of pilot responsibility regarding seatbelts and 	seats in aircraft?

Answer

1. Part 91.129 requires the Classes B, C, and D fly the IFR departure 	where one exists.

2. There are no departure minimums for Part 91 aircraft.

3. Other than Part 91 the other FAR parts are limited by visibility.  	Where climb requirements higher than standard exist there may also 	be ceiling minimum.

4. 400’

5. Commercial operations must have approved weather and alternates where

field is below landing minimims.

6. 1.3Vso at maximum certicated landing weight

7. Send in a written report of that deviation on FAA request.

8. A written report to the facilty on request within 48 hours.  FAR 	91.123(d)

9. There are no takeoff minimums under Part 91.  FAR 91.175

10. On being below the MDA without at least one of the visual 	requirements in sight.

11. When on a VASI runway you must remain at or above the VASI 	glideslope.

12. 200 knots  FAR 91.117(b)

13. The pilot

14. The FAA after something happens to prove the pilot wrong.

15. The owner/operator

16. FAR 91.405 allows aircraft without an MEL to operate if inoperative

items are properly placarded and disabled.

17. An aircraft can be flown for 30 days without an ELT.

18. Each person must...a. be briefed on seatbelt operation; b. be 	notified to fasten bests before takeoff and landing; c. occupy

an approved seat with secured belts as provided for takeoff and

landings; Children under two years do not need separate seats.

Question

1. When on approach you begin to see the ground occasionally.  How

does this influence your MAP decision?

2. What do you do if you are told that braking is nil when on the

approach

3. What do you do if you find that you cannot depart before a clearance

void time?

4. When on an ILS approach you find that you are in the clear and can 	see much of the runway but must enter fog to continue the 	approach?

5. There are no runway lights as you are on the ILS approach at dusk.

You are Part 91.  What do you do.

6. If your vectors to final set you up at the marker but far too high.

What do you do?

7. what is required to land if you find that inside the marker the 	minimum visibility is below requirements?

8. You note a thunderstorm off the departure end of the runway, just as

you are cleared for departure after a 45 minutes wait.  What to 	do?

9. Do you fly if your medical is out of date by only one day?

10. after landing you find that you had neglected to set the altimeter 	on an approach that was down to minimums.  Who do you tell?

11. You are well equipped with two glide slopes.  Their indications 	differ.  What do you do?

12. Reported surface temperate is 5 degrees centigrade.  You have some

ice on the leading edge as you descend in IMC.  What to do?

13. You have the long runway in view as you break out and see that it

is covered with snow.  What to do?

14. On short final a passenger says that the gear is not down.  You have

three green.  What do you do?

15. You have just flown a very ragged approach but landed successfully;.

what do you do?

16. When within a few miles of your home field, you note that the oil

pressure is slightly low.  What to do?

17. Your flying companion does something very poorly.  What do you do?

18. What do you say to the controler if you arrive for the approach low 	on fuel?

Answer

1. Without the references required by 91.175 the missed is required.

2. Go to your alternate.

3. No go.  AIM 5-2-4

4. Missed is required.  Cancel IFR if conditions allow.

5. Not a problem for Part 91.

6. Go around and come back on altitude and speed.

7. Part 91 lets you take a look.

8. Cancel

9. No.

10. Do a phase check on yourself.

11. Checking altitude at outer marker would tell you which is o.k.

12. Continue but add speed

13. Don’t land

14. Go around and recheck gear

15. Plan some proficiency rides.

16. Get on the ground as soon as possible.

17. If it is a safety issue do not ignore.

18. “Minimum Fuel”.

Question

1. What segment is used  for the outbound heading?

2. What segment is used for the inbound heading?

3. What segment is after the final approach fix?

4. What glide slope references are available for non-precision 	approaches?

5. What is the safety margin for obstacle clearance on a Cat1 ILS?

6. When is a Mode C not required?

7. How much altitude variation is allowable in a transponder Mode C?

8. How often must a transponder be certified for accuracy?

9. In Class E airspace how high can you fly without a transponder?

10. What happens to Mode C if you are on the incorrect altimeter 	setting?

11. If a VFR aircraft is assigned a visual hold at a particular point, 	what are you supposed to do?

12. When ATC sees that your flight is compressing the allowable 	separation allowed, what can you expect.

13. If issued a clearance limit short of your destination, what are you

options?

14. What are the elements of a holding clearance?

Answer

1. The initial segment is the outbound course line.

2. The intermediate segment is the inbound course line.

3. The final segment is from the final approach fix to the missed 	approach point.

4. a non-precision approach may have a visual descent point or a

VASI or VAPI.

5. Near the runway it may be as little as 60 feet.

6. Mode C is not required below 10,000’ and when outside the Class B 	Mode C 30 NM veil.

7. If your transponder shows over a 300’ error after you have set	the 	altimeter, you may expect ATC to have you stop use of Mode C.

8. The transponder accuracy check is required for all aircraft every two 	years.

9. Any flight above 10,000 feet MSL requires the use of a transponder 	capable of Mode C.  The exception within 	2500’ of terrain.

10. Mode C altitude is determined by ATC computers that use the current 	altimeter setting.  If your altimeter is incorrectly set, it does 	not affect the accuracy of your Mode C.  You can expect ATC to 	give you

11.  You are expected to circle the point instead of flying a racetrack.

12. Vectors.

13. Query ATC as to your clearance status, option to request slower 	speed	or vectors exist.

14. Name of fix, altitude, holding direction, and EFC time.