# Click on page number for instant scrolling.

Mimimumsã

Circling to Land

Circling Not Allowed

Missed Approach

Minimums

Altitude Minimums

1) Maximum Elevation minimums

2) Below Minimums Part 91 Operations

3) Standard minimums

4) Takeoff minimums

(4a) Zero-Zero

5) Climb Minimums

6) Non-standard minimums

7) Sidestep minimums

8) Obstacle Clearance minimums

Obstacle clearance minimums chart

9) Minimum vector altitudes

10) Minimum enroute altitude (MEA)

11) Minimum IFR altitude

12) Minimum safe altitude

13) Minimums Off Airways

14) Minimum Descent Altitude (MDA)

15) Descent Minimums

16) Circling Minimums:

17) Non-ILS circling minimums

18) Straight-in minimums

19) Estimated time of arrival minimums

20) Alternate airport Minimums

Circling to land

Circling to land is a relatively dangerous maneuver.  It contains the worst elements of IFR flight.  There is a minimum obstruction clearance, a limited space in which to maneuver, an absence of visual reference, and trying to keep the runway in sight while preparing to land.  At night it is a quite a bit more than dangerous.  The required continuous turn in marginal conditions that keeps the airport in sight is hazardous.  Many approaches that are aligned with the runway will have only circling minimums.  You still may be able to land straight-in.  Set up the missed approach procedure before beginning to circle.

Because of the hazards associated with circling, flying the airplane must not be part of the problem.  You must know the power settings and airspeeds that provide some safety margins.  Trim the

aircraft for the lightest possible touch.  Let go when you look for the airport.  Practice the flying about an isolated airport in good VFR

to hone the MDA skills required in MVFR or LVFR conditions as for

actual conditions.

During training you should make every circle to land approach with as close and short approach as you can handle without exceeding a 30-degree bank and your short field approach speed.  This will give you a steep approach.  If your circle puts you on a right downwind you might opt (tell ATC) to make a left downwind circle for better visual to the runway.  Don't hesitate to take advantage of any wind to get down steeper and shorter.  You are required to keep an identifiable part of the airport in distinct view except when concealed by a banked wing.

On an ILS approach you must have a localizer approach with a circle to land option.  Most other approaches circle to land minimums published as well and straight-in.  Circling minimums only are allowed for all approaches with a letter designation such as VOR-A. No straight-

in minimums are published if the approach is more than 30° off the runway or a normal descent cannot made from MDA altitude.   However, you are allowed to land straight in with the runway in sight and an ATC clearance.  Request a 360 if you need to get down.  In a any circling approach keep the runway in sight and stay at MDA until clear of obstructions.   If the VOR plate has a runway number it means that the

approach is within 30° of the runway and straight-in minimums will be published.

If you lose sight of the runway, overfly the airport, turn to your missed approach heading, and climb.  The published missed approach is valid only when begun at the missed approach point at the MDA or higher.  You are not promised terrain clearance if you do not follow the published procedure.  No turns should be made until reaching the published missed approach point.  Any other time you are on your own.

The circling minimums presume a high degree of pilot proficiency and

familiarity with the airport.

You have planned both the circling maneuver toward the airport and the missed procedure.  You have pre-decided, because of the ATIS information, the approach angle to which side to look for the airport.  There are no required circling maneuvers.  With ATC clearance you are free to maneuver to a landing as long as the runway is in sight.  Requirements of distance from airport center (Category A 1.5 nm) and 300' clearance above obstacles are figured into the circling minimums.

Each category has a different minimum circling radii requirement that is stated in nautical miles for the aircraft category.  Conversion of this to statute miles equals the visibility in statute miles as stated on the

approach plate.  The minimukm obstacle clearance within each approach category circling maneuvering area is 300 feet. Height Above Aiport

(HAA) is 350 feet for Category A planes.  VASI and PAPI equipped runways allow descent when within 10 degrees of interception.  Publish minimums provide obstacle clearance only within the protected airspace.

ILS descent must not be below circling minimums for circling to be allowed.   If there is a circling approach listed for the airport the circling altitude is the MDA.  The MAP is based upon the FAF for the procedure as shown on the chart.  An ILS approach for runway 27, circle to land runway 9 has its missed approach point at the designated missed approach point based on DME or on time and is a NON-PRECISION approach.

Every circle-to-land approach is a non-precision approach.  the MDA of a circling approach gives an obstacle clearance of 300' but this is usually still lower than the normal pattern altitude.  Check the charts to see if circling can only be in one direction.  Only losing sight of the field due to banking is allowed.  Any other failure to see the field requires that you start your climb to the missed-approach altitude, then turn in the direction of the protected obstacle-clearance region and fly the missed-approach heading.  If you leave the MDA before

intercepting the final approach course the FAA will blame you if you run

into anything.

‘

One way to fly circling minimums is to fly the glideslope down to the published circling minimums for the localizer and then proceed at MDA on the localizer by timing or DME if authorized.  Another way is to fly the LOC to the straight-in minimums and landing straight in. If the published ILS approach is to be flown as a non-precision it makes sense to use the glide slope down to circling minimums.  Basic rule is to fly the shortest path to base of downwind leg even if requiring you to overfly the airport.  When appropriate all turns should be to the left.

Since a high descent rate usually has circling minimums only, you must be proficient in getting down quickly and in cleaning up during the missed. If you become visual with the runway by knowing where to look for the runway and are using a created VDP a straight-in is possible.  Use no more that 30-degree banks in turns and do not rise above MDA by more than 50 feet.  Only normal maneuvers are allowed during descent from MDA.  If at any point you lose visual contact with the airport and runway you must execute the missed.  Don’t depart from the electronic glide slope until within one mile of the airport.

You could use a 90 left/270 right course reversal in a circle to land situation if conditions require.  If possible, the turns should be made in the direction that gives the pilot in command the best view of the runway during the turns.  Unless otherwise directed, the AIM procedure, if you lose sight of the runway, requires you to execute a climbing turn back to the landing runway and continue the turn to establish yourself on the published missed approach course.  You are required to miss the circling approach when an identifiable part of the airport is not distinctly visible.   If ceilings permit fly higher than

MDA.  A single pilot forced into right turns should not be circling.

The circling approach area is determined by approach category ("A" for C-172).  It is a series of arcs 1.3 miles in  radius using all the runway ends and sides at the airport.  Its size is the largest area possible made from these arcs.  Category B is 1.5 miles. Primary obstacle clearance is only 300'.  If obstacles are in the way no circling will be  allowed.   Obstacles of less than 400' may not be charted on the plates!!!!  Circling in a specific direction may not be allowed.  Study the chart.  Occasionally it is necessary to remain at a higher than published altitude when transitioning to the approach.  Occasionally, even the minimums vary according to the approach category.  If the descent requires you to fly at a higher approach speed you must use the minimums for the higher category.  Likewise, you must fly the missed approach published for the higher category.   Obstacle clearance is assured only when using higher minimums for the speed flown.  Higher minimums apply if an aircraft is flown at a higher category airspeed.

FAA figures a 400 fpm descent rate for the full approach but when circling is required the 400fpm applies only to the MDA.  A higher rate

may be required below the MDA for the straight in approach.  Don’t force it.  Let the tower know if you do not plan to make a straight in.

# Circling Not Allowed

A circling approach cannot be made if straight in descent has gone below circling minimums.  Any circling must (should) be to keep the airport and runway to be used in sight from the pilot's side.  There are no circling minimums for an ILS approach.  Minimum descent altitudes may be published for straight-in-landings, circles-to-land or both.  Don’t use a lower straight in minimum for a circle because the 250’ obstacle

clearance only applies to the centerline of the approach.  Because of obstacles the direction to circle may be restricted to higher minimums.

Read the notes.

# Missed Approach

While the preflight preparation included a complete run-through of the missed procedure, a review is done outside the FAF perhaps by noting

the procedure written in crayon on the windshield.  The prepared student

asks questions about radio settings, identifiers, and any changes from

‘as planned’.  Among the vitals of position awareness are your equipment

settings.

You already have the missed checklist out.  At the missed you must have pre-decided the sequence of events that you will perform.  In addition to the sequence you have pre-decided the timing you will use.  Talking is way down on the list.  Get the power in, go to the heading, set pitch attitude, and remove some flaps.  Gear later, talking later.

A mandatory reporting point.  The IFR missed approach is more demanding that a VFR go around because it will occur in IFR conditions.  The first consideration in low IFR conditions is to relate these to your personal minimums.  If a missed approach is a possibility, it would be best to hand fly the approach instead of using autopilot.  Controllers may be unfamiliar with where a Missed Approach procedure goes.  The published missed approach is often a non-radar procedure and not very efficient.  The missed approach course usually approximates the approach

course.  If you forget the missed procedure, add full power and climb on runway heading.

We often become slaves to our training and fly the total time to the missed approach.  Rather, at our personal minimums and Visual descent point we would execute the missed.   Get away from the ground and obstacles, climb on course and don't turn until the published time runs out.  Don't clean up the plane until everything is under control.  Since you have memorized the first part of the missed approach procedure and radios are pre-set you don't need to refer to the plate. Don't do anything in a turn during the climbing missed except control the turn.

Should you experience a full CDI deflection at any time you can climb but you MUST not turn before reaching the missed approach point.  There is no guaranteed obstacle clearance protection beyond the full deflection of the CDI until we reach the missed approach point.  An early turn may present obstacles that exceed your maximum climb rate.

You are likely to make the mistake of not applying full power only once.  Power comes first, then pitch, positive rate, gear and flaps.  Any change in the order is going to be exciting.  Every missed starts with a climb.  If you happen to have dual navs set them both to the approach.  The other option of using the second nav for intersections or missed or for distance means that re-setting is required.   This distraction may (will) create problems.  With flip-flop nav critical

fixes become relatively easy.

The ILS missed approach point is a given when the glide slope meets the decision altitude.  If you are inside the marker don’t try to salvage a poor intercept.  Some say not to time for a localizer approach if it detracts from the ILS.  Fly one approach at a time.

VDPs (Visual Descent Points) are not charted when DME is available or when there is no local weather advisory. The VDP gives a charted point from which to commence a normal landing approach with the runway in view.  Use of the VDP is discretionary.   While there are complex ways to use the DME to find the VDP for normal and possible landing slope to the runway the easiest way that will work consists of:  Take 10% the MDH, subtract that number in seconds from the approach time.  At that time, if you do not have the required runway visual indicators commence your missed approach climb and turn when the time runs out.  This method flexes and works for both altitude and airspeed differences.

For the straight-in non-precision approaches nothing beats having a pre-planned visual descent point.  There are several ways of getting a self-made VDP.  The easiest way is to take the MDA (HAT or HAA) and drop the last digit.  Use the remaining figures (2) as seconds to be subtracted from the approach time.   The 3° descent rate usually desired can be obtained by multiplying your ground speed by five.  At 100 knots a 500 fpm descent is close to a three degree descent.

Missed approaches do not occur often but the missed approach should be a part of your proficiency training after you have the rating.  Get to a safe altitude before you become upset with the weather forecast, ATC or yourself.  The unexpected missed is the most difficult IFR procedure of all.  It will be much harder if some instrument fails at this time.  Use your no-peekies on the missed for practice.

The missed approach has several problem areas.  Following an incorrect procedure can fly you into an obstacle at worst and at best may not return you to the correct fix.  An approach flown too far due to failure to determine the actual ground speed creates a distraction problem.  Looking for the airport is the most usual accident causing distraction.  A mistake in your ground speed that makes you look for the airport after you have already passed it can be a fatal distraction.  Use the ATIS wind direction and speed to predetermine the indicated airspeed you will use on the approach.

Preparation for the missed begins with the weather.  An ATIS can be up to an hour old.  I would suggest never making a nonprecision approach if the weather makes the outcome doubtful.  Except for training purposes successful non-precision approaches in minimum conditions will create dangerous habits and presumptions.  Save yourself the aggravation and proceed to a precision approach facility.

The stress level of a second approach after the missed approach is much greater than for the first.  If the miss is because of procedure or technique, you could try again.  If the miss is because of weather, find an alternate.  The temptation to go below your personal minimums is greater on a second approach.  The second approach has a disproportionate number of fatal accidents.

AN ATC warning of course or altitude DEVIATION is sufficient notice to begin THE MISSED.  If you do not continue to fly the course procedure until the time has run out you will not have guaranteed terrain clearance.  Do not turn until past the missed approach point.  A climb is allowed but no turns.

# Minimums

Some minimum altitudes are published on charts. The published minimums are designed for the competent pilot.  If you are less than proficient raise the minimums.  Other minimums are only published for controller use.  It is possible to request a lower than published altitude, if it exists it may be authorized.  Such distinctions exist where flight along VOR to VOR routes have published altitudes but a vector airport to airport may allow a lower altitude via RNAV or radar vectors.

IFR airspace flows with the terrain.  Minimum IFR altitudes in feet MSL are based upon obstacle clearance.  Approaches are based upon descent angles, maneuvering area, aircraft speeds, etc.  FAR 91 operations have no landing minimums but according to aircraft and pilot capability the safety of FARs 121 and 135 should be followed.

# Altitude Minimums

ATC expects you to hold right on assigned altitudes as per FAR 91.123 regarding clearance deviation.  In terminal areas ATC has a 200 foot margin before they get on your case.  This allows for turbulence and altimeter errors.

Center and terminal rules have longitudinal, lateral and vertical

minimums that vary somewhat if in radar or non-radar environment.   Laterally in terminals it 3 nautical, in centers it 5 nautical with radar.  Without radar it varies.  Aircraft given vertical separation

above 10,000 get merging traffic calls regardless but below 10,000 ATC

is not required to give a call.

By asking for a visual climb, descent or separation the pilot has

agreed to maintain VFR and visual separation.  ATC likes this because

it relieves them of responsibility.

# 1) Maximum Elevation minimums

Terrain figures for chart purposes are rounded off the nearest 100 feet and then 200 feet is added.  The chart figures thus makes the elevation with a safety factor of from 201 to 299 feet.  For obstacles  made by man the figures are rounded up to the nearest 100 feet and then another 100 feet is added.  the safety margin is then from 101 feet to 199 feet.  The pilot is well advised to add some more.

# 2) Below Minimums Part 91 Operations

Under FAR Part 91 flight rules:

- Any type of FAA approved weather forecast may be used to determine if minimums are met under FAR 912.169(c), including Area Forecasts.

- Ceiling not a factor

- Visibility as determined from the aircraft is key factor

This flight visibility can not be less than required by procedure

- No descent below DH or MDA unless:

Position to make normal landing, and

Approach light system; threshold markings; runway end identifier 			lights;  VASI; touchdown zone, its markings; or its lights 			or runway, its markings, or its lights are visible and 			identifiable, and

If  1) is missing a missed approach is initiated, or

If  2) not even one of the features is visible and identifiable

a missed approach is initiated.

The FAA investigates all IFR landings made where below minimums visibility is reported.  I suggest that you have difficulty hearing any

ATC request for your flight visibility after an approach.

# 3) Standard minimums

- For a precision approach (glide slope) 600' ceiling and 2 miles

- Obstacle clearance for precision approach is 121’ trapezoidal area tapering to runway.

- Obstacle clearance on intermediate segment (procedure turn) is 500’

- Obstacle clearance on feeder and transition routes is 1000’ + depending on terrain.

- For a non-precision approach 800'  and 2 mile visibility

- Obstacle clearance is minimum of 250’ trapezoidal area with no greater clearance in any one area over another.  Procedures with

- a FAF all have 250’; a VOR without FAF is 300’; NDB with FAF is

- 300’ and without is 350’.

- Obstacle clearance on intermediate segment (procedure turn) is 500’.

- Obstacle clearance on feeder and transition routs is 1000’ + depending on terrain.

# 4) Takeoff minimums

Takeoff minimums are not mandatory on FAR 91 operations but the Part 91 pilot should be aware of the FAR 121,129 and 135 minimums. A chart may show takeoff minimums and still not have an IFR departure (Not Part 91)

# (4a) Zero-Zero

There are no accident figures that preclude zerzero takeoffs as

being dangerous.  The acceleration of the aircraft can cause a somatogravic illusion that you are more nose up than it seems.  Pilots

who habitually make zero-zero Part 91 departures tend to be confident

of their abilities and the maintenance of their aircraft.

# 5) Climb Minimums

FAA climbs and climbs to minimum altitudes are predicated on crossing the departure threshold at 35'.  You are required to report to ATC anytime you are unable to climb 500 feet per minute.

# 6) Non-standard minimums

An airport that meets the weather forecast may not be legal if it has non-standard minimums. (Jepp on airport diagram sheet).  A NOS chart may show an A for an airport where Jepp charts may not because Jepp does not chart for Category E aircraft.

# 7) Sidestep minimums

Sidestep landings to parallel runways have higher minimums than straight in approaches.  Sidestep should be done as soon as runway is in sight.

# 8) Obstacle Clearance minimums

From end of runway away from MAP you have 1/2 nautical mile obstacle clearance area.  This area extends in an ever widening angle to 13.5 NM.  At 13.5 NM the primary safe area extends 4 nm to each side of center line and the secondary safety area is an additional 2 nm to each side.

MOCAs insure obstacle clearance  by 2000' lateral and 1000' vertical and navigational signal within 22 nautical of Navaid.  In some instances as  over the hills of Vallejo and Hayward the proximity of the terrain is much closer.

# Obstacle clearance minimums chart

APPROACH SEGMENT (5)       PRIMARY AREA    SECONDARY AREA

Initial                         1000'            500'

Intermediate                     500'            500'                    Final

ILS If terrain is above runway elevation the obstacle clearance may be a little as 120’ even though the DH is at 200’.  The further away from the runway the greater the obstacle clearance.  Laterally the protected airspace is 1000’ wide when 200’ from the threshold.  At ten miles the lateral dimension is five miles to each side of the centerline with the outside mile tapered upward.  Your obstacle clearance may be as little as 70 feet.

VOR/DME, VORTAC/FAF, Straight-in   250             250

VOR/DME, radial or arc final      500             500

NDB on airport, no FAF, straight-in 350             350

NDB obstacle clearances usually become wider toward the runway.  This accounts for the higher minimum descent altitudes

NDB w/FAF, straight in            300             300

Localizer, LDA, SDF                250             250

GPS approaches usually become narrower toward the runway.  Therefore, the MDAs are lower.

Circling                          300     Tapers to 0

Holding                         1,000             500

DF Approach 			    500

In the secondary area the obstacle clearance tapers to zero

at the outer edge.  The greatest clearance is on the center line.

Approaches with 500’ minimums will have higher obstacles.  Check the

circling area.

If you intend to land, you must at some point descend below the

MDA and DH.  The closer you get to the runway the less your obstacle

clearance becomes.  The non-precision minimum is 250’.   The visibility minimums of all approaches is determined by the slope to the runway.  It is possible to have an obstacle intrude into the glide path inside the DH if visibility minimums are over one mile.

# 9) Minimum vector altitudes

Controllers are required to issue an altitude to maintain that provides obstacle clearance until on a published segment of the approach.  MVAs are used by radar controllers depending on radar quality and the distance from the antenna.  Controllers often put in their personal fudge factor to protect themselves.

# 10) Minimum enroute altitude (MEA)

- The MEA provides at least 2000' of clearance above enroute obstacles when the route is over mountainous terrain and 1000' otherwise.

- An MEA assures reception of VOR signals.  VHF communications are usually but not always possible.

- MEA guarantees radio communication on airways. Communication is guaranteed at FAF and as missed approach point.  There is no assurance of communications anywhere else during the approach.

- Don’t accept as gospel the MEA altitude during very cold conditions.

Request or fly at least a thousand above MEA.

# 11) Minimum IFR altitude

The MIA is on unpublished direct ATC routes established by the facility.  Any ATC clearance will give restriction to keep pilot above MIA.

# 12) Minimum safe altitude

If there is no nav facility within 30 nm of the airport, there will be no MSA.  MSA provides 1000' clearance above the highest obstacle in a defined sector.  Be sure to check plates to see facility used to determine MSA distances.

# 13) Minimums Off Airways

IFR pilots operating on an IFR flight plan off a designated airway must ensure they operate at a safe altitude based upon the requirement of FAR 91.177.  Actual flight information can only be determined by use of sectional plus any NOTAMed new obstacles.

The pilot is responsible when operating off-airways on a direct routing for determining the minimum en route altitude.  In mountainous terrain the minimum IFR altitude for off-airway operations is 2000' above the highest obstruction within 4 miles either side of your course.

# 14) Minimum Descent Altitude (MDA)

As written by the FAA the lowest altitude, expressed in feet above mean sea level, to which descent is authorized on final approach or during circle-to-land maneuvering, in execution of a standard instrument approach procedure where no electronic glideslope is provided.

Simplified:

Lowest mean sea level altitude during non-precision approaches.

This is one of the most dangerous flight altitudes in all aviation.  Single pilot operations that require the pilot to find the runway indicators make the presence of an flight ‘looking’ assistant very desirable.

# 15) Descent Minimums

You cannot deviate from the published approach procedure unless you are in VFR conditions and you have canceled IFR with ATC.  The published IFR altitude for a final approach segment is the minimum descent altitude. (MDA)

Requirements for descent below Minimum Descent Altitudes. (MDA)

1.  Aircraft in position to make normal approach and landing

2.  Approach threshold or approach lights or other airport 				markings are visible to pilot.

3.  Flight visibility is not less than prescribed visibility for 			the approach.

4.  Arrival at missed approach point or thereafter requires both 			1, 2,	and 3 above or a missed approach must be commenced.  			FAR 91.175

5.  Doing the published approach may give lower (ice free)

altitudes than taking vectors due to vector minimums.

#1 of the above creates a particular problem for non-precision approaches where the runways are typically shorter than for precision approaches.  Any excess airspeed carried in the approach but not required for landing must be dissipated.  Diving for the runway will not work.  The placement of the non-precision missed approach point precludes a normal landing unless the landing decision can be made prior to reaching the missed approach point.

# 16) Circling Minimums:

1.  Do not descend or use flaps until on final

2.  Any airspeed greater than 91 kts raises your category.

3.  Lose sight of runway--GO AROUND by flying runway heading in a 		climb.  You much have an identifiable portion of the airport 		in view visible to the pilot while above the MDA.

4.  Only circling minimums are published in a letter coded 				approach (VOR-A) straight-in minimums are not authorized but 		if conditions allow you may land straight-in.

5.  You do not need to worry about other aircraft inside the 			circling maneuvering area when in Class D airspace.

6.  You may well be concerned about other aircraft when flying 			circling minimums at an uncontrolled airport.  Left patterns 		unless contra indicated.

7.  Circling at night is more dangerous than during the day.

8.  Do not descend below MDA until on base leg.  If at an 				unfamiliar airport fly the MDA of the class D aircraft for 			greater flexibility and obstacle avoidance.

# 17) Non-ILS circling minimums

One way to fly glideslope down to localizer circling minimums.  Then proceed at MDA on the localizer by timing or DME if authorized.  Another way is to fly the LOC to the VDP and landing straight in or fly the ILS to the circling minimums for the circle.

# 18) Straight-in minimums

A straight-in approach has minimums of alignment and descent.  A descent gradient of 300' per mile from the FAF to threshold is standard but 400' can be used.  The approach course must be within 30 degrees.  Some straight-in approaches have only circling minimums because of obstacles.  If visibility allows you see the runway requirements at the circling minimums you can land straight-in.  You should have a VDP pre-decided in such an event.

A straight in approach mandates than a procedure turn may not be performed unless a specific clearance is obtained.

Non-precision criteria for minimums can have obstacles above the

MDA within one mile of the approach fix.  Chop it and drop it procedures

are not ALWAYS the best way to fly.  A straight in may have only circling published minimums if the approach will not allow a normal descent.  If you maintain the circling MDA and can make a straight in

without excessive maneuvering or rate of descent, get the clearance and do it.

# 19) Estimated time of arrival minimums

If you find your ETA to a fix is going to be off by over THREE minutes you should advise ATC.  The safest ETA estimates seems to be "6" minutes.

# 20) Alternate airport Minimums

FAR 91.103 says that pre-flight includes knowing runway length, aircraft performance and alternatives available if the planned flight cannot be completed.  FAR 91.167 adds fuel required to destination, then to alternate and then for 45 minutes.  Based on these FARs, especially FAR 91.169, an alternate (plan)is required for all IFR flights except when destination has an approach and is forecast to be 2000' and three.

An approved alternate must have weather reporting (It can be an area forecast) and the instrument procedure must be continuously monitored.	If an airport is not authorized as an alternate one of these factors is probably missing.

Where a planned alternate has more than one approach, one of tham may be listed as NA as an altrernate.  The airport can be used only if another

apjproach is used.

Required if not:

Standard

600'& 2 precision;  800' & 2 non-precision. Requires  				operating weather observer on site or AWOS/ASOS

Mnemonic: ABCD123 = 1 hour After/Before, Ceilings 2000', Distance 3 miles.

Non-standard

Published in SIDs, STARs, Terms and  Tables.

The prudent pilot	sets his sights on the "filed paper' alternate early on.  If the ceiling is forecast at the destination to be less than 2,000' AGL and visibility less than 3 miles, an alternate is required.  A terminal forecast (not Area) is required for determining alternate weather minimums.  "Occasional implies 50% chance or greater and "chance" is less than 50%.

Terminal weather observation and reporting facilities must be available for the airport to serve as an filed paper alternate airport.  If the weather is good as, required in FAR 91.169 to descend from MEA and land in basic VFR, all you need for planning is an area forecast saying so and a runway. Even when an alternate is not required you must have options in mind.

ATC has no record of your filed alternate option. As a Part 91 pilot you should have immediately at hand a takeoff alternate with plates available.  The idea is that if you can't return to your departure field you should have a near-by airport available as an real emergency as required alternate.  (Locally I would consider Angwin, on a mountain top near Napa, as a good avection/ground fog choice for VFR conditions)  The ultimate alternate is to cancel and try an emergency at Travis using their radar assistance.  Your life should be worth the paper work.