# Click on page number for instant scrolling.

# scan basicã

Know thy Self

Reaction

Ready for IFR

Scan-Flying Problems

Turns to Headings

Getting a Scan

Knowing Where to Look

Night IFR

Cautions at night:

Runway Incursions

Altitude Deviation

Climb/descent in VFR

Special VFR (SVFR)

GPS Instruction

GPS in Emergency

IFR Ground

IFR Mnemonics

An approach checklist

IFR checklist

Pre-approach mnemonic

IFR Climb and Descent

Rule of Thumb Climb:

Rule of Thumb Descent

Rule of Thumb Cold Wx Altimeter

Standardized Controlled IFR Flying

Control instruments

Performance instruments

Power, Power Changes and Trim

Level cruise

Approach cruise

Level to Climbing Turn

Steep Turn

Cruise Descent

Slow Cruise Descent

Chop/Drop descent

Approach

Level bank

Level to Descent

Level Turn

Descent to level

Cruise to Approach

Climb and Descent-Scan

Detecting Instrument Failure

The Silent Emergency

SCAN During System Failure

Failures on Approach

Primary and Supporting - Scan

Scan Priority

IFR Scan

# Know thy Self

Inexperienced pilots tend to over-control when beginning on simulated instruments and then again when they are exposed to actual conditions.

The difficulty is related to fully understanding the relationship between the actual horizon and that displayed on the AI.  Some pilots make consatnt adjustments to controls even when they are not needed under the presumption that they should be doing something all the time.

Every pilot has fears and weaknesses.  Ideally, the pilot should make a list and work toward elimination of fears and weaknesses one at a time.  We must beware of perpetuating that weakness in someone else and in ourselves.  My most apparent IFR weakness is failing to identify navaids and keep their volume at a recognition level during approaches.  Some volume controls are overly sensitive in this regard.  It is difficult to set the level because the potentiometer of the volume control has only too loud and off. (07U is an example).  If this listening technique is part of your   standard of IFR then you will quickly pick up if you have an instructor induced navaid failure.  If a student fails to identify and listen to a navaid, expect that the instructor will disable that navaid.  An instructor's failure to do so is allowing a double standard of IFR performance to exist.

The single-pilot IFR flight should have a condensed set of lists that covers route, clearances, frequencies, minimums and missed.  He should have made a dry run through radio frequencies and settings.  He should have rehearsed what he expects to hear during the flight and his anticipated (word for word) responses.  The pilot's efficiency in organization of what the procedures require is the best way to lighten the workload when in flight.  In additional, he should have a pop-up IFR checklist for the end of those VFR flights that require an instrument approach.  If you are on a VFR flight plan you MUST close it even if you are opening an IFR flight plan.

There are advantages to flying the full procedure using your own navigation.  You will always know where you are.  while on vectors, it is easy to lose orientation due to vector changes.  If the unplanned workload increases it is very easy to overlook critical items.  Reading back a clearance does not mean you have adequately prepared for that routing.  A well organized pilot who is at least even with the aircraft is best able to deal with the unanticipated.

Many pilots agree more with the 'follow your gut'  crowd than the 'personal

minima' crowd only because my gut seems to be far better at calculating the quantum variables intuitively much faster than you can reason it all out while bouncing through the clouds.  Confidence in your decisions is a requirement to executing a good approach.  If that confidence isn't there it's time to consider the alternatives."

Develop physiological reaction to those situations that are over your head, and when after 250 hrs you may have learned to heed it. When the altimeter reads too close to DH and it's still dark and gloomy outside, or there's one of those "runway in the storm window" crosswinds, a sour taste at the back of my throat rises.  This means "Go around, you idiot", or "take the missed approach and land somewhere else".

It's much more reliable than a hard-and-fast rule like "I won't land in

less than 500 ft. ceilings".  Experience shows that very few IFR missed approaches actually occur due to weather.  The fact that so few occur is

the cause of the ‘expectancy’ problem.  The pilot may view the situation

as how he would have it be rather than as they are.

Number one is flying the aircraft.  Do not let a distraction such as changing frequencies change this priority.  Mastery of the aircraft should never by in doubt regardless of the workload.  Use the speed you can be comfortable with.  If ATC requests a higher speed, advise that you will accept vectors to allow faster traffic to pass.  In no event maintain an uncomfortably speed on the final portion of the approach.

All approaches should be flown at the selected airspeed with the gear down, using power to control rate of descent and flaps as required at the bottom of the approach.  The primary consideration is airplane handling and performance during the final phase of the procedure.  This means you must get the aircraft set up at a definite point ahead of time.  Stick with the same configuration for all approaches.  If you have good basic attitude instrument skills, with control over airspeed, descent and heading you can fly a good instrument approach.  Descent rate approximates G.S x 10.

2

Whether or not to use flaps on an approach depends very much on how they affect the performance of the aircraft when initiating the missed approach.  By themselves flaps do not affect the approach but on Cessnas the addition of full power in higher powered singles can create overpowering trim forces on flap removal.  Be careful and prepared by locking the elbow on power application.

# Reaction

In searching for an appropriate place for this material I was surprised at how often the term existed, the variety of context, and application to both plane and pilot.  In many situations you must be decisive, selective and accurate.  You must know what to do, do it, and do it correctly.  Most often the word reaction occurs in my writing in conjunction with the word anticipation.  There are times when there can be no anticipation as in cataclysmic engine failure.

Reaction time is based upon how your senses work together.  Hearing and smell may provide advance warning but most often we are not as sensitive to these as to the tactile sense.  Vision evokes the quickest reactions but this quickness may be instinctive and counter productive.  Where you look is in a maneuver will enable you to counter visual illusion and associated instinctive reactions.  How the maneuver feels and sounds will augment vision once the proper parameters are practiced and imprinted.

We need to practice reactions in flying situation so that they can be anticipated, decisive, selective, and accurate.  Landings, takeoffs, steep turns, stalls, airspeeds, minimum controllable, and unusual attitudes are areas where we can organize our senses to get our performance and the required reaction under control.  Along with correct performance of the maneuvers we should expose ourselves to incorrect (read instinctive) reaction situations.  The inadvertent event is the one most likely to be met with instinctive reaction.  IFR could well mean I Feel Reaction.

Recognition of the need and correctness in reaction is, to me, more important than the speed.  Maybe this is because, with age, I have replaced most of my reaction time with anticipation.  I feel that the shift from reaction to anticipation is one of the large personality changes that distinguish pilots from the less fortunate.  It is certainly one of the things I look for when I train an IFR student.

Knowing what to expect from yourself, the plane, ATC, and the weather greatly reduces the need for reaction and increases the presence of anticipation.  The practice element that I would concentrate on would be; 1) Awareness of how deceptive our senses can be, 2) How anticipation can affect selection, speed, and accuracy of any reaction, and 3) Situations where reaction is all you have available.

# Ready for IFR

FAA studies say that the amount of prior flight time has little effect on the acquisition and retention of instrument flight skills.  Others believe that a certain amount of seasoning through independent flying is a good thing.  No matter what kind of flying you do, you should be making improvements in your existing skills.  There is no apparent advantage in waiting to get your instrument rating.

Regardless of how you plan to learn instrument flying, needle and ball, control and performance or primary and support, there are fundamentals that apply to all three.  The flying of an approach is technique; the route, speed and altitudes are procedure.  I would place the first requirement as aircraft control.  You must be able to make the plane do as you want when you want.  The mechanical operation of the aircraft power should be habitually ingrained into the pilot as to sound and throttle position.  Likewise pitch can be preset for any flight condition by knowing the initial position and the amount of change required.  Competence in these two flight skills considerably reduces the scan speed and changes required.

The pilot must be able to (1) fly the plane, (2) use the system, and (3) know where he is.  The pilot must be able to fly, position, and operate the aircraft through a variety of configurations and performance parameters in such a way that flying does not get in the way of the other two elements above.  There are at least ten distinctly different skills that must be mastered.  The skills are inter-dependent and any weakness in one will be reflected in another.  The smart pilot does everything possible to optimize the resources at his disposal.  This necessitates standardization of procedures.

Pitch attitude, power settings, and trim position separately and/or in combination must be known to and achievable by the pilot.  Being able to fly means being able to fly a heading, maintain an airspeed, remain on altitude, and make changes as required.  The competent pilot is able to vary his attention to the proper place, on time and in sequence.  He understands the principles of energy as it applies to flying.  He uses his knowledge of instrument peculiarities as they interpret energy change.

Properly trimmed with wings level IFR is relatively simple as long as attention is paid and kept ahead of the aircraft.  Use the rudder to keep things in order when you need both hands.   Without rudder input holding headings and making heading corrections of less than 10 degrees is nearly impossible.  Use the rudder to hold headings and make corrections of less than 5 degrees.  It's quicker, more accurate, and less likely to induce a correction back in the other direction.  Sure, it's a skid, just hold the wings level and kick the rudder as required.  You can't begin to meet the scan requirements of IFR until you can fly your aircraft through all the required power settings and attitudes.

You must be able to make smooth rapid  corrections that will average out the errors and keep the selected line of flight within limits.  You should know approximate movements of yoke and throttle required.  You will never exceed a standard rate bank.  If a more rapid change is desired the movements may be made in excess for a few seconds but should then be normalized.  Gentle wins every time.  Pitch makes changes faster than power.

IFR is a complex combination of flight skills, cockpit management, communications, decisions and priorities.  Of themselves any one choice is not difficult; it is in combination that difficulty rears its ugly head.

# Scan-Flying Problems

Instrument flying problems usually are from faulty scan.  Instrument interpretation, fixation, selection and control reactions individually or in combination cause a flight problem.  Fixation on an instrument, the AI, is an acceptable procedure when redirecting the airplane, reacting to vertigo and flying in turbulence.

The most important element of a standardized scan is knowing when to look for what.  Scanning fatigue occurs when we are moving our eyes without purpose.  Try moving your eyes side to side a few times (15) and see what I mean.  Also note that while your eyes are moving you cannot see anything.  You will see without fatigue when your eyes are stopped and have been moved for a purpose.

Problem:

Chasing the VSI or using the slope needle for pitch changes is an 	early cause of vertical S's on the glide slope.

Solution: Pitch changes are made on the AI.

Problem:

S-turns while tracking a needle.  Student is probably trying to fly 	plane 'level' by reference to HI.

Solution: Use a reference heading on the HI but hold the heading by 	reference to the AI..

Problem:

Small variations of altitude.

Solution: Use pitch (AI)  to 	correct.  Above 100' use both AI and power 	for correcting.

Every pilot will have some flight/scan problem in one form or another.  Be prepared to tell the instructor where you are looking during the scan and what is making you react as you do.  Because of cockpit limitations the instructor can only guess as probable causes.

Instruction:

Make N-Peeky stick.  Tack No-Peeky to 2 foot rod.

# Turns to Headings

Cover HI  Have student get to straight and level.  Allow student a quick look of HI to note heading and a selected heading 90-degrees right or left.  Cover HI and have student initiate 90-degree level turn using AI as rate indicator.  Every airspeed has a AI angle for standard rate turns.  Remove no-peeky two or three times for quick HI look so student can plan rollout to 90-degree heading.  Intent is to show student that HI is only used occasionally and briefly during turns.  Know when to look where.

# Getting a Scan

You are moving your eyes across each instrument for information.  Move the eyes too fast and you will lower your ability to make interpretations correctly.  A slow scan can cause fixation.

Initially don't try for accuracy.  Visualize putting the airplane where you want it by watching the attitude indicator.

- Scan from the AI to the TC and VSI to see the trend.

- Verify that the AI, TC and VSI are in agreement.

- Go to the numbers, altimeter, power, and HI

- Trim and begin total scan.

# Knowing Where to Look

The competent IFR pilot knows what to look for and when to expect a change.  He knows when to do something and how much of it to do. The Practical Test Standards (PTS) give the objectives and skill level requirements but don't tell how easy or difficult the acquisition may be.  Most IFR students have 60 hours or instrument time before taking their checkride.

Pitch

AI, Altimeter, VSI, Airspeed indicator

Power

Airspeed, Manifold pressure, RPM

Bank

AI, HI, TC, and Magnetic compass

Rate

TC, VSI, Clock, AI

Decide what you want.  Decide where to look for the performance you seek.  Greatest area of misinterpretation is in making compass turns without the aid of the clock.  a normal scan is clockwise beginning with the AI.  For a rate turn the initial scan begins counterclockwise from the AI.  No one instrument gives all the information.  Each instrument must be scanned to obtain confirmation of information either sought or required.

# Night IFR

Part of the preflight is to know the approach procedure and the lighting system activation that may be available.  Statistics show that night flight and IFR night flight has proportionately higher accident rates.  There is nothing more difficult flying at night on instruments providing you can see the instruments.  Good lighting is essential.  Preservation of night vision is of reduced importance.  Usually the accident is caused by controlled flight into obstacles while transition from IFR to VFR.   An accident at this time is called 'pilot error' or an attempt to use Braille.

Depth perception and obstacle determination is more difficult at night.  The brightness of runway lights and others cause an illusion of distance that is deceptive.  This is especially true if the area is unfamiliar.  Transition to visual conditions may require exceptional area awareness.  Fuel exhaustion at night seems to become a factor only if multiple approaches are involved.

If you are going to fly night IFR do considerable practice in those conditions.  Fly only precision approaches into controlled airports that are straight in.  No circling approaches.  Use the VASI or PAPI. If you must go into an uncontrolled airport cancel IFR only after landing.  Set higher minimums for night.  A vacuum back-up is essential at night.

Don't fly if ice is a possibility and don't take off if it is a probability.  Nights are colder and ice is more likely.  On the bright side (pun) thunder storms are less likely.  Night flight into broken cloud conditions are quite conducive to creation of vertigo.  In night IFR avoid looking outside until you really need to.  Prevention is easier than recovery from symptoms of illusion.  At night you can’t obtain visual impressions of where the weather is or what it looks like.

As with all IFR flight, it is important that you fly the charted routes.  Radar vectors requires you to totally trust someone else.   Better yet, combine 'own nav' with radar advisories specifically asking for obstacle clearance.  Use the second pilot to call the plate numbers to supplement your own checks.  Plate calls would include; ATIS, pattern altitude, IAF altitude and track, inbound track and altitude, approach fix, minimums and time, missed approach track and climb to altitude before turning.

Flying a consistent profile is essential to safe night IFR.  Be so situation aware that you do not descend below 1500' AGL until you are within 5 nm of the destination. (You can remain above the charted altitudes.)	 The worst night IFR situation is a non-precision approach to circle-to-land at a strange airport.

The worst case scenario related to night IFR actual conditions flight is engine failure where without power, most instruments, and little opportunity to select a landing place everything is at risk.  The risk management of flight in these conditions begins with the proficiency of the pilot.  A smoke filled cockpit becomes IFR with your eyes closed.  Without auto pilot you haven’t a chance.  Standby vacuum doesn’t help if the windmilling propeller provides no differential.  Catastrophic failures can occur any time but are far more likely to occur where poor maintenance practices are allowed to exist.

# Cautions at night:

- Personal minimums should be very high for night IFR.

- Carry extra fuses, flashlights.

- Have current Airport/Facilities Directory to determine hours of 			operation, lighting and facilities available.

- Get L-type notams

- Do not expect ATC to know about FDC notamed changes to procedures.

- Get the FDCs as part of your flight planning.

- Chose route as nearly airport vicinity as possible

- Double check MDA and DH for any night restrictions

- Night vision is affected by IFR MOCA altitudes due to lack of oxygen.

- Ability to retain night vision very difficult.

- Bright runway lights on a wide runway make you feel closer to the 		ground; dim lights in haze on a narrow runway may cause you to 		descend below the glide slope.

- Your worst landing will be when you follow the landing light into the 	ground.

- Night approaches with only the approach lights visible creates 		illusions of altitude and direction from only the slightest bank.

- Trust the instruments at night

- Wing mounted taxi lights make taxiing difficult

- There should be some night IFR instruction but it should be in the form 	of a review of previously flown day light procedures.

- Learning a new procedure at night is both difficult, inefficient, and 	dangerous.

- After the IFR rating is acquired then additional night IFR instruction 	should be used in maintaining the 6-6-6 requirements.

- The most difficult approach ever do will be the night circling approach 	followed closely by the step-down approach.

- More than 50% of accidents which result during approaches to below 	landing minima are fatal.

- A  single pilot delayed "go around' is most frequent single cause.

- The FAA reviews all landings reported as being made below weather 	minimums.

- Consider coming in SVFR as a viable option since altitude minimums do 	not apply.

- The accident rate on night IFR approaches is 60% of all IFR approach 	accidents.

- Only 4% of all general aviation flying is done at night.

- most ILS accidents (20-30 per year) seem to occur within a mile or less 	of the runway but 1/3  crash on the runway.

- Over half of the ILS accidents occur at night when only 1/4 of the ILS 	approaches are made.

- A pilot should make his personal IFR minimums for night flight very 	igh.

- Transitioning to the visual is the most demanding and dangerous part of 	an ILS but even more so at night.

- There is no margin for error in an ILS carried to minimums.

- Runway accidents seems to be related to contact on slick runways and 	higher than normal speed.

- 1/6 of ILS accidents occur while making second or third approaches.

- An ATC warning of course or altitude deviation is sufficient notice to 	begin the missed.

- Flights on the ILS to an airport known to be below minimums should be 	flown to DH for practice only with a planned missed.

- Don't fly a no-approach light ILS at night.

# Runway Incursions

Through greater emphasis on standardized radio communications pilot will be involved in fewer runway transgressions, enroute course deviations, and other aviation safety incidents.

# Altitude Deviation

Most altitude deviations are the result of a communications failure.  The failure may be with ATC or the pilot.  The major cause of the failure is not using standard phraseology usually by the pilot who fails to read back the ATC clearance completely with the aircraft call sign.  If there is cockpit doubt as to the clearance make a confirmation call to ATC.

# Climb/descent in VFR

Clearance or ATC instruction when:

IFR flight requests climb/descent in VFR conditions

Noise abatement not met by IFR route/altitudes

Practice approach not on IFR flight plan.  Must comply with

VFR rules.  ATC separation provided in Classes B and C.

VFR climbs help you to avoid circuitous charted IFR routes often caused by terrain.  Such a visual climb clearance requires that you fly above

minimum safe altitudes requires in FAR 91.119 and provide your own terrain clearance.

# Special VFR (SVFR)

SVFR will be available in B, C, D, surface-based E, or surface foot-print of the airspace.  SVFR in primary airport of Class B makes SVFR unavailable to fixed wing aircraft.

# GPS Instruction

Begin by running through the GPS approach set-ups.  Do several without flying them.  The set-up will be different full approaches, approaches with vectors and when an initial approach fix is used.

Fly the approaches at 1,800’ AGL above the published altitudes so you can observe the GPS perform sans altitude, climbs and descents.  After such a dry run try the full approach with missed included.

GPS is not condusive to windshield scan.  Best to have a third person aboard who is supposed to watch for other aircraft.

# GPS in Emergency

You can use GPS to position yourself on short final to a runway under the worst conditions if you have practiced a relatively simple procedure.  There are several requirements.

- you must be able to fly timed distances on precise headings.

- You must have an aircraft that is willing to remain airborne.

- You must have a GPS receiver with a suitable data base.

You will need to fly a modified airport pattern much as you would in VFR conditions.  Try this lesson in VFR conditions around a seldom used airport until you perfect the procedure.  GPS locates an airport by a reference point called the ARP.  This point is usually near the center of the airport itself.

You want to remain relatively close to the airport.  You intercept a direction to the ARP that is 90 degrees to the longest runway.  Since you want to be about 1/2 mile from the runway on your abeam the numbers downwind you will anticipate your turn to downwind by turning when .3 from the ARP.  A standard rate turn should position you about 1/2 from the runway on a parallel track.

Turn base at 1 mile from the ARP and make an early turn to final to a heading that will allow a 30 degree intercept to the bearing to the ARP.  The descent throughout the procedure should be from pattern altitude and at approach speed before turning base.  A normal descent should place you over the runway with some excess altitude as a safety margin.  Never accept an ATC final approach speed higher than your flap or gear speed.

# IFR Ground

Much of the IFR ground information should be self studied from FAA and other texts.  Material herein should be considered supplementary only.

# IFR Mnemonics

Turn, Time, Twist, Throttle, Talk, Track are the 6 Ts mnemonic used at several points during an approach.

The terminal arrival mnemonics of TIMS, MARTHA-A, and FLAGS at the FAF gets the important things done.

T = Tune		M = Missed		F = Flags (avionics)

I = Identify	A = Approach	L = Lists

M = Monitor		R = Radio		A = Altitude

S = Select		T = Time		G = Gear

H = Headings	S = Start (watch)_

A = Altitude

A = Airspeed

# An approach checklist

Mis - memorize missed

T  - Time set

R - Radios set

I - instruments set

A - altitude memorized MDA/DA

L - Landing configuration/checklist

# IFR checklist

The very best single pilot IFR checklist is the flow checklist.  Follow a logical series of checklist stops across the panel as required to set up a given flight task.  The flow can be accomplished quickly from memory and even more quickly confirmed by reference to the checklist.

A - ATIS

A - Altimeter set to ATIS

H - Heading indicator set to compass

C - Com #1 on approach

C - Com #2 on tower

N - Nav #1 on approach facility

N - Nav #2 on intersection or missed

D - DME on appropriate NAVAID

A - ADF on approach frequency

T - Timer set

M - Markers on/tested

M - Missed memorized

L - Landing Checklist

G - Gas

U - Under carriage

M - Mixture as required (altitude)

P - Prop fine pitch

H - Heat (carburetor)

I - Instrument check

# Pre-approach mnemonic

MACARTHAR

M missed procedure

A Atis/altimeter

C compass check

A approach procedure

R radios set

T timing set

H headings-heights

A altimeter/altitudes

R reduce speed

## TTTTTTTT

turn, time, twist, throttle, talk, track, tires

MARTHA, for approach

M Missed

A ATIS

R radio

T times

H headings

A altitudes

CRAFT for clearances

C Course/cleared

R route

A altitude

F frequency

T transponder

C C C C C for missed approach

C Cram the throttle

C Climb

C Clean up (gear/flaps)

C Cool (cowl flaps)

C Communicate

BE SUPER SHARP, MICHAEL'S COMING. For fixed gear Cessna

B fuel on both

S security (seats, belts, doors)

S stopwatch set

M mixture rich

C compass

WRIM-TM for final approach

W weather (wind ceiling, visibility

R radios (set-ident)

I instruments (radials/intercepts)

M minimums

T time

M missed procedures

ADEF ACEfor lost communications and altitudes

A as assigned

D direct route

E expected route

F as filed

A assigned

C charted

E expected

(whichever is highest)

PTAEN for position reporting		IPTAR

P position				I identification

T time				P position

A altitude				T time

E estimate				A arrival (next and ETA next expected)

N next				R remarks

FAT for reporting at holding pattern

F fix

A altitude

T time

HATMA RAAD SHELFF GIMPI STAT

Heading, Altitude, Time, Missed Approach

Radios, ATIS, Altimeter, Directional gyro

Seats, Harness, Engine, Luggage, Lights, Flaps, Fuel (pump)

Gear, Ice (C.H.), Mixture, Prop, Inbound

Speed, Time, (FAF), Altimeter, Talk

# IFR Climb and Descent

You must include in your IFR planning the vertical aspect.  Every IFR departure has a climb gradient that your are expected to meet or exceed.  Ground speed determines your gradient.  Groundspeed divided by 60 equals vertical speed divided by gradient.  Jeppeson has a chart of gradients.

Adequate gradient figures for either climb and descent can be obtained by rounding the feet per minute by the distance in miles.

540'    rounded to      5  Gives angle of 2.5 degrees

over distance of two miles     2

To find gradient per mile you just multiply angle by 100

Angle of 2.5 x 100 = 250' per mile

To convert angle to rate of climb/descent use E6B or work proportion

Ground speed 		VSI

60		feet per mile (gradient)

# Rule of Thumb Climb:

Memorize aircraft speed in knots per minute.  Multiply by required feet per mile climb gradient.  75 kts = 1.25 a minute x300 fpm (feet per mile)=375 fpm

# Rule of Thumb Descent

(150 kt range of ground speeds)

# 1. Take 1/3 of altitude-to-lose to get distance, descent at 1/2 ground speed.

# 2. Take half of ground speed and add a zero for descent rate.

# 3. Take the distance to field times three and add two zeros.  This will give the altitude that the distance will give a three degree glide slope (hypotenuse) to the field.

# Rule of Thumb Cold Wx Altimeter

Any error is proportional to difference of actual and standard temperatures and the altitude of the aircraft above the altimeter setting source.  This assums standard lapse rate.  Error should be figured as four-feet per thousand feet for each Celsius degree difference.

Example:

Surface temperature is -20°C which is 35°C	below standard.

Aircraft at 5000’ indicated would be 4 x 5 x 35 or 700’ low. 		4300’ adjusted.

IFR departures have obstacle-clearance gradient of 1.5 degrees. that begins 35' above departure end of runway.  A .5 degree safety margin is built in so you must make good a 2-degree climb rate.  If you accept a SID with a higher rate required you are expected to perform.  ATC enroute climb rates are 150 feet per mile below 5000'; 120 between 5k and 10k; and, 100' above 10k.

If below 5,000' you are told to gain 3000' before crossing a particular fix you would do the following.

At 150 feet per mile required converts to 1.5 angle.  3000' converts to 30, divide by 1.5 = 20.  You must begin climb 20 miles out.

ATC usually expects a climb or descent rate of 500 feet per minute.  A pilot-discretion clearance means you can choose both when to initiate and at what rate.  Once an altitude has been left it cannot be attained again without an amended ATC clearance.  Any SID clearance that

has a climb gradient is concerned with terrain clearance.  Crossing  restrictions have more to do with traffic routes that may conflict.  When climb and crossing restrictions appear together be careful.

Don’t hesitate to request radar vector to assure clearance of terrain.

This puts clearance responsibility back to ATC.

300 ft/nm is the maximum decimate gradient for the intermediate segment; 400 ft/nm is the maximum fro the FAF to TDZ elevation.  The more closely you fly the required elevations and descents the more likely will be your approach a stabilized one.

# Standardized Controlled IFR Flying

# Control instruments

Attitude Indicator and Power (Tachometer or manifold)

The proficient pilot knows by sound the present power setting and airspeed.  He knows how much change is required to go to another performance situation.  He knows by both feel and sound the change made. Then he confirms by looking at the control instruments. I have found that most pilots tend to over-control when using power.  I recommend that you make the power change and then take half of it off, immediately.  Put in 2" manifold and then back off 1".  Seems to work.  I have seen an ILS flown beautifully using just power adjustments, trim and rudder.  Any procedure you use must work in the worst case situation.

# Performance instruments

- Airspeed indicator

- Altimeter

- Turn coordinator

- Directional gyro

- Vertical speed indicator

1.  Know your power settings and aircraft configurations.

2.  Performance instruments are your means of determining the effect of 		control input.

Airspeed is set with power.

Reset your HI and set approach configuration before descending. Descent is initiated by power reduction. Know the reference descent rate and bracket it with power changes.

3.  All turns related to flying are pre-selected on the HI but entering and stopping the turn is with the AI.  Minor corrections can be done with the rudder.

# Power, Power Changes and Trim

Once you have mastered the power, power change and trim required for two climb speeds, two descent speeds, and at least two level speeds, using only the AI, you are ready go on to...

Redirect the aircraft:

1.  Set power and AI (attitude), trim as required

2.  The trend of movement is shown on the turn coordinator and the 		VSI.

3.  Check trend.  If unacceptable redo the AI.

Check the numbers information

FAA Primary instruments have numbers on them.  They allow 	 		you to fine tune your performance.

4.  Fine trim for hands off.  This must be done without error or 			hesitation.  If a problem exists practice aircraft control 			basics.

5.  Look for meaningful information

Set the AI, check the trend then read the numbers

# Level cruise

- A constant altitude with power and pitch to obtain cruise speed.

- Power for airspeed

- At desired speed, lean for economy

- Frequent full power flight is good for engine

- Set AI, check TC and VSI for trend

- Fine tune: Alt for pitch, HI for bank, Tach for power

- At level cruise the altimeter is primary pitch.  Airspeed, AI, altimeter and VSI are affected by elevator and throttle.  Any change in power only results in change in airspeed or altitude.  Check altitude before adjusting airspeed by power. Changes in straight and level airspeeds require adjustment of pitch, power, and (bank) rudder.

# Approach cruise

- Fine tune: Speed at top of white arc, Vfe

- Reduced airspeed that may be best endurance

- 1/2 bar high on AI

- Set AI, check TC and VSI for trend

- Alt for pitch, HI for bank, power for airspeed.

# Level to Climbing Turn

(As in a missed approach)

- Airspeed for pitch, TC for bank, Tach for power

- AI set, then power (full), airspeed and turn coordinator (TC)

- Check trend.  If unacceptable redo #1.

If acceptable go to #2 & #3.

- Check the numbers information

Bank information is AI/TC or HI for constant heading (wings 		level)

Pitch information is airspeed/power, altimeter, or VSI

Power information is tachometer or airspeed

# Steep Turn

Alt for pitch, AI for bank, Tach for power

# Cruise Descent

- No flaps since G-load is set lower

- Speed limited by ATC, Vne, Vno, or Va

- One bar low on HI for 500 fpm descent

- VSI for pitch, HI/TC for bank, airspeed POH

# Slow Cruise Descent

- Full C.H. and 1500 rpm

- Extend Gear and partial flaps

- Less than 1/2 bar down on AI

- Power for airspeed at or just below Vfe

- Prepare for go-around or landing

# Chop/Drop descent

Airspeed/VSI for pitch, TC for bank, tach for power

# Approach

VSI for pitch, HI for bank, airspeed for power

# Level bank

During a level bank the compass, HI, turn coordinator and AI, four instruments in all move.  If you don't like what's happening to any of them, adjust the AI

# Level to Descent

The type of aircraft makes a difference in how descents are initiated.  In fixed gear aircraft you can get 5 for 5 almost every time.  500 fpm for 500 rpm.  This works if you maintain the same speed in the descent as you had in level flight.  If you are going to descend at a slower speed than the level speed, it may be best to slow and trim for the descent speed while in level flight and then use the five for five descent parameter.

Retractables have vastly differing drag coefficients with the gear down which will necessitate developing a level to descent chart.  It is a waste of time to try to fly an approach without having in your memory bank exactly how to attain the speed and descent rate standard for your aircraft.  The more 'constants' you have developed for flying the aircraft the less likely will flying the aircraft be part of the problem.

# Level Turn

Personally, I prefer to hand fly my level turns and always anticipate the forward pressure on roll-out.

# Descent to level

If you know that you will be leveling off shortly after initiating a descent as though descending from the FAF to a circle to land altitude, you would be best not to change the trim.  Make the descent by power reduction only in fixed gear aircraft or in retractables that have their gear down before the FAF.  If the gear/flaps are not down at the FAF use the gear/flaps to initiate the descent and plan to adjust power and trim at the leveling altitude.

# Cruise to Approach

In fixed gear the change in speed is initiated with an immediate power reduction to a predetermined setting.  You must know and have charted the power setting that will give a given approach speed.  Trim off pressure slowly while holding heading and altitude.  Keep track of the trim setting.

The process may be made faster by initially taking off more power.  Application of trim must then be quicker but to a known setting.  Pre-determined power must be brought in just as speed reaches approach speed.  Missing this point of power application will exacerbate problems associated with attaining approach speed.

Retractables should include dropping the gear and any flap application as a part of the speed change to approach speed.  Variations of just when  to do what will vary from aircraft to aircraft.

## Best Angle Climb, Vx

- 3-bar high on AI

- Maximum power

- Speed increases with altitude

- About 5-10 knots below Vy

(Minimum sink speed on engine failure)

## Best Rate Climb

- 2+ bar nose high on AI

- Maximum power

- Fastest rate of climb

- Caution on traffic avoidance and engine heat.

## Cruise Climb

- Constant airspeed or constant vertical speed

- One-bar nose high

- Better speed, visibility, and cooling

- Advise ATC if not 500 fpm

## Climb to Level:

- AI set(#1), then to altimeter and heading indicator (numbers)

adjust AI(#1) then back to Alt and HI.

- Check trend.  If unacceptable redo #1.

If acceptable go to #2 & # 3.

- Check the numbers information

Bank information is AI/TC or HI for constant heading (wings 		level)

Pitch information is airspeed/power, altimeter, or VSI

Power information is tachometer or airspeed

# Climb and Descent-Scan

When climbing and descending at constant power, airspeed is primary pitch.  To maintain airspeed during descent, lower nose on AI only and immediately reduce power to predetermined setting to hold airspeed.  Trading airspeed for altitude or altitude for airspeed may be preferable. If both airspeed and altitude are high or low you must adjust both pitch and power.  With gear down anticipate initiating descent with glideslope is 1/4 dot above center.  Where manifold pressure is used for power the descent causes an increase.  Power must be regularly reduced to maintain airspeed.  A rule of thumb for establishing a 3 degree slope descent is to multiply your ground speed by 5 and add 50.  Not exact but good enough.

Example:

Climb

- Set attitude on AI

- Climb power

- Primary is altimeter

- Supporting is airspeed and VSI

## Climbing Rate Turn

Initial phase

AI

Power phase

Airspeed-primary

Stabilized phase

VSI - primary for pitch

Airspeed - primary for power

RPM - support for power

Turn coordinator - primary for bank

AI supporting for bank and pitch

A climb is not as sensitive to pitch as to power and airspeed.  Once speed and power are set, pitch is locked by use of trim.  Leveling off is more difficult.

Instruction:

Level

Scan begins and ends with AI.  Lower nose to AI horizon while avoiding any roll input.  Major trim adjustment.  Check VSI.  Check AI.  Listen for acceleration to speed desired.  Altitude check. Set power.  Minor trim adjustments.  AI check.

Instruction:

Cockpit Operations

Go from heading to attitude indicator to altimeter.  Always include AI as a scan segment between every part of writing, changing frequencies or any cockpit operation.

## Airspeed adjustment

Airspeed set but VSI high or altitude low.  Raise pitch and add power.

Anytime airspeed is constant any change in pitch requires an opposite

hand movement to change power. Practice the vertical S in which each cycle decreases by 100'

Rule of thumb:

1. Standard distance between runway lights is 200'.  Have passenger 		count lights to determine your aircraft performance.

2. Takeoff and landing distances in the book are of maximum performance 		type.  For normal operations double the book figures.

Rule of thumb:

The more near the throttle is to full power the more even leaning 		will be among the cylinders.

Rule of thumb:

Cruise speed for planing purposes should be increased by one knot for 		every 100 pounds below gross weight

Having the airspeed indicator fail at night or in actual IFR conditions is perhaps the most confusing instrument failure that can

occur.  Such failures are not common but they do happen and nearly half of them result in accidents.  Failure usually results from pitot system blockages with ice the usual culprit.  The very improbability of airspeed failure makes it so that pilots trust airspeed before they trust gyros.

The pilot’s first recourse with airspeed failure is immediate reference to the attitude indicator and the VSI.  By knowing the power settings for level cruise, descent, approach speed you can get these

speeds by setting the aircraft attitude.  This would be a worthwhile

partial panel exercise for proficiency training.

# Detecting Instrument Failure

Bank is compared with the AI (vacuum), TC (electric).  Turn is compared by adding HI to the   Compare the pitch information of the AI (vacuum)and the VSI (static).  The AI and the VSI can fail independently as can the airspeed (ice) or power.  However, the airspeed and altimeter (using alternate air) can check the AI and the VSI reliability.  An HSI and TC can fail together since both are electric.

When in doubt let go of the controls and sort things out.  Compass is used to check AI and TC for wings level or direction of turn.  HI is unreliable for this check because it uses same vacuum as AI.  HSI  and TC are electric and can be checked against AI.

You know that AI has a set angle for standard rate turns for every airspeed.  Knowing this, it is possible to cross check the operation of the AI against the HI and turn coordinator for failure.

# The Silent Emergency

Pneumatic (suction) system failures are emergencies.  If you do not have a backup system declare an emergency and request a vector to the nearest VFR.  All to many accidents occur after the pilot has flown up to an hour of cruise partial panel and loses it because of the higher skill demands of the approach.

Pneumatic systems can fail suddenly or slowly.  A complete failure shows immediately on the gauge followed by a failure of gyros.  You do include the suction gauge in your scan?  An initial lack of recognition of pneumatic system failure gives a conflict between instrument indications and interpretation.  A pilot must scan all instruments when there is a conflict of information.  A decision must be made of what to ignore and to cover that instrument.

Aircraft can be flown safely with pneumatic system failure.  If you know the weather behind a 180 may be the best maneuver.  Disconnect the autopilot since its information may come from the pneumatic inputs.

# SCAN During System Failure

The prelude to an accident is when everything seems to be going right.  The cross-check scan to the second and third level instruments is more important when you have time because everything appears to be going right.  Instruments are much like politicians and lawyers; they can be lying any time or all the time.

The triad of pitch instruments are,

one, the vertical speed indicator (VSI) powered by static air,

two, the attitude indicator whose gyro is vacuum driven and a poor 		choice for

three between airspeed and altimeter.   Airspeed and altimeter are 		also parts of the pitot/static power system.

Airspeed and its changes can be determined independently by sound.

Summary:

- A.  Change of attitude: Lock AI for attitude, set power and trim.

- B.  Scan AI, TC, VSI for trend.  If wrong redo A. If O.K. go to C.

- C.  Scan the numbers.  If wrong redo A and B.  If O.K. go to D.

- D.  Fine trim for rudder only flight and go to basic scan.

# Failures on Approach

If a major navigation instrument fails on approach, execute the

missed approach procedure.  Stop the descent, proceed on heading until the time runs out before turning.  Don't get involved with trying to salvage an approach.

# Primary and Supporting - Scan

The FAA system of instrument use and interpretation requires that the student acquire knowledge as to what instruments are primary and supporting.

The basic rule is that the instrument with the numbers is primary and others are supporting.  ATC assigns numbers.  Numbers are primary instruments.  This method requires pilot to have knowledge of operation and interpretation of all instruments.  There are three groupings of instruments related to every control function and aircraft performance.  For each part of a maneuver there is a primary and support instruments.  When a gyro dies a primary instrument dies.  Support instruments then become primary. (See other material which indicates the FAA primary is really the third and final level of performance ...more)  This material is NOT the FAA system you are required to explain on the PTS.

Use the attitude indicator to set attitude.  Use power instruments to confirm power settings.  The performance instruments show how well you have set the attitude and power.

# Scan Priority

Heading	Heading indicator and course as reflected by a

navigation instrument

Altitude    Level as assigned or mental note of altitude climbing

or descending to

Looking for A mental prompt to note next intersection, time, or

action point.  You are never looking for just an 				altitude you may be looking for more than one thing.  			You could be looking for an intersection, DME fix, 				time set for approach.  On an ILS you are looking for 			an altitude endpoint.

# IFR Scan

A good scan should include skimming over the attitude indicator each time your scan moves on to a new instrument.  A sequence could be

AI to DI to altimeter across AI to airspeed across AI to VSI up and down through AI to turn coordinator.  Draw this out and see how it goes on

a panel picture of write it out and finger point it across the aircraft panel.  Every third scan should include navigation and engine instruments.

Efficient safe IFR flight requires instrument cross-check, interpretation, and aircraft control.  Scan is moving the eyes continuously with out pauses or fixating over the instrument panel.  Scan cross-checking will detect a change.  Interpretation uses other instruments concur. Small control input pressures make a correction that must be cross-checked and verified by scan. Scan is the first IFR skill to deteriorate without practice.

The sloth is an animal noted for its slow deliberate style of movement.  It knows just when and where it is going to make any movement.  Skilled IFR pilots could well use the sloth as an icon.  Every eye movement and stop is based on knowing when to look for what.  Every body movement is predetermined, deliberate and selective as to quality and quantity.  As soon as un-sloth-like tensions enter into your flying both your scan and flying will suffer.

Partial panel IFR develops and broadens the scan to include instruments that may become less used in general practice.  Becoming overly dependent on the autopilot limits your skill and ability in interpreting  raw instrument data.  Without exception, all instruments must be cross-checked for less than normal indications.   Any digression from the scan for over three seconds can cause unconscious yoke inputs to become significant.  Lock your elbow and arm before breaking the scan away to other things.

First comes fly the plane.  Second, comes the scan/cross-check.  You must be able to keep your eyes moving without fixating.  Students tend to watch the instrument that moves the most. (VSI)  Lastly, you must be able to interpret the instruments.  Interpretation includes the sounds and feelings you get from the aircraft.  You want to know what is happening.  Your scan will tell you what is happening.  You want to know what needs to be done.  This is your interpretation of what needs to be done.  You need to do what needs to be done.

Recognize in every situation that there is a standardized attitude and power setting to control everything.  Concentrate on your aircraft attitude and power settings.  "Attitude plus power equals performance."   Scan is an individualized process determined mainly on how well the pilot flies. The better you fly the less reliance must be placed on scan.  The fact is that very few IFR pilots have learned by, and fly the FAA 'recommended' system.  One desirable scan is based on hub-and-spoke centered on the attitude indicator.  Another uses a system called the inverted V.  Scan according to the sense priority you have developed for yourself.  Every pilot I know has made his individual adaptations.  How you scan is an individualized skill developed for each flight condition and maneuver.  Regardless of what your scan system or pattern may be, you must keep your eyes moving.  When you scan is very individual.  Related to how strong your sense of survival may be.

Flying the aircraft, holding a heading and altitude are #1.  Scan the flight instruments at varying speeds for the situation, yes, but take quick looks to check charts, engine, avionics, ATIS, etc.  Any look of over three seconds is likely to affect aircraft attitude.  Keep your elbow and wrist locked.  Poor control results from over concentration on one instrument, an instinctive reaction to perceived condition, incorrect bank and/or pitch in AI or failure to lock arm and wrist.   Fatigue is a probable cause of such problems.  Much of your IFR flying should be considered as endurance training.