IFR Competencyã

An instrument proficient pilot is a combination of proper training and intelligent judgment.  He obeys the FARs but understands personal limits as well a legal limits.  Instrument flying can be unforgiving of pilots who fail to recognize their shortcomings.   Past experience does not count because IFR proficiency is a perishable commodity.  Instrument flying must be used or lost.  Any cockiness in either student or instructor must be replaced by desire to find and correct any deficiencies.  The good instructor will give the student room to make mistakes before intervening.  Waiting until the student notices the mistake is the best learning teaching technique.

One aspects of competency is to have personal minimums and laternate plans that will give you a way out.  If you begin to have IFR difficulty the first

move should be to lower the gear and adding some flaps.  The worst thing that can happen is to have the aircraft accelerate due to spatial disorientation.  Slow up to give youself a chance to catch-up.  Hood time

does not prepare you for the actual donditions of turbulence that can be

expected in actual conditions.  It you are short of actual IFR experience, always have an alternative plan.

Certain aspects of instrument competency and flying can be equated with sports training.  Your performance is the result of applying knowledge and skill to attain a selected outcome.  To raise your level of performance

you should make it more difficult to perform.  Like running with weights.

Set a standard of zero tolerance for radio communications.  Keep busy so you don’t become mentally lazy or bored.

While flying weekly may make you current, it doesn't follow that you are proficient.  Partial panel skills are quickly lost.  If you are flying partial panel, fly to an airport that offers an ASR approach.  Since partial panel is always a possibility, have a plan and use all available resources.

Pilots seldom have opportunity to practice partial panel.  Those who fail to remain proficient believe they will never have an instrument fail..  (True, only if you never fly.)

The Instrument Competency Check (ICC) is a good way to review your established skills and detect weaknesses.  It is a combination evaluation and learning experience.  Weak areas seems to be aircraft systems and weather interpretation.  Partial panel skills and situational awareness in non-radar environments cause problems.  Single radio and ADF operations offer an added challenge.  The best way to fly with competence is to have

high standards.

Preparation for an ICC should include study and review of the POH, FARs, and AIM.  Prepare the planned approach especially terrain clearance requirements.  Rember that currency is used to meet the FAR requirements of the FAA.  Proficiency is what we do to fly with competence and safety.  Time

alone does not give either competence or proficiency.

The whole of proficiency includes preflight planning of procedures, the getting of clearances , the actual flying the procedures, the use of radio and navigation aids, doing approaches and emergencies.

The weak area of flight preparation seems to be getting the appropriate notams for the flight.  On a long flight the local notams may not be available and must be acquired enroute.  The proficient pilot is able to

get the desired information in the most efficient manner and in the least time.  Once underway, the pilot should review clearance expectations and then be prepared to copy a clearance that does not meet these expectations.

Proficiency in the flying of IFR cannot be practiced in VFR.  Altitude holding and changes require constant practice.  Airspeed changes are an essential proficiency skill not easily acquired but quickly lost.  Rat maneuvers of climb and descent are equally required.

INSTRUMENT COMPETENCY CHECK PLAN AND CHECKLIST

Date____________

Name _______________________Certificate No______________

Certificate and Ratings _________________________________

Date of Last Check _______________

Class of Medical_________________Dated____________

Total time_____________ Time in type_____________

Total instrument time: Hood______, Actual______, Trainer____

In last 6 months:      Hood______, Actual______, Trainer____

Approaches/Last 6 months:  Precision____, Non-precision _____

Aircraft to be used _______________ Registration #__________

Location of check __________________________________________

KNOWLEDGE PORTION OF COMPETENCY CHECK.

A.FAR Part 91 review

1. Instrument Flight Rules

2. Equipment, Instrument, and Certificate Requirements

3. Maintenance

B. Instrument en-route and approach charts, SIDs and STARs

C. Weather analysis and knowledge

D. Preflight planning, including performance data, fuel, alternate, NOTAMS, 	and FAA publications.

E. Aircraft systems as related to IFR operations

F. Aircraft flight instruments and navigation equipment, including emergency 	procedures such as lost communications

G. Airworthiness status of aircraft and avionics for IFR flight

H. ATC procedures, clearances, and pilot/controller

responsibilities

I. Other areas:

II SKILL PORTION OF COMPETENCY CHECK AT ________________

A. Instrument cockpit check

B. Intercepting/tracking VOR/NDB

C. Steep turns

Stalls

D. Recovery form unusual attitudes

Basic coordination maneuvers

Elevator-throttle coordination

Constant speed transitions

Vertical S-1

Vertical S-2

E. Basic attitude instrument flying

Straight-and-level hand on/off

Level turns to headings

Airspeed climbs/descents

Airspeed turning climbs/descents

Rate climbs/descents

Rate turns in climbs/descents

F. VOR approach, orientation, intercept

G. NDB approach, orientation, intercept

H. ILS approach, orientation, intercept

I. Holding procedures

NDB, VOR, intersection

J. Missed approach

VOR, NDB, ILS

K. Circling approach

Short approach

L. Radio failure

Emergency procedures

M. Other areas:

Performance Areas of the Competency Check

- Ask for clarification of any part of a clearance that is dubious.

- Don’t ask the one giving the check what was said.  This is a single pilot operation.

- Don’t delegate responsibility.

- Get ATIS off ATC or via speaker.

- Always check the facility ident.

- Use the heading bug before changing heading.

- Develop and retain a quick scan of the instruments.

- Read any chart little bits at a time.  Three seconds max.

- Aircraft control is first.

- Don’t break any minimums ever.  Set personal minimums 100’ high if you are out of currency.

- Don’t accept an ATC clearance, hold, or vector if you have lost situational awareness and are not certain that it is safe.

III OVERALL COMPLETION OF COMPETENCY CHECK

Remarks:_____________________________________________________

_____________________________________________________________

________________________ Date_______

Eugene L. Whitt CFII

# 1876572 Expires ______

I have received an instrument competency check which consisted

of the knowledge review and skill demonstration of the

procedures noted.

______________________ Date__________

(Signature of pilot)

Staying Current

Paperwork

logbook, certificate, medical, POH, wt/balance papers,

Recurrent training

Maneuvers and emergencies likely to be encountered

2 hours flight time:

Taxiing and Runup

Instrument check

Nav checks

Electrical check

Instrument checks (vacuum etc.)

Radio setup

Departure

Steep turns

Stalls

Unusual attitudes

(dirty tricks)

During unusual attitudes flip HI 180 degrees

Then cover AI and HI with no-peeky

Fly to charted intersection

Uncover only HI and pull gear breaker

(Must remember to set HI to compass)

When established toward intersection give holding instructions			(Fail VOR that has ILS)

Hold at VOR and NDB

(Fail alternator, smoke in cabin, pitot/static

failure, gear procedure)

Approaches

NDB (Pilot should discover gear breaker pulled)

VOR approach using compass

Fly ILS or LOC with AI covered

Fly ILS with full panel

Basic Instrument Flying

Required Performance

Begin with the basics. Start out with the 10-10-100

- Straight and level

- Straight climbs and descents by constant airspeed and rate

- Rate level, climb, and descent turns to headings.

- Speed changes Cruise to low cruise to slow flight to minimum controllable and back again.  Most difficult of basic exercises.

- Steady state attitudes trimmed for hands-off

Speed Parameters for C-172

- Vso 50

- Vref 60

- Initial approach 105

- Intercept final 90

- Glideslope 90

- Landing 70

- Missed Approach 90

Required Skills

- Instrument cross check

- Interpretation of instruments

- Control of pitch, power, and bank

What is the 2/2/20 club?

Being able to consistently fly IFR within:

2 degrees of heading;

2 knots of airspeed;

and 20 feet of altitude.

Works for VFR too.

Emergencies

The idea is to see how soon the pilot recognizes and corrects the problem.

- Assigned headings that intercept false glide slopes

- Surprise holding situations

- 180° change in heading indicator

- Situational awareness exercises

When first beginning IFR training the multiplicty of tasks so divides our attention that the putting together of the puzzle seems impossible.

There are some ways not only to fly better but to fly smarter.

A good pilot will anticipate rather than react.  Thinking ahead of the airplane is a necessity, not a choice.

Scan is an essential ingredient to good IFR piloting.  A proficient scan can only be maintained by continual practice.

You can get away with some memorized checklists.  Written checklists

are the preferred method of experienced IFR pilots.

Be exact in your actions.  Know how much power, trim, rudder, lkit takes to do what.  Organize the flow path of what you do.

Know where you arein the world around you.  Nothing, but nothing, so

disables the throught processes as being lost, confused or misplaced.

Learn enough of the codes so as to know tha navaid identifier when you hear it.

Organize your flight materials.  there is considerable difference between having something and knowing just where you have it.

Use all your ATC and cockpit resources.  Every radio and navaid should be used effectively.  Knowing you need assistance requires a companion factor of being able to ask for it.

Divide the things you do into priorities.  Do the primary things in order.  Secondary things must wait both their turn and the time of doing.

Standards are just averages.  Fly to a higher level in maintaining altitude, heading  and airspeed.

The pre-approach preparation of weather, getting plate essentials;l, setting radios and navaids is completed before reaching the initial approach fix.

If you are not doing something then there must be something you should be doing.

Getting behind is part of life and flying.  Slowing down the airplane is the best way to catch up to it in the cockpit.

Beginning with aircraft control we find that flying the airplane must be removed as part of the equation.  As a student instrument pilot or as a retread, you must know where every power setting, trim change, and attitude

is for a particular aircraft.you must know where you want the aircraft to be relative to speed, attitude, configuration.  You are FULL of anticipation.

You anticipate the required throttle movement, anticipate the required trim

and anticipate the required attitude.  No reactions, all anticipation.

With anticipation comes smoothness.  Controls are pressed lightly.  Controls are pressed into position and trimmed to stay there.  Don’t press a

control unless it needs to be pressed.  Always apply half as much pressure as seems to be needed and you will achieve the smoothness of a favorite

drink.  Fatigue becomes a factor in instrument flying but it is not

physical.  The instrument pilot flies so lightly that the controls spend

most of their time unmoving.  Things stay where they are supposed to stay

because they were put there in the first place.

The argument as to whether you use elevator or throttle to control airspeed and altitude is moot.  Neither work independently of the other to control airspeed and altitude.  Elevator, by itself, controls attitude.  Power, by itself, controls thrust.  Stabilized flight conditions such as

level or glide slope do require that elevator control altitude and power to control airspeed.  In another situation, where by design, power is not a variable, elevator is used to adjust speed.  Elevator gives relatively fine

speed control when speed is a priority.  To do this altitude must be available to lose or gain.  Power tends to be coarse, slow, and inaccurate when controlling airspeed.

The instrument pilot is thinking ahead of the airplane.  There is

a specific altitude, heading, and airspeed for every situation.  He is mentally there ahead of the aircraft and presses it (the airplane) into position.  Once the airplane is controlled, instrument flying skills moves

to the instruments.

The scan, or crosscheck is probably best learned from a specialist.

Some, like me, learned in a Link Trainer as my own teacher.  What works for me is probably not desirable for you.  Go to a simulator facility and take lessons specifically in cross-check skills.  Even with the training you will

put your own touch into how you scan.

Scan, done properly is the major fatigue factor of flying.  You must

move beyond looking, into seeing, and finally into understanding. (interpretation).  The best illustration of this skill is one I used on rainy days while teaching elementary school.  I would have a row of children

put their heads down while I ‘hid’ a coin in plain sight but in an unusual place.  The children would then leave their seats and look for the coin.

The first one to see the coin would take their seat without revealing where the coin was. The coin would be looked at over and over but not seen.  Some

children always seemed to have better ‘seeing’ skills than others.  Some of the seers would grab at the coin instead of sitting down.  Try it in the pilots lounge some time.  Seeing is a developed skill.  It can be taught.

Having a scan is not enough.  Most any pilot can move his eyes in an

ordered sequence as required for a particular maneuver.  Doing the correct scan at the correct time at the correct speed is where the fatigue factor

comes into play.  A slow cross-check that suffices for level flight must shift gears when making a turn, shift still further when making a turning

descent, and even further when making a timed turning descent to specific heading.  Eye muscles can be trained for strength and rapid movement.  Some

opthamoligists/optometrists have a device to do this.  The use of a computer generated panel may help but I question if the amount of movement meets actual cockpit requirements.

The competent basic instrument pilot must know the specific scann and

speed required for every given situation.  Steady state flight has a scan for level, descent, and climb.  Turns add a new dimension and a new scan

Different kinds of turns have still different cross-check requirements.

You need to know and use the scan and cross-check required.  A cross-check

that lacks the proper speed, sequence, stop, and interpretaton is going to break down.  Fixing the results of an improper cross-check is much more

difficult than getting it right and keeping it in the first place.

If you know the cross-check sequence you are going to be aware

of the hazard of fixation.  You are far more likely to fixate on a

movement.  If the movement is not where it is supposed to be fixation

is even more likely.  As you detect, by quick glances, the movement of

instruments you should note that some movements are in the proper direction and proper amount.  Your scan sequnce is seeking out the distinctions between what should be and what is.

As a basic instrument pilot you can improve your scan, cross-check, and seeing skill and speed.  To make the improvement you must deliberately

push you speed.  You can’t push your speed well unless you know where your

eyes are supposed to move.  You should avoid going back to a single instrument twice in a row.  Never, is a dirty word in flying but it this

instance it applies.  If you know the information required for a given maneuver you can keep your cross-check moving through the instruments that

have and give that information.  Knowing where to look lets you see change

as it happens.  Seeing something happen must conform to your mental picture of what is supposed to happen.  If the pictures are in conflict you press

controls to bring the mental picture desired/required into an overlay of what the instruments show.  Easy to say, not so easy to do at first.

Flying to Practical Test Standards

New Instrument Rating Pratical Test Standards

FAA-S-8081-4B  $2.50

Get, study, and make performance cards from POH.  Make not of special system procedures such as vacuum backup, gear, and emergency.

Review procedures for getting weather, notams, and different methods of filing flight plans.  You should be familiar with pop-up, tower Pitot,

void-time, and pre-filed procedures.  You cannot maintain proficiency by only flying 6, 6 and 6.

Once learned, procedures are best practiced in an airplane.  Non-precision which require circle-to-land require special attention.  There are several altitude 'busts' possible and you must be able to maneuver close to the airport for landing.

Each objective in the PTS is given in four parts.  There is a certain amount of knowledge required, a recommended scan technique,  a performance standard, and partial panel performance.  Straight-and-level requires heading within 10 degrees, altitude within 100' and airspeed within 10 knots.  For an instructor to sign off a student as competent these parameters should be cut in half.

FAR 61.45 requires applicant to bring an airplane in which all required tasks can be demonstrated.

Reason for PTS Checkride Failure

1. Inability to perform DME arc approaches.  If DME exist in aircraft

DME arc must be performed.

2. Partial panel approaches.  Use partial panel approaches from 			beginning of instruction.

3. Aircraft Systems must be understood as required by PTS.

4. Orientation and situational awareness due to lack of experience.

5. Radio failure procedures.  Memorization won’t work.  Fly practice 		IFR flights as VFR with radio failure simulations that require 		use of procedures.

PAST STUDENT ERRORS

kNOW WHAT YOU WANT TO DO BEFORE YOU TAKEOFF

THERE IS NO CODE IDENTIFYING ATC AS A NAVIGATION TOOL

IF YOU CAN'T DEPART VFR AND STAY VFR REMAIN ON THE GROUND AND GET A VOID CLEARANCE TIME.

GETTING TIME

OFF

TIME AT VOR'S

TIME AT INTERSECTIONS

TIME AT FAF

READING CLEARANCES

REPEATING ESSENTIALS

CHECKING

IDENTIFYING

VOR, LDA, ILS, NDB

CARD SETTING AND FREQUENCIES

COMMUNICATIONS

MOVING #2 TO #1 AND SETTING NEW #2

SETTING SELECTOR SWITCH

INITIATING CLIMB/DESCENTS

AIRSPEED CONTROL,

LEVELING OFF,

HOLDING ALTITUDE

CHECKPOINT CHECKLIST

TIME, TURN, TUNE, THROTTLE, TALK

GUAGES CHECK

COMPASS/HEADING INDICATOR CHECK

GETTING ATIS EARLY

USING NDB FOR ORIENTATION EVEN THOUGH

NOT PART OF APPROACH

IF VOR DOES NOT IDENT-CHECK VIA RADIO ON OPERATION

USE APPROACH TO CALL POSITION IF EQUIPMENT IS DOUBTFUL

ATC MAKES MISTAKES-

FAILS TO REQUIRE FREQUENCY CHANGE-

OVERSHOOT OF VECTOR TO FAF WITHOUT ADVISING---

THE BETTER YOU KNOW WHAT

IS SUPPOSED TO HAPPEN THE BETTER YOU CAN PROTECT YOURSELF

DON'T BE AFRAID TO ASK ATC FOR SPECIAL CONSIDERATIONS--

DESCENT TO MINIMUMS-

STRAIGHT IN-

SHORT APPROACH

IF UNREASONABLE PERFORMANCE IS REQUESTED DON'T HESITATE

TO LET ATC KNOW AND SUGGEST ALTERNATIVE

IF YOU ARE TOO BUSY TO COPY A CLEARANCE-SAY SO-SUGGEST

DELAY

FAILING TO COPY A FREQUENCY OR TRANSPONDER CODE

FAILING TO ACKNOWLEDGE A HEADING--DO THIS WHILE TURNING

FAILING TO GET A TRANSPONDER CODE PRIOR TO DEPARTURE

RAILING TO SLOW DOWN BEFORE ARRIVAL AT HOLDING OR APPROACH

FAILING TO SET RADIO SELECTOR SWITCH TO CORRECT RADIO

LOOKING AT APPROACH PLATE IN LAST MINUTE OF APPROACH TO

CONFIRM MDA OR TIME OR DH.  MUCH TOO LATE

INSTRUCTOR WENT TO RADIOS BEFORE STUDENT HAD CHANCE TO

DEAL WITH TOTAL COM SYSTEM FAILURE AT TRAVIS

INSTRUCTOR MISSED CLEARANCE

USING WRONG VOR TO GET INTERSECTION

FAILING TO SET UP MISSED APPROACH WHEN CLEARED FOR APPROACH

APPROACH FAILED TO SET VOR INTERSECTION RADIALS

WHEN ON NDB APPROACH VECTOR HEADING AT ANGLE TO APPROACH MUST

BE USED TO DETERMINE WHEN TO TURN ON FINAL.

VECTOR TO FINAL BY ATC MAY NOT ALLOW SUFFICIENT DESCENT TIME.

KEEP TRACK OF POSITION AND REQUEST LOWER IF NEEDED.

ON BEING IDENTED ADF WAS NOT CHANGED FROM REC TO ADF.

PREFLIGHT REVIEW OF FREQUENCIES AND OBS SETTINGS NOT DONE.

RESULT WAS FAILURE TO GET INTERSECTIONS.

FSS CONTACT/FILING FLIGHT PLAN

IFR PRESTART CHECKLIST

FREQUENCY LIST

DEPARTURE/ARRIVAL PLATES

CLEARANCE PRIOR TO TAXI PROCEDURES

ALL RADIOS SET AND ON

IFR TAXIING CHECKLIST

INSTRUMENT CHECK

PRE TAKEOFF CHECKLIST

TIME

TRANSPONDER

HEADING CHECK

IDENT VOR?

Competence Trap

After completing IFR basics combine steep turns, unusual attitudes and interception of a VOR radial to create an orientation trap.  Advise the student that the next series of maneuvers will be steep turns, unusual attitudes and VOR radial interception.

Place student under hood and have him enter a steep 360-degree turn with recovery on original heading.  Do this in both left and right turns.

Have the student prepare for entering unusual attitudes with head down and eyes closed.  Reach over and give the HI a 180-degree reverse setting.  Give the student several unusual attitude situations before initiating the VOR radial interception.  If the student fails to check and reset the HI a severe case of positional unawareness should occur along with possible loss of control.