T-38 Takeoff and Landing Restrictions

MCI  11-238 / AETC Sup 1 / CAFB Sup 1 / Dash 1

Takeoff:

Intersection takeoffs are prohibited.	AETC

Minimum runway length is 8000'.	11-238

OG/CC may waive minimum to7000'.	AETC

Takeoffs behind a full stop require 6000' separation.	AETC

Rolling takeoffs may only be performed:	AETC

day, dual, single ship, ARL > CFL by 500'.

Maximum cross-wind limits:

dry (RCR 23)			30 kts	Dash 1

wet (RCR 12)			20 kts	Dash 1

icy / standing water (RCR 5)	10 kts	Dash 1

solo student			15 kts	AETC

Formation:

Two ship per element maximum.	11-238

Use single ship TOLD for formation takeoffs.	AETC

Formation solo, wing takeoff only,	11-238

50' wingtip spacing.	AETC

If cross-wind > 5 kts, put wingman on upwind side.	11-238

Minimum WX for takeoff:

500' / 1 ½ nm or PWC whichever is higher.	CAFB

Interval takeoff between aircraft / elements:

10 sec minimum spacing.	11-238

Min WX for VFR rejoin, 3000' / 3nm.	AETC

Trail departure, 20 sec minimum spacing.	11-238

No formation takeoffs if:

standing water , snow, or ice on runway	11-238

runway width < 125'	11-238

cross-wind > 15 kts	11-238

CFL within 1000' of ARL	AETC

Cruise:

Do not climb or descend through forecast icing

more severe than light rime.	11-238

Do not cruise in any known icing conditions.	AETC

Landing:

240 kts minimum to pull closed.	AETC

If landing roll exceeds 80% of runway, land at alternate.	11-238

Do not land over any raised web barrier MA-1A, BAK-15.	11-238

Similar a/c, alternate sides of runway, 3000' spacing.	AETC

Dissimilar a/c, or same side of runway, 6000' spacing.	11-238

Practice single engine go-around, 300' AGL minimum.	AETC

Emergency chase, 300' AGL minimum.	11-238

Restricted low approach, 500' AGL minimum.	11-238

Cross-wind > 15 kts, land on center of upwind side.	Dash 1

Maximum cross-wind limits:

Dry (RCR 23)			30 kts	Dash 1

Touch and Go			25 kts	AETC

Wet (RCR 12)			20 kts	Dash 1

Solo student			15 kts	AETC

Icy, standing water (RCR 5)	10 kts	Dash 1

Formation:

Two ship only.	11-238

Accomplish from a straight in approach.	11-238

If cross-wind > 5 kts, put wingman on upwind side.	11-238

10' wingtip spacing when stacked level.	11-238

Formation low approaches, 100' AGL minimum.	11-238

Solo formation low approach, 300' AGL minimum.	AETC

Minimum WX for landing:

500' / 1 ½ nm or PWC whichever is higher.	11-238

No formation landings if:

nightime	AETC

solo student	AETC

cross-wind > 15 kts	11-238

runway is wet; ice, slush, or snow on runway	11-238

runway width < 125'	11-238

|  | Normal recovery | Solo recovery | Min fuel | Solo min fuel | Emer fuel |
| --- | --- | --- | --- | --- | --- |
| 11-238 | 800 | X | 600 | X | 400 |
| AETC | X | 900 | 700 | 800 | 500 |
| Dual rwy op CAFB | X | X | 1000 | 1200 | X |

CAFB single runway ops: land with no less than 1200 Lbs, student solos full stop as soon as practical.

| 21 Sep 97ACROBATICS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proficiency KIAS | Proficiency Power | Post-prof. KIAS | Post-prof. Power |
| Lazy 8 | 350 | 95% | 300 - 400 | 90% - mil |
| Barrell roll | 400 | 95% | 300 - 500 | 90% - mil |
| Chandelle | 400 | 95% | 350 - 450 | 90% - mil |
| Split S | 200 | mil | 200 | 90% - mil |
| Cloverleaf | 450 | mil | 400 -500 | 550c - mil |
| Loop | 500 | mil | 450 - 500 | 550c - mil |
| Immelman | 500 | mil | 450 - 500 | 550c - mil |
| Cuban 8 | 500 | mil | 450 -500 | 550c - mil |

| Aircraft Handling Characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pitchback | 450 -500 kias | 550c - mil |
| Sliceback | 200 -300 kias | 90% - mil |
| Stab Demo | 60 nose high | 85% min |
| Aft Stick Stall |  | 80% min |
| Sim Trim Failure | < FL 200 | 300 kias |
| Rudder Effectivness |  | 85% - mil |
| Aileron Effectivness |  | 85% - mil |
| Accel Exercise |  | mil - max |
| Turn Reversals | 300 - 400 kias | min of mil, 4G |

| Instrument Exercises |  |  |  |  |
| --- | --- | --- | --- | --- |
| Steep Turns | 45 - 60 bank | 300 or 350 kias |
| Wingover | 350 kias | 95% |
| Aileron Roll | 300 kias (min) | 85% (min) |

| Miscellaneous |  |  |  |  |
| --- | --- | --- | --- | --- |
| G - Warmup | 400 +/- 20 kias | 4 G then 5 G |
| TP Stalls | 8000AGL to FL200 | 80% min |
| Slow Flight | bug - 10 kias |  |