STUDY NOTES

Contact Phase Test

# Best single engine climb: 60% flaps = 190 knots, 0% flaps = 220 knots (high gross weight)

- If engine seizes wait until 220 kts to raise flaps

- Oil system capacity = 4 qt with 1 quart expansion space

- Fuel low pressure light is a valid indication of a boost pump failure if the throttle is out of off, fuel shutoff is in normal and crossfeed off.

- No takeoffs from Live Oak allowed if CFL is greater than 6000’

- Standard VASI has 2.5 – 3.0 glideslope, aims 750’ down the runway and should allow touchdown in the first 2000’ of the runway

- Non-standard VASI has 3.0 – 3.5 glideslope, aims 450’ before the runway and should allow touchdown in the first 1000’ of the runway

- On a 3 glideslope, you are 300AGL 1nm prior to the threshold

- Actual stall is indicated by excessive sink and heavy buffet

- The three basic factors that effect stall recovery are AOA, back and thrust

- What causes a stall?  Exceeding the critical AOA

- CDI must center within 4 of the CSW to a known bearing

- ADI and HIS warmup time is 3.5 minutes

- ADI off flag goes out of view in 1 minute after initial AC power is applied

- TACAN warmup time is 90 sec

- Standby ADI warmup time is 3 minutes

- Standby ADI is good for 9 minutes after loss of power 6

- ADI off flags:

- 12 o’clock – localizer signal is bad (course warning flag)

- 9 o’clock – glideslope signal is bad

- 7 o’clock – ADI off flag – may not show if internal failure or generator phase failure

- Don’t taxi within 25’ of an object without wing walkers, never taxi within 10’ from an object

- Day solo flights can’t be accomplished in the 15 minute extended daylight window

- Min fuels:

- Normal Ops:  600 Dual, 800 Solo

- Dual Runway Ops:  1000 Dual, 1200 Solo

- Bug speed is the highest of ARS, CEFS and DS

- Rolling takeoffs are allowed on wet runways – the rolling takeoff requirements are Dual, Day, Single ship & CFL not within 1000’ of runway length

- Min airspeed to request closed = 220 kts, min airspeed to pull closed = 240 kts, min airspeed in closed pullup = 200 kts

- Closed downwind fly a max of 240 kts and a min of final turn speed

- Climb and level off checks can be combined below FL180

- Minimum CEFS is 110 unless corrected for RCR

- As TOF rises, SETOS rises

- Min airspeed for MA-1 barrier engagement is 60 kts, aircraft damage may occur above 120 kts, MA-1 dynamic limit is 150 kts

- Airframe mounted gearbox shaft failure will give master caution, generator and hydraulics lights

- Oil pump shaft failure will give simultaneous RPM indication drop to zero and complete oil pressure loss – you’ll also loose the T5 system, so over temps may occur above 97%

- A 2-3g pull on the top side up the cloverleaf should give a 175 – 200 knot speed over the top

- Hydraulic caution light will illuminate for pressure under 1500psi and will go out when the pressure goes about 1800psi – it will also illuminate for an excessive temperature

- Restricted low approach is 500AGL, low approach as wingman during an emergency is 300AGL, normal low approach just don’t touch down

- Add ½ of gust factor to final approach and touchdown airspeeds – not to final turn airspeed

- Recover in pattern anytime stall indication or excessive sink rate exists

- Add headwind (subtract tailwind) from ARS & RS

- Adjust landing roll with RCR using charts

- Max endurance AOA is 0.3, max range is 0.18, optimum AOA in the pattern is 0.6

- Throttle bursts from IDLE to MIL in one second or less can cause blade failure

- Oil limits at 0g is 10 seconds, negative g’s is 30 seconds

- Max airspeed with flaps from 1 - 45 is 300 kts, 45 - 60 is 240 kts, 60 - 100 is 220 kts

- Fast slave cycle starts:

- Interruption of AC power

- Switch from Dir Gyro – Mag

- When the fast slave button is pressed

- With a RS of 160kts, an RCR = 10 drops RS to 124 kts

- Utility hydraulics powers: flight controls, stability augmentation, nosewheel steering, landing gear, speed brake (FANGS)

- After aborting a start, purge with air for 2 minutes before attempting another start

- When told to standby after requesting closed you can turn crosswind at the end of the runway, or wait until 2 miles past the departure end

- RS assumptions are:

- 3 second reaction time

- optimum braking in a 3 point attitude

- 1 engine at IDLE, 1 engine windmilling

- In a MOA on an IFR clearance, you need 3 miles visibility and must remain clear of clouds to do aerobatics

- On an IFR flight plan the pilot is still ultimately responsible for clearing

- Landing with 60% flaps increases landing roll by 500’

- Landing distances:

- Normal = 2500 + fuel + flare

- 60% flaps = 2500 + 500 + fuel + flare

- No flap = (2500 + fuel)x2 + flare

- Crosswind procedures with greater than 15 kts crosswind

- Land on upwind side of runway

- Stop aircraft without aerobraking – keep landing attitude

- 6000’ spacing

- Full aerobrake is approximately 12 nose high

- Normal level off lead point is 10% of VVI.  If VVI is pegged, cut climb rate in half 2000’ prior

- Rolling takeoffs increase takeoff distance by 150 – 300’

- With an inop airspeed indicator, fly known pitch and power settings and use 0.6 AOA on final

- Initial takeoff abort – you can roll full flaps, but you must be below full flap takeoff speed and at full aft stick prior to rolling flaps to full

- Speedbrake becomes ineffective below 250 kts

- Restricted climb is accomplished by attaining 400 kts passing 10,000’

- With flap-slab interconnect failure & no locking device, a fully configured aircraft will have an abrupt pitchup & stall immediately (not after 3 seconds) – reaction (MAX and full forward stick) within 3 seconds is required to keep from losing altitude

- Landable configurations for the T-38:

- All gear down

- All gear up with

- Mains down, nosegear up

- Gear door position doesn’t matter

- Know ops limits for lineup – fluctuations out of the normal range are bad

- Stalls must be accomplished between 8000 AGL and FL200 – use at least 80% RPM