Table 1 The measured data of rotameter float height, manometer positions, mass of water and time taken by water with the related mass flow rate.

| Rotameter height (mm) | Manometer position (mm) |  |  |  | Mass (kg) | Time (s) | Mass flow rate (kg.s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Position 1 (water entering venturi meter ) | Position 2 (water leaving venturi meter) | Position 5 (water entering orifice plate) | Position 6 (water leaving orifice plate) |  |  |  |
| 165 + 1 | 297 + 1 | 152 + 1 | 286 + 1 | 123 + 1 | 15 + 0.3 | 42.0 + 0.5 | 0.357 |
| 145 + 1 | 264 + 1 | 150 + 1 | 255 + 1 | 126 + 1 | 15 + 0.3 | 48.0 + 0.5 | 0.313 |
| 126 + 1 | 243 + 1 | 153 + 1 | 235 + 1 | 135 + 1 | 15 + 0.3 | 54.0 + 0.5 | 0.278 |
| 105 + 1 | 223 + 1 | 157 + 1 | 216 + 1 | 143 + 1 | 15 + 0.3 | 63.5 + 0.5 | 0.236 |
| 84 + 1 | 205 + 1 | 157 + 1 | 198 + 1 | 143 + 1 | 7.5 + 0.3 | 37.0 + 0.5 | 0.203 |
| 62 + 1 | 187 + 1 | 155 + 1 | 182 + 1 | 147 + 1 | 7.5 + 0.3 | 46.5 + 0.5 | 0.161 |
| 43 + 1 | 184 + 1 | 163 + 1 | 179 + 1 | 157 + 1 | 7.5 + 0.3 | 58.0 + 0.5 | 0.129 |
| 25 + 1 | 177 + 1 | 164 + 1 | 174 + 1 | 160 + 1 | 7.5 + 0.3 | 74.0 + 0.5 | 0.101 |
| 1 + 1 | 207 + 1 | 201 + 1 | 205 + 1 | 200 + 1 | 7.5 + 0.3 | 117.0 + 0.5 | 0.064 |

Table 2 The calculated results for the venturi meter

| Rotameter height (mm) | Change in manometer height (m) | Change in pressure (Pa) | Velocity (m.s-1) | Kinetic energy per unit volume (J. m -3) |
| --- | --- | --- | --- | --- |
| 165 + 1 | 0.145 | 1422 | 1.345 | 905 |
| 145 + 1 | 0.114 | 1118 | 1.177 | 693 |
| 126 + 1 | 0.090 | 883 | 1.046 | 547 |
| 105 + 1 | 0.066 | 647 | 0.890 | 396 |
| 84 + 1 | 0.048 | 471 | 0.764 | 292 |
| 62 + 1 | 0.032 | 314 | 0.608 | 185 |
| 43 + 1 | 0.021 | 206 | 0.487 | 119 |
| 25 + 1 | 0.013 | 128 | 0.382 | 73 |
| 1 + 1 | 0.006 | 59 | 0.241 | 29 |

Table 3 The calculated results for the orifice plate

| Rotameter height (mm) | Change in manometer height (m) | Change in pressure (Pa) | Velocity (m.s-1) | Kinetic energy per unit volume (J. m -3) |
| --- | --- | --- | --- | --- |
| 165 + 1 | 0.163 | 1599 | 0.541 | 146 |
| 145 + 1 | 0.129 | 1265 | 0.473 | 112 |
| 126 + 1 | 0.100 | 981 | 0.421 | 89 |
| 105 + 1 | 0.073 | 716 | 0.358 | 64 |
| 84 + 1 | 0.055 | 540 | 0.307 | 47 |
| 62 + 1 | 0.035 | 343 | 0.244 | 30 |
| 43 + 1 | 0.022 | 216 | 0.196 | 19 |
| 25 + 1 | 0.014 | 137 | 0.154 | 12 |
| 1 + 1 | 0.005 | 49 | 0.097 | 5 |

Venturi meter constant:

m = Δy = 1500   (Pa) = 1.61

Δx      930 (J.m-3)

Therefore the constant for the venturi meter is 1.61.

Orifice plate constant:

m = Δy = 1780   (Pa) = 11.13

Δx      160 (J.m-3)

Hence the constant for the orifice plate is 11.13.