UMTS RF
Optimization Guideline

xxx-xxx-xxx
Issue 1.0
August 2003

The copyright laws of the United States and other countries protect this guideline. It may not be reproduced, distributed, or altered in any fashion by any entity (either internal or external to Lucent Technologies), except in accordance with applicable agreements, contracts, or licensing, without the express written consent of the Author.

For permission to reproduce or distribute, please contact:

Lucent Technologies Network Systems GmbH

Network & RF Engineering Department in EMEA

Thurn-und-Taxis-Strasse 10
90411 Nuremberg, Germany

Contact: 	Andreas Conradi (aconradi@lucent.com)
Phone: +49 911 525 3177

Notice

Every effort was made to ensure that the information in this information product was complete and accurate at the time of printing.  However, information is subject to change.

1.	Introduction	8

2.	Overview of UMTS System Operation	10

2.1.	High Level Architecture Model	10

2.2.	Radio Interface Protocol Architecture	11

2.3.	Sub-Layers Functionality	12

2.3.1.	Physical Layer	12

2.3.2.	Medium Access Control (MAC) Protocol	12

2.3.3.	Radio Link Control (RLC) Protocol	13

2.3.4.	Packet Data Convergence Protocol (PDCP)	14

2.3.5.	Broadcast/Multicast Control (BMC) Protocol	14

2.3.6.	Radio Resource Control (RRC) Protocol	14

2.4.	UMTS Channels	15

2.4.1.	Logical Channels	16

2.4.2.	Transport Channels	16

2.4.3.	Physical Channels	17

2.5.	RRC States and State Transitions	20

2.5.1.	General Description of Idle Mode	20

2.5.2.	UTRA RRC Connected Mode	22

2.5.3.	States Transitions	24

2.6.	Synchronization  and Scrambling Codes	26

2.6.1.	Scrambling Codes	27

2.6.2.	Synchronization Channels and Cell Search Procedure	28

3.	RF Optimization Aspects	31

3.1.	RF Coverage	31

3.2.	Cell Breathing	31

3.3.	Pilot Pollution	32

3.4.	Near-Far Problem	33

3.5.	Around-the-Corner Problem	33

3.6.	Handover	34

3.7.	Missing Neighbors	34

4.	RF Drive Test Based Optimization Process	35

4.1.	Process Overview	35

4.2.	Site Readiness	35

4.2.1.	General	35

4.2.2.	Spectrum Clearance Verification	36

4.2.3.	Antenna Audit	36

4.2.4.	Sector Verification	36

4.2.5.	Baseline Existing System	37

4.3.	Optimization Planning	37

4.3.1.	General	37

4.3.2.	Perform RF Parameter Audit	37

4.3.3.	Validate Initial Neighbor Lists	37

4.3.4.	Tool Readiness	38

4.3.5.	Define Clusters	38

4.3.6.	Drive Route Planning	38

4.4.	RF Optimization Execution	39

4.4.1.	General	39

4.4.2.	Cluster Optimization	39

4.4.3.	System Verification	41

5.	RF Optimization Tools	43

5.1.	General	43

5.2.	Agilent	44

5.3.	CAIT3G	46

5.4.	WINDS	47

5.5.	LDAT 3G	49

5.6.	Friendly Viewer	51

5.7.	UMTS Test Terminals	52

5.8.	Supplementary Tools	52

5.9.	RF Tool Kit	53

5.10.	Attenuation Box	54

5.11.	Tools and features for simulated cell loading	54

5.12.	RF Call Trace	55

5.13.	Pre-Optimization Tool Ocelot	56

5.14.	Service Measurement Tools – SPAT3G	56

6.	Generation of Voice & Data Sessions	58

6.1.	General	58

6.2.	Voice Call	58

6.3.	UDP Transfers	59

6.4.	FTP Transfers	59

6.5.	Ping	60

6.6.	Test Scripts	60

7.	UMTS Performance Metrics	61

7.1.	General	61

7.2.	Collecting Key Performance Data	62

7.3.	System Loading	63

7.4.	Key Performance Indicators	63

7.4.1.	General	63

7.4.2.	Voice KPI’s	63

7.4.3.	Data KPI’s	64

7.4.4.	Summary Tables for Acceptance Tests	65

7.4.5.	Summary Tables for Internal Tests	66

8.	UMTS RF Parameters	69

8.1.	General	69

8.2.	Primary Parameters	70

8.2.1.	Pilot Channel Power	70

8.2.2.	Antenna Parameter	70

8.2.3.	Neighbor Lists	71

8.3.	Secondary Parameters	71

8.3.1.	Cell Selection / Re-Selection	71

8.3.2.	Access Procedure	72

8.3.3.	Soft/Softer Handover	73

8.4.	UMTS ParCat	74

8.5.	NDP Database	74

8.6.	OMC-U	74

9.	Vocabulary	75

10.	Definitions Equations	84

11.	Abbreviations	88

References	93

Sources

For the preparation of this Engineering Guideline the following documents were used:

[]	3GPP TS 23.101 V4.0.0, General UMTS Architecture

[]	3GPP TS 25.301 V3.8.0, Radio Interface Protocol Architecture

[]	3GPP TS 25.211 V3.8.0, Physical channels and mapping of transport channels onto physical channels (FDD)

[]	3GPP TS 25.331 V3.8.0, RRC Protocol Specification

[]	3GPP TS 25.304 V3.8.0, UE Procedures in Idle Mode and Procedures for Cell Reselection in Connected Mode

[6]	3GPP TS 25.922 V3.6.0, Radio resource management strategies

[7]	3GPP TS 25.214 V3.9.0, Physical layer procedures (FDD)

[8]	Optimum power setting for pilot and control channel, Frank Beyer, Lucent

[10]	1xEV RF Optimization Guidelines, version 1.09, 07/11/2002, by: Vladan Jovanovic, Anu Sandhu, Hayder Kammona, Amit Shah

[11]	3G1x RF Optimization Procedures and Guidelines for PCS and Cellular CDMA Systems, 
	Version 1.0, Dec 2001, by Devesh Patel

[12]	CDMA RF Optimization Procedures for 1.9 GHz PCS Systems, Version 1.53, November 6, 1996, by V. DaSilva, M. Feuerstein, J. McElroy, S. Shio, X. Wang

[13]	CDMA RF PERFORMANCE ANALYSI S & TROUBLESHOOTING GUIDE REV 20.0

Version History

| Version | Changes |
| --- | --- |
| 0.1 | First draft version |
| 0.9 | Preliminary Version ready for review. |
| 1.0 | Preliminary Released Version |

1

# Introduction

This document presents a set of procedures and guidelines for RF Optimization of a Lucent Technologies’ UMTS network. RF Optimization consists of assessing and improving the network performance so that it meets contractual and technical objectives. RF Optimization is used initially during new UMTS deployments prior to a commercial launch. Nevertheless RF Optimization often ends up being a continuous process, as there are network configuration changes due to the addition of new cells and/or increased traffic.

The primary target audience for this document is the Lucent RF personnel responsible for the preparation and execution of the RF Optimization tasks. RF Optimization engineers are expected to be familiar with the Translation Application Notes for UMTS as well as with the corresponding Methods and Procedures documents. All references to these M&P documents can be found in Appendix A.

The overall UMTS RF Optimization process consists of the following phases:

- Pre-Optimization

- Drive Test Based Optimization

- Service Based Optimization

The Pre-Optimization is performed using Lucent Technologies’ optimization tool Ocelot (see chapter 5.13). Ocelot (pre-) optimizes antenna tilts, azimuths and power settings to improve coverage and capacity in the UMTS network. A pre-optimized network can drastically minimize the amount of drive testing required for the primary RF Optimization phase.

The RF Drive Test Based Optimization is the primary optimization phase. It is generally performed prior to a commercial network launch. The most significant objective of the RF Drive Test Based Optimization is to ensure adequate coverage, capacity, performance and quality in the network. Please refer also to MSS-RF-D028 UMTS RF Optimization – Drive Test Based

Similar procedures are followed during the Service Based Optimization phase.  This phase is performed in a commercial network with live network traffic. Lucent Technologies’ software tool, SPAT3G, and specific network performance counters are used to identify and optimize the problem areas. For general information regarding SPAT3G please refer to chapter 5.14. Refer also to MSS-RF-C005 Service Measurement Based RF Optimization.

The focus of this document is on Drive Test Based Optimization. Future versions of this document will have additional specifics related to ongoing RF Optimization scenarios.

Prior to any successful RF Optimization an adequate UMTS network design is necessary. The RF design guidelines for UMTS are covered in a separate document (see UMTS RF Engineering Guidelines).

In general the following deployment scenarios are found during the RF Optimization activities:

- A Greenfield scenario is defined as a brand new wireless mobility network. No history of 2G wireless systems is available to baseline RF performance.

- An Overlay scenario is defined as a wireless mobility system built over an existing system of a different air interface technology.

- A Swapout scenario is defined as a wireless mobility system replacing an existing system of the same air interface technology.

- A Network Expansion scenario is defined as an addition of cell sites or cells to an existing UMTS network to enhance coverage and capacity.

- An Additional Carrier scenario is defined as an addition of carriers to an existing UMTS network.

This optimization guideline is applicable for all of the aforementioned scenarios. Specific RF Optimization techniques, such as Hierarchical Cell Structure or Micro Cell implementation, need to be addressed on a case-by-case basis.

2

# Overview of UMTS System Operation

## High Level Architecture Model

In 3GPP the UMTS architecture is described in terms of entities- User Element (UE), UMTS Terrestrial Radio Access Network (UTRAN) and Core Network (CN). The radio interface Uu and the CN-UTRAN interface Iu are the reference points between the subsystems.

The protocols over the Uu and Iu are divided into two structures: User Plane protocols, i.e. the protocols implementing the actual radio access bearer (RAB) service: and Control Plane protocols i.e. the protocols for controlling the RAB and the connection between CN and UE.

The UMTS high-level functional communication can be grouped into Access Stratum (AS) and Non-Access Stratum (NAS).

The Access Stratum is the functional grouping of protocols specific to the access technique and it includes protocols:

- for supporting the transfer of radio- related information

- for coordinating the use of radio resources between UE and access network

- for supporting access from the serving network to the resources provided by the access network

The Access Stratum offers services through Service Access Points (SAP) to the Non-Access Stratum (CN- related signaling and services).

The Non-Access Stratum is the functional grouping of protocols aimed:

- at Call Control for circuit switched voice and data

- at Session Management for packet switched data

- at Mobility Management

- at RAB management for re-establishment of RABs, which still have a PDP context active.

- The radio access bearer (RAB) is a service provided by the Access Stratum to the Non-Access Stratum in order to transfer user data between UE and CN. A bearer is described by a set of parameters or attributes which define the particular traffic aspect or Quality of Service (QoS) profile of that particular service or application.

## Radio Interface Protocol Architecture

The radio interface protocols are needed to set up, reconfigure, and release the Radio Bearer services. The radio interface consists of three protocol layers:

- Physical layer (L1)

- Data link layer (L2) contains MAC, RLC, PDCP and BMC as sub-layers

- Network layer (L3) contains only RRC

- The different protocols are described in the section 2.3. Figure 1 shows the protocol architecture.

Figure 1: UTRA FDD radio interface protocol architecture

## Sub-Layers Functionality

### Physical Layer

The physical layer offers data transport services to higher layers. The access to these services is through the use of transport channels via the MAC sub-layer. The physical layer is expected to perform the following functions in order to provide the data transport service.

- Macro diversity distribution/combining and soft handover execution

- Error detection on transport channels and indication to higher layers

- Forward Error Correction (FEC) encoding/decoding of transport channels

- Multiplexing of transport channels and de-multiplexing of coded composite transport channels (CCTrCHs)

- Rate matching of coded transport channels to physical channels

- Mapping of coded composite transport channels on physical channels

- Power weighting and combining of physical channels

- Modulation and spreading/demodulation and de-spreading of physical channels

- Frequency and time (chip, bit, slot, frame) synchronization

- Radio characteristics measurements including BLER, SIR, Interference Power, etc., and indication to higher layers

- Inner - loop power control

### Medium Access Control (MAC) Protocol

The Medium Access Control Protocol is responsible for:

- Mapping the logical channels onto appropriate transport channels.

- Multiplexing and de-multiplexing functions of RLC packet data units (PDU) into and from transport blocks (TB) delivered to and from the physical layer on common and dedicated channels.

- Traffic volume measurements reported to RRC based on which the RRC performs transport channels switching decisions.

The data transfer services of the MAC layer are provided on logical channels. The type of information transferred defines each logical channel type.

### Radio Link Control (RLC) Protocol

The Radio Link Control Protocol is responsible for:

- Segmentation and reassembly of the upper layer PDUs

- Error correction in the acknowledgement mode (AM) mode

- Flow control (rate control at which the peer RLC may send information)

- Ciphering and maintenance of the QoS as defined by the upper layers.

- Each RLC is configured by the RRC to operate in one of the three modes

- Transparent mode (TM) where no overhead is added to higher layer data

- Unacknowledged mode (UM) where no retransmission protocol is in use and data delivery is not guaranteed.

- Acknowledged mode (AM) where the automated repeat request (ARQ) mechanism is used for error correction.

The RLC transfer mode affects the configuration parameters of outer-loop power control in the RNC and the user bit rate.

### Packet Data Convergence Protocol (PDCP)

This protocol exists only in the user plane and only for services from the PS domain. The main functions are compression of redundant protocol control information (e.g. TCP/IP and RTP/UDP/IP headers) at the transmitting entity and decompression at the receiving entity, and multiplexing radio bearers into one RLC entity, transfer of user data.

### Broadcast/Multicast Control (BMC) Protocol

Like the PDCP, the BMC protocol exits only in the user plane and it provides a broadcast/multicast transmission service on the radio interface for common user data in TM or UM mode. It utilizes UM RLC using CTCH logical channel mapped onto FACH

### Radio Resource Control (RRC) Protocol

The understanding of the Radio Resource Control (RRC) functions and signaling procedures is essential for radio network tuning and optimization. Through the RRC protocol analysis, it is possible to monitor system information broadcast in the cell, paging messages, cell selection and reselection procedures, the establishment, maintenance and release of RRC connections between the UE and the UTRAN, the UE measurement reporting criteria and their control and the forward power control.

The RRC Protocol is responsible for:

- Controlling the mobility of the UE in connected mode

- Broadcast the information related to the NAS and AS

- Establish, reconfigure and release of the Radio Bearers

- Setting up and controlling the UE measurement reporting criteria and the forward outer-loop power control

- Initial cell selection and cell reselection

- Carrying all the parameters required to set up, modify and release L2 and L1 protocol entities.

## UMTS Channels

The following section provides a general classification of the UMTS channels. These channels can be divided into three main classes:

- Logical channels,

- Transport channels

- Physical channels

Each of these classes being managed by different protocols in different layers. Figure 2 shows the channel mapping between the logical, transport, and physical channels.

Figure 2: UMTS Channel Mapping

### Logical Channels

The logical channels are divided into control and traffic channels.

#### Control Channels

- Broadcast Control Channel (BCCH) for system control information in the downlink.

- Paging Control Channel (PCCH) for transferring paging information in the downlink (used when the network does not know the cell location of the UE, or when the UE is in cell connected mode).

- Common Control Channel (CCCH) for transmitting control information between network and UE in both directions commonly used by UE when accessing a new cell after cell reselection and by UE’s having no RRC connection.

- Dedicated Control Channel (DCCH) is a point-to-point bi-directional channel established through the RRC connection setup procedure.

#### Traffic Channels

- Dedicated Traffic Channel (DTCH) is a point-to-point channel dedicated to one UE for transfer of user information (can exist in both directions).

- Common Traffic Channel (CTCH) is a point-to-multipoint unidirectional channel for transfer of dedicated user information for all or a group of UE’s.

### Transport Channels

In UTRAN the data generated at higher layers is carried over the air interface using transport channels mapped onto different physical channels.

In 3GPP all transport channels are defined as unidirectional, uplink or downlink or relay-link depending on services and state. The UE can have simultaneously one or more transport channels in downlink and one or more of them in the uplink. Two types of transport channels exist, dedicated channels and common channels.

#### Dedicated Transport Channels

The only dedicated transport channels specified in 3GPP is the Dedicated Channel (DCH), which supports variable bit rate and services multiplexing. It carries all user information and it is mapped onto the dedicated physical data channel (DPDCH). The DPDCH is characterized by closed-loop power control and fast data rate change on a frame-by-frame basis and it supports soft/softer handover.

#### Common Transport Channels

The common transport channels are a resource divided between all users or a group of users in a cell. They do not support soft/softer handover but some of them like Common Packet Channel (CPCH) and Downlink Shared Channel (DSCH) can have fast power control.

- Broadcast Channel (BCH). This is used to transmit information (random access codes, cell access slots, cell type transmit diversity, etc) specific to the UTRAN in a given cell and it is mapped onto the Primary Common Control Physical Channel (P-CCPCH) which is a downlink data channel only.

- Forward Access Channel (FACH). This carries downlink control information to terminals known to be located in the given cell. It is used to transmit a small amount of downlink packet data. There can be more than one FACH in a cell even mapped onto the same Secondary Common Control Physical Channel (S-CCPCH), which may supports slow power control.

- Paging Channel (PCH) carries data relevant to the paging procedure and can be transmitted in a single cell or several cells according to the system configuration. It is mapped onto S-CCPCH.

- Random Access Channel (RACH) carries uplink control information, such as a request to set up an RRC connection it is used also to send small amounts of uplink packet data. It is mapped onto PRACH.

- Uplink Common Packet Channel (CPCH) carries uplink packet-based user data. It supports uplink inner loop power control, with the aid of a downlink Dedicated Physical Control Channel (DPCCH). Its transmission may span several radio frames and it is mapped onto Physical Common Packet Channel (PCPCH).

- Downlink Shared Channel (DSCH) carries dedicated user data and/or control information and can be shared in time between several users. As a pure data channel it is always associated with a downlink DCH. It supports downlink inner loop power control based on the associated uplink DPCCH. It is mapped onto Physical Downlink Shared Channel (PDSCH).

The common transport channels needed for basic operation are RACH, FACH, and BCH while DSCH and CPCH may or may not be used.

### Physical Channels

A physical channel is identified by a specific carrier frequency, scrambling code, channelization code (optional), duration and, on the uplink, relative phase (0 or /2). In UMTS the transmission of a physical channel in normal mode is continuous, but in compressed mode it is interrupted to allow the UE to monitor cells on other FDD and from other radio access technologies. A detailed description of the physical channels is found in the 3GPP TS 25.211 specification.

#### Uplink Physical Channels

- Dedicated uplink physical channels: There are two types of uplink dedicated physical channels, the uplink Dedicated Physical Data Channel (uplink DPDCH) and the uplink Dedicated Physical Control Channel (uplink DPCCH).
The DPDCH and the DPCCH are I/Q code multiplexed within each radio frame.
The uplink DPDCH is used to carry the DCH transport channel. There may be zero, one, or several uplink DPDCHs on each radio link.
The uplink DPCCH is used to carry control information generated at Layer 1. The Layer 1 control information consists of known pilot bits to support channel estimation for coherent detection, transmit power control (TPC) commands, feedback information (FBI), and an optional transport format combination indicator (TFCI). The transport format combination indicator informs the receiver about the instantaneous transport format combination of the transport channels mapped to the simultaneously transmitted uplink DPDCH radio frame. There is one and only one uplink DPCCH on each radio link.

- Physical Random Access Channel (PRACH): The PRACH is used to carry the RACH transport channel. The random access transmission is based on a Slotted ALOHA approach with fast acquisition indication.

- Physical Common Packet Channel (PCPCH): The PCPCH is used to carry the CPCH transport channel. The CPCH is like the RACH with fast power control and longer allocation time, and with the possibility of using higher bit rates to transfer larger amounts of data with a more controlled access method. It is intended to carry packet switched user data in the uplink direction. One of its main advantages is a short access delay with a high bit rate, which makes it especially suitable for burst data.

#### Downlink Physical Channels

- Dedicated downlink physical channels: There are two types of downlink dedicated physical channels, the downlink Dedicated Physical Data Channel (downlink DPDCH) and the downlink Dedicated Physical Control Channel (downlink DPCCH). Both channels are time multiplexed with complex scrambling.

- Common Pilot Channel (CPICH): There are two types of common pilot channel, the primary and secondary CPICH. There are transmitted at a fixed rate (30 Kbps, spreading factor 256) and carry a predefined symbol sequence.
The primary common pilot channel (P-CPICH) is characterized by a fixed channelization code (CCh, 256, 0) and is always scrambled using a primary scrambling code. There is one P-CPICH per cell and is broadcast over the entire cell. The Primary CPICH is a phase reference for the following downlink channels: SCH, Primary CCPCH, AICH, AP-AICH, PICH, CD/CA-ICH, CSICH, DL-DPCCH for CPCH and the S-CCPCH. By default, the Primary CPICH is also a phase reference for downlink DPCH. The UE is informed by higher layer signaling if the P-CPICH is not a phase reference for a downlink DPCH.
The secondary common pilot channel (S-CPICH) is characterized by an arbitrary channelization code with a spreading factor of 256 and is scrambled by either a primary or a secondary scrambling code. In a cell there may be no, one, or several S-CIPICHs. Each S-CPICH may be transmitted over the entire cell or over only a part of the cell.

- Primary Common Control Physical Channel (P-CCPCH): The P-CCPCH is a fixed rate (30 Kbps, spreading factor 256) downlink physical channel used to carry the BCH. It is a pure data channel characterized by a fixed channelization code (CCh, 256, 1). The P-CCPCH is broadcast over the entire cell and is not transmitted over the first 256 chips of each slot, when the P-SCH and S-SCH are transmitted instead.

- Secondary Common Control Physical Channel (S-CCPCH): The S-CCPCH is used to carry the transport channel FACH and PCH, which can be mapped onto the same S-CCPCH (same frame) or onto separate S-CCPCHs. The S-CCPCH spreading factor ranges from 256 (30 Kbps) down to 4 (1920 Kbps).

- Synchronization Channel (SCH): The SCH is a downlink signal channel used for cell search. The SCH consists of two sub channels, the Primary and Secondary SCH. The 10 ms radio frames of the Primary and Secondary SCH are divided into 15 slots, each of length 2560 chips.
The primary synchronization channel (P-SCH) consists of a modulated code of length 256 chips. The primary synchronization code (PSC) is transmitted once every slot, which allows downlink slot synchronization in a cell and is identical in every cell of the system.
The secondary synchronization channel (S-SCH) consists of a 15 sequence repeatedly transmitted of modulated codes of the length 256 chips. This sequence permits downlink frame synchronization and indicates which of the code groups the downlink primary scrambling code belongs to.

- Physical Downlink Shared Channel (PDSCH) is used to carry the downlink shared transport channel (DSCH). The DSCH is targeted to transfer bursts non real time packet switched data. The basic idea of the DSCH is to share a single downlink physical channel, i.e. an orthogonal downlink channelization code, between several users.

- Acquisition Indicator Channel (AICH) is a fixed rate (SF=256) physical channel used to carry Acquisition Indicators (AI). The Acquisition Indicator corresponds to signature on the PRACH. Once the base station has received a preamble, the same signature that has been detected on the PRACH preamble is then sent back to the UE using the AICH.

- Access Preamble Acquisition Indicator Channel (AP-AICH) is a fixed rate (SF=256) physical channel used to carry AP acquisition indicators (API) of CPCH. AP acquisition indicator APIs corresponds to AP signature transmitted by UE.

- Collision Detection Channel Assignment Indicator Channel (CD/CA-ICH) is a fixed rate (SF=256) physical channel used to carry CD Indicator (CDI) only if the CA is not active, or CD Indicator/CA Indicator (CDI/CAI) at the same time if the CA is active. CD/CA-ICH and AP-AICH may use the same or different channelization codes.

- Paging Indicator Channel (PICH): The PICH is a physical channel used to carry paging indicators (PI). This channel is transmitted at a fixed rate (SF=256) and is always associated with a S-CCPCH, where the paging channel (PCH) is mapped. One PICH radio frame of length 10 ms consists of 300 bits

- CPCH Status Indicator Channel (CSICH) is a fixed rate (SF=256) physical channel used to carry CPCH status information. A CSICH is always associated with a physical channel used for transmission of CPCH AP-AICH and uses the same channelization and scrambling code.

## RRC States and State Transitions

This section will provide a brief overview of the enumerated states and transitions of the UE. The UE comprises two main states Idle Mode and Connected Mode. Figure 3 shows the RRC states in the UTRA RRC Connected Mode, including transitions between UTRA RRC connected mode and GSM connected mode for CS domain services, and between UTRA RRC connected mode and GSM/GPRS packet modes for PS domain services. It also shows the transitions between Idle Mode and UTRA RRC Connected Mode and furthermore the transitions within UTRA RRC connected mode.

Figure 3: RRC States and State Transitions

In the following paragraphs the Idle and UTRA RRC Connection Mode are described as well the transition states within the UTRA RRC Connected Mode.

### General Description of Idle Mode

When a UE is switched on, a public land mobile network (PLMN) is selected and the UE searches for a suitable cell of this PLMN to camp on. The particular PLMN to be contacted may be selected either automatically or manually.

The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

The UE searches for a suitable cell of the chosen PLMN and chooses that cell to provide available services, and tunes to its control channel. This choosing is known as "camping on the cell". The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the registration area of the chosen cell. If the UE finds a more suitable cell, it reselects onto that cell and camps on it. If the new cell is in a different registration area, location registration is performed. If necessary, the UE will look for more suitable cells on other PLMNs at regular time intervals, which is referred to as PLMN-reselection. This is described in (3GPP TS 22.011: "Service accessibility").

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The purpose of camping on a cell in idle mode is fourfold:

- It enables the UE to receive system information from the PLMN.

- When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

- If the PLMN receives a call for the registered UE, it knows (in most cases) the registration area of the cell in which the UE is camped. It can then send a "paging" message for the UE on control channels of all the cells in the registration area. The UE will then receive the paging message because it is tuned to the control channel of a cell in that registration area and the UE can respond on that control channel.

- It enables the UE to receive cell broadcast services.

If the UE is unable to find a suitable cell to camp on, or the USIM is not inserted, or if the location registration failed, it attempts to camp on a cell irrespective of the PLMN identity and enters a "limited service" state in which it can only attempt to make emergency calls.

The idle mode tasks can be subdivided into three processes:

- PLMN selection and reselection;

- Cell selection and reselection;

- Location registration.

The relationship between these processes is illustrated in Figure 4.

Figure 4: Overall Idle Mode Process

### UTRA RRC Connected Mode

The UTRA RRC Connected Mode is entered when the RRC connection is established. The RRC connection is defined as a point-to-point bi-directional connection between RRC peer entities in the UE and the UTRAN characterized by the allocation of a UMTS Radio Network Temporary Identity (U-RNTI). A UE has either zero or one RRC connection. The RRC connection establishment procedure can only be initiated by the UE sending an RRC connection request message to the radio access network. The event is triggered either by a paging request from the network or by a request from the upper layers in the UE. When the network releases the RRC connection, the signaling link and all radio bearers (RBs) between the UE and the UTRAN are released.

#### URA_PCH or CELL_PCH State

In the URA_PCH or CELL_PCH neither a DCCH nor DTCH (both logical channels) are available. In both states no uplink activity is possible and the UE is known by the SRNC on a cell level. The UE moves into the CELL_FACH state first if paged by the network or through any uplink access e.g. initiated through by the UE for cell reselection (Cell Update Procedure). In the URA_PCH or CELL_PCH state the UE shall perform the following actions:

- if the UE is "in service area":

- maintain up-to-date system information as broadcast by the serving cell

- perform cell reselection process

- perform a periodic search for higher priority PLMNs

- monitor the paging occasions and PICH monitoring occasions

- receive paging information on the PCH mapped on the S-CCPCH selected by the UE

- act on RRC messages received on PCCH and BCCH

- perform measurements process according to measurement control information

- maintain up-to-date Broadcast/Multicast Control (BMC) data if it supports Cell Broadcast Service (CBS)

- run periodical URA update if the UE is in URA_PCH or for periodical cell update if the UE is in CELL_PCH (Timer T305)

- if the UE is "out of service area":

- perform cell reselection process

- start control timer for UE T316 when the UE detects "out of service area" in URA_PCH or CELL_PCH state (3GPP TG 25.331).

- start control timer for UE T305, which means entering CELL_FACH or URA_PCH or CELL_PCH state. Reception of CELL UDPATE CONFIRM/URA UPDATE CONFIRM (3GPP TG 25.331)

The difference between the URA_PCH and the CELL_PCH state is that in the URA_PCH state the UE executes the cell update procedure only if the UTRAN Registration Area (URA) is changed. One cell can belong to one or several URAs in order to avoid ping-pong effects. The UE may be moved to the URA_PCH state via explicit signaling.

#### CELL_FACH State

In the CELL_FACH state the DCCH and if configured the DTCH (both logical channels) are available. In this state the UE shall perform the following actions:

- if the UE is "in service area":

- maintain up-to-date system information as broadcast by the serving cell

- perform cell reselection process

- perform measurements process according to measurement control information

- run periodical cell update (Timer T305)

- listen to all FACH transport channels mapped on the S-CCPCH selected by the UE

- act on RRC messages received on BCCH, CCCH and DCCH (logical channels)

- if the UE is "out of service area":

- perform cell reselection process

- run timers T305 (periodical cell update), and T317 (cell update when re-entering "in service") or T307 (transition to Idle mode)

#### CELL_DCH State

The CELL_DCH state is characterized by a dedicated physical channel, which is allocated to the UE in uplink and downlink. The UE is known on cell level according to its current active set. The CELL_DCH-state is entered from the Idle Mode through the setup of an RRC connection, or by establishing a dedicated physical channel from the CELL_FACH state. A PDSCH may be assigned to the UE in this state.

In the CELL_DCH state the UE shall perform the following actions:

- read system information broadcast on FACH

- perform measurements process according to measurement control information as specified in 3GPP TS 25.331 sub clause 8.4 “Measurement Procedures” and in clause 14 “Specific Functions”

- act on RRC messages received on DCCH

- act on RRC messages received on BCCH

### States Transitions

#### Transition from Idle Mode to UTRA RRC Connected Mode

The transition to the UTRA RRC Connected Mode from the Idle Mode can only be initiated by the UE by transmitting a request for an RRC Connection. The event is triggered either by a paging request from the network or by a request from upper layers in the UE.

When the UE receives a message from the network that confirms the RRC connection establishment, the UE enters the CELL_FACH or CELL_DCH state of UTRA RRC Connected Mode.

In the case of a failure to establish the RRC Connection the UE goes back to Idle Mode. Possible causes are radio link failure, a received reject response from the network or lack of response from the network (timeout).

#### Transition from CELL_DCH to Idle Mode

Transition to Idle Mode is realized through the release of the RRC connection.

#### Transition from CELL_DCH to CELL_FACH State

Transition to CELL_FACH state occurs when all dedicated channels have been released and when explicit signaled (e.g. Physical Channel Reconfiguration, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration, etc.).

#### Transition from CELL_DCH to CELL_PCH State

Transition to CELL_PCH state occurs when explicit signaled (e.g. Physical Channel Reconfiguration, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration, etc.).

#### Transition from CELL_DCH to URA_PCH State

Transition to URA_PCH state occurs when explicit signaled (e.g. Physical Channel Reconfiguration, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration, etc.).

#### Transition from CELL_FACH to CELL_DCH State

The transition occurs when a dedicated physical channel is established through explicit signaling (e.g. Physical Channel Reconfiguration, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration, etc.).

#### Transition from CELL_FACH to CELL_PCH State

The transition occurs when UTRAN orders the UE to move to CELL_PCH state, which is done through explicit signaling (e.g. Cell Update Confirm, Radio Bearer Reconfiguration, etc.).

#### Transition from CELL_FACH to Idle Mode

The transition of the UE to the idle mode occurs on release of the RRC connection.

#### Transition from CELL_FACH to URA_PCH State

The transition occurs when UTRAN orders the UE to move to URA _PCH state, which is done through explicit signaling (e.g. URA Update Confirm, Radio Bearer Reconfiguration, etc.).

#### Transition from CELL_PCH to CELL_FACH State

The UE is transferred to CELL_FACH state:

- by paging from UTRAN (PAGING TYPE1 message)

- through any uplink access

#### Transition from URA_PCH to CELL_FACH State

The UE is transferred to CELL_ FACH state:

- if RACH Uplink access is performed

- by paging from UTRAN ( PAGING TYPE1 message ).

The release of an RRC connection is not possible in the URA_PCH State. The UE will first move to CELL_FACH State to perform the release signaling.

## Synchronization  and Scrambling Codes

In UTRAN spreading codes (also called channelization codes) are used to differentiate physical channels from the same transmitter, and scrambling codes are used to differentiate transmitters. A UE needs to code and time synchronize to the scrambling code used by the serving cell before any communications with the base station can take place.

In an asynchronous system like UTRAN, using distinct scrambling codes in neighbor cells can only identify cells. UTRAN uses 512 scrambling code sets, allowing unique cell identification in a cluster of 512 cells. The process of searching for a cell and synchronizing to its downlink scrambling code is often referred to as cell search. Cell search is necessary after the UE has switched on (initial search) and during idle and active modes, for identifying new camping cells or hand over candidates, respectively.

Idle and active mode search is also called target cell search. The performance of cell search impacts the perceived switch-on delay (initial search), stand-by time (idle mode search), and link quality and capacity (active mode search). In addition to code and time uncertainty, the degree of frequency uncertainty can be large during initial search.

In UTRAN code and time synchronization is done with help of the downlink channels SCH and P-CPICH. These channels are permanently transmitted in downlink direction with constant power. Setting these powers too low increases the time needed by the UE for getting synchronized. This means for example that soft hand over links cannot be established fast enough. Thus UE’s near the cell edge suffer from bad link quality and might be dropped. On the other side if the power reserved for P-CPICH and SCH is set too high, fewer traffic can be established. Thus power settings for P-CPICH and SCH directly influence the system capacity.

The 512 scrambling code sets are organized in 64 groups, each of 8 code sets. Hence, cell search is normally carried out in three steps.

- Slot synchronization

- Frame synchronization and code-group identification

- Scrambling code identification.

For initial cell search, the ultimate goal is to decode the cell identity of the acquired signal. To achieve this, two extra stages are needed

- Frequency acquisition

- Detection of cell identity

From a performance point of view, initial cell search is more critical than target cell search. But initial cell search takes place much less than target cell search. Thus we focus on the first three steps for target cell search.

Network operators can control the target cell search performance up to some degree by proper scrambling code planning. It is revealed that there exists a trade-off between implementation complexity and receiver performance. This should be considered in scrambling code planning.

### Scrambling Codes

In UTRAN the scrambling codes are based on length 218-1 Gold sequences truncated to frame interval, which is 38400 chips for the chip rate 3.84 Mchips/s. A total of 218-1 = 262143 scrambling codes, numbered from 0 to 262142, can be generated, but due to the truncation a length of 213-1=8192 is used. The scrambling codes are divided into 512 code sets each of a primary scrambling code and 15 secondary scrambling codes. The primary scrambling codes are in number of n = 16i, where i = 0,, 511. The code set number is enter on a sector base into the OMC-U. The total number of scrambling codes that can be generated is m = 16i+o, where o (= 1,, 15) the index of the secondary code is.. Hence, scrambling codes m = 0,, 8191 are used. The code sets are further organized into 64 code groups, each consisting of 8 code sets. The jth scrambling code group can be determined by n = 168j+16k, where j = 0, , 63 and k = 0, , 7 (the code group set index indicates the primary scrambling code in the scrambling code group).

The following relationship allows calculating the primary scrambling code, the code group and the code group set index:

Code Set Number i, i = 0,, 511

Primary Scrambling Code  n = 16 i = 16 8 j + 16 k, 
                      j = 0,,63; k = 0,,7

Code Set Number  i = 8j+k

Code Group Set Index k = i modulo 8

Example: A cell uses the code set with number i = 111. According to the above this is equivalent to number n = 16111 = 1776 which represents the primary scrambling code. The number 1776 can also be written as 1776 = 16813+167.

111

104

7

8

13

Code group number j

Code Group Set Index K

### Synchronization Channels and Cell Search Procedure

For the cell search procedure, three channels are used, namely the P-SCH, the S-SCH and the P-CPICH. The P-SCH and the S-SCH are referred to as the SCH. Figure 5 bellow illustrates the slot and frame format of these channels. Each frame of 38400 chips (or 10 ms duration) is divided into 15 slots, of each 2560 chips (or 0.67 ms duration). Observe that both P-SCH and S-SCH have a 10% duty factor. The remaining 90% of each slot are reserved for the P-CCPCH. The power of each of the three channels is configurable and can be set to a different value.

P-SCH:

Power P2

S-SCH:

Power P1

P-CPICH:

Power P3

One slot (0.67 ms)

Figure 5 - Frame and slot structures for CPICH, P-SCH, S-SCH

The P-CPICH, which is used to carry the downlink common pilot symbols, is scrambled by the primary downlink scrambling code of the cell within each P-CPICH time slot, there are 10 pilot symbols, and each spread by 256 chips. All symbols are QPSK modulated, and the modulation values of the pilot symbols are known once the UE knows the frame boundary. The spreading sequence of the P-CPICH is taken from the OVSF codes maintaining mutual orthogonal between P-CPICH and other downlink channels also spread by OVSF codes.

Unlike P-CPICH, neither the P-SCH nor the S-SCH is scrambled by the primary downlink scrambling code. Instead of the OVSF codes, other sequences of length 256 chips are used.

There is one P-SCH sequence that is transmitted in the same position in every slot and is used for detecting the slot boundary. Furthermore, all cells use the same P-SCH sequence.

The S-SCH is used to identify the frame boundary and scrambling code group number. Unlike the P-SCH sequence, the S-SCH sequences vary from slot to slot. There are 16 S-SCH sequences, numbered from 1 to 16. A frame (15 slots) of 15 such S-SCH symbols forms a code word taken from a codebook of 64 code words. These 64 code words correspond to the 64 code groups used throughout the system. Thus a code group can be detected by identifying the code word transmitted in each S-SCH frame. Furthermore, the 64 code words are all chosen to have distinct code phase shifts, and any phase shift of a code word is different from all phase shifts of all other code words. With these properties, the frame boundary can be detected by identifying the correct starting phase of the S-SCH symbol sequence.

Example: If the code set 111 is used, this is the 7th code within the group 13. The 13th code word in the codebook is c = (1,8,14,10,14,1,15,15,8,5,11,4,10,5,4). These numbers indicate which S-SCH sequence is sent in the corresponding time slot.  The code word and hence the 15 S-SCH is repeated in each frame.

To minimize the cross-channel interference the 16 S-SCH sequences and the P-SCH sequence are mutual orthogonal.

#### Slot Synchronization

In stage 1 of the cell search process, the P-SCH matched filter is used to detect the slot boundary. The sequence s = (s0,,s255) of length 256 used in the P-SCH is the Kronecker product of two sequences of length 16.

The maximum absolute value of these output samples indicates the beginning of a slot. In order to get reliable statistics, the matched filter output has to be combined over many slots. It is suggested to add the outputs non-coherently, i.e. not the true outputs are added together but the absolute values of the outputs. Because no channel estimation can be applied in case of cell search, the phase difference between outputs of different slots leads to large errors if coherent combination is used. Taking the maximal value after non-coherent combination leads to the estimation of the boundary of the slot.

A typical value may be 15, which is the number of slots per frame. It is proposed to decompose the matched filter of length 256 in several short matched filters. This reduces the combination loss even further if the frequency inaccuracy is large which is normally the case during initial cell search.

The estimation of the slot boundary is used by the second stage in the cell search process.

#### Code Group Identification

In stage 2, the receiver operation start with correlating the received signal with all 16 S-SCH sequences, and then accumulates S-SCH correlations over N time slots according to the 64 RS code words used, each with 15 hypothesized frame boundaries. The total number of hypotheses is therefore 960 (64 code words times 15 shifts). At the end, the hypothesis with the largest accumulated metric is chosen as the candidate for frame boundary and code group, respectively. This candidate is given to stage 3 for scrambling code identification.
Hence, stage 2 in the cell search process can be decomposed in two sub stages:

- Matched filtering and S-SCH sequence detection

- Decoding of the Read Salomon Code (RS Code).

As in stage 1, non-coherent accumulation has to be used for combining S-SCH correlations from slot to slot if the phase rotations are not corrected. However, since the P-SCH is always transmitted along with S-SCH, the P-SCH correlation can be used as a phase reference to correct the phase inaccuracies of the S-SCH correlations. Both detection methods are described below.

First of all, the received signal must be passed through matched filters. As in stage 1 these filters are matched to the S-SCH sequences, no channel estimation is available. Because it is unknown to the receiver in advance which S-SCH sequence has been used in a certain time slot, 16 matched filters, each matched to one S-SCH sequence, has to be applied.

RS decoding can be done either using soft decision or hard decision values. Because only 960 hypotheses are to be checked, it is very likely that the UE processes soft information. This requires that per slot one soft information value is available for each of the 16 S-SCH sequences.  These reliabilities are calculated in a different way, depending whether phase correction is applied or not. The 64 code words used for the S-SCH form a non-linear subset. Because of the non-linearity and the low dimension of the subset, it is very likely that Maximum Likelihood decoding is used in the UE. This means that for all 960 hypotheses a metric is calculated and this hypothesis with the largest metric value is chosen as decoding result.

#### Scrambling Code Identification

After identifying the code group and achieving frame synchronization the P-CPICH is identified. It is very likely that the P-CPICH is used for this task because the pilot symbols lying under the scrambling code are known to the UE once the frame boundary is estimated. The second reason is that the power used for P-CPICH is higher than for other channels with know symbols, like DPCCH. Thus the performance of the third stage is best using P-CPICH.

As already mentioned the P-CPICH contains 10 QPSK modulated symbols. Each symbol is spread with the first OVSF code of length 256. This channel is continuously transmitted in each slot.

Each scrambling code group consists of 8 scrambling code sets. But not all of these codes may be used in the neighbor cells. In the target cell search process the scrambling codes used in the neighbor cells are known to the UE.

The filters applied in the third stage of the cell search process are matched to product of the pilot symbols and the scrambling codes used in the identified code group.

3

# RF Optimization Aspects

The most common challenges of RF Optimization are RF Coverage, Cell Breathing, Pilot Pollution, Around-the-Corner-Problem, Handovers and Missing Neighbors.

## RF Coverage

The RF coverage area is defined as an area where two conditions are met: pathloss < maximum allowed pathloss and Ec/Io > minimum signal-to-noise ratio (e.g. pathloss of 145 dB and Ec/Io of –11 dB for 384kbps CS Data). Pathloss and Ec/Io depend on Services and Quality defined for the specific network and can be verified during drive test RF Optimization. Antennas can be downtilted or re-oriented and NodeB output power can be adjusted to improve coverage.

The UE receive power is not an accurate measure of pathloss for spread spectrum technologies. The UE may have strong receive power due to many overlapping sectors but no pilot fulfills the above mentioned coverage conditions. Therefore the Ec/Io ratio and the Ec signal strength (connected to the pathloss) of the Primary Common Pilot Channel are used as an accurate measures for the RF coverage.

## Cell Breathing

An UMTS system has the characteristic of cell breathing, which is dependent on the network loading. An increase of the network load is associated with an increase of the network interference. High interference lowers the quality of service at the initial cell coverage border and thus shrinking the coverage area. Inversely, low load leads to low network interference, which increases the cell coverage. Please refer to Figure 6.

Due to cell breathing, RF Optimization is performed according to either the live or simulated network traffic load. Lucent Technologies OCNS generates downlink interference to simulate live traffic during the RF Optimization. On the uplink, an attenuator attached to the UE simulates the loading.

Cell A

Cell B

30% Loading

50% Loading

Figure 6 - Cell Breathing

## Pilot Pollution

Pilot Pollution is interference where there are overlapping pilots with similar signal strengths.  In this case, the lack of a dominant pilot causes low Ec/Io ratios.

Problem areas with low Ec/Io ratios may be mis-interpreted as pilot pollution areas and lead to iterative drive testing and unnecessary parameter changes in attempts to establish a dominant pilot. If the pilot doesn’t have sufficient Ec signal strength (extensive pathloss), the problem area is considered as a coverage hole. If the pilot has sufficient Ec signal strength (low pathloss), the problem area has pilot pollution. The RF Optimization engineer needs to determine whether the Ec/Io ratio is poor due to excessive pathloss or Pilot Pollution.

The optimization goal for Pilot Pollution is to increase the coverage of the dominant pilot and reduce coverage (here interference) of the weaker pilots. At the same time, continuous coverage through the soft handover must be ensured (advantage of soft handover gain).

The RF Optimization techniques for minimizing pilot pollution are antenna tilt changes and P-CPICH channel power changes.

Pilot Pollution is also considered if the number of present pilots is greater than the actual active set size of the UE. Present pilots which cannot be added into the active set cause interference.

Another aspect for interference is the multipath reception. Each received pilot is accompanied by 2-3 strong multipaths. The UE uses a rake receiver to exploit multipath reception. Since the rake receiver has a limited number of fingers, unused multipaths act as interference. Consequently, a six-finger rake receiver is fully occupied when receiving three pilots (each with 2 multipaths). The additional pilots and multipaths are interference.

Common trouble spots are bridges, upper floors in buildings, elevated highways, street intersections, and large bodies of water.

## Near-Far Problem

The Near-Far Problem occurs when an UE transmits on high power near the cell site, thus creating excessive interference for an UE located far away from the cell site. (See Figure 7)

The goal of the cell site is to receive all UE’s at equal signal strengths. Therefore fast closed loop power control is needed to direct mobiles to power up/down very quickly.

The optimization goal is to ensure that all power control algorithms are working properly. Power control parameters are tuned only when there are obvious power control failures. An indication of power control failure is if NodeB or the UE is always transmitting on full power despite satisfying block error rates (e.g. <5%).

Figure 7 - Near - Far Problem

## Around-the-Corner Problem

The Around-the-Corner Problem occurs when an UE travels beyond an obstruction and if there is significant downlink interference from a new sector with low pathloss. The downlink degrades momentarily until the handover is performed or the downlink power control reacts to compensate the interference. (See Figure 8)  When the UE goes into handover with the new cell site, fast power control will be needed to quickly reduce cell site transmit power.

The optimization goal is similar to the Near-Far goals.  The power control mechanism should be inspected to ensure it is functioning properly. The Around-the-Corner Problem is a continual and unavoidable issue. For known trouble spots, such as an elevated highway or a street intersection, one solution is to increase the handover margin to prevent the interferer.

Figure 8 - Around the Corner Problem

## Handover

Unnecessary delays in Handovers may cause uplink/downlink interference. Quick Handovers are required when there are rapid changes in pathloss between the UE and the sector due to fading. Also, unnecessary handovers due non-contiguous UMTS coverage or pilot pollution lead to excessive handover activity, require additional signaling resources, and increase downlink interference.

Time delays due to resource allocation (channel units, transmission links to RNC, OVSF codes) degrade call quality and reduce the throughput of data calls.

The goal here is to optimize the Handover performance by careful selection of thresholds and timers.

## Missing Neighbors

Missing Neighbors are pilots that are not in the neighbor list.  When received pilots, not in the neighbor list are present, the pilots cannot be added to the active set and thus these pilots will cause interference.

It is important that all received UMTS sectors are either eliminated or declared in the neighbor list.

4

# RF Drive Test Based Optimization Process

## Process Overview

The RF Drive Test Based Optimization process consists of three phases encompassing the following activities:

- Site Readiness

            - Spectrum Clearance

            - Antenna Audit

            - Sector Verification

            - Baseline Existing System

- RF Optimization Planning

            - Perform Parameter Audit

            - Validate Initial Neighbor Lists

            - Tool Readiness

            - Define Clusters

            - Drive Route Planning

- RF Optimization Execution

            - Cluster Optimization

            - System Verification

## Site Readiness

### General

The Site Readiness procedures are health checks that ensure all cells are operating as required. These procedures are usually performed after deploying a new network or when introducing new cell sites required for a professional service. Once these health checks have been performed and it a satisfactory performance of all cells can be guaranteed, these health checks are no longer a prerequisite of the RF Optimization.

### Spectrum Clearance Verification

The spectrum clearance assures that no external interference is present and sufficient guard bands are obeyed.

The detection of the interferences can be a very time consuming and difficult task once the UMTS system is up and running. It is desirable to have a very high degree of confidence that the spectrum is cleared prior to any testing.

More details about the Spectrum Clearance Verification are found in MSS-RF-EP088 Spectrum Clearance Verification Audit.

### Antenna Audit

This phase involves a series of quality checks to ensure proper installation of the antenna system. The number of audited cell sites will depend on the customer contract. There is a recommended audit minimum of 25% of cell sites in a cluster. The selection of cell sites must be done with input from the customer. Swapouts, cell sites of the underlying system being swapped, with low RF performance must be considered for audits.  If more than 50% of the audited antennas uncover installation errors, the remaining antennas in the cluster must also be audited. Based on the results and the confidence level of the antenna installations, the percentage of cell sites to be audited may vary for successive clusters.

The audit process consists of various inspections on antenna height, antenna azimuth, antenna type, antenna mechanical down-tilt, cable length, etc.

More details about Antenna Audit can be found in MSS-RF-D020 Antenna Audit.

### Sector Verification

The sector tests include verification of basic call processing functions including origination, termination and handover tests. Measurements are made on UMTS signal levels to verify that each sector is transmitting with the appropriate power levels and the correct scrambling code. These basic functional tests are intended to detect hardware -, software -, configuration - and parameter errors for each cell site in the cluster prior to further drive testing. Sector drives should be executed for each sector in the system or according to contractual obligations. Due to the simple nature of the drives, sector drives do not require customer approval.

The sector tests are performed using the measurement system CAIT3G including a UMTS test terminal. Once all data from the sector tests are collected the measurement data can be post-processed using LDAT3G. In case of existing sector problems, appropriate actions need to be undertaken. The sector test should be repeated until all tests succeed.

More details about Sector Verification can be found in MSS-RF-D011 Sector Verification.

### Baseline Existing System

The objective for the Baseline Existing System is to collect the RF performance metrics of the existing UMTS system equipment. Baseline driving should be performed prior to any RF Optimization activity and contains measuring of the Key Performance Indicators. Drive routes and Key Performance Indicators will be the same as the ones used later for System Verification. It is important to keep the drive routes and KPI’s identical for performance validation and comparison purposes. Drive routes and KPI’s must be agreed upon with the customer.

More information on Baseline Existing System can be found in MSS-RF-D019 Baseline System Performance.

## Optimization Planning

### General

The Optimization Planning phase ensures system and tool readiness for RF Optimization before beginning the actual drive testing.  For more information please refer to MSS-RF-D010 Optimization Planning.

### Perform RF Parameter Audit

At the beginning of the RF Optimization process, RF parameters must be inspected for consistency with the UMTS parameter catalogue.

The RF parameter settings used in the network can be obtained from the NDP project database. These settings are then audited using the UMTS parameter catalogue ParCat.

More information about the NDP project database and the UMTS ParCat database are found in the chapters 8.4 and 8.5. More details about the Parameter Audit can be found in MSS-RF-EP097U Verifying Translation Data for UMTS.

### Validate Initial Neighbor Lists

An important step within the RF Optimization preparation phase is associated with the neighbor list verification. The complete neighbor lists in the UMTS network are required to compare the neighbor relations with network design plots. Neighbor relations need to be verified for recent updates, validity and appropriateness. The recommended strategy is to have a minimum number of neighbor relations in the neighbor lists. The neighbor lists used in the network can also be obtained from the NDP project database.

More details about Neighbor List verification are found in MSS-RF-ER055U Verifying and Updating Neighbor Lists for UMTS.

### Tool Readiness

Appropriate drive test tools and post-processing tools, as discussed in chapter 5, need to be prepared for optimization. Please refer to MSS-RF-ER039U Setting Up Data Collection Tools for UMTS and to the individual tool M&P documents referenced in Appendix A.

### Define Clusters

Approximately 15-19 cell sites should be combined into one cluster. The actual number used is based on the network expansion as well on topographical environment. The clusters are selected to provide a center cell site with two rings of surrounding cell sites as shown below in Figure 9. It may be worthwhile to utilize natural barriers such as hills, water bodies, etc. for cluster separation to minimize overlap and influence between the clusters. A little cell site overlap should remain between each cluster to ensure continuity across the boundaries. Please refer to MSS-RF-ER034 Cluster Selection.

Figure 9 - Cluster Selection

### Drive Route Planning

Drive routes need to be defined for Sector Verification, Cluster Optimization and System Verification. Coverage prediction plots, morphology and clusters can define all drive test routes.

- Planning Drive Routes for Sector Verification

Each cell site is driven approximately around the entire cell site. The selected drive route should maintain a distance equal to 1/2 of the cell site radius. Sector drive routes usually do not require customer approval.

- Planning Drive Routes for Cluster Optimization

The routes for Cluster Optimization shall consist of major roads, highways and hotspots. Total time to drive all routes in a typical cluster should be approximately 6 to 8 hours. One control route per cluster is chosen to verify system performance. A control route is a subset of the optimization route and should be limited to about 1 to 2 hours. Additional border routes are chosen to verify system performance on overlapping cluster regions. A border route is chosen by the way it crosses the cluster borders without going into the cluster areas. Control routes and border routes are exit drives and must be agreed upon with the customer

- Planning Drive Routes for System Verification

The System Verification drive routes are used to collect the metrics for the Exit Criteria.  The routes are a combination of the cluster control routes and routes between the individual clusters. System Verification drive routes must be agreed upon with the customer.

For more details about Drive Routes Planning please refer to MSS-RF-ER042 Defining Drive Routes.

## RF Optimization Execution

### General

The RF Optimization Execution consists of drive tests, problem area identification, verification drives, and final drives to ensure completion of Exit Criteria. The core activity is to provide system tuning, as well as data collection and reporting. Design changes relating to cell site layout modifications or adding a new cell site may be considered if critical coverage holes are discovered during optimization.

Antennae corrective actions are more frequent for new deployments, such as Greenfield or Overlay scenarios. They are uncommon in existing systems, such as Network Expansion or Additional Carrier System. Fine tuning of the transmit powers is the most effective procedure in already optimized networks.

The quality and performance of a network depend on the actual load in the system. Unloaded network conditions can skew acceptance tests, as less interference is present. If traffic increases and the load rises, the network performance will be diminished and previous acceptance tests become invalid. It is the customer’s interest to verify and modify an UMTS network under loaded network conditions.

Methods to determine various problem categories including troubleshooting procedures are covered by the Troubleshooting Guideline. Please refer also to the M&P Optimization Process documents, which are referenced Appendix A.

### Cluster Optimization

Cluster Optimization consists of procedures performed on geographical groupings of cell sites that are large enough to have meaningful multi-cell site optimization. Several factors make it worthwhile to optimize the system in manageable sized clusters. There is a better focus on the area optimized, as smaller sector numbers make it easier it is to track the parameter changes and the impact of their performance. Another benefit to smaller cluster optimization is that multiple teams can optimize different clusters simultaneously. Each team is able to maintain focus on its cluster with minimal impact from other teams. In addition, smaller cluster optimization aids in speeding up the system tests for commercial operation. Optimization in equipped clusters can proceed simultaneously with installation of other clusters.

Cluster Optimization should be performed for network sections that are fully deployed. This avoids a re-testing of already optimized clusters in case cell sites are later integrated. All cell sites in the network (or a network section) are switched on. Each cluster is tested under unloaded and loaded conditions. If live traffic exists, cells in the tested clusters must be barred for all users except for the test users (optimization team).

During multiple cluster testing the optimization teams working advisable on neighbor clusters must coordinate activities especially regarding neighbor relations, loading conditions or eventual overshooting sites.

It is recommended to finish the unloaded cluster tests for all clusters within the network or network sections before continuing with the loaded cluster tests. After a small set of adjacent clusters pass the Exit Criteria, a border exit drive must be performed. The border exit drive is performed under loaded conditions in order to verify and confirm the Exit Criteria at the borders of the clusters.

The required data collection, processing and analysis tools for Cluster Optimization are a phone-based data collection tool kit including CAIT3G, a UMTS terminal, WINDS as well as the post-processing tool LDAT3G. In addition to the phone-based tool kit, the scanner-based tool Agilent can be used during the Cluster Optimization. The Agilent scanner is an important tool due to its multiple pilot measurement capability, which is especially useful for more in depth coverage analysis (e.g. pilot pollution) in challenging RF environments (e.g. large water-bodies, bridges, un-even terrain, etc.)

The Cluster Optimization consists of three phases:

- Unloaded Cluster Optimization

During the first Cluster Optimization phase, a measurement drive is performed under unloaded network conditions using the optimization route. Once the data from the first phase are collected, problem spots are identified and optimized. The unloaded drive test identifies coverage holes, handover regions and multiple pilot coverage areas. It also spots eventual overshooting sites (as interference is minimal) from areas belonging to neighbor clusters. The first pass might lead to correction of neighbor lists and adjustments of the fundamental RF parameters such as transmit powers and/or antenna azimuths and antenna tilts. The drive test information highlights fundamental flaws in the RF design under best-case conditions.

- Loaded Cluster Optimization

The second Cluster Optimization phase is performed under loaded conditions. The drive routes for the loaded Cluster Optimization will be exactly the same routes as those used for the unloaded measurement drives. Loaded testing produces a rise in the noise floor, which has the effect of shrinking the coverage area (cell breathing). This will cause an increase of negative Ec/Io values, identify potential coverage holes, result in higher BLER, result in lower mobility throughput, and more dropped calls. The objective is to fix the problems observed by the field teams. This involves the fine-tuning of RF parameters such as the transmit power or handover parameters. Antenna re-adjustments (e.g. down-tilts, azimuths, patterns/types or heights) are also occasional performed.

Problem areas may be re-driven after implementing changes. It is not recommended to drive a problem area more than three times. If the problem cannot be solved after three test drives, either a root cause analysis is performed or Cluster Optimization proceeds with the next cluster. It is generally not recommended to attempt resolution of complex time-intensive performance issues, such as location-specific problems like cell site equipment failures. For such problems, it is advisable to report the behavior and proceed with the next cluster. The problem cluster can be verified at a later stage.

- Cluster Performance Verification

In the third phase, the cluster performance is measured against the cluster Exit Criteria. The exit drive’s purpose is to verify and to confirm specific Exit Criteria demanded by the customer.

The final statistics from the cluster exit drive are presented to the customer for approval. These statistics contain plots as well as data in tabular form. The approval to exit the cluster is based on the terms of the contract. Approval with exceptions allows the cluster to be exited under the condition that any problems will be resolved during system wide optimization. If the cluster is not approved, loaded Cluster Optimization must be continued until the troubles are resolved. A report specifying the reasons why the exit drive did not pass the Exit Criteria is required.

For more details regarding Cluster Optimization please refer to MSS-RF-D012 Cluster Optimization.

### System Verification

System Verification is the final phase of the RF Drive Test Based Optimization activity and it focuses specifically on collecting overall performance statistics. System Verification will begin after all clusters in the UMTS network have been tested.  It is performed under loaded conditions with all cells activated. System Verification involves fusion of the previously optimized clusters and once again is required to demonstrate that Exit Criteria are met system-wide. The exact system test requirements are defined in the customer contract.

The System Verification will be a comprehensive drive test covering the major highways and primary roads in the defined coverage area.  There is a focus on the problem areas identified during the Cluster Optimization (System Verification driving routes). The procedures and analysis are identical to those used in Cluster Performance Verification. Performance data will be collected and statistics will be made to characterize coverage and performance over the entire network.

The system drive routes should not be used for optimization. System drives do not allow changing parameters due to side effects. Optimizing a system route can result in very good performance on the System Verification driving routes but poor performance elsewhere. System optimization is a continuation of Cluster Performance Verification. The main difference is the larger contiguous area of coverage.

Specific problem areas identified by the System Verification will be addressed on a case-by-case basis after the entire drive has been completed. Individual Cluster Optimization drives are used to fix existing coverage problems by adjusting transmit powers and neighbor lists. In extreme situations, handover thresholds, channel power parameters or other low tuning parameters may require modification. After any parameter changes are made, another drive test must be completed to ensure the surrounding regions are still performing properly.

The final statistics from the System Verification are presented to the customer for approval. The same tools that were used for Cluster Optimization will be used for the System Verification. At the end of the system-wide drive test phase, the RF Optimization procedure is considered complete.  The UMTS network is ready for live traffic testing leading into commercial service. Once significant loading with live traffic is present on the network, additional tuning of system parameters will be required to accommodate uneven traffic conditions (e.g. traffic hot spots) and other dynamic effects which cannot modeled with simulated traffic loading.

It is possible for problem areas to remain after System Verification is complete. An example would be a coverage hole that will be fixed by a future cell site addition. Such items must be well documented with solutions agreed upon by the customer.

For more details regarding the system Exit Criteria please refer to chapter 7 – UMTS Performance Metrics. For more details regarding the System Verification please refer to MSS-RF-D013 System Verification.

5

# RF Optimization Tools

## General

The ability to measure the performance metrics is key for the RF Optimizing. The tools can be classified into data collection and data analysis tools. The focus in this chapter is on the tools utilized for Drive Test Based Optimization. This does not negate the tools for Pre-Optimization (Ocelot) and Service Measurement Based Optimization (SPAT3G), for which a short introduction is given.

The following tools will be described:

- Drive Test Based Optimization

- Agilent Scanner

- CAIT3G

- WINDS

- LDAT3G

- Friendly Viewer

- UMTS Test Terminals

- RF Drive Test Kit

- Tools/features for simulated cell loading

- UMTS feature RF Call Trace

- Pre-Optimization Tools

  - Ocelot

- Service Measurement Tools

  - SPAT3G

Information and software for tools can be found under the Global RF Core Support Homepage.

## Agilent

The Agilent scanner drive test system (Agilent system E7476A) known as the VIPER system is a scalable integrated air interface measurement system used to obtain comprehensive RF measurement data. The full UMTS Agilent system planned for the future will include a 3GPP phone and is called NITRO, similar to the COBRA system.

The Agilent system measures the UMTS physical layer. The system performs absolute and relative channel power measurements of the Primary Synchronization Channel (PSCH), Secondary Synchronization Channel (SSCH) and the Primary Common Pilot Channel (P-CPICH). These three channel measurements can be performed for multiple scrambling codes at the same time. All channel measurements can be executed simultaneously or in conjunction with other types of scanner measurements such as spectral analysis, total channel power or CW power measurements

The UMTS channel power measurements can be executed without using a UMTS terminal and hence a U-SIM card, thus the Agilent scanner works autonomously.

An overview of the Agilent drive test system is shown in the Figure 10.

Figure 10 - Agilent Drive Test System

The Agilent drive test system consists of:

- Agilent Scanner (receiver E6455C) for UMTS

- Agilent Scanner PC software (PC is arbitrary but comes with the RF Kit)

- Hardware Security Key (License Dongle)

- GPS and W-CDMA antennas

The receiver measurement functions of the Agilent E7476A are composed of three primary elements:

- Spectrum Analyzer

- Channel and CW Power Measurements

- W-CDMA (UMTS) Scrambling Code Analyzer.

The Scrambling Code Analysis executes the three following measurements:

- TOP N – Measures all valid primary scrambling codes above a certain threshold and

reports the following main measurements:

  - Primary and Secondary Synchronization Code Power Ec and Ec/Io (P+SSCH)

  - Scrambling Code Power Ec and Ec/Io (CPICH)

  - Scrambling Code Group and Scrambling Code Number

  - RSSI (Io)

- User List – Measures user defined primary scrambling codes and reports
   the following main measurements:

- Scrambling Code Power Ec and Ec/Io (CPICH) of user defined SC’s

- Primary SCH Scan – Measures the primary synchronization channel and reports

the following main measurements:

- Multiple Primary Synchronization Code Power Ec and Ec/Io (PSCH)

- Delay Spread

Aside from the Channel and CW Power Measurement element the Spectrum Analyzer and Scrambling Code Analyzer are used during the Drive Test Based Optimization. The Spectrum Analyzer is used for spectrum clearance tests and the Scrambling Code Analyzer is used during sector tests, network pilot coverage analysis, neighbor verifications or interference analysis (pilot pollution). Nevertheless a complete Sector Verification or Cluster Optimization requires a phone-based drive test measurement system and the Agilent system is currently only a scanner based measurement system.

The Agilent Drive Test System is mainly used for pilot coverage surveys. Measurement data are collected within the cluster drives to analyze pilot coverage, best server and pilot pollution. MapInfo is used to post-process the data. Nevertheless Agilent’s diagnostic software allows also effective analyzes already during the measurement test drives.

Refer to MSS-RF-ER072U Data Collection Using the Agilent Scanner

## CAIT3G

The CDMA Air Interface Tester (CAIT3G) is a drive test diagnostic and collection tool. It can log and analyze both UL/DL messages (Layer 1-3) and DL performance measurements (e.g. DL BLER) from an UMTS test terminal.

CAIT3G can help to assess, debug and optimize the RF system performance by using real time displays and/or post-processing logging files using the post-processing tools LDAT 3G and Friendly Viewer.

The CAIT3G drive test system requires an UMTS test terminal based on the Qualcomm chipset and diagnostic software. Current UMTS test terminals used with CAIT3G are the test mobile QC5200 from Qualcomm and the PC data card Merlin from Novatel.

The current software version CAIT3G 2.9 has limited 3G functionalities, especially regarding real time displays. Therefore the CAIT3G system is currently primarily used to log air interface drive test data in order to post-process them using LDAT3G, utilized for Sector Verification, Cluster Optimization and System Verification.

An overall overview of the CAIT3G drive test system is shown in the Figure 11.

Figure 11 - CAIT3G Measurement System

The CAIT3G drive test system consists of:

- CAIT3G diagnostic software

- Security Key (License)

- UMTS test terminal

- CAIT3G data cable

- GPS antenna

- External W-CDMA antenna

CAIT3G’s key features are:

- View and evaluate RF conditions using Signal Graphs (e.g. TX/RX power), Temporal Analyzer (SC numbers including P-CPICH Ec/Io’s) and analyze displays (Layer1 State or Block Error Rate).

- Log position data along with over the air messages to and from the UMTS test terminal in order to post-process them with LDAT3G.

- Collect call performance statistics using the Call Monitor (e.g. call termination/origination).

- Automate phone testing by utilizing a large set of scripting commands from the Scripting Command window (e.g. automate call termination/origination intervals).

Refer to MSS-RF-EP080 Setup and Data Collection Using CAIT

## WINDS

WINDS is an UDP-based application that acts as a constant configurable data source and receiver. The key characteristic of the User Datagram Protocol (UDP) is that no retransmissions on the user protocol are performed. Thus, these UDP test data transfers are preferential used to test and characterize the air interface performance.

WINDS is one of the primary tool used during the Drive Test Based Optimization. It is used for Sector Verification, Cluster Optimization and System Verification. WINDS data measurements are usually post-processed by LDAT3G.

WINDS runs on a Server connected to the fixed (core) network, and will communicate with any number of WINDS applications installed on wireless terminals. Please see for an overview Figure 12. The key features of this tool that make it ideal for RF drive-testing purposes are as follows:

- Data transfers over both links (Downlink and Uplink) can be originated from the wireless terminal (there need not be an operator at the Server end).

- WINDS will recover from dropped calls even if the wireless terminal has assigned a new IP address on reactivation. The WINDS application at the Wireless Terminal is able to do this by detecting dropped calls and subsequently providing constant feedback to the Server with its latest IP address. This feedback process is terminated as soon as the Server successfully redirects the packet stream to the new IP address and the Wireless Terminal successfully receives these packets.

- WINDS supports comprehensive data logging at the application layer. This includes both per-packet logs as well as periodic rollups at user-defined intervals. The application will also log important events such as loss of data (typically when call drops). A summary at the end of the log provides details on the entire session such as overall throughput and number of lost calls.

- WINDS supports multiple adapter cards installed in the laptop and is able to read or send to each of these adapters explicitly. This is a very useful feature because the test plan often requires a separate phone to be used for downlink and uplink transfer. With WINDS, one laptop may be configured with multiple adapters to achieve this. As the RF toolkits are enhanced to support multiple Data phones, the existing WINDS multi-adaptor feature will become very useful.

- Data transfers may be initiated from a single WINDS Server to multiple WINDS Wireless Terminals over the Downlink (up to 10), as well as from multiple Wireless Terminals to the Server on the Uplink.

Figure 12 - WINDS Usage Overview

- WINDS displays detailed real-time statistics and other key information during the drive. This information includes: Instantaneous throughput, average session throughput, number of lost calls (loss of data transfer), number of application layer packet errors, etc.

- WINDS has the ability to read GPS time and position directly from a GPS receiver connected to the COM port. The log files will assign coordinates to each periodic and event log record based on these measurements.

- The timestamps will also be synchronized with GPS making it easy to correlate these logged data with that from other data collection tools such as CAIT3G. This time synchronization will also be useful when correlating logs with Layer 3 measurements using tools such as Friendly Viewer or LDAT3G.

- An extension of the LDAT3G post-processing tool is the ability to read and process WINDS log files. LDAT3G will be able to plot application layer statistics such as throughput and packet error rate. This may be used to troubleshoot application layer performance problems through correlating these application layer plots with Layer 1 to 3 plots and events.

The following features are expected to be integrated into WINDS in the future:

- Integration of ICMP functionality (ping) into WINDS to characterize absolute delays by measuring round-trip delay.

- WINDS has the ability to set up different call scenarios. An example would be originations drives where data calls are constantly being initiated and terminated at pre-defined intervals.

- Ability to model TCP protocol and its various tuneable parameters. This is still a long-term goal, and not expected to be implemented in the near future.

Refer to MSS-RF-ER111 How to Use WINDS

## LDAT 3G

Lucent’s Data Analysis Tool for 3G (LDAT3G) is used to post process the drive test data to aid in performance analysis by providing several metrics. Consequently LDAT3G helps to evaluate and determine the low performance areas in a UMTS network.

LDAT3G allows to create and visualize performance metrics from CAIT3G and WINDS. The CAIT3G and the WINDS measurement data can be analysed in LDAT3G independently.  gives an overview about the usage of LDAT3G and its possible inputs.

LDAT3G provides the ability to display and print the metrics in three different ways:

- Geographical mapping

- Time series plots

- Histograms

Several metrics can be combined in one map or a plot to facilitate correlation.

Figure 13 - LDAT 3G Post-Processing Tool

The following main metrics are available to LDAT3G menu for UMTS performance analysis:

Pilot Survey

- CPICH Ec/Io Max Finger

- CPICH Ec/Io Aggregate Finger

- Ec/Io Dominant Pilot

- Ec/Io Specific Pilot

- Dominant Pilot Scrambling Code

- Number of Pilots within Range of Best Pilot

- Number of Pilots within Active Set

- Total to Aggregate Active Ec/Io Ratio

Power Measurements

- UE Receive Power

- UE Transmit Power

- UE Max Transmit Power Margin

- Dominant Pilot Ec Power

Error Rate

- BLER Composite – Downlink

- BLER per Channel – Downlink

Throughput

- Physical Layer Data Rate Composite – Downlink

- Physical Layer Data Rate per Channel – Downlink

- Physical Layer Throughput Composite – Downlink

- Physical Layer Throughput per Channel – Downlink

WINDS Metrics

- All metrics plotted independently for Downlink and Uplink.

- Available Metrics (for Downlink and Uplink separately):

  - UDP Application Layer Throughput

  - UDP Application Layer Error Rate

  - Missed Packet Rate

  - Number of Missed Packet

- Available Events:

  - Data Transfer Start

  - Data Transfer Complete (Normal Termination)

  - Data Transfer Timeout

  - Data Transfer Abort by Sender

  - Data Transfer Abort by Receiver

RF Call Trace Metrics

- NodeB measurements (e.g. UL RSSI, DL carrier power)

- RNC measurements (e.g. UL SIR, Transmit Code Power)

Refer to the LDAT3G Homepage and MSS-RF-ER052 Data Processing Using LDAT3G and Other Tools

## Friendly Viewer

Friendly Viewer is a software tool from Qualcomm in order to parse and view binary CAIT3G log files. The Friendly Viewer will parse the log file and display the contents e.g. specific layer over the air messages using its own text browser. Friendly Viewer is most often used to debug suspected Layer 3 problems, or in-depth analysis of failure scenarios. Friendly Viewer is used specifically for root cause analysis during the RF Optimization. Refer to Friendly Viewer user documents.

## UMTS Test Terminals

The UMTS Qualcomm test mobile MSM5200 Form Factor Accurate (FFA) is the primary test terminal used during the RF Optimization, if not other specified by the customer.

Other terminals might be considered in the future based on their commercial availability, CAIT3G compatibility, maturity, and logging capabilities. But at this time no test terminals other than the QC 5200 are recommended for RF Optimization. The QC5200 is used together with the CAIT3G diagnostic software to collect mobile call information going over the air.

Figure 14 - QC 5200

At this point also the UMTS PCMCIA cards Merlin (Novatel) and Option Card (Option) should be mentioned. Both UMTS cards are used to perform UMTS data calls with the PC.  Unlike the Option Card, the Novatel Card Merlin is also compatible to CAIT3G and can be used to collect mobile call information going over the air using CAIT3G. (Creating log data is not working correctly yet, however it allows to monitor the pilot during a call and idle mode)

Refer to Lucent’s Qualcomm Support Homepage and to MSS-RF-ER085U Setting Up Handsets for UMTS for Optimization.

## Supplementary Tools

Some supplementary tools are helpful during the optimization process.

- MapInfo which is used in parallel to LDAT3G to plot Agilent measurement data.

- Lucent Cell Application Tool (LCAT), which can be used optional to generate the network cell sites and its sector information for LDAT3G displays.

## RF Tool Kit

Required RF Optimization equipment will be shipped as RF Tool Kits. Each RF Too Kit includes all the equipment needed to outfit one vehicle. The picture below shows the RF Too Kit.

Figure 15 – RF Tool Kit

A RF kit can include either a complete scanner based drive system (Agilent) or a complete phone based drive system (CAIT3G) including supplementary devises and a powerful Laptop.

The drive test kit is usually equipped with the following components:

- With either the CAIT3G drive test system or Agilent drive test system

- Laptop (for diagnostic software and data recording)

- UMTS test mobile (QC5200)

- Separate Attenuation boxes for uplink and downlink (used to compensate for vehicle penetration loss and for introducing simulated uplink load)

- Test mobile isolation box to provide RF isolation

- GPS unit

- Trimble GPS Placer

- Power supply power box for toolkit components

- External RF antenna

For details about the RF Tool Kit, please refer to Deployment Toolkit Team Homepage.

## Attenuation Box

Attenuators are used to provide common conditions of a phone that meets standard specifications. The attenuation box of the RF tool kit provides two attenuators, one in the transmit path and one in the receive path. Uplink and downlink attenuation is added to compensate / simulate cable losses, external mobile antenna gain or penetration losses. Additional uplink attenuation is added to simulate uplink loading (see next chapter).

Studies and tests are ongoing. For the time being please refer to PCS CDMA RF Optimization Handbook 602 subsection 321 together with the Attenuator Setting worksheet.

## Tools and features for simulated cell loading

Uplink Loading

Uplink loading can be simulated by implementation of attenuators at the mobile (UE) in the uplink path (mobiles TX path). The attenuation must be equivalent with the noise rise (cell load) specified in the link budgets and upon agreed by the customer. For example 5dB attenuation should be employed in the uplink to correspond to 68% uplink loading. The exact used attenuation is given in the ATP (Acceptance Test Plan) that is committed by SAE (System & Architecture Engineering).

Downlink Loading

Lucent Technologies’ Orthogonal Noise Channel Simulator (OCNS) is software feature that is used to simulate UMTS downlink loading by activating Channel Elements (CEs) in a given sector.

Primarily, OCNS will be used to test network capacity and to verify performance parameters of the network. This is particularly necessary during deployment and optimization phase, when the network is initially installed where cluster testing and system wide testing is required.

OCNS is initiated from the OMC-U. For a particular cell up to three OCNS objects can be created. Per OCNS object specific inputs are required:

  - OCNS mode (Voice/Data)

  - Spreading Factor (e.g. 128/32/8 for Voice [12.2kbps], Data [64kbps], Data [384kbps])

  - Average Power per User (e.g. 23, 27, 35 for Voice [12.2kbps], Data [64kbps], Data [384kbps])

  - Number of Users (e.g. 75 voice users to simulate 50% load)

Spreading Factor and Average Power per User (per service) are given in the traffic model committed by SAE (System & Architecture Engineering). The number of users gives the actual simulated cell load.

An additional OCNS option is the Power Variation Flag. This feature can vary the individual user’s power in each frame to simulate inner loop power control. It is recommended to be cautious using OCNS with power variation for RF Optimization purposes since appearing RF issues cannot be concluded anymore to the present cell load.

For more details about simulated download and uplink load refer to Translation Application Note OCNS and to MSS-RF-ER069U Performing Loaded Tests Using OCNS for UMTS.

## RF Call Trace

RF Call Trace is a Lucent Technologies developed UMTS software feature, which is used to monitor the downlink and uplink link performance of a UMTS call. This feature can be used during every phase of UMTS RF Optimization from sector testing through final system optimization.

The purpose of this feature is to collect RF measurements and related parameters during the operation of a call. The RF Call Trace capability allows the gather radio related information associated to one or more cells. The data and information collected is used to manage and support:

- Network optimization

- Performance Troubleshooting (RF as well as data performance)

- Warranty and exit criteria for customer contracts

The tracing functionality by the RNC is performed by the collection of signaling messages on the Uu, Iub and Iu interfaces. These traces will aid in the following objectives:

- Verification of call establishment

- Performance and maintenance of radio links

- Radio link quality and coverage

The approach to RF call trace within the UTRAN is for the OMC-U to invoke a trace on a particular UE by creating a RF trace object. The RF Call Trace measurement file is created at the RNC as soon as the RNC gets RF call trace initiation command from the OMC-U. It is named by the RNC, which includes the RNC identity, the IMSI and the time when the trace is initiated, e.g. <imsi_timestamp_rncid>. The RF call trace data can be collected from the RNC by a simple FTP session. The file is in XML format and a style sheet for this format is available.

The RF Call Trace measurements can be post-processed using LDAT3G. In order to display the RF Call Trace measurements location based within LDAT3G, a CAIT3G measurement file is required that was collected in parallel to RF Call Trace measurement file and that recorded also the GPS information. LDAT3G is able to correlate the two files by synchronizing the time stamps and hence to display all RF Call Trace measurements location based.

Once the RF call trace is invoked, information pertaining to radio links used by the UE are collected while connected to the RNC. This information is collected by measurements performed at the UE and NodeB at the request of the RNC. In addition, the RNC itself performs a set of measurements such as BLER.

Some of the most common RF Trace measurement are displayed below:

UE Measurements

- CPICH Ec/No (pilot SIR)

- CPICH RSCP (pilot level)

NodeB Measurements

- UL RSSI (per cell)

- DL Transmitted Carrier Power (per cell)

- UL SIR

RNC Measurements

- UL SIR target

- Radio Bearer ID

For details about the UMTS feature RF Call Trace, especially how to set up the RF Call Trace measurements please refer to MSS-RF-ER075U	RF Call Trace for UMTS and to the Translation Application Note RF Call Trace.

## Pre-Optimization Tool Ocelot

The first step in optimizing a UMTS network may be the usage of Lucent Technologies’ Ocelot tool. Ocelot optimizes a network in terms of maximum Coverage or Capacity - or, normally the optimum trade off of both. The tool calculates optimum Antenna tilt settings, azimuths and power settings to achieve the given Coverage and/or Capacity target. This can be done for the whole network or just for pre-defined clusters. Ocelot can be used prior to build out the network after finishing the nominal RF plan. Ocelot calculates the required parameters like Azimuth or tilts which influences the civil construction plans of the cell sites. Ocelot can also be used at a later stage in building out a network, when first clusters are getting shape during the site acquisition process.

Refer to M&P Refine RF Design Using Ocelot.

## Service Measurement Tools – SPAT3G

Service measurement tools are used after network launch when live traffic exists. These tools are usually installed at the OMC-U in order to collect network performance data according network performance counters. The GUI runs on a PC in order to generate service measurement metrics based on the data received from the OMC-U. Thus, a rapid identification of trouble spots is ensured. Such tools, like Lucent’s SPAT3G need to be used.

SPAT3G is a System Performance Analysis tool that can be used to quickly trouble shoot and improve UMTS network performance. It easily gives the user access to a wealth of information at the system, cell, sector, and carrier level that can be displayed graphically or in tabular formats.

SPAT3G is used after the network launch when live traffic exists, and when the system needs improvements in performance. SPAT3G is not used during the initial optimisation phase carry out before the network launch.

SPAT3G software is composed of two components - the OMC-U script and the PC GUI tool.  The OMC-U script is installed and runs in the OMC-U in order to collect network performance data according network performance counters (live traffic via hourly service measurements) while the PC GUI runs on the PC in order to generate service measurement metrics based on the data received from the OMC-U.

The SPAT3G PC GUI provides performance network data or statistics for one or multiple metrics as single charts and/or geographical maps on a cell, sector, and carrier level. Please refer to the figure below that shows an example of a geographical output.

SPAT3G is currently under development. As soon more information are available this chapter will be updated.

Please refer to M&P MSS-RF-C005 Service Measurement Based RF Optimization.

6

# Generation of Voice & Data Sessions

## General

An UMTS system requires various data or voice tests to verify the network performance. It is always preferable to perform these tests under loaded conditions, having either artificial load or live traffic conditions. Artificial load is produced for the downlink by Lucent Technologies’ OCNS, and for the uplink by appropriate attenuators together with the measurement equipment.

This chapter discusses the generics of the different applications used together with the measurement equipment CAIT3G and WINDS. For running the application a Laptop is used, for data application a server within the core network shall be available.

Final measurement procedures will be obtained from the ATP (Acceptance Test Plan) that is committed by SAE (System & Architecture Engineering).

## Voice Call

Currently voice measurement tests using QC5200 and CAIT3G are based on origination and termination test calls only. For fixed data rates of the voice channel (e.g.12.2kbps) the Mean Opinion Score (MOS) might be efficient for testing the voice quality. MOS is a measure of the voice quality based on the acoustic assessment by the user.

For AMR (Adaptive Multi-Rate) voice channels a full rate voice call would be required to best assess and hence optimize the performance of voice calls (e.g. BLER). Up to now no special feature or application like Lucent Technologies’ CDMA feature “Markov Call (full rate call)” is known. Tools like QVoice for UMTS which can automatically generate and analyze voice calls will be introduced here later.

For assessing and optimizing the voice performance an individual agreement with the customer shall be made.

## UDP Transfers

UDP is the preferred application layer protocol for data load generation in all RF Optimization procedures. The preferred UDP test tool is “Winds” which is a by Lucent developed application’s running on a Windows PC (Server Machine) connected to the Gi Interface. The same software is installed on the UE test equipment (Client Laptop) to form the end-to-end connection. One server can support a number of clients.

Once the Winds application is set up and launched on the data server side, both uplink and downlink transfers can be initiated from the mobile side without an operator at the server end. Uplink and downlink data streams can be defined independently, both in terms of the throughput and packet sizes. In case of a call drop, Winds shall recover even if the IP address of the mobile terminal gets reassigned.

For more information regarding the WINDS tool please refer to chapter 5.4.

## FTP Transfers

FTP sessions are not recommenced for testing except the operator insists that TCP/IP protocol must be used instead of UDP. FTP uploads or downloads from or to a FTP server are often simpler to use in the sense that no physical or remote presence is required on the network side.

If FTP transfers are used, optimization work can be done normally with down - and uploads of data files (preferable TEXT files).

Essential for any FTP-based testing is the correct setting of TCP and PPP parameters at the laptop. Except for the appropriate permissions at the FTP server, another prerequisite for FTP testing is to have several data files available at the network side for downloads, as well as at the laptop for uploads. On the server side a file of around 10Mbytes should be prepared. On the laptop, a file of around 1Mbytes should be prepared in the similar manner. While 10MB and 1MB files cover most drive testing needs during optimization, in practice it is often convenient to prepare a series of files on both sides with different sizes, i.e. 1MB, 3MB, 10MB and 30MB on the server side, and 100kB, 300kB, 1MB and 3MB on the terminal side. As a rule of thumb, the file size should be chosen in such a manner that the downloads last at least one minute but not more than five minutes.

Larger files for example will be needed during sector throughput tests at the Sector Verification phase. Smaller files will be required in multi-user tests when several terminals are sharing the bandwidth. As a general guidance, the file sizes should be scaled down proportional to the number of users so that download times between one and three minutes are achieved. Note that the attempt by several terminals for downloading the same file from the data server often results in a file sharing violation report. A simple practice for multi-user tests is the generation of series of files with suffixes corresponding to the numbers assigned to each test laptop and invoke different files from each one based on the laptop number.

Both uplink and downlink FTP transfers can be initiated via a user-friendly GUI interface or from the DOS window by entering the common FTP commands. Current experience is that it is more convenient to create batch files for uploads and downloads with shortcuts on the desktop.

## Ping

A simple and very useful command for testing data connections is making the command:

<ping –t [IP address of Data Server]>

from a batch file or a DOS window.

Ping with “–t” option would keep on sending the ICMP echo request message continuously which can test the connectivity, packet error rate and the round-trip-time to the pinged server. Instead of “-t” option a “-n” option with a number can be used to generate a number of pings and stop. Default pings are 32 bytes long (can be changed with the “–l” option to specify number of bytes to be sent. Larger pings are often useful when the backbone network connectivity is tested. ICMP requests are sent once per second (no option to change the period in Windows 2000, but can be changed in most Unix implementations). Another useful option is “-w”, which specifies the wait time for ICMP echo in milliseconds. To pause ping with –t option, one can do Cntrl+Break in which case ping shows statistics for the number of packets sent and received, percentage of packets in error, and the average roundtrip- time. Cntl+C shows the same statistics but stops the pinging process. With the default ping size and 1-second repetition period the equivalent throughput is rather low (~500bps), so that the process can ran in parallel to any UDP or TCP/IP application with negligible throughput impact. Pinging is useful in early testing to show quickly if there are any problems with RF links.

## Test Scripts

The plan is to incorporate test scripts which automatically originate and terminate data calls. Data calls can be periodically originated and terminated for specific data rates with specified duration and pause in-between.

Such scripts will be provided within CAIT3G or WINDS. Up to now no scripts were used during the RF Optimization, therefore this section will be updated as soon experience is gathered.

7

# UMTS Performance Metrics

## General

Specific quality and performance criteria within an UMTS network are assessed by certain measures and events. Such assessments may be used as a general health check on a network or in warranty situations where it is important to ascertain whether the deployed network is achieving a level of performance consistent with customer design requirements.

These specific measures and events are the performance metrics that are composed of a series of quality indicators. Since there is a high amount of quality indicators used for function and performance tests, a subset of key indicators is chosen that best can represent the quality and performance of an UMTS network. These Key Performance Indicators (KPI’s) are utilized for RF Optimization.

The Key Performance Indicators are used for warranty purposes to meet contractual and technical objectives. These Metrics shall be clear, simple, and measurable. The exact warranty targets, also called Exit Criteria used in the RF Optimization are customer and market specific. It is expected that prior to the actual RF Optimization values will be defined based on the design criteria for the market. For this reason specific Exit Criteria cannot be provided in this guideline. Final acceptance values and precise measurement procedures will be obtained from the ATP (Acceptance Test Plan) that is committed by SAE (System & Architecture Engineering).

The following chapter describes the Key Performance Indicators, the associated methods of measurement and warranty targets generally used during the optimization process of both voice and data. Metrics vary depending on the contract and additional unlisted metrics may be necessary.

A summary of Key Performance Metrics is given below. For each one a target is set.

Voice:

- Channel Block Error Rate (BLER)

- Drop Call Rate

- Call Success Rate (originations and terminations)

Data:

- Single UE throughput in an unloaded system

- Single UE throughput in a loaded system

- Delay

## Collecting Key Performance Data

Key performance data need to be collected during drive tests so they can be evaluated against Exit Criteria. Final acceptance drives are usually conducted on a per-cluster basis and on a per-system basis and are referred to as cluster exit drive and system exit drive respectively.

All tests should be performed using test vehicles equipped with a test kit using a UMTS test mobile (UE) and a PC for data transfers. If not other specified by the customer the following equipment is proposed:

- Lucent’s optimization drive test kit includes the Qualcomm 5200 test mobile and a Laptop for the data transfers.

- The diagnostic software CAIT3G for collecting the performance data.

- The test mobiles (UE) shall have fixed attenuators connected between the transceiver and the antenna to compensate for vehicle penetration loss and additional vehicle height.  Additional attenuation may be added to simulate system load.

- LDAT3G for the data analyses and for producing final plots and statistics for quality and performance acceptance.

The performance data should be collected for both uplink and downlink and shall be gathered within the design coverage area that is agreed upon with the customer. The design coverage area consists of those locations where coverage exists as determined and provided by the design prediction. Coverage plots for the clusters and the system shall be available prior to any test drive.

There are minimum data collection requirements necessary to receive accurate analysis, which will also be obtained from the ATP (Acceptance Test Plan).

In-building coverage via external UMTS infrastructure shall not be tested as part of acceptance; however, any building penetration margins specified in the design (i.e., in the link budget) may be verified at the street level by adjusting the attenuation in the test van setup.

For purposes of data collection and analysis the routes shall be divided into spatial subdivisions called geographic bins. These bin size shall be agreed upon with the customer. During data collection test routes shall be driven or sampled at speeds agreed with the customer to be representative for subscriber behavior.

Regarding the in the previous mentioned specific bin size, LDAT3G has the capability to transform the collected measurement data into geographical bins of a certain size. The data points per geographic bin are averaged and then evaluated against the Exit Criteria.

## System Loading

The network must perform properly under loaded conditions. Therefore the majority of the performance data shall be collected under loaded network conditions. Simulated load shall be introduced separately for uplink and downlink. The levels of simulated load will also be obtained from the ATP (Acceptance Test Plan).

## Key Performance Indicators

### General

The following sections present a set of KPI’s for Voice and Data. They are applicable for both Packet Switched (PS) and Circuit Switched (CS) services. Further differentiations for the specific service classes (i.e. QoS classes) will be added in future versions.

### Voice KPI’s

#### Block Error Rate (BLER)

Under simulated DL/UL load the BLER shall be measured for downlink or uplink (preferable for downlink). A continuous call shall be maintained along the test drive route. If the call drops a new call shall be set up immediately. During the call measurement the error rate per time bin is computed and collected using CAIT3G. These data are post-processed using LDAT3G where the time bin values within a geographic bin are averaged within a certain bin size.

For acceptance the average BLER shall not exceed x%.(Common values might be 2%-5%.)

#### Drop Calls

Under simulated DL/UL load, a sequence of test calls shall be placed along the test drive route. Those calls that reach the connected state and the number that drop prior to a hold time of e.g. 90 seconds shall be recorded. The drop call rate shall be the ratio of drop calls to the total number of calls that entered a connected state.

Drop Call Rate = Drop Calls / Total Calls *100

For acceptance the drop call rate shall be x% or less. (Common values might be 2%-5%.)

#### Call Success Rate

The all Success Rate for call originations and terminations are defined as follows.

Originations

Under simulated DL/UL load, a sequence of valid test calls shall be placed along the test drive route.  A valid test call is defined as a correctly dialed call to a non-busy number at the switch. The origination success rate shall be computed as the ratio of the total number of successful originations to total number of valid test calls.  A successful origination shall be defined as a call that reaches the voice state.

Origination Success Rate = Successful Originations / Total Originations *100

For acceptance the origination success rate shall meet or exceed x%. (Common values might be 90%-95%.)

Terminations

Under simulated DL/UL load, a sequence of valid test calls shall be placed to the test mobile (UE) along the test drive route.  A valid test call is defined as a correctly dialed call at the switch to the test mobile (UE). The termination success rate shall be computed as the ratio of total number of successful terminations to the total number of valid test calls.  A successful termination shall be defined as a call that reaches the disconnected state.

Terminations Success Rate = Successful Terminations / Total Terminations *100

For acceptance, the terminations success rate shall meet or exceed x%. (Common values might be 90%-95%.)

### Data KPI’s

#### Unloaded Throughput

Under unloaded network conditions, a continuous data transfer shall be set up along the test drive route using e.g. a 384 kbps bearer. The average mobile (UE) throughput shall be computed. If the bearer drops it shall immediately be re-established.

For acceptance tests, the observed average value of throughput for the active e.g. 384 kbps bearer shall satisfy:

- Mean of distribution is greater than or equal to ‘x’ kbps.

- Standard deviation of distribution is less than or equal to ‘x’ kbps.

The observed average value of throughput shall fall within the standard deviation of the mean. The mean distribution and standard deviation can be computed by e.g. LDAT3G.

#### Loaded Throughput

Under simulated DL/UL load, a continuous data transfer shall be set up along the test drive route using e.g. a 384 kbps bearer. The average mobile (UE) throughput shall be computed.

For acceptance tests, the observed average value of throughput for the active e.g. 384 kbps bearer shall satisfy:

- Mean of distribution is greater than or equal to ‘x’ kbps.

- Standard deviation of distribution is less than or equal to ‘x’ kbps.

The observed average value of throughput shall fall within the standard deviation of the mean.

#### Round Trip Delay

The round trip delay shall be measured via repeated pinging along the test drive routes. The payload used for ping shall be small, e.g. 64 Bytes.

Round-trip delay shall be measured using a computer connected via a mobile terminal to a server on the GGSN. The delay measurements shall be taken under unloaded conditions.

The delays for the different bearer shall be characterized separately. [E.g. for 64 kbps uplink and 64 kbps downlink (64/64) and 64 kbps uplink and 384 kbps downlink (64/384)].

For acceptance, e.g. 95% of the delays measured within the design coverage area with a 64 kbps uplink and a 64 kbps downlink (64/64) shall not exceed ‘x’ milliseconds.

### Summary Tables for Acceptance Tests

The following KPI tables are proposals for Cluster and System exit drives. The examples for data KPI’s are only related to two bearers, 64k/64k and 64k/384k.

#### Voice:

| BLER | UL/DL |  | Dropped calls |  |
| --- | --- | --- | --- | --- |
| Number of samples (see note 6) | / |  | Number of equivalent 90s calls |  |
| Mean | / |  | Number of drops |  |
| Standard deviation | / |  | Dropped call percentage (see note 3) |  |
| Percent less than 1.5% | / |  | Loading condition (see note 1) | UL/DL |
| Percent less than 5% | / |  |  |  |
| Percent greater than 10% | / |  |  |  |
| Loading condition (see note 1) | / |  |  |  |

| Call originations |  |  | Call terminations |  |
| --- | --- | --- | --- | --- |
| Number of origination attempts (see note 3) |  |  | Number of termination attempts (see note 3) |  |
| Number of successful originations |  |  | Number of successful terminations |  |
| Origination success rate |  |  | Termination success rate |  |
| Loading condition (see note 1) | UL/DL |  | Loading condition (see note 1) | UL/DL |

#### Data:

| Unloaded data throughput UL/DL | 64k /64k |  | Loaded data throughput UL/DL | 64k/64k |
| --- | --- | --- | --- | --- |
| Number of throughput samples | / |  | Number of throughput samples | / |
| Mean | / |  | Mean | / |
| Standard deviation | / |  | Standard deviation | / |
| Loading condition (see note 1) | / |  | Loading condition (see note 1) | / |

| Unloaded data throughput UL/DL | 64k/384k |  | Loaded data throughput UL/DL | 64k/384k |
| --- | --- | --- | --- | --- |
| Number of throughput samples (see note 4) | / |  | Number of throughput samples (see note 4) | / |
| Mean | / |  | Mean | / |
| Standard deviation | / |  | Standard deviation | / |
| Loading condition (see note 1) | / |  | Loading condition (see note 1) | / |

| Unloaded PING (64Bytes) delay | 64k/384k |  | Loaded PING (64Bytes) delay | 64k/384k |
| --- | --- | --- | --- | --- |
| Number of throughput samples (see note 4) |  |  | Number of throughput samples (see note 4) |  |
| Mean (see note 2) |  |  | Mean (see note 2) |  |
| Standard deviation |  |  | Standard deviation |  |
| Loading condition (see note 1) | / |  | Loading condition (see note 1) | / |
| Unloaded PING (64Bytes) delay | 64k/64k |  | Loaded PING (64Bytes) delay | 64k/64k |
| Number of throughput samples |  |  | Number of throughput samples |  |
| Mean (see note 2) |  |  | Mean (see note 2) |  |
| Standard deviation |  |  | Standard deviation |  |
| Loading condition (see note 1) | / |  | Loading condition (see note 1) | / |

### Summary Tables for Internal Tests

The following Key Performance Indicators are used for RF Optimization to assess, optimize and verify system performance. For more information please also to the Troubleshooting Guideline.

#### Voice and Data

| UE transmit power UL/DL | Voice or Data ( / ) |  | Node B transmit power per UE UL/DL | Voice or Data ( / ) |
| --- | --- | --- | --- | --- |
| Number of samples (see note 8) | / |  | Number of throughput samples (see note 9) | / |
| Mean | / |  | Mean | / |
| Standard deviation | / |  | Standard deviation | / |
| Loading condition (see note 1) | / |  | Loading condition (see note 1) | / |

| Primary CPICH Ec/Io | Voice or Data ( / ) |  | Handover success rate | Voice or Data ( / ) |
| --- | --- | --- | --- | --- |
| Number of samples (see note 10) |  |  | Number of samples (see note 5) |  |
| Mean |  |  | Number of successful handovers |  |
| Standard deviation |  |  | Handover success rate | UL/DL |
| Loading condition (see note 1) | UL/DL |  |  |  |

| SIR downlink | Voice or Data ( / ) |  | SIR error downlink | Voice or Data ( / ) |
| --- | --- | --- | --- | --- |
| Number of samples (see note 11) | / |  | Number of samples (see note 12) | / |
| Mean | / |  | Mean | / |
| Standard deviation | / |  | Standard deviation | / |
| Loading condition (see note 1) |  |  | Loading condition (see note 1) |  |
| Please attach raw data file |  |  | Please attach raw data file |  |

#### Data

| BLER | Data ( / ) |
| --- | --- |
| Number of samples (see note 6) | / |
| Mean | / |
| Standard deviation | / |
| Percent less than 1.5% | / |
| Percent less than 5% | / |
| Percent greater than 10% | / |
| Loading condition (see note 1) | / |

| PDP Context Success Rate | Data ( / ) |  | PDP Context Drop Rate | Data ( / ) |
| --- | --- | --- | --- | --- |
| Number of origination PDP Context attempts (see note 3) |  |  | Number of equivalent 90s calls |  |
| Number of successful PDP Contexts |  |  | Number of drops |  |
| PDP Contexts success rate |  |  | Dropped call percentage (see note 3) |  |
| Loading condition (see note 1) | UL/DL |  | Loading condition (see note 1) | UL/DL |

Notes:

- For “unloaded” tests, only one mobile is in the network. For “loaded” tests, additional attenuation is inserted in the uplink and OCNS is used for downlink. UL attenuation or DL artificial load shall be noted.

- Start 100 pings for each packet size. Calculate average and standard deviation.

- There can be 10 second gap between each call. Normal holding time for a call is about 90 seconds, dependent on the measurement object. In case of call originations/terminations and PDP context a holding time of 10 seconds may be efficient.  According to gap and holding time, as many as possible calls should be performed during the defined driving route.

- For drive tests, the number of throughput samples within a cell or sector should be at least 30 with a file of 2MB with both TCP applications (e.g. FTP) and UDP applications (e.g. WINDS). For Exit drives UDP applications should be used since the RF team cannot warrant the performance on the server side.

- The number of samples for handover tests should be as many as possible, please see note 3. Handover events can be monitored via layer 3 messages.

- Downlink BLER can be collected from CAIT using a Qualcomm UE.

- Uplink BLER can be collected from RF call trace.

- UE transmit power can be collected from CAIT using a Qualcomm UE.

- Node B transmit power per UE can be collected from RF call trace. Samples can be collected from at a frequency of 100 ms.

- Primary CPICH Ec/Io can be monitored via CAIT using a Qualcomm UE or by a scanner.

- In downlink, SIR and associated SIR_error can be collected from CAIT using a Qualcomm UE. In uplink, SIR and associated SIR_error can be collected from RF call trace.

8

# UMTS RF Parameters

## General

RF Optimization requires the adjustment of several RF parameters. Some of the parameters have complex interactions with one another affecting the system in terms of coverage, capacity and call quality. Therefore, it is important to prioritize the parameters depending on their ability to improve performance with minimal complexity and trade-offs.

Some parameters require frequent tuning depending on the local RF environment, thus these parameters have variable final values.  Certain parameters need very infrequent adjustments to influence performance on a system wide basis.

Regarding their tuning occurrence the RF parameters can be classified into three classes. The three classes are Primary, - Secondary - and Fixed parameters.

- Primary Parameters

These parameters require frequent adjustments, often from one cell site to another. These include:

- Pilot Channel Power

- Neighbor Lists

- Antenna Parameter (antenna configurations changes as antenna tilt, azimuth and height)

- Secondary Parameters

The secondary parameters can be used for further fine-tuning, especially in specific problem areas. These parameters include:

- Handover parameters

- Access parameters

- Cell Selection / Re-selection parameters

- Fixed Parameters

The fixed parameters are not normally adjusted during RF Optimization. Changing these parameters can create complex interactions in key system performance such as coverage, capacity, voice quality, data throughput, etc. The impact is not easily characterized or predictable, and can vary from network to network or within a network. These parameters should be adjusted only after consulting the subject matter experts, e.g. system engineering (SAE). These parameters include:

- Power Control parameters

- Load Control parameters

- Common Channel powers (e.g. AICH, P+SSCH, BCH)

- Access parameters which are not part of the secondary parameters

- Handover parameters which are not part of the secondary parameters

The following sections present the primary and secondary parameters. The fixed parameters are not discussed here.

It is important to be familiar with the UMTS Translation Application Notes. The following sections will be updated, as soon new knowledge is available.

## Primary Parameters

### Pilot Channel Power

The pilot transmit power can be adjusted to address coverage overshoot problems and multiple pilot coverage regions (pilot pollution). In some cases, transmit powers can be adjusted to provide fill-in coverage for weak signal strength areas. Additional alternatives such as changing antenna downtilts or antenna pattern can mitigate problem areas where there are insufficient transmit power adjustments.

| Lcell:primaryCPICH.power | Total power allocated to the primary common pilot channel (CPICH). This power is important for the 3rd stage of cell search. |
| --- | --- |

Please refer to TAN Cell Selection Reselection.

### Antenna Parameter

Antenna Parameter configuration can be adjusted for reasons addressed in the previous chapter. Antenna configuration modifications should be considered after power adjustments are not sufficient to eliminate a problem area.

### Neighbor Lists

The optimization of neighbor lists might be easier during cluster tests where there are a small number of active cell sites. The optimization of neighbor lists during system-wide tests is more complicated as all sectors are active and have complex interactions. The general approach is to reduce the neighbor relations to an optimum minimum.

## Secondary Parameters

The secondary RF Optimization parameters can have system-wide performance impacts, and should be adjusted with caution. For example, small changes in soft handover parameters can impact overall system capacity and channel element utilization.

### Cell Selection / Re-Selection

When a UE is switched on, the UE searches for a suitable cell. When the UE is in idle mode, the UE continuously compares the strength of the current pilot with all other available pilots. When the mobile finds another sector of sufficiently greater signal strength, it will perform a re-selection.

The parameters given below are used to change the requirements for a mobile to perform cell selection or re-selection. Special network conditions might require special settings for these parameters. For example, in dense networks (multiple pilots) mobiles shall meet high requirements for re-selection to avoid ping-pong effects and unnecessary signaling (interference). On the other hand, in loose networks (rural) the requirements for cell selection / re-selection shall be low to allow mobiles network access that experience higher pathloss.

Please refer to TAN Cell Selection Reselection.

- Cell Selection

| Lcell:sIB3Q QualMin | Minimum required quality level (Ec/No) a cell must have for selection in idle mode. |
| --- | --- |
| Lcell:sIB3Q RXLevMin | Minimum required RX level (RSCP) a cell must have for selection in idle mode. |

- Cell Reselection

When the UE is in idle mode, it constantly compares the signal strength of the current pilot with all other available pilots. When the mobile finds another sector of sufficiently greater signal strength, it will perform a reselection.

| Lcell:sIB3CellSelect ReselMeasure | Defines whether CPICH RSCP (1) or CPICH Ec/No (0) is used as the cell reselection quality measure. This value is consolidated. Recommended is CPICH Ec/No (0), so Qhyst2 and Qoffset2 is used. |
| --- | --- |

| Qoffset1s,n (dB) | This specifies the offset between the two cells (Hysteresis value prioritizing the ranking of the serving cell). It is used for TDD and GSM cells and for FDD cells in case the quality measure for cell selection and re-selection is set to CPICH RSCP. [Not specified for 01.01] |
| --- | --- |
| Qoffset2s,n (dB) | This specifies the offset between the two cells (Hysteresis value prioritizing the ranking of the serving cell). It is used for FDD cells in case the quality measure for cell selection and re-selection is set to CPICH Ec/No. [Not specified for 01.01] |
| Lcell:sIB3Qhyst1(dB) | Hysteresis value prioritizing the ranking of the serving cell (dB) if CPICH RSCP is used as quality measure. Value is not used if the recommended Ec/No quality measure is used. |
| Lcell:sIB3Qhyst2 (dB) | Hysteresis value prioritizing the ranking of the serving cell (dB) if CPICH Ec/No is used as quality measure. This value should be increased if ping-pong reselections are experienced. |
| Lcell:sIB3Q QualMin | This specifies the minimum required Pilot quality level in the cell in dB. |
| Lcell:sIB3Q RXLevMin | This specifies the minimum required Pilot RX level in the cell in dBm. |
| Lcell:sIB3TReselection (sec) | Time value [sec] which defines how long a neighbor cell must be ranked higher than the serving cell before this cell is selected. A longer time will make ping-pong reselections unlikely but also delay the reselection of a new sector. |

### Access Procedure

The mobile goes into the network access stage when a call is originated. The mobile selects the initial transmit power probe based on the received signal strength and some adjustment factors. The mobile subsequently ramps up the power on successive probe attempts for every unacknowledged probe. The purpose of the access parameters is to minimize the power transmitted while maximizing the access success rate and minimizing the access delay.

| PRACH:powerOffsetPm (dB) | The Power offset P p-m = Pmessage-control – Ppreamble, measured in dB Its setting has to be considered along with setting of parameter PRACH constant Val since low values of PRACH constant Val lead RACH preamble to be detected at a very low Eb/N0, thus to send the RACH message part at a power level too low to be decoded correctly by the NodeB. High values improve the RACH success rate of RACH but also increase of UL interference. |
| --- | --- |
| PRACH:constant Val (dB) | Too low values may cause RACH Preambles to be detected at very low Eb/N0. Otherwise with too high values, RACH Preambles may be transmitted with more power than is necessary to be detected, thus increasing the uplink interference. |
| PRACH:powerRampStep (dB) | The power-ramping factor Power Ramp Step [integer > 0]. Trade-off between minimising UL interference and speed up the successful UE access to the network. |

Please refer to TAN Access Procedures.

### Soft/Softer Handover

The tuning of Handover parameters has the following goals (Please refer to TAN Handover):

- Ensure a smooth coverage area (ensure HO when required)

- Use the advantage of Handover gain (find best trade-off between handover gain versus network capacity)

- Coordinate traffic distribution, e.g. for bridges or city highways

- Handover Measurement Reporting

| ReportingCriteria: rcReportingInterval | Periodicity of reporting handover events (1A&1C) from the UE to the UTRAN. This parameter affects the speed of the handover procedure. |
| --- | --- |
| IntraFreqMeas:measQty: filterCoefficient | Average window of the event measurements by the UE, means for which period an handover event is fulfilled before sending a measurement report to the UTRAN. A narrow window will speed up the handover procedure, but would also cause ping-pong effect. |

- Handover Algorithm

| ReportingCriteria:Reporting DeactivationThreshold | The parameter set the maximum active set size for which event 1A can be reported. The optimized value should consist in a good trade-off between uplink-limited scenarios (where better performances are achieved with higher number of active set cells) and downlink-limited scenarios (vice versa). The optimum point is also different for different data rates. |
| --- | --- |
| ReportingCriteria:Reporting ActivationThreshold | The parameter set the minimum active set size for event 1C to be reported. This parameter should be always set to a value greater than parameter Deactivation Threshold in order to allow Event 1C to be reported |
| ReportingCriteria: ReportingRange (dB) | The parameter set the reporting range for a candidate pilot to be able to trigger event 1A and 1B. The smaller the value of this parameter, the less restrictive it is. The optimized value should consist in a good trade-off between uplink limited and downlink limited scenarios. The value of this parameter must be considered along with parameter Hysteresis. |
| ReportingCriteria: Hysteresis (dB) | In case of for event 1A the given value decreases the global hysteresis factor for event 1A making the triggering condition less restrictive. In case of event 1B the given value increases the global hysteresis factor for event 1B making the triggering condition less restrictive. In case of event 1C this hysteresis parameter is the only one that controls the range for triggering condition and the given value means that the new candidate cell will have to be by the individual setting better than the worst pilot included in the active set for event 1C to be triggered. |
| ReportingCriteria: timeToTrigger (dB) | This parameter is used to limit the measurement signalling load avoiding Measurement Report message to be sent by the UE for a defined period of time during which the triggering conditions for the related event have existed. For event 1A and 1C it is not recommended to delay the occurrence of the event, whereas for event 1B delaying the occurrence is important in order to ensure that no link is dropped from the active set due to a short fade in the received signal. |
| Lcell:cellIndividualOffset | The offset on a cell basis can be either positive or negative. In case of events 1A or 1B the offset is added to the measurement quantity of the cell candidate to enter or leave the active set respectively. In case of event 1C one offset is added to the measurement quantity of the new candidate cell to enter the active set and one offset is added to the measurement quantity of the worst cell included in the active set. |

## UMTS ParCat

The UMTS ParCat is an Oracle database with a Web based GUI (Portal). It is an indispensable tool for RF Optimization. The UMTS ParCat database supports the managed object model of the OMC-U for describing a network element as specified by System Engineering (SAE).

It holds the following information:

- Objects

- Parameter Names

- Default Values

- Parameter Ranges

## NDP Database

The UMTS NDP Database is also an Oracle database with a Web based GUI (Portal). It is linked with the UMTS ParCat. The purpose of this database is to capture all required parameters for a UMTS network in order to transfer them to the OMC-U. This process is used only for UMTS network deployments and is not applicable for network modifications (e.g. RF parameters). The NDP database is used during the RF Optimization to the RF configurations currently used in the network. Network modifications need to be performed at the OMC-U.

For more information please refer also to UMTS RF Data Generation for NDP Database.

## OMC-U

The Operations & Maintenance Center (OMC-U) is state-of-the-art operations and maintenance platform designed specifically for UMTS networks. The OMC-U maintains and ensures configuration management, fault management and performance management.

For more information please refer to OMC-U homepage.

9

# Vocabulary

A

Access delay: The value of elapsed time between an access request and a successful access (source: ITU-T X.140).

Active Set: Set of radio links simultaneously involved in a specific communication service between an UE and a UTRAN access point.

Air Interface User Rate: The user rate between Mobile Termination and IWF. For T services it is the maximum possible AIUR not including padding. For NT services it is the maximum possible AIUR.

Application: an application is a service enabler deployed by service providers, manufacturers or users. Individual applications will often be enablers for a wide range of services.

Application protocol: The set of procedures required by the application.

B

Bearer: A information transmission path of defined capacity, delay and bit error rate, etc.

Bearer service: A type of telecommunication service that provides the capability of transmission of signals between access points.

C

Call: a logical association between several users (this could be connection oriented or connection less).

Cell: Radio network object that can be uniquely identified by a User Equipment from a (cell) identification that is broadcasted over a geographical area from one UTRAN Access Point.

Cell Site: Cell Site refers actually to the cell as previous explained. Within the UMTS RF Engineering the term Cell Site is rather used to specify unmistakable the site property with its cell UMTS equipment (NodeB). A cell site consists usually of three sectors.

Common Channel: A Channel not dedicated to a specific UE.

Connected Mode: Connected mode is the state of User Equipment switched on and an RRC connection established.

Connection: A communication channel between two or more end-points (e.g. terminal, server etc.).

Connection mode: The type of association between two points as required by the bearer service for the transfer of information. A bearer service is either connection-oriented or connectionless. In a connection oriented mode, a logical association called connection needs to be established between the source and the destination entities before information can be exchanged between them. Connection oriented bearer services lifetime is the period of time between the establishment and the release of the connection. In a connectionless mode, no connection is established beforehand between the source and the destination entities; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

Control channel: A logical channel that carries system control information.

Controlling RNC: A role an RNC can take with respect to a specific set of UTRAN access points. There is only one Controlling RNC for any UTRAN access point. The Controlling RNC has the overall control of the logical resources of its UTRAN access point's.

Core network: An architectural term relating to the part of 3GPP System which is independent of the connection technology of the terminal (eg radio, wired).

Coverage area: Area over which a 3GPP System service is provided with the service probability above a certain threshold.

Current serving cell: This is the cell on which the MS is camped.

D

Delivered QoS: Actual QoS parameter values with which the content was delivered over the lifetime of a QoS session.

Downlink: Unidirectional radio link for the transmission of signals from a UTRAN access point to a UE. Also in general the direction from Network to UE.

Drift RNS: The role an RNS can take with respect to a specific connection between a UE and UTRAN. An RNS that supports the Serving RNS with radio resources when the connection between the UTRAN and the User Equipment need to use cell(s) controlled by this RNS is referred to as Drift RNS.

E

Enterprise Systems: Information Systems that are used in the telecommunication organisation but are not directly or essentially related to the telecommunications aspects (Call Centre's, Fraud Detection and Prevention Systems, Invoicing etc).

H

Handoff Gain/Loss (dB): This is the gain/loss factor (+ or -) brought by handoff to maintain specified reliability at the cell boundary.

Handover: The transfer of a user’s connection from one radio channel to another (can be the same or different cell).

Handover: The process in which the radio access network changes the radio transmitters or radio access mode or radio system used to provide the bearer services, while maintaining a defined bearer service QoS.

Hard Handover: Hard handover is a category of handover procedures where all the old radio links in the UE  are abandoned before the new radio links are established.

I

Idle mode: The state of UE switched on but which does not have any established RRC connection.

Information Data Rate: Rate of the user information, which must be transmitted over the Air Interface. For example, output rate of the voice codec.

Inter cell handover: A handover between different cells. An inter cell handover requires network connections to be altered.

Interactive service: A service which provides the means for bi-directional exchange of information between users. Interactive services are divided into three classes of services: conversational services, messaging services and retrieval services (source: ITU-T I.113).

Intra cell handover: A handover within one sector or between different sectors of the same cell. An intra cell handover does not require network connections to be altered.

Iu: Interconnection point between an RNC or a BSC and a 3G Core Network. It is also considered as a reference point.

Iub: Interface between an RNC and a Node B.

Iur: A logical interface between two RNC. Whilst logically representing a point to point link between RNC, the physical realisation may not be a point to point link.

L

Logical Channel: A logical channel is an information stream dedicated to the transfer of a specific type of information over the radio interface. Logical Channels are provided on top of the MAC layer.

Logical O&M: Logical O&M is the signalling associated with the control of logical resources (channels, cells,) owned by the RNC but physically implemented in the Node B. The RNC controls these logical resources. A number of O&M procedures physically implemented in Node B impact on the logical resources and therefore require an information exchange between RNC and Node B. All messages needed to support this information exchange are classified as Logical O&M forming an integral part of NBAP.

M

Maximum output Power: For UE, this  is a measure of the maximum power supported by the UE (i.e. the actual power as would be measured assuming no measurement error) (TS 25.101). For FDD BS, the mean power level per carrier of the cell site measured at the antenna connector in a specified reference condition (TS 25.104). For TDD BS this refers to the measure of power when averaged over the transmit timeslot at the maximum power setting (TS 25.105).

Maximum Transmitter Power Per Traffic Channel (dBm): The maximum power at the transmitter output for a single traffic channel.

Mean bit rate: A measure of throughput. The average (mean) bit rate available to the user for the given period of time (source: ITU-T I.210).

Medium Access Control: A sub-layer of radio interface layer 2 providing unacknowledged data transfer service on logical channels and access to transport channels.

Mobile evaluated handover: Mobile evaluated handover (MEHO) is a type of handover triggered by an evaluation made in the mobile.  The mobile evaluates the necessity of handover based on the measured radio environment and based on criteria defined by the network.  When the evaluation meets the hand-off criteria the necessary information is sent from the mobile to the network.  The network then decides on the necessity of the handover based on the reported evaluation result and other conditions, e.g. uplink radio environment and/or availability of network resources, the network may then execute the handover.

Mobility: The ability for the user to communicate whilst moving independent of location.

Mobility Management: A relation between the mobile station and the UTRAN that is used to set-up, maintain and release the various physical channels.

N

Negotiated QoS: In response to a QoS request, the network shall negotiate each QoS attribute to a level that is in accordance with the available network resources. After QoS negotiation, the bearer network shall always attempt to provide adequate resources to support all of the negotiated QoS profiles.

Network connection: An association established by a network layer between two users for the transfer of data, which provides explicit identification of a set of network data transmissions and agreement concerning the services to be provided by the set (source: ITU-T X.213 / ISO-IEC 8348).

Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

Node B: A logical node responsible for radio transmission / reception in one or more cells to/from the User Equipment. Terminates the Iub interface towards the RNC.

O

Orthogonal Channel Noise Simulator a mechanism used to simulate the users or control signals on the other orthogonal channels of a downlink

P

Packet data protocol (PDP): Any protocol which transmits data as discrete units known as packets, e.g., IP, or X.25.

Packet transfer mode: Also known as packet mode. A transfer mode in which the transmission and switching functions are achieved by packet oriented techniques, so as to dynamically share network transmission and switching resources between a multiplicity of connections (source: ITU-T I.113).

Performance: The ability to track service and resource usage levels and to provide feedback on the responsiveness and reliability of the network.

Physical Channel: In FDD mode, a physical channel is defined by code, frequency and, in the uplink, relative phase (I/Q). In TDD mode, a physical channel is defined by code, frequency, and time-slot.

Point-to-point: A value of the service attribute "communication configuration", which denotes that the communication involves only two network terminations.

Protocol: A formal set of procedures that are adopted to ensure communication between two or more functions within the within the same layer of a hierarchy of functions (source: ITU-T I.112).

Q

QoS profile: a QoS profile comprises a number of QoS parameters. A QoS profile is associated with each QoS session. The QoS profile defines the performance expectations placed on the bearer network.

QoS session: Lifetime of PDP context. The period between the opening and closing of a network connection whose characteristics are defined by a QoS profile. Multiple QoS sessions may exist, each with a different QoS profile.

Quality of Service: The collective effect of service performances which determine the degree of satisfaction of a user of a service. It is characterised by the combined aspects of performance factors applicable to all services, such as;

- service operability performance;

- service accessibility performance;

- service retainability performance;

- service integrity performance; and

- other factors specific to each service.

R

Radio access bearer: The service that the access stratum provides to the non-access stratum for transfer of user data between User Equipment and CN.

Radio Access Mode: Mode of the cell, FDD or TDD.

Radio Access Network Application Part: Radio Network Signalling over the Iu.

Radio Bearer: The service provided by the Layer 2 for transfer of user data between User Equipment and UTRAN.

Radio frame: A radio frame is a numbered time interval of 10 ms duration used for data transmission on the radio physical channel. A radio frame is divided into 15 time slots of 0.666 ms duration.  The unit of data that is mapped to a  radio frame (10 ms  time interval) may also be referred to as radio frame.

Radio interface: The "radio interface" is the tetherless interface between User Equipment and a UTRAN access point. This term encompasses all the functionality required to maintain such interfaces.

Radio link: A "radio link" is a logical association between single User Equipment and a single UTRAN access point. Its physical realisation comprises one or more radio bearer transmissions.

Radio link addition: The procedure where a new radio link is added to the active set.

Radio Link Control: A sublayer of radio interface layer 2 providing transparent, unacknowledged and  acknowledged data transfer service.

Radio link removal: The procedure where a radio link is removed from the active set.

Radio Link Set: A set of one or more Radio Links that has a common generation of Transmit Power Control (TPC) commands in the DL

Radio Network Controller: This equipment in the RNS is in charge of controlling the use and the integrity of the radio resources.

Radio Network Subsystem Application Part: Radio Network Signalling over the Iur.

Radio Network Subsystem: Either a full network or only the access part of a UTRAN offering the allocation and the release of specific radio resources to establish means of connection in between an UE and the UTRAN.

A Radio Network Subsystem is responsible for the resources and transmission/reception in a set of cells.

Received Signal Code Power: Given only signal power is received, the average power of the received signal after despreading and combining.

Receiver Antenna Gain (dBi): The maximum gain of the receiver antenna in the horizontal plane (specified as dB relative to an isotropic radiator).

Receiver Noise Figure (dB): Receiver noise figure is the noise figure of the receiving system referenced to the receiver input.

Receiver Sensitivity (dBm): This is the signal level needed at the receiver input that just satisfies the required Eb/(No+Io).

Requested QoS: a QoS profile is requested at the beginning of a QoS session. QoS modification requests are also possible during the lifetime of a QoS session.

Required Eb/(No+Io) (dB): The ratio between the received energy per information bit to the total effective noise and interference power density needed to satisfy the quality objectives.

RRC Connection: A point-to-point bi-directional connection between RRC peer entities on the UE and the UTRAN sides, respectively. An UE has either zero or one RRC connection.

S

Seamless handover: "Seamless handover" is a handover without perceptible interruption of the radio connection.

Sector: A "sector" is a sub area of a cell. All sectors within one cell are served by the same cell site. A radio link within a sector can be identified by a single logical identification belonging to that sector.

Service: a component of the portfolio of choices offered by service providers to a user, a functionality offered to a user.

Service Area: The  Service Area is defined in the same way as the Service Area according to ITU-T Recommendation Q.1001 [4]. In contrast to the PLMN area it is not based on the coverage of a PLMN. Instead it is based on the area in which a fixed network user can call a mobile user without knowing his location. The Service Area can therefore change when the signalling system is being extended, for example.

Service bit rate: The bit rate that is available to a user for the transfer of user information (source: ITU-T I.113).

Serving RNS: A role an RNS can take with respect to a specific connection between an UE and UTRAN. There is one Serving RNS for each UE that has a connection to UTRAN. The Serving RNS is in charge of the RRC connection between a UE and the UTRAN. The Serving RNS terminates the Iu for this connection.

Shared Channel: A radio resource (transport channel or physical channel) that can be shared dynamically between several UEs.

(U)SIM code group: Combination of the (U)SIM code and the associated network subset and network codes (it is equivalent to the IMSI).

(U)SIM personalization: Enables a user to personalize a ME so that it may only be used with particular (U)SIM(s).

Soft Handover: Soft handover is a category of handover procedures where the radio links are added and abandoned in such manner that the UE always keeps at least one radio link to the UTRAN.

Suitable Cell: This is a cell on which an UE may camp. It must satisfy certain conditions.

T

Terminal: A device into which a UICC can be inserted and which is capable of providing access to 3GPP System services to users, either alone or in conjunction with a UICC.

Throughput: A parameter describing service speed. The number of data bits successfully  transferred in one direction between specified reference points per unit time (source: ITU-T I.113).

Total power dynamic range: The difference between the maximum and the minimum total transmit output power for a specified reference condition (TS25.104).

Traffic channel: A "traffic channel" is a logical channel which carries user information.

Transmitter Antenna Gain (dBi): The maximum gain of the transmitter antenna in the horizontal plane (specified as dB relative to an isotropic radiator.

Transport Block: Transport Block is defined as the basic data unit exchanged between L1 and MAC.  An equivalent term for Transport Block is “MAC PDU”.

Transport channel: The channels offered by the physical layer to Layer 2 for data transport between peer L1 entities are denoted as Transport Channels. Different types of transport channels are defined  by how and with which characteristics data is transferred on the physical layer, e.g. whether using dedicated or common physical channels.

U

UE Service Capabilities: Capabilities that can be used either singly or in combination to deliver services to the user. The characteristic of UE Service Capabilities is that their logical function can be defined in a way that is independent of the implementation of the 3GPP System (although all UE Service Capabilities are of course constrained by the implementation of the 3GPP System). Examples: a data bearer of 144 kbps; a high quality speech teleservice; an IP teleservice; a capability to forward a speech call.

Universal Subscriber Identity Module (USIM): An application residing on the UICC used for accessing services provided by mobile networks, which the application is able to register on with the appropriate security.

Uplink: An "uplink" is a unidirectional radio link for the transmission of signals from a UE  to a cell site, from a Mobile Station to a mobile cell site or from a mobile cell site to a cell site.

User: An entity, not part of the 3GPP System , which uses 3GPP System services. Example: a person using a 3GPP System mobile station as a portable telephone.

User-network interface: The interface between the terminal equipment and a network termination at which interface the access protocols apply (source: ITU-T I.112).

User Services Profile: Contains identification of subscriber services, their status and reference to service preferences.

Uu: The Radio interface between UTRAN and the User Equipment.

10

# Definitions Equations

Following RF relevant formulas are given as specified by 3GPP.

| CPICHIor | The ratio of the received energy per PN chip of the CPICH to the total transmit power spectral density at the Node_B (SS) antenna connector. |
| --- | --- |
| Ec | Average energy per PN chip. |
| EcIor | The ratio of the average transmit energy per PN chip for different fields or physical channels to the total transmit power spectral density. |
| Io | The total received power spectral density, including signal and interference, as measured at the UE antenna connector. |
| OCNS | Average energy per PN chip for the OCNS. |
| OCNSIor | The ratio of the average transmit energy per PN chip for the OCNS to the total transmit power spectral density. |
| P−CPICH | Average* energy per PN chip for P-CPICH. |

UTRAN measurement abilities

CPICH RSCP

| Definition | Received Signal Code Power, the received power on one code measured on the Primary CPICH. The reference point for the RSCP shall be the antenna connector of the UE. If Tx diversity is applied on the Primary CPICH the received code power from each antenna shall be separately measured and summed together in [W] to a total received code power on the Primary CPICH. |
| --- | --- |
| Applicable for | Idle, Connected Intra, Connected Inter |

UTRA carrier RSSI

| Definition | The received wide band power, including thermal noise and noise generated in the receiver, within the bandwidth defined by the receiver pulse shaping filter. The reference point for the measurement shall be the antenna connector of the UE. |
| --- | --- |
| Applicable for | Idle, Connected Intra, Connected Inter |

CPICH Ec/No

| Definition | The received energy per chip divided by the power density in the band. The CPICH Ec/No is identical to CPICH RSCP/UTRA Carrier RSSI. Measurement shall be performed on the Primary CPICH. The reference point for the CPICH Ec/No shall be the antenna connector of the UE. If Tx diversity is applied on the Primary CPICH the received energy per chip (Ec) from each antenna shall be separately measured and summed together in [Ws] to a total received chip energy per chip on the Primary CPICH, before calculating the Ec/No. |
| --- | --- |
| Applicable for | Idle, Connected Intra, Connected Inter |

Transport channel BLER

| Definition | Estimation of the transport channel block error rate (BLER). The BLER estimation shall be based on evaluating the CRC of each transport block associated with the measured transport channel after RL combination. The BLER shall be computed over the measurement period as the ratio between the number of received transport blocks resulting in a CRC error and the number of received transport blocks. When either TFCI or guided detection is used, the measurement “Transport channel BLER” may only be requested for a transport channel when the associated CRC size is non zero and at least one transport format in the associated transport format set includes at least one transport block. When neither TFCI nor guided detection is used, the measurement “Transport channel BLER” may only be requested for a transport channel when the associated CRC size is non zero and all transport formats in the associated transport format set include at least one transport block. The measurement “Transport channel BLER” does not apply to transport channels mapped on a P-CCPCH and a S-CCPCH. The UE shall be able to perform the measurement “Transport channel BLER” on any transport channel configured such that the measurement “Transport channel BLER” can be requested as defined in this section. |
| --- | --- |
| Applicable for | Connected Intra |

SIR

| Definition | Signal to Interference Ratio, is defined as: (RSCP/ISCP)SF. Measurement shall be performed on the DPCCH of a Radio Link Set. In compressed mode the SIR shall not be measured in the transmission gap. The reference point for the SIR measurements shall be the Rx antenna connector. where: RSCP = Received Signal Code Power, unbiased measurement of the received power on one code. ISCP = Interference Signal Code Power, the interference on the received signal. SF=The spreading factor used on the DPCCH. |
| --- | --- |

SIRerror

| Definition | SIRerror = SIR – SIRtarget_ave, where: SIR = the SIR measured by UTRAN, defined in section 5.2, given in dB. SIRtarget_ave = the SIRtarget averaged over the same time period as the SIR used in the SIRerror calculation. In compressed mode SIRtarget=SIRcm_target shall be used when calculating SIRtarget_ave. In compressed mode the SIRtarget_ave shall not be calculated over the transmission gap. The averaging of SIRtarget shall be made in a linear scale and SIRtarget_ave shall be given in dB. |
| --- | --- |

Transmitted carrier power

| Definition | Transmitted carrier power, is the ratio between the total transmitted power and the maximum transmission power. Total transmission power is the mean power [W] on one carrier from one UTRAN access point. Maximum transmission power is the mean power [W] on one carrier from one UTRAN access point when transmitting at the configured maximum power for the cell. Measurement shall be possible on any carrier transmitted from the UTRAN access point. The reference point for the transmitted carrier power measurement shall be the Tx antenna connector. In case of Tx diversity the transmitted carrier power for each branch shall be measured and the maximum of the two values shall be reported to higher layers, i.e. only one value will be reported to higher layers. |
| --- | --- |

Transmitted code power

| Definition | Transmitted code power, is the transmitted power on one channelisation code on one given scrambling code on one given carrier. Measurement shall be possible on the DPCCH-field of any dedicated radio link transmitted from the UTRAN access point and shall reflect the power on the pilot bits of the DPCCH-field. When measuring the transmitted code power in compressed mode all slots shall be included in the measurement, e.g. also the slots in the transmission gap shall be included in the measurement. The reference point for the transmitted code power measurement shall be the Tx antenna connector. In case of Tx diversity the transmitted code power for each branch shall be measured and summed together in [W]. |
| --- | --- |

Transport channel BER

| Definition | The transport channel BER is an estimation of the average bit error rate (BER) of the DPDCH data of a Radio Link Set. The transport channel (TrCH) BER is measured from the data considering only non-punctured bits at the input of the channel decoder in Node B. It shall be possible to report an estimate of the transport channel BER for a TrCH after the end of each TTI of the TrCH. The reported TrCH BER shall be an estimate of the BER during the latest TTI for that TrCH. Transport channel BER is only required to be reported for TrCHs that are channel coded. |
| --- | --- |

Physical channel BER

| Definition | The Physical channel BER is an estimation of the average bit error rate (BER) on the DPCCH of a Radio Link Set. An estimate of the Physical channel BER shall be possible to be reported after the end of each TTI of any of the transferred TrCHs. The reported physical channel BER shall be an estimate of the BER averaged over the latest TTI of the respective TrCH. |
| --- | --- |

Round trip time

| Definition | Round trip time (RTT), is defined as RTT = TRX – TTX, where TTX = The time of transmission of the beginning of a downlink DPCH frame to a UE. The reference point for TTX shall be the Tx antenna connector. TRX = The time of reception of the beginning (the first detected path, in time) of the corresponding uplink DPCCH/DPDCH frame from the UE. The reference point for TRX shall be the Rx antenna connector. Measurement shall be possible on DPCH for each RL transmitted from an UTRAN access point and DPDCH/DPCCH for each RL received in the same UTRAN access point. |
| --- | --- |

11

# Abbreviations

A

AICH	Acquisition Indicator Channel

AM	Acknowledged Mode

AN	Access Network

ARFCN	Absolute Radio Frequency Channel Number

AS	Access Stratum

ASC	Access Service Class

AWGN	Additive White Gaussian Noise

B

BCH	Broadcast Channel

BER	Bit Error Ratio

BLER	Block Error Ratio

C

CCCH	Common Control Channel

CCH	Control Channel

CCPCH	Common Control Physical Channel

Cct	Circuit

CCTrCH	Coded Composite Transport Channel

CDMA	Code Division Multiple Access

CI	Cell Identity

CPICH	Common Pilot Channel

CPCH	Common Packet Channel

CRC	Cyclic Redundancy Check

CS	Circuit Switched

CSD	Circuit Switched Data

CTCH	Common Traffic Channel

CW	Continuous Wave (unmodulated signal)

D

DCCH	Dedicated Control Channel

DCH	Dedicated Channel

DL	Downlink (Forward Link)

DPCCH	Dedicated Physical Control Channel

DPCH	Dedicated Physical Channel

DPDCH	Dedicated Physical Data Channel

DRAC	Dynamic Resource Allocation Control

DRNS	Drift RNS

DSCH	Downlink Shared Channel

DTCH	Dedicated Traffic Channel

E

ETS	European Telecommunication Standard

ETSI	European Telecommunications Standards Institute

F

FACH 	Forward Access Channel

FDD	Frequency Division Duplex

FER	Frame Erasure Rate, Frame Error Rate

FR	Full Rate

G

GGSN	Gateway GPRS Support Node

GTP	GPRS Tunneling Protocol

GTP-U	GPRS Tunnelling Protocol for User Plane

GUI	Graphical User Interface

H

HO	Handover

HSCSD	High Speed Circuit Switched Data

HTTP	Hyper Text Transfer Protocol

IM	Intermodulation

INAP	Intelligent Network Application Part

IP	Internet Protocol

ISCP	Interference Signal Code Power

ISO	International Organisation for Standardisation

ISP	Internet Service Provider

ITU	International Telecommunication Union

L

L1	Layer 1 (physical layer)

L3	Layer 3 (network layer)

LAN	Local Area Network

LLC	Logical Link Control

Lm	Traffic channel with capacity lower than a Bm

M

MAC	Medium Access Control (protocol layering context)

Mcps	Mega-chips per second

N

NAS	Non-Access Stratum

NBAP	Node B Application Part

NCELL	Neighbouring (of current serving) Cell

O

O&M	Operations & Maintenance

OCNS	Orthogonal Channel Noise Simulator

OSI RM	OSI Reference Model

OTA	Over-The-Air

OVSF	Orthogonal Variable Spreading Factor

P

P-CCPCH	Primary Common Control Physical Channel

P-CPIH	Primary Common Pilot Channel

PC	Power Control

PCCCH	Packet Common Control Channel

PCCH	Paging Control Channel

PCH 	Paging Channel

PCPCH	Physical Common Packet Channel

PCU	Packet Control Unit

PDCH	Packet Data Channel

PDN	Packet Data Network

PDP	Packet Data Protocol

PDSCH	Physical Downlink Shared Channel

PDTCH	Packet Data Traffic Channel

PDU	Protocol Data Unit

PHY	Physical layer

PhyCH	Physical Channel

PN	Pseudo Noise

PPCH	Packet Paging Channel

PPP	Point-to-Point Protocol

PRACH	Physical Random Access Channel

PS	Packet Switched

PSC	Primary Synchronisation Code

PSCH	Physical Shared Channel

PSPDN	Packet Switched Public Data Network

Q

QoS	Quality of Service

R

RA	Routing Area

RAB	Radio Access Bearer

RAN	Radio Access Network

RB	Radio Bearer

RL	Radio Link

RLC	Radio Link Control

RLCP	Radio Link Control Protocol

RLP	Radio Link Protocol

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RNSAP	Radio Network Subsystem Application Part

RR	Radio Resources

RRC	Radio Resource Control

RRM	Radio Resource Management

RSAT	Radio System Acceptance Test

RSCP	Received Signal Code Power

RSSI	Received Signal Strength Indicator

RWB	Resolution Bandwidth

RX	Receive

RXLEV	Received signal level

RXQUAL	Received Signal Quality

S

S-CCPCH	Secondary Common Control Physical Channel

S-CPICH	Secondary Common Pilot Channel

SAP	Service Access Point

SB	Synchronization  Burst

SCCH	Synchronisation Control Channel

SCH	Synchronisation Channel

SDCCH	Stand-Alone Dedicated Control Channel

SDU	Service Data Unit

SF	Spreading Factor

SFN	System Frame Number

SHCCH	Shared Channel Control Channel

SIR	Signal-to-Interference Ratio

SM	Session Management

SMS	Short Message Service

SMS-CB	SMS Cell Broadcast

SN	Subscriber Number

SP	Switching Point

SQN	Sequence number

SRB	Signalling Radio Bearer

SRNC	Serving Radio Network Controller

SRNS	Serving RNS

SS7	Signalling System No. 7

SSC	Secondary Synchronisation Code

STTD	Space Time Transmit Diversity

T

TA	Timing Advance

TC-TR	Technical Committee Technical Report

TCP	Transmission Control Protocol

TD-CDMA	Time Division-Code Division Multiple Access

TE	Terminal Equipment

TF	Transport Format

TN	Timeslot Number

TPC	Transmit Power Control

TPDU	Transfer Protocol Data Unit

TrCH	Transport Channel

TRX	Transceiver

TS	Time Slot

TSC	Training Sequence Code

TSG	Technical Specification Group

TSTD	Time Switched Transmit Diversity

TX	Transmit

TXPWR	Transmit PoWeR; Tx power level in the MS_TXPWR_REQUEST and MS_TXPWR_CONF parameters

U

UARFCN	UTRA Absolute Radio Frequency Channel Number

UARFN	UTRA Absolute Radio Frequency Number

UDD	Unconstrained Delay Data

UDP	User Datagram Protocol

UE	User Equipment

UI	User Interface

UL	Uplink (Reverse Link)

UM	Unacknowledged Mode

UMTS	Universal Mobile Telecommunications System

UP	User Plane

URAN	UMTS Radio Access Network

URB	User Radio Bearer

USCH	Uplink Shared Channel

USF	Uplink State Flag

USIM	Universal Subscriber Identity Module

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

UUI	User-to-User Information

V

VA	Voice Activity factor

W

WAP	Wireless Application Protocol

WCDMA	Wideband Code Division Multiple Access

WDP	Wireless Datagram Protocol

WLAN	Wireless Local Area Network

WTDD	Wideband Time Division Duplexing

APPENDIX A

References

This bibliography chapter is intended to provide additional literature to processes and tools addressed in this guideline. The references are listed in a chronological order.

Chapter 3 - RF OPTIMIZATION OVERVIEW

- UMTS RF Engineering Guidelines

- UMTS RF Troubleshooting Guidelines

- Drive Test RF Optimization (M&P MSS-RF-C003)

Chapter 5 - RF OPTIMIZATION PROCESS

- Optimization Planning (M&P MSS-RF-D010)

- Spectrum Clearance Verification Audit (M&P MSS-RF-EP088)

- Verifying Translation Data for UMTS (M&P MSS-RF-EP097U)

- Verifying Neighbor Lists for UMTS (M&P MSS-RF-ER055U)

- Cluster Selection (M&P MSS-RF-ER034)

- Defining Drive Routes (M&P MSS-RF-ER042)

- Setting Up Data Collection Tools for Drive Testing (M&P MSS-RF-ER039U)

- Baseline System Performance (M&P MSS-RF-D019)

- Sector Verification (M&P MSS-RF-D011)

- Antenna Audit (M&P MSS-RF-D020)

- Cluster Optimization (M&P MSS-RF-D012)

- System Verification (M&P MSS-RF-D013)

- Service Measurement-Based RF Optimization (M&P MSS-RF-C005)

Chapter 6 - RF OPTIMIZATION TOOLS

- Deployment Toolkit Team (User Documents)

- RF Performance Analysis Tools Group (User Documents)

- RF Tools By Technology (User Documents)

- UE Vendor Information (Qualcomm) (User Documents)

- Data Collection Using Agilent Scanner (M&P MSS-RF-ER072U)

- Setup and Data Collection Using CAIT (M&P MSS-RF-EP080)

- How to Use WINDS (M&P MSS-RF-ER111)

- Data Processing Using LDAT 3G and Other Tools (M&P MSS-RF-ER052CK)

- Setting Up Handsets for UMTS Optimization (M&P MSS-RF-ER085U)

- Refine RF Design Using Ocelot (M&P MSS-RF-D022U)

- Performing Loaded Tests Using OCNS for UMTS (M&P MSS-RF-ER069U)

- RF Call Trace for UMTS (M&P MSS-RF-ER075U)

- Service Measurement-Based RF Optimization (M&P MSS-RF-C005)

- Friendly Viewer (User Documents)

The indicated division within Idle Mode is only included for clarification and shall not be interpreted as states.